3GPP TSG-RAN WG4 Meeting #92                 	 R4-1908185

Ljubljana, Slovenia, August 26th – 30th, 2019

Agenda Item:	9.1.2.3

Source:	MediaTek Inc.

Title:	Consideration on sync raster design for NR-U

Document for:		Discussion

# Introduction

In last RAN4 (#91) meeting, the issue of sync raster for NR-U was discussed. Some agreements from the meeting report are listed below.

Agreement: To define reduced number of SS raster entries for NR-U. => Agreement on Friday Based on further RAN1 LS R4-1907801, RAN4’s common understanding is RAN1 SSB multiplexing scheme is waiting for RAN4 decision on the SSB placement. From RAN4 persepctive, NR-U sync raster shall be based on the global NR sync raster design in Rel-15. RAN4 intension is to reduce the number of SS entry per 20MHz for NR-U. Wheterh to restrict the SSB placement in the middle or edge will be defined in RAN4 considering the cell search performance.

In this paper, we provide our view on sync raster design.

# Discussion

NR-U may be deployed on different bands. But so far, only n46 was agreed. Therefore, we focus on n46 in this paper. In n46 which ranges from 5150MHz to 5925MHz, the frequency band has already been partitioned in to pre-defined channels (chunks), depending on bandwidth 10MHz, 20MHz, 40MHz, 80MHz or 160MHz. (Since RAN4 already agreed that 10MHz BW will not be used for standalone mode, we skip 10MHz in the following discussions.)

One of the design target of the sync raster is to guarantee that every possible channel allocation has at least one available SSB for UE for initial access. This target becomes very easy to be achieved in n46, because all channels were already pre-defined, as some example shown in Table 1. In this case, if all 20MHz channel is allocated with one sync raster, then all channels with different BWs will have at least one sync raster. To us, there is no intention to add a 2nd sync raster in each 20MHz channel. Therefore, in order to reduce the time and complexity for initial access (which is already expected to be increased due to LBT), it should be sufficiently enough to define only one sync raster in each 20MHz channel.

Proposal 1: In n46, define a single sync raster for each 20MHz channel.

Table 1. Examples for channels in n46

|  | Channel index |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20MHz | 40MHz | 80MHz | 160MHz |
| 5150–5170 | 32 | 34 & 38 |  |  |
| 5170–5190 | 36 |  | 42 | 50 |
| 5190–5210 | 40 |  |  |  |
| 5210–5230 | 44 | 46 |  |  |
| 5230–5250 | 48 |  |  |  |
| 5250-5270 | 52 | 54 | 58 |  |
| 5270-5290 | 56 |  |  |  |
| 5290-5310 | 60 | 62 |  |  |
| 5310-5330 | 64 |  |  |  |

Regarding the location of sync raster in each 20MHz channel, there was 2 options. One is to locate it at the end of the channel, and the other is at the center. Basically, each option has its own pros and cons, which are summarized in Table 2.

Table 2. Pros and cons for 2 options of sync raster placement

| Sync raster location | Advantages | Disadvantages |
| --- | --- | --- |
| channel edge | - Rate matching on RMSI may not be needed. Better RMSI decoding performance is expected due to consecutive DMRS in frequency domain. In Rel-15 baseline UE assumption, PDSCH does not need to rate match around SSB. Therefore, it brings less additional UE design effort. | - Degradation in UE reception performance at band edge. Initial access and measurement performance could be compromised. - 2 different raster may be needed for 15KHz and 30KHz SCS. More complicated spec is expected |
| channel center | - Better initial access and measurement performance - Raster can be shared by both 15KHz and 30KHz. | - Rate matching on RMSI may be needed, complicates UE design and compromising decoding performance. Additional design effort is needed for R15 UE to upgrade |

In our understanding, we slightly prefer to the channel edge placement to reduce UE’s design effort. But we are open for more discussions.

Proposal 2: The single sync is placed at the edge of the 20MHz band.

# Summary

In this paper, we provide our view on the sync raster design for NR-U. We have the following proposals.

Proposal 1: In n46, define a single sync raster for each 20MHz channel.

Proposal 2: The single sync is placed at the edge of the 20MHz band.

# References