3GPP TSG-RAN WG4 Meeting #92		R4-1908191

# Ljubljana, SI, 26th– 30th Aug, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.133 on TCI state switching requirements (Section 8.10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-08-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Definition of known TCI state is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated value of X in definition of known TCI state |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Definition of known TCI state is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 38.533 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change

## 8.10	Active TCI state switching delay

### 8.10.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section.

### 8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

-	TCI state switch is within X ms of last transmission of the resource for beam reporting/ measurement for the target TCI state

- X = 1280 for UE supporting Power Class 1

- X = 160 for UE supporting Power Class 2/3/4

-	The UE has sent at least 1 measurement report for the target TCI state

-	The TCI state shall remain detectable during the TCI state switching period

-	SNR of the TCI state is > -3dB

Otherwise, the TCI state is unknown.

Start of Change