3GPP TSG-RAN WG4 Meeting #92	R4-1908596

Ljubljana, SI, 26th– 30th Aug, 2019

Agenda item:	9.17.1

Source: 		NTT DOCOMO, INC.

Title: 	DM-RS configurations for normal demodulation test in FR1

Document for:	Discussion

1. Introduction

At RAN4#88, following agreements were reached regarding the number of DM-RS [1].

- DMRS

    - Type 1 single symbol front loaded

    - 1 additional DMRS for Low (5/10Hz) and Medium (100 Hz) Speed scenarios

      - i.e. for full DL slots OFDM symbols with DMRS: #2 and #9

    - 2 additional DMRSs for High (400 Hz) Speed scenarios

      - i.e. for full DL slots OFDM symbols with DMRS: #2, #6 and #9

    - FDM is applied between DMRS and data for test cases with 1 Layer and 2 Layers

In this contribution, we discuss issues on test coverage of DM-RS configurations for RAN4 test.

2. Discussion

## 2.1 Number of additional DM-RS

For the Rel. 15 UE demodulation, 1 additional DM-RS is used for the test cases with Doppler frequency of 5, 10 and 100 Hz, while 2 additional DM-RS is used for that of 400 Hz. The test design seems natural, since larger number of additional DM-RS is required for high mobility scenarios. On the other hand, in the commercial network, the number of high-speed UEs is not always large. For instance, for the urban and rural scenarios, most of the UEs are low-to-mid speed, while there are some high-speed UEs in trains or vehicles in highways. Ideally, gNB should estimate mobility for each UE and adjust the number of additional DM-RSs with UE-specific manner. However, at least for the early NR deployment, it is not easy to implement accurate UE speed estimation and RRC re-configuration of the number of additional DM-RSs. Moreover, if the number of additional DM-RS is UE-specifically changed, it will result in scheduling restriction for MU-MIMO operation, since UEs with different DM-RS configuration can’t be multiplexed together for MU-MIMO. With the reasons above, we think it is more natural to apply same number of additional DM-RS in a cell and the number of additional DM-RS should be one considering that most of the UEs are low-to-mid mobility.

Observation 1: Followings were specified for Rel. 15 FR1 UE normal demodulation tests.

- For low-to-middle Doppler scenarios, 1 additional DMRS tests (no 2 additional DM-RS tests)

- For high Doppler scenarios, 2 additional DMRSs tests (no 1 additional DM-RS tests)

Observation 2: For early NR deployment, it is typical to use same number of additional DM-RS within a cell.

Observation 3: Considering that most of the UEs are low-to-mid mobility, cell common number of additional DM-RS is 1, if applicable.

Proposal: Introduce high Doppler normal demodulation test, e.g., 400 Hz, with 1 additional DM-RS.

- Alt. 1: Introduce one additional test

- Alt. 2: Replace one high Doppler test with 1 additional DM-RS

## 2.2 Simulation results

We performed link-level simulation in order to verify the impact of the number of additional DM-RS. Simulation results will be added in a revised contribution.

3. Conclusion

In this contribution, we presented our views on DM-RS configurations. Our observations and proposal are shown as follows.

Observation 1: Followings were specified for Rel. 15 FR1 UE normal demodulation tests.

- For low-to-middle Doppler scenarios, 1 additional DMRS tests (no 2 additional DM-RS tests)

- For high Doppler scenarios, 2 additional DMRSs tests (no 1 additional DM-RS tests)

Observation 2: For early NR deployment, it is typical to use same number of additional DM-RS within a cell.

Observation 3: Considering that most of the UEs are low-to-mid mobility, cell common number of additional DM-RS is 1, if applicable.

Proposal: Introduce high Doppler normal demodulation test, e.g., 400 Hz, with 1 additional DM-RS.

- Alt. 1: Introduce one additional test

- Alt. 2: Replace one high Doppler test with 1 additional DM-RS

References

[1] 3GPP, R4-1811394, Intel, “Way forward on NR UE PDSCH demodulation requirements and General aspects,” Aug. 2018.

[2] 3GPP, “TS38.211”.