3GPP TSG-RAN WG4 Meeting #92	R4-1908661

Ljubljana, SI, 26th– 30th Aug, 2019

Agenda item:	8.14.4	BS Demodulation 	[LTE_high_speed_enh2-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	BS demodulation for LTE high speed

Document for:	Approval

1.	Introduction

In RAN4 #91, WF on BS demodulation for high speed in Rel.16 was approved [1]. In this contribution, we discuss on open issues and provide our views.

RAN4 #91 [1] WF on PUSCH for HST for evaluation (not for requirement) - Deployment scenarios and related parameters (e.g., Ds, Dmin, fd) will be decided based on evaluation results. The following parameters should be considered in the evaluation. - Maximum Doppler shift will be based on the outcome of simulation results and could be less than 2 times downlink Doppler shift. - Note: This agreement does not exclude work on unidirectional scenario.

2.	Discussion

2.1	HST scenario

In WF [1], the options for maximum Doppler shift for evaluation were shown as below:

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Open space |  | Tunnel for multi-antennas |  |
|  | Bi-directional (1st priority) | Uni-directional | Bi-directional (1st priority) | Uni-directional |
| Ds | 1000 m |  | 300 m |  |
| Dmin | 50 m |  | 2 m |  |
| v | 500 km/h (lower speeds are not precluded) |  | 500 km/h (lower speeds are not precluded) |  |
| fd | 1944 Hz, 1894 Hz, 1844 Hz, 1794 Hz, 1744 Hz, 1694 Hz |  |  |  |

In the UE demodulation discussion, the maximum Doppler shift with 972Hz was agreed [2]. The doubled value of maximum Doppler shift for UE demodulation is 1944Hz. If a lower value than 1944Hz is adopted as the maximum Doppler shift, the deployment scenario for BS demodulation does not align with that for UE demodulation.

Observation 1: If the maximum Doppler frequency shift lower than 1944Hz is adopted, HST scenario for BS demodulation does not align with for UE demodulation.

Table 1 shows UE velocity based on Band 1 (2.1GHz) and calculated from candidate maximum Doppler shift assumptions. Band 1 should be considered for HST scenario since the existing BS HST scenario is based on Band 1 assumption. If the lower value than 1944Hz is adopted as a maximum Doppler shift, the performance under HST with UE velocity up to 500km/h cannot guaranteed.

Observation 2: If the maximum Doppler frequency shift lower than 1944Hz is adopted, LTE cannot support 500km/h at Band 1.

From the above observations, to support up to 500km/h UE velocity, we need to consider 1944 Hz as a maximum Doppler shift. Otherwise, the performance under HST with 500km/h UE velocity conditions is not guaranteed.

Proposal 1: Agree to adopt the maximum Doppler shift 1944Hz for BS HST scenario.

Proposal 2: Keep 500km/h as the UE velocity assumption regardless of maximum Doppler shift assumptions.

Table 1: UE velocity corresponding to maximum Doppler shift at Band 1

| Maximum Doppler Shift | UE velocity |
| --- | --- |
|  | @Band 1 (2.1GHz) |
| 1944Hz | 500km/h |
| 1894Hz | 486km/h |
| 1844Hz | 473km/h |
| 1794Hz | 460km/h |
| 1774Hz | 455km/h |
| 1694Hz | 435km/h |

3.	Conclusion

In this contribution, we provide our views on BS demodulation requirements for LTE high speed in Rel.16. The following proposals are obtained.

Observation 1: If the maximum Doppler frequency shift lower than 1944Hz is adopted, HST scenario for BS demodulation does not align with for UE demodulation.

Observation 2: If the maximum Doppler frequency shift lower than 1944Hz is adopted, LTE cannot support 500km/h at Band 1.

Proposal 1: Agree to adopt the maximum Doppler shift 1944Hz for BS HST scenario.

Proposal 2: Keep 500km/h as the UE velocity assumption regardless of maximum Doppler shift assumptions.

References

- R4-1907747, “Way forward on Rel-16 LTE HST BS demodulation requirements”, Nokia, NTT DOCOMO, INC., Ericsson, RAN4 #91, May 2019

- R4-1907745, “Way forward on UE demodulation enhancement for HST”, Qualcomm, RAN4 #91, May 2019