3GPP TSG-RAN WG4 Meeting #92	R4-1908725

# Ljubljana, SI, 26th– 30th Aug, 2019

Source:	LG Electronics

Title:	Antenna isolation and A-MPR for B41/n41 EN-DC

Agenda item:	10.14.2

Document for:	Approval

Introduction

In RAN4#91 meeting, the WF [1] on B41/n41 A-MPR assumptions was agreed as follows:

- Request UE vendors to provide information on actual and feasible antenna isolations for B41/n41 frequencies to validate improved antenna isolation assumptions. - Request component vendors to provide measurements of A-MPR requirements with new assumptions: - Three values for antenna isolation of 20 dB, 16 dB, and 13 dB - Post PA loss of 4 dB - Power Class 2 Tx chains (LTE and NR) - Equal Power on LTE and NR - 1RB + 1 RB allocations - If results of 1 RB + 1RB appear promising, additional allocations, with equal allocation sizes, to align with existing curve inflections points - Determine backoff required to meet -13, -25, and -30 dBm/MHz limits

In this contribution, minimum A-MPR values required to satisfy spurious emissions (-13, -25, and -30 dBm/MHz) limits are provided based on measurements with actual antenna isolation for B41/n41 intra-band contiguous/non-contiguous EN-DC.

Intermodulation distortion (IMD) analysis for B41/n41 EN-DC

Before analyzing the measurement results of A-MPR, let’s first take a look at the IMD analysis for B41/n41 intra-band contiguous/non-contiguous EN-DC as summarized in Table 1.

| 3GPP TS38.101-3, TS38.101-1, and TS36.101 |  | B41/n41 Intra-band contiguous EN-DC UE | B41/n41 Intra-band non-contiguous EN-DC UE |
| --- | --- | --- | --- |
| Additional spurious emissions (NS_04) | -25dBm/MHz | IM3 into the A-SEM range |  |
| General Spurious emissions (-30dBm/MHz) | -30dBm/MHz | IM5 into G-SE | IM3 into G-SE |
| General Spurious emissions (NS_04) | -25dBm/MHz | IM5 into G-SE | IM3 into G-SE |
| A-SEM(NS_04) |  | IM3 into A-SEM (-13dBm/MHz spec. limit) | IM3 into A-SEM (-25dBm/MHz spec. limit) |
| General SEM |  | IM3 into G-SEM (-13dBm/MHz spec. limit) | IM3 into G-SEM (-25dBm/MHz spec. limit) |
| EN-DC ACLR |  | IM3 into ACLREN_DC range |  |
| EN-DC UE-to-UE Co-existence | B40 Rx | IM3 into B40 |  |
|  | B30 Rx | IM5 into B30 | IM3 into B30 |
|  | B1/B4/B66 Rx | IM7 into B1/B4/B66 | IM5 into B1/B4/B66 |

Table 1. IMD analysis for B41/n41 intra-band contiguous/non-contiguous EN-DC

As shown in Table 1, IMD3 emissions fall into additional spurious emissions of NS_04 (-13 dBm/MHz and -25 dBm/MHz), general spurious emissions of NS_04 (-25 dBm/MHz) and general spurious emissions (-30 dBm/MHz) regions. To satisfy these limits, RAN4 has defined the A-MPR for NS_04 and it is used instead of MPR defined in 6.2B2.2 (EN-DC MPR = 0) in [2].

For EN-DC UE-to-UE Co-existence, IMD3, IMD5, and IMD7 emissions fall into adjacent bands (B1, B4, B66, B30, and B40) and these adjacent bands were included to spurious emission requirements as protected band except for Band 40. The reason for removing the band 40 from B41/n41 EN-DC co-existence table can be found in [3].

Observation 1: IMD 3 emissions fall into additional spurious emissions of NS_04 (-13 dBm/MHz and -25 dBm/MHz), general spurious emissions of NS_04 (-25 dBm/MHz) and general spurious emissions (-30 dBm/MHz) regions for B41/n41 intra-band contiguous/non-contiguous EN-DC.

A-MPR measurement of B41/n41 intra-band contiguous/non-contiguous EN-DC

RAN4 has been reviewing and discussing on a potential improved A-MPR for B41/n41 EN-DC, and there was a request to UE vendors to provide actual and feasible antenna isolations in RAN4#91 meeting. New assumptions are listed to do measurements of A-MPR requirements as follows:

- Three values for antenna isolation of 20 dB, 16 dB, and 13 dB

- Post PA loss of 4 dB

- Power class 2 Tx chains (LTE and NR)

- Equal power on LTE and NR

- 1RB + 1 RB allocations

  - If results of 1RB + 1RB appear promising, additional allocations, with equal allocation sizes, to align with existing curve inflections points

- Determine back-off required to meet -13, -25, and -30 dBm/MHz limits

For the A-MPR measurement of B41/n41 intra-band contiguous EN-DC, actual antenna isolation of 10 dB was used with the assumptions that are listed above. From UE vendor perspective, the current implementable antenna isolation value is 10 dB considering the form factor (size, shape, and positions of components) of a mobile phone. Details of parameters and a result of test measurements are shown in Table 2.

Observation 2: From UE vendor perspective, the current implementable antenna isolation value is 10 dB.

Proposal 1: RAN4 should use the antenna isolation value as 10 dB regardless of EN-DC band combinations.

| Intra-band contiguous 41/n41 EN-DC ▪ UL non-contiguous resource allocation. ▪ LTE SC-FDMA/QPSK, NR DFT-s-OFDM/QPSK |  |  |  |  |  |  | Tx power backoff [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE 20MHz |  | NR 40/50/60/80/100 MHz |  | Total UL RB BW [MHz] | EN-DC PC2 3GPP Rel-15 AMPR value TS38.101-3 |  | 29dBm UE for B41/n41 - Post PA loss of 4 dB, -Power Class 2 Tx chains (LTE and NR) - Equal Power on LTE and NR |  |  |  |  |  |  |  |
| SCS | transmitted RB | SCS | transmitted RB |  | Type1 UE 6.2B.3.1.2.2 |  | NS04 Additional spurious Emissions | NS04/ General SEM | EN-DC ACLR | EN-DC UE co-existence |  |  | Spurious emissions | Spurious emissions |
|  |  |  |  |  |  |  |  |  |  |  | TS38.101-3 |  |  |  |
|  |  |  |  |  | “B” [MHz] | EN-DC AMPR | 38.101-3 | 38.101-3 38.101-1 36.101 | 38.101-3 | B40 | B30 Rx | B1/4/66 Rx | 38.101-3 38.101-1 | -30dBm /MHz |
|  |  |  |  |  |  |  | -25dBm/MHz f < 2490.5 | -13dBm /MHz f<ENBW |  | -40dBm /MHz | -50dBm /MHz |  | -25dBm /MHz |  |
| 15KHz | 1 | 30KHz | 1 | 0.54 | <1M | 15 | 14dB | 8dB | 6dB | 17dB | 11dB | 6dB | 5dB | 7dB |
| 15KHz | 6 | 30KHz | 1 | 1.44 | 1~2M | 14 | 14dB | 8dB | 6dB | 17dB | 10dB | 4dB | 3dB | 6dB |
| 15KHz | 5 | 30KHz | 5 | 2.7 | 2~5M | 13 | 13dB | 6dB | 6dB | 16dB | 9dB | 4dB | 3dB | 5dB |
| 15KHz | 12 | 30KHz | 1 | 2.52 |  | 13 | 12dB | 5dB | 6dB | 15dB | 8dB | 3dB | 2dB | 4dB |
| 15KHz | 10 | 30KHz | 10 | 5.4 | ≥5M | 12 | 13dB | 7dB | 6dB | 16dB | 10dB | 3dB | 2dB | 4dB |
| 15KHz | 15 | 30KHz | 15 | 8.1 |  | 12 | 11dB | 4dB | 6dB | 14dB | 8dB | 3dB | 1dB | 3dB |
| 15KHz | 30 | 30KHz | 1 | 5.76 |  | 12 | 11dB | 5dB | 6dB | 15dB | 8dB | 3dB | 1dB | 2dB |
| 15KHz | 20 | 30KHz | 20 | 10.8 |  | 12 | 10dB | 4dB | 6dB | 14dB | 7dB | 2dB | 1dB | 2dB |
| 15KHz | 25 | 30KHz | 25 | 13.5 |  | 12 | 9dB | 3dB | 6dB | 13dB | 7dB | 2dB | - | 2dB |
| 15KHz | 30 | 30KHz | 30 | 16.2 |  | 12 | 9dB | 3dB | 6dB | 13dB | 7dB | 1dB | - | 1dB |
| 15KHz | 35 | 30KHz | 35 | 18.9 |  | 12 | 9dB | 3dB | 7dB | 12dB | 6dB | 1dB | - | 1dB |
| 15KHz | 40 | 30KHz | 40 | 21.6 |  | 12 | 8dB | 2dB | 7dB | 12dB | 6dB | 1dB | - | 1dB |
| 15KHz | 45 | 30KHz | 45 | 24.3 |  | 12 | 8dB | 2dB | 7dB | 12dB | 6dB | 1dB | - | 1dB |
| 15KHz | 50 | 30KHz | 50 | 27 |  | 12 | 8dB | 2dB | 7dB | 12dB | 6dB | 1dB | - | 1dB |

Table 2. Test measurements result of A-MPR for B41/n41 intra-band contiguous EN-DC with 10 dB antenna isolation

It can be observed that the minimum power back-off values of 14 dB, 8 dB, and 7 dB are required to satisfy -25, -13, and 
-30 dBm/MHz limits, respectively for the worst case where 1 RB (LTE) and 1 RB (NR) are allocated. The measured power back-off value of -30 dBm/MHz limit is relatively small compared to others because IMD5 falls into this spurious emissions region.

Observation 3: When 10 dB antenna isolation is used for 29 dBm UE supporting B41/n41 intra-band contiguous EN-DC, the minimum values of A-MPR to satisfy the requirements as follows:

- -25 dBm/MHz additional spurious emissions of NS_04: 14 dB

- -13 dBm/MHz general SEM of NS_04: 8 dB

- -30 dBm/MHz spurious emissions: 7 dB

The same antenna isolation of 10 dB was used for test measurement of B41/n41 intra-band non-contiguous EN-DC and its measured power back-off values are shown in Table 3.

| Intra-band non-contiguous 41/n41 EN-DC Case ▪ UL non-contiguous resource allocation. ▪ LTE SC-FDMA/QPSK, NR DFT-s-OFDM/QPSK |  |  |  |  |  |  | Tx power backoff [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE 20MHz |  | NR 40/50/60/80/100 MHz |  |  | EN-DC PC2 3GPP Rel-15 AMPR value TS38.101-3 |  | 29dBm UE Power Class for LTE Band 41 and NR Band n41 - Post PA loss of 4 dB - Power Class 2 Tx chains (LTE and NR) - Equal Power on LTE and NR |  |  |  |  |  |  |  |  |
| SCS | transmitted RB | SCS | transmitted RB | Total UL RB BW [MHz] | Type1 UE 6.2B.3.1.2.2 |  | NS04 Additional spurious emissions | NS04 SEM | General SEM | EN-DC ACLR | EN-DC UE co-existence |  |  | Spurious emissions | Spurious emissions |
|  |  |  |  |  |  |  |  |  |  |  | 38.101-3 |  | 38.101-3 |  |  |
|  |  |  |  |  | “B” [MHz] | EN-DC AMPR | 38.101-3 | 38.101-3, 38.101-1 |  |  | B30 | B40 | B1/4/66 Rx | 38.101-3 38.101-1 | -30dBm /MHz |
|  |  |  |  |  |  |  | -25dBm /MHz f < 2490.5 | -25dBm /MHz |  |  | -40dBm /MHz | -40dBm /MHz | -50dBm /MHz | -25dBm /MHz |  |
| 15KHz | 1 | 30KHz | 1 | 0.54 | <1M | 15 | 14dB | 12dB | 12dB | 6dB | 17dB | 17dB | 9dB | 13dB | 15dB |
| 15KHz | 6 | 30KHz | 1 | 1.44 | 1~2M | 14 | 14dB | 12dB | 12dB | 6dB | 17dB | 17dB | 9dB | 13dB | 14dB |
| 15KHz | 5 | 30KHz | 5 | 2.7 | 2~5M | 13 | 13dB | 10dB | 10dB | 6dB | 16dB | 16dB | 9dB | 11dB | 14dB |
| 15KHz | 12 | 30KHz | 1 | 2.52 |  | 13 | 12dB | 9dB | 9dB | 6dB | 15dB | 15dB | 9dB | 10dB | 13dB |
| 15KHz | 10 | 30KHz | 10 | 5.4 | ≥5M | 12 | 13dB | 11dB | 11dB | 6dB | 16dB | 16dB | 8dB | 12dB | 12dB |
| 15KHz | 15 | 30KHz | 15 | 8.1 |  | 12 | 11dB | 8dB | 8dB | 6dB | 14dB | 14dB | 8dB | 8dB | 12dB |
| 15KHz | 30 | 30KHz | 1 | 5.76 |  | 12 | 11dB | 9dB | 9dB | 6dB | 15dB | 15dB | 8dB | 10dB | 11dB |
| 15KHz | 20 | 30KHz | 20 | 10.8 |  | 12 | 10dB | 7dB | 7dB | 6dB | 14dB | 14dB | 8dB | 7dB | 11dB |
| 15KHz | 25 | 30KHz | 25 | 13.5 |  | 12 | 9dB | 6dB | 6dB | 6dB | 13dB | 13dB | 7dB | 7dB | 11dB |
| 15KHz | 30 | 30KHz | 30 | 16.2 |  | 12 | 9dB | 6dB | 6dB | 6dB | 13dB | 13dB | 7dB | 7dB | 10dB |
| 15KHz | 35 | 30KHz | 35 | 18.9 |  | 12 | 9dB | 6dB | 6dB | 7dB | 12dB | 12dB | 7dB | 7dB | 10dB |
| 15KHz | 40 | 30KHz | 40 | 21.6 |  | 12 | 8dB | 6dB | 6dB | 7dB | 12dB | 12dB | 7dB | 7dB | 10dB |
| 15KHz | 45 | 30KHz | 45 | 24.3 |  | 12 | 8dB | 5dB | 5dB | 7dB | 12dB | 12dB | 7dB | 6dB | 9dB |
| 15KHz | 50 | 30KHz | 50 | 27 |  | 12 | 8dB | 5dB | 5dB | 7dB | 12dB | 12dB | 6dB | 6dB | 9dB |

Table 3. Test measurements result of A-MPR for B41/n41 intra-band non-contiguous EN-DC with 10 dB antenna isolation

It can be observed that the minimum values of power back-off are 14 dB, 12 dB, and 15 dB to satisfy -25, -25, and -30 dBm/MHz limits, respectively for the worst case where 1 RB (LTE) and 1 RB (NR) are allocated. Since IMD3 emissions fall into -30 dBm/MHz spurious emissions region, the power back-off value is bigger than the value in intra-band contiguous case.

Observation 4: When 10 dB antenna isolation is used for 29 dBm UE supporting B41/n41 intra-band non-contiguous EN-DC, the minimum values of A-MPR to satisfy the requirements as follows:

- -25 dBm/MHz additional spurious emissions of NS_04: 14 dB

- -25 dBm/MHz general SEM of NS_04: 12 dB

- -30 dBm/MHz spurious emissions: 15 dB

Conclusions

In this contribution, the test measurements of A-MPR for B41/n41 intra-band contiguous/non-contiguous EN-DC with 10 dB antenna isolation are provided.

Observation 1: IMD 3 emissions fall into additional spurious emissions of NS_04 (-13 dBm/MHz and -25 dBm/MHz), general spurious emissions of NS_04 (-25 dBm/MHz) and general spurious emissions (-30 dBm/MHz) regions for B41/n41 intra-band contiguous/non-contiguous EN-DC.

Observation 2: From UE vendor perspective, the current implementable antenna isolation value is 10 dB.

Observation 3: When 10 dB antenna isolation is used for 29 dBm UE supporting B41/n41 intra-band contiguous EN-DC, the minimum values of A-MPR to satisfy the requirements as follows:

- -25 dBm/MHz additional spurious emissions of NS_04: 14 dB

- -13 dBm/MHz general SEM of NS_04: 8 dB

- -30 dBm/MHz spurious emissions: 7 dB

Observation 4: When 10 dB antenna isolation is used for 29 dBm UE supporting B41/n41 intra-band non-contiguous EN-DC, the minimum values of A-MPR to satisfy the requirements as follows:

- -25 dBm/MHz additional spurious emissions of NS_04: 14 dB

- -25 dBm/MHz general SEM of NS_04: 12 dB

- -30 dBm/MHz spurious emissions: 15 dB

Proposal 1: RAN4 should use the antenna isolation value as 10 dB regardless of EN-DC band combinations.

Reference

[1] R4-1907495, “Way Forward on B41/n41 A-MPR assumptions,” Sprint, Skyworks

[2] TS 38.101-3

[3] R4-1902002, “Draft CR to 38.101-3 on DC_n41-41 – B40 coexistence,” Qualcomm Incorporated