3GPP TSG-RAN WG4 Meeting #92	R4-1908779

Ljubljana, Slovenia, 26 - 30 August, 2019

Agenda item:	10.1.2

Source:	Dish Network

Title:	Finalizing the Intra-band CA requirements for bands <2700MHz

Document for:	Approval

# 1	Introduction

This contribution recaps the development of Intra-band CA requirements <2700MHz (a.k.a NR legacy bands) for Class B and class C in Release 15 and in subsequent Rel-16 specification and then makes proposals how to close the remaining topics.

# 2	Discussion

The requirements for Generic Intra-band contiguous CA Class B were agreed in RAN4#89 [1][2]. These requirements were for Rel16 specification, that was to be published after RAN#84 (June 2019). Then in RAN4 #90 it was agreed, due to n41 deployment schedule to introduce Intra-band contiguous CA class C for Rel15 [3]. Class B requirements were done to match respective LTE requirements as much as possible as well as the table format used for NR non-CA requirements. Class C requirements in NR are much different story, because for instance the maximum aggregated bandwidth in NR is 200MHz while in LTE the largest aggregated bandwidth is 100MHz (for LTE class F).

All this put together made it pretty impossible to have Rel15 and Rel16 aligned for Class B and Class C <2700MHz. Draft Rel-16 version was first time shown in RAN4#91 (May 2019) where it was noticed that some requirements for Intra-band contiguous CA Class B requirements are missing even the CR is approved i.e CR [2] is only partially implemented. This is mainly due to different table formats and so on. Correcting that during the meeting would have been too time consuming so it was agreed RAN4 management correction CR in August.

We illustrate what is missing for Class B <2700MHz from Rel-16 that was agreed in [1]. After that we suggest how to solve the problem and finalize Class B requirements <2700MHz in Rel16.

Maximum Input level:

Current Rel16 specification is as follows.

### 7.4A.1	Maximum input level for Intra-band contiguous CA

For intra-band contiguous carrier aggregation maximum input level is defined as the maximum mean power received at the UE antenna port, over the Transmission bandwidth configuration of each CC.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4A.1-1 for each component carrier.

Table 7.4A.1-1: Maximum input level for Intra-band contiguous CA

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232 | -252 | -262 |
|  |  | -253 | -273 | -283 |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in subclause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |  |

In [1] there was no specific change to this requirement, because the text during the approval of [1] was generally applicable to all future BW classes:

### 7.4A.1	Maximum input level for Intra-band contiguous CA

For carrier aggregation maximum input level is defined as the exact wording TBD, over the aggregated receiver bandwidth, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel over each component carrier. The minimum requirement is the same as the one specified in Table 7.4-1.

Now that there is a table in place, a requirement for class B is needed.

Proposal1: Add NR CA Bandwidth Class B into table 7.4A.1-1 as follows.

Table 7.4A.1-1: Maximum input level for Intra-band contiguous CA

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -252 | -232 | -252 | -262 |
|  |  | -273 | -253 | -273 | -283 |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in subclause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |  |  |

Adjacent Channel Sensitivity:

Current Rel16 specification is as follows.

### 7.5A.1		Adjacent channel selectivity for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.5A.1-1 and 7.5A.1-1a for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5A.1-2, 7.5A.1-2a, 7.5A.1-3 and 7.5A.1-3a.

Table 7.5A.1-1: ACS for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  |  | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
| Rx Parameter | Units | C | D | E |
| ACS | dB | 33.0 | 25.2 | 24.0 |

Table 7.5A.1-1a: ACS for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

|  |  | NR CA bandwidth class |  |
| --- | --- | --- | --- |
| Rx Parameter | Units | C |
| ACS | dB | 17.0 |

Table 7.5A.1-2: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 31.5 dB | Aggregated power + 23.7 dB | Aggregated power + 22.5 dB |
| BWInterferer | MHz | BWchannel CA | 50 | 50 |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA | 25 + Foffset / -25 -Foffset | 25 + Foffset / -25 -Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |

Table 7.5A.1-2a: Test parameters for intra-band contiguous CA with FDL_low<2700 MHz and FUL_low<2700 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 15.5 dB |
| BWInterferer | MHz | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5A.1-3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 | -48.7 + 10log(NRB,c/NRB_agg) | -47.5 + 10log(NRB,c/NRB_agg) |
| PInterferer | dBm | -25 | 25 | 25 |
| BWInterferer | MHz | BWchannel CA | 50 | 50 |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA | 25 + Foffset / -25 -Foffset | 25 + Foffset / -25 -Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |

Table 7.5A.1-3a: Test parameters for intra-band contiguous CA with FDL_low <2700 MHz and FUL_low<2700 MHz, case 2

| Rx Parameter | Units | NR CA Bandwidth Class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -40.5 + 10log(NRB,c/NRB_agg) |
| PInterferer | dBm | -25 |
| BWInterferer | MHz | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

In [1] the following was agreed for Intra-band contiguous CA class B.

### 7.5A.1	Adjacent channel selectivity for Intra-band contiguous CA

For intra-band contiguous carrier aggregation with two component carriers and aggregated bandwidth BWChannel_CA shall be configured at nominal channel spacing tothe PCC. The UE shall fulfil the minimum requirement specified in Table 7.5A.1-1 and Table 7.5A.1-4 for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2  (with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.2.1 and A.5.2.2) with parameters specified in Tables 7.5A.1-2 and 7.5A.1-3 and Table 7.5A.1-5 and 7.5A.1-6.

Table 7.5A.1-4: ACS for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz |
| ACS | dB | [24] | [24] | [24] | [24] | [22.2] |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 50 MHz |  |  |  |  |
| ACS | dB | [22.2] |  |  |  |  |

Table 7.5A.1-5: Test parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | Aggregated power + [22.5] dB |  |  |  | Aggregated power + [20.7] dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 50 MHz |  |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | Aggregated power + [20.7] dB |  |  |  |  |
| BWinterferer | MHz | 5 |  |  |  |  |
| Finterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

Table 7.5A.1-6: Test parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz |  |
| Power in transmission bandwidth configuration, Per CC | dBm | -47.5 +10log10(NRB,c/NRB agg) |  |  |  |  | -45.7 +10log10(NRB,c/NRB agg) |
| Pinterferer | dBm | -25 |  |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |  |
| Finterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |  |
|  |  | 50 MHz |  |  |  |  |  |
| Power in transmission bandwidth configuration | dBm | -45.7 +10log10(NRB,c/NRB agg) |  |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |  |
| BWinterferer | MHz | 5 |  |  |  |  |  |
| Finterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |  |

The obvious difference between Class C for bands <2700MHz and non-implemented Class B CR is that in former ACS is defined per Bandwidth class while in latter the ACS is separately defined for each aggregated bandwidth. In order to align these two we propose to adopt the principle of defining only one ACS number power bandwidth class used for Class C also in Class B.

However, this re-creates an issue that was had earlier been solved in [1] by having specific ACS numbers for each aggregated bandwidth: Ensuring NR requirements are similar to those in LTE. In [1] ACS requirements from LTE Class C are used for NR aggregated bandwidths up to 40MHz and requirements from LTE Class D are used for NR aggregated bandwidths 45MHz and 50MHz. Now, when there is only one ACS requirement for BW class, we have to choose the less stringent one, that is to use LTE class D (i.e up to 60MHz aggregated BW) requirements for NR class B. The same principle is used through the entire document and accompanying CR [4].

Proposal2: Define ACS for Intra-band contiguous CA class B in tables 7.5A.1-1a, 7.5A.1-2a, and 7.5A.1-3a as follows.

Table 7.5A.1-1a: ACS for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

|  |  | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C |
| ACS | dB | 22.2 | 17.0 |

Table 7.5A.1-2a: Test parameters for intra-band contiguous CA with FDL_low<2700 MHz and FUL_low<2700 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 20.7 dB | Aggregated power + 15.5 dB |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

Table 7.5A.1-3a: Test parameters for intra-band contiguous CA with FDL_low <2700 MHz and FUL_low<2700 MHz, case 2

| Rx Parameter | Units | NR CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -45.7 + 10log(NRB,c/NRB_agg) | -40.5 + 10log(NRB,c/NRB_agg) |
| PInterferer | dBm | -25 | -25 |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

In-band blocking for CA:

Current Rel16 specification is as follows.

#### 7.6A.2.1 In-band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.6A.2.1-1 and 7.6A.2.1-1a for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.6A.2.1-1: In-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx Parameter | Units | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Pw in Transmission Bandwidth Configuration, per CC | dB | REFSENS + CA bandwidth class specific value below |  |  |
|  |  | 6 | 13.8 | 15.0 |
| BWInterferer | MHz | BWchannel CA | 50 | 50 |
| FIoffset, case 1 | MHz | BWchannel CA+ BWchannel CA/2 | 75 | 75 |
| FIoffset, case 2 | MHz | BWInterferer + FIoffset, case 1 | 125 | 125 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattrn OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |

Table 7.6A.2.1-1a: In-band blocking parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA bandwidth class specific value below |
|  |  | 19.0 |
| BWInterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |

Table 7.6A.2.1-2: In-band blocking for intra-band contiguous CA  with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 3BWchannel CA to FDL_high + 3BWchannel CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.1-2a: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n41 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

In [1] the following was agreed for Intra-band contiguous CA class B.

#### 7.6A.2.1 In-band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggregation with two component carriers and aggregated bandwidth BWChannel_CA, the SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.6A.2.1-1 and in Table 7.6A.2.1-3 for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.2.1 and A.5.2.2).

Table 7.6A.2.1-3: In-band blocking parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 25 MHz | 30 MHz | 35 MHz | 40 MHz |  | 45 MHz |
| Power in transmission bandwidth configuration, Per CC | dBm | REFSENS + channel specific value below |  |  |  |  |  |
|  | dB | 12 |  |  |  | 13.8 |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 |  | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 | 7.5 | 7.5 |  | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 | 12.5 | 12.5 |  | 12.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |  |
|  |  | 50 MHz |  |  |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |  |
|  | dB | 13.8 |  |  |  |  |  |
| BWinterferer | MHz | 5 |  |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |  |

Table 7.6A.2.1-4: In-band blocking for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | [-56] | [-44] |
| n66 | Finterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
|  | Finterferer (range) |  | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 |  |  |  |  |

Proposal3: Define In-band blocking for Intra-band contiguous CA Class B in tables 7.6A.2.1-1a and 7.6A.2.1-2a as follows.

Table 7.6A.2.1-1a: In-band blocking parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA bandwidth class specific value below | REFSENS + NR CA bandwidth class specific value below |
|  |  | 13.8 | 19.0 |
| BWInterferer | MHz | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |

Table 7.6A.2.1-2a: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n41, n66, n71 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

Out of band blocking:

The current Rel16 specification is as follows.

#### 7.6A.3.1	Out-of-band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggreagation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test.

The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.6A.3-1, Table 7.6A.3-1a, Table 7.6A.3-2 and Table 7.6A.3-2a being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.6A.3-1: Out-of-band blocking parameters for intra-band contiguous CA

| RX parameter | Units | CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Power in transmission bandwidth configuration | dBm | REFSENS + CA bandwidth class specific value below |  |  |
|  | dB | 9 | [9] | [9] |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |

Table 7.6A.3-1a: Out-of-band blocking parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 50 MHz |  |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.6A.3-2: Out of-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n41 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n77, n78 (NOTE 3) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(200,3CBWchannel CA) or FDL_high+ MAX(200,3CBWchannel CA) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(150,3CBWchannel CA) or FDL_high + MAX(150,3CBWchannel CA) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

Table 7.6A.3-2a: Out of-band blocking for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n66 | Pinterferer | dBm | -45 | -30 | -15 |
|  | Finterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |

The content is actually same as in [1] for class B, but improvements should be made for consistency. First, the content of table 7.6A.3-1a should be combined into table 7.6A.3-1. Secondly, the content of table 7.6A.3-2a should be embedded in table 7.6A.3-2.

Proposal4: Define Out of band blocking for Intra-band contiguous CA Class B in tables 7.6A.3-1a and 7.6A.3-2 as follows.

Table 7.6A.3-1: Out-of-band blocking parameters for intra-band contiguous CA

| RX parameter | Units | CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E |
| Power in transmission bandwidth configuration | dBm | REFSENS + CA bandwidth class specific value below |  |  |  |
|  | dB | 9 | 9 | [9] | [9] |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |

Table 7.6A.3-2: Out of-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n41, n66, n71 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n77, n78 (NOTE 3) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(200,3CBWchannel CA) or FDL_high+ MAX(200,3CBWchannel CA) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(150,3CBWchannel CA) or FDL_high + MAX(150,3CBWchannel CA) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

Narrow band blocking:

The current Rel16 specification is as follows.

#### 7.6A.4.1	Narrow band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggregation, the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6A.4.1-1 with the uplink configuration. For UE(s) supporting one uplink, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-1. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.6A.4.1-1 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6A.4.1-1.

Table 7.6A.4.1-1: Narrow-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | NR CA bandwidth class |
| --- | --- | --- | --- |
|  |  |  | C |
| n41 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NA CA Bandwidth Class specific value below |
|  |  |  | 16 |
|  | Puw (CW) | dBm | -55 |
|  | Fuw (offset forf = 15 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 |
|  | Fuw (offset forf = 30 kHz) | MHz |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |

In [1] the following was agreed for Intra-band contiguous CA class B.

#### 7.6A.4.1 Narrow band blocking for	Intra-band contiguous CA

For intra-band contiguous carrier aggregation with FDL_low < 2700 MHz and FUL_low < 2700 MHz the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6A.4-1 with the uplink configuration set according to Table 7.3A.2.2-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.6A.4-1 being on either side of the aggregated signal. The throughput of each carrier shall be ≥  95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.2.1 and A.5.2.2) with parameters specified in Table 7.6A.4-1.

Table 7.6A.4-1: Narrow-band blocking for Intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Parameter | Unit | Channel Bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz |
| Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |
|  |  | 16 | 16 | 16 | 16 | 16 | 16 |
| Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 | -55 |
| Fuw (offset for SCS = 15 kHz) | MHz | TBD | TBD | TBD | TBD | TBD | TBD |
| Fuw (offset for SCS= 30 kHz) | MHz |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L as defined in subclause 6.2.4. NOTE 2: Reference measurement channel is specified in Annex A.2.2, A.2.3 and A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.2.1/A.5.2.2. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. |  |  |  |  |  |  |  |

Proposal5: Define Narrowband blocking for Intra-band contiguous CA Class B in tables 7.6A.3-1a and 7.6A.3-2 as follows.

Table 7.6A.4.1-1: Narrow-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | NR CA bandwidth class |  |
| --- | --- | --- | --- | --- |
|  |  |  | B | C |
| n41 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NA CA Bandwidth Class specific value below | REFSENS + NA CA Bandwidth Class specific value below |
|  |  |  | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 |
|  | Fuw (offset forf = 15 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
|  | Fuw (offset forf = 30 kHz) | MHz |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

Spurious response:

Current Rel16 specification is as follows.

### 7.7A.1	Spurious response for Intra-band contiguous CA

Table 7.7A-1: Spurious response parameters for intra-band contiguous CA

| RX parameter | Units | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Power in transmission bandwidth configuration | dBm | REFSENS + CA bandwidth class specific value below |  |  |
|  | dB | 9 | 9 | 9 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |

Table 7.7A-2: Spurious response for CA

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

Table 7.7A-3: Spurious response parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 50 MHz |  |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.7A-4: Spurious response for CA

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

The content is actually same as in [1] for class B, but improvements should be made for consistency. The content of table 7.7A-3 should be combined into table 7.7A-1.

Proposal 6: Define Spurious Response for Intra-band contiguous CA Class B in tables 7.7A.1 as follows.

Table 7.7A-1: Spurious response parameters for intra-band contiguous CA

| RX parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E |
| Power in transmission bandwidth configuration | dBm | REFSENS + CA bandwidth class specific value below |  |  |  |
|  | dB | 9 | 9 | 9 | 9 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |

Wide-band Intermodulation:

The current Rel16 specification is as follows.

#### 7.8A.2.1	Wide band intermodulation for Intra-band contiguous CA

Table 7.8A.2.1-1: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 | REFSENS + 13.8 | REFSENS + 15 |
| PInterferer 1 (CW) | dBm | -46 |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |
| BWInterferer 2 | MHz | BWChannel_CA | 50 | 50 |
| FInterferer 1 (Offset) | MHz | -2BWChannel_CA / +2BWChannel_CA | -Foffset-75 / Foffset+75 | -Foffset-75 / Foffset+75 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |

Table 7.8A.2.1-2: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| Rx parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 22 |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -Foffset-7.5 / Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

In [1] the following was agreed for Intra-band contiguous CA class B.

#### 7.8A.2.1	Wide band intermodulation for Intra-band contiguous CA

Table 7.8A.2.1-1: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 12 |  |  |  | REFSENS + 13.8 |  |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |
| BWInterferer 2 | MHz | 5 |  |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) | MHz | –Foffset-7.5 / + Foffset+7.5 |  |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is TBD. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in TBD. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |  |

Proposal7: Define Wideband Intermodulation for Intra-band contiguous CA Class B in table 7.8A.2.1-2 as follows.

Table 7.8A.2.1-2: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| Rx parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 13.8 | REFSENS + 22 |
| PInterferer 1 (CW) | dBm | -46 | -46 |
| PInterferer 2 (Modulated) | dBm | -46 | -46 |
| BWInterferer 2 | MHz | 5 | 5 |
| FInterferer 1 (Offset) | MHz | -Foffset-7.5 / Foffset+7.5 | -Foffset-7.5 / Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 | 2*FInterferer 1 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |

# 3	Conclusions

Recap of Intra-band contiguous CA Class B and Class C requirements <2700MHz was provided and the following proposals were made to capture the missing Class B requirements in Rel16.

Proposal1: Add NR CA Bandwidth Class B into table 7.4A.1-1 as follows.

Table 7.4A.1-1: Maximum input level for Intra-band contiguous CA

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -252 | -232 | -252 | -262 |
|  |  | -273 | -253 | -273 | -283 |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in subclause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |  |  |

Proposal2: Define ACS for Intra-band contiguous CA class B in tables 7.5A.1-1a, 7.5A.1-2a, and 7.5A.1-3a as follows.

Table 7.5A.1-1a: ACS for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

|  |  | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C |
| ACS | dB | 22.2 | 17.0 |

Table 7.5A.1-2a: Test parameters for intra-band contiguous CA with FDL_low<2700 MHz and FUL_low<2700 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 20.7 dB | Aggregated power + 15.5 dB |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

Table 7.5A.1-3a: Test parameters for intra-band contiguous CA with FDL_low <2700 MHz and FUL_low<2700 MHz, case 2

| Rx Parameter | Units | NR CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -45.7 + 10log(NRB,c/NRB_agg) | -40.5 + 10log(NRB,c/NRB_agg) |
| PInterferer | dBm | -25 | -25 |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

Proposal3: Define In-band blocking for Intra-band contiguous CA Class B in tables 7.6A.2.1-1a and 7.6A.2.1-2a as follows.

Table 7.6A.2.1-1a: In-band blocking parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA bandwidth class specific value below | REFSENS + NR CA bandwidth class specific value below |
|  |  | 13.8 | 19.0 |
| BWInterferer | MHz | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |

Table 7.6A.2.1-2a: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n41, n66, n71 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

Proposal4: Define Out of band blocking for Intra-band contiguous CA Class B in tables 7.6A.3-1a and 7.6A.3-2 as follows.

Table 7.6A.3-1: Out-of-band blocking parameters for intra-band contiguous CA

| RX parameter | Units | CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E |
| Power in transmission bandwidth configuration | dBm | REFSENS + CA bandwidth class specific value below |  |  |  |
|  | dB | 9 | 9 | [9] | [9] |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |

Table 7.6A.3-2: Out of-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n41, n66, n71 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n77, n78 (NOTE 3) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(200,3CBWchannel CA) or FDL_high+ MAX(200,3CBWchannel CA) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(150,3CBWchannel CA) or FDL_high + MAX(150,3CBWchannel CA) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

Proposal5: Define Narrowband blocking for Intra-band contiguous CA Class B in tables 7.6A.3-1a and 7.6A.3-2 as follows.

Table 7.6A.4.1-1: Narrow-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | NR CA bandwidth class |  |
| --- | --- | --- | --- | --- |
|  |  |  | B | C |
| n41 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NA CA Bandwidth Class specific value below | REFSENS + NA CA Bandwidth Class specific value below |
|  |  |  | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 |
|  | Fuw (offset forf = 15 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
|  | Fuw (offset forf = 30 kHz) | MHz |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

Proposal 6: Define Spurious Response for Intra-band contiguous CA Class B in tables 7.7A.1 as follows.

Table 7.7A-1: Spurious response parameters for intra-band contiguous CA

| RX parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E |
| Power in transmission bandwidth configuration | dBm | REFSENS + CA bandwidth class specific value below |  |  |  |
|  | dB | 9 | 9 | 9 | 9 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |

Proposal7: Define Wideband Intermodulation for Intra-band contiguous CA Class B in table 7.8A.2.1-2 as follows.

Table 7.8A.2.1-2: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| Rx parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 13.8 | REFSENS + 22 |
| PInterferer 1 (CW) | dBm | -46 | -46 |
| PInterferer 2 (Modulated) | dBm | -46 | -46 |
| BWInterferer 2 | MHz | 5 | 5 |
| FInterferer 1 (Offset) | MHz | -Foffset-7.5 / Foffset+7.5 | -Foffset-7.5 / Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 | 2*FInterferer 1 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |

# 3	References

[1] R4-1816226, “Intra-band CA UE RX requirements for <2700MHz”, RAN4#89, Dish Network

[2] R4-1815797, “Draft CR to add NR intra-band FR1 in TS 38.101-1”, RAN4#89, Ericsson, Dish Network

[3] R4-1902194, “Draft CR for 38.101-1: adding DL intra-band contiguous CA requirements for frequency less than 2700MHz”, RAN4#90, Huawei, HiSilicon, CMCC

[4] R4-1908776, “Finalizing Generic Intra-band Contiguous CA Class B requirements”, RAN4#92, Dish Network