3GPP TSG-RAN4 Meeting #92	R4-XXXXX

Ljubljana, SI, 26th – 30th Aug 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: Addition of 2D compact range MU evaluation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The general description and process of DBCR was agreed. The MU evalaution of 2D compact range methord should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The 2D compact range MU budget is added in TR 37.843. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 2D compact range test methord is not finished. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2, 10.3.2 and Annex E. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGE>

10.2.2.1	General

The EIRP accuracy uncertainty budget was re-assessed and adjusted slightly according as detailed in the following tables. The MU budget for the CATR and 2D compact range are shown as it is this MU that determines the agreed MU and TT.

Table 10.2.2-1: Test system specific measurement uncertainty values for the EIRP test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 6 GHz |
| Compact Antenna Test Range | 1.33 | 1.40 |
| 2D campact range | 1.23 | 1.36 |
| Common maximum accepted test system uncertainty | 1.3 | 1.4 |

10.2.2.3	2D Compact Range

10.2.2.3.1	MU assessment

10.2.2.3.1.1	MU budget

Table 10.2.2.3.1.1-1:2D Compact Range uncertainty budget format for EIRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT and pointing error | E2-1 |
| 2 | Measurement equipment | F1 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (DUT connector terminated and test range antenna connector cable terminated) | E2-4 |
| 5 | Quiet zone ripple DUT | E2-5 |
| 6 | Frequency flatness | E2-16 |
| 7 | Time nonlinearity | E6-1 |
| 8 | Power nonlinearity | E6-2 |
| 9 | Miscellaneous uncertainty | E2-14 |
| Stage 1: Calibration measurement |  |  |
| 10 | Calibration equipment | F1 |
| 11 | Mismatch of receiver chain | E2-7 |
| 12 | Insertion loss of receiver chain | E2-8 |
| 13 | RF leakage (SGH connector terminated and test range antenna connector cable terminated) | E2-4 |
| 14 | Influence of reference antenna feed cable (flexing cables, adapters, attenuators, connector repeatability) | E2-9 |
| 15 | Uncertainty of absolute gain of reference antenna | F1 |
| 16 | Misalignment positioning system | E2-11 |
| 17 | Misalignment of calibration antenna & pointing error | E2-1 |
| 18 | Rotary joints | E2-12 |
| 19 | Standing wave between reference antenna and test range antenna | E2-3 |
| 20 | Quiet zone ripple reference antenna | E2-5 |
| 21 | Switching uncertainty | E2-15 |
| 22 | Frequency flatness | E2-16 |

10.2.2.3.1.2	MU value

| EIRP uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty Value |  | Distribution | Divisor | ci | Std. uncertainty ui [dB] |  |
|  |  | f ≦ 3GHz | 3GHz ≦ f < 6GHz |  |  |  | f ≦ 3GHz | 3GHz ≦ f < 6GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 2 | Measurement equipment | 0.00 | 0.00 | Gaussian | 1 | 1 | 0.00 | 0.00 |
| 3 | Standing wave between DUT and test range antenna | 0.05 | 0.14 | Rectangular | √3 | 1 | 0.03 | 0.08 |
| 4 | RF leakage (calibration antenna connector terminated) | 0.086 | 0.086 | Normal | 1 | 1 | 0.09 | 0.09 |
| 5 | Quiet zone ripple DUT | 0.35 | 0.41 | Rectangular | √3 | 1 | 0.20 | 0.24 |
| 6 | Frequency flatness | 0.13 | 0.13 | Rectangular | √3 | 1 | 0.08 | 0.08 |
| 7 | Time nonlinearity | 0.017 | 0.017 | Rectangular | √3 | 1 | 0.0098 | 0.0098 |
| 8 | Power nonlinearity | 0.06 | 0.06 | Rectangular | √3 | 1 | 0.035 | 0.035 |
| 9 | Miscellaneous Uncertainty | 0.20 | 0.20 | Normal | 1 | 1 | 0.20 | 0.20 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Calibration equipment (i.e. reference receiver, measurement receiver and generator) | 0.059 | 0.059 | Normal | 1 | 1 | 0.06 | 0.06 |
| 11 | Mismatch (i.e. reference receiver, reference antenna, generator and reference cable) | 0.06 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.03 |
| 12 | Insertion loss variation in receiver chain | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 13 | RF leakage (calibration antenna connector terminated) | 0.086 | 0.086 | Normal | 1 | 1 | 0.09 | 0.09 |
| 14 | Influence of the calibration antenna feed cable | 0.103 | 0.104 | Exp. normal | 2 | 1 | 0.06 | 0.06 |
| 15 | Uncertainty of the absolute gain of the calibration antenna | 0.29 | 0.25 | Exp. normal | 2 | 1 | 0.15 | 0.13 |
| 16 | Misalignment of positioning system | 0.00 | 0.00 | Exp. normal | 2 | 1 | 0.00 | 0.00 |
| 17 | Misalignment of calibration antenna & pointing error | 0.5 | 0.5 | Rectangular | √3 | 1 | 0.29 | 0.29 |
| 18 | Rotary joints | 0.00 | 0.00 | U-shaped | √2 | 1 | 0.00 | 0.00 |
| 19 | Standing wave between reference antenna and test range antenna | 0.12 | 0.12 | Rectangular | √3 | 1 | 0.07 | 0.07 |
| 20 | Quiet zone ripple reference antenna | 0.10 | 0.12 | Rectangular | √3 | 1 | 0.06 | 0.07 |
| 21 | Switching uncertainty | 0.02 | 0.02 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 22 | Field calibration repeatability | 0.13 | 0.31 | Rectangular | √3 | 1 | 0.08 | 0.18 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.63 | 0. 69 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.23 | 1.36 |

10.3.2	OTA sensitivity

The OTA sensitivity MU was derived in TR 37.842 [4] and 2D compact range methord  as follows:

Table 10.3.2-1: Test system specific measurement uncertainty values for the EIS test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1.22 | 1.25 |
| Compact Antenna Test Range | 1.33 | 1.40 |
| One Dimensional Compact Range Chamber | 1.29 | 1.43 |
| Near Field Test Range | 1.24 | 1.24 |
| 2D compact range | 1.30 | 1.33 |
| Common maximum accepted test system uncertainty | 1.3 | 1.4 |

10.3.2.1  2D compact range

10.3.2.1.1	MU assessment

10.3.2.1.1.1	MU Budget

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT and pointing error | E2-1 |
| 2 | Measurement equipment（signal generator） | F1 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (DUT connector terminated and test range antenna connector cable terminated) | E2-4 |
| 5 | Quiet zone ripple DUT | E2-5 |
| 6 | Frequency flatness | E2-16 |
| 7 | Time nonlinearity | E6-1 |
| 8 | Power nonlinearity | E6-2 |
| 9 | Miscellaneous uncertainty | E2-14 |
| Stage 1: Calibration measurement |  |  |
| 10 | Calibration equipment | F1 |
| 11 | Mismatch of receiver chain | E2-7 |
| 12 | Insertion loss of receiver chain | E2-8 |
| 13 | RF leakage (SGH connector terminated and test range antenna connector cable terminated) | E2-4 |
| 14 | Influence of reference antenna feed cable (flexing cables, adapters, attenuators, connector repeatability) | E2-9 |
| 15 | Uncertainty of absolute gain of reference antenna | F1 |
| 16 | Misalignment positioning system | E2-11 |
| 17 | Misalignment of calibration antenna & pointing error | E2-1 |
| 18 | Rotary joints | E2-12 |
| 19 | Standing wave between reference antenna and test range antenna | E2-3 |
| 20 | Quiet zone ripple reference antenna | E2-5 |
| 21 | Switching uncertainty | E2-15 |
| 22 | Frequency flatness | E2-16 |

10. 10.3.2.1.1.2	MU Value

| UID | Uncertainty Source | Uncertainty Value |  | Distribution | Divisor | ci | Std. uncertainty ui [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f ≦ 3GHz | 3GHz ≦ f < 4.2GHz |  |  |  | f ≦ 3GHz | 3GHz ≦ f < 4.2GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 2 | Measurement equipment（signal generator） | 0.46 | 0.46 | Gaussian | 1 | 1 | 0.46 | 0.46 |
| 3 | Standing wave between DUT and test range antenna | 0.05 | 0.14 | Rectangular | √3 | 1 | 0.03 | 0.08 |
| 4 | RF leakage (calibration antenna connector terminated) | 0.086 | 0.086 | Normal | 1 | 1 | 0.09 | 0.09 |
| 5 | Quiet zone ripple DUT | 0.35 | 0.41 | Rectangular | √3 | 1 | 0.20 | 0.24 |
| 6 | Frequency flatness | 0.13 | 0.13 | Rectangular | √3 | 1 | 0.08 | 0.08 |
| 7 | Time nonlinearity | 0.017 | 0.017 | Rectangular | √3 | 1 | 0.0098 | 0.0098 |
| 8 | Power nonlinearity | 0.06 | 0.06 | Rectangular | √3 | 1 | 0.035 | 0.035 |
| 9 | Miscellaneous Uncertainty | 0.20 | 0.20 | Normal | 1 | 1 | 0.20 | 0.20 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Calibration equipment (i.e. reference receiver, measurement receiver and generator) | 0.059 | 0.059 | Normal | 1 | 1 | 0.06 | 0.06 |
| 11 | Mismatch (i.e. reference receiver, reference antenna, generator and reference cable) | 0.06 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.03 |
| 12 | Insertion loss variation in receiver chain | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 13 | RF leakage (calibration antenna connector terminated) | 0.086 | 0.086 | Normal | 1 | 1 | 0.09 | 0.09 |
| 14 | Influence of the calibration antenna feed cable | 0.103 | 0.104 | Exp. normal | 2 | 1 | 0.06 | 0.06 |
| 15 | Uncertainty of the absolute gain of the calibration antenna | 0.29 | 0.25 | Exp. normal | 2 | 1 | 0.15 | 0.13 |
| 16 | Misalignment of positioning system | 0.00 | 0.00 | Exp. normal | 2 | 1 | 0.00 | 0.00 |
| 17 | Misalignment of calibration antenna & pointing error | 0.5 | 0.5 | Rectangular | √3 | 1 | 0.29 | 0.29 |
| 18 | Rotary joints | 0.00 | 0.00 | U-shaped | √2 | 1 | 0.00 | 0.00 |
| 19 | Standing wave between reference antenna and test range antenna | 0.12 | 0.12 | Rectangular | √3 | 1 | 0.07 | 0.07 |
| 20 | Quiet zone ripple reference antenna | 0.10 | 0.12 | Rectangular | √3 | 1 | 0.06 | 0.07 |
| 21 | Switching uncertainty | 0.02 | 0.02 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 22 | Field calibration repeatability | 0.13 | 0.31 | Rectangular | √3 | 1 | 0.08 | 0.18 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.65 | 0.68 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.30 | 1.33 |

E.6	2D Compact Range

E6-1 Time nonlinearity

In the process of Base station OTA testing, these chips must working continually for about more than 1 hours, So the stability of the test system need to be considered.

E6-2 Power nonlinearity

When performing EIRP test, the base station performs beamforming. Although the beam peak has not been formed yet, there is already a beam shape, so the 2D compact range antennas array will form different high and low power level. At this time, it is necessary to consider whether the response of the amplitude modulation phase modulator chips to different powers is linear. If there is nonlinearity, a test error will be introduced.

<END OF CHANGE>