3GPP TSG-RAN WG4 Meeting 92	R4-1908839

# Ljubljana, Slovenia, 26th – 30th August 2019

Source: 	Skyworks Solutions, Inc.

Title: 	Initial Measurement Results for Intra-band Contiguous NR UL CA for FR1

Agenda Item:	9.12.4 Intra-band contiguous UL CA for FR1 power class 3 [NR_RF_FR1]

Document for:	Approval

# Introduction

In Release 16, one of the key UE RF step compared to Release 15 is the addition of intra-band UL CA for NR with both contiguous and non-contiguous cases. This contribution provides a first set of measurement results for contiguous intra-band NR UL CA to assess the required back-off PA back-off based on ACLR and IMD3 and 5 levels for SEM. Some of the data can be further analyzed to provide first guess on non-contiguous case by applying different criteria on IM3 and 5. The evaluated back off is based on the assumptions and requirements proposed in [1] and looks into the behavior of inner and outer allocations as defined in [2]. This first step focuses on PC3.

# Discussion

## Assumptions for Back-off Evaluation

As usual for 3GPP studies, the PA is calibrated for 1 dB MPR for 100RB0 20 MHz DFT-s-OFDM QPSK and 30dBc ACLR. The other assumptions are according to [1]:

- Transceiver impairments of 28dBc image and carrier leakage are added

- Single transmit path, equal PSD and equal back-off is used

- CCs: same SCS and channel BW between 20 and 100 MHz

- SEM and ACLR requirements are based on the aggregated bandwidth with ACLR measurement bandwidths as follows:

  - 100% aggregated bandwidth for wanted and 99% aggregated bandwidth for adjacent in FR1

  - 98% aggregated bandwidth for wanted and adjacent in FR2

- In band emissions are ignored in this first set of measurements for inner allocation until further agreement is achieved.

- Back-off is evaluated for inner and outer definitions of chapter 2.1.2 of [2]

Measurements are done in Band n41 as it is today the largest spectrum available to a single operator with 190 MHz in the U.S. and 160 MHz in China. It shall be considered though that aggregated bandwidths above 100MHz will exceed the 4% maximum UL bandwidth criteria.

Even if Band n41 is used as a first step, NS01 criteria is used (no OOB requirement and spurious emission level at -30dBm) instead of NS04. But the data can be further analyzed for NS04.

## Measurement Plan

The complete measurement plan was as follow:

- Channel combinations measured: 20+20 MHz, 50+50 MHz, 100+100 MHz, 60+100 MHz

- DFT-s-OFDM and CP-OFDM QPSK waveforms

- Total Power sweep from 13 to 23 dBm in 0.5 dB steps and equal PSD between CCs

- Were measured: each CC power, total power, ACLR, SEM, lower and upper IMD3/5/7 against SEM and spurious emission levels (-30 dBm/MHz)

- In terms of allocations, a careful selection of inner and outer allocation across variable RB length is done to use best and worst cases that just at both sides of the limit of the inner/outer criteria with symmetric and asymmetric CC1 and CC2 allocations, as much as possible the same allocations are used for DFT-s-OFDM and CP-OFDM but in some cases there is a slight difference due to the 2^x*3^y*5^z limitation. Following descriptions are used here assuming CC1 and CC2 have same bandwidth:

  - Inner best: xRB+xRB as close as possible

  - Outer worst: xRB+xRB as far as possible

  - Inner symmetric: based on the inner definition if on CC allocation gets closer to the center of the two CCs the other one can be further away. In this case the two allocation are at the same distance to the center of the 2CCs

  - Inner CCRB0: lowest possible inner allocation in CC1 with lowest possible allocation in CC2

  - Inner asymmetric: CC1 and CC2 allocation are different distance to the two CC center.

  - Outer marginal_sym: outer allocation that is just at the limit of the symmetric inner region. If delta is small then inner allocation region has margin.

  - Outer top: outer positioned just outside the largest possible inner allocation

  - Outer top+1: 1RB more than maximum possible inner in each CC.

  - Then higher outer allocations up to full allocation

- Results provides: back-off rounded to the lowest 0.5 dB, limiting factor and total RB bandwidth.

## Contiguous NR UL CA Results

The results are provided in a set of tables that gives for CP-OFDM and DFT-s-OFDM:

- Back-off needed rounded to the lowest 0.5dB: !!!note that it has no margin applied which will be necessary to account for variations especially for narrow allocations and memory effects creating asymmetric IMDs

- Limiting factor: ACLR, SEM…

- Total RB bandwidth to look into correlation of back-off with IMD bandwidth

If these measured results are not as exhaustive as simulations and their “nice” triangle results, this measurement set already provides a large amount of data. Since for 2CC case there is no real presentation feasible. Still, to ease the analysis, the results are plotted in Figure 1 with back-off versus total RB bandwidth inner and outer allocations use different markers (inner can fit in the corresponding outer marker for CP-OFDM and DFT-s-OFDM, while the different channel configurations use different colors.

Still the tables are important because it provides the corresponding limitation

- Table 1: provides results for 20 MHz+20 MHz cases

- Table 2: provides results for 50 MHz+50 MHz cases

- Table 3: provides results for 100 MHz+100 MHz cases

- Table 4: provides results for 60 MHz+100 MHz cases

Code for Figure 1

- 20+20 MHz (blue), 50+50 MHz (red), 100+100 MHz (purple), 60+100 MHz (green)

- Outer CP-OFDM: Diamond, Outer DFT-s-OFDM: Square, Inner CP-OFDM: +, Inner DFT-s-OFDM: x

Figure 1: PA back-off versus various contiguous CC channel and RB allocations

Observations, only relative back-off behavior is important, not the absolute values, at this time:

- CP-OFDM Outer allocations require at least 6.5 dB back-off and it should be noted that the largest 100+100 MHz bandwidth needs an extra dB back-off

- DFT-s-OFDM outer allocations require at least 5 dB back-off

- Inner CP-OFDM allocations require up to 2 dB back-off

- Inner DFT-s-OFDM allocations needs up to 1.5 dB

- Best inner allocations need no back-off but adding in-band requirements may re-consider this.

- There is a lot of inner cases that match an outer case, this is for the outer cases than are just outside the inner region and shows that there is margin in the way the inner region is defined

To further analyze what is driving the back-off further observations shall be taken from the tables.

Table 1: Results for 20 MHz+20 MHz case

Table 2: Results for 50 MHz+50 MHz case

Table 3: Results for 100 MHz+100 MHz case

Table 4: Results for 60 MHz+100 MHz case

Observations from tables:

- Main limitations come from ACLR (larger allocations) or -13 dBm/MHz SEM (small allocations) region which is also in the adjacent region.

- Difference in back-off between inner and outer comes from the fact that IM3 falls in adjacent region for outer allocations while its IM5 for inner allocations

- Most inner or marginally outer allocation for DFT-s-OFDM are not limited by SEM or ACLR (but possibly by in-band requirements)

- This is also the case for best case CP-OFDM and DFT-s-OFDM inner allocations which do not have even IM5 falling outside the CCs

- The behavior is consistent across all CC configurations

With more time, using IMD3 and IMD5 results of the measurement set, an initial assessment could also be provided for the non-contiguous UL CA case. If time allows, this may be provided in a revision of this document after the deadline for information.

As discussed, it is too early to provide targets but the following proposal should help to focus the work and provide comparable results for measurements and simulations, allowing contributions from more companies.

Proposal for contiguous and non-contiguous NR UL CA MPR evaluation based on measurements or simulations:

- Inner and outer allocations (as in [2]) are evaluated for different MPR (at least for QPSK and 16QAM)

- Different MPR values can be assessed for CP-OFDM and DFT-s-OFDM waveforms

- To finalize inner allocation assessment, the need for in band-emissions and EVM requirements in 2CC configuration must be decided

- Priority is to settle MPR for QPSK and 16QAM and contiguous UL CA case

## Worst case for NS04 and NS27

Although a complete analysis would be needed to set AMPR requirements for NS04 and NS27 (including using a 3.5 GHz PA) some worst case data can be extracted from the measurements, especially relating to the strict OOB requirements for band n41 and n48.

For NS04 n41C UL CA, for contiguous CCs at the band edge, IMD3 products will have to comply with OOB emissions of -13 dBm/MHz (equivalent to -1 3dBm/MHz NS01 SEM cases), but also will need to meet up to -25 dBm/MHz, this will also apply to IMD5 for inner allocations

The worst back-off data for OOB NS04 based on current evaluation:

- For -13 dBm/MHz IM3 (outer case): > 6 dB for CP-OFDM, > 5 dB for DFT-s-OFDM

- > 6 dB for -25 dBm/MHz IM3 (outer case): > 10 dB for CP-OFDM, > 8.5 dB for DFT-s-OFDM

- > 6 dB for -25 dBm/MHz IM5 (inner case): > 5.5 dB for CP-OFDM, > 5 dB for DFT-s-OFDM

For NS27 n48C UL CA, for contiguous CCs at the band edge, IMD3 to IMD7 products will have to comply with OOB emissions of -40 dBm/MHz.

The worst back-off data for OOB NS04 based on current evaluation which did not use large enough dynamic range to reach -40 dBm/MHz for IM3, in this case, the IMD level at largest back-off is provided instead of back-off value:

- For -40 dBm/MHz IM3 (outer case) : -30 dBm/MHz at 11 dB back-off for CP-OFDM, -32 dBm/MHz at 11 dB back-off for DFT-s-OFDM

- For -40dBm/MHz IM5: > 10 dB for CP-OFDM, > 8.5dB for DFT-s-OFDM

- For -40dBm/MHz IM7: > 8 dB for CP-OFDM, > 6dB for DFT-s-OFDM

# Conclusions

In this contribution we provide an initial set of back-off measurements using requirements and assumption proposed in [1] and inner/outer allocation definitions from [2]. Assuming those can be agreed, we make the further proposals below as a way forward to structure the RAN4 work on NR UL CA for Release 16.

Proposal for contiguous and non-contiguous NR UL CA MPR evaluation based on measurements or simulations:

- Inner and outer allocations (as in [2]) are evaluated for different MPR (at least for QPSK and 16QAM)

- Different MPR values can be assessed for CP-OFDM and DFT-s-OFDM waveforms

- To finalize inner allocation assessment, the need for in band-emissions and EVM requirements in 2CC configuration must be decided

- Priority is to settle MPR for QPSK and 16QAM and contiguous UL CA case

The following observations are useful for NR intra-band UL CA MPR/AMPR studies.

Observations, only relative back-off behavior is important not the absolute values at this time:

- CP-OFDM Outer allocations require at least 6.5dB back-off and it should be noted that the largest 100+100MHz bandwidth needs an extra dB back-off

- DFT-s-OFDM outer allocations require at least 5dB back-off

- Inner CP-OFDM allocations require up to 2dB back-off

- Inner DFT-s-OFDM allocations needs up to 1.5dB

- Best inner allocations need no back-off but adding in-band requirements may re-consider this.

- There is a lot of inner cases that match an outer case, this is for the outer cases than are just outside the inner region and shows that there is margin in the way the inner region is defined

The worst back-off data for OOB NS04 based on current evaluation:

- For -13dBm/MHz IM3 (outer case): > 6dB for CP-OFDM, > 5dB for DFT-s-OFDM

- > 6dB for -25dBm/MHz IM3 (outer case): > 10dB for CP-OFDM, > 8.5dB for DFT-s-OFDM

- > 6dB for -25dBm/MHz IM5 (inner case): > 5.5dB for CP-OFDM, > 5dB for DFT-s-OFDM

The worst back-off data for OOB NS04 based on current evaluation:

- For -40dBm/MHz IM3 (outer case) : -30dBm/MHz at 11dB back-off for CP-OFDM, -32dBm/MHz at 11dB back-off for DFT-s-OFDM

- For -40dBm/MHz IM5: > 10dB for CP-OFDM, > 8.5dB for DFT-s-OFDM

- For -40dBm/MHz IM7: > 8dB for CP-OFDM, > 6dB for DFT-s-OFDM

# References

[1]	R4-190xxxx Proposals on intra-band NR UL CA requirements for FR1, Skyworks Solutions Inc., RAN4#92

[2]	R4-190xxxx Discussion on 2UL allocation types and their impact on required back-off and MSD, Skyworks Solutions Inc., RAN4#92