3GPP TSG-RAN WG4 Meeting #92	R4-1908925

Ljubljana, Slovenia, 26th – 30th August, 2019

Source:	Huawei, HiSilicon

Title:	TP for TR 36.716-03-01 for CA_2A-7A-29A, CA_2A-7C-29A , CA_2A-7A-7A-29A

Agenda item:	8.3.3

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_2A-7A-29A, CA_2A-7C-29A , CA_2A-7A-7A-29A as defined in the revised WID [1].

Reference

[1]	RP-191120, “Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL”, Huawei, HiSilicon

Text Proposal

---Start of changes---

5.X	CA_2-7-29

5.X.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-7A-29A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-7C-29A | - | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-7A-7A-29A | - | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |

5.X.2		ΔTIB,c and ΔRIB,c values

ΔRIB,c values for CA_2-7-29 are derived from CA_2-7, CA_2-29 and CA 7-29.  For the ΔTIB,c the  values from CA_2-7 are used. Values will be defined in the following tables in TS 36.101.

Table 5.x.2-1: ΔTIB,c (four bands)

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-7-29, CA_2-7-7-29 | 2 | 0.5 |
|  | 7 | 0.5 |

Table 5.x.2-2: ΔRIB,c (four bands)

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-7-29, CA_2-7-7-29 | 2 | 0 |
|  | 7 | 0 |

5.X.3	REFSENS requirements

The REFSENS requirements for CA_2A-7A-29A, CA_2A-7C-29A and CA_2A-7A-7A-29A need to be added in TS 36.101 as in tables below:

Table 7.3.1A-0d: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |
| CA_2A-7A-29A CA_2A-7C-29A CA_2A-7A-7A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 29 |  |  | -97 | -94 |  |  |  |

---End of changes---