3GPP TSG-RAN WG4 Meeting #92              	R4-1908986

Ljubljana, SI, 26th – 30th Aug, 2019

Agenda item:	8.14.4

Source:	Samsung

Title:	Discussion and simulation results for BS performance with HST in LTE Rel-16

Document for:	Discussion

# 1	Introduction

During RAN4#91 meeting, the general issues about BS demodulation scenarios and requirements for high speed scenario in LTE Rel-16 were discussed.  Some agreements for PUSCH are captured in the WF [1] as follows:

    - The following deployment scenario are considered:

      - Open space

        - Bi-directional (1st priority)

        - Uni-directional

      - Tunnel for multi-antennas

        - Bi-directional (1st priority)

        - Uni-directional

    - BS-Railway track distance (Dmin) and Initial distance of the train from BS (Ds/2):

      - Same as existing HST scenario 1 for open space and 3 for tunnel

      - Other options are not precluded

  - WF on PUSCH for HST evaluation (not for requirement)

    - Deployment scenarios and related parameters (e.g., Ds, Dmin, fd) will be decided based on evaluation results. The following parameters should be considered in the evaluation

    - Maximum Doppler shift will be based  on the outcome of simulation results and could be less than 2 times downlink Doppler shift

For initial evaluation about the impact of high Doppler shift, some test cases are introduced.

In this contribution, the initial results are presented based on initial simulation assumption.

# 2	Simulation results

## 2.1	Performance requirements for PUSCH

As captured in the WF [1], there are four options Doppler value listed for initial evaluation purpose.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Open space |  | Tunnel for multi-antennas |  |
|  | Bi-directional (1st priority) | Uni-directional | Bi-directional (1st priority) | Uni-directional |
| Ds | 1000 m |  | 300 m |  |
| Dmin | 50m |  | 2m |  |
| V | 500km/h (lower speeds are not precluded) |  | 500km/h(Other options are not precluded) |  |
| fd | 1944Hz, 1894Hz,1844Hz,1794Hz,1744Hz,1694Hz |  |  |  |

In this section, the initial simulation result is provided. In table 1, we provide the simulation assumptions for BS demodulation, which is reused in TR36.878.

Table 1: simulation assumptions for PUSCH demodulation performance evaluation

| Parameters | Unit | Values |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| MCS for PUSCH |  | QPSK 1/3 |
| Propagation condition and correlation matrix |  | AWGN |
| Antenna configuration |  | 1x2 |
| Reference receiver |  | MRC |
| Noise estimation |  | Practical |
| Time and frequency track |  | Practical |

In Figure 1, we provide the ideal simulation results for PUSCH under in Open Space for bi-directional scenario with QPSK 1/3.

Figure 1: Simulation results of BS demodulation performance with High Doppler shift value (to be updated)

# 3	Conclusion

In this contribution, the initial simulation result for performance requirements of LTE HST in Rel-16 is provided.

# References

R4-1907747, Way forward on LTE HST BS demodulation performance requirement, NTT DOCOMO