3GPP TSG-RAN WG4 Meeting #92	R4-1909169

Ljubljana, Slovenia, 26 – 30 August 2019

Agenda Item:	9.17.1.3

Source:	Ericsson

Title:	Simulation assumption of UE demodulation requirements for LTE-NR co-existence scenario in TDD

Document for:	Approval

# 1	Introduction

We propose the simulation assumption of UE demodulation requirements for LTE-NR co-existence scenario in TDD [1].

# 2	Simulation assumption

Table 1	Parameters for LTE-NR co-existence scenario in TDD.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 for Test 1-1 11 for Test 1-2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |

Table 2 Test list for LTE-NR co-existence scenario in TDD.

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 (2Rx) | R.PDSCH.1-x.1 TDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |
| 1-2 (2Rx) | R.PDSCH.1-x.2 TDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |
| 1-1 (2Rx) | R.PDSCH.1-x.1 TDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x4, ULA Low | 70 | TBD |
| 1-2 (2Rx) | R.PDSCH.1-x.2 TDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x4, ULA Low | 70 | TBD |

Table 3	FRC for LTE-NR co-existence scenario with TDD UL-DL pattern FR1.15-2.

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-x.1 TDD | R.PDSCH.1-x.2 TDD |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 |  |  |  |
| Number of consecutive PDSCH symbols |  | 9 | 11 |  |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |
| Target Coding Rate |  | 0.30 | 0.30 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 18 | 18 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} for i from {0,…,19} | Bits | N/A | N/A |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} for i from {0,…,19} | Bits | 2472 | 3240 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} for i from {0,…,19} | Bits | N/A | N/A |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} for i from {0,…,19} | Bits | 16 | 16 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} for i from {0,…,19} | CBs | N/A | N/A |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} for i from {0,…,19} | CBs | 1 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} for i from {0,…,19} | Bits | N/A | N/A |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} for i from {0,…,19} | Bits | 8384 | 10880 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 0.989 | 1.296 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with LTE PBCH/PSS/SSS |  |  |  |  |  |  |

Table 4	NR TDD UL-DL configurations

| Parameter |  | Unit | UL-DL pattern | UL-DL pattern |
| --- | --- | --- | --- | --- |
|  |  |  | FR1.15-1 | FR1.15-2 |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSU | DSUDD |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U | 10D+2G+2U |
| referenceSubcarrierSpacing |  | kHz | 15 | 15 |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) |  |  |  |  |
| Pattern 1 | dl-UL-TransmissionPeriodicity | ms | 5 | 3 |
|  | nrofDownlinkSlots |  | 3 | 1 |
|  | nrofDownlinkSymbols |  | 10 | 10 |
|  | nrofUplinkSlot |  | 1 | 1 |
|  | nrofUplinkSymbols |  | 2 | 2 |
| Pattern 2 | dl-UL-TransmissionPeriodicity | ms | N/A | 2 |
|  | nrofDownlinkSlots |  | N/A | 2 |
|  | nrofDownlinkSymbols |  | N/A | 0 |
|  | nrofUplinkSlot |  | N/A | 0 |
|  | nrofUplinkSymbols |  | N/A | 0 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 4 if mod(i,5) = 0 3 if mod(i,5) = 1 2 if mod(i,5) = 2 6 if mod(i,5) = 3 | 2 if mod(i,5) = 0 6 if mod(i,5) = 1 4 if mod(i,5) = 2 3 if mod(i,5) = 3 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9}. |  |  |  |  |

# References

- R4-1909168, “Discussion on UE demodulation requirements for LTE-NR co-existence scenario in TDD”, Ericsson.