3GPP TSG-RAN WG4 Meeting #92                                                               R4-1909175

Ljubljana, SI, 26th-30th August, 2019

Source: 	Huawei, HiSilicon

Title:	TP on NB-IoT operating in NR guard band

Agenda Item:	8.12.2

Document for:	Approval

1	Introduction

In last RAN4 meeting #91, following agreements on NB-IoT operation in NR guard band were approved for NB-IoT coexistence with NR.

- Agreements on NB-IoT operation in NR guard band

  - Considering legacy deployments, even if one NB-IoT 15 kHz sub-carrier would operate in NR guard band, still consider this case as NB-IoT operation in NR in-band.

  - NB-IoT operating in NR guard band is handled as implementation issue and RF requirements are not specified in Rel-15 (nor in Rel-16 unless new Rel-16 features make this essential).

  - Define in-band, guard band and standalone operation with NR.

In this contribution we provide TP on NB-IoT operating in NR guard band.

Text proposal

5.3	NB-IoT operating in NR guard-band

Compared to LTE which has fixed spectrum utilization for channel bandwidth larger than and equal to 3 MHz, the spectrum utilization has been improved for NR especially for 15 kHz SCS. Spectrum utilization values in Rel-15 are the results of companies’ compromise. NB-IoT operating in NR guard band need further increase the spectrum utilization hence NB-IoT should not be considered for operating in NR guard band for 15 KHz SCS. For SCS other than 15 KHz, in some scenarios, the spectrum utilization may be less than that in LTE. For those scenarios some extra guard need to be considered for the co-existence of mixed numerology. As agreed in Clause 5.2.3, no requirement would be specified for mixed numerology. Again the spectrum utilization was optimized for 30 kHz and 60 kHz SCS, the operation in guard band will depends on the implementation margin and need to be investigated case by case. For these reasons RAN4 made following agreements on NB-IoT operation in NR guard band.

  - Considering legacy deployments, even if one NB-IoT 15 kHz sub-carrier would operate in NR guard band, still consider this case as NB-IoT operation in NR in-band.

  - NB-IoT operating in NR guard band is handled as implementation issue and RF requirements are not specified in Rel-15 (nor in Rel-16 unless new Rel-16 features make this essential).

  - Define in-band, guard band and standalone operation with NR.

References

[1] R4-1907809, WF on coexistence of NB-IoT with NR, Huawei, HiSilicon, Ericsson, Nokia,

ZTE, DISH, T-Mobile USA, CHTTL