3GPP TSG-RAN WG4 Meeting #92	R4-1909181

Ljubljana, Slovenia, 26th– 30th August, 2019

Source: 	Huawei, HiSilicon

Title:	A-MPR for n38 40 MHz co-existence

Agenda Item:	10.20.1

Document for:	Discussion

1	Introduction

In last RAN plenary meeting #83, a new WI was approved to add wider channel bandwidths in NR band n38 [1]. The objective of this WI is to add to NR band n38 the support of channel bandwidths of 40 MHz.

In this contribution we provide update on simulation results on A-MPR for n38 40 MHz co-existence. RBend is used as the variation. Furthermore, A-MPR table is suggested for 40 MHz.

2	Discussion

For the wider channel bandwidths in NR band n38, one of expected impacts is band 7 protection. In Release 15, the supported maximum channel bandwidth for NR band n38 is 20 MHz which is the same as in LTE.

Table 2-1: Requirements for spurious emissions for UE co-existence for n38

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

It can be found from the co-existence table that the protection of band 7 is achieved through uplink transmission restrictions. For 15 MHz and 20 MHz, the transmission bandwidth is up to 54 RB. While for 40 MHz channel bandwidth, addition to wide band spectral regrowth the IM3 and CIM3 of small RB allocation will exceed the emission limits, as verified in LTE CA_38C.

A-MPR simulation results for 40 MHz channel bandwidth are shown in Figure 2-1 and 2-2. The additional emission limits are according to the UE-UE coexistence table. The RF impairments assumptions are 28dB IQ image and LO suppression, 60 dB CIM3 suppression. And PA calibration point is set so that ACLR is just met with full QPSK allocation.

Figure 2-1: A-MPR for 40 MHz CBW DFT-s-OFDM

Figure 2-2: A-MPR for 40 MHz CBW CP-OFDM)

Similar back-off regions as CA_NS_05 in TS 36.101are adopted as starting point. Due to the different SU and SCS from LTE, which may result different regions and A-MPR values. A-MPR is shown in Table 2 and 3 for further discussion.

Table 2: A-MPR for NS_XX

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS | LCRB*12*SCS |  |
|  |  | MHz | MHz |  |
| 40 MHz | 2590 ≤ FC ≤ 2595 | ≥0, <2.88 | >0 | A1 |
|  |  | ≥2.88, <14.4 | >max (0, 12*SCS*RBend - 3.6) | A2 |
|  |  | ≥14.4, <23.4 | >10.8 | A3 |
|  |  | ≥23.4, <32.4 | >16.2 | A4 |
|  |  | ≥32.4 | >0 | A5 |

Table 3: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | [5] | [2] | [3] | [4] | [12] |
|  | QPSK | [5] | [2] | [3] | [4] | [12] |
|  | 16 QAM | [5] | [2] | [3] | [4] | [12] |
|  | 64 QAM | [5] | [2] | [3] | [4] | [12] |
|  | 256 QAM | [5] | [2] | [3] | [4] | [12] |
| CP-OFDM | QPSK | [5] | [4] | [5] | [6] | [12] |
|  | 16 QAM | [5] | [4] | [5] | [6] | [12] |
|  | 64 QAM | [5] | [4] | [5] | [6] | [12] |
|  | 256 QAM | [5] | [4] | [5] | [6] | [12] |

3	Conclusions

In this contribution we provide simulation results and A-MPR proposals on protection of band 7 for channel bandwidth 40 MHz of band n38.

4	References

[1] RP-190392, New WI proposal: addition of new wider channel bandwidth in Band n38

[2] R4-1905570, A-MPR for n38 40 MHz co-existence