3GPP TSG-RAN WG4 Meeting #92	R4-1909206

Ljubljana, Slovenia, 26th– 30th August, 2019

Source: 	Huawei, HiSilicon

Title:	Discussion on adding wider channel bandwidths in Band n77/n78

Agenda Item:	10.23

Document for:	Discussion

1	Introduction

In last RAN plenary meeting #84, a new WI was approved to add wider channel bandwidths in NR band n77 and n78 [1].

In this contribution we provide an overview on the affected RAN4 specifications.

2	Discussion

38.101-1

For band n77 and n78, no additional emission requirements are required, i.e. NS_01 will be used and A-MPR of 0 dB shall be used.

25 MHz and 30 MHz UE channel bandwidth are already included for other NR bands in Rel-15, while 70 MHz has never been a UE channel bandwidth before. Hence general requirements for 70 MHz should be introduced, e.g. spectrum utilization related limits. It is proposed to re-use the 70 MHz SU defined in BS specification. The following update is needed correspondingly.

5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each UE channel bandwidth and SCS is specified in Table 5.3.3-1,

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 552.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 905 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1610 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

UE reference sensitivity is a band specific requirement. For all 3 new channel bandwidths, UE reference sensitivity need to be defined. The requirements can be derived by scaling on the transmission bandwidth configuration.

38.104

The BS RF requirements for different channel bandwidth and SCS are defined in a band agnostic manner. It is anticipated that there is no specific BS RF requirement need to be supplemented. Hence the only change needed for TS 38.104 is to add these channel bandwidths in the Table 5.3.5-1.

= = = Required change for 38.104 Table 5.3.5-1= = =

Table 5.3.5-1: BS channel bandwidths and SCS per operating band in FR1

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

Optional support in Rel-15

Current 38.307 does not include the channel bandwidth as a release independent feature. Channel bandwidth is rather a capability than a feature. One approach is to add these channel bandwidths in Rel-15 38.101 and mark them as optional. Further discussion and decision is needed.

As discussed, 25 MHz and 30 MHz UE channel bandwidth are already included for other NR bands in Rel-15, and the signalling can support these channel bandwidths. Meanwhile for 70 MHz channel bandwidth, UE capability signalling for 70 MHz CBW need to be introduced in RAN2 specifications.

Enable optional UE support of 25 MHz

3	Conclusions

In this contribution we provide an overview on the affected RAN4 specifications for the addition of new channel bandwidth for band n77 and n78. The corresponding CRs can be found in [2], [3] and LS to RAN2 can be found in [4].

4	References

[1] RP-191606, New WI proposal: Addition of channel bandwidths in Band n77 and n78

[2] R4-1909207, draft CR for TS 38.101-1: adding wider channel bandwidths, Huawei

[3] R4-1909209, draft CR for TS 38.104: adding wider channel bandwidths, Huawei

[4] R4-1909208, LS to RAN2 on adding 70MHz UE CBW in Band n77/n78