3GPP TSG-RAN WG4 Meeting #92												  	     		    R4-1909272

Ljubljana, Slovenia, 26 – 30 August, 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Initial IAB-Node coexistence simulation results

Agenda item:			9.5.2

Document for:			Approval

# 1	Introduction

Part of the Integrated Access and Backhaul work item is defining the RF requirements needed to guarantee coexistence with IAB-Nodes and legacy networks. In this contribution we provide initial simulation results.

# 2	Discussion

In RAN4#91 initial simulation assumptions were agreed in [1] and [2], and these assumptions have been used in the simulations. Not all required detailed parameters, such as the activity factor of IAB-Nodes have been set in the simulation assumptions, and for those parts the simulation model has been described more in detail in this contribution. The main simulation parameters have been summarized in Table 1.

| Parameters |  |
| --- | --- |
| Duplex mode | TDD (50% UL, 50% DL) |
| Carrier Frequency | 30 GHz |
| Beamforming | Yes |
| Simulation bandwidth | 200 MHz |
| Number of UEs in the network | 5 UEs/sector |
| gNB Tx power | 33 dBm for macro and micro |
| maximum IAB node Tx power | 33 dBm |
| maximum gNB EIRP | 57 dBm |
| maximum IAB node EIRP | 57 dBm |
| gNB antenna height | 25m (macro cells); 10m (micro cells) |
| IAB node antenna height | 25m (macro cells); 10m (micro cells) |
| gNB receiver noise figure | 10 dB |
| IAB node receiver noise figure | 10 dB |
| UE Tx power (dBm) | 22.4 dBm EIRP (13.4 dBm conducted) |
| UE noise figure (dB) | 10 |
| Subcarrier spacing | 120 kHz |
| Maximum number of IAB hops to egress node | 3 |
| Traffic Model | Full buffer |
| Transmit Method | Rank adaptive SU-MIMO with maximum rank of 2 |
| CSI Feedback | CRI/PMI/RI/CQI, 10ms period, 5ms delay |
| Maximum modulation order | 256QAM |

The initial simulations were performed using current BS type 2-O ACLR and ACS requirements, and to continue the work the intention is to study different ACS and ACLR values to find the minimum required ACLR and ACS which still enables minimal performance degradation to legacy networks. Only wideband resource blocks allocations were used, i.e. no frequency domain scheduler was used in the simulations.

In Figure 1 results for layout 1 are shown. The baseline results show UE UL and DL throughputs in victim network when no IAB-Node exists in the layout. When IAB-Nodes are added, the results remain practically unchanged. Due to the used power control, IAB-Node Tx powers ended up being from 25 dBm to -17 dBm (TRP).

Figure 1: Co-existence simulation results for layout 1.

In Figure 2 results for layout 2 are shown. In scenario 1 IAB-Node transmissions towards the egress node take place during UL-timeslots and in scenario 2 during DL timeslots. The difference in performance with and without IAB-Node aggressor and with IAB-Nodes are minimal. There is virtually no difference between scenario 1 and scenario 2 when it comes to victim network throughput. However, as DL timeslots are shared between backhaul and access transmission in the IAB-Node, the network capacity is worse in scenario 2. While IAB-parents need to share their DL timeslots between the child backhaul and access link transmission in scenario 1, in scenario 2 DL timeslots are additionally consumed for backhaul link reception. Due to the used power control, IAB-Node Tx powers ended up being from 33 dBm to 5 dBm (TRP).

Figure 2: Co-existence simulation results for layout 2.

In both layouts the SNR-driven power control scheme results in significant power backoffs. As the transmissions towards access users and backhaul parent may share the same RF hardware, applying such backoffs would bring a need to very quickly reconfigure the transmitter chain to apply up to 50 dB backoff. This may introduce additional delay in transitioning from backhaul transmission to access transmission, which degrade system performance. Therefore, it is proposed that RAN4 works further on possible restrictions on the power control.

Proposal 1: RAN4 to work further on power control scheme and possible restrictions on the power control dynamic range.

# 3	Conclusion

In this contribution initial coexistence simulation results were provided. It can be seen that there are no coexistence issues when current BS ACLR and ACS are applied. Further work is needed to see if these values can be relaxed. Additionally, the following proposal was made.

Proposal 1: RAN4 to work further on power control scheme and possible restrictions on the power control dynamic range.

# 4	References

[1]	R4-1907824 IAB Ad Hoc meeting minutes, Qualcomm Incorporated

[2]	R4-1907825 WF on simulation assumptions for IAB co-existence study.