3GPP TSG-RAN4 Meeting #92	R4-1909331

# Ljubljana, Slovenia, 26th - 30th Aug, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Scheduling restriction on TCI state switching delay (section 8.10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-06-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - There is no scheduling restriction for MAC CE based TCI state switch. UE is able to receive on the old TCI state until slot n+ THARQ +3ms +TOk*(Tfirst-SSB). During TSSB-proc, UE may retune its Rx beam to the new TCI state to perform time-frequency synchronization if the old and new TCI state is not QCLed. Thus the scheduling restriction shall be specified, that is, UE is not required to receive PDCCH during TSSB-proc. - The unit is not uniform(slot and millisecond) - TO shall be TOuk rather than TOk in unknown case |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding scheduling restriction for MAC CE based TCI state switch. - Slot and millisecond is alligned - Correct TOk in unknown case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ (THARQ +3ms +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length. The UE shall be able to receive on the old TCI state until slot n+ (THARQ +3ms +TOk*(Tfirst-SSB)) / NR slot length and is not required to receive PDCCH during TSSB-proc.

Where THARQ is the timing between DL data transmission and acknowledgement as specified in [7];

Tfirst-SSB is time to first SSB transmission after TCI state command is received by the UE;

TSSB-proc =2ms;

TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ (THARQ +3ms + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc)) / NR slot length . The UE shall be able to receive on the old TCI state until slot n+ (THARQ +3ms+ TL1-RSRP +TOuk*(Tfirst-SSB))/ NR slot length and is not required to receive PDCCH during TSSB-proc.

Where T L1-RSRP  is the time for L1-RSRP measurement for Rx beam refinement, defined as

  - TBM_Measurement_Period_SSB for SSB in FR2 as specified in section 9.5.4.1, with the assumption of M=1

  - TBM_Measurement_Period_CSI-RS for CSI-RS  in FR2 as specified in section 9.5.4.2

    - with the assumption of M=1 for periodic CSI-RS

    - for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE;

TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement

If the target TCI state is not measured and reported by the UE prior to TCI state switching, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ (THARQ +3ms + TL1-RSRP + TOuk* (Tfirst-SSB + TSSB-proc)) / NR slot length . The UE shall be able to receive on the old TCI state until slot n+ (THARQ +3ms+ TL1-RSRP +TOuk*(Tfirst-SSB)) / NR slot length and is not required to receive PDCCH during TSSB-proc.

During MAC CE based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in section 8.2.

### 8.10.4	DCI based TCI switch delay

If the target TCI state is known, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling the PDSCH at slot n, UE shall be able to receive PDSCH or transmit PUSCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.306[14].

The known condition for TCI state defined in section 8.10.2 is applied.

During DCI based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in section 8.2.

### 8.10.5	RRC based TCI state delay

If the target TCI state is known, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ (TRRC_processing  +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length. Where TRRC_processing is the RRC processing delay, Tfirst-SSB, TSSB-proc and TOk are as defined in Section 8.10.3. The UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH until the end of switching period.

If the target TCI state is unknown, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ (TRRC_processing  +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length . Where TRRC_processing is the RRC processing delay, Tfirst-SSB and TOuk are as defined in Section 8.10.3. The UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH until the end of switching period.

During RRC based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in section 8.2.

### 8.10.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new TCI state no later than slot n+ (THARQ +3ms +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length, where THARQ, Tfirst-SSB, TSSB-proc and TOk are as defined in Section 8.10.3.

<End of Change 1>