3GPP TSG-RAN4 Meeting # 92	 R4-1909366

# Ljubljana, Slovenia,  Aug 26 - 30

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 1238 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR of adding LTE B42/B43 for UE category NB1 in R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson; Softbank |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_bands_R16_M1_NB1-Perf |  |  |  |  |  | Date: |  |  | 2019-8-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Release 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding B42/B43 for NB1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding requirement for B42/B43 for NB1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing B42/B43 for NB1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 7.2.5, 7.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < start of changes >

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1. Unless stated otherwise, requirements specified for the TDD duplex mode apply for downlink and uplink operations in Frame Structure Type 2.

NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 25, 26, 28, 31, 41 (in certain regions), 42, 43, 65, 66, 70, 71, 72, 73, 74, 85, 87, 88 which are defined in Table 5.5-1.

## << Unchanged part is omitted>>

### 7.2.5	Test requirement

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.5-1 for Wide Area BS, in Table 7.2.5-2 for Local Area BS, in Table 7.2.5-3 for Home BS and in Table 7.2.5-4 for Medium Range BS.

Table 7.2-1: E-UTRA Wide Area BS reference sensitivity levels

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |
| 1.4 | FRC A1-1 in Annex A.1 | -106.1 | -105.8 |
| 3 | FRC A1-2 in Annex A.1 | -102.3 | -102.0 |
| 3 | FRC A1-6 in Annex A.1 for E-UTRA with NB-IoT in-band operation Note 3 | -102.3 Note 2 | N/A |
| 5 | FRC A1-3 in Annex A.1 | -100.8 | -100.5 |
| 5 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation | -100.8 Note 2 | N/A |
| 10 | FRC A1-3 in Annex A.1 Note 1 | -100.8 | -100.5 |
| 10 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation Note 4 | -100.8 Note 2 | N/A |
| 15 | FRC A1-3 in Annex A.1 Note 1 | -100.8 | -100.5 |
| 15 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation Note 4 | -100.8 Note 2 | N/A |
| 20 | FRC A1-3 in Annex A.1 Note 1 | -100.8 | -100.5 |
| 20 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation Note 4 | -100.8 Note 2 | N/A |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. Note 2: The requirements apply to BS that supports E-UTRA with NB-IoT in-band operation. Note 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of FRC A1-6 mapped to the 12 E-UTRA resource blocks adjacent to the NB-IoT PRB. Note 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of FRC A1-7 mapped to the 24 E-UTRA resource blocks adjacent to the NB-IoT PRB (location of which is specified in sub-clause 4.7.3), and for each consecutive application of a single instance of FRC A1-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |

Table 7.2-2: E-UTRA Local Area BS reference sensitivity levels

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| 1.4 | FRC A1-1 in Annex A.1 | -98.1 | -97.8 |  |
| 3 | FRC A1-2 in Annex A.1 | -94.3 | -94.0 |  |
| 3 | FRC A1-6 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 4) | -94.3 (Note 3) |  |  |
| 5 | FRC A1-3 in Annex A.1 | -92.8 | -92.5 |  |
| 5 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation | -92.8 (Note 3) |  |  |
| 10 | FRC A1-3 in Annex A.1 (Note 1) FRC A1-8 in Annex A.1 (Note 2) | -92.8 | -92.5 | -94.7 |
| 10 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 5) | -92.8 (Note 3) |  |  |
| 15 | FRC A1-3 in Annex A.1 (Note 1) | -92.8 | -92.5 |  |
| 15 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 5) | -92.8 (Note 3) |  |  |
| 20 | FRC A1-3 in Annex A.1 (Note 1) FRC A1-9 in Annex A.1 (Note 2) | -92.8 | -92.5 | -94.7 |
| 20 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 5) | -92.8 (Note 3) |  |  |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. This reference measurement channel is not applied for Band 46 and Band 49. Note 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be tested for at least one interlace of FRC A1-8 (if supported) and A1-9. This reference measurement channel is only applied for Band 46 and Band 49. Note 3: The requirements apply to BS that supports E-UTRA with NB-IoT in-band operation. Note 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of FRC A1-6 mapped to the 12 E-UTRA resource blocks adjacent to the NB-IoT PRB. Note 5: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of FRC A1-7 mapped to the 24 E-UTRA resource blocks adjacent to the NB-IoT PRB (location of which is specified in sub-clause 4.7.3), and for each consecutive application of a single instance of FRC A1-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |  |

Table 7.2-3: E-UTRA Home BS reference sensitivity levels

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |
| 1.4 | FRC A1-1 in Annex A.1 | -98.1 | -97.8 |
| 3 | FRC A1-2 in Annex A.1 | -94.3 | -94.0 |
| 3 | FRC A1-6 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 3) | -94.3 (Note 2) |  |
| 5 | FRC A1-3 in Annex A.1 | -92.8 | -92.5 |
| 5 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation | -92.8 (Note 2) |  |
| 10 | FRC A1-3 in Annex A.1 (Note 1) | -92.8 | -92.5 |
| 10 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 4) | -92.8 (Note 2) |  |
| 15 | FRC A1-3 in Annex A.1 (Note 1) | -92.8 | -92.5 |
| 15 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 4) | -92.8 (Note 2) |  |
| 20 | FRC A1-3 in Annex A.1 (Note 1) | -92.8 | -92.5 |
| 20 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 4) | -92.8 (Note 2) |  |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each Note 2: The requirements apply to BS that supports E-UTRA with NB-IoT in-band operation. Note 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of FRC A1-6 mapped to the 12 E-UTRA resource blocks adjacent to the NB-IoT PRB. Note 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of FRC A1-7 mapped to the 24 E-UTRA resource blocks adjacent to the NB-IoT PRB (location of which is specified in sub-clause 4.7.3), and for each consecutive application of a single instance of FRC A1-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |

Table 7.2-4: E-UTRA Medium Range BS reference sensitivity levels

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| 1.4 | FRC A1-1 in Annex A.1 | -101.1 | -97.8 |  |
| 3 | FRC A1-2 in Annex A.1 | -97.3 | -94.0 |  |
| 3 | FRC A1-6 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 4) | -97.3 (Note 3) |  |  |
| 5 | FRC A1-3 in Annex A.1 | -95.8 | -92.5 |  |
| 5 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation | -95.8 (Note 3) |  |  |
| 10 | FRC A1-3 in Annex A.1 (Note 1) FRC A1-8 in Annex A.1 (Note 2) | -95.8 | -92.5 | -97.7 |
| 10 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 5) | -95.8 (Note 3) |  |  |
| 15 | FRC A1-3 in Annex A.1 (Note 1) | -95.8 | -92.5 |  |
| 15 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 5) | -95.8 (Note 3) |  |  |
| 20 | FRC A1-3 in Annex A.1 (Note 1) FRC A1-9 in Annex A.1 (Note 2) | -95.8 | -92.5 | -97.7 |
| 20 | FRC A1-7 in Annex A.1 for E-UTRA with NB-IoT in-band operation (Note 5) | -95.8 (Note 3) |  |  |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. This reference measurement channel is not applied for Band 46. Note 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be tested for at least one interlace of FRC A1-8 (if supported) and A1-9. This reference measurement channel is only applied for Band 46. Note 3: The requirements apply to BS that supports E-UTRA with NB-IoT in-band operation. Note 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of FRC A1-6 mapped to the 12 E-UTRA resource blocks adjacent to the NB-IoT PRB. Note 5: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of FRC A1-7 mapped to the 24 E-UTRA resource blocks adjacent to the NB-IoT PRB (location of which is specified in sub-clause 4.7.3), and for each consecutive application of a single instance of FRC A1-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |  |

For NB-IoT standalone BS or E-UTRA BS with NB-IoT (in-band and/or guard band), NB-IoT throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A with parameters specified in Table 7.2-5 for Wide Area BS, Table 7.2-6 for Local Area BS, Table 7.2-7 for Home BS and Table 7.2-8 for Medium Range BS.

Table 7.2-5: NB-IoT Wide Area BS reference sensitivity levels

| NB-IoT channel bandwidth [kHz] | NB-IoT Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] (f≤3.0 GHz) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -126.6 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -132.6 |

Table 7.2-6: NB-IoT Local Area BS reference sensitivity levels

| NB-IoT channel bandwidth [kHz] | NB-IoT Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] (f≤3.0 GHz) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -118.6 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -124.6 |

Table 7.2-7: NB-IoT Home BS reference sensitivity levels

| NB-IoT channel bandwidth [kHz] | NB-IoT Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |  |
| --- | --- | --- | --- | --- |
|  |  |  |  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |
| 200 |  | 15 |  | FRC A14-1 in Annex A.14 |  | -118.6 | -118.3 |
| 200 |  | 3.75 |  | FRC A14-2 in Annex A.14 |  | -124.6 | -124.3 |

Table 7.2-8: NB-IoT Medium Range BS reference sensitivity levels

| NB-IoT channel bandwidth [kHz] | NB-IoT Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] (f≤3.0 GHz) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -121.6 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -127.6 |

For E-UTRA BS with subPRB allocation, subPRB allocation throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A with parameters specified in Table 7.2-9 for Wide Area BS, in Table 7.2-10for Local Area BS, in Table 7.2-11 for Home BS and in Table 7.2-12 for Medium Range BS.

Table 7.2-9: SubPRB allocation for Wide Area BS reference sensitivity levels

| SubPRB transmission bandwidth [kHz] | subPRB Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |
| --- | --- | --- | --- |
| 30 | 15 | FRC A23-1 in Annex A. | -124 |

Table 7.2-10: subPRB allocation for Local Area BS reference sensitivity levels

| SubPRB transmission bandwidth [kHz] | subPRB Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |
| --- | --- | --- | --- |
| 30 | 15 | FRC A23-1 in Annex A. | -116 |

Table 7.2-11: subPRB allocation for Home BS reference sensitivity levels

| SubPRB transmission bandwidth [kHz] | subPRB Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |
| --- | --- | --- | --- |
| 30 | 15 | FRC A23-1 in Annex A. | -116 |

Table 7.2-12: subPRB allocation for Medium Range BS reference sensitivity levels

| SubPRB transmission bandwidth [kHz] | subPRB Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |
| --- | --- | --- | --- |
| 30 | 15 | FRC A23-1 in Annex A. | -119 |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The relationship between Minimum Requirements and Test Requirements is defined in subclause 4.1 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

## << Unchanged part is omitted>>

### 7.6.5	Test Requirements

#### 7.6.5.1	General requirement

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS antenna input using the parameters in Tables 7.6-1, 7.6-1a, 7.6-1b, 7.6-1c and 7.6-2. The reference measurement channel for the wanted signal is specified in Tables 7.2-1, 7.2-2, 7.2-3 and 7.2-4 for each channel bandwidth and further specified in Annex A.

For each measured NB-IoT carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS antenna input using the parameters in Tables 7.6-1d, 7.6-1e, 7.6-1f, 7.6-1g, 7.6-1h, 7.6-1i, 7.6-1j, 7.6-1k, 7.6-2a and 7.6-2b. The reference measurement channel for the wanted signal is specified in Table 7.2-5 for each subcarrier spacing option and further specified in Annex A.

The blocking requirement is always applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For a BS operating in non-contiguous spectrum within any operating band, the blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in Table 7.6-2. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a BS capable of multi-band operation, the requirement in the in-band blocking frequency ranges applies for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in Table 7.6-2.

For a BS capable of multi-band operation, the requirement in the out-of-band blocking frequency ranges apply for each operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to Tables 7.6-1, 7.6-1a and 7.6-1c shall be excluded from the out-of-band blocking requirement.

For the Public Safety LTE BS in Korea from 718 to 728 MHz in band 28, the wanted and the interfering signal coupled to the BS antenna input are specified in Tables G-2.2, G-2.3, G-2.4 and G-2.5 for the band blocking requirements in annex G.2 of [2]. The reference measurement channel for the wanted signal is A.1-3 for 10 MHz channel bandwidth and further specified in Annex A.

Table 7.6-1: Blocking performance requirement for Wide Area BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] * | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18, 19, 21-23, 24, 27, 30, 33-45, 48, 50, 52, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -43 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -43 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -43 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -43 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -43 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -43 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -43 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |  |
| Note*: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [2] subclause 7.2.1. Note**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to PREFSENS + 1.4 dB. |  |  |  |  |  |  |  |

NOTE:	Table 7.6-1 assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6-1a: Blocking performance requirement for Local Area BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] * | Interfering signal centre frequency minimum frequency offset from the lower(upper) edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13-14, 18,19,21-23, 24, 27, 30, 33-45, 48-53, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -35 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -35 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -35 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -35 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -35 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -35 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 46 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB* | See table 7.6-2 | See table 7.6-2 |
|  | (FUL_low -500) (FUL_high +20) | to to | (FUL_low -20) (FUL_high +500) | -35 | PREFSENS +6dB* |  | CW carrier |
|  | 1 (FUL_high +500) | to to | (FUL_low -500) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -35 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |  |
| Note*: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [2] subclause 7.2.1. Note**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to PREFSENS + 1.4 dB. |  |  |  |  |  |  |  |

NOTE:	Table 7.6-1a assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6-1b: Blocking performance requirement for Home BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] * | Interfering signal centre frequency minimum frequency offset from the channel edge of the wanted signal [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-44, 48, 50-52, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -27 | PREFSENS +14dB | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -27 | PREFSENS +14dB | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -27 | PREFSENS +14dB | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -27 | PREFSENS +14dB | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -27 | PREFSENS +14dB | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +14dB |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -27 | PREFSENS +14dB | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB |  | CW carrier |
| 74 | (FUL_low -20) | to | (FUL_high +5) | -27 | PREFSENS +14dB | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -27 | PREFSENS +14dB | See table 7.6-2 | See table 7.6-2 |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB |  | CW carrier |  |
| Note*: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [2] subclause 7.2.1. |  |  |  |  |  |  |  |

NOTE:	Table 7.6-1b assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6-1c: Blocking performance requirement for Medium Range BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] * | Interfering signal centre frequency minimum frequency offset to the lower (higher) edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-45, 48, 50, 52-53, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -38 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -38 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -38 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -38 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -38 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -38 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 46 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB* | See table 7.6-2 | See table 7.6-2 |
|  | (FUL_low -500) (FUL_high +20) | to to | (FUL_low -20) (FUL_high +500) | -35 | PREFSENS +6dB* |  | CW carrier |
|  | 1 (FUL_high +500) | to to | (FUL_low -500) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -38 | PREFSENS +6dB** | See table 7.6-2 | See table 7.6-2 |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |  |
| Note*: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [2] subclause 7.2.1. Note**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to PREFSENS + 1.4 dB. |  |  |  |  |  |  |  |

NOTE:	Table 7.6-1c assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1d: Blocking performance requirement for Wide Area BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 11, 13-14,18,19, 21, 26, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6dB* | See table 7.6.2a | See table 7.6. 2a |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -43 | PREFSENS +6dB* | See table 7.6.2a | See table 7.6. 2a |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -43 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -43 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -43 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -43 | PREFSENS +6dB* | See table 7.6-2a | See table 7.6-2a |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -43 | PREFSENS +6dB* | See table 7.6-2a | See table 7.6-2a |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -43 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |  |
| Note*: PREFSENS is specified in TS 36.104 [2] subclause 7.2.1 Note**: Up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1d assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1e: Blocking performance requirement for Wide Area BS for E-UTRA with NB-IoT in-band/guard band operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 11, 13-14,18,19, 21, 26, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -43 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -43 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -43 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -43 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -43 | PREFSENS +6dB* | See table 7.6-2b | See table 7.6-2b |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -43 | PREFSENS +6dB* | See table 7.6-2b | See table 7.6-2b |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -43 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |  |
| Note*: PREFSENS depends on the channel bandwidth or supported subcarrier spacing as specified in TS 36.104 [2] subclause 7.2.1. Note**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power is equal to PREFSENS + 1.4 dB. Note***: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1e assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1f: Blocking performance requirement for Local Area BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 11, 13-14,18,19, 21, 26, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB* | See table 7.6.2a | See table 7.6. 2a |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -35 | PREFSENS +6dB* | See table 7.6.2a | See table 7.6. 2a |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -35 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -35 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -35 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -35 | PREFSENS +6dB* | See table 7.6-2a | See table 7.6-2a |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 31, 72, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -35 | PREFSENS +6dB* | See table 7.6-2a | See table 7.6-2a |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -35 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |  |
| Note*: PREFSENS is specified in TS 36.104 [2] subclause 7.2.1 Note**: Up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1f assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1g: Blocking performance requirement for Local Area BS for E-UTRA with NB-IoT in-band/guard band operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 11, 13-14,18,19, 21, 26, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -35 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -35 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -35 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -35 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -35 | PREFSENS +6dB* | See table 7.6-2b | See table 7.6-2b |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 31, 72, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -35 | PREFSENS +6dB* | See table 7.6-2b | See table 7.6-2b |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -35 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |  |
| Note*: PREFSENS depends on the channel bandwidth or supported subcarrier spacing as specified in TS 36.104 [2] subclause 7.2.1. Note**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power is equal to PREFSENS + 1.4 dB. Note***: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1g assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1h: Blocking performance requirement for Home BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 11, 13-14,18,19, 21, 26, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -27 | PREFSENS +14dB* | See table 7.6.2a | See table 7.6. 2a |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -27 | PREFSENS +14dB* | See table 7.6.2a | See table 7.6. 2a |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -27 | PREFSENS +14dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -27 | PREFSENS +14dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14B* |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -27 | PREFSENS +14dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -27 | PREFSENS +14dB* | See table 7.6-2a | See table 7.6-2a |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 74 | (FUL_low -20) | to | (FUL_high +5) | -27 | PREFSENS +14dB* | See table 7.6-2a | See table 7.6-2a |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -27 | PREFSENS +14dB* | See table 7.6. 2a | See table 7.6. 2a |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |  |
| Note*: PREFSENS is specified in TS 36.104 [2] subclause 7.2.1 Note**: Up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1h assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1i: Blocking performance requirement for Home BS for E-UTRA with NB-IoT in-band/guard band operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 11, 13-14,18,19, 21, 26, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -27 | PREFSENS +14dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -27 | PREFSENS +14dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -27 | PREFSENS +14dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -27 | PREFSENS +14dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -27 | PREFSENS +14dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -27 | PREFSENS +14dB* | See table 7.6-2b | See table 7.6-2b |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 74 | (FUL_low -20) | to | (FUL_high +5) | -27 | PREFSENS +14dB* | See table 7.6-2b | See table 7.6-2b |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -27 | PREFSENS +14dB* | See table 7.6.2b | See table 7.6.2b |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +14dB* |  | CW carrier |  |
| Note*: PREFSENS depends on the channel bandwidth or supported subcarrier spacing as specified in TS 36.104 [2] subclause 7.2.1. Note**: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1i assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1j: Blocking performance requirement for Medium Range BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 11, 13-14,18,19, 21, 26, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB* | See table 7.6.2a | See table 7.6. 2a |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -38 | PREFSENS +6dB* | See table 7.6.2a | See table 7.6. 2a |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -38 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -38 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -38 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -38 | PREFSENS +6dB* | See table 7.6-2a | See table 7.6-2a |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 31, 72, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -38 | PREFSENS +6dB* | See table 7.6-2a | See table 7.6-2a |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -38 | PREFSENS +6dB* | See table 7.6. 2a | See table 7.6. 2a |  |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15** | PREFSENS +6dB* |  | CW carrier |  |
| Note*: PREFSENS is specified in TS 36.104 [2] subclause 7.2.1 Note**: Up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1j assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1k: Blocking performance requirement for Medium Range BS for E-UTRA with NB-IoT in-band/guard band operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 11, 13-14,18,19, 21, 26, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -38 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -38 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -38 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -38 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -38 | PREFSENS +6dB* | See table 7.6-2b | See table 7.6-2b |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 31, 72, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -38 | PREFSENS +6dB* | See table 7.6-2b | See table 7.6-2b |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -38 | PREFSENS +6dB* | See table 7.6.2b | See table 7.6.2b |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15*** | PREFSENS +6dB* |  | CW carrier |
| Note*: PREFSENS depends on the channel bandwidth or supported subcarrier spacing as specified in TS 36.104 [2] subclause 7.2.1. Note**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power is equal to PREFSENS + 1.4 dB. Note***: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1k assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6-2: Interfering signals for blocking performance requirement

| E-UTRA channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±2.1 | 1.4MHz E-UTRA signal |
| 3 | ±4.5 | 3MHz E-UTRA signal |
| 5 | ±7.5 | 5MHz E-UTRA signal |
| 10 | ±7.5 | 5MHz E-UTRA signal |
| 15 | ±7.5 | 5MHz E-UTRA signal |
| 20 | ±7.5 | 5MHz E-UTRA signal (Note 1) |
| 20 | ±30 | 20 MHz E-UTRA signal (Note 2) |
| Note 1: This type of interfering signal is not applied for Band 46. Note 2: This type of interfering signal is only applied for Band 46. |  |  |

Table 7.6.2a: Interfering signals for blocking performance requirement for NB-IoT standalone operation

| NB-IoT channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 0.2 | ±7.5 | 5MHz E-UTRA signal |

Table 7.6-2b: Interfering signals for blocking performance requirement for E-UTRA with NB-IoT in-band/guard band operation

| E-UTRA channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 3 | ±4.5 | 3MHz E-UTRA signal |
| 5 | ±7.5 | 5MHz E-UTRA signal |
| 10 | ±7.5 | 5MHz E-UTRA signal |
| 15 | ±7.5 | 5MHz E-UTRA signal |
| 20 | ±7.5 | 5MHz E-UTRA signal |

NOTE: 	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The relationship between Minimum Requirements and Test Requirements is defined in subclause 4.1 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

## < end of changes >