3GPP TSG-RAN WG4 meeting #92	R4-1909519
Ljubljana, Slovenia, 26th– 30th Aug, 2019

Title:	draft LS reply to RAN2 on UE initial channel bandwidth

Reply to:	R2-1908301

Release:	Rel-15

Work Item:	NR_newRAT

Source:	ZTE Corporation

To:	RAN2

Cc:	RAN1

Contact Persons:

#### Name:               Aijun CAO

#### E-mail Address: cao.aijun@zte.com.cn

Attachments: 	None

1. Overall Description:

RAN4 thanks RAN2 for the LS on UE initial channel bandwidth issue, and RAN4 discusses the issue and reaches the common understandings on the answers to the questions raised in the LS:

Q1: Which BWP-bandwidths is a UE is expected to support: Only the BWP-bandwidths matching exactly the supported channel bandwidths or also values less than the exact channel bandwidth (possibly including any value - in number of PRBs - lower than the supported channel bandwidths)?

A1: Values equals to or less than the exact channel bandwidth.

Q2: Can the network make any assumptions regarding supported initial BWP bandwidths (when UE capabilities are not yet known)?

A2: When UE capabilities are not known to the network, the network may assume that there is at least one channel bandwidth supported by the UE which covers the necessary bandwidth parts to complete the initial process, and the channel bandwidth does not to be the same as the channel bandwidth broadcast in SIB1.

Q3: Does the RAN2 agreement that UEs shall support an initial UL BWP bandwidth equal to CORESET#0 have any impact to RAN1/4 specifications?

A3: No, the RAN2 agreement does not have any impact on RAN4 specifications.

Q4: clarify which channel bandwidth the UE assumes/applies during initial access (e.g. how a UE not supporting the channel bandwidth indicated in SIB1 behaves during the initial access)?

A4: UE may set its initial channel bandwidth as the channel bandwidth broadcast in SIB1 if it supports it, or choose one channel bandwidth from its supported channel bandwidth set which should at least cover the bandwidth part necessary to complete the initial process, e.g., for DL, at least CORESET#0 + initial DL BWP, and for UL, at least initial BWP + PRACH.

Q5: clarify how a UE not supporting the channel bandwidth indicated in SIB1 would behaves if it’s never provided with dedicated channel bandwidth?

A5: As seen in A4, a UE may set its initial channel bandwidth without providing the network its capability on the supported channel bandwidths. If the network does not know the UE’s capability and never configures the UE dedicated channel bandwidth, it assumes that the UE supports the minimum channel bandwidth of the corresponding band which covers the necessary bandwidth parts to complete the initial process.

RAN4 kindly asks RAN2/RAN1 to consider the above understandings in the future work for UE initial channel bandwidth.

2. Actions:

To: RAN WG2/WG1

ACTION:

RAN4 kindly asks RAN2/RAN1 to consider the above understandings in the future work for UE initial channel bandwidth.

3. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #92bis	 14 - 18 Oct 2019                                     Chongqing, China

TSG RAN WG4 Meeting #93	 18 - 22 Nov 2019                                     Reno, USA