3GPP TSG-RAN4 Meeting #92	R4-1909676

# Ljubljana, SI, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6620 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing euCA related interruption requirements for NE-DC in 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2019-06-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As agreed in RAN4#84, euCA related interruption requirements should be added as part of Rel-16 CA/DC enhancement WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce euCA related interruption requirements for NE-DC in 36.133 1) interruption caused by direct SCell activation/hibernation 2) interruption caused by state transitions around dormant state 3) interruption caused by measurement on dormant SCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Objective of the CA/DC enhancement WI is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 7.32.2.7, 7.32.2.8, 7.32.2.9, 7.32.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 7.36.2.5	Interruptions during measurements on SCC

##### 7.36.2.5.1	Interruptions during measurements on deactivated NR SCC

PSCell interruptions due to measurements when an NR SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [50] is 640 ms or longer. The UE is only allowed to cause interruptions immediately before and immediately after an SMTC. Each interruption shall not exceed X1 subframes for synchronous intraband NE-DC, 1 subframe for synchronous interband NE-DC or 2 subframes for asynchronous interband NE-DC: X1 is equal to the duration of the SMTC of the deactivated NR SCell + 1 ms. The interruption is based on assumption that the cell specific reference signals from both cells are available in the same slot.

SCell interruptions due to measurements when an NR SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [50] is 640 ms or longer. The UE is only allowed to cause interruptions immediately before and immediately after an SMTC. Each interruption shall not exceed X1 subframes for synchronous intraband NE-DC, 1 subframe for synchronous interband NE-DC or 2 subframes for asynchronous interband NE-DC: X1 is equal to the duration of the SMTC of the deactivated NR SCell + 1 ms. The interruption is based on assumption that the cell specific reference signals from both cells are available in the same slot.

##### 7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC

PSCell interruptions due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. Each interruption shall not exceed:

-	1 subframes if the PSCell is not in the same band as the deactivated SCell

-	5 subframes if the PSCell is in the same band as the deactivated SCell

SCell interruptions due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. Each interruption shall not exceed:

-	1 subframes if the SCell is not in the same band as the deactivated SCell

-	5 subframes if the SCell is in the same band as the deactivated SCell

##### 7.36.2.5.3	Interruptions during measurements on dormant E-UTRA SCC

PSCell interruptions due to CQI and RRM measurements when an SCell is dormant are allowed as defined in clause 7.8.2.17.

<End of Change 1>

<Start of Change 2>

#### 7.36.2.7	Interruptions at SCell activation/deactivation of dormant SCell

When one dormant SCell belonging to SCG is activated or deactivated, UE is allowed interruptions on PSCell and any activated SCell(s) as defined in clause 7.8.2.14, 7.8.2.15 and 7.8.2.16.

#### 7.32.2.8	Interruptions at SCell hibernation

When one activated or deactivated SCell belonging to SCG is hibernated, UE is allowed interruptions on PSCell and any activated SCell(s) as defined in clause 7.8.2.16a.

#### 7.32.2.9	Interruptions at direct SCell activation/hibernation

When one SCell belonging to SCG is directly activated or hibernated as defined in [2], UE is allowed interruptions on PSCell and any activated SCell(s) as defined in clause 7.8.2.18.

<End of Change 2>