3GPP TSG-RAN WG4 Meeting #92	R4-1909802

Ljubljana, Slovenia, 26 August – 30 August 2019

Source:	Ericsson, T-Mobile US

Title:	TP to TR 37.716.21-21: Addition of CA configuration for DC_66A_n25A-n41A

Agenda item:	10.8.2

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 v0.5.0 add the missing MSD values for DC_66_n25-n41 as defined in WID [1] and also updates the TIB,c and RIB values in accordance with the agreements made during the work on DC_66_n41.

# Text Proposal

##### ---Start of changes---

### 6.26.4	∆TIB and ∆RIB values

For DC_66_n25-n41, the TIB,c and RIB values are derived from LTE CA_2-7-66 and combined with the findings from the work on DC_66_n41. The values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n41 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.x.4-2: ΔRIB

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n41 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 6.26.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 66 and n41 that impact the reference sensitivity of NR band n25. The required MSD is shown in the following table.

Table 6.26.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_66A_n25A-n41A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n25A-n41A | 66 | IMD4 | ¦2*fB66 -2*fn41¦ | 1715 | 5 | 25 | 2115 | N/A |
|  | n41 |  |  | 2685 | 10 | 50 | 2685 |  |
|  | n25 |  |  | 1860 | 5 | 25 | 1940 | 11.0 |

The update needed in 38-101-3 as in following table.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | DL LCRB | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n25A-n41A | 66 | 1715 | 5 | 25 | 2115 | 5 | N/A | FDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | 10 | N/A | TDD |  |
|  | n25 | 1860 | 5 | 25 | 1940 | 5 | 11.0 | FDD | IMD4 |

##### ---End of changes---

# Reference

[1]	RP-190151, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics