3GPP TSG-RAN WG4 Meeting #92	R4-1909890

Ljubljana, Slovenia, Aug. 26-30th

Agenda item:	9.17.1.1

Source: 		NTT DOCOMO, INC.

Title: 	Power imbalance requirement for intra-band EN-DC/NR CA

Document for:	Approval

1. Introduction

In this contribution, we propose to specify the power imbalance requirement for intra-band EN-DC and NR CA.

2. Discussion

In LTE, the power imbalance requirement was introduced from the first release of LTE-Advanced (Rel.10) with 6dB power imbalance of adjacent two component carriers, and the original target scenario is CA scenario #3. Also, the motivation to introduce this requirement is originally to confirm the proper implementation of Rx image rejection at UE side, e.g. [1-2], but this requirement is also interpreted as the verification of proper AGC implementation at UE RF component.

From Rel-15 NR WI, some band combinations for intra-band EN-DC/NR CA were also introduced, e.g. DC_3_n3, DC_41_n41, DC_42_n78, CA_n78C [3]. Note the DC_42_n78 has different band number for NR and LTE side, but the LTE band 42 (3.4-3.6GHz) is completely contained in the NR band n78 (3.3-3.8GHz), so UE RF requirements of intra-band EN-DC are applied for this band combination [3]. For the intra-band EN-DC/NR CA cases, only collocated scenario is considered based on the RRM specification [4], however, reception power of LTE and NR would be different according to deployment scenario. Roughly speaking, the following three deployment scenarios could be considered for intra-band EN-DC:

- Scenario #1: LTE and NR have different antenna/beam pattern, e,g., advanced beamforming technology for NR

- Scenario #2: LTE and NR have same antenna pattern, but the transmission precoding matrix (TPMI) is different.

- Scenario #3: LTE and NR have same antenna pattern and transmission precoding matrix by strict coordination of LTE/NR schedulers at eNB/gNB

Figure 1: Deployment scenario for intra-band EN-DC

The reception power of LTE and NR would be the same only in the scenario #3, and it would be different for other scenarios. The same analysis could be possible for intra-band NR CA. Hence, to ensure the PDSCH demodulation performance in the scenario #1 and #2, we propose the followings:

Proposal 1: Specify the UE demodulation requirement with power imbalance for intra-band EN-DC/NR CA in Rel.16 WI.

- Detailed simulation assumption is FFS

Proposal 2: 6 dB power imbalance is assumed for above requirement as the same level as LTE CA

3. Conclusion

In this contribution, we discussed the necessity of power imbalance requirement for intra-band EN-DC/NR CA. Our proposals are summarized below.

Proposal 1: Specify the UE demodulation requirement with power imbalance for intra-band EN-DC/NR CA in Rel.16 WI.

- Detailed simulation assumption is FFS

Proposal 2: 6 dB power imbalance is assumed for above requirement as the same level as LTE CA

References

- R4-103677, Nokia, Nokia Siemens Networks, Image Rejection in intraband carrier aggregation, RAN4 2010 AH#4

- R4-114882, Huawei, HiSilicon, Demodulation requirements on CA PDSCH with power imbalance, RAN4 #60bis

- TS38.101-3 V15.6.0

- TS38.133 V15.4.0