3GPP TSG-RAN WG4 Meeting 92												  	         R4-1909942

# Ljubljana, Slovenia, 26th – 30th August 2019

Source: 	Skyworks Solutions, Inc.

Title: 		Experimental Measurements for n7 30,40,50 CBW REFSENS

Agenda Item: 			10.15.1 UE RF  [NR_n7_BW-Core]

Document for:	Approval

# 1	Introduction

This document presents Tx noise in Rx band power level measurements and proposes REFSENS levels for n7 operation in 30,40,50 MHz Channel Bandwidth (CBW) based on uplink allocations proposed in [1].

# 2	Experimental Measurements

## 2.1	Power Amplifier Calibration and Test Waveforms

- A Band 7 power amplifier (PA) is calibrated to meet 1 dB MPR using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at channel edge.

- For Tx noise in Rx band power level measurements, the following waveforms are used in which the allocated Resource Blocks (RBs) are located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth.

  - 30 MHz CBW: QPSK DFT-s-OFDM SCS 15 kHz with 64RBs (i.e. 64RB96),

  - 40 MHz CBW: QPSK DFT-s-OFDM SCS 15 kHz with 45RBs (i.e. 64RB171),

  - 50 MHz CBW: QPSK DFT-s-OFDM SCS 15 kHz with 45RBs (i.e. 64RB225).

  - Image and Local Oscillator (LO) rejection is set to -28 dBc

  - Counter-IM3 (C-IM3) set to -60 dBc.

Figure 1 below shows the 50 MHz CBW source waveform as an example.

Figure 1: Test waveform spectrum plot example used to measure n7 Tx noise levels in Rx band for 50 MHz channel bandwidth operation.

## 2.2. Experimental results

We assume 4 dB post-PA loss, so Tx power levels at PA output port are set to 27 dBm for PC3 operation. An example plot of the PA output spectrum at 27 dBm is presented in Figure 2. The impact of C-IM3 can be clearly seen.

Figure 2: Example of spectrum observed at PA output port at 27 dBm for 50MHz CBW operation with C-IM3.

The measured Tx noise in Rx band power levels in each respective bandwidth are:

- 30 MHz: -55.1 dBm

- 40 MHz: -51.9 dBm

- 50 MHz: -37.3 dBm.

# 3 MSD and Reference Sensitivity Estimation

We assume:

- 50 dB Transmit port to Receiver port duplexer rejection in Rx band, as a worst case assumption,

- 40 dB Transmit port to Antenna port duplexer rejection in Rx band, as a worst case assumption,

- 10 dB Primary antenna to diversity antenna isolation,

- 4 dB Post PA loss,

- 4 dB Antenna to LNA front-end losses,

- 3 dB Antenna diversity gain

- -1 dB SNR.

- Previously agreed rules to compute correlated and uncorrelated MRC contributions.

Table 1 below summarizes the results of MSD analysis for 30,40,50 MHz CBW.

Table 1: MSD Analysis for n7 PC3 Operation in 30,40,50 MHz CBW

Observation 1:

- At 30 MHz and 40 MHz CBW, MSD can be neglected,

- At 50 MHz, worst case MSD is 5 dB.

We therefore propose to define n7 REFSENS as follows.

Proposal 1:

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) |  | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) |  | 100 MHz (dBm) |  |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  | -90.7 | -89.9 | -88.6 | -82.7 |  |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  | -90.8 | -90.0 | -88.6 | -82.7 |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  | -91.0 | -90.1 | -88.8 | -82.8 |  |  |  |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity (from 38.101-1)

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n7 | 15 | 25 | 501 | 751 | 751 | 721 | 641 | 451 | 451 |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 361 | 361 | 321 | 201 | 201 |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 181 | 181 | 161 | 101 | 101 |  |  |  |  |  |
| NOTE 1: 1 Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 4	Conclusion

This contribution presents experimental Tx noise in Rx band Power amplifier measurements for PC3 operation in n7 at 30,40,50 MHz channel bandwidth and proposes to adopt the following reference sensitivity levels.

Proposal 1:

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) |  | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) |  | 100 MHz (dBm) |  |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  | -90.7 | -89.9 | -88.6 | -82.7 |  |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  | -90.8 | -90.0 | -88.6 | -82.7 |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  | -91.0 | -90.1 | -88.8 | -82.8 |  |  |  |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity (from 38.101-1)

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n7 | 15 | 25 | 501 | 751 | 751 | 721 | 641 | 451 | 451 |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 361 | 361 | 321 | 201 | 201 |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 181 | 181 | 161 | 101 | 101 |  |  |  |  |  |
| NOTE 1: 1 Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# References

[1]	R4-1905337, n7 new channel BW REFSENS, Nokia.