3GPP TSG-RAN WG4 Meeting #92 	R4-1909984

Ljubljana, Slovenia, 26th August -30th August, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | CRNum | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for PRACH configuration indices of RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-04-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FR1 PRACH configurations 1-3 use prach-ConfigurationIndex of 87 and FR1 PRACH configuration 4 uses prach-ConfigurationIndex of 0. Both these PRACH configuration indices use subframe #9 in every 16th radio frame. On the other hand, RAN4 uses a TDD UL/DL configuration of DSUUD (consisting of pattern 1 and pattern 2 configs) and 3D1S4U2D in 15 kHz and 30 kHz SCS respectively. Hence, the FR1 PRACH configuration indices of current versions of 38.133 fall entirely in DL portions of the cell specific TDD UL/DL configuration. A PRACH config becomes invalid if it falls in DL portions of TDD-UL-DL pattern. Hence, it is essential to select prach-ConfigurationIndex of FR1 so that some PRACH occasions fall in UL portions of TDD-UL-DL pattern. Besides, existing FR1 and FR2 PRACH configurations in RAN4 tests have 160 ms periodicity. This increases latency in all PRACH tests, e.g. handover, beam failure recovery, etc. Reducing FR1 and FR2 configuration latency to 10 ms will reduce the latency of all tests that need RACH transmission. Current 133 spec does not have a specific SSB CFRA configuration to convey BFR. However, the existing SSB CFRA configuration for FR1 and FR2 can be mostly used to convey BFR. The only difference is that the ra-PreambleIndex IE of SSB-CFRA-BFR configurations lies within BFR-SSB-Resource IE, instead of the ssb-ResourceList IE that get used for “regular CFRA” and are shown in existing FR1 and FR2 PRACH configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We change prach-ConfigurationIndex of FR1 PRACH configurations 1-4 from 87 and 0 to 8. Similarly, we change prach-ConfigurationIndex of FR2 PRACH configurations 1-4 from 173 and 144 to 190. We add a new row in FR1 and FR2 PRACH configurations 2 named “BFR-SSB-Resource”. Depending on the reason of CFRA transmission, i.e., if it is a “regular” CFRA or BFR-CFRA, UE selects the IEs within ‘ssb-ResourceList’ or ‘BFR-SSB-Resource’ list. We also correct some typos in these sections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR1 tests will not have any valid PRACH occasion. Any FR1 test that needs PRACH transmission cannot be successfully performed. FR2 PRACH tests will have 160 ms periodicity and lead to longer latency that require PRACH transmission. During SSB based BFR, UE will not find necessary fields to transmit CFRA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | ……….. |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

## A.3.8	PRACH configurations

### A.3.8.1	Introduction

This section provides the typical PRACH configurations used for RRM test cases defined in Annex A. To note that for other parameters not listed in this section, either it can be derived from the set up of each test or it is subjected to RAN5 specifications.

### A.3.8.2	PRACH configurations in FR1

#### A.3.8.2.1	FR1 PRACH configuration 1

FR1 PRACH configuration 1 in this section provides the typical PRACH configuration for SSB-based contention based random access in FR1.

Table A.3.8.2.1-1: Parameters for FR1 PRACH configuration 1

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 87102 | 16010ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.2.2	FR1 PRACH configuration 2

FR1 PRACH configuration 2 in this section provides the typical PRACH configuration for SSB based non-contention based random access in FR1.

Table A.3.8.2.2-1: Parameters for FR1 PRACH configuration 2

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 87102 | 16010ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.2.3	FR1 PRACH configuration 3

FR1 PRACH configuration 3 in this section provides the typical PRACH configuration for CSI-RS based non-contention based random access in FR1.

Table A.3.8.2.3-1: Parameters for FR1 PRACH configuration 3

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 87102 | 16010ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| csirs-ResourceList | ra-PreambleIndex = 50 | Associated with CSI-RS configured |
| ra-OccasionList | 1 | RA occasions allowed corresponding to CSI-RS |
| rsrp-ThresholdCSI-RS | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.2.4	FR1 PRACH configuration 4

FR1 PRACH configuration 4 in this section provides the PRACH configuration for CSI-RS based non-contention based random access in FR1 to convey BFR.

Table A.3.8.2.4-1: Parameters for FR1 PRACH configuration 4

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 08 | 16010ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n200 | Max number of RA preamble transmission performed before declaring a failure is 200 |
| ra-ResponseWindow | sl1 | 1 slot |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 93 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| BFR-CSIRS-Resource | ra-PreambleIndex = 50 | Associated with CSI-RS configured |
| ra-OccasionList | 1 | RA occasions allowed corresponding to CSI-RSh |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

### A.3.8.3	PRACH configurations in FR2

#### A.3.8.3.1	FR2 PRACH configuration 1

FR2 PRACH configuration 1 in this section provides the typical PRACH configuration for SSB-based contention based random access in FR2.

Table A.3.8.3.1-1: Parameters for FR2 PRACH configuration 1

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 173190 | Preamble Format C2, with 16010ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.3.2	FR2 PRACH configuration 2

FR2 PRACH configuration 2 in this section provides the typical PRACH configuration for SSB based non-contention based random access in FR2.

Table A.3.8.3.2-1: Parameters for FR2 PRACH configuration 2

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 173190 | Preamble Format C2, with 16010ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.3.3	FR2 PRACH configuration 3

FR2 PRACH configuration 3 in this section provides the typical PRACH configuration for CSI-RS based non-contention based random access in FR2.

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 173190 | Preamble Format C2, with 16010ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| csirs-ResourceList | ra-PreambleIndex = 50 | Associated with CSI-RS configured |
| ra-OccasionList | 1 | RA occasions allowed corresponding to CSI-RS |
| rsrp-ThresholdCSI-RS | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

#### A.3.8.3.4	FR2 PRACH configuration 4

FR2 PRACH configuration 4 in this section provides the PRACH configuration for CSI-RS based non-contention based random access in FR2 to convey BFR.

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 144190 | Preamble Format C2, with 16010ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n200 | Max number of RA preamble transmission performed before declaring a failure is 200. |
| ra-ResponseWindow | sl40 | 40 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| BFR-CSIRS-Resource | ra-PreambleIndex = 50 | Associated with CSI-RS configured |
| ra-OccasionList | 1 | RA occasions allowed corresponding to CSI-RS |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

<< End of changes>>