3GPP TSG-RAN WG4 Meeting # 92	R4-1910199

Ljubljana, Slovenia, 26th – 30th August, 2019

Source:	Huawei, HiSilicon

Title:	TP to TR 38.716.02-00 for CA_n7-n66

Agenda item:	10.2.2

Document for:	Approval

Background

This contribution provides text proposal on the NR CA band combination CA_n7A-n66A as defined in Revised WID on NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) RP-191565 [1].

Text Proposal

---Start of changes---

6.x	CA_n7-n66

6.x.1	Common for 1 band UL and 2 bands UL CA

6.x.1.1 Operating bands for CA

Table 6.x.1-1: CA band combination of band n7 + n66

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n7-n66 | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.x.1.2	Channel bandwidths per operating band for CA

Table 8.x.2-1: Supported bandwidths per CA band combination of band n7+n66

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n7A-n66A | CA_n7A-n66A | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |

6.x.1.3	Co-existence studies

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n7-n66.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |

Based on above table, there is no harmonics issue for the band combination of n7 and n66.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 |  |  |

Based on above table, there is no harmonics mixing issue for the band combination of n7 and n66.

6.x.1.4	∆TIB and ∆RIB values

For CA_n7A-n66A, the TIB,c and RIB values are given in the tables below.

Table 6.x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |

Table 6.x.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |

6.x.1.5	REFSENs requirements

There are no specific REFSENS requirements for 1 band UL

6.x.2		Specific for 2 bands UL CA

6.x.2.1		UE co-existence studies

Table 6.x.2.1-1 gives IMD interference analysis for CA_ n7-n66 with 2 ULs.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 720 | 860 | 4210 | 4350 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 850 | 1060 | 3220 | 3430 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5920 | 6130 | 6710 | 6920 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 2560 | 2840 | 5720 | 6000 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 7630 | 7910 | 9210 | 9420 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1440 | 1720 | 8420 | 8700 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 4270 | 4620 | 8220 | 8570 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9340 | 9690 | 11710 | 12060 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 10 | 340 | 3940 | 4290 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 10920 | 11270 | 10130 | 10480 |

Based on above table, two-tone 4th order IMD products may fall into the own Rx Band of Band 7.

Table 6.x.2.1-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.x.2.1-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7-n66 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 66, 71, , 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |

6.x.2.2		REFSENS requirements

The IMD issue for CA_n7-n66 is similar as LTE CA_4A-7A. Hence the MSD value for CA_4A-7A can be re-used for CA_n7-n66.

Table 6.x.2.2-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7A-n66A | n7 | 2535 | 10 | 50 | 2655 | 15 | FDD | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |

---End of changes---

Reference

[1]	RP-191565, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation