3GPP TSG-RAN WG4 Meeting #92				  												R4-1910200

Ljubljana, Slovenia, 26th Aug. 2019 - 30th Aug. 2019

Source: 	Nokia, Nokia Shanghai Bell, T-Mobile USA

Title: 						TP to TR 38.716-02-00: CA_n41-n66

Agenda Item: 			10.2.2 [NR_CADC_R16_2BDL_xBUL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 38.716-02-00 [1] to introduce the combinations listed in the table below following the WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) [2].

| CA configuration | Uplink Configuration | BCS | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | CA_n41A | 0 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | None |
| CA_n41A-n66A | CA_n66A | 0 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | None |

# 2	Text Proposal

*********************************** Start of the TP ***********************************

## 6.x	CA_n41-n66

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n41+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2469 MHz | – | 2690 MHz | 2469 MHz | – | 2690 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n41+n66

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum Aggregated bandwidth [MHz] | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | - | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 140 | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |

#### 6.x.1.3		UE co-existence studies

Table 6.18.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA _ n41-n66.

# Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |

# Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 |  |  |

Based on above tables, there is no harmonic issue.

#### 6. x.1.4		∆TIB and ∆RIB values

For CA_n41-n66, the TIB,c and RIB,c values for low high combination are reused from DC_66_n41 as given in the tables below.

Table 6. x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n66 | n41 | 0.5 |
|  | n66 | 0.5 |

Table 6. x.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n66 | n41 | 0.5 |
|  | n66 | 0.5 |

#### 6. x.1.5		REFSENS requirements

A reference sensitivity exception (MSD) already exists in 38.101-3 due to cross band isolation for EN-DC of band n41 and 66 in NR FR1. The same should apply for CA_n41-n66 as given in Table 6.x.1.5-1 with uplink configuration specified in Table 6.x.1.5-2.

# Table 6.x.1.5-1: MSD for the CA configuration for asynchronous operation and cross band isolation for CA

| NR CA Configuration CA_n41-n66 | NR UL band | NR DL band | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
|  | n411 | n66 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied.. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Table 6.x.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for CA

| Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR UL band | NR DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n66 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |

*********************************** End of the TP ***********************************

# 3	References

[1] 	R4-1905855 TR 38.716-02-00 v0.5.0, ZTE.

[2] 	RP-191350 Revised WID:  NR inter-band CA/DC for 2 bands DL with up to 2 bands UL, ZTE.