3GPP TSG-RAN WG4#92 Meeting	R4-1910357

Ljubljana, Slovenia, 26 – 30 August 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editor CR to TS38.133 after RAN4#92 (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2019-09-06 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Section 1-3: - Endorsed CRs: R4-1908032, R4-1908898, R4-1910029; - Editorial change (R4-1910029) - Section 3.5.3 - Change 1: Change NR_TDD_FR2_G --> NR_TDD_FR2_L for FR2 PC2 operating bands. - Section 3.6.2.4 - Change 1: Remove support of SUL carriers for NR-DC. - Section 4: - Endorsed CR: R4-1910035, R4-1910181 - Editorial CR R4-1910035, - Updating ranking margin when rangetobestcell is configured in idle mode R4-1910181 - Section 5: - Endorsed CRs: R4-1908904 - Change 1: Editorial changes. Section 5.2 change to Void - Section 6.1: - Endorsed CR: R4-1908240, R4-1909323 - Editorial update is needed on handover core requirement. - For FR2-FR2 and FR1-FR2 handover, if the target cell is an unknown cell, UE performs cell detection firstly however the cell detection (without SSB index) can only abtain the symbol level synchronization. In order to acquire the fine timing or the full timing information (including SSB index), one more SMTC is needed. - Dual SMTC configuration is applicable for intra-frequency measurement, then the related description in FR1-FR2 and FR2-FR1 handover shall be removed and in FR1-FR1 handover shall be added. - Section 6.2: - Implemented CRs endorsed in RAN4#92: R4-1908086 - Change-1:. Change “receiving a NACK on msg3” by “UL grant for msg3 retransmission”, for more precise descriptioin; Square bracket clean-up; “RRC connected mode” should be replaced by “RRC_CONNECTED state”; “RRCConnectionRelease” is not correct, while “RRCRelease” is the correct name for the message, and other editorial changes, based on R4-1908086. - Section 7.1 and 7.3: - Endorsed CR: R4-1910038 - To correct the wording to align with the following clauses. - To correct the unit in Table 7.1.2-1, 7.1.2.1-1 and 7.1.2.2-1. - To modify the note in Table 7.1.2-2. - To correct the unit in Table 7.3.2.2-1 - Section 7.5 and 7.6: - Endorsed CRs : R4-1910037, R4-1908180, R4-1910554 - Section 7.5.1 - Change 1 : Editorial change - add NR-DC operation - Section 7.5.4 - Change 1 : Editorial change - Change 2 : Clarify MTTD requirements for inter-band cross FR1-FR2 CA apply to the pair of carriers or TAGs - Section 7.5.5 - Change 1 : Editorial change - Section 7.5.6 - Change 1 : Editorial change - Section 7.6.4 - Change 1 : Clarify MRTD requirements for inter-band cross FR1-FR2 CA apply to the pair of carriers or TAGs - Change 2 : A note is added in Table 7.6.4-1 clarifying that there will be performance degradation when actual receive time difference is larger than the cyclic prefix of the respective CC. - Section 7.6.5 - Change 1 : Editorial change - Section 7.6.6 - Change 1 : Editorial change - Section 7.7: - Endorsed CR: R4-1909732; - Change 1: editorial change. - Section 8.1: - Endorsed CRs: R4-1907977, R4-1908901, R4-1909079, R4-1909334, R4-1910039, - Section 8.1: - Change 1: R4-1910039, Editorial changes. - Section 8.1.2.1 & section 8.1.3.1: - Change 1: R4-1907977, Correct unit of Bandwidth from MHz to PRBs in RLM transmission parameters - Section 8.1.1: - Change 1: R4-1908901, Remove the description which is overlapping part with the agreed CR of measurement restrictions in the last meeting in Reno. - Section 8.1.2.2: - Change 1: R4-1909079, The value of Psharing factor in the delay requirements of RLM is updated. - Section 8.1.7.3: - Change 1: R4-1909334, Adding the scheduling availability due to SSB as RLM –RS and CORESET for RMSI scheduling multiplexing patterns 2/3. - Section 8.2: - Endorsed CRs: R4-1909344, R4-1908183, R4-1909337, R4-1909338, R4-1909339, R4-1909641 - R4-1909344: Editorial CR - R4-1908183:Clarify the current requirement applies only to the case that DCI/timer-based BWP switch happens in only one single CC. - R4-1909337: When one E-UTRA SCell is added /released or activation/deactivation, the victim intra-band NR cells shall be on FR1. Thus the intra-band interruption on NR serving cell shall exclude the cell with 120kHz SCS. - R4-1909338: Adding the following requirements for NR-DC, - -transitions between active and non-active during DRX, or - -transitions from non-DRX to DRX. - R4-1909339: The title shall changed to Interruptions during measurements on deactivated SCC in which both intra-band and inter-band cases are included. - R4-1909641: Update the the interruption requirements for inter-frequency SFTD measurements - Section 8.3: - Implement endorsed draft CRs: R4-1910040, R4-1909604 - Change 1: Square brackets removed for SCell activation delay requirements for FR1 and FR2; - Change 2: TSMTC_SCell is replaced with Trs whereever applicable, TSMTC_SCell is replaced with Trs whereever applicable; - Change 3: SCell activation delay requirements for known target SCell when there is no serving cell on FR2 band is specified, SCell activation delay requirements for unknown SCell, when there is no active serving cell on FR2 band is specified; - Change 4: Add UE behaviour when a L1-RSRP report provided during SCell activation contains random values and valid values, respectively. - Section 8.4: - Make some editorial corrections (R4-1909345) - Section 8.5: - Endorsed CRs: R4-1908682, R4-1908828, R4-1908902, R4-1908906, R4-1909335, R4-1909336, R4-1909615 - Section 8.5: Change 1: R4-1908907, Editorial changes. - Section 8.5.2.1:Change 1: R4-1908902, Update Bandwidth value in PDCCH transmission parameters for SSB based BFD - Section 8.5.2.2, 8.5.5.2:Change 1: R4-1908682, The value of Psharing factor in the delay requirements of BFD, CBD and L1-RSRP is updated. - Section 8.5.3.2:Change 1: R4-1908828, Add an exception for CSI-RS based BFD in FR2. - Section 8.5.6.2:Change 1: R4-1908902, Update the reference to measurement restriction for CSI-RS based CBD in CSI-RS based CBD requirement - Section 8.5.7.3:Change 1: R4-1909335, Adding the measurement restriction for BFD due to SSB and CORESET for RMSI scheduling multiplexing patterns 2/3. - Section 8.5.8.2:Change 1: R4-1909615, To clairfiy the scheduling res restrictions on TRS and SRS for CBD measurements. - Section 8.5.8.3:Change 1: R4-1909336, Adding the scheduling availability for CBD due to SSB and CORESET for RMSI scheduling multiplexing patterns 2/3. - Change 1: R4-1909615, To clairfiy the scheduling res restrictions on TRS and SRS for CBD measurements. - Section 8.6: - Endorsed CR: R4-1910564 - Change 1: (R4-1910564) Clarify that if UE does not have the information on the indicated TCI-state to decode PDCCH and activated TCI-state to decode PDSCH in the new BWP, UE re-uses those TCI-states in old BWP till new MAC CE is received to update the TCI-states for PDCCH and PDSCH - Change 2: (R4-1910564) Clarify the current requirement applies only to the case that DCI/timer-based BWP switch happens in only one single CC - Change 3: (R4-1910564) Clarify RRC-based BWP swtich covers 2 cases. One is to change the active BWP, and the other is to change the parameter in the active BWP - Section 8.9 - Endorsed CR R4-190177, - Removing information related to PSCell in FR1. Clarifying that PCell is in FR1 and PSCell in FR2. - Correcting reference to applicable delay uncertainty for random access towards PSCell in FR2. Corrected table number and technical specification number. - Modified the text to make it more concise with respect to Trs and applicability of the requirement. - This CR captures missing parts of PSCell addition delay requirements as endorsed in Reno meeting in R4-1907767 - Section 8.10: - Change 1: R4-1910042, captures missing parts of the TCI state switching delay requirements in section 8.10. - Update definition of known TCI state. - Update TL1-RSRP with TReport = 0. - Update requirement for L1-RSRP measurement time with different QCL type. - Update the wording - Section 9.1: - Endorsed CR: R4-1908867, R4-1909320, R4-1909614, R4-1909679, R4-1909733, R4-1910031, R4-1910115, R4-1910116, R4-1910180, R4-1910560 - Editorial CR (R4-1909733) - Section 9.1.2 - Change 1 (R4-1908867): Modify the wording to consider random access procedure - Change 2 (R4-1909320): Add a note that It is left to UE implementation to determine which measurement gap sharing scheme specified in the table to be applied, when MeasGapSharingScheme is absent and there is no stored value in the field. - Change 3 (R4-1909679): Correct the titles of Table 9.1.2-4 and Table 9.1.2-4a and related text descriptions, such that for NE-DC the interrupted slots due to measurement gaps are defined in Table 9.1.2-4. - Change 4 (R4-1910031): Delete notes that specified the capability signalling of measurement gap patterns - Section 9.1.4 - Change 1 (R4-1910560): To clarify the requirements for reporting criteria so as to be aligned with applicability of requirements in section 3.6 - Section 9.1.5 - Change 1 (R4-1909614): To correct the titles for section 9.1.5.1.2 and 9.1.5.1.3. To introduce section 9.1.5.1.4 for NE-DC mode. - Change 2 (R4-1910115): Explicitly define pool A and pool B. Define the conditions for membership of pool A and pool B, and define CSSFwithin_gap,I according to the competing measurement objects in pool A and pool B. Delete the editor notes: FFS if better description exists for the case when the number of configured interfrequency and interRAT measurement objects is zero and the UE is configured with per-UE gap - Change 3 (R4-1910116): Specify that inter-frequency SFTD measurement object is a candidate to be measured in all measurement gaps - Change 4 (R4-1910180): Correct the reference of section numbers to refer to the correct sections for SFTD requriements. Also, for inter-frequency SFTD, only the interruption based measurement will impact the application of CSSF based requriements. - Section 9.2: - R4-1908178: - Restructure the paragraph on SSB-toMeasure by specifically indicating the scnearios with K_layer1_measurement = 1 and leave all the other cases to K_layer1_measurement = 1.5. - R4-1908360 : - Add the condition “with the same Spatial Rx parameter” to the definition of detectable cell - R4-1909333: - Replace processing time of X slots with [2] slots between the paging reception and the PDCCH decoding for the updated SI is need for UE. - R4-1909547: - Correct errors and improve wording where it is unclear. - Section 9.3: - Endorsed CRs: R4-1908360, R4-1908907, R4-1910558 - Section 9.3 - Change 1: R4-1908907, Editorial changes. - Section 9.3.6.3 - Change 1: R4-1908360, Add the condition “with the same Spatial Rx parameter” to the definition of detectable cell - Section 9.3.8 - Change 1: R4-1910558, Add conditions for when interruption based SFTD measurement requirements apply. - Section 9.4: - R4-1909453, Editorial updates (section 9.4), Ericsson - R4-1910113, E-UTRAN time acquisition requirements for E-UTRA FDD RSTD measurements (section 9.4), Ericsson - R4-1910114, CR for inter-RAT RSTD measurement on TDD E-UTRA carrier (section 9.4.4.2), Huawei, HiSilicon - Section 9.5: - Endorsed CR: R4-1908682, R4-1908830, R4-1909333, R4-1909657 - Section 9.5.4 - Update P sharing factor. (R4-1908682) - Remove Editor notes with FFS on measurement restriction. (R4-1909657) - Editorial changes. (R4-1908830) - Section 9.5.5 - Editorial changes. (R4-1908830) - Section 9.5.6 - X=[2] slots, the processint time between the paging reception and the PDCCH decoding for the updated SI is need for UE. (R4-1909333) - Editorial changes. (R4-1908830) - Section 9.6: - Editorial CR (R4-1909733) - Section 10.1: - Implemetned CRs: R4-1909683, R4-1909684, R4-1909530 - - 10 and 10.1.1: remove editor notes already resolved - - 10.1.2.1: correct terminology of SS-RSRP - - 10.1.2.2: void CSI-RS measurement accuracy requriements in Rel-15 - - 10.1.3.1: correct terminology of SS-RSRP, add missing condition and note - - 10.1.3.2: void CSI-RS measurement accuracy requriements in Rel-15 - - 10.1.4.2: void CSI-RS measurement accuracy requriements in Rel-15 - - 10.1.5.1: add missing note - - 10.1.5.2: void CSI-RS measurement accuracy requriements in Rel-15 - - 10.1.6: correct L1-RSRP report range in the texts, clarify the relative report mapping is for L1-RSRP - - 10.1.9.1: add missing min Io condition for FR1 - - 10.1.19 add missing condition and correct note - - 10.1.20 add missing condition - - 10.1.21.2: remove square bracket - - 10.1.21.3: add new section for inter-frequency SFTD accuracy requriement - Section A.3: - R4-1907980 Setup for Test cases with NR cells in both FR1 and FR2, Add a new clause A.3.7A for NR FR1-FR2 test setup, which states that the Test System shall provide a stable and noise-free NR FR1 signal without need of precise propagation modelling, path loss and polarization control. - R4-1909044 Configuration of msg1-SubcarrierSpacing is removed from FR1 PRACH configuration 4. - R4-1909160 Section editor CR for editorical changes - R4-1909278 Add DRX configuration 11 with DRX cycle=20 ms - R4-1909619 To add the modified parameters for beam failure detection and link recovery testing with 4 RX antenna connection - R4-1909658 Update the RS in TCI.State.2 and TCI.State.3 - R4-1910048 Schedule CSI-RS for CFRA and RLM tests every 5ms in the next slot of SSB transmisson - R4-1910161 Two new SMTC patterns are used with SMTC offset = 10 ms with SMTC duration of 1 ms and 5 ms while SMTC periodicity = 20 ms in both patterns - R4-1909984 We change prach-ConfigurationIndex of FR1 PRACH configurations 1-4 from 87 and 0 to 8. Similarly, we change prach-ConfigurationIndex of FR2 PRACH configurations 1-4 from 173 and 144 to 190. We add a new row in FR1 and FR2 PRACH configurations 2 named “BFR-SSB-Resource”. Depending on the reason of CFRA transmission, i.e., if it is a “regular” CFRA or BFR-CFRA, UE selects the IEs within ‘ssb-ResourceList’ or ‘BFR-SSB-Resource’ list. We also correct some typos in these sections. - Section A.4: - R4-1908908 Editorial correction - R4-1909040 Corrections are made on the active BWP switch test cases, Correct delay requirements and Correct Io values - R4-1909343 Remove brackets and Remove the empty section - R4-1909562 Io, nominal in Ch BW for NR Cells 2 and 3 was corrected. AWGN and propagation condition are specified per freq for inter-freq test cases. Removed square brackets from measurement reporting delay values and Added TRS configuration. Editorial corrections - R4-1909741 Editorial corrections. Added TRS, SSB, SMTC configuration. Added SRS configuration for 15 kHz config with same periodicity as DRx cycle - R4-1909745 Editorial corrections; Codebook based SRS transmission as non-codebook is optional. Updated value of k based on 38.213 - R4-1909749 Editorial corrections; Derived power params (SS-RSRP, Es/Iot) corrected in intra-freq test - R4-1909761 Editorial corrections; Added TRS configuration; Event reporting should be B1 instead of A4 since it is inter-RAT; PRACH configuration is changed periodicity is 10ms which changes requirements - R4-1909763 Editorial corrections; SSB based test should have timeRestrictionForChannelMeasurements configured; Updated requirements for SSB based L1-RSRP based on cell detection time of 600ms. Making CSI-RS L1-RSRP test case with periodic reports; Power levels in CSI-RS based test to be in alingment with SSB based test; Removing TBD’s - R4-1909765 Editorial corrections; Requirement statement in SSB based RSRP corrected - R4-1909769 Editorial corrections; Adding that UE needs to be provided with timing advance commanda to restart time alignment coounter in DRx test cases - R4-1909773 Editorial corrections; TRS configuration is missing for FDD and 30 kHz TDD; TCI state is extraneous - R4-1910028 Remove square brackets in corresponding tests; Remove redundant statement in A.7.6.2.4 - R4-1910121 Update Noc level and remove Test 2 for 30kHz SCS. - R4-1910132 Editorial corrections; Update notation of DRX configuration in Table A.4.5.1.3.1-2 and Table A.4.5.1.4.1-2. Update values of T1~T5 and D1. Add one row for Noc in unit of dBm/SCS. Remove brackets in Table A.4.5.1.1.1-2, Table A.4.5.1.1.2-2, Table A.4.5.1.1.3-2, and Table A.4.5.1.1.4-2. Correct the CSI-RS configurations for CSI reporting. Complete the test requirements - R4-1910143 Editorial corrections; Updated BWP configuration; SSB ARFCN for cell3 should be freq2 - R4-1910160 Further clarify in test purpose and environment that UE only need to pass tests with one single measurement gap pattern, if multiple gap patterns are configured to verify the same requirement. Change gapoffset in gap pattern #4 from 19 to 10 - R4-1910163 Square brackets are removed in FR1 tests; Some TBDs for TRS and TCI state configuration are updated to correct table references; Spelling errors are corrected; Some superflous T prefixes are removed; Add missing antenna configurations; Add missing propagation conditions; Add missing TRS configurations; Add missing gap pattern IDs - R4-1910175 “RRC CONNECTION REQUEST” is replaced by “RRCSetupRequest”. Square brackets clean-up. RRCConnectionRelease” is not correct, while “RRCRelease” is the correct name for the message. Remove unnecessary notes in test setup tables. Other editorial changes. - R4-1910538 Update the values of time duration in test cases; Add BWP configuration, correct some parameters; Correction typos, remove brackets. - R4-1910539 Modify SSB configuration to include two SSBs. Modify CSI-RS configuration for CSI reporting to be periodic. Add RLM-RS configuration and remove the editor’s notes. Modify the values of D1 for non-DRX test and the values of T1/T2/T3/T4/T5/D1 for DRX test. Update the cell specific test parameters to add the SNR level for RLM-RS. Remove the measurement gap configuration for DRX test in order to reduce the total test time. - R4-1910541 Clairfiy CSI-RS configuration for q0 and q1. Add CSI-RS configuration for CSI reporting. Add RLM-RS configuration and remove the editor’s notes. Modify the values of D1 for non-DRX test and the values of T1/T2/T3/T4/T5/D1 for DRX test. Update the cell specific test parameters to add the SNR level for RLM-RS. Remove the measurement gap configuration - Section A.5: - List of implemented CRs: ,R4-1908409,R4-1908908,R4-1909141,R4-1909343,R4-1909742,R4-1909746,R4-1909764,R4-1909766,R4-1909770,R4-1910028,R4-1910131,R4-1910134,R4-1910137,R4-1910141,R4-1910144,R4-1910154,R4-1910157,R4-1910160,R4-1910163,R4-1910164,R4-1910166,R4-1910168,R4-1910169,R4-1910171,R4-1910172,R4-1910173,R4-1910175,R4-1910540,R4-1910545 - Update test configurations for interruption test cases - Removal of brackets on the RLM scheduling restriction test case - Remove the empty section A.5.5.4 - Added TRS configuration and OTA params table and changed SRS periodicity for UE transmit timing test case - Added table for OTA parameters, Codebook based SRS transmission for timing advance test cases - Corrected test configurations for L1-RSRP test cases - Corrected test confgurrations for L1-RSRP performance test cases - Corrected test configurations for intra-frequency measurement test cases - Editorial update on test cases for measurement procedure - Corrections on FR2 random access test cases - Corrected the SSB-based RLM test cases - Corrected the CSI-RS RLM tests for FR2 EN-DC - Add new test cases for scheduling restriction due to SSB based Beam Failure Detection and Link recovery procedures in FR2 - Corrections are made on the active BWP switch test cases - Update on inter-frequency measurement test cases - Correction of editorial errors in 38.133 measurement accuracy tests - Specify OTA parameters in intrafrequency RSRP accuracy tests - Update Intra-frequency RSRQ test cases - Update SS-SINR measurement accuracy test cases - Update test configurations for PSCell addition test cases - Adding test case for MAC-CE based TCI state switch - Adding test case for RRC based TCI state switch - Update SSB based BFD and link recovery tests for EN-DC in FR2 - Update the CSI-RS based BFD and link recovery tests for EN-DC in FR2 - Editorial corrections - Section A.6: - R4-1908235 (Section A.6.3.1) - Add TRS configuration to HO cases - R4-1908908 (Section A.6.5.2) - Editorial changes to interruption test cases - R4-1909042 (Section A.6.5.6) - Corrections are made on the active BWP switch test cases, - Correct delay requirements - Correct Io values - Editorial changes - R4-1909048 (Section A.6.3.1) - Parameters related to inter-RAT E-UTRAN measurement are removed from TC A.6.3.1.5. - Time instant T3 is changed to T2 in test requirements since there is no longer T3. - R4-1909052 (Section A.6.3.2) - Io values in TC 6.3.2.3.1 and 6.3.2.3.2 are corrected. - Brackets in test requirements of TC 6.3.2.3.1 and 6.3.2.3.2 are removed. - R4-1909343 (Section A.6.5.4) - Removal of brackets - R4-1909563 (Section A.6.6.2) - Corrected time period values in general test parameters tables. - Re-introduced previously deleted clauses (as void). - Editorial corrections. - R4-1909743 (Section A.6.4.1) - Editorial corrections - Added TRS,SMTC configuration - Added SRS configuration for 15 kHz config with same periodicity as DRx cycle - R4-1909747 (Section A.6.4.3) - Editorial corrections - Codebook based SRS transmission as non-codebook is optional. - Updated value of k based on 38.213 - R4-1909751 (Section A.6.7.1) - Editorial corrections - Some derived power params (SS-RSRP, Es/Iot) were inaccurate in intra-freq test - R4-1909767 (Section A.6.7.4) - Editorial corrections - Requirement statement in SSB based RSRP was incorrect - R4-1909771 (Section A.6.6.1) - Editorial corrections - Adding that UE needs to be provided with timing advance commanda to restart time alignment coounter in DRx test cases - R4-1910028 (no changes for A.6) - R4-1910122 (Section A.6.7.2) - Updated Noc level for Test2 for 30 kHz SCS - R4-1910133 (Section A.6.5.1) - Update notation of DRX configuration in Table A.6.5.1.3.1-2 and Table A.6.5.1.4.1-2. - Update values of T1~T5 and D1. - Add one row for Noc in unit of dBm/SCS. - Remove brackets and correct typos in Table A.6.5.1.1.1-2, Table A.6.5.1.1.2-2, Table A.6.5.1.1.3-2, and Table A.6.5.1.1.4-2. - Correct the CSI-RS configurations for CSI reporting. - R4-1910136 (Section A.6.5.1) - Update the values of time duration in test cases - Add BWP configuration, update some parameters - Correction typos, remove brackets. - R4-1910145 - Changes corresponding to A.6.5.3 were already there ins v15.6.0 - R4-1910162 - Further clarify in test purpose and environment that UE only need to pass tests with one single measurement gap pattern, if multiple gap patterns are configured to verify the same requirement. - Change gapoffset in gap pattern #4 from 19 to 10 - R4-1910163 - Square brackets are removed in FR1 tests - Some TBDs for TRS and TCI state configuration are updated to correct table references - Spelling errors are corrected - Some superflous T prefixes are removed - Add missing antenna configurations - Add missing propagation conditions - Add missing TRS configurations - Add missing gap pattern IDs - R4-1910175 - The following editorial changes are provided in this CR: - SSB configuration is added for handover test cases, for A.6.3.1.1, A.6.3.1.2, A.6.3.1.3, A.7.3.1.1, A.7.3.1.2 and A.7.3.1.3. - SMTC configuration is corrected for handover test cases, for A.6.3.1.1, A.6.3.1.2, A.6.3.1.3, A.7.3.1.1, A.7.3.1.2 and A.7.3.1.3. - “RRC CONNECTION REQUEST” is replaced by “RRCSetupRequest”. - Square brackets clean-up. - “RRCConnectionRelease” is not correct, while “RRCRelease” is the correct name for the message. - Remove unnecessary notes in test setup tables. - Other editorial changes. - R4-1910546 - Modify SSB configuration to include two SSBs. - Modify CSI-RS configuration for CSI reporting to be periodic. - Add RLM-RS configuration and remove the editor’s notes. - Modify the values of D1 for non-DRX test and the values of T1/T2/T3/T4/T5/D1 for DRX test. - Update the cell specific test parameters to add the SNR level for RLM-RS. - Remove the measurement gap configuration for DRX test in order to reduce the total test time. - Correct some typos. - R4-1910548 - Clairfiy CSI-RS configuration for q0 and q1. - Add CSI-RS configuration for CSI reporting. - Add RLM-RS configuration and remove the editor’s notes. - Modify the values of D1 for non-DRX test and the values of T1/T2/T3/T4/T5/D1 for DRX test. - Update the cell specific test parameters to add the SNR level for RLM-RS. - Remove the measurement gap configuration. - Section A.7: - Implemented CRs endorsed in RAN4#92: R4-1908326, R4-1908908, R4-1909142, R4-1909343, R4-1909744, R4-1909748, R4-1909768, R4-1909772, R4-1910028, R4-1910130, R4-1910131, R4-1910135, R4-1910138, R4-1910142, R4-1910145, R4-1910154, R4-1910155, R4-1910159, R4-1910162, R4-1910163, R4-1910165, R4-1910167, R4-1910168, R4-1910170, R4-1910175, R4-1910547 and R4-1910549. - Change-1: Delete redundant “ NR FR2” from the title of TC A.7.5.6.2.1, based on R4-1908326. - Change-2: Editorial correction for TC A.7.5.2.1, based on R4-1908908. - Change-3: Editorial correction for TC A.7.5.1.9, based on R4-1909142. - Change-4: Removal of the empty section of A.7.5.4, based on R4-1909343. - Change-5: Adding OTA parameter table, changing codebook based SRS transmission as non-codebook to be optional feature, updated value of k based TS38.213 and other editorial corrections for TC A.7.4.3.1, based on R4-1909744. - Change-6: Adding OTA parameter table, SMTC configuration, and changed SRS periodicity to align with DRx, and other editorial corrections for TC A.7.4.1.1, based on R4-1909748. - Change-7: Removal of TBDs in OTA parameters, corrected requirement statement in SSB based RSRP, and other editorial corrections for TC A.7.7.4.1 and A.7.7.4.2, based on R4-1909768. - Change-8: Adding that UE needs to be provided with timing advance commanda to restart time alignment coounter in DRx test cases, and other editorial corrections for TCs in A.7.6.1, based on R4-1909772. - Change-9: Removal of redundant statement and square brackets for TCs in A.7.6.1 and A.7.6.2, based on R4-1910028. - Change-10: Correction of values for Noc, Es/Noc and etc for FR1 cell, added TRS and SSB configuraiton, and updated handover delay according to core requirement and PRACH configuration for TC A.7.3.1.1, based on R4-1910130. - Change-11: For random access test cases, using 50MHz bandwidth instead of 100MHz due to the limitation of testability for achievable baseband SNR, providing target PRACH power values, removal of ss-PBCH-BlockPower due to its dependence on OTA chamber implementation, and specifying FR2 test cases with no artificial noise transmitted for TC A.7.3.2.2.1 and A.7.3.2.2.2, based on R4-1910131. - Change-12: Corrections of the CSI-RS configurations for CSI reporting, TCI states for PDCCH/PDSCH, SNR configurations, the configuration of time periods (T1, T2, etc.), T310 duration, the DRX configurations and other editoiral changes for TC A.7.5.1.1-4, based on R4-1910135. - Change-13: Corrections for CSI-RS based RLM TCs on TRS resurce set, BWP configuration, 640ms DRX cycle, longer T2 and D1, and two CORESETs configured, and other corrections, based on R4-1910138. - Change-14: Adding BWP configuration, OTA parameters and other editorial corrections for TC A.7.5.2.1, based on R4-1910142. - Change-15: Updating the parameters for SCell activation and deactivation for TC A.7.5.3.1 and A.7.5.3.2, based on R4-1910145. - Change-16: Adding new test cases for scheduling restriction due to SSB based Beam Failure Detection and Link recovery procedures in FR2 as TC A.7.5.5.5, based on R4-1910154. - Change-17: Updating the parameters in the test cases for BWP switch test cases for TC A.7.5.6.1.1-3, based on R4-1910155. - Change-18: Corrections on delay requirements, Io values and other editorial corrections for Active BWP switch test cases in A.7.5.6, based on R4-1910159. - Change-19: Clarification that UE only need to pass tests with one single measurement gap pattern, and change gapoffset configuration for TC A.7.6.2.1-8, based on R4-1910162. - Change-20: Editorial corrections, adding missing antenna configuration, propagation condition, TRS configuration and gap pattern configurations, for measurement accuracy tests for TCs in A.7.7, based on R4-1910163. - Change-21: Specify OTA parameters for TC A.7.7.1.1, based on R4-1910165. - Change-22: Updated band group, removal of Test 2, chaning to no artificial noise for Test 3, and corrections of OTA parameters for TC A.7.7.2.1, based on R4-1910167. - Change-23: Corrections on OTA parameters for TC A.7.7.2.2, based on R4-1910168. - Change-24: Adding noise and signal power levels, deleting one sub-test, and modifying AoA configuration, for SINR measurement accuracy test, TC A.7.7.3.1 and A.7.7.3.2, based on R4-1910170. - Change-25: Correction on SMTC and SSB configuration for TC A.7.3.1.1, A.7.3.1.2 and A.7.3.1.3, and other editorial corrections on TCs in A.7.1, A.7.3, based on R4-1910175. - Change-26: Modifying SMTC, CSI-RS configurations, adding RLM-RS configuration, modifying the values of T1/T2/T3/T4/D1, adding the SNR level and removal of the measurement gap configuration for DRX test in order to reduce the total test time, for TC A.7.5.5.1 and A.7.5.5.2, based on R4-1910547. - Change-27: Clarifying CSI-RS configruation for q0 and q1, modifying SMTC configuraiton, CSI-RS configuration for CSI reporting, adding RLM-RS configuration, modifying the values of T1/T2/T3/T4/D1, adding the SNR level and removal of the measurement gap configuration, for TC A.7.5.5.3 and A.7.5.5.4, based on R4-1910549. - Section A.8: - Endorsed CR: R4-1908326, R4-1910175, R4-1910174 - Correct FR1 to FR2 in the title and in the description of TC A.8.4.2.8. - “RRC CONNECTION REQUEST” is replaced by “RRCSetupRequest”. - Square brackets clean-up. - The noise and signal power levels of measurement accuracy test cases are added for A.8.5.2 - For A.8.5.2, delete one sub-test for RSRP FR1 and FR2, RSRQ FR2, inter-RAT measurement which was just a place holder - Delete redundant TCI state description for A.8.5.2 - Modify the AoA configuration for A.8.5.2 - Section B - Endorsed CRs: R4-1907982, R4-1909613,R4-1910106 - Update and finalise FR2 side condition values for Power classes 2 and 3 in: - Table B.1.2-2: Conditions for intra-frequency cell re-selection - Clause B.2.1.3 Derivation of Minimum SSB_RP values - Table B.2.2-2: Conditions for intra-frequency measurements - Table B.2.3-2: Conditions for inter-frequency measurements - B.2.4.1-2: Conditions for SSB based L1-RSRP measurements - B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements - B.2.5-2: Conditions for RRC connection release, redirection to NR - B.2.6.1-2: Conditions for SSB based UE transmit timing - B.2.6.2-2: Conditions for CSI-RS based UE transmit timing - In FR1 Tables B.2.6.1-1 and B.2.6.2-1, remove [ ] from Es/Iot values - Remove the conditions for CSI-RS based UE transmit timing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1, 2, 3, 4, 5, 6.1, 6.2, 7.1, 7.3, 7.5, 7.6, 7.7, 8.1, 8.2, 8.3, 8.4, 8.5, 8.6, 8.9, 8.10, 9.2, 9.3, 9.4, 9.5, 10.1, A.3, A.4, A.5, A.6, A.7, A.8, B |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

3GPP TS 38.133 V16.0.0 (2019-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Requirements for support of radio resource management

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	20

1	Scope	20

2	References	20

3	Definitions, symbols and abbreviations	21

3.1	Definitions	21

3.2	Symbols	22

3.3	Abbreviations	22

3.4	Test tolerances	24

3.5	Frequency bands grouping	24

3.5.1	Introduction	24

3.5.2	NR operating bands in FR1	24

3.5.3	NR operating bands in FR2	24

3.6	Applicability of requirements in this specification version	25

3.6.1	RRC connected state requirements in DRX	25

3.6.2	Number of serving carriers	26

3.6.2.1	Number of serving carriers for SA	26

3.6.2.2	Number of serving carriers for EN-DC	26

3.6.2.3	Number of serving carriers for NE-DC	26

3.6.2.4	Number of serving carriers for NR-DC	26

3.6.3	Applicability for SSB Rx beam in intra-band FR2	26

3.6.4	Applicability for FR2 UE power classes	27

3.6.5	Applicability of requirements for NGEN-DC operation	27

4	SA: RRC_IDLE state mobility	27

4.1	Cell Selection	27

4.2	Cell Re-selection	27

4.2.1	Introduction	27

4.2.2	Requirements	27

4.2.2.1	UE measurement capability	27

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	30

4.2.2.6	Maximum interruption in paging reception	31

4.2.2.7	General requirements	32

5	SA: RRC_INACTIVE state mobility	33

5.1	Cell Re-selection	33

5.1.1	Introduction	33

5.1.2	Requirements	33

5.1.2.1	UE measurement capability	33

5.1.2.2	Measurement and evaluation of serving cell	33

5.1.2.3	Measurements of intra-frequency NR cells	33

5.1.2.4	Measurements of inter-frequency NR cells	33

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	33

5.1.2.6	Maximum interruption in paging reception	33

5.1.2.7	General requirements	33

5.2	RRC_INACTIVE Mobility Control	33

6	RRC_CONNECTED state mobility	34

6.1	Handover	34

6.1.1	NR Handover	34

6.1.1.1	Introduction	34

6.1.1.2	NR FR1 - NR FR1 Handover	34

6.1.1.2.1	Handover delay	34

6.1.1.2.2	Interruption time	34

6.1.1.3	NR FR2- NR FR1 Handover	35

6.1.1.3.1	Handover delay	35

6.1.1.3.2	Interruption time	35

6.1.1.4	NR FR2- NR FR2 Handover	35

6.1.1.4.1	Handover delay	35

6.1.1.4.2	Interruption time	36

6.1.1.5	NR FR1- NR FR2 Handover	36

6.1.1.5.1	Handover delay	36

6.1.1.5.2	Interruption time	36

6.1.2	NR Handover to other RATs	37

6.1.2.1	NR – E-UTRAN Handover	37

6.1.2.1.1	Introduction	37

6.1.2.1.2	Handover delay	37

6.1.2.1.3	Interruption time	37

6.2	RRC Connection Mobility Control	38

6.2.1	SA: RRC Re-establishment	38

6.2.1.1	Introduction	38

6.2.1.2	Requirements	38

6.2.1.2.1	UE Re-establishment delay requirement	38

6.2.2	Random access	40

6.2.2.1	Introduction	40

6.2.2.2	Requirements	40

6.2.2.2.1	Contention based random access	40

6.2.2.2.2	Non-Contention based random access	41

6.2.2.2.3	UE behaviour when configured with supplementary UL	42

6.2.3	SA: RRC Connection Release with Redirection	42

6.2.3.1	Introduction	42

6.2.3.2	Requirements	42

6.2.3.2.1	RRC connection release with redirection to NR	42

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	43

7	Timing	44

7.1	UE transmit timing	44

7.1.1	Introduction	44

7.1.2	Requirements	44

7.2	UE timer accuracy	45

7.2.1	Introduction	45

7.2.2	Requirements	45

7.3	Timing advance	46

7.3.1	Introduction	46

7.3.2	Requirements	46

7.3.2.1	Timing Advance adjustment delay	46

7.3.2.2	Timing Advance adjustment accuracy	46

7.4	Cell phase synchronization accuracy	46

7.4.1	Definition	46

7.4.2	Minimum requirements	46

7.5	Maximum Transmission Timing Difference	47

7.5.1	Introduction	47

7.5.2	Minimum Requirements for inter-band EN-DC	47

7.5.3	Minimum Requirements for intra-band EN-DC	47

7.5.4	Minimum Requirements for NR Carrier Aggregation	47

7.5.5	Minimum Requirements for inter-band NE-DC	48

7.5.6	Minimum Requirements for inter-band NR-NR DC	48

7.6	Maximum Receive Timing Difference	49

7.6.1	Introduction	49

7.6.2	Minimum Requirements for inter-band EN-DC	49

7.6.3	Minimum Requirements for intra-band EN-DC	50

7.6.4	Minimum Requirements for NR Carrier Aggregation	50

7.6.5	Minimum Requirements for inter-band NE-DC	50

7.6.6	Minimum Requirements for inter-band NR-NR DC	51

7.7	deriveSSB-IndexFromCell tolerance	51

7.7.1	Minimum requirements	51

7.8	Maximum Allowed UE Transition Times for TDD Intra-band Carrier Aggregation	52

7.8.1	Requirements	52

8	Signalling characteristics	52

8.1	Radio Link Monitoring	52

8.2	Interruption	60

8.2.1.2	Requirements	60

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	60

8.2.1.2.3	Interruptions at SCell addition/release	61

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	63

8.2.1.2.7	Interruption due to Active BWP switching Requirement	64

8.2.2.2.1	Interruptions at SCell addition/release	65

8.2.2.2.2	Interruptions at SCell activation/deactivation	65

8.2.2.2.3	Interruptions during measurements on SCC for intra-band CA	66

8.2.2.2.5	Interruption due to Active BWP switching Requirement	67

8.2.3	NE-DC Interruptions	68

8.2.3.1	Introduction	68

8.2.3.2	Requirements	68

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	68

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	69

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	71

8.2.3.2.7	Interruption due to Active BWP switching Requirement	71

8.2.4	NR-NR DC: Interruptions	71

8.2.4.1	Introduction	71

8.2.4.2	Requirements	72

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	72

8.2.4.2.2	Interruptions at SCell activation/deactivation	72

8.2.4.2.3	Interruptions during measurements on SCC	73

8.2.4.2.5	Interruption due to Active BWP switching Requirement	74

8.3	SCell Activation and Deactivation Delay	74

8.3.1	Introduction	74

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	74

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	75

8.4	UE UL carrier RRC reconfiguration Delay	76

8.4.1	Introduction	76

8.4.2	UE UL carrier configuration Delay Requirement	76

8.4.3	UE UL carrier deconfiguration Delay Requirement	76

8.5.7	Scheduling availability of UE during beam failure detection	84

8.6	Active BWP switch delay	86

8.6.1	Introduction	86

8.6.2	DCI and timer based BWP switch delay	86

8.6.3	RRC based BWP switch delay	87

8.7	Void	87

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	87

8.8.1	Introduction	87

8.8.2	E-UTRAN PSCell Addition Delay Requirement	87

8.8.3	E-UTRAN PSCell Release Delay Requirement	88

9	Measurement Procedure	89

9.1	General measurement requirement	89

9.1.1	Introduction	89

9.1.2	Measurement gap	89

9.1.2.1	NSA: Measurement Gap Sharing	97

9.1.2.1a	SA: Measurement Gap Sharing	97

9.1.2.1b	NE-DC: Measurement Gap Sharing	98

9.1.2.1c	NR-DC: Measurement Gap Sharing	98

9.1.3	UE Measurement capability	98

9.1.3.1	NSA: Monitoring of multiple layers using gaps	98

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	99

9.1.3.2	NSA: Maximum allowed layers for multiple monitoring	100

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	101

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	101

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	102

9.1.4.1	Introduction	102

9.1.4.2	Requirements	102

9.1.5	Carrier-specific scaling factor	103

9.1.5.1	Monitoring of multiple layers outside gaps	103

9.1.5.1.1	NSA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	103

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps without NR-DC operation	104

9.1.5.1.3	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps with NR-DC operation	104

9.1.5.2	Monitoring of multiple layers within gaps	105

9.1.5.2.1	NSA mode: carrier-specific scaling factor for SSB-based measurements performed within gaps	105

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed within gaps	106

9.1.6	Minimum requirement at transitions	107

9.2	NR intra-frequency measurements	107

9.2.1	Introduction	107

9.2.2	Requirements applicability	108

9.2.3	Number of cells and number of SSB	108

9.2.4	Measurement Reporting Requirements	109

9.2.5	Intrafrequency measurements without measurement gaps	109

9.2.5.3.1	Scheduling availability of UE performing measurements with a same subcarrier spacing as PDSCH/PDCCH on FR1	113

9.2.6	Intrafrequency measurements with measurement gaps	114

9.3	NR inter-frequency measurements	116

9.3.2	Requirements applicability	116

9.3.2.1	Void	116

9.3.2.2	Void	116

9.3.3	Number of cells and number of SSB	116

9.3.4	Inter frequency cell identification	117

9.3.4.1	Void	118

9.3.4.2	Void	118

9.3.5	Inter frequency measurements	118

9.3.5.1	Void	119

9.3.5.2	Void	119

9.3.5.3	Void	119

9.3.6	NR Inter frequency measurements reporting requirements	119

9.3.7	Void	120

9.4	Inter-RAT measurements	120

9.4.1	Introduction	120

9.4.2	SA: NR − E-UTRAN FDD measurements	121

9.4.3	SA: NR − E-UTRAN TDD measurements	123

9.4.4	SA: Inter-RAT RSTD measurements	125

9.4.5	SA: Inter-RAT E-CID measurements	132

10	Measurement Performance requirements	139

10.1.1	Introduction	139

10.1.21	SFTD accuracy requirements	168

10.1.21.1	SFTD acuracy requirements for NE-DC	168

10.2	E-UTRAN measurements	169

10.2.1	Introduction	169

10.2.2	E-UTRAN RSRP measurements	170

10.2.3	E-UTRAN RSRQ measurements	170

10.2.4	E-UTRAN RSTD measurements	170

10.2.5	E-UTRAN RS-SINR measurements	170

11	Measurements Performance Requirements for NR network	170

Annex A (normative):	Test Cases	171

A.1	Purpose of annex	171

A.2	Requirement classification for statistical testing	171

A.2.1	Types of requirements in TS 38.133	171

A.2.1.1	Time and delay requirements on UE higher layer actions	171

A.2.1.2	Measurements of power levels, relative powers and time	171

A.2.1.3	Implementation requirements	172

A.2.1.4	Physical layer timing requirements	172

A.3	RRM test configurations	173

A.3.1	Reference measurement channels	173

A.3.1.1	PDSCH	173

A.3.1.1.1	FDD	173

A.3.1.1.2	TDD	174

A.3.1.2	CORESET for RMSI scheduling	177

A.3.1.2.1	FDD	177

A.3.1.2.2	TDD	178

A.3.1.3	CORESET for RMC scheduling	181

A.3.1.3.1	FDD	181

A.3.1.3.2	TDD	182

A.3.1.4	TDD UL/DL configuration	184

A.3.2	OFDMA channel noise generator (OCNG)	185

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	185

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	185

A.3.2.2	Void	186

A.3.3	Reference DRX configurations	186

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	186

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	186

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	186

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms	187

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms	187

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = Infinity	187

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	188

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	188

A.3.4	Test Cases with Different Channel Bandwidths	188

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	188

A.3.4.1.1	Introduction	188

A.3.4.1.2	Principle of testing	188

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	189

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	189

A.3.5.1.1	Introduction	189

A.3.5.1.2	Principle of Testing	189

A.3.6	Antenna configurations	189

A.3.6.1	Antenna configurations for FR1	189

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	189

A.3.6.1.1.1	Introduction	189

A.3.6.1.1.2	Principle of testing	189

A.3.6.2	Antenna configurations for FR2	191

A.3.7	EN-DC test setup	191

A.3.7.1	Introduction	191

A.3.7.2	E-UTRAN Serving Cell Parameters	191

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	191

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	193

A.3.8	PRACH configurations	194

A.3.8.1	Introduction	194

A.3.8.2	PRACH configurations in FR1	194

A.3.8.2.1	FR1 PRACH configuration 1	194

A.3.8.2.2	FR1 PRACH configuration 2	195

A.3.8.2.3	FR1 PRACH configuration 3	195

A.3.8.2.4	FR1 PRACH configuration 4	196

A.3.8.3	PRACH configurations in FR2	197

A.3.8.3.1	FR2 PRACH configuration 1	197

A.3.8.3.2	FR2 PRACH configuration 2	198

A.3.8.3.3	FR2 PRACH configuration 3	199

A.3.8.3.4	FR2 PRACH configuration 4	200

A.3.9	BWP configurations	201

A.3.9.1	Introduction	201

A.3.9.2	Downlink BWP configurations	202

A.3.9.2.1	Initial BWP	202

A.3.9.2.2	Dedicated BWP	202

A.3.9.3	Uplink BWP configurations	202

A.3.9.3.1	Initial BWP	202

A.3.9.3.2	Dedicated BWP	203

A.3.10	SSB Configurations	203

A.3.10.1	SSB Configurations for FR1	203

A.3.10.1.1	SSB pattern 1 in FR1: SSB for SSB SCS=15 KHz in 10 MHz	203

A.3.11	SMTC Configurations	206

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	206

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	206

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	206

A.3.12	Test Cases with Different CC Configurations	207

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	207

A.3.12.1.1	Introduction	207

A.3.12.1.2	Principle of testing	207

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	207

A.3.12.2.1	Introduction	207

A.3.12.2.2	Principle of testing	207

A.3.13	Test Cases in Standalone and Non-Standalone Operations	207

A.3.13.1	Introduction	207

A.3.13.2	Principle of Testing	207

A.3.14	CSI-RS configurations	208

A.3.14.1	FDD	208

A.3.14.2	TDD	210

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	213

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	213

A.3.16	TCI State Configuration	213

A.3.16.1	Introduction	213

A.3.16.2	TCI states	213

A.3.17	Configurations of CSI-RS for tracking	214

A.3.17.1	Configuration of CSI-RS for tracking for FR1	214

A.3.17.1.1	FDD	214

A.3.17.1.2	TDD	215

A.3.17.2	Configuration of CSI-RS for tracking for FR2	216

A.3.17.2.1	TDD	216

A.4	EN-DC tests with all NR cells in FR1	216

A.4.1	Void	216

A.4.2	Void	216

A.4.3	RRC_CONNECTED state mobility	216

A.4.3.1	Void	216

A.4.3.2	RRC Connection Mobility Control	216

A.4.3.2.1	Void	216

A.4.3.2.3	Void	223

A.4.4	Timing	223

A.4.4.1	UE transmit timing	223

A.4.4.1.1	NR UE Transmit Timing Test for FR1	223

A.4.4.1.1.1	Test Purpose and environment	223

A.4.4.1.1.2	Test requirements	225

A.4.4.2	UE timer accuracy	226

A.4.4.3	Timing advance	226

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	226

A.4.4.3.1.1	Test Purpose and Environment	226

A.4.4.3.1.2	Test Parameters	226

A.4.4.3.1.3	Test Requirements	228

A.4.5	Signaling characteristics	229

A.4.5.1	Radio link Monitoring	229

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	229

A.4.5.1.1.1	Test Purpose and Environment	229

A.4.5.1.1.2	Test Requirements	232

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	232

A.4.5.1.2.1	Test Purpose and Environment	232

A.4.5.1.2.2	Test Requirements	236

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	236

A.4.5.1.3.1	Test Purpose and Environment	236

A.4.5.1.3.2	Test Requirements	239

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	239

A.4.5.1.4.1	Test Purpose and Environment	239

A.4.5.1.4.2	Test Requirements	242

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	242

A.4.5.1.5.1	Test Purpose and Environment	242

A.4.5.1.5.2	Test Requirements	245

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	246

A.4.5.1.6.1	Test Purpose and Environment	246

A.4.5.1.6.2	Test Requirements	249

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	249

A.4.5.1.7.1	Test Purpose and Environment	249

A.4.5.1.7.2	Test Requirements	254

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	254

A.4.5.1.8.1	Test Purpose and Environment	254

A.4.5.1.8.2	Test Requirements	259

A.4.5.2	Interruption	259

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	259

A.4.5.2.1.2	Test Requirements	261

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	262

A.4.5.2.2.1	Test Purpose and Environment	262

A.4.5.2.2.2	Test Requirements	264

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	264

A.4.5.2.3.1	Test Purpose and Environment	264

A.4.5.2.3.2	Test Requirements	266

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	267

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	270

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	272

A.4.5.2.7	Void	275

A.4.5.4	UE UL carrier RRC reconfiguration Delay	283

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	283

A.4.5.4.1.1 Test Purpose and Environment	283

A.4.5.4.1.2	Test Requirements	288

A.4.5.5	Beam Failure Detection and Link recovery procedures	288

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	288

A.4.5.5.1.1	Test Purpose and Environment	288

A.4.5.5.1.2	Test Requirements	292

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	292

A.4.5.5.2.1	Test Purpose and Environment	292

A.4.5.5.2.2	Test Requirements	296

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	297

A.4.5.5.3.1	Test Purpose and Environment	297

A.4.5.5.3.2	Test Requirements	301

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	302

A.4.5.5.4.1	Test Purpose and Environment	302

A.4.5.5.4.2	Test Requirements	307

A.4.5.6	Active BWP switch delay	308

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	308

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	308

A.4.5.6.1.1.1	Test Purpose and Environment	308

A.4.5.6.1.1.2	Test Requirements	311

A.4.5.6.2	RRC-based Active BWP Switch	312

A.4.5.6.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	312

A.4.5.6.2.1.1	Test Purpose and Environment	312

A.4.5.6.2.2.2	Test Requirements	314

A.4.5.7	PSCell addition and release delay	314

A.4.5.7.1	Addition and Release Delay of known NR PSCell	314

A.4.5.7.1.1	Test purpose and environment	314

A.4.5.7.1.2	Test Requirements	318

A.4.6	Measurement procedure	318

A.4.6.1	Intra-frequency Measurements	318

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	318

A.4.6.1.1.1	Test purpose and Environment	318

A.4.6.1.1.2	Test parameters	318

A.4.6.1.1.3	Test Requirements	320

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	320

A.4.6.1.2.1	Test purpose and Environment	320

A.4.6.1.2.2	Test parameters	321

A.4.6.1.2.2	Test Requirements	323

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	323

A.4.6.1.3.1	Test purpose and Environment	323

A.4.6.1.3.2	Test parameters	323

A.4.6.1.3.3	Test Requirements	325

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	325

A.4.6.1.4.1	Test purpose and Environment	325

A.4.6.1.4.2	Test parameters	326

A.4.6.1.4.3	Test Requirements	328

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	328

A.4.6.1.5.1	Test purpose and Environment	328

A.4.6.1.5.2	Test parameters	328

A.4.6.1.5.3	Test Requirements	330

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	331

A.4.6.1.6.1	Test purpose and Environment	331

A.4.6.1.6.2	Test parameters	331

A.4.6.1.6.3	Test Requirements	332

A.4.6.2	Inter-frequency Measurements	333

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	333

A.4.6.2.1.1	Test Purpose and Environment	333

A.4.6.2.1.2	Test Requirements	336

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	336

A.4.6.2.2.1	Test Purpose and Environment	336

A.4.6.2.2.2	Test Requirements	339

A.4.6.2.3	Void	340

A.4.6.2.4	Void	340

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	340

A.4.6.2.5.1	Test Purpose and Environment	340

A.4.6.2.5.2	Test Requirements	343

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	343

A.4.6.2.6.1	Test Purpose and Environment	343

A.4.6.2.6.2	Test Requirements	346

A.4.6.2.7	Void	347

A.4.6.2.8	Void	347

A.4.7	Measurement Performance requirements	347

A.4.7.1	SS-RSRP	347

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	347

A.4.7.1.1.1	Test Purpose and Environment	347

A.4.7.1.1.2	Test parameters	347

A.4.7.1.1.3	Test Requirements	350

A.4.7.1.3	Void	353

A.4.7.2	SS-RSRQ	353

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	353

A.4.7.2.1.1	Test Purpose and Environment	353

A.4.7.2.1.2	Test Parameters	353

A.4.7.2.1.3	Test Requirements	356

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	356

A.4.7.2.2.1	Test Purpose and Environment	356

A.4.7.2.2.2	Test Parameters	356

A.4.7.2.2.3	Test Requirements	359

A.4.7.3	SS-SINR	359

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	359

A.4.7.3.1.1	Test Purpose and Environment	359

A.4.7.3.1.2	Test Parameters	359

A.4.7.3.1.3	Test Requirements	362

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	362

A.4.7.3.2.1	Test Purpose and Environment	362

A.4.7.3.2.2	Test Parameters	362

A.4.7.3.2.3	Test Requirements	365

A.4.7.4	L1-RSRP measurement for beam reporting	366

A.4.7.4.1	SSB based L1-RSRP measurement	366

A.4.7.4.1.1	Test Purpose and Environment	366

A.4.7.4.1.2	Test parameters	366

A.4.7.4.1.3	Test Requirements	368

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	369

A.4.7.4.2.1	Test Purpose and Environment	369

A.4.7.4.2.2	Test parameters	369

A.4.7.4.2.3	Test Requirements	371

A.4.7.5	SFTD accuracy	371

A.4.7.5.1	Test Purpose and Environment	371

A.4.7.5.2	Test Requirements	372

A.4.7.5.3	Test Requirements	374

A.4.8	Void	374

A.5	EN-DC tests with one or more NR cells in FR2	375

A.5.1	Void	375

A.5.2	Void	375

A.5.3	RRC_CONNECTED state mobility	375

A.5.3.1	Void	375

A.5.3.2	RRC Connection Mobility Control	375

A.5.3.2.1	Void	375

A.5.3.2.2	Random Access	375

A.5.3.2.2.1	Contention based random access test in FR2 for PSCell/SCell in EN-DC	375

A.5.3.2.2.2	Non-contention based random access test in FR2 for PSCell/SCell in EN-DC	379

A.5.3.2.3	Void	383

A.5.4	Timing	383

A.5.4.1	UE transmit timing	383

A.5.4.1.1	NR UE Transmit Timing Test for FR2	383

A.5.4.1.1.1	Test Purpose and environment	383

A.5.4.1.1.2	Test requirements	385

A.5.4.2	UE timer accuracy	386

A.5.4.3	Timing advance	386

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	386

A.5.4.3.1.1 Test Purpose and Environment	386

A.5.4.3.1.2 Test Parameters	386

A.5.4.3.1.3	Test Requirements	389

A.5.5	Signaling characteristics	389

A.5.5.1	Radio link Monitoring	389

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	389

A.5.5.1.1.1	Test Purpose and Environment	389

A.5.5.1.1.2	Test Requirements	392

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	392

A.5.5.1.2.1	Test Purpose and Environment	392

A.5.5.1.2.2	Test Requirements	395

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	395

A.5.5.1.3.1	Test Purpose and Environment	395

A.5.5.1.3.2	Test Requirements	398

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	398

A.5.5.1.4.1	Test Purpose and Environment	398

A.5.5.1.4.2	Test Requirements	402

A.5.5.2	Interruption	420

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	420

A.5.5.2.1.1	Test Purpose and Environment	420

A.5.5.2.1.2	Test Requirements	422

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	423

A.5.5.2.2.1	Test Purpose and Environment	423

A.5.5.2.2.2	Test Requirements	425

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	425

A.5.5.2.3.1	Test Purpose and Environment	425

A.5.5.2.3.2	Test Requirements	428

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	428

A.5.5.2.4.1	Test Purpose and Environment	428

A.5.5.2.4.2	Test Requirements	431

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	431

A.5.5.2.5.1	Test Purpose and Environment	431

A.5.5.2.5.2	Test Requirements	434

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	434

A.5.5.2.6.1	Test Purpose and Environment	434

A.5.5.2.6.2	Test Requirements	437

A.5.5.4	UE UL carrier RRC reconfiguration Delay	442

A.5.5.5	Beam Failure Detection and Link recovery procedures	442

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	442

A.5.5.5.1.1	Test Purpose and Environment	442

A.5.5.5.1.2	Test Requirements	446

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	446

A.5.5.5.2.1	Test Purpose and Environment	446

A.5.5.5.2.2	Test Requirements	450

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	451

A.5.5.5.3.1	Test Purpose and Environment	451

A.5.5.5.3.2	Test Requirements	455

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	456

A.5.5.5.4.1	Test Purpose and Environment	456

A.5.5.5.4.2	Test Requirements	460

A.5.5.6	Active BWP switch delay	461

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	461

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	461

A.5.5.6.1.1.1	Test Purpose and Environment	461

A.5.5.6.1.1.2	Test Requirements	464

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 DL active BWP switch with FR2 SCell in non-DRX in synchronous EN-DC	465

A.5.5.6.1.2.1	Test Purpose and Environment	465

A.5.5.6.1.2.2	Test Requirements	468

A.5.5.6.2	RRC-based Active BWP Switch	468

A.5.5.6.2.1.1	Test Purpose and Environment	468

A.5.5.6.1.2.2	Test Requirements	470

A.5.6	Measurement procedure	471

A.5.6.1	Intra-frequency Measurements	471

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	471

A.5.6.1.1.1	Test purpose and Environment	471

A.5.6.1.1.2	Test Requirements	473

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	473

A.5.6.1.2.1	Test purpose and Environment	473

A.5.6.1.2.2	Test Requirements	476

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	476

A.5.6.1.3.1	Test purpose and Environment	476

A.5.6.1.3.2	Test Requirements	479

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	479

A.5.6.1.4.1	Test purpose and Environment	479

A.5.6.1.4.2	Test Requirements	482

A.5.6.2	Inter-frequency Measurements	482

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	482

A.5.6.2.1.1	Test Purpose and Environment	482

A.5.6.2.1.2	Test Requirements	485

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	485

A.5.6.2.2.1	Test Purpose and Environment	485

A.5.6.2.2.2	Test Requirements	488

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	488

A.5.6.2.3.1	Test Purpose and Environment	488

A.5.6.2.3.2	Test Requirements	492

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	492

A.5.6.2.4.1	Test Purpose and Environment	492

A.5.6.2.2.4	Test Requirements	495

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	495

A.5.6.2.5.1	Test Purpose and Environment	495

A.5.6.2.5.2	Test Requirements	499

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	500

A.5.6.2.6.1	Test Purpose and Environment	500

A.5.6.2.6.2	Test Requirements	503

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	504

A.5.6.2.7.1	Test Purpose and Environment	504

A.5.6.2.7.2	Test Requirements	507

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	508

A.5.6.2.8.1	Test Purpose and Environment	508

A.5.6.2.8.2	Test Requirements	511

A.5.7	Measurement Performance requirements	512

A.5.7.1	SS-RSRP	512

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	512

A.5.7.1.1.1	Test Purpose and Environment	512

A.5.7.1.1.2	Test parameters	512

A.5.7.1.1.3	Test Requirements	514

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	514

A.5.7.1.2.1	Test Purpose and Environment	514

A.5.7.1.2.2	Test parameters	514

A.5.7.1.2.3	Test Requirements	516

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	516

A.5.7.1.3.1	Test Purpose and Environment	516

A.5.7.2	SS-RSRQ	520

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	520

A.5.7.2.1.1	Test Purpose and Environment	520

A.5.7.2.1.2	Test Parameters	520

A.5.7.2.1.3	Test Requirements	522

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	523

A.5.7.2.2.1	Test Purpose and Environment	523

A.5.7.2.2.2	Test Parameters	523

A.5.7.2.2.3	Test Requirements	525

A.5.7.3	SS-SINR	525

A.5.7.3.1	Intra-frequency case	525

A.5.7.3.1.1	Test Purpose and Environment	525

A.5.7.3.1.2	Test Parameters	525

A.5.7.3.1.3	Test Requirements	527

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	527

A.5.7.3.2.1	Test Purpose and Environment	527

A.5.7.3.2.2	Test Parameters	527

A.5.7.3.2.3	Test Requirements	529

A.5.7.4	L1-RSRP measurement for beam reporting	530

A.5.7.4.1	SSB based L1-RSRP measurement	530

A.5.7.4.1.1	Test Purpose and Environment	530

A.5.7.4.1.2	Test parameters	530

A.5.7.4.1.3	Test Requirements	532

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	532

A.5.7.4.2.1	Test Purpose and Environment	532

A.5.7.4.2.2	Test parameters	533

A.5.7.4.2.3	Test Requirements	535

A.5.8	PSCell addition and release delay	535

A.5.8.1	Addition and Release Delay of NR PSCell	535

A.5.8.1.1	Test purpose and environment	535

A.5.8.1.2	Test Requirements	537

A.6	NR standalone tests with all NR cells in FR1	539

A.6.1	SA: RRC_IDLE state mobility	539

A.6.1.1	Cell re-selection to NR	539

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	539

A.6.1.1.1.1	Test Purpose and Environment	539

A.6.1.1.1.2	Test Parameters	539

A.6.1.1.1.3	Test Requirements	542

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	542

A.6.1.1.2.1	Test Purpose and Environment	542

A.6.1.1.2.2	Test Parameters	542

A.6.1.1.2.3	Test Requirements	544

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	545

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	545

A.6.1.2.1.1	Test Purpose and Environment	545

A.6.1.2.1.2	Test Parameters	545

A.6.1.2.1.3	Test Requirements	548

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	548

A.6.1.2.2.1	Test Purpose and Environment	548

A.6.1.2.2.2	Test Parameters	548

A.6.1.2.2.3	Test Requirements	551

A.6.2	SA: RRC_INACTIVE state mobility	552

A.6.3	RRC_CONNECTED state mobility	552

A.6.3.1	Handover	552

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	552

A.6.3.1.1.1	Test Purpose and Environment	552

A.6.3.1.1.2	Test Parameters	552

A.6.3.1.1.3 Test Requirements	554

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	554

A.6.3.1.2.1	Test Purpose and Environment	554

A.6.3.1.2.2	Test Parameters	554

A.6.3.1.2.3	Test Requirements	556

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	556

A.6.3.1.3.1	Test Purpose and Environment	556

A.6.3.1.3.2	Test Parameters	556

A.6.3.1.3.3 Test Requirements	558

A.6.3.1.4	 SA NR - E-UTRAN handover	558

A.6.3.1.4.1	Test Purpose and Environment	558

A.6.3.1.4.2	Test Requirements	562

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	562

A.6.3.1.5.1	Test Purpose and Environment	562

A.6.3.1.5.2	Test Requirements	565

A.6.3.2	RRC Connection Mobility Control	566

A.6.3.2.1	SA: RRC Re-establishment	566

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	566

A.6.3.2.1.1.1	Test Purpose and Environment	566

A.6.3.2.1.1.2	Test Requirements	568

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	568

A.6.3.2.1.2.1	Test Purpose and Environment	568

A.6.3.2.1.2.2	Test Requirements	570

A.6.3.2.2	Random Access	571

A.6.3.2.2.1	Contention based random access test in FR1 for NR standalone	571

A.6.3.2.2.2	Non-Contention based random access test in FR1 for NR standalone	574

A.6.3.2.3	SA: RRC Connection Release with Redirection	577

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	577

A.6.3.2.3.1.1	Test Purpose and Environment	577

A.6.3.2.3.1.2	Test Parameters	577

A.6.3.2.3.1.3	Test Requirements	579

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	580

A.6.3.2.3.2.1	Test Purpose and Environment	580

A.6.3.2.3.2.2	Test Parameters	580

A.6.3.2.3.2.3	Test Requirements	583

A.6.4	Timing	584

A.6.4.1	UE transmit timing	584

A.6.4.1.1	NR UE Transmit Timing Test for FR1	584

A.6.4.1.1.1	Test Purpose and environment	584

A.6.4.1.1.2	Test requirements	586

A.6.4.2	UE timer accuracy	587

A.6.4.3	Timing advance	587

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	587

A.6.4.3.1.1	Test Purpose and Environment	587

A.6.4.3.1.2	Test Parameters	587

A.6.4.3.1.3	Test Requirements	590

A.6.5	Signaling characteristics	590

A.6.5.1	Radio link Monitoring	590

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	591

A.6.5.1.1.1	Test Purpose and Environment	591

A.6.5.1.1.2	Test Requirements	594

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	594

A.6.5.1.2.1	Test Purpose and Environment	594

A.6.5.1.2.2	Test Requirements	598

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	598

A.6.5.1.3.1	Test Purpose and Environment	598

A.6.5.1.3.2	Test Requirements	601

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	601

A.6.5.1.4.1	Test Purpose and Environment	601

A.6.5.1.4.2	Test Requirements	605

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	605

A.6.5.1.5.1	Test Purpose and Environment	605

A.6.5.1.5.2	Test Requirements	608

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	609

A.6.5.1.6.1	Test Purpose and Environment	609

A.6.5.1.6.2	Test Requirements	612

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	612

A.6.5.1.7.1	Test Purpose and Environment	612

A.6.5.1.7.2	Test Requirements	617

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	617

A.6.5.1.8.1	Test Purpose and Environment	617

A.6.5.1.8.2	Test Requirements	622

A.6.5.3	SCell Activation and Deactivation Delay	625

A.6.5.4	UE UL carrier RRC reconfiguration Delay	630

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	630

A.6.5.4.1.1	Test Purpose and Environment	630

A.6.5.4.1.2	Test Requirements	634

A.6.5.4.2	Test Requirements	635

A.6.5.5	Beam Failure Detection and Link recovery procedures	635

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	635

A.6.5.5.1.1	Test Purpose and Environment	635

A.6.5.5.1.2	Test Requirements	639

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	639

A.6.5.5.2.1	Test Purpose and Environment	639

A.6.5.5.2.2	Test Requirements	643

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	643

A.6.5.5.3.1	Test Purpose and Environment	643

A.6.5.5.3.2	Test Requirements	647

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	648

A.6.5.5.4.1	Test Purpose and Environment	648

A.6.5.5.4.2	Test Requirements	652

A.6.5.6	Active BWP switch delay	653

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	653

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	653

A.6.5.6.1.1.1	Test Purpose and Environment	653

A.6.5.6.1.1.2	Test Requirements	656

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	656

A.6.5.6.1.2.1	Test Purpose and Environment	656

A.6.5.6.1.2.2	Test Requirements	659

A.6.5.6.2	RRC-based Active BWP Switch	659

A.6.5.6.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	659

A.6.6	Measurement procedure	662

A.6.6.1	Intra-frequency Measurements	662

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	662

A.6.6.1.1.1	Test purpose and Environment	662

A.6.6.1.1.2	Test parameters	662

A.6.6.1.1.3	Test Requirements	664

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	664

A.6.6.1.2.1	Test purpose and Environment	664

A.6.6.1.2.2	Test parameters	664

A.6.6.1.2.3	Test Requirements	666

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	666

A.6.6.1.3.1	Test purpose and Environment	666

A.6.6.1.3.2	Test parameters	666

A.6.6.1.3.3	Test Requirements	668

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	668

A.6.6.1.4.1	Test purpose and Environment	668

A.6.6.1.4.2	Test parameters	669

A.6.6.1.4.3	Test Requirements	671

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	671

A.6.6.1.5.1	Test purpose and Environment	671

A.6.6.1.5.2	Test parameters	671

A.6.6.1.5.3	Test Requirements	673

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	673

A.6.6.1.6.1	Test purpose and Environment	673

A.6.6.1.6.2	Test parameters	673

A.6.6.1.6.3	Test Requirements	675

A.6.6.2	Inter-frequency Measurements	675

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	675

A.6.6.2.1.1	Test Purpose and Environment	675

A.6.6.2.1.2	Test Requirements	678

A.6.6.2.2 	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	678

A.6.6.2.2.1	Test Purpose and Environment	678

A.6.6.2.2.2	Test Requirements	681

A.6.6.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	682

A.6.6.2.3.1	Test Purpose and Environment	682

A.6.6.2.3.2	Test Requirements	685

A.6.6.2.4	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	685

A.6.6.2.4.1	Test Purpose and Environment	685

A.6.6.2.4.2	Test Requirements	688

A.6.6.3	Inter-RAT Measurements	688

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	688

A.6.6.3.1.2	Test Requirements	691

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	692

A.6.7	Measurement Performance requirements	696

A.6.7.1	SS-RSRP	696

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	696

A.6.7.1.1.1	Test Purpose and Environment	696

A.6.7.1.1.2	Test parameters	696

A.6.7.1.1.3	Test Requirements	699

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	699

A.6.7.1.2.1	Test Purpose and Environment	699

A.6.7.1.2.2	Test parameters	699

A.6.7.1.2.3	Test Requirements	701

A.6.7.1.3	Void	701

A.6.7.2	SS-RSRQ	701

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	701

A.6.7.2.1.1	Test Purpose and Environment	701

A.6.7.2.1.2	Test Parameters	702

A.6.7.2.1.3	Test Requirements	704

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	705

A.6.7.2.2.1	Test Purpose and Environment	705

A.6.7.2.2.2	Test Parameters	705

A.6.7.2.2.3	Test Requirements	708

A.6.7.3	SS-SINR	708

A.6.7.3.1	Intra-frequency case	708

A.6.7.3.1.1	Test Purpose and Environment	708

A.6.7.3.1.2	Test Parameters	708

A.6.7.3.1.3	Test Requirements	711

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	711

A.6.7.3.2.1	Test Purpose and Environment	711

A.6.7.3.2.2	Test Parameters	711

A.6.7.3.2.3	Test Requirements	714

A.6.7.4.2.2	Test parameters	717

A.7.3.1	Handover	726

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	726

A.7.3.1.1.1	Test Purpose and Environment	726

A.7.3.1.1.2	Test Parameters	726

A.7.3.1.1.3	Test Requirements	729

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	729

A.7.3.1.2.1	Test Purpose and Environment	729

A.7.3.1.2.2	Test Parameters	729

A.7.3.1.2.3	Test Requirements	731

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	731

A.7.3.1.3.1	Test Purpose and Environment	731

A.7.3.1.3.2	Test Parameters	731

A.7.3.1.3.3 Test Requirements	733

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	736

A.7.6.2	Inter-frequency Measurements	829

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	859

A.7.7.1.3.1	Test Purpose and Environment	859

A.7.7.3	SS-SINR	866

A.8	E-UTRA standalone tests for NR RRM	876

A.8.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	876

A.8.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	876

A.8.1.1.1	Test Purpose and Environment	876

A.8.1.1.2	Test Requirements	879

A.8.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	879

A.8.1.2.1	Test Purpose and Environment	879

A.8.1.2.2	Test Requirements	880

A.8.1.3	SFTD accuracy	881

A.8.1.3.1	Test Purpose and Environment	881

A.8.1.3.2	Test Requirements	881

A.8.1.3.3	Test Requirements	884

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	886

B.1	Conditions for NR RRC_IDLE state mobility	886

B.1.1	Introduction	886

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	886

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	887

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	887

B.2.1	Introduction	887

B.2.2	Conditions for NR intra-frequency measurements	887

B.2.3	Conditions for NR inter-frequency measurements	888

B.2.4	Conditions for NR L1-RSRP reporting	888

B.2.4.1	Conditions for SSB based L1-RSRP reporting	888

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	889

B.2.5	Conditions for RRC connection release with redirection to NR	890

B.3	RRM Requirements Exceptions	890

B.3.1	Introduction	890

B.3.2	Receiver sensitivity relaxation for CA	891

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	891

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	891

B.3.2.2.1	Inter-band carrier aggregation	891

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	891

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	891

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	892

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	892

B.3.2.4.1	Intra-band contiguous carrier aggregation	892

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	892

B.3.3	Receiver sensitivity relaxation for DC	892

B.3.4	Receiver sensitivity relaxation for SUL	892

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	892

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	892

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	892

Annex C (informative):	Change history	893