3GPP TS 38.141-2 V16.0.0 (2019-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Base Station (BS) conformance testing

Part 2: Radiated conformance testing

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	14

1	Scope	15

2	References	15

3	Definitions, symbols and abbreviations	17

3.1	Definitions	17

3.2	Symbols	20

3.3	Abbreviations	22

4	General radiated test conditions and declarations	24

4.1	Measurement uncertainties and test requirements	24

4.1.1	General	24

4.1.2	Acceptable uncertainty of OTA Test System	26

4.1.2.1	General	26

4.1.2.2	Measurement of transmitter	27

4.1.2.3	Measurement of receiver	30

4.1.2.4	Measurement of performance requirement	31

4.1.3	Interpretation of measurement results	32

4.2	Radiated requirement reference points	33

4.3	Base station classes	34

4.4	Regional requirements	34

4.5	BS configurations	35

4.5.1	Transmit configurations	35

4.5.2	Receive configurations	36

4.5.3	Power supply options	37

4.5.4	BS with integrated Iuant BS modem	37

4.6	Manufacturer’s declarations	37

4.7	Test configurations	45

4.7.1	General	45

4.7.2	Test signal configurations	46

4.7.2.1	Test signal used to build Test Configurations	46

4.7.2.2	NRTC1: Contiguous spectrum operation	46

4.7.2.2.1	NRTC1 generation	46

4.7.2.2.2	NRTC1 power allocation	47

4.7.2.3	NRTC2: Contiguous CA occupied bandwidth	47

4.7.2.3.1	NRTC2 generation	47

4.7.2.3.2	NRTC2 power allocation	47

4.7.2.4	NRTC3: Non-contiguous spectrum operation	48

4.7.2.4.1	NRTC3 generation	48

4.7.2.4.2	NRTC3 power allocation	48

4.7.2.5	NRTC4: Multi-band test configuration for full carrier allocation	48

4.7.2.5.1	NRTC4 generation	48

4.7.2.5.2	NRTC4 power allocation	49

4.7.2.6	NRTC5: Multi-band test configuration with high PSD per carrier	49

4.7.2.6.1	NRTC5 generation	49

4.7.2.6.2	NRTC5 power allocation	50

4.8	Applicability of requirements	50

4.8.1	Requirement set applicability	50

4.8.2	Applicability of test configurations for single-band RIB	50

4.8.3	Applicability of test configurations for multi-band RIB	51

4.9	RF channels and test models	52

4.9.1	RF channels	52

4.9.2	Test models	53

4.9.2.1	General	53

4.9.2.2	NR FR2 test models	53

4.9.2.2.1	NR FR2 test model 1.1 (NR-FR2-TM1.1)	55

4.9.2.2.2	NR FR2 test model 2 (NR-FR2-TM2)	55

4.9.2.2.3	NR FR2 test model 3.1 (NR-FR2-TM3.1)	56

4.9.2.3	Data content of physical channels and signals for NR-FR2-TM	56

4.9.2.3.1	PDCCH	56

4.9.2.3.2	PDSCH	57

4.10	Requirements for contiguous and non-contiguous spectrum	57

4.11	Requirements for BS capable of multi-band operation	58

4.12	Co-location requirements	58

4.12.1	General	58

4.12.2	Co-location test antenna	59

4.12.2.1	General	59

4.12.2.2	Co-location test antenna characteristics	59

4.12.2.3	Co-location test antenna alignment	59

4.13	Format and interpretation of tests	60

4.14	Reference coordinate system	61

5	Operating bands and channel arrangement	63

6	Radiated transmitter characteristics	64

6.1	General	64

6.2	Radiated transmit power	64

6.2.1	Definition and applicability	64

6.2.2	Minimum requirement	64

6.2.3	Test purpose	65

6.2.4	Method of test	65

6.2.4.1	Initial conditions	65

6.2.4.2	Procedure	65

6.2.5	Test requirement	66

6.3	OTA base station output power	66

6.3.1	Definition and applicability	66

6.3.2	Minimum requirement	66

6.3.3	Test purpose	67

6.3.4	Method of test	67

6.3.4.1	Initial conditions	67

6.3.4.2	Procedure	67

6.3.5	Test requirement	68

6.3.5.1	BS type 1-O	68

6.3.5.2	BS type 2-O	68

6.4	OTA output power dynamics	68

6.4.1	General	68

6.4.2	OTA RE power control dynamic range	68

6.4.2.1	Definition and applicability	68

6.4.2.2	Minimum requirement	68

6.4.2.3	Method of test	68

6.4.3	OTA total power dynamic range	69

6.4.3.1	Definition and applicability	69

6.4.3.2	Minimum requirement	69

6.4.3.3	Test purpose	69

6.4.3.4	Method of test	69

6.4.3.4.1	Initial conditions	69

6.4.3.4.2	Procedure	69

6.4.3.5	Test requirement	70

6.4.3.5.1	BS type 1-O	70

6.4.3.5.2	BS type 2-O	71

6.5	OTA transmit ON/OFF power	71

6.5.1	OTA transmitter OFF power	71

6.5.1.1	Definition and applicability	71

6.5.1.2	Minimum requirement	72

6.5.1.3	Test purpose	72

6.5.1.4	Method of test	72

6.5.1.5	Test requirements	72

6.5.2	OTA transmitter transient period	72

6.5.2.1	Definition and applicability	72

6.5.2.2	Minimum requirement	72

6.5.2.3	Test purpose	73

6.5.2.4	Method of test	73

6.5.2.4.1	Initial conditions	73

6.5.2.4.2	Procedure	73

6.5.2.4.2.1	General procedure	73

6.5.2.4.2.2	BS type 1-O	73

6.5.2.4.2.3	BS type 2-O	74

6.5.2.5	Test requirements	74

6.5.2.5.1	BS type 1-O	74

6.5.2.5.2	BS type 2-O	74

6.6	OTA transmitted signal quality	75

6.6.1	General	75

6.6.2	OTA frequency error	75

6.6.2.1	Definition and applicability	75

6.6.2.2	Minimum Requirement	75

6.6.2.3	Test purpose	75

6.6.2.4	Method of test	75

6.6.2.4.1	Initial conditions	75

6.6.2.5	Test Requirements	75

6.6.3	OTA modulation quality	75

6.6.3.1	Definition and applicability	75

6.6.3.2	Minimum Requirement	76

6.6.3.3	Test purpose	76

6.6.3.4	Method of test	76

6.6.3.4.1	Initial conditions	76

6.6.3.4.2	Procedure	76

6.6.3.5	Test requirements	77

6.6.3.5.1	BS type 1-O	77

6.6.3.5.2	BS type 2-O	79

6.6.4	OTA time alignment error	80

6.6.4.1	Definition and applicability	80

6.6.4.2	Minimum requirement	80

6.6.4.3	Test purpose	80

6.6.4.4	Method of test	80

6.6.4.4.1	Initial conditions	80

6.6.4.4.2	Procedure	81

6.6.4.5	Test Requirement	81

6.6.4.5.1	BS type 1-O	81

6.6.4.5.2	BS type 2-O	81

6.7	OTA unwanted emissions	82

6.7.1	General	82

6.7.2	OTA occupied bandwidth	82

6.7.2.1	Definition and applicability	82

6.7.2.2	Minimum requirement	82

6.7.2.3	Test purpose	83

6.7.2.4	Method of test	83

6.7.2.4.1	Initial conditions	83

6.7.2.4.2	Procedure	83

6.7.2.5	Test requirement	84

6.7.2.5.1	BS type 1-O	84

6.7.2.5.2	BS type 2-O	85

6.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)	85

6.7.3.1	Definition and applicability	85

6.7.3.2	Minimum requirement	85

6.7.3.3	Test purpose	85

6.7.3.4	Method of test	85

6.7.3.4.1	Initial conditions	85

6.7.3.4.2	Procedure	85

6.7.3.5	Test requirements	86

6.7.3.5.1	BS type 1-O	86

6.7.3.5.2	BS type 2-O	89

6.7.4	OTA operating band unwanted emissions	92

6.7.4.1	Definition and applicability	92

6.7.4.2	Minimum requirement	92

6.7.4.3	Test purpose	92

6.7.4.4	Method of test	92

6.7.4.4.1	Initial conditions	92

6.7.4.4.2	Procedure	92

6.7.4.5	Test requirements	93

6.7.4.5.1	BS type 1-O	93

6.7.4.5.1.1	Wide Area BS (Category A)	94

6.7.4.5.1.2	Wide Area BS Category B (Option 1)	96

6.7.4.5.1.3	Wide Area BS Category B (Option 2)	98

6.7.4.5.1.4	Medium Range BS (Category A and Category B)	99

6.7.4.5.1.5	Local Area BS (Category A and Category B)	101

6.7.4.5.1.6	Additional requirements	103

6.7.4.5.1.6.1	Limits in FCC Title 47	103

6.7.4.5.1.6.2	Protection of DTT	103

6.7.4.5.2	BS type 2-O	104

6.7.4.5.2.1	General	104

6.7.4.5.2.2	OTA operating band unwanted emission limits (Category A)	104

6.7.4.5.2.3	OTA operating band unwanted emission limits (Category B)	105

6.7.5	OTA transmitter spurious emissions	105

6.7.5.1	General	105

6.7.5.2	General OTA transmitter spurious emissions requirements	106

6.7.5.2.1	Definition and applicability	106

6.7.5.2.2	Minimum requirement	106

6.7.5.2.3	Test purpose	106

6.7.5.2.4	Method of test	106

6.7.5.2.4.1	Initial conditions	106

6.7.5.2.4.2	Procedure	107

6.7.5.2.5	Test requirement	108

6.7.5.2.5.1	Test requirement for BS type 1-O	108

6.7.5.2.5.2	Test requirement for BS type 2-O	108

6.7.5.2.5.2.1	General	108

6.7.5.2.5.2.2	OTA transmitter spurious emissions (Category A)	109

6.7.5.2.5.2.3	OTA transmitter spurious emissions (Category B)	109

6.7.5.3	Protection of the BS receiver of own or different BS	109

6.7.5.3.1	Definition and applicability	109

6.7.5.3.2	Minimum requirements	110

6.7.5.3.3	Test purpose	110

6.7.5.3.4	Method of test	110

6.7.5.3.4.1	Initial conditions	110

6.7.5.3.4.2	Procedure	110

6.7.5.3.5	Test requirements	111

6.7.5.3.5.1	Test requirement for BS type 1-O	111

6.7.5.4	Additional spurious emissions requirements	111

6.7.5.4.1	Definition and applicability	111

6.7.5.4.2	Minimum Requirement	111

6.7.5.4.3	Test purpose	112

6.7.5.4.4	Method of test	112

6.7.5.4.4.1	Initial conditions	112

6.7.5.4.4.2	Procedure	112

6.7.5.4.5	Test requirement	113

6.7.5.4.5.1	Test requirement for BS type 1-O	113

6.7.5.5	Co-location requirements	119

6.7.5.5.1	Definition and applicability	119

6.7.5.5.2	Minimum requirements	119

6.7.5.5.3	Test purpose	119

6.7.5.5.4	Method of test	119

6.7.5.5.4.1	Initial conditions	119

6.7.5.5.4.2	Procedure	119

6.7.5.5.5	Test requirements	120

6.7.5.5.5.1	Test requirement for BS type 1-O	120

6.8	OTA transmitter intermodulation	124

6.8.1	Definition and applicability	124

6.8.2	Minimum requirement	125

6.8.3	Test purpose	125

6.8.4	Method of test	125

6.8.4.1	Initial conditions	125

6.8.4.2	Procedure	125

6.8.5	Test requirements	127

6.8.5.1	Requirement for BS type 1-O	127

7	Radiated receiver characteristics	128

7.1	General	128

7.2	OTA sensitivity	128

7.2.1	Definition and applicability	128

7.2.2	Minimum requirement	129

7.2.3	Test Purpose	129

7.2.4	Method of test	129

7.2.4.1	Initial conditions	129

7.2.4.2	Procedure	129

7.2.5	Test requirements	130

7.2.5.1	General	130

7.2.5.2	Test requirements for BS type 1-H and BS type 1-O	130

7.2.5.3	Test requirements for BS type 2-O	130

7.3	OTA reference sensitivity level	130

7.3.1	Definition and applicability	130

7.3.2	Minimum requirement	131

7.3.3	Test Purpose	131

7.3.4	Method of test	131

7.3.4.1	Initial conditions	131

7.3.4.2	Procedure	131

7.3.5	Test requirements	132

7.3.5.1	General	132

7.3.5.2	Test requirements for BS type 1-O	132

7.3.5.3	Test requirements for BS type 2-O	133

7.4	OTA dynamic range	134

7.4.1	Definition and applicability	134

7.4.2	Minimum requirement	134

7.4.3	Test purpose	134

7.4.4	Method of test	134

7.4.4.1	Initial conditions	134

7.4.4.2	Procedure	135

7.4.5	Test requirement	135

7.4.5.1	General	135

7.4.5.2	Test requirements for BS type 1-O	135

7.5	OTA in-band selectivity and blocking	144

7.5.1	OTA adjacent channel selectivity	144

7.5.1.1	Definition and applicability	144

7.5.1.2	Minimum requirement	144

7.5.1.3	Test purpose	144

7.5.1.4	Method of test	144

7.5.1.4.1	Initial conditions	144

7.5.1.4.2	Procedure	144

7.5.1.5	Test requirement	145

7.5.1.5.1	General	145

7.5.1.5.2	Test requirements for BS type 1-O	145

7.5.1.5.3	Test requirements for BS type 2-O	146

7.5.2	OTA in-band blocking	147

7.5.2.1	Definition and applicability	147

7.5.2.2	Minimum requirement	147

7.5.2.3	Test purpose	147

7.5.2.4	Method of test	147

7.5.2.4.1	Initial conditions	147

7.5.2.4.2	Procedure	148

7.5.2.5	Test requirement	148

7.5.2.5.1	General	148

7.5.2.5.2	Test requirements for BS type 1-O	149

7.5.2.5.3	Test requirements for BS type 2-O	152

7.6	OTA out-of-band blocking	153

7.6.1	Definition and applicability	153

7.6.2	Minimum requirement	153

7.6.3	Test purpose	153

7.6.4	Method of test	153

7.6.4.1	Initial conditions	153

7.6.4.2	Procedure	154

7.6.4.2.1	BS type 1-O procedure for out-of-band blocking	154

7.6.4.2.2	BS type 1-O procedure for co-location blocking	154

7.6.4.2.3	BS type 2-O procedure for out-of-band blocking	155

7.6.5	Test requirements	156

7.6.5.1	Requirement for BS type 1-O	156

7.6.5.1.1	General	156

7.6.5.1.2	Co-location requirement	156

7.6.5.2	Requirement for BS type 2-O	157

7.6.5.2.1	General requirement	157

7.7	OTA receiver spurious emissions	157

7.7.1	Definition and applicability	157

7.7.2	Minimum requirement	158

7.7.3	Test purpose	158

7.7.4	Method of test	158

7.7.4.1	Initial conditions	158

7.7.4.2	Procedure	159

7.7.5	Test requirement	159

7.7.5.1	Test requirement for BS type 1-O	159

7.7.5.2	Test requirement for BS type 2-O	160

7.8	OTA receiver intermodulation	160

7.8.1	Definition and applicability	160

7.8.2	Minimum requirement	160

7.8.3	Test purpose	161

7.8.4	Method of test	161

7.8.4.1	Initial conditions	161

7.8.4.2	Procedure	161

7.8.5	Test requirement	162

7.8.5.1	BS type 1-O	162

7.8.5.2	BS type 2-O	166

7.9	OTA in-channel selectivity	166

7.9.1	Definition and applicability	166

7.9.2	Minimum requirement	166

7.9.3	Test purpose	167

7.9.4	Method of test	167

7.9.4.1	Initial conditions	167

7.9.4.2	Procedure	167

7.9.5	Test requirement	168

7.9.5.1	BS type 1-O	168

7.9.5.2	BS type 2-O	170

8	Radiated performance requirements	171

8.1	General	171

8.1.0	Scope and definitions	171

8.1.1	OTA demodulation branches	171

8.1.2	Applicability rule	172

8.1.2.0	General	172

8.1.2.1	Applicability of PUSCH performance requirements	172

8.1.2.1.1	Applicability of requirements for different subcarrier spacings	172

8.1.2.1.2	Applicability of requirements for different channel bandwidths	172

8.1.2.1.3	Applicability of requirements for different configurations	172

8.1.2.2	Applicability of PUCCH performance requirements	172

8.1.2.2.1	Applicability of requirements for different formats	172

8.1.2.2.2	Applicability of requirements for different subcarrier spacings	172

8.1.2.2.3	Applicability of requirements for different channel bandwidths	172

8.1.2.2.4	Applicability of requirements for different configurations	173

8.1.2.2.5	Applicability of requirements for multi-slot PUCCH	173

8.1.2.3	Applicability of PRACH performance requirements	173

8.1.2.3.1	Applicability of requirements for different formats	173

8.1.2.3.2	Applicability of requirements for different subcarrier spacings	173

8.1.2.3.3	Applicability of requirements for different channel bandwidths	173

8.2	OTA performance requirements for PUSCH	173

8.2.1	Performance requirements for PUSCH with transform precoding disabled	173

8.2.1.1	Definition and applicability	173

8.2.1.2	Minimum Requirement	173

8.2.1.3	Test purpose	173

8.2.1.4	Method of test	174

8.2.1.4.1	Initial conditions	174

8.2.1.4.2	Procedure	174

8.2.1.5	Test Requirement	175

8.2.1.5.1	Test requirement for BS type 1-O	175

8.2.1.5.2	Test requirement for BS type 2-O	178

8.2.2	Performance requirements for PUSCH with transform precoding enabled	182

8.2.2.1	Definition and applicability	182

8.2.2.2	Minimum Requirement	182

8.2.2.3	Test Purpose	182

8.2.2.4	Method of test	182

8.2.2.4.1	Initial Conditions	182

8.2.2.4.2	Procedure	182

8.2.2.5	Test Requirement	184

8.2.2.5.1	Test requirement for BS type 1-O	184

8.2.2.5.2	Test requirement for BS type 2-O	184

8.2.3	Performance requirements for UCI multiplexed on PUSCH	185

8.2.3.1	Definition and applicability	185

8.2.3.2	Minimum Requirement	185

8.2.3.3	Test purpose	186

8.2.3.4	Method of test	186

8.2.3.4.1	Initial conditions	186

8.2.3.4.2	Procedure	186

8.2.3.5	Test Requirement	188

8.2.3.5.1	Test requirement for BS type 1-O	188

8.2.3.5.2	Test requirement for BS type 2-O	189

8.3	OTA performance requirements for PUCCH	190

8.3.1	Performance requirements for PUCCH format 0	190

8.3.1.1	Definition and applicability	190

8.3.1.2	Minimum Requirement	190

8.3.1.3	Test purpose	191

8.3.1.4	Method of test	191

8.3.1.4.1	Initial conditions	191

8.3.1.4.2	Procedure	191

8.3.1.5	Test Requirement	192

8.3.1.5.1	Test requirement for BS type 1-O	192

8.3.1.5.2	Test requirement for BS type 2-O	192

8.3.2	Performance requirements for PUCCH format 1	193

8.3.2.1	NACK to ACK detection	193

8.3.2.1.1	Definition and applicability	193

8.3.2.1.2	Minimum Requirement	193

8.3.2.1.3	Test purpose	193

8.3.2.1.4	Method of test	193

8.3.2.1.4.1	Initial Conditions	193

8.3.2.1.4.2	Procedure	194

8.3.2.1.5	Test Requirement	195

8.3.2.1.5.1	Test Requirement for BS type 1-O	195

8.3.2.1.5.2	Test Requirement for BS type 2-O	195

8.3.2.2	ACK missed detection	196

8.3.2.2.1	Definition and applicability	196

8.3.2.2.2	Minimum Requirement	196

8.3.2.2.3	Test purpose	196

8.3.2.2.4	Method of test	196

8.3.2.2.4.1	Initial Conditions	196

8.3.2.2.4.2	Procedure	196

8.3.2.2.5	Test Requirement	198

8.3.2.1.5.1	Test Requirement for BS type 1-O	198

8.3.2.2.5.2	Test Requirement for BS type 2-O	198

8.3.3	Performance requirements for PUCCH format 2	198

8.3.3.1	ACK missed detection performance requirements	198

8.3.3.1.1	Definition and applicability	198

8.3.3.1.2	Minimum Requirement	199

8.3.3.1.3	Test Purpose	199

8.3.3.1.4	Method of test	199

8.3.3.1.4.1	Initial conditions	199

8.3.3.1.4.2	Procedure	199

8.3.3.1.5	Test requirement	200

8.3.3.1.5.1	Requirements for BS type 1-O	200

8.3.3.1.5.2	Requirements for BS type 2-O	201

8.3.3.2	UCI BLER performance requirements	201

8.3.3.2.1	Definition and applicability	201

8.3.3.2.2	Minimum Requirement	201

8.3.3.2.3	Test Purpose	201

8.3.3.2.4	Method of test	201

8.3.3.2.4.1	Initial conditions	201

8.3.3.2.4.2	Procedure	202

8.3.3.2.5	Test requirement	203

8.3.3.2.5.1	Requirements for BS type 1-O	203

8.3.3.2.5.2	Requirements for BS type 2-O	203

8.3.4	Performance requirements for PUCCH format 3	204

8.3.4.1	Definition and applicability	204

8.3.4.2	Minimum requirement	204

8.3.4.3	Test purpose	204

8.3.4.4	Method of test	204

8.3.4.4.1	Initial conditions	204

8.3.4.4.2	Procedure	205

8.3.4.5	Test requirement	206

8.3.4.5.1	Test requirement for BS type 1-O	206

8.3.4.5.2	Test requirement for BS type 2-O	206

8.3.5	Performance requirements for PUCCH format 4	207

8.3.5.1	Definition and applicability	207

8.3.5.2	Minimum requirement	207

8.3.5.3	Test purpose	207

8.3.5.4	Method of test	207

8.3.5.4.1	Initial conditions	207

8.3.5.4.2	Procedure	207

8.3.5.5	Test requirement	209

8.3.5.5.1	Test requirement for BS type 1-O	209

8.3.5.5.2	Test requirement for BS type 2-O	209

8.4	OTA performance requirements for PRACH	210

8.4.1	PRACH false alarm probability and missed detection	210

8.4.1.1	Definition and applicability	210

8.4.1.2	Minimum requirement	210

8.4.1.3	Test purpose	210

8.4.1.4	Method of test	210

8.4.1.4.1	Initial conditions	210

8.4.1.4.2	Procedure	211

8.4.1.5	Test requirement	212

8.4.1.5.1	Test requirement for BS type 1-O	212

8.4.1.5.2	Test requirement for BS type 2-O	213

Annex A (normative):	Reference measurement channels	214

A.1	Fixed Reference Channels for OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity (QPSK, R=1/3)	214

A.2	Fixed Reference Channels for OTA dynamic range (16QAM, R=2/3)	215

A.3	Fixed Reference Channels for performance requirements (QPSK, R=193/1024)	216

A.4	Fixed Reference Channels for performance requirements (16QAM, R=658/1024)	221

A.5	Fixed Reference Channels for performance requirements (64QAM, R=567/1024)	224

A.6	PRACH Test preambles	226

Annex B (normative):	Environmental requirements for the BS equipment	227

B.1	General	227

B.2	Normal test environment	227

B.3	Extreme test environment	227

B.3.1	Extreme temperature	227

B.4	Vibration	228

B.5	Power supply	228

B.6	Measurement of test environments	228

B.7	OTA extreme test methods	229

B.7.1	Direct far field method	229

B.7.2	Relative method	229

Annex C (informative):	Test tolerances and derivation of test requirements	231

C.1	Measurement of transmitter	232

C.2	Measurement of receiver	235

C.3	Measurement of performance requirements	237

Annex D (normative):	Calibration	238

Annex E (informative):	OTA measurement system set-up	239

E.1	Transmitter	239

E.1.1	Radiated transmit power, OTA output power dynamics, OTA transmitted signal quality, OTA occupied bandwidth, and OTA transmit ON/OFF power (BS type 2-O)	239

E.1.2	OTA base station output power, OTA ACLR, OTA operating band unwanted emissions	240

E.1.3	OTA spurious emissions	240

E.1.4	OTA co-location emissions, OTA transmit ON/OFF power (BS type 1-O)	241

E.1.5	OTA transmitter intermodulation	242

E.2	Receiver	243

E.2.1	OTA sensitivity and OTA reference sensitivity level	243

E.2.2	OTA dynamic range	243

E.2.3	OTA adjacent channel selectivity, general OTA blocking, and OTA narrowband blocking	244

E.2.4	OTA blocking	245

E.2.4.1	General OTA out-of-band blocking	245

E.2.4.2	OTA co-location blocking	246

E.2.5	OTA receiver spurious emissions	246

E.2.6	OTA receiver intermodulation	247

E.2.7	OTA in-channel selectivity	247

E.3	Performance requirements	248

Annex F (normative):	Void	249

Annex G (informative):	Transmitter spatial emissions declaration	250

G.1	General	250

G.2	Declarations	250

Annex H (normative):	Characteristics of the interfering signals	252

Annex I (normative):	TRP measurement procedures	253

I.1	General	253

I.2	Spherical equal angle grid	253

I.2.1	General	253

I.2.2	Reference angular step criteria	253

I.3	Spherical equal area grid	255

I.4	Spherical Fibonacci grid	255

I.5	Orthogonal cut grid	256

I.6	Wave vector space grid	257

I.7	Orthogonal 2 cuts with pattern multiplication	257

I.8	Orthogonal 2 or 3 cut with dense sampling	258

I.8.1	Operating band unwanted emissions	258

I.8.2	Spurious unwanted emissions	258

I.9	Full sphere with sparse sampling	258

I.10 Beam-based directions	259

I.11	Peak method	259

I.12	Equal sector with peak average	259

I.13 Pre-scan	259

Annex J (normative):	Propagation conditions	261

J.1	Static propagation condition	261

J.2	Multi-path fading propagation conditions	261

J.2.1	Delay profiles	261

J.2.1.1	Delay profiles for FR1	262

J.2.1.2	Delay profiles for FR2	263

J.2.2	Combinations of channel model parameters	264

J.2.3	MIMO channel correlation matrices	264

J.2.3.1	MIMO correlation matrices using Uniform Linear Array	264

J.2.3.1.1	Definition of MIMO correlation matrices	264

J.2.3.1.2	MIMO correlation matrices at high, medium and low level	266

J.2.3.2	Multi-antenna channel models using cross polarized antennas	268

J.2.3.2.1	Definition of MIMO correlation matrices using cross polarized antennas	268

J.2.3.2.2	Spatial correlation matrices at UE and gNB sides	269

J.2.3.2.2.1	Spatial correlation matrices at UE side	269

J.2.3.2.2.2	Spatial correlation matrices at gNB side	269

J.2.3.2.3	MIMO correlation matrices using cross polarized antennas	269

Annex K (informative):	Measuring noise close to noise-floor	271

Annex L (normative):	In-channel TX tests	272

L.1	General	272

L.2	Basic principles	272

L.2.1	Output signal of the TX under test	272

L.2.2	Ideal signal	272

L.2.3	Measurement results	273

L.2.4	Measurement points	273

L.3	Pre-FFT minimization process	274

L.4	Timing of the FFT window	275

L.5	Resource element TX power	276

L.6	Post-FFT equalisation	276

L.7	EVM	278

L.7.1	Averaged EVM (FDD)	278

L.7.2	Averaged EVM (TDD)	279

Annex M (informative):	Change history	280

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the Radio Frequency (RF) test methods and conformance requirements for NR Base Station (BS) type 1-H, BS type 1-O and BS type 2-O. These have been derived from, and are consistent with the radiated requirements for BS type 1-H, BS type 1-O and BS type 2-O in BS specification defined in TS 38.104 [2].

A BS type 1-C only has conducted requirements so it does not require compliance to this specification.

A BS type 1-H has both conducted and radiated requirements so it requires compliance to the applicable requirements of this specification and TS 38.141-1 [3].

BS type 1-O and BS type 2-O have only radiated requirements so they require compliance to this specification only.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.104: "NR Base Station (BS) radio transmission and reception"

[3]	3GPP TS 38.141-1: "NR, Base Station (BS) conformance testing, Part 1: Conducted conformance testing"

[4]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[6]	3GPP TR 37.842: "E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)"

[7]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[8]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Section 4: Stationary use at non-weather protected locations"

[9]	IEC 60 721: "Classification of environmental conditions"

[10]	IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[11]	IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[12]	IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[13]	Recommendation ITU-R M.328: "Spectra and bandwidth of emissions"

[14]	FCC publication number 662911: "Emissions Testing of Transmitters with Multiple Outputs in the Same Band".

[15]	ECC/DEC/(17)06: "The harmonised use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)"

[16]	3GPP TR 37.843: "E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) radiated requirements"

[17]	3GPP TR 38.817-02: "NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR"

[18]	3GPP TS 36.104: "E-UTRA; Base Station (BS) radio transmission and reception"

[19]	3GPP TS 38.212: "NR; Multiplexing and channel coding"

[20]	3GPP TS 38.211: "NR; Physical channels and modulation"

[21]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[23]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

[24]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[25]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[26]	ERC Recommendation 74-01: "Unwanted emissions in the spurious domain"

[27]	3GPP TR 25.942: "RF system scenarios".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

aggregated BS channel bandwidth: the RF bandwidth in which a Base Station transmits and receives multiple contiguously aggregated carriers. The aggregated BS channel bandwidth is measured in MHz

antenna connector: connector at the conducted interface of the BS type 1-C

Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [5] used for the formulation of unwanted emission requirements for FR1

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain BS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is found

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink

NOTE 1:	The BS channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	It is possible for the BS to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the BS transmission bandwidth configuration, in any part of the BS transmission bandwidth configuration.

BS receiver: composite receiver function of a BS receiving in an operating band

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

channel edge: lowest or highest frequency of the NR carrier, separated by the BS channel bandwidth

carrier aggregation: aggregation of two or more component carriers in order to support wider transmission bandwidths

carrier aggregation configuration: a set of one or more operating bands across which the BS aggregates carriers with a specific set of technical requirements

co-location reference antenna: a passive antenna used as reference for base station to base station co-location requirements

contiguous carriers: set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block

contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s)

demodulation branch: single input of the BS receiver to the demodulation algorithms

directional requirement: requirement which is applied in a specific direction within the OTA coverage range for the Tx and when the AoA of the incident wave of a received signal is within the FR1 OTA REFSENS RoAoA or FR2 OTA REFSENS RoAoA or the minSENS RoAoA as appropriate for the receiver

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

NOTE:	Isotropic directivity is equal in all directions (0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

NOTE 1:	The sensitivity is the minimum received power level at which specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (0 dBi).

fractional bandwidth: fractional bandwidth FBW is defined in percent as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%

highest carrier: the carrier with the highest carrier frequency transmitted/received in a specified frequency band

inter-band carrier aggregation: carrier aggregation of component carriers in different operating bands

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

intra-band contiguous carrier aggregation: contiguous carriers aggregated in the same operating band

intra-band non-contiguous carrier aggregation: non-contiguous carriers aggregated in the same operating band

Inter-band gap: The frequency gap between two supported consecutive operating bands

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

lowest Carrier: the carrier with the lowest carrier frequency transmitted/received in a specified frequency band

lower sub-block edge: frequency at the lower edge of one sub-block

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

maximum carrier TRP: mean power level measured per RIB during the transmitter ON period for a specific carrier in a specified reference condition and corresponding to the declared rated carrier TRP (Prated,c,TRP)

maximum transmitter TRP: mean power level measured per RIB during the transmitter ON period in a specified reference condition and corresponding to the declared rated transmitter TRP (Prated,t,TRP)

measurement bandwidth: RF bandwidth in which an emission level is specified

minSENS: the lowest declared EIS value for the OSDD’s declared for OTA sensitivity requirement.

minSENS RoAoA: The reference RoAoA associated with the OSDD with the lowest declared EIS

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

multi-carrier transmission configuration: set of one or more contiguous or non-contiguous carriers that a BS is able to transmit simultaneously according to the manufacturer’s specification

non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s)

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

NOTE:	The operating band(s) for a BS is declared by the manufacturer according to the designations in TS 38.104 [2].

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE:     The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

OTA REFSENS RoAoA: Is the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction assuming that for any AoA, the receiver gain is optimized for that AoA

NOTE:	This contour will be related to the average element/sub-array radiation pattern 3dB beam width.

OTA sensitivity directions declaration: set of manufacturer declarations comprising at least one set of declared minimum EIS values (with BS channel bandwidth), and related directions over which the EIS applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

Radio Bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier

rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period

rated carrier TRP: mean power level declared by the manufacturer per carrier, for BS operating in single carrier, multi-carrier, or carrier aggregation configurations that the manufacturer has declared to be available at the RIB during the transmitter ON period

rated transmitter TRP: mean power level declared by the manufacturer, that the manufacturer has declared to be available at the RIB during the transmitter ON period

reference beam direction pair: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the OTA peak directions set

receiver target: AoA in which reception is performed by BS types 1-H, BS type 1-O and BS types 2-O

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to particular OSDD

receiver target reference direction: direction inside the OTA sensitivity directions declaration declared by the manufacturer for conformance testing. For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA

reference RoAoA: the sensitivity RoAoA associated with the receiver target reference direction for each OSDD

requirement set:	one of the NR base station requirement’s set as defined for BS type 1-C, BS type 1-H, BS type 1-O, and BS type 2-O

sensitivity RoAoA: RoAoA within the OTA sensitivity directions declaration, within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific BS direction setting

single-band RIB: operating band specific RIB supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band RIB

sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band

sub-block: one contiguous allocated block of spectrum for transmission and reception by the same base station

NOTE:	There may be multiple instances of sub-blocks within a Base Station RF Bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within a Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band

TAB connector: transceiver array boundary connector

total radiated power: the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations. Total radiated power is defined in both the near-field region and the far-field region.

total RF bandwidth: maximum sum of Base Station RF Bandwidths in all supported operating bands

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: RF Bandwidth of an instantaneous transmission from a UE or BS, measured in resource block units

transmitter OFF period: time period during which the BS transmitter is not allowed to transmit

transmitter ON period: time period during which the BS transmitter is transmitting data and/or reference symbols

transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa

upper sub-block edge: frequency at the upper edge of one sub-block

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BeWθ	The beam width in θ

BeWφ	The beam width in ϕ

BeWθ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees, applicable for FR1 only

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees, applicable for FR1 only

BWChannel	BS channel bandwidth

BWChannel_CA	Aggregated BS channel bandwidth, expressed in MHz. BWChannel_CA = Fedge_high- Fedge_low

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12

BWContiguous	Contiguous transmission bandwidth, i.e. BS channel bandwidth for single carrier or aggregated BS channel bandwidth for contiguously aggregated carriers. For non-contiguous operation within a band the term is applied per sub-block.

BWtot	Total RF bandwidth

f	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔFGlobal	Global frequency raster granularity

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔFR2_REFSENS	Offset applied to the FR2 OTA REFSENS depending on the AoA

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔminSENS	Difference between conducted reference sensitivity and EISminsens

ΔOTAREFSENS	Difference between conducted reference sensitivity and OTA REFSENS

ΔFRaster	Channel raster granularity

Δsample	The difference between the nominal and extreme power measurements during extreme EIRP testing, Pmax,sample,nom - Pmax,sample,ex

ΔSUL	Channel raster offset for SUL

EISminsens	The EIS declared for the minSENS RoAoA

EISREFSENS	OTA REFSENS EIS value

EISREFSENS_50M	Declared OTA reference sensitivity basis level for FR2 based on a reference measurement channel with 50MHz BS channel bandwidth

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

FC	RF reference frequency on the channel raster

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC_low	The Fc of the lowest carrier, expressed in MHz

FC_high	The Fc of the highest carrier, expressed in MHz

Fedge_low	The lower edge of Aggregated BS channel bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset_low

Fedge_high	The upper edge of Aggregated BS channel bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset_high

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset_low

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset_high

Foffset_high	Frequency offset from FC_high to the upper Base Station RF Bandwidth edge, or from F C,block, high to the upper sub-block edge

Foffset_low	Frequency offset from FC_low to the lower Base Station RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

f_offset	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF,SUL	RF reference frequency for Supplementary Uplink (SUL) bands

Fstep,X	Frequency steps for the OTA transmitter spurious emissions (Category B)

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Iuant	gNB internal logical interface between the implementation specific O&M function and the RET antennas and TMAs control unit function of the gNB

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

NRB	Transmission bandwidth configuration, expressed in resource blocks

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

PEM,n50,ind	Declared emission level for Band n50 in the band 1518-1559 MHz; ind = a, b

Pmax,c,EIRP	The maximum carrier EIRP when the BS is configured at the maximum rated carrier TRP (PRated,c,TRP)

Pmax,c,EIRP, extreme	The maximum carrier EIRP when the BS is configured at the maximum rated carrier TRP (PRated,c,TRP) under extreme conditions, either measured directly or calculated

Pmax,c,TRP	Maximum carrier TRP measured at the RIB(s), and corresponding to the declared rated carrier TRP (Prated,c,TRP)

Pmax,sample,nom	The measured sample power in extreme conditionals chamber when the BS is configured at the rated carrier TRP (Prated,c,TRP), under nominal conditions

Pmax,sample,ext	The measured sample power in extreme conditionals chamber when the BS is configured at the rated carrier TRP (Prated,c,TRP), under extreme conditions

Prated,c,EIRP	The rated carrier EIRP when the BS is configured at the rated carrier TRP (Prated,c,TRP)

Prated,c,FBWhigh	The rated carrier EIRP for the higher supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,FBWlow	The rated carrier EIRP for the lower supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,TRP	Rated carrier TRP declared per RIB

Prated,t,TRP	Rated transmitter TRP declared per RIB

PREFSENS	Conducted reference Sensitivity power level

SSREF	SS block reference frequency position

TRPEstimate	Numerically approximated TRP

TTOTA	Test tolerance for OTA requirements

Wgap	Sub-block gap or Inter RF Bandwidth gap size

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AA	Antenna Array

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

AWGN	Additive White Gaussian Noise

BS	Base Station

BW	Bandwidth

CA	Carrier Aggregation

CACLR	Cumulative ACLR

CLTA	Co-Location Test Antenna

CW	Continuous Wave

DM-RS	Demodulation Reference Signal

DUT	Device Under Test

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

FBW	Fractional Bandwidth

FR	Frequency Range

GSCN	Global Synchronization Channel Number

ICS	In-Channel Selectivity

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

LNA	Low Noise Amplifier

MR	Medium Range

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OBUE	Operating Band Unwanted Emissions

OSDD	OTA Sensitivity Directions Declaration

OTA	Over The Air

PT-RS	Phase Tracking Reference Signal

RDN	Radio Distribution Network

REFSENS	Reference Sensitivity

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RS	Reference Signal

RX	Receiver

RoAoA	Range of Angles of Arrival

SCS	Sub-Carrier Spacing

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TDD	Time Division Duplex

TRP	Total Radiated Power

TT	Test Tolerance

# 4	General radiated test conditions and declarations

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in TS 38.141-2 (the present document), i.e. to all radiated tests defined in FR1 for BS type 1-H, BS type 1-O and radiated tests defined in FR2 for BS type 2-O. The frequency ranges FR1 and FR2 are defined in subclause 5.1 of TS 38.104 [2].

The minimum requirements are given in TS 38.104 [2]. Test Tolerances for the radiated test requirements (TTOTA) explicitly stated in the present document are given in annex C.

Test Tolerances are individually calculated for each test. Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

The requirements are classified according to spatial charactersitics as shown in table 4.1.1-1 and table 4.1.1-2.

Table 4.1.1-1: Overview of radiated Tx requirements

| Tx requirement |  | Classification | Coverage range |  | Number of conformance directions |
| --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 |  |
| Radiated transmit power |  | Directional | OTA peak directions set | OTA peak directions set | 5 |
| OTA BS output power |  | TRP | See annex I |  |  |
| OTA output power dynamics |  | Directional | OTA peak directions set | OTA peak directions set | 1 |
| OTA transmitter OFF power |  | Co-location | See subclause 4.12 | N/A | See subclause 4.12 |
|  |  | Directional | N/A | OTA peak directions set (Note 2) | 1 |
| OTA transient period |  | Co-location | See subclause 4.12 | N/A | See subclause 4.12 |
|  |  | Directional | N/A | OTA peak directions set (Note 2) | 1 |
| OTA modulation quality |  | Directional | OTA coverage range | OTA coverage range | 5 |
| OTA frequency error |  | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA time alignment error |  | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA occupied bandwidth |  | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA ACLR |  | TRP | N/A | N/A | See annex I |
| OTA operating band unwanted emission |  | TRP | N/A | N/A | See annex I |
| OTA transmitter spurious emission | General requirement | TRP | N/A | N/A | See annex I |
|  | Protection of the BS receiver of own or different BS | Co-location | See subclause 4.12 | N/A | See subclause 4.12 |
|  | Additional spurious emissions | TRP | N/A | N/A | See annex I |
|  | Co-location with other base stations | Co-location | See subclause 4.12 | N/A | See subclause 4.12 |
| OTA transmitter intermodulation |  | Co-location | See subclause 4.12 | N/A | See subclause 4.12 |
| NOTE 1: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. NOTE 2: For FR2, RF Core requirements are defined on TRP levels. Conformance requirements are verified by EIRP measurements in the reference direction. |  |  |  |  |  |

Table 4.1.1-2: Overview of radiated Rx requirements

| Rx requirement |  | Classification | Applicability levels |  | Coverage range |  | Number of conformance directions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 | FR1 | FR2 |  |
| OTA sensitivity |  | Directional | Minimum EIS | N/A | OSDD | N/A | 5 |
| OTA reference sensitivity |  | Directional | OTA REFSENS | OTA REFSENS | OTA REFSENS RoAoA |  | 5 |
| OTA Dynamic range |  | Directional | OTA REFSENS | N/A | OTA REFSENS RoAoA | N/A | 1 |
| OTA adjacent channel selectivity |  | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA in-band blocking |  | Directional | OTA REFSENS and minSENS | OTA REFSENS | OTA REFSENS RoAoA and minSENS RoAoA | OTA REFSENS RoAoA | 5 |
| OTA out-of-band blocking | General requirement | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
|  | Co-location with other base stations | Co-location (Note 2) | minSENS | N/A | minSENS RoAoA | N/A | 1 |
| OTA receiver spurious emissions |  | TRP | See sub-clause 7.7 | See sub-clause 7.7 | N/A | N/A | See annex I |
| OTA receiver intermodulation |  | Directional | OTA REFSENS and minSENS | OTA REFSENS | OTA REFSENS RoAoA and minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA in-channel selectivity |  | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| NOTE 1: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. NOTE 2: The compliance direction for co-location blocking is applicable for the wanted signal only but not the interfering signal. |  |  |  |  |  |  |  |

### 4.1.2	Acceptable uncertainty of OTA Test System

#### 4.1.2.1	General

The maximum acceptable uncertainty of the OTA Test System is specified below for each radiated test defined explicitly in the present specification, where appropriate.

The OTA Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the DUT to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95% is the measurement uncertainty tolerance interval for a specific measurement that contains 95% of the performance of a population of test equipment.

For details on measurement uncertainty budget calculation, OTA measurement methodology description (including calibration and measurement stage for each test range), MU budget format and its contributions, refer to TR 37.843 [16].

#### 4.1.2.2	Measurement of transmitter

The maximum OTA Test System uncertainty for OTA transmitter tests minimum requirements are given in tables 4.1.2.2-1 and 4.1.2.2-2. Details for derivation of OTA Test System uncertainty are given in corresponding subclauses in TR 38.817-02 [17].

Table 4.1.2.2-1: Maximum OTA Test System uncertainty for FR1 OTA transmitter tests

| Subclause | Maximum OTA Test System uncertainty |
| --- | --- |
| 6.2 Radiated transmit power | Normal condition: ±1.1 dB, f ≤ 3 GHz ±1.3 dB, 3 GHz < f ≤ 6 GHz |
|  | Extreme condition: ±2.5 dB, f ≤ 3 GHz ±2.6 dB, 3 GHz < f ≤ 6 GHz |
| 6.3 OTA base station output power | ±1.4 dB, f ≤ 3.0 GHz ±1.5 dB, 3.0 GHz < f ≤ 4.2 GHz ±1.5 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 6.4.2 OTA RE power control dynamic range | N/A |
| 6.4.3 OTA total power dynamic range | ±0.4 dB |
| 6.5.1 OTA transmitter OFF power | ±3.4 dB, f ≤ 3.0 GHz ±3.6 dB, 3.0 GHz < f ≤ 6 GHz (NOTE) |
| 6.5.2 OTA transmitter transient period | N/A |
| 6.6.2 OTA frequency error | ±12 Hz |
| 6.6.3 OTA modulation quality | ±1 % |
| 6.6.4 OTA time alignment error | ±25 ns |
| 6.7.2 OTA occupied bandwidth | ±100 kHz, BWChannel 5 MHz, 10 MHz ±300 kHz, BWChannel 15 MHz, 20 MHz, 25 MHz, 30 MHz, 40 MHz, 50 MHz ±600 kHz, BWChannel 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz |
| 6.7.3 OTA ACLR/CACLR | f ≤ 3.0 GHz ±1 dB, BW ≤ 20MHz ±1 dB, BW > 20MHz 3.0 GHz < f ≤ 6.0 GHz ±1.2 dB, BW ≤ 20MHz ±1.2 dB, BW > 20MHz Absolute power ±2.2 dB, f ≤ 3.0 GHz Absolute power ±2.7 dB, 3.0 GHz < f ≤ 4.2 GHz Absolute power ±2.7 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 6.7.4 OTA operating band unwanted emissions | Absolute power ±1.8 dB, f ≤ 3.0 GHz Absolute power ±2 dB, 3.0 GHz < f ≤ 4.2 GHz Absolute power ±2 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 6.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz < f ≤ 6 GHz ±4.2 dB, 6 GHz < f ≤ 26 GHz |
| 6.7.5.3 OTA transmitter spurious emissions, protection of BS receiver | ±3.1 dB, f ≤ 3 GHz ±3.3 dB, 3 GHz < f ≤ 4.2 GHz ±3.4, 4.2 GHz < f ≤ 6 GHz (NOTE) |
| 6.7.5.4 OTA transmitter spurious emissions, additional spurious emissions requirements | ±2.6 dB, f ≤ 3 GHz ±3.0, 3 GHz < f ≤ 4.2 GHz ±3.5, 4.2 GHz < f ≤ 6 GHz |
| 6.7.5.5 OTA transmitter spurious emissions, co-location | ±3.1 dB, f ≤ 3 GHz ±3.3 dB, 3 GHz < f ≤ 4.2 GHz ±3.4, 4.2 GHz < f ≤ 6 GHz (NOTE) |
| 6.8 OTA transmitter intermodulation | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests (6.6.1, 6.6.2 and 6.6.4) which have to be carried out in the presence of the interferer. ±3.2 dB, f ≤ 3.0 GHz ±3.4 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.5 dB, 4.2 GHz < f ≤ 6 GHz (NOTE) |
| NOTE: Fulfilling the criteria for CLTA selection and placement in subclause 4.12 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate measurement uncertainty budget in TR 37.843 [16], subclause 10.6 shall be used for evaluating the test system uncertainty. |  |

Table 4.1.2.2-2: Maximum OTA Test System uncertainty for FR2 OTA transmitter tests

| Subclause | Maximum OTA Test System uncertainty |
| --- | --- |
| 6.2 Radiated transmit power | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 40 GHz) |
|  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 40 GHz) |
| 6.3 OTA base station output power | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 40 GHz) |
| 6.4.2 OTA RE power control dynamic range | N/A |
| 6.4.3 OTA total power dynamic range | ±0.4 dB |
| 6.5.1 OTA transmitter OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 40 GHz) |
| 6.5.2 OTA transmitter transient period | N/A |
| 6.6.2 OTA frequency error | ±12 Hz |
| 6.6.3 OTA modulation quality | 1% |
| 6.6.4 OTA time alignment error | ±25 ns |
| 6.7.2 OTA occupied bandwidth | [600] kHz |
| 6.7.3 OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 40 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 40 GHz) |
| 6.7.4 OTA operating band unwanted emissions | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 40 GHz) |
| 6.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |
| 6.7.5.4 OTA transmitter spurious emissions, additional spurious emissions requirements | FFS |

#### 4.1.2.3	Measurement of receiver

The maximum OTA Test System uncertainty for OTA receiver tests minimum requirements are given in tables 4.1.2.3-1 and 4.1.2.3-2. Details for derivation of OTA Test System uncertainty are given in corresponding subclauses in TR 38.817-02 [17].

Table 4.1.2.3-1: Maximum OTA Test System uncertainty for FR1 OTA receiver tests

| Subclause | Maximum OTA Test System uncertainty |
| --- | --- |
| 7.2 OTA sensitivity | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz ±1.6 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 7.3 OTA reference sensitivity level | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz ±1.6 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 7.4 OTA dynamic range | ±0.3 dB |
| 7.5.1 OTA adjacent channel selectivity | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.4 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 7.5.2 In-band blocking (General) | ±1.9 dB, f ≤ 3.0 GHz ±2.2 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.5 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 7.5.2 In-band blocking (Narrowband) | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.4 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 7.6 OTA out-of-band blocking (General) | fwanted ≤ 3.0 GHz: ±2.0 dB, finterferer ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < finterferer ≤ 6.0 GHz ±3.5 dB, 6.0 GHz < finterferer ≤ 12.75 GHz 3 GHz < fwanted ≤ 4.2 GHz: ±2.0 dB, finterferer ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < finterferer ≤ 6.0 GHz ±3.6 dB, 6.0 GHz < finterferer ≤ 12.75 GHz 4.2 GHz < fwanted ≤ 6.0 GHz: ±2.2 dB, finterferer ≤ 3.0 GHz ±2.3 dB, 3.0 GHz < finterferer ≤ 6.0 GHz ±3.6 dB, 6.0 GHz < finterferer ≤ 12.75 GHz |
| 7.6 OTA out-of-band blocking (Co-location) (NOTE) | fwanted ≤ 3.0 GHz: ±3.4 dB, finterferer ≤ 3.0 GHz ±3.5 dB, 3.0 GHz < finterferer ≤ 4.2 GHz ±3.7 dB, 4.2 GHz < finterferer ≤ 6.0 GHz 3 GHz < fwanted ≤ 4.2 GHz: ±3.5 dB, finterferer ≤ 3.0 GHz ±3.6 dB, 3.0 GHz < finterferer ≤ 4.2 GHz ±3.7 dB, 4.2 GHz < finterferer ≤ 6.0 GHz 4.2 GHz < fwanted ≤ 6.0 GHz: ±3.6 dB, finterferer ≤ 3.0 GHz ±3.7 dB, 3.0 GHz < finterferer ≤ 4.2 GHz ±3.8 dB, 4.2 GHz < finterferer ≤ 6.0 GHz |
| 7.7 OTA receiver spurious emissions | ±2.5 dB, 30 MHz ≤ f ≤ 6.0 GHz ±4.2 dB, 6.0 GHz < f ≤ 26 GHz |
| 7.8 OTA receiver intermodulation | ±2.0 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.2 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 7.9 OTA in-channel selectivity | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.4 dB, 4.2 GHz < f ≤ 6.0 GHz |
| NOTE: Fulfilling the criteria for CLTA selection and placement in subclause 4.12 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate measurement uncertainty budget in TR 37.843 [16], subclause 10.6 shall be used for evaluating the test system uncertainty. |  |

Table 4.1.2.3-2: Maximum OTA Test System uncertainty for FR2 OTA receiver tests

| Subclause | Maximum OTA Test System uncertainty |
| --- | --- |
| 7.3 OTA reference sensitivity level | ±2.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±2.4 dB, 37 GHz < f ≤ 40 GHz |
| 7.5.1 OTA adjacent channel selectivity | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 40 GHz |
| 7.5.2 In-band blocking (General) | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 40 GHz |
| 7.6 OTA out-of-band blocking | ±4.1 dB |
| 7.7 OTA receiver spurious emissions | ±2.5 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |
| 7.8 OTA receiver intermodulation | ±3.9 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.9 dB, 37 GHz < f ≤ 40 GHz |
| 7.9 OTA in-channel selectivity | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 40 GHz |

#### 4.1.2.4	Measurement of performance requirement

Table 4.1.2.4-1: Maximum OTA Test System uncertainty for FR1 OTA performance requirements

| Subclause | Maximum OTA Test System uncertainty | Derivation of OTA Test System uncertainty |
| --- | --- | --- |
| 8 PUSCH, PUCCH, PRACH with [single antenna port] and fading channel | ± [0.6] dB | [Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB] |
| 8 PRACH with [single antenna port] and AWGN | ± [0.3] dB | [Signal-to-noise ratio uncertainty ±0.3 dB] |
| 8 PUSCH with [two antenna port] and fading channel | ± [0.8] dB | [Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO] |

Table 4.1.2.4-2: Maximum OTA Test System uncertainty for FR2 OTA performance requirements

| Subclause | Maximum OTA Test System uncertainty | Derivation of OTA Test System uncertainty |
| --- | --- | --- |
| 8 PUSCH, PUCCH, PRACH with [single antenna port] and fading channel | ± [0.6] dB | [Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB] |
| 8 PRACH with [single antenna port] and AWGN | ± [0.3] dB | [Signal-to-noise ratio uncertainty ±0.3 dB] |
| 8 PUSCH with [two antenna port] and fading channel | ± [0.8] dB | [Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO] |

### 4.1.3	Interpretation of measurement results

The measurement results returned by the OTA Test System are compared - without any modification - against the test requirements as defined by the Shared Risk principle in Recommendation ITU-R M.1545 [4].

The actual measurement uncertainty of the OTA Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the OTA Test System uncertainty shall be, for each OTA measurement, equal to or lower than the appropriate figure in subclause 4.1.2 of this specification.

If the OTA Test System for an OTA test is known to have a measurement uncertainty greater than that specified in subclause 4.1.2, it is still permitted to use this apparatus provided that an adjustment is made as follows:

Any additional uncertainty in the OTA Test System over and above that specified in subclause 4.1.2 shall be used to tighten the OTA test requirement, making the test harder to pass. For some tests e.g. receiver tests, this may require modification of stimulus signals. This procedure will ensure that an OTA Test System not compliant with subclause 4.1.2 does not increase the chance of passing a DUT where that device would otherwise have failed the test if an OTA Test System compliant with subclause 4.1.2 had been used.

## 4.2	Radiated requirement reference points

Radiated characteristics for BS type 1-H, BS type 1-O and BS type 2-O are defined over the air (OTA) where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. For BS type 1-H the requirements are defined for two points of reference, signified by radiated requirements at the RIB and the conducted requirements at transceiver array boundary (TAB). The OTA requirements of BS type 1-H are tested in the far field (Fraunhofer) region.

General architecture and reference points of BS type 1-H, BS type 1-O and BS type 2-O are presented on the following figures 4.2-1 – 4.2-2.

Figure 4.2-1: General architecture of BS type 1-H

This specification details only radiated test requirements and hence only requires the radiated reference points.

Figure 4.2-2: General architecture of BS type 1-O and BS type 2-O

The transceiver unit array is part of the composite transceiver functionality generating modulated transmit signal structures and performing receiver combining and demodulation.

The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to receive/send parallel independent modulated symbol streams.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network that distributes the RF power between the transceiver array boundary and the antenna array, in an implementation specific way.

## 4.3	Base station classes

The requirements in this specification apply to Wide Area Base Stations, Medium Range Base Stations and Local Area Base Stations unless otherwise stated. The associated deployment scenarios and definitions of BS classes are exactly the same for BS with and without connectors.

BS classes for BS type 1-H are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum coupling loss equal to 45 dB.

BS classes for BS type 1-O and BS type 2-O are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

The manufacturer shall declare the intended class of the BS under test.

## 4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.4-1: List of regional requirements

| Subclause | Requirement | Comments |
| --- | --- | --- |
| 5 | Operating bands | Some NR operating bands may be applied regionally. |
| 6.7.2 | OTA occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the occupied bandwidth according to the definition in present specification. |
| 6.7.4 | OTA operating band unwanted emissions | Category A or Category B operating band unwanted emission limits may be applied regionally. |
| 6.7.4.5.1.6.1 | OTA operating band unwanted emissions Limits in FCC Title 47 | The BS may have to comply with the applicable emission limits established by FCC Title 47, when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.7.4.5.1.6.2 | OTA operating band unwanted emissions Protection of DTT | The BS operating in Band n20 may have to comply with the additional requirements for protection of DTT, when deployed in certain regions. |
| 6.7.5.2 | General OTA transmitter spurious emissions | Category A or Category B spurious emission limits, as defined in ITU-R Recommendation SM.329 [5], may apply regionally. The emission limits for BS type 1-O specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |
| 6.7.5.4 | Additional OTA transmitter spurious emissions | These requirements may be applied for the protection of system operating in frequency ranges other than the BS operating band. |
| 6.8.5.1 | OTA transmitter intermodulation | Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are not excluded from the requirement in Japan in Band n77, n78, n79. |
| 7.7 | OTA receiver spurious emissions | The emission limits for BS type 1-O specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |

## 4.5	BS configurations

### 4.5.1	Transmit configurations

Unless otherwise stated, the radiated transmitter characteristics in clause 6 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Editor’s note: to be aligned with the figures for the RIB interfaces and co-location concept.

Figure 4.5.1-1: Transmitter test interfaces

Figure 4.5.1-2: Transmitter test interfaces for co-location concept

### 4.5.2	Receive configurations

Unless otherwise stated, the radiated receiver characteristics in clause 7 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Editor’s note: to be aligned with the figures for the RIB interfaces and co-location concept.

Figure 4.5.2-1: Receiver test interfaces

Figure 4.5.2-2: Receiver test interfaces for co-location concept

### 4.5.3	Power supply options

If the BS is supplied with a number of different power supply configurations, it may not be necessary to test RF parameters for each of the power supply options, provided that it can be demonstrated that the range of conditions over which the equipment is tested is at least as great as the range of conditions due to any of the power supply configurations.

### 4.5.4	BS with integrated Iuant BS modem

Unless otherwise stated, for the tests in the present document, the integrated Iuant BS modem shall be switched OFF.

## 4.6	Manufacturer’s declarations

The following BS manufacturer’s declarations listed in table 4.6-1, when applicable to the BS under test, are required to be provided by the manufacturer for radiated requirements testing for BS type 1-H, BS type 1-O and BS type 2-O.

For the BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3], subclause 4.6.

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 3) | x | x | x |
| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Supported bands declared for every beam (D.3). (Note 4) | c | x | x |
| D.5 | BS requirements set | Declaration of one of the NR base station requirement’s set as defined for BS type 1-H, BS type 1-O, or BS type 2-O. | c | x | x |
| D.6 | BS class | Declared as Wide Area BS, Medium Range BS, or Local Area BS. | c | x | x |
| D.7 | BS channel band width and SCS support | BS supported SCS and channel bandwidth per supported SCS. Declared for each beam (D.3) and each operating band (D.4). | c | x | x |
| D.8 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | x | x | x |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.11 | Rated beam EIRP | The rated EIRP level per carrier (PRated,c,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared for every beam (D.3). (Note 12, 14) | x | x | x |
| D.12 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3). | x | x | x |
| D.13 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.12) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. | x | x | x |
| D.14 | Parallel beams | List of beams which have been declared equivalent (D.13) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. | x | x | x |
| D.15 | Number of carriers at maximum TRP | The number of carriers per operating band the BS is capable of generating at maximum TRP declared for every beam (D.3). | n/a | x | x |
| D.16 | Operating bands with multi-band dependencies | List of operating bands which are generated using transceiver units supporting operation in multiple operating bands through common active RF components. Declared for each operating band for which multi-band transceiver is used. | c | x | n/a |
| D.17 | Maximum radiated Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band, declared for each supported operating band (D.4). (Note 15) | c | x | x |
| D.18 | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D.16). | c | x | n/a |
| D.19 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (D.52). | c | x | x |
| D.20 | CA-only operation | Declared of CA-only (with equal power spectral density among carriers) but not multiple carriers operation, declared per operating band (D.4) and per beam (D.3). | c | x | x |
| D.21 | Total number of supported carriers for operating bands with multi-band dependencies | Total number of supported carriers for operating bands declared to have multi-band dependencies (D.16). | c | x | n/a |
| D.22 | Contiguous or non-contiguous spectrum operation support | Ability of BS to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier in an operating band. | c | x | x |
| D.23 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a |
| D.24 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.23). (Note 5) | x | x | n/a |
| D.25 | OTA sensitivity supported BS channel bandwidth and SCS | The BS supported SCS and channel bandwidth per supported SCS by each OSDD. | x | x | n/a |
| D.26 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD. | x | x | n/a |
| D.27 | Minimum EIS for FR1 (EISminSENS) | The minimum EISminSENS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared per NR supported channel BW for the OSDD (D.30). The lowest EIS value for all the declared OSDD’s is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. (Note 6) | x | x | n/a |
| D.28 | EIS REFSENS for FR2 (EISREFSENS_50M) | The EISREFSENS_50M level applicable in the OTA REFSENS RoAoA, (used as a basis for the derivation of the FR2 EISREFSENS for other channel bandwidths supported by BS). (Note 7) | n/a | n/a | x |
| D.29 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D.31) for each OSDD. | x | x | n/a |
| D.30 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. (Note 8) | x | x | n/a |
| D.31 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D.30). (Note 9) | x | x | n/a |
| D.32 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D.33). | x | x | n/a |
| D.33 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. | x | x | n/a |
| D.34 | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. (Note 10) | x | x | x |
| D.35 | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (D.34). (Note 11) | x | x | x |
| D.36 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. | x | x | x |
| D.37 | The rated carrier OTA BS power, PRated,c,TRP | PRated,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band. (Note 12, 14) | n/a | x | x |
| D.38 | Rated transmitter TRP,Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 12,14) | n/a | x | x |
| D.39 | CLTA placement for co-location test | The manufacturer shall declare the side of DUT where radiating elements are placed closest to the edge of DUT when applicable. The CLTA shall be placed at the DUT side where radiating elements are placed closest. | n/a | x | n/a |
| D.40 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | c | x | x |
| D.41 | Additional operating band unwanted emissions | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.7.4 apply. (Note 16) | c | x | x |
| D.42 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or PHS operating in another operating band are deployed. | c | x | x |
| D.43 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD and/or E-UTRA operating in another operating band. | c | x | n/a |
| D.44 | Single-band RIB or multi-band RIB | List of single-band RIB and/or multi-band RIB for the supported operating bands (D.4). | c | x | n/a |
| D.45 | Single or multiple carrier | BS capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RIB. (Note 17) | c | x | x |
| D.46 | Maximum number of supported carriers per operating band | Maximum number of supported carriers. Declared per supported operating band, per RIB. (Note 15) | c | x | x |
| D.47 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands. Declared per RIB. | c | x | x |
| D.48 | Other band combination multi-band restrictions | Declare any other limitation under simultaneous operation in the declared band combinations (D.16), which have any impact on the test configuration generation. | c | x | n/a |
| D.49 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band. Declared per operating band (D.4). | c | n/a | n/a |
| D.50 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band. Declared per operating band (D.4). | c | x | x |
| D.51 | Maximum supported power difference between carriers is different operating bands | Maximum supported power difference between any two carriers in any two different supported operating bands. Declared per operating bands combination (D.52). | c | x | n/a |
| D.52 | Operating band combination support | List of operating bands combinations supported by single-band RIB(s) and/or multi-band RIB(s) of the BS. | c | x | n/a |
| D.53 | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. | n/a | x | x |
| D.54 | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (D.53). | n/a | x | x |
| D.55 | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. | n/a | x | x |
| D.56 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. | x | x | x |
| D.57 | Rated beam EIRP at lower end of the fractional bandwidth (Prated,c,FBWlow) | The rated EIRP level per carrier at lower frequency range of the fractional bandwidth (Prated,c,FBWlow), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges (D.56). (Note 12, 13, 14, 15) | x | x | x |
| D.58 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh) | The rated EIRP level per carrier at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges in (D.56). (Note 12, 13, 14 ,15) | x | x | x |
| D.59 | Relation between supported maximum RF bandwidth, number of carriers and Rated maximum TRP | If the rated transmitter TRP and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated transmitterTRP; - The reduced total output power at the maximum number of supported carriers. | n/a | x | x |
| D.60 | Inter-band CA | Declaration of operating band(s) combinations supporting inter-band CA. Declared per operating band combination (D.52). | c | x | x |
| D.61 | Intra-band contiguous CA | Declaration of operating band(s) supporting intra-band contiguous CA. Declared per operating band with CA support. | c | x | x |
| D.62 | Intra-band non-contiguous CA | Declaration of operating band(s) supporting intra-band non-contiguous CA. Declared per operating band with CA support. | c | x | x |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type for FR1 as specified in TS 38.211 [20], i.e., type A, type B or both. | c | x | n/a |
| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2, i.e., pos0, pos1, or both. | n/a | n/a | x |
| D.102 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [20], i.e., format 0, format 1, format 2, format 3, format 4. | c | x | x |
| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [20], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [20], i.e.: - For BS type 1-O: 15 kHz, 30 kHz or both. - For BS type 1-O: 60 kHz, 120 kHz or both. | c | x | x |
| D.104 | Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.105 | Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.106 | PUSCH PT-RS | Declaration of PT-RS in PUSCH support: without PT-RS, with PT-RS or both. | n/a | n/a | x |
| D.107 | PUCCH multi-slot | Declaration of multi-slot PUCCH support. | c | x | n/a |
| NOTE 1: Manufacturer declarations applicable per BS requirement set were marked as “x”. Manufacturer declarations not applicable per BS requirement set were marked as “n/a”. NOTE 2: For BS type 1-H, the only radiated declarations are related to EIRP and EIS requirements. For BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3]. For declarations marked as ‘c’, related conducted declarations in TS 38.141-1 [3] apply. NOTE 3: Depending on the capability of the system some of these beams may be the same. NOTE 4: These operating bands are related to their respective single-band RIBs. NOTE 5: As each identified OSDD has a declared minimum EIS value (D.27), multiple operating band can only be declared if they have the same minimum EIS declaration. NOTE 6: If the BS type 1-H or BS type 1-O is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. NOTE 7: Although EISREFSENS_50M level is based on a reference measurement channel with BWChannel = 50 MHz, it does not imply that BS has to support 50 MHz channel bandwidth. NOTE 8: Not applicable for BS type 2-O. NOTE 9: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. NOTE 10: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error, TAE or EVM. NOTE 11: The OTA coverage reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 12: If a BS type 2-O is capable of 64QAM DL operation then two rated output power declarations may be made. One declaration is applicable when configured for 64QAM transmissions and the other declaration is applicable when not configured for 64QAM transmissions. NOTE 13: If D.57 and D.58 are declared for certain frequency range (D.56), there shall be no “Rated beam EIRP” declaration (D.11) for the operating band containing that particular frequency range. NOTE 14: If a BS type 1-H or BS type 1-O is capable of 256QAM DL operation then two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions and the other declaration is applicable when not configured for 256QAM transmissions. NOTE 15: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. NOTE 16: If BS is declared to support Band n20 (D.4), the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 17: in case of BS type 1-H, this declaration applies per TAB connector. |  |  |  |  |  |

## 4.7	Test configurations

### 4.7.1	General

The test configurations shall be constructed using the methods defined below subject to the parameters declared by the manufacturer as listed in subclause 4.6.

The applicable test models for generation of the carrier transmit test signal are defined in subclause 4.9.2.

NOTE:	In case, carriers are shifted to align with the channel raster Foffset.

### 4.7.2	Test signal configurations

#### 4.7.2.1	Test signal used to build Test Configurations

The signal’s BS channel bandwidth and subcarrier spacing used to build NR Test Configurations shall be selected according to tables 4.7.2.1-1 and 4.7.2.1-2.

Table 4.7.2.1-1: Signal to be used to build NR TCs for BS type 1-H and BS type 1-O

| Operating band characteristics |  | FDL_high – FDL_low < 100 MHz | FDL_high – FDL_low ≥ 100 MHz |
| --- | --- | --- | --- |
| TC signal characteristics | BWchannel | 5 MHz (Note) | 20 MHz (Note) |
|  | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |  |
| Note: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal characteristics | BWchannel | 100 MHz (Note 1, Note 2) |
|  | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| Note 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. Note 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

#### 4.7.2.2	NRTC1: Contiguous spectrum operation

The purpose of test configuration NRTC1 is to test all BS requirements excluding CA occupied bandwidth.

For NRTC1 used in receiver tests only the two outermost carriers within each supported operating band need to be generated by the test equipment.

##### 4.7.2.2.1	NRTC1 generation

NRTC1 shall be constructed on a per band basis using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth for contiguous operation (D.17).

-	Select the carrier to be tested according to 4.7.2.1 and place it adjacent to the lower Base Station RF Bandwidth edge. Place same signal adjacent to the upper Base Station RF Bandwidth edge.

-	For transmitter tests, select as many carriers (according to 4.7.2.1) that the beam supports within a band and that fit in the rest of the declared maximum Base Station RF Bandwidth. Place the carriers adjacent to each other starting from the upper Base Station RF Bandwidth edge. The nominal carrier spacing defined in TS 38.104 [2] subclause 5.4.1 shall apply;

The test configuration should be constructed on a per band basis for all component carriers of the inter-band CA bands declared to be supported by the beam (D.59). All configured component carriers are transmitted simultaneously in the tests where the transmitter should be on.

##### 4.7.2.2.2	NRTC1 power allocation

Set the number of carriers to the number of carriers at maximum TRP (D.15).

For EIRP accuracy requirements set each beam to rated beam EIRP (D.11) for the tested beam direction pair.

For all other requirements ensure the total radiated power is set to rated carrier TRP PRated,c,TRP (D.37).

For a beam declared to support CA-only operation (D.20), set the power spectral density of each carrier to the same level so that the sum of the carrier power equals the same value as above.

#### 4.7.2.3	NRTC2: Contiguous CA occupied bandwidth

NRTC2 in this subclause is used to test CA occupied bandwidth.

##### 4.7.2.3.1	NRTC2 generation

NRTC2 shall be constructed on a per band basis using the following method:

-	All component carrier combinations supported by the beam, which have different sum of channel bandwidths of component carrier, shall be tested. For all component carrier combinations which have the same sum of channel bandwidths of component carriers, only one of the component carrier combinations shall be tested.

-	Of all component carrier combinations which have same sum of channel bandwidths of component carrier, select those with the narrowest carrier with the smallest supported subcarrier spacing declared per operating band (D.7) at the lower Base Station RF Bandwidth edge.

-	Of the combinations selected in the previous step, select one with the narrowest carrier with the smallest supported subcarrier spacing declared per operating band (D.7) at the upper Base Station RF Bandwidth edge.

-	If there are multiple combinations fulfilling previous steps, select the one with the smallest number of component carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier with the smallest supported subcarrier spacing declared per operating band (D.7) being adjacent to the lowest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier with the smallest supported subcarrier spacing declared per operating band (D.7) being adjacent to the highest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier with the smallest supported subcarrier spacing declared per operating band (D.7) being adjacent to the carrier which has been selected in the previous step.

-	If there are multiple combinations fulfilling previous steps, repeat the previous step until there is only one combination left.

-	The nominal channel spacing defined in TS 38.104 [2] subclause 5.4.1 shall apply.

##### 4.7.2.3.2	NRTC2 power allocation

Set the number of carriers to the number of carriers at maximum TRP (D.15).

For EIRP accuracy requirements set each beam to rated beam EIRP (D.11) for the tested beam direction pair.

For all other requirements ensure the total radiated power is set to rated carrier TRP PRated,c,TRP (D.37).

For a beam declared to support CA-only operation (D.20), set the power spectral density of each carrier to the same level so that the sum of the carrier power equals the same value as above.

#### 4.7.2.4	NRTC3: Non-contiguous spectrum operation

The purpose of NRTC3 is to test NR multicarrier non-contiguous aspects.

For NRTC3 used in receiver tests, outermost carriers for each sub-block need to be generated by the test equipment; other supported carriers are optional to be generated.

##### 4.7.2.4.1	NRTC3 generation

NRTC3 is constructed on a per band basis using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth for non-contiguous operation (D.17). The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum radiated Base Station RF Bandwidth for non-contiguous operation (D.17).

-	Select the carrier to be tested according to 4.7.2.1. Place it adjacent to the upper Base Station RF Bandwidth edge and another similar carrier adjacent to the lower Base Station RF Bandwidth edge.

-	For single-band operation receiver tests, if the remaining gap is at least 15 MHz (or 60 MHz if channel bandwidth of the carrier to be tested is 20 MHz) for FR1 or 150 MHz for FR2 plus two times the channel bandwidth used in the previous step and the beam supports at least 4 carriers, place a NR carrier of this channel bandwidth adjacent to each already placed carrier for each sub-block. The nominal channel spacing defined in TS 38.104 [2] subclause 5.4.1 shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset_high and FOffset_low for the carriers adjacent to the sub-block gap.

##### 4.7.2.4.2	NRTC3 power allocation

Set the number of carriers to the number of carriers at maximum TRP (D.15).

For EIRP accuracy requirements set each beam to rated beam EIRP (D.11) for the tested beam direction pair.

For all other requirements ensure the total radiated power is set to rated carrier TRP PRated,c,TRP (D.37).

#### 4.7.2.5	NRTC4: Multi-band test configuration for full carrier allocation

The purpose of NRTC4 is to test beams which have been generated using transceiver units supporting operation in multiple operating bands through common active electronic components(s), considering maximum supported number of carriers.

##### 4.7.2.5.1	NRTC4 generation

NRTC4 is based on re-using the existing test configuration applicable per band on beams generated using Multi-band transceiver units and hence have declared multi-band dependencies (D.16). It is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth (D.17).

-	The number of carriers of each supported operating band shall be the declared total number of supported carriers for operating bands with multi-band dependencies in each band (D.21). Carriers shall be selected according to 4.7.2.1 and shall first be placed at the outermost edges of the declared maximum radiated Radio Bandwidth (D.18). Additional carriers shall next be placed at the edges of Base Station RF Bandwidth, if possible.

-	The allocated Base Station RF Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum radiated Radio Bandwidth (D.18).

-	Each concerned band shall be considered as an independent band and the corresponding test configuration shall be generated in each band. The mirror image of the single band test configuration shall be used in the highest band being tested for the beam.

--	If an operating band with multi-band dependencies supports three carriers only, two carriers shall be placed in one band according to the relevant test configuration while the remaining carrier shall be placed at the edge of the maximum Radio Bandwidth in the other band.

-	If the sum of the maximum base station RF bandwidths of each of the supported operating bands is greater than the declared total RF bandwidth BWtot (D.19) of transmitter and receiver for the declared band combinations of the BS, then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared total RF bandwidth is not exceeded and vice versa.

-	If the sum of the maximum number of supported carrier of each supported operating bands with multi-band dependencies (D.16) is larger than the declared total number of supported carriers for operating bands with multi-band dependencies (D.21), repeat the steps above for test configurations where in each test configuration the number of carriers of one of the operating band shall be reduced so that the total number of supported carriers is not be exceeded and vice versa.

##### 4.7.2.5.2	NRTC4 power allocation

Set the number of carriers to the total number of supported carriers for operating bands with multi-band dependencies (D.21).

For EIRP accuracy requirements set each beam to rated beam EIRP (D.11) for the tested beam direction pair.

For all other requirements ensure the total radiated power is set to rated carrier TRP PRated,c,TRP (D.37).

If the allocated number of carriers in an operating band exceeds the declared number of carriers at maximum TRP in an operating band (D.15) the carriers should if possible be allocated to a different operating band.

#### 4.7.2.6	NRTC5: Multi-band test configuration with high PSD per carrier

The purpose of NRTC5 is to test multi-band operation aspects considering higher PSD cases with reduced number of carriers and non-contiguous operation (if supported) in multi-band mode.

##### 4.7.2.6.1	NRTC5 generation

NRTC5 is based on re-using the existing test configuration applicable for operating bands using multi-band transceiver units and hence have declared multi-band dependencies (D.16). It is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth (D.17).

-	The allocated Radio Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum Radio Bandwidth of the operating band with multi-band dependencies (D.18).

-	The maximum number of carriers is limited to two per band. Carriers shall be selected according to 4.7.2.1 and shall be placed at the outermost edges of the declared maximum Radio Bandwidth of the operating band with multi-band dependencies (D.18).

-	Each concerned band shall be considered as an independent band and the carrier placement in each band shall be according to NRTC3, where the declared parameters for multi-band operation shall apply. Narrowest supported BS channel bandwidth with the smallest subcarrier spacing declared per operating band (D.7) shall be used in the test configuration.

-	If an operating band with multi-band dependencies supports three carriers only, two carriers shall be placed in one band according to the relevant test configuration while the remaining carrier shall be placed at the edge of the maximum Radio Bandwidth in the other band.

-	If the sum of the maximum base station RF bandwidths of each of the supported operating bands is greater than the declared total RF bandwidth BWtot (D.19) of transmitter and receiver for the declared band combinations of the BS, then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared total RF bandwidth BWtot of the operating band with multi-band dependencies (D.18) is not exceeded and vice versa.

##### 4.7.2.6.2	NRTC5 power allocation

Set the number of carriers to the total number of supported carriers for operating bands with multi-band dependencies (D.21).

For EIRP accuracy requirements set each beam to rated beam EIRP (D.11) for the tested beam direction pair.

For all other requirements ensure the total radiated power is set to rated carrier TRP PRated,c,TRP (D.37).

If the sum of the TRP for all carriers in an operating band(s) exceeds the sum of the rated carrier TRP output power PRated,c,TRP (D.37) for the number of carriers at maximum TRP (D.15) in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the EIRP allocated for a carrier exceeds the declared rated TRP, the exceeded power shall, if possible, be reallocated into the other carriers.

## 4.8	Applicability of requirements

### 4.8.1	Requirement set applicability

In table 4.8.1-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement subclause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.8.1-1: Requirement set applicability

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O |
| Radiated transmit power | 6.2 | 6.2 | 6.2 |
| OTA base station output power | NA | 6.3 | 6.3 |
| OTA output power dynamics |  | 6.4 | 6.4 |
| OTA transmit ON/OFF power |  | 6.5 | 6.5 |
| OTA transmitted signal quality |  | 6.6 | 6.6 |
| OTA occupied bandwidth |  | 6.7.2 | 6.7.2 |
| OTA ACLR |  | 6.7.3 | 6.7.3 |
| OTA out-of-band emission |  | 6.7.4 | 6.7.4 |
| OTA transmitter spurious emission |  | 6.7.5 | 6.7.5 |
| OTA transmitter intermodulation |  | 6.8 | NA |
| OTA sensitivity | 7.2 | 7.2 | NA |
| OTA reference sensitivity level | NA | 7.3 | 7.3 |
| OTA dynamic range |  | 7.4 | NA |
| OTA in-band selectivity and blocking |  | 7.5 | 7.5 |
| OTA out-of-band blocking |  | 7.6 | 7.6 |
| OTA receiver spurious emission |  | 7.7 | 7.7 |
| OTA receiver intermodulation |  | 7.8 | 7.8 |
| OTA in-channel selectivity |  | 7.9 | 7.9 |
| Radiated performance requirements |  | 8 | 8 |

### 4.8.2	Applicability of test configurations for single-band RIB

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to subclause 4.6. The generation and power allocation for each test configuration is defined in subclause 4.7. This subclause contains the test configurations for single-band RIB.

For a BS declared to be capable of single carrier operation only, a single carrier (SC) shall be used for testing.

For a single-band RIB declared to support multi-carrier and/or CA operation in contiguous spectrum operation, the test configurations in the second column of table 4.8.2-1 shall be used for testing.

For a single-band RIB declared to support multi-carrier and/or CA operation in contiguous and non-contiguous spectrum and where the parameters in the manufacturer's declaration according to subclause 4.6 are identical for contiguous (C) and non-contiguous (NC) spectrum operation, the test configurations in the third column of table 4.8.2-1 shall be used for testing.

For a single-band RIB declared to support multi-carrier and/or CA in contiguous and non-contiguous spectrum and where the parameters in the manufacture's declaration according to subclause 4.6 are not identical for contiguous and non-contiguous spectrum operation, the test configurations in the fourth column of table 4.8.2-1 shall be used for testing.

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported BS channel bandwidth with the smallest supported subcarrier spacing declared per operating band (D.7).

Table 4.8.2-1: Test configurations for a single-band RIB

| BS test case | Contiguous spectrum capable BS | C and NC capable BS with identical parameters | C and NC capable BS with different parameters |
| --- | --- | --- | --- |
| Radiated transmit power | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA base station maximum output power | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| OTA total power dynamic range | SC | SC | SC |
| OTA transmit ON/OFF power (only applied for NR TDD BS) | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| OTA error Vector Magnitude | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA time alignment error | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA Occupied bandwidth | SC, NRTC2 (Note 1) | SC, NRTC2 (Note 1) | SC, NRTC2 (Note 1) |
| OTA ACLR | NRTC1 | NRTC1,NRTC3 | NRTC1, NRTC3 |
| OTA CACLR | - | NRTC3 | NRTC3 |
| OTA operating band unwanted emissions | NRTC1, SC (Note 2) | NRTC1, NRTC3, SC (Note 2) | NRTC1, NRTC3, SC (Note 2) |
| OTA transmitter spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA transmitter intermodulation | NRTC1 | NRTC1, NRTC3 | NRTC1, NRTC3 |
| OTA sensitivity | SC | SC | SC |
| OTA reference sensitivity level | SC | SC | SC |
| OTA dynamic range | SC | SC | SC |
| OTA adjacent channel selectivity | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| In-band blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA out-of-band blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA receiver spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA receiver intermodulation | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA in-channel selectivity | SC | SC | SC |
| Note 1: NRTC2 is only applicable when contiguous CA is supported. Note 2: OBUE SC shall be tested using the widest supported channel bandwidth and the highest supported subcarrier spacing. |  |  |  |

### 4.8.3	Applicability of test configurations for multi-band RIB

For a multi-band RIB, the test configuration in table 4.8.3-1 shall be used for testing.

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported BS channel bandwidth with the smallest supported subcarrier spacing declared per operating band (D.7).

NOTE:	The applicability of test configurations in table 4.8.3-1 are not applicable to BS type 2-O.

Table 4.8.3-1: Test configuration for a multi-band RIB

| BS test case | Test configuration |
| --- | --- |
| Radiated transmit power | NRTC1/3 (Note 1), NRTC4 |
| OTA base station maximum output power | NRTC1/3 (Note 1), NRTC4 |
| OTA RE power control dynamic range | Tested with Error Vector Magnitude |
| OTA total power dynamic range | SC |
| OTA transmit ON/OFF power (only applied for NR TDD BS) | NRTC4 |
| OTA frequency error | Tested with Error Vector Magnitude |
| OTA Error Vector Magnitude | NRTC1/3 (Note 1), NRTC4 |
| OTA time alignment error | NRTC1/3 (Note 1), NRTC5 (Note 2) |
| OTA occupied bandwidth | SC, NRTC2 (Note 3) |
| OTA ACLR | NRTC1/3 (Note 1), NRTC5 (Note 4) |
| OTA CACLR | NRTC3 (Note 1), NRTC5 (Note 4) |
| OTA operating band unwanted emissions | NRTC1/3 (Note 1), NRTC5, SC (Note 5) |
| OTA transmitter spurious emissions | NRTC1/3 (Note 1), NRTC5 |
| OTA transmitter intermodulation | NRTC1/3 (Note 1) |
| OTA sensitivity | SC |
| OTA reference sensitivity level | SC |
| OTA dynamic range | SC |
| OTA adjacent channel selectivity | NRTC5 |
| In-band blocking | NRTC5 |
| OTA out-of-band blocking | NRTC5 |
| OTA receiver spurious emissions | NRTC1/3 (Note 1), NRTC5 |
| OTA receiver intermodulation | NRTC5 |
| OTA in-channel selectivity | SC |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. Note 2: NRTC5 is only applicable when inter-band CA is supported. Note 3: NRTC2 is only applicable when contiguous CA is supported. Note 4: NRTC5 may be applied for Inter RF Bandwidth gap only. Note 5: OBUE SC shall be tested using the widest supported channel bandwidth and the highest supported sub-carrier spacing. |  |

## 4.9	RF channels and test models

### 4.9.1	RF channels

For the single carrier testing many tests in this TS are performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the BS. These are denoted as RF channels B (bottom), M (middle) and T (top).

Unless otherwise stated, the test shall be performed with a single carrier at each of the RF channels B, M and T.

Many tests in this TS are performed with the maximum Base Station RF Bandwidth located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BRFBW (bottom), MRFBW (middle) and TRFBW (top).

Unless otherwise stated, the test shall be performed at BRFBW, MRFBW and TRFBW defined as following:

-	BRFBW: maximum Base Station RF Bandwidth located at the bottom of the supported frequency range in the operating band.

-	MRFBW: maximum Base Station RF Bandwidth located in the middle of the supported frequency range in the operating band.

-	TRFBW: maximum Base Station RF Bandwidth located at the top of the supported frequency range in the operating band.

For a BS capable of multi-band operation and capable of dual-band operation, unless otherwise stated, the test shall be performed at BRFBW_T'RFBW and B'RFBW_TRFBW defined as following:

-	BRFBW_ T'RFBW: the Base Station RF Bandwidths located at the bottom of the supported frequency range in the lower operating band and at the highest possible simultaneous frequency position, within the maximum Radio Bandwidth, in the upper operating band.

-	B'RFBW_TRFBW: the Base Station RF Bandwidths located at the top of the supported frequency range in the upper operating band and at the lowest possible simultaneous frequency position, within the maximum Radio Bandwidth, in the lower operating band.

NOTE:	BRFBW_T'RFBW = B'RFBW_TRFBW = BRFBW_TRFBW when the declared maximum Radio Bandwidth (see subclause 4.6) spans both operating bands. BRFBW_TRFBW means the Base Station RF Bandwidths are located at the bottom of the supported frequency range in the lower operating band and at the top of the supported frequency range in the upper operating band.

Occupied bandwidth test in this TS are performed with the aggregated BS channel bandwidth and sub-block bandwidths located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BBW Channel CA(bottom), MBW Channel CA (middle) and TBW Channel CA (top) for contiguous spectrum operation.

Unless otherwise stated, the test for contiguous spectrum operation shall be performed at BBW Channel CA, MBW Channel CA and TBW Channel CA defined as following:

-	BBW Channel CA: aggregated BS channel bandwidth located at the bottom of the supported frequency range in each operating band;

-	MBW Channel CA: aggregated BS channel bandwidth located close in the middle of the supported frequency range in each operating band;

-	TBW Channel CA: aggregated BS channel bandwidth located at the top of the supported frequency range in each operating band.

When a test is performed by a test laboratory, the position of B, M and T for single carrier, BRFBW, MRFBW and TRFBW for single band operation, BBW Channel CA, MBW Channel CA and TBW Channel CA for contiguous spectrum operation in the operating band, the position of BRFBW_T'RFBW and B'RFBW_TRFBW in the supported operating band combinations shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

### 4.9.2	Test models

#### 4.9.2.1	General

The following subclauses will describe the NR test models needed for BS type 2-O. Note the NR FR1 test models described in TS 38.141-1 [3] are also applicable for BS type 1-O conformance testing.

#### 4.9.2.2	NR FR2 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR- FR2-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-2 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For NR FR2 TDD, test models are derived based on the uplink/downlink configuration as shown in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon as defined in TS 38.331 [22].

Table 4.9.2.2-1: Configurations of TDD for BS type 2-O test models

| Field name | Value |  |
| --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 60 | 120 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25 | 1.25 |
| nrofDownlinkSlots | 3 | 7 |
| nrofDownlinkSymbols | 10 | 6 |
| nrofUplinkSlots | 1 | 2 |
| nrofUplinkSymbols | 2 | 4 |

Common physical channel parameters for all FR2 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 and table 4.9.2.2-4 for PDSCH. Specific physical channel parameters for FR2 test models are described in subclauses 4.9.2.2.1 to 4.9.2.2.3.

Table 4.9.2.2-2: Common physical channel parameters for BS type 2-O PDCCH

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for BS type 2-O PDSCH

| Parameter | Value |
| --- | --- |
| mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | 'pos2' |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 0 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |
| PTRS configuration and density | LPT-RS = 4 KPT-RS = 2 krefRE=00 |
| Ratio of PT-RS EPRE to DM-RS EPRE | 0 dB |

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 2-O test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |

##### 4.9.2.2.1	NR FR2 test model 1.1 (NR-FR2-TM1.1)

This model shall be used for tests on:

-	Radiated transmit power

-	BS output power

-	Transmit ON/OFF power

-	TAE

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Receiver spurious emissions

Common physical channel parameters are defined in section 4.9.2.2. Specific physical channel parameters for NR-FR2-TM1.1 are defined in table 4.9.2.2.1-1.

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR2-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH | NRB - 3 |
| Modulation PDSCH | QPSK |
| Starting RB location of PDSCH | 3 |
| Modulation of PDSCH | QPSK |
| Starting RB location of PDSCH | 0 |

##### 4.9.2.2.2	NR FR2 test model 2 (NR-FR2-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power)

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in subclause 4.9.2.2. Specific physical channel parameters for NR-FR2-TM2 are defined in table 4.9.2.2.2-1.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| # of PDSCH PRBs which are not allocated | NRB-1 |

##### 4.9.2.2.3	NR FR2 test model 3.1 (NR-FR2-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

NOTE:	EVM shall be evaluated over PDSCH allocated PRBs with nRNTI=0 and nRNTI=2.

Common physical channel parameters are defined in subclause 4.9.2.2. Specific physical channel parameters for NR-FR2-TM3.1 shall be defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 64QAM.

#### 4.9.2.3	Data content of physical channels and signals for NR-FR2-TM

Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS 38.211 [20], subclause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of '0' bits shall be generated prior to the scrambling.

Initialization of the scrambler and RE-mappers as defined in TS 38.211 [20] use the following additional parameters:

-	NIDcell=1 for the lowest configured carrier, NIDcell=2 for the 2nd lowest configured carrier, …  NIDcell=n for the nth configured carrier

-	Antenna ports starting with 1000 for PDSCH

-	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	Rank 1, single layer (except for TAE requirement of 2 layer MIMO transmission)

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], subclause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: 1(# of PDCCH) * 1(# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in subclause 4.4.9.2.2.

-	Generate this amount of bits according to 'all 0' data.

-	1 CCE shall be according to TS 38.211 [20], subclause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH occupies the first two symbols for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], subclause 7.3.2.3.

-	NID=NIDcell in DM-RS sequence generation in TS 38.211 [20], subclause 7.4.1.3.

-	nRNTI=0 in scrambling sequence generation in TS 38.211 [20], subclause 7.3.2.3.

-	Perform mapping to REs according to TS 38.211 [20], subclause 7.3.2.5.

##### 4.9.2.3.2	PDSCH

-	For each slot generate the required amount of bits for all PRBs according to 'all 0' data.

-	NR-FR2-TMs utilize 1 or 2 user PDSCH transmissions distinguished by nRNTI. For each NR-FR2-TM, PRBs are mapped to user (nRNTI) as follows:

Table 4.9.2.3.2-1: Mapping of PRBs to  for NR-FR2-TM

| Test model |  | Number of users |
| --- | --- | --- |
| NR-FR2-TM1.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |
| NR-FR2-TM2 | 2 for all PRBs | 1 |
| NR-FR2-TM3.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |

-	Perform user specific scrambling according to TS 38.211 [20], subclause 7.3.1.1.

-	nID=NIDcell

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [20], subclause7.3.1.2.

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [20], subclause 7.3.1.3.   Complex-valued modulation symbols  for codeword q shall be mapped onto the layers ,  where υ is equal to 1.

-	Perform PDSCH mapping type A according to TS 38.211 [20].

-	PDSCH resource allocation according to TS 38.214 [21] as following;

-	NR-FR2-TM1.1, NR-FR2-TM3.1: type 1 for PDSCH with nRNTI = 0, nRNTI = 2

-	NR-FR2-TM2: type 1 for PDSCH with nRNTI = 2

-	DM-RS sequence generation according to TS 38.211 [20], subclause 7.4.1.1.1 where l is the OFDM symbol number within the slot with symbols indicated by table 4.9.2.2-3.

-	NIDnSCID=NIDcell

-	nSCID=0

-	DM-RS mapping according to TS 38.211 [20], subclause 7.4.1.1.2 with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS sequence generation according to TS 38.211 [20], subclause 7.4.1.2.1, with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS mapping according to TS 38.211 [20], subclause 7.4.1.2.2, with parameters listed in table 4.9.2.2-3.

## 4.10	Requirements for contiguous and non-contiguous spectrum

A spectrum allocation where a BS operates can either be contiguous or non-contiguous. Unless otherwise stated, the requirements in the present specification apply for BS configured for both contiguous spectrum operation and non-contiguous spectrum operation.

For BS operation in non-contiguous spectrum, some requirements apply both at the Base Station RF Bandwidth edges and inside the sub-block gaps. For each such requirement, it is stated how the limits apply relative to the Base Station RF Bandwidth edges and the sub-block edges respectively.

## 4.11	Requirements for BS capable of multi-band operation

For multi-band RIB, the radiated test requirements in clause 6 and 7 apply separately to each supported operating band, unless otherwise stated. For some radiated test requirements, it is explicitly stated that specific additions or exclusions to the test requirement apply at multi-band RIB(s) as detailed in the requirement subclause.

BS type 1-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.

For multi-band RIBs supporting the bands for TDD, the radiated test requirements in the present specification assume no simultaneous uplink and downlink occur between the bands.

The radiated test requirements for multi-band RIBs supporting bands for both FDD and TDD are FFS and are not covered by the present release of this specification.

## 4.12	Co-location requirements

### 4.12.1	General

Co-location requirements are requirements which are based on assuming the BS type 1-O is co-located with another BS of the same base station class. They ensure that both co-located systems can operate with minimal degradation to each other.

The co-location requirements in table 4.12.1-1 rely on a co-location reference antenna used to mimic a base station to base station co-location scenario.

Table 4.12.1-1: Co-location requirements

| Subclause number | Requirement | Co-location reference antenna operation | Type |
| --- | --- | --- | --- |
| 6.5 | OTA transmit ON/OFF power for FR1 | Measure emission | Mandatory |
| 6.7.5.3 6.7.5.5 | OTA spurious emission: Protection of the BS receiver of own or different BS | Measure emission | Optional based on declaration |
|  | OTA spurious emission: Co-location with other base stations |  |  |
| 6.8 | OTA transmitter intermodulation | Inject the interferer signal | Mandatory |
| 7.6.3 | OTA out-of-band blocking: Co-location with other base stations | Inject the interferer signal | Optional based on declaration |

The OTA transmit ON/OFF power requirement and OTA transmitter intermodulation requirement are mandatory requirements where the test requirement is derived using the co-location reference antenna, which represents the worst-case scenario.

The co-location reference antenna is defined in TS 38.104 [2].

### 4.12.2	Co-location test antenna

#### 4.12.2.1	General

Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA).

A CLTA is a practical antenna which can be used to test conformance to the co-location requirements.

#### 4.12.2.2	Co-location test antenna characteristics

A co-location test antenna is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. A CLTA shall comply with the requirements specified in table 4.12.2.2-1.

Translation of the requirements to other test antennas are not precluded but suitable translations between the co-location reference antenna and test antenna must be provided to demonstrate that the method is within the specified MU.

The currently defined CLTAs are suitable for testing BS type 1-O implemented with a planar antenna array. The method for testing BS with other antenna array implementations is FFS and not covered by the present release of this specification.

Table 4.12.2.2-1: CLTA characteristics

| Parameter | In-band CLTA | Out-of-band CLTAs |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | N/A |
| Horizontal beam width | 65° ± 10° | 65° ± 10° |
| Vertical beam width | N/A | The half-power vertical beam width of the CLTA equals the narrowest declared (D.3) vertical beamwidth ±3° |
| Polarization | Match | Match to in-band |
| Conducted interface return loss | > 10 dB | > 10 dB |
| NOTE: If a multi-column or multi-band antenna is used the column closest to the NR BS shall be selected while other columns are terminated during testing. |  |  |

#### 4.12.2.3	Co-location test antenna alignment

The alignment between the NR BS under test and the co-location test antenna is described in table 4.12.2.3-1 and figure 4.12.2.3-1. The same physical alignment applies to in-band and out-of-band co-location requirements.

Table 4.12.2.3-1: CLTA alignment tolerances

| Parameter |  |
| --- | --- |
| Edge-to-edge separation between the NR BS and the CLTA, d | 0.1 m ± 0.01 m |
| Vertical alignment | Centre ± 0.01 m |
| Front alignment | Radome front ± 0.01 m |

Figure 4.12.2.3-1: Alignment of NR BS and CLTA

## 4.13	Format and interpretation of tests

Each test has a standard format:

X	Title

All tests are applicable to all equipment within the scope of the present document, unless otherwise stated.

X.1	Definition and applicability

This subclause gives the general definition of the parameter under consideration and specifies whether the test is applicable to all equipment or only to a certain subset. Required manufacturer declarations may be included here.

X.2	Minimum requirement

This subclause contains the reference to the subclause to the 3GPP reference (or core) specification which defines the minimum requirement.

X.3	Test purpose

This subclause defines the purpose of the test.

X.4	Method of test

X.4.1	General

In some cases there are alternative test procedures or initial conditions. In such cases, guidance for which initial conditions and test procedures can be applied are stated here. In the case only one test procedure is applicable, that is stated here.

X.4.2y	First test method

X.4.2y.1	Initial conditions

This subclause defines the initial conditions for each test, including the test environment, the RF channels to be tested and the basic measurement set-up. The OTA Test System is assumed to be correctly calibrated as part of the initial conditions. Calibration is not explicitly mentioned.

X.4.2y.2	Procedure

This subclause describes the steps necessary to perform the test and provides further details of the test definition like domain (e.g. frequency-span), range, weighting (e.g. bandwidth), and algorithms (e.g. averaging). The procedure may comprise data processing of the measurement result before comparison with the test requirement (e.g. average result from several measurement positions).

X.4.3y	Alternative test method (if any)

If there are alternative test methods, each is described with its initial conditions and procedures.

X.5	Test requirement

This subclause defines the pass/fail criteria for the equipment under test, see subclause 4.1.3 (Interpretation of measurement results). Test requirements for every minimum requirement referred in subclause X.2 are listed here. Cases where minimum requirements do not apply need not be mentioned.

## 4.14	Reference coordinate system

Radiated requirements are stated in terms of electromagnetic characteristics (e.g. EIRP and EIS) at certain angles with respect to the base station. To be able to declare radiated characteristics part of radiated requirements a reference coordinate system is required. The reference coordinate system is should be associated to an identifiable physical feature on the base station enclosure. The location of the origin and the orientation of the reference coordinate system are for the base station manufacturer to declare.

The reference coordinate system is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () as showed in figure 4.14-1.

Figure 4.14-1: Reference coordinate system

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°, inclusive. Note that  is defined as positive along the down-tilt angle.

# 5	Operating bands and channel arrangement

For the NR operating bands specification, their channel bandwidth configurations, channel spacing and raster, as well as synchronization raster specification, refer to TS 38.104 [2], clause 5 and its relevant subclauses.

For radiated testing purposes in this specification, FR1 and FR2 operating bands are considered.

# 6	Radiated transmitter characteristics

## 6.1	General

General test conditions for transmitter tests are given in clause 4, including interpretation of measurement results and configurations for testing. BS configurations for the tests are defined in subclause 4.5.

If beams have been declared equivalent and parallel (D.13, D.14), only a representative beam is necessary to be tested to demonstrate conformance.

## 6.2	Radiated transmit power

### 6.2.1	Definition and applicability

Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each declared beam, the requirement is based on declarations captured in subclause 4.6 for a beam identifier (D.3), reference beam direction pair (D.8), rated beam EIRP (D.11) at the beam's reference direction pair, OTA peak directions set (D.9), the beam direction pairs at the maximum steering directions (D.10) and their associated rated beam EIRP and beamwidth(s) for reference beam direction pair and maximum steering directions (D.12).

For a declared beam identifier and beam direction pair, the rated beam EIRP level is the maximum power that the BS is declared to radiate at the associated beam peak direction during the transmitter ON period.

For each beam peak direction associated with a beam direction pair within the OTA peak directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in subclause 6.2.4.1.

NOTE 1:	The OTA peak directions set for a beam is the complete continuous or discrete set of all beam direction for which the EIRP accuracy is intended to be achieved for the beam.

NOTE 2:	A beam direction pair consists of a beam centre direction and an associated beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

For operating bands where the supported fractional bandwidth (FBW) is larger than 6%, two rated carrier EIRP may be declared by manufacturer:

-	Prated,c,FBWlow for lower supported frequency range, and

-	Prated,c,FBWhigh for higher supported frequency range.

For frequencies in between FFBWlow and FFBWhigh the rated carrier EIRP is:

-	Prated,c,FBWlow, for the carrier whose carrier frequency is within frequency range FFBWlow ≤ f < (FFBWlow +FFBWhigh) / 2,

-	Prated,c,FBWhigh, for the carrier whose carrier frequency is within frequency range (FFBWlow +FFBWhigh) / 2 ≤ f ≤FFBWhigh.

Radiated transmit power is directional requirement applicable to BS type 1-H, BS type 1-O and BS type 2-O.

### 6.2.2	Minimum requirement

Radiated transmit power minimum requirement for BS type 1-H and BS type 1-O is defined in TS 38.104 [2], subclause 9.2.2.

Radiated transmit power minimum requirement for BS type 2-O is defined in TS 38.104 [2], subclause 9.2.3.

### 6.2.3	Test purpose

The test purpose is to verify the ability to accurately generate and direct radiated power per beam, across the frequency range and under normal conditions, for all declared beams of the BS type 1-H, BS type 1-O and BS type 2-O.

### 6.2.4	Method of test

#### 6.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: B, M and T; see subclause 4.9.1.

Base station RF bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW, MRFBW and TRFBW in single-band operation, see subclause 4.9.1,

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested:

-	OTA peak directions set reference beam direction pair (D.8), and

-	OTA peak directions set maximum steering directions (D.10).

Beams to be tested:	Declared beam with the highest intended EIRP for the narrowest intended beam corresponding to the smallest BeWθ, or for the narrowest intended beam corresponding to the smallest BeWϕ (D.3, D.11).

In addition for the BS type 1-O and BS type 2-O, a single test case shall be performed under extreme test environment as defined in annex B.3. In this case, it is sufficient to test on a single combination of one NR-ARFCN, one RF bandwidth position and with only one applicable test configuration defined in subclause 4.7. Direction to be tested is only at OTA peak directions set reference beam direction pair (D.8).

NOTE:	Tests under extreme power supply also test extreme temperature.

#### 6.2.4.2	Procedure

For normal test environment conditions in OTA domain, the test procedure is as follows:

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model(s) in subclause 4.9.2.

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model(s) in subclause 4.9.2 on all carriers configured.

6)	Measure EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Test steps 3 to 6 are repeated for all declared beams (D.3) and their reference beam direction pairs and maximum steering directions (D.8 and D.10).

For multi-band capable BS and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carriers activated in the other band.

For extreme conditions tests the methods in annex B.7 may be used	.

### 6.2.5	Test requirement

For each declared conformance beam direction pair, the EIRP measurement results in subclause 6.2.4.2 shall remain within the values provided in table 6.2.5-1, relative to the manufacturer's declared rated beam EIRP (D.11) value:

Table 6.2.5-1: Test requirement for radiated transmit power

|  | Normal test environment | Extreme test environment |
| --- | --- | --- |
| BS type 1-H | f ≤ 3 GHz: ± 3.3 dB | N/A |
|  | 3 GHz < f ≤ 6 GHz: ± 3.5 dB |  |
| BS type 1-O | f ≤ 3 GHz: ± 3.3 dB | f ≤ 3 GHz: ± 5.2 dB |
|  | 3 GHz < f ≤ 6 GHz: ± 3.5 dB | 3 GHz < f ≤ 4.2 GHz: ± 5.3 dB |
|  |  | 4.2 GHz < f ≤ 6 GHz: ± 5.3 dB |
| BS type 2-O | 24.15 GHz < f ≤ 29.5 GHz: ± 5.1 dB 37 GHz < f ≤ 40 GHz: ± 5.4 dB … | 24.15 GHz < f ≤ 29.5 GHz: ± 7.6 dB 37 GHz < f ≤ 40 GHz: ± 7.8 dB |

## 6.3	OTA base station output power

### 6.3.1	Definition and applicability

OTA BS output power is declared as rated carrier TRP, with the output power accuracy requirement defined at the RIB during the transmitter ON period.

The BS rated carrier TRP for BS type 1-O shall be within limits as specified in table 6.3.1-1.

Table 6.3.1-1: BS rated carrier TRP limits for BS type 1-O

| BS class | Prated,c,TRP |
| --- | --- |
| Wide Area BS | (note) |
| Medium Range BS | ≤ + 47 dBm |
| Local Area BS | ≤ + 33 dBm |
| NOTE: There is no upper limit for the Prated,c,TRP of the Wide Area Base Station. |  |

There is no upper limit for the rated carrier TRP of BS type 2-O.

Despite the general requirements for the BS output power described in TS 38.104 [2] subclauses 9.3.2 – 9.3.3, additional regional requirements might be applicable.

NOTE:	In certain regions, power limits corresponding to BS classes may apply for BS type 2-O.

### 6.3.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.3.3.

### 6.3.3	Test purpose

The test purpose is to verify the accuracy of the maximum carrier TRP (Pmax,c,TRP) across the frequency range for all RIBs.

### 6.3.4	Method of test

#### 6.3.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: B, M, T; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW, MRFBW and TRFBW in single band operation; see subclause 4.9.1.

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Beams to be tested:

As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

#### 6.3.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in in annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 5, and 7.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Configure the BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

4)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model(s) in subclause 4.9.2.

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model(s) in subclause 4.9.2 on all carriers configured.

5)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

6)	Measure the radiated power for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

If the test chamber is a reverberation chamber measure TRP directly.

7)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I).

8)	Calculate TRP using the EIRP measurements.

For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 6.3.5	Test requirement

#### 6.3.5.1	BS type 1-O

The TRP measurement result in step 9 of subclause 6.3.4.2 shall remain:

-	within +3.4 dB and -3.4 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP carrier frequency f ≤ 3.0 GHz;

-	within +3.5 dB and –3.5 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

-	within +3.5 dB and –3.5 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP for carrier frequency 4.2 GHz < f ≤ 6.0 GHz.

#### 6.3.5.2	BS type 2-O

The TRP measurement result in step 9 of subclause 6.3.4.2 shall remain:

-	within +5.1 dB and -5.1 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP carrier frequency 24.25 GHz < f ≤ 29.5 GHz.

-	within +5.4 dB and –5.4 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP for carrier frequency 37 GHz < f ≤ 40 GHz.

Editor’s note: more frequency divisions for the measuring accuracy may be introduced.

## 6.4	OTA output power dynamics

### 6.4.1	General

The requirements in subclause 6.4 apply during the transmitter ON period. Transmit signal quality (as specified in subclause 6.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are single direction requirements and apply to the beam peak directions over the OTA peak directions set.

### 6.4.2	OTA RE power control dynamic range

#### 6.4.2.1	Definition and applicability

The OTA RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,EIRP) for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

#### 6.4.2.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.4.2.2.

#### 6.4.2.3	Method of test

No specific test or test requirements are defined for RE power control dynamic range. The Error Vector Magnitude test, as described in subclause 6.6 provides sufficient test coverage for this requirement.

### 6.4.3	OTA total power dynamic range

#### 6.4.3.1	Definition and applicability

The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE:	The upper limit of the OTA total power dynamic range is the BS maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

#### 6.4.3.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.4.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.4.3.3.

#### 6.4.3.3	Test purpose

The test purpose is to verify that the total power dynamic range is within the limits specified by the minimum requirement.

#### 6.4.3.4	Method of test

##### 6.4.3.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see subclause 4.9.1.

Beams to be tested:	Declared beam with the highest intended EIRP for the narrowest intended beam corresponding to the smallest BeWθ, or for the narrowest intended beam corresponding to the smallest BeWϕ (D.3, D.11).

Directions to be tested: The OTA peak directions set reference beam direction pair (D.8).

##### 6.4.3.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna..

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	For BS type 1-O, set the BS to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test models:

-	NR-FR1-TM3.1a in TS 38.141-1 [3] subclause 4.9.2.2.6 if 256QAM is supported by BS without power back off;

-	or NR-FR1-TM3.1 in TS 38.141-1 [3] subclause 4.9.2.2.5 if 256QAM is not supported by BS;

-	or NR-FR1-TM3.1in TS 38.141-1 [3] subclause 4.9.2.2.5 if 256QAM is supported by BS with power back off;

For BS type 2-O, set the BS to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test models:

-	NR-FR2-TM3.1 with 64QAM signals if 64QAM is supported by BS without power back off;

-	or NR-FR2-TM3.1 with highest modulation order supported without power back off if 64QAM is not supported by BS;

-	or NR-FR2-TM3.1with highest modulation order supported without power back off if 64QAM is supported by BS with power back off;

6)	Measure the average OFDM symbol power as defined in annex F by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	For BS type 1-O, set the BS to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test models:

-	NR-FR1-TM2a in TS 38.141-1 [3] subclause 4.9.2.2.4 if 256QAM is supported by BS;

-	or NR-FR1-TM2 in TS 38.141-1 [3] subclause 4.9.2.2.3 if 256QAM is not supported by BS;

For BS type 2-O, set the BS to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test models:

-	NR-FR2-TM2 if 64QAM is supported by BS;

-	or NR-FR2-TM2 with highest modulation order supported if 64QAM is not supported by BS;

8)	Measure the average OFDM symbol power as defined in annex F by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

The measured OFDM symbols shall not contain RS or SSB.

In addition, for multi-band RIB(s), the following steps shall apply:

9)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.4.3.5	Test requirement

##### 6.4.3.5.1	BS type 1-O

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.4.3.5.1-1.

Table 6.4.3.5.1-1: Total power dynamic range

| BS channel bandwidth (MHz) | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.5 | 10 | N/A |
| 10 | 16.7 | 13.4 | 10 |
| 15 | 18.5 | 15.3 | 12.1 |
| 20 | 19.8 | 16.6 | 13.4 |
| 25 | 20.8 | 17.7 | 14.5 |
| 30 | 21.6 | 18.5 | 15.3 |
| 40 | 22.9 | 19.8 | 16.6 |
| 50 | 23.9 | 20.8 | 17.7 |
| 60 | N/A | 21.6 | 18.5 |
| 70 | N/A | 22.3 | 19.2 |
| 80 | N/A | 22.9 | 19.8 |
| 90 | N/A | 23.4 | 20.4 |
| 100 | N/A | 23.9 | 20.9 |

NOTE:	Additional test requirements for the Error Vector Magnitude (EVM) at the lower limit of the dynamic range are defined in subclause 6.6.

##### 6.4.3.5.2	BS type 2-O

OTA total power dynamic range minimum requirement for BS type 2-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 6.4.3.5.2-1.

Table 6.4.3.5.2-1: Minimum requirement for BS type 2-O total power dynamic range

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | OTA total power dynamic range (dB) |  |  |  |
| 60 | 17.7 | 20.8 | 23.8 | N.A |
| 120 | 14.6 | 17.7 | 20.8 | 23.8 |

NOTE:	Additional test requirements for the EVM at the lower limit of the dynamic range are defined in subclause 6.6.

## 6.5	OTA transmit ON/OFF power

### 6.5.1	OTA transmitter OFF power

#### 6.5.1.1	Definition and applicability

OTA transmitter OFF power requirements apply only to TDD operation of NR BS.

OTA transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

For BS supporting intra-band contiguous CA, the transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the aggregated BS channel bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2 during the transmitter OFF period. N = SCS/15, where SCS is the smallest supported Sub Carrier Spacing in kHz in the aggregated BS channel bandwidth.

For BS type 1-O, the transmitter OFF power is defined as the output power at the co-location test antenna conducted output(s). For BS type 2-O the transmitter OFF power is defined as TRP.

For multi-band RIBs or single band RIBs supporting transmission in multiple bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

#### 6.5.1.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.2.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.2.3.

#### 6.5.1.3	Test purpose

The purpose of this test is to verify the OTA transmitter OFF power is within the limits of the minimum requirements.

#### 6.5.1.4	Method of test

Requirement is tested together with transmitter transient period, as described in subclause 6.5.2.4.

#### 6.5.1.5	Test requirements

The conformance testing of transmit OFF power is included in the conformance testing of transmitter transient period; therefore, see subclause 6.5.2.5 for test requirements.

### 6.5.2	OTA transmitter transient period

#### 6.5.2.1	Definition and applicability

The OTA transmitter transient period requirements apply only to TDD operation of BS.

The OTA transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The OTA transmitter transient period is illustrated in figure 6.5.2.1-1.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.5.2.1-1: Illustration of the relations of transmitter ON period, transmitter OFF period and transmitter transient period

For BS type 1-O, this requirement applies for RIB supporting transmission in the operating band and is measured at the co-location test antenna conducted outputs. For BS type 2-O, the requirement applies at each RIB supporting transmission in the operating band.

#### 6.5.2.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.3.3.

#### 6.5.2.3	Test purpose

The purpose of this test is to verify the OTA transmitter transient periods are within the limits of the minimum requirements.

#### 6.5.2.4	Method of test

##### 6.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

Directions to be tested:

-	The requirement for BS type 1-O is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

-	The requirement for BS type 2-O is verified by an EIRP measurement at a direction corresponding to the OTA peak directions set reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

##### 6.5.2.4.2	Procedure

###### 6.5.2.4.2.1	General procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

###### 6.5.2.4.2.2	BS type 1-O

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Place the co-location test antenna as specified in subclause 4.12.

5)	Configure the beam peak direction of the BS according to the declared beam direction pair.

6)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2.

For a BS declared to be capable of multi-carrier and/or CA operation, use the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2 on all carriers configured.

7)	Measure the mean power spectral density at the output(s) of co-location test antenna as power sum over two orthogonal polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

8)	For an BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over two orthogonal polarizations over 70/N μs filtered with a square filter of bandwidth equal to the aggregated BS channel bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is the smallest supported Sub Carrier Spacing in kHz in the aggregated BS channel bandwidth.

In addition, for a multi-band RIB, the following steps shall apply:

9)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 6.5.2.4.2.3	BS type 2-O

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model NR-FR2-TM1.1 and set of physical channels in subclause 4.9.2.

For a BS declared to be capable of multi-carrier and/or CA operation, use the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model NR-FR2-TM1.1 and set of physical channels in subclause 4.9.2 on all carriers configured.

5)	Measure the mean EIRP spectral density as the power sum over two orthogonal polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period - 3 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

NOTE: Make sure that the measurement receiver is not overloaded.

6)	For an NR BS supporting contiguous CA, measure the mean EIRP spectral density as the power sum over two orthogonal polarizations over 70/N μs filtered with a square filter of bandwidth equal to the aggregated BS channel bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period – 3 μs. N = SCS/15, where SCS is the smallest supported Sub Carrier Spacing in kHz in the aggregated BS channel bandwidth.

#### 6.5.2.5	Test requirements

##### 6.5.2.5.1	BS type 1-O

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

##### 6.5.2.5.2	BS type 2-O

The measured mean EIRP spectral density according to subclause 6.5.2.4.2 shall be less than -33.1 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 24.15 GHz < f ≤ 29.5 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density according to subclause 6.5.2.4.2 shall be less than -32.7 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 37 GHz < f ≤ 40 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

## 6.6	OTA transmitted signal quality

### 6.6.1	General

Unless otherwise stated, the requirements in clause 6.6 apply during the transmitter ON period.

### 6.6.2	OTA frequency error

#### 6.6.2.1	Definition and applicability

OTA frequency error is the measure of the difference between the actual BS transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

OTA frequency error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 6.6.2.2	Minimum Requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.6.1.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.6.1.3.

#### 6.6.2.3	Test purpose

The test purpose is to verify that OTA frequency error is within the limit specified by the minimum requirement.

#### 6.6.2.4	Method of test

Requirement is tested together with OTA modulation quality test, as described in subclause 6.6.3.

##### 6.6.2.4.1	Initial conditions

Directions to be tested: OTA coverage range reference direction (D.35).

#### 6.6.2.5	Test Requirements

The modulated carrier frequency of each NR carrier configured by the BS shall be accurate to within the accuracy range given in table 6.6.2.5-1 observed over 1 ms.

Table 6.6.2.5-1: OTA frequency error test requirement for BS type 1-O and BS type 2-O

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±(0.05 ppm + 12 Hz) |
| Medium Range BS | ±(0.1 ppm + 12 Hz) |
| Local Area BS | ±(0.1 ppm + 12 Hz) |

### 6.6.3	OTA modulation quality

#### 6.6.3.1	Definition and applicability

OTA modulation quality is defined by the difference between the measured carrier signal and an idealsignal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector.

OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 6.6.3.2	Minimum Requirement

The minimum requirement for BS type 1-O, is in TS 38.104 [2], subclause 9.6.2.2.

The minimum requirement for BS type 2-O, is in TS 38.104 [2], subclause 9.6.2.3.

#### 6.6.3.3	Test purpose

The test purpose is to verify that OTA modulation quality is within the limit specified by the minimum requirement.

#### 6.6.3.4	Method of test

##### 6.6.3.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier:

-	B and T; see subclause 4.9.1.

Base station RF bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW and TRFBW in single-band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested:

-	The OTA coverage range reference direction (D.35).

-	The OTA coverage range maximum directions (D.36).

Polarizations to be tested: For dual polarized systems the requirement shall be tested and met for both polarizations.

##### 6.6.3.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna..

4)	Configure the beamforming settings of the BS according to the direction to be tested.

5)	Set the BS to output according to the applicable test configuration in subclause 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2.

For BS type 1-O declared to be capable of single carrier operation only, set the BS to transmit a signal according to:

-	NR-FR1-TM3.1a if 256QAM is supported by BS without power back off

-	or NR-FR1-TM3.1a if 256QAM is supported by BS with power back off, at manufacturer’s declared rated output power (PRated,c,EIRP) and NR-FR1-TM3.1 at maximum power

-	or NR-FR1-TM3.1 if highest modulation order supported by BS is 64QAM

-	or NR-FR1-TM3.2 if highest modulation order supported by BS is 16QAM

-	or NR-FR1-TM3.3 if highest modulation order supported by BS is QPSK.

For BS type 1-O declared to be capable of multi-carrier and/or CA operation, set the BS to transmit according to the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test models on all carriers configured:

-	NR-FR1-TM3.1a if 256QAM is supported by BS without power back off

-	or NR-FR1-TM3.1a if 256QAM is supported by BS with power back off, at manufacturer’s declared rated output power (PRated,c,EIRP) and NR-FR1-TM3.1 at maximum power

-	or NR-FR1-TM3.1 if highest modulation order supported by BS is 64QAM

-	or NR-FR1-TM3.2 if highest modulation order supported by BS is 16QAM

-	or NR-FR1-TM3.3 if highest modulation order supported by BS is QPSK.

For BS type 2-O declared to be capable of single carrier operation only, set the BS to transmit a signal according to the applicable test signal configuration and corresponding power setting specified in subclause 4.7.2 and 4.8 using the corresponding test models on all carriers configured:

-	NR-FR2-TM3.1 with 64QAM signal if 64QAM is supported by BS without power back off

-	or NR-FR2-TM 3.1 with highest modulation order without power back off if 64QAM is not supported by BS

-	or if 64 QAM is supported by BS with power back off, NR-FR2-TM 3.1 with 64QAM at manufacturer’s declared rated output power (PRated,c,EIRP) and NR-FR2-TM3.1 with highest modulation order supported at maximum power.

For BS type 2-O declared to be capable of multi-carrier and/or CA operation, set the BS to transmit according to:

-	NR-FR2-TM3.1 with 64QAM signal if 64QAM is supported by BS without power back off

-	or NR-FR2-TM3.1 with highest modulation order supported without power back off if 64QAM is not supported by BS

-	or if 64QAM is supported by BS with power back off, NR-FR2-TM3.1 with 64QAM signal at manufacturer’s declared rated output power (PRated,c,EIRP) and NR-FR2-TM3.1 with highest supported modulation order at maximum power

For NR-FR1-TM 3.1a and NR-FR2-TM 3.1, power back-off shall be applied if it is declared.

6)	For each carrier, measure the EVM and frequency error as defined in annex L.

7)	Repeat steps 5 and 6 for NR-FR1-TM2 if 256QAM is not supported by BS type 1-O or for NR-FR1-TM2a if 256QAM is supported by BS type 1-O. For NR-FR1-TM2 and NR-FR1-TM2a the OFDM symbol power (in the conformance direction) shall be at the lower limit of the dynamic range according to the test procedure in subclause 6.4.3.4.2 and test requirements in subclause 6.4.3.5.1.

Repeat steps 5 and 6 for NR-FR2-TM2 for BS type 2-O. For NR-FR2-TM2 the OFDM symbol power (in the conformance direction) shall be at the lower limit of the dynamic range according to the test procedure in subclause 6.4.3.4.2 and test requirements in subclause 6.4.3.5.2..

In addition, for multi-band RIB, the following steps shall apply:

8)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.6.3.5	Test requirements

##### 6.6.3.5.1	BS type 1-O

For BS type 1-O, the EVM of each NR carrier for different modulation schemes on PDSCH shall be less than the limits in table 6.6.3.5.1-1.

Table 6.6.3.5.1-1: EVM requirements for BS type 1-O

| Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 18.5 |
| 16QAM | 13.5 |
| 64QAM | 9 |
| 256QAM | 4.5 |

EVM shall be evaluated for each NR carrier over all allocated resource blocks and downlink slots. Different modulation schemes listed in table 6.6.3.5.1-1 shall be considered for rank 1.

For NR, for all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink slots within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

Table 6.6.3.5.1-2, 6.6.3.5.1-3, 6.6.3.5.1-4 below specify the EVM window length (W) for normal CP for BS type 1-O.

Table 6.6.3.5.1-2: EVM window length for normal CP, FR1, 15 kHz SCS

| Channel bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| 10 | 1024 | 72 | 28 | 40 |
| 15 | 1536 | 108 | 44 | 40 |
| 20 | 2048 | 144 | 58 | 40 |
| 25 | 2048 | 144 | 72 | 50 |
| 30 | 3072 | 216 | 108 | 50 |
| 40 | 4096 | 288 | 144 | 50 |
| 50 | 4096 | 288 | 144 | 50 |
| NOTE: These percentages are informative and apply to a slot’s symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

Table 6.6.3.5.1-3: EVM window length for normal CP, FR1, 30 kHz SCS

| Channel bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | 8 | 40 |
| 10 | 512 | 36 | 14 | 40 |
| 15 | 768 | 54 | 22 | 40 |
| 20 | 1024 | 72 | 28 | 40 |
| 25 | 1024 | 72 | 36 | 50 |
| 30 | 1536 | 108 | 54 | 50 |
| 40 | 2048 | 144 | 72 | 50 |
| 50 | 2048 | 144 | 72 | 50 |
| 60 | 3072 | 216 | 130 | 60 |
| 70 | 3072 | 216 | 130 | 60 |
| 80 | 4096 | 288 | 172 | 60 |
| 90 | 4096 | 288 | 172 | 60 |
| 100 | 4096 | 288 | 172 | 60 |
| NOTE: These percentages are informative and apply to a slot’s symbols 1 through 13. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Table 6.6.3.5.1-4: EVM window length for normal CP for NR, FR1, 60 kHz SCS

| Channel bandwidth (MHz) | FFT size | Cyclic prefix length -in FFT samples | EVM window length W | Ratio of W to total CP- (Note) (%) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 18 | 8 | 40 |
| 15 | 384 | 27 | 11 | 40 |
| 20 | 512 | 36 | 14 | 40 |
| 25 | 512 | 36 | 18 | 50 |
| 30 | 768 | 54 | 26 | 50 |
| 40 | 1024 | 72 | 36 | 50 |
| 50 | 1024 | 72 | 36 | 50 |
| 60 | 1536 | 108 | 64 | 60 |
| 70 | 1536 | 108 | 64 | 60 |
| 80 | 2048 | 144 | 86 | 60 |
| 90 | 2048 | 144 | 86 | 60 |
| 100 | 2048 | 144 | 86 | 60 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 2. Symbol 0 of slot 0 and slot 2 may have a longer CP and therefore a lower percentage. |  |  |  |  |

##### 6.6.3.5.2	BS type 2-O

For BS type 2-O, the EVM of each NR carrier for different modulation schemes on PDSCH shall be less than the limits in table 6.4.3.5.2-1.

Table 6.6.3.5.2-1: EVM requirements for BS type 2-O

| Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 18.5 |
| 16QAM | 13.5 |
| 64QAM | 9 |

EVM requirements shall apply for each NR carrier over all allocated resource blocks and downlink slots. PT-RS should be configured for localized setting for every fourth symbol for every second RB. Different modulation schemes listed in table 6.6.3.5.2-1 shall be considered for rank 1.

For NR, for all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink slots within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

Table 6.6.3.5.2-2 and 6.6.3.5.2-3 below specify the EVM window length (W) for normal CP for BS type 2-O.

Table 6.6.3.5.2-2: EVM window length for normal CP, FR2, 60 kHz SCS

| Channel bandwidth (MHz) | FFT size | Cyclic prefix lengthin FFT samples | EVM window length W | Ratio of W to total CP length -(Note) (%) |
| --- | --- | --- | --- | --- |
| 50 | 1024 | 72 | 36 | 50 |
| 100 | 2048 | 144 | 72 | 50 |
| 200 | 4096 | 288 | 144 | 50 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 2. Symbol 0 of slot 0 and slot 2 may have a longer CP and therefore a lower percentage. |  |  |  |  |

Table 6.6.3.5.2-3: EVM window length for normal CP, FR2, 120 kHz SCS

| Channel bandwidth (MHz) | FFT size | Cyclic prefix length in FFT samples | EVM window length W | Ratio of W to total CP length -(Note) (%) |
| --- | --- | --- | --- | --- |
| 50 | 512 | 36 | 18 | 50 |
| 100 | 1024 | 72 | 36 | 50 |
| 200 | 2048 | 144 | 72 | 50 |
| 400 | 4096 | 288 | 144 | 50 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 4. Symbol 0 of slot 0 and slot 4 may have a longer CP and therefore a lower percentage. |  |  |  |  |

### 6.6.4	OTA time alignment error

#### 6.6.4.1	Definition and applicability

This requirement shall apply to frame timing in MIMO transmission, carrier aggregation and their combinations.

Frames of the NR signals present in the radiated domain are not perfectly aligned in time. In relation to each other, the RF signals present in the radiated domain may experience certain timing differences.

For a specific set of signals/transmitter configuration/transmission mode, the OTA Time Alignment Error (OTA TAE) is defined as the largest timing difference between any two different NR signals. The OTA time alignment error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 6.6.4.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.6.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.6.3.3.

#### 6.6.4.3	Test purpose

To verify that the OTA time alignment error is within the limit specified by the minimum requirement.

#### 6.6.4.4	Method of test

##### 6.6.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested: OTA coverage range reference direction (D.35).

Polarizations to be tested: For dual polarized systems the requirement shall be tested and met considering both polarisations. If the measurement antenna does not support dual polarization, time alignment error shall be measured under the condition that measurement antenna is aligned between the BS polarisations such that it receives half the power from each polarisation.

##### 6.6.4.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna..

4)	Configure the beamforming settings of the BS according to the direction of the testing.

5)	Set the BS type 1-O to transmit NR-FR1-TM1.1 or any DL signal using MIMO transmission or carrier aggregation, using the configuration with the minimum number of cells and reference signals.

Set the BS type 2-O to transmit NR-FR2-TM 1.1 or any DL signal using MIMO transmission or carrier aggregation, using the configuration with the minimum number of cells and reference signals.

NOTE:	For MIMO transmission, different ports may be configured in NR-FR1-TM1.1 and NR-FR2-TM 1.1 (using DMRS ports p = 1000 and 1001 with CDM).

For an  BS declared to be capable of single carrier operation only, set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model at manufacturer's declared rated output power, PRated,c,TRP.

If the BS supports intra band contiguous or non-contiguous Carrier Aggregation set the BS to transmit using the applicable test configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8.

If the BS supports inter band carrier aggregation set the BS to transmit, for each band, a single carrier or all carriers, using the applicable test configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8.

For BS type 1-O declared to be capable of multi-carrier operation, set the BS to transmit according to the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model on all carriers configured.

For BS type 2-O declared to be capable of multi-carrier operation, set the BS to transmit according to the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model on all carriers configured.

6)	Measure the time alignment error between the different reference symbols on different beams on the carrier(s).

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.6.4.5	Test Requirement

##### 6.6.4.5.1	BS type 1-O

For MIMO transmission, at each carrier frequency, OTA TAE shall not exceed 90 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 285 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3.025 µs.

For inter-band carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3.025 µs.

##### 6.6.4.5.2	BS type 2-O

For MIMO transmission, at each carrier frequency, OTA TAE shall not exceed 90 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 155 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 285 ns.

For inter-band carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3.025 µs.

## 6.7	OTA unwanted emissions

### 6.7.1	General

OTA unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 [5]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the BS channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The OTA out-of-band emissions requirement for the BS type 1-O and BS type 2-O transmitter is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and operating band unwanted emissions (OBUE). The OTA Operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. OTA Unwanted emissions outside of this frequency range are limited by an OTA spurious emissions requirement.

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The value of ΔfOBUE is defined in table 6.7.1-1 for BS type 1-O and BS type 2-O for the NR operating bands.

Table 6.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL_high – FDL_low ≤ 900 MHz | 40 |
| BS type 2-O | FDL_high – FDL_low ≤ 3250 MHz | 1500 |

The OTA unwanted emission requirements are applied per cell for all the configurations. Requirements for OTA unwanted emissions are captured using TRP, directional requirements or co-location requirements as described per requirement.

There is in addition a requirement for OTA occupied bandwidth.

### 6.7.2	OTA occupied bandwidth

#### 6.7.2.1	Definition and applicability

The OTA occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also recommendation ITU-R SM.328 [13].

The value of /2 shall be taken as 0.5%.

The OTA occupied bandwidth requirement applies during the transmitter ON period for a single transmitted carrier. The minimum requirement below may be applied regionally. There may also be regional requirements to declare the OTA occupied bandwidth according to the definition in the present clause.

The OTA occupied bandwidth is defined as a directional requirement and shall be met in the manufacturer’s declared OTA coverage range at the RIB.

#### 6.7.2.2	Minimum requirement

The minimum requirement for BS type 1-O and BS type 2-O is in TS 38.104 [2], subclause 9.7.2.2.

#### 6.7.2.3	Test purpose

The test purpose is to verify that the emission at the RIB does not occupy an excessive bandwidth for the service to be provided and is, therefore, not likely to create interference to other users of the spectrum beyond undue limits.

#### 6.7.2.4	Method of test

##### 6.7.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Directions to be tested: OTA coverage range reference direction (D.35).

Beams to be tested: Declared beam with the highest intended EIRP for the narrowest intended beam corresponding to the smallest BeWθ, or for the narrowest intended beam corresponding to the smallest BeWϕ (D.3, D.11).

Aggregated BS channel bandwidth positions to be tested for contiguous carrier aggregation: MBW Channel CA; see subclause 4.9.1.

For a BS declared to be capable of single carrier operation, start transmission according to the applicable test configuration in subclause 4.8 using the corresponding test model NR-FR1-TM1.1 for BS type 1-O or NR-FR2-TM1.1 for BS type 2-O in subclause 4.9.2 at manufacturers declared rated carrier EIRP (PRated,c,EIRP, D.11).

For a BS declared to be capable of contiguous carrier aggregation operation, set the base station to transmit according to NR-FR1-TM1.1 for BS type 1-O or NR-FR2-TM1.1 for BS type 2-O in subclause 4.9.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclauses 4.7.2.3.1 and 4.8.

##### 6.7.2.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna..

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	Set the BS to transmit signal.

6)	Measure the spectrum emission of the transmitted signal using at least the number of measurement points, and across a span, as listed in table 6.7.2.4.2-1 and table 6.7.2.4.2-2. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less.

NOTE:	The detection mode of the spectrum analyzer will not have any effect on the result if the statistical properties of the out-of-OBW power are the same as those of the inside-OBW power. Both are expected to have the Rayleigh distribution of the amplitude of Gaussian noise. In any case where the statistics are not the same, though, the detection mode is power responding. There are at least two ways to be power responding. The spectrum analyser can be set to "sample" detection, with its video bandwidth setting at least three times its RBW setting. Or the analyser may be set to respond to the average of the power (root-mean-square of the voltage) across the measurement cell.

Table 6.7.2.4.2-1: Span and number of measurement points for OBW measurements for FR1

| Bandwidth | BS channel bandwidth BWChannel (MHz) |  |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | > 20 | > 20 |
| Span (MHz) | 10 | 20 | 30 | 40 | 2×BWChannel |  |
| Minimum number of measurement points | 400 | 400 | 400 | 400 | 2×BWChannel100kHz |  |

Table 6.7.2.4.2-2: Span and number of measurement points for OBW measurements for FR2

| Bandwidth | BS channel bandwidth BWChannel (MHz) |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 | > 50 |
| Span (MHz) |  |  |  |  |  |
| Minimum number of measurement points |  |  |  |  |  |

7)	Compute the total of the EIRP, P0, (in power units, not decibel units) of all the measurement cells in the measurement span. Compute P1, the EIRP outside the occupied bandwidth on each side. P1 is half of the total EIRP outside the bandwidth. P1 is half of (100 % - (occupied percentage)) of P0. Measure the EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

8)	Determine the lowest frequency, f1, for which the sum of all EIRP in the measurement cells from the beginning of the span to f1 exceeds P1.

9)	Determine the highest frequency, f2, for which the sum of all EIRP in the measurement cells from the end of the span to f2 exceeds P1.

10)	Compute the OTA occupied bandwidth as f2 - f1.

In addition, for multi-band RIB(s), the following steps shall apply:

11) For multi-band RIBs and single band tests, repeat the steps 6) - 10) above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

#### 6.7.2.5	Test requirement

##### 6.7.2.5.1	BS type 1-O

The OTA occupied bandwidth for each NR carrier shall be less than the channel bandwidth as defined in TS 38.104 [2], table 5.3.5-1. For contiguous CA, the occupied bandwidth shall be less than or equal to the aggregated BS channel bandwidth as defined in TS 38.104 [2], subclause 5.3A.

##### 6.7.2.5.2	BS type 2-O

The OTA occupied bandwidth for each NR carrier shall be less than the channel bandwidth as defined in TS 38.104 [2], table 5.3.5-2. For contiguous CA, the occupied bandwidth shall be less than or equal to the aggregated BS channel bandwidth as defined in TS 38.104 [2], subclause 5.3A.

### 6.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

#### 6.7.3.1	Definition and applicability

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

For both BS type 1-O and BS type 2-O, for a RIB operating in multi-carrier or contiguous CA, the OTA ACLR requirements in subclause 6.7.3.2 apply to BS channel bandwidths of the outermost carrier.

For BS type 1-O, for a RIB operating in non-contiguous spectrum, the OTA ACLR requirements in subclause 6.7.3.2 shall apply inside sub-block gaps for the frequency ranges defined in table 6.7.3.5.1-2a, while the CACLR requirement in subclause 6.7.3.2 shall apply in sub block gaps for the frequency ranges defined in table 6.7.3.5.1-3. In addition, for a multi-band RIB, the ACLR requirement in subclause 6.7.3.2 shall apply in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.7.3.5.1-2a, while the CACLR requirement in subclause 6.7.3.2 shall apply in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.7.3.5.1-3.

For BS type 2-O, for a RIB operating in non-contiguous spectrum, the OTA ACLR requirements in subclause 6.7.3.2 shall apply inside any sub-block gap for the frequency ranges defined in table 6.7.3.5.2-3, while the CACLR requirement in subclause 6.6.3.2 shall apply in sub block gaps for the frequency ranges defined in table 6.7.3.5.2-3.

The requirement shall be applied per RIB during the transmitter ON period.

#### 6.7.3.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.7.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.7.3.3.

#### 6.7.3.3	Test purpose

To verify that the OTA adjacent channel leakage ratio requirement shall be met as specified by the minimum requirement.

#### 6.7.3.4	Method of test

##### 6.7.3.4.1	Initial conditions

Test environment:	normal; see annex B.2.

RF channels to be tested for single carrier:	B and T; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW and TRFBW in single-band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operaton, see subclause 4.9.1.

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

##### 6.7.3.4.2	Procedure

##### The following procedure for measuring TRP is based on the directional power measurements as described in in Annex I, an alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so follow steps 1, 3, 4, 6, 8, 9, 10, 11, 12 and 13.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)

The measurement devices characteristics shall be:

- measurement filter bandwidth: defined in subclause 6.7.3.5.

- detection mode: true RMS voltage or true power averaging.

4)	For single carrier operation, set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model(s) in subclause 4.9.2 at manufacturers declared rated carrier output power (PRated,c,TRP).

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model(s) in subclause 4.9.2 on all carriers configured.

5)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

6)	Measure the absolute power of the assigned channel frequency and the (adjacent channel frequency).

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex I).

8)	Calculate TRPEstimate for the absolute total radiated power of the wanted channel and the adjacent channel using the measurements made in Step 7.

9) 	Calculate relative ACLR estimate.

NOTE 1:	ACLR is calculated by the ratio of the absolute TRP of the assigned channel frequency and the absolute TRP of the adjacent frequency channel.

NOTE 2:	For FR1 the measurement uncertainty of the reverberation chamber for the relative ACLR is higher than the measurement uncertainty in subclause 4.1.2 the test requirements in table 6.7.3.5.1-1 should be offset by 0.4 dB (0 to 3 GHz) and 0.3 dB (3 to 6 GHz).

10)	Measure OTA ACLR for the frequency offsets both side of channel frequency as specified in table 6.7.3.5.1-1 for BS type 1-O or table 6.7.3.5.2-1for BS type 2-O respectively. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

11)	For the OTA ACLR requirement applied inside sub-block gap for non-contiguous spectrum operation or inside Inter RF Bandwidth gap for multi-band operation:

a)	Measure OTA ACLR inside sub-block gap or Inter RF Bandwidth gap, if applicable.

b)	Measure OTA CACLR inside sub-block gap or Inter RF Bandwidth gap, if applicable.

12)	Repeat the test with the channel set-up using NR- FR1-TM1.2 defined in subclause 4.9.2 in TS 38.141-1 [3] for BS type 1-O.

In addition, for multi-band RIB, the following steps shall apply:

13)	For BS type 1-O and multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.7.3.5	Test requirements

##### 6.7.3.5.1	BS type 1-O

For the OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.1-1/2a or the OTA ACLR absolute limits in table 6.7.3.5.1-2 shall apply, whichever is less stringent. The OTA CACLR limits in table 6.7.3.5.1-3 or the OTA CACLR absolute limits in table 6.7.3.5.1-3a shall apply, whichever is less stringent.

The CACLR in a sub-block gap and Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.1-3 and the filters on the assigned channels are defined in table 6.7.3.5.1-4.

For operation in paired and unpaired spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1-1.

Table 6.7.3.5.1-1: BS type 1-O ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0 – 3 GHz) | OTA ACLR limit (3 – 6 GHz) |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.1-2.

Table 6.7.3.5.1-2: BS type 1-O ACLR absolute limit

| BS category / BS class | OTA ACLR absolute limit |
| --- | --- |
| Category A Wide Area BS | -4 dBm/MHz |
| Category B Wide Area BS | -6 dBm/MHz |
| Medium Range BS | -16 dBm/MHz |
| Local Area BS | -23 dBm/MHz |
| NOTE 1: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 2: Void |  |

For operation in non-contiguous spectrum or multiple bands, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1-2a.

Table 6.7.3.5.1-2a: BS type 1-O ACLR limit in non-contiguous spectrum or multiple bands

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0-3GHz) | OTA ACLR limit (3-6GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |  |

The OTA CACLR measurement result shall not less than the OTA CACLR limit specified in table 6.7.3.5.1-3.

Table 6.7.3.5.1-3: BS type 1-O CACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA CACLR limit (0-3 GHz) | OTA CACLR limit (3-6 GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | 5 ≤ Wgap < 15 (Note 3) 5 ≤ Wgap < 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 10 < Wgap < 20 (Note 3) 10 ≤ Wgap < 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| 25, 30, 40, 50, 60, 70, 80,90, 100 | 20 ≤ Wgap < 60 (Note 4) 20 ≤ Wgap < 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 40 < Wgap < 80 (Note 4) 40 ≤ Wgap < 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA CACLR absolute limit specified in table 6.7.3.5.1-3a.

Table 6.7.3.5.1-3a: BS type 1-O CACLR absolute limit

| BS category / BS class | OTA CACLR absolute limit |
| --- | --- |
| Category A Wide Area BS | -4 dBm/MHz |
| Category B Wide Area BS | -6 dBm/MHz |
| Medium Range BS | -16 dBm/MHz |
| Local Area BS | -23 dBm/MHz |
| NOTE 1: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 2: Void |  |

Table 6.7.3.5.1-4: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| NR | NR of same BW with SCS that provides largest transmission bandwidth configuration |

##### 6.7.3.5.2	BS type 2-O

For the OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.2-1/3 or the OTA ACLR absolute limits in table 6.7.3.5.2-2 shall apply, whichever is less stringent. The OTA CACLR limits in table 6.7.3.5.2-4 or the OTA CACLR absolute limits in table 6.7.3.5.2-4a shall apply, whichever is less stringent.

The CACLR in a sub-block gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.2-4 and the filters on the assigned channels are defined in table 6.7.3.5.2-5.

The OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.2-1.

Table 6.7.3.5.2-1: BS type 2-O ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |  |  |

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.2-2

Table 6.7.3.5.2-2: BS type 2-O ACLR absolute limit

| BS class | ACLR absolute limit |
| --- | --- |
| Wide-area BS | -10.3dBm/MHz |
| Medium-range BS | -17.3 dBm/MHz |
| Local-area BS | -17.3 dBm/MHz |

For operation in non-contiguous spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.2-3.

Table 6.7.3.5.2-3: BS type 2-O ACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest NR carrier transmitted (MHz) | Sub-block gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (MHz) |
| --- | --- | --- | --- | --- | --- |
| 50, 100 | Wgap ≥ 100 (Note 5) Wgap ≥ 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| 200, 400 | Wgap ≥ 400 (Note 6) Wgap ≥ 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50 or 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200 or 400 MHz. |  |  |  |  |  |

For operation in non-contiguous spectrum, the CACLR for carriers located on either side of the sub-block gap shall be less than the value specified in table 6.7.3.5.2-4.

Table 6.7.3.5.2-4: BS type 2-O CACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest NR carrier transmitted (MHz) |  | Sub-block gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA CACLR limit (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 50, 100 |  | 50 ≤ Wgap < 100 (Note 5) 50 ≤ Wgap < 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| 200, 400 |  | 200 ≤ Wgap < 400 (Note 6) 200 ≤ Wgap < 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50 or 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200 or 400 MHz. |  |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA CACLR absolute limit specified in table 6.7.3.5.2-4a.

Table 6.7.3.5.2-4a: BS type 2-O CACLR absolute limit

| BS class | CACLR absolute limit |
| --- | --- |
| Wide area BS | -10.3 dBm/MHz |
| Medium range BS | -17.3 dBm/MHz |
| Local area BS | -17.3 dBm/MHz |

Table 6.7.3.5.2-5: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| NR | NR of same BW with SCS that provides largest transmission bandwidth configuration |

### 6.7.4	OTA operating band unwanted emissions

#### 6.7.4.1	Definition and applicability

The OTA limits for operating band unwanted emissions are specified as TRP per RIB, unless otherwise stated.

For BS type 1-O, for a RIB operating in multi-carrier or contiguous CA, the requirements apply to BS channel bandwidths of the outermost carrier. In addition, for a RIB operating in non-contiguous spectrum, the requirements shall apply inside any sub-block gap. In addition, for a multi-band RIB, the requirements shall apply inside any Inter RF Bandwidth gap.

For BS type 2-O, for a RIB operating in multi-carrier or contiguous CA, the requirements apply to the frequencies (ΔfOBUE) starting from the edge of the contiguous transmission bandwidth. In addition, for a RIB operating in non-contiguous spectrum, the requirements apply inside any sub-block gap.

#### 6.7.4.2	Minimum requirement

The minimum requirement for BS type 1-O is defined in TS 38.104 [2], subclause 9.7.4.2.

The minimum requirement for BS type 2-O is defined in TS 38.104 [2], subclause 9.7.4.3.

#### 6.7.4.3	Test purpose

This test measures the emissions of the BS, close to the assigned channel bandwidth of the wanted signal, while the BS is in operation.

#### 6.7.4.4	Method of test

##### 6.7.4.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier: B, M and T; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW, MRFBW and TRFBW in single-band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

##### 6.7.4.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in in annex I, an alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so follow steps 1, 3, 4, 6 and 9.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)

The measurement devices characteristics shall be:

- measurement filter bandwidth: defined in subclause 6.7.4.5.

- detection mode: true RMS voltage or true power averaging.

4)	For single carrier operation, set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model(s) in subclause 4.9.2 at manufacturers declared rated carrier output power (PRated,c,TRP).

For a BS declared to be capable of multi-carrier and/or CA operation, use the applicable test signal configuration and corresponding power setting specified in subclause 4.7.2 and 4.8 using the corresponding test model(s) in subclause 4.9.2 on all carriers configured.

5) Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

6)	Sweep the centre frequency of the measurement filter in contiguous steps and measure emission power within the specified frequency ranges with the specified measurement bandwidth.

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex I).

8)	Calculate TRPEstimate using the measurements made in step 6.

9) For BS type 1-O and multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.7.4.5	Test requirements

##### 6.7.4.5.1	BS type 1-O

The emission measurement result shall not exceed the maximum levels specified in tables 6.7.4.5.1.1-1 to 6.7.4.5.1.5-3, where:

-	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band RIB inside any Inter RF Bandwidth gaps with Wgap < 2*ΔfOBUE, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The test requirement for Base Station RF Bandwidth edge is specified in the tables 6.7.4.5.1.1-1 to 6.7.4.5.1.5-3 below, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the Base Station RF Bandwidth edge.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band RIB, the operating band unwanted emission limits apply also in a supported operating band without any carrier transmitted, in the case where there are carrier(s) transmitted in another supported operating band. In this case, no cumulative limit is applied in the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted and

-	In case the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted is less than 2*ΔfOBUE, f_offsetmax shall be the offset to the frequency ΔfOBUE MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause, shall apply across both downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause for the largest frequency offset (fmax), shall apply from ΔfOBUE MHz below the lowest frequency, up to ΔfOBUE MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

For a multicarrier single-band RIB or a single-band RIB configured for intra-band contiguous or non-contiguous carrier aggregation the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the upper edge of the carrier transmitted at the highest carrier frequency within a specified frequency band.

In addition inside any sub-block gap for a single-band RIB operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The test requirement for each sub block is specified in the tables 6.7.4.5.1.1-1 to 6.7.4.5.1.5-3 below, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

###### 6.7.4.5.1.1	Wide Area BS (Category A)

For a RIB operating in Bands n5, n8, n12, n14, n28, n71, emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.1-1.

Table 6.7.4.5.1.1-1: Wide Area BS operating band unwanted emission limits 
(NR bands ≤ 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm - 7/5(f_offset/MHz - 0.05) dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -4 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -4 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

For a RIB operating in Bands n1, n2, n3, n7, n25, n30, n34, n38, n39, n40, n41, n50, n65, n66, n70, n74, n75, n77, n78, n79,  emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.1-2 to 6.7.4.5.1.1-4:

Table 6.7.4.5.1.1-2: Wide Area BS operating band unwanted emission limits 
(1 GHz < NR bands ≤ 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -4 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.1-3: Wide Area BS operating band unwanted emission limits 
(3 GHz < NR bands ≤ 4.2 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 4 dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -4 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.1-4: Wide Area BS operating band unwanted emission limits 
(4.2 GHz < NR bands ≤ 6 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 4 dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -4 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

###### 6.7.4.5.1.2	Wide Area BS Category B (Option 1)

For Category B operating band unwanted emissions, there are two options for the limits that may be applied regionally. option 1 is as follows.

For a RIB operating in Bands n5, n8, n12, n20, n28, n71, emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.2-1:

Table 6.7.4.5.1.2-1: Wide Area BS operating band unwanted emission limits 
(NR bands ≤ 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -7 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -7 dBm/ 100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

For a RIB operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n50, n65, n66, n70, n75, n77, n78, n79, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.2-2 to 6.7.4.5.1.2-4:

Table 6.7.4.5.1.2-2: Wide Area BS operating band unwanted emission limits 
(1 GHz < NR bands ≤ 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.2-3: Wide Area BS operating band unwanted emission limits 
(3 GHz < NR bands ≤ 4.2 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 4 dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.2-4: Wide Area BS operating band unwanted emission limits 
(4.2 GHz < NR bands ≤ 6 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 4 dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

###### 6.7.4.5.1.3	Wide Area BS Category B (Option 2)

The limits in this subclause are intended for Europe and may be applied regionally for a RIB operating in bands n1, n3, n8.

For a RIB operating in bands n1, n3, n8, n65 emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.3-1:

Table 6.7.4.5.1.3-1: Regional Wide Area BS operating band unwanted emission limits for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limit (Note 1, 2, 5) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015 MHz f_offset < 0.215 MHz | -3.2 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 4) | 1.015 MHz f_offset < 1.5 MHz | -15.2 dBm | 30 kHz |
| 1 MHz f min( 10 MHz, fmax) | 1.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -2.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | - 6 dBm (Note 3) | 1 MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -6 dBm/1MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 6: Void |  |  |  |

###### 6.7.4.5.1.4	Medium Range BS (Category A and Category B)

For Medium Range BS class in NR bands ≤ 3 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.4-1 and 6.7.4.5.1.4-4.

For Medium Range BS class in 3GHz <NR bands ≤ 4.2 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.4-2 and 6.7.4.5.1.4-5.

For Medium Range BS class in 4.2GHz <NR bands ≤ 6 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.4-3 and 6.7.4.5.1.4-6.

Table 6.7.4.5.1.4-1: Medium Range BS operating band unwanted emission limits, 40 < Prated,c,TRP  47 dBm (NR bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP – 58.2 dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP - 60 dB, -16 dBm) (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,c,TRP – 60 dB, -16 dBm)/100kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.4-2: Medium Range BS operating band unwanted emission limits, 40 < Prated,c,TRP  47 dBm ( 3 GHz < NR bands ≤ 4.2 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP - 58 dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP – 60 dB, -16 dBm) (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,c,TRP – 60 dB, -16 dBm)/100kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.4-3: Medium Range BS operating band unwanted emission limits, 40 < Prated,c,TRP  47 dBm ( 4.2 GHz < NR bands ≤ 6 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP - 58 dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP – 60 dB, -16 dBm) (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,c,TRP – 60 dB, -16 dBm)/100kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.4-4: Medium Range BS operating band unwanted emission limits, Prated,c,TRP  40 dBm

(NR bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -18.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -20 dBm/100kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.4-5: Medium Range BS operating band unwanted emission limits, Prated,c,TRP  40 dBm

(3 GHz < NR bands ≤ 4.2 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -18 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -20 dBm/100kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.4-6: Medium Range BS operating band unwanted emission limits, Prated,c,TRP  40 dBm

(4.2 GHz < NR bands ≤ 6 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -18 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -20 dBm/100kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

###### 6.7.4.5.1.5	Local Area BS (Category A and Category B)

For Local Area BS class in NR bands ≤ 3 GHz, emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.5-1.

For Local Area BS class in 3 GHz < NR bands ≤ 4.2 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.5-2.

For Local Area BS class in 4.2 GHz < NR bands ≤ 6 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.5-3.

Table 6.7.4.5.1.5-1: Local Area BS operating band unwanted emission limits (NR bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -26.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -28 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -28 dBm/100kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.5-2: Local Area BS operating band unwanted emission limits (3 GHz < NR bands ≤ 4.2 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -26 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -28 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -28 dBm/100kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

Table 6.7.4.5.1.5-3: Local Area BS operating band unwanted emission limits (4.2 GHz < NR bands ≤ 6 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2, 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -26 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -28 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -28 dBm/100kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void |  |  |  |

###### 6.7.4.5.1.6	Additional requirements

6.7.4.5.1.6.1	Limits in FCC Title 47

In addition to the requirements in tables 6.7.4.5.1.1-1 to 6.7.4.5.1.5-3, the BS may have to comply with the applicable emission limits established by FCC Title 47 [14], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

6.7.4.5.1.6.2	Protection of DTT

In certain regions the following requirement may apply for protection of DTT. For BS type 1-O operating in Band n20, the level of emissions in the band 470-790 MHz, measured in an 8 MHz filter bandwidth on centre frequencies Ffilter according to table 6.7.4.5.1.6.2-1, shall not exceed the maximum emission TRP level shown in the table. This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.7.4.5.1.6.2-1: Declared emissions levels for protection of DTT

| Case | Measurement filter centre frequency | Condition on BS maximum aggregate EIRP / 10 MHz, PEIRP_10MHz (NOTE) | Maximum level PEIRP,N,MAX | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| A: for DTT frequencies where broadcasting is protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz 59 dBm | 0 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PEIRP_10MHz < 59 dBm | PEIRP_10MHz – 59 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz < 36 dBm | -23 dBm | 8 MHz |
| B: for DTT frequencies where broadcasting is subject to an intermediate level of protection | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz 59 dBm | 10 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PEIRP_10MHz < 59 dBm | PEIRP_10MHz – 49 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz < 36 dBm | -13 dBm | 8 MHz |
| C: for DTT frequencies where broadcasting is not protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | N.A. | 22 dBm | 8 MHz |
| NOTE: PEIRP_10MHz (dBm) is defined by PEIRP_10MHz = P10MHz + Gant + 9dB, where Gant is 17 dBi. |  |  |  |  |

##### 6.7.4.5.2	BS type 2-O

###### 6.7.4.5.2.1	General

The requirements of either subclause 6.7.4.5.2.2 (Category A limits) or subclause 6.7.4.5.2.3 (Category B limits) shall apply. The application of either Category A or Category B limits shall be the same as for General OTA transmitter spurious emissions requirements (BS type 2-O) in subclause 6.7.5.2.5.2.The emission measurement result shall not exceed the maximum levels specified in the tables below, where:

-	f is the separation between the contiguous transmission bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the contiguous transmission bandwidth edge.

-	f_offset is the separation between the contiguous transmission bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE outside the downlink operating band, where ΔfOBUE is defined in table 6.7.1-1.

In addition, inside any sub-block gap for a RIB operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The test requirement for each sub-block is specified in the subclauses 6.7.4.5.2.2 and 6.7.4.5.2.3 below, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.7.4.5.2-1: Void

Table 6.7.4.5.2-2: Void

###### 6.7.4.5.2.2	OTA operating band unwanted emission limits (Category A)

The power of any spurious emission shall not exceed the limits in table 6.7.4.5.2.2-1 or 6.7.4.5.2.2-2.

Table 6.7.4.5.2.2-1: OBUE limits applicable in the frequency range 24.25 – 33.4 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 32.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| NOTE: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. |  |  |  |

Table 6.7.4.5.2.2-2: OBUE limits applicable in the frequency range 37 GHz – 52.6 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 30.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| NOTE: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. |  |  |  |

###### 6.7.4.5.2.3	OTA operating band unwanted emission limits (Category B)

The power of any spurious emission shall not exceed the limits in table 6.7.4.5.2.3-1 or 6.7.4.5.2.3-2.

Table 6.7.4.5.2.3-1: OBUE limits applicable in the frequency range 24.25 – 33.4 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 32.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < 2*BWcontiguous | 0.1* BWcontiguous +0.5 MHz f_offset < 2* BWcontiguous +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| 2*BWcontiguous f < fmax | 2 BWcontiguous +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -10 dBm)) | 10 MHz |
| NOTE: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. |  |  |  |

Table 6.7.4.5.2.3-2: OBUE limits applicable in the frequency range 37 – 52.6 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-2.3 dBm, Max(Prated,t,TRP – 30.3 dB, -9.3 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < 2*BWcontiguous | 0.1* BWcontiguous +0.5 MHz f_offset < 2* BWcontiguous +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| 2*BWcontiguous f < fmax | 2 BWcontiguous +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -10 dBm)) | 10 MHz |
| NOTE: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. |  |  |  |

### 6.7.5	OTA transmitter spurious emissions

#### 6.7.5.1	General

Unless otherwise stated, all requirements are measured as mean power.

The OTA transmitter spurious emissions limits are specified as TRP per RIB, unless otherwise stated.

The OTA transmitter spurious emission limits for FR1 shall apply from 30 MHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in subclause 6.7.1. For some operating bands, the upper limit of the spurious range might be higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [5].

For multi-band RIB each supported operating band and the ΔfOBUE MHz around each band are excluded from the OTA transmitter spurious emissionsrequirements.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer’s specification.

BS type 1-O requirements consists of OTA transmitter spurious emission requirements based on TRP and co-location requirements not based on TRP.

The OTA transmitter spurious emission limits for FR2 shall apply from 30 MHz to 2nd harmonic of the upper frequency edge of the downlink operating band, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in subclause 6.7.1.

#### 6.7.5.2	General OTA transmitter spurious emissions requirements

##### 6.7.5.2.1	Definition and applicability

The general OTA transmitter spurious emissions requirements are specified as TRP per RIB, per cell, unless otherwise specified.

##### 6.7.5.2.2	Minimum requirement

The minimum requirement for BS type 1-O is specified in TS 38.104 [2], subclause 9.7.5.2.2.

The minimum requirement for BS type 2-O is specified in TS 38.104 [2], subclause 9.7.5.3.2.

##### 6.7.5.2.3	Test purpose

The test purpose is to verify if the radiated spurious emissions from the BS at the RIB are within the specified minimum requirements.

##### 6.7.5.2.4	Method of test

###### 6.7.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier, see subclause 4.9.1:

-	For FR1:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	For FR2:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in single-band multi-carrier operation, see subclause 4.9.1:

-	For FR1:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 12.75 GHz (or 5th harmonic)

-	For FR2:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in multi-band multi-carrier operation, see subclause 4.9.1:

-	For FR1:

-	BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

-	B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

###### 6.7.5.2.4.2	Procedure

The following procedure for measuring TRP is based on directional power measurements as described in in Annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 5, 7 and 10.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 6.7.5.2.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the BS to transmit

-	For RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test model in subclause 4.9.2 (i.e. NR-FR1-TM1.1 for BS type 1-O and NR-FR2-TM1.1 for BS type 2-O), at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier and/or CA operation, set the RIB to transmit according to the corresponding test model in subclause 4.9.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.7.2 and 4.8.

6)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

7)	Measure the emission at the specified frequencies with specified measurement bandwidth.

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I).

NOTE 1: the TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2: the frequency sweep or the TRP measurement grid sweep may be done in any order.

9)	Calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For BS type 1-O and multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.7.5.2.5	Test requirement

###### 6.7.5.2.5.1	Test requirement for BS type 1-O

For a BS meeting category A the TRP of any spurious emission shall not exceed the limits in table 6.7.5.2.5.1-1.

Table 6.7.5.2.5.1-1: General OTA BS transmitter spurious emission limits for BS type 1-O, Category A

| Spurious frequency range | Test limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 + X dBm | 100 kHz | Note 1, Note 6 |
| 1 GHz – 12.75 GHz |  | 1 MHz | Note 1, Note 2, Note 6 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3, Note 6 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 4: Void. NOTE 5: Void. NOTE 6: X = 9 dB, unless stated differently in regional regulation. |  |  |  |

For a BS meeting category B the TRP of any spurious emission shall not exceed the limits in table 6.7.5.2.5.1-2.

Table 6.7.5.2.5.1-2: General OTA BS transmitter spurious emission limits for BS type 1-O, Category B

| Spurious frequency range | Test limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -36 + X dBm | 100 kHz | Note 1, Note 5 |
| 1 GHz – 12.75 GHz | -30 + X dBm | 1 MHz | Note 1, Note 2, Note 5 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75GHz. NOTE 4: Void. NOTE 5: X = 9 dB, unless stated differently in regional regulation. |  |  |  |

###### 6.7.5.2.5.2	Test requirement for BS type 2-O

###### 6.7.5.2.5.2.1	General

The requirements of either subclause 6.7.5.2.5.2.2 (Category A limits) or subclause 6.7.5.2.5.2.3 (Category B limits) shall apply. The application of either Category A or Category B limits shall be the same as for Operating band unwanted emissions in subclause 6.7.1.

Table 6.7.5.2.5.2-1: Void

NOTE:	Table 6.7.5.2.5.2-1 is moved to subclause 6.7.5.2.5.2.2 as table 6.7.5.2.5.2.2-1.

###### 6.7.5.2.5.2.2	OTA transmitter spurious emissions (Category A)

The power of any spurious emission shall not exceed the limits in table 6.7.5.2.5.2-1.

Table 6.7.5.2.5.2.2-1: General OTA BS transmitter spurious emission limits for BS type 2-O

| Spurious frequency range | Test limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – min(2nd harmonic of the upper frequency edge of the DL operating band in GHz; [60] GHz) |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Measurement bandwidth as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. |  |  |  |

###### 6.7.5.2.5.2.3	OTA transmitter spurious emissions (Category B)

The power of any spurious emission shall not exceed the limits in table 6.7.5.2.5.2.3-1.

Table 6.7.5.2.5.2.3-1: BS radiated Tx spurious emission limits in FR2 (Category B)

| Frequency range (Note 4) | Test limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 18 GHz | -30 dBm | 1 MHz | Note 1 |
| 18 GHz Fstep,1 | -20 dBm | 10 MHz | Note 2 |
| Fstep,1 Fstep,2 | -15 dBm | 10 MHz | Note 2 |
| Fstep,2 Fstep,3 | -10 dBm | 10 MHz | Note 2 |
| Fstep,4 Fstep,5 | -10 dBm | 10 MHz | Note 2 |
| Fstep,5 Fstep,6 | -15 dBm | 10 MHz | Note 2 |
| Fstep,6 min(2nd harmonic of the upper frequency edge of the DL operating band in GHz; [60] GHz) | -20 dBm | 10 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [5], s4.1 NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [26], annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 4: The step frequencies Fstep,X are defined in table 6.7.5.2.5.2.3-2. |  |  |  |

Table 6.7.5.2.5.2.3-2: Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [26], annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in table 6.7.1-1. |  |  |  |  |  |  |

#### 6.7.5.3	Protection of the BS receiver of own or different BS

##### 6.7.5.3.1	Definition and applicability

This requirement shall be applied for NR FDD operation in order to prevent the receivers of own or a different BS of the same band being desensitised by emissions from a BS type 1-O.

This requirement is a co-location requirement as defined in subclause 4.9, in TS 38.104 [2], the power levels are specified at the CLTA output.

##### 6.7.5.3.2	Minimum requirements

The minimum requirement for BS type 1-O is defined in TS 38.104 [2], subclause 9.7.5.2.

##### 6.7.5.3.3	Test purpose

For OTA co-location spurious emission, the test purpose is to verify that the emission is within the specified requirement limits at the CLTA conducted output(s).

##### 6.7.5.3.4	Method of test

###### 6.7.5.3.4.1	Initial conditions

Test environment:	Normal; see annex B.2.

RF channels to be tested for single carrier:	M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band RIB, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band RIB, see subclause 4.9.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, emission testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, emission testing below the lowest operating band may be omitted.

Directions to be tested: The requirement is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

The co-location spurious emission is measured at the CLTA conducted output(s).

###### 6.7.5.3.4.2	Procedure

1)	Select and place the NR BS and CLTA as described in subclause 4.12 with parameters as specified in table 4.12.2.2-1 and table 4.12.2.3-1.

2)	Several CLTAs might be required to cover the whole co-location spurious emission frequency ranges.

3)	Place test antenna in reference direction at far-field distance, aligned in all supported polarizations (single or dual) with the NR BS as depicted in annex E.1.3.

4)	The test antenna shall be dual (or single) polarized with the same frequency range as the NR BS for co-location spurious emission test case.

5)	Connect test antenna and CLTA to the measurement equipment as depicted in annex E.1.3.

6)	OTA co-location spurious emission is measured as the power sum over all supported polarizations at the CLTA conducted output(s).

7)	The measurement device (signal analyzer) characteristics shall be:

-	Detection mode: True RMS.

8)	Set the BS type 1-O to transmit:

-	Set the NR BS to transmit maximum power according to the applicable test configuration in subclause 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2.

-		For the NR BS declared to be capable of multi-carrier and/or CA operation, set the BS to transmit according to NR-FR1-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.7.2 and 4.8.

9)	Measure the emission at the specified frequencies with specified measurement bandwidth.

NOTE:	An alternative measurement method to be used for measuring the OTA emission is described in annex K.

In addition, for multi-band RIB, the following steps shall apply:

10)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.7.5.3.5	Test requirements

###### 6.7.5.3.5.1	Test requirement for BS type 1-O

This requirement shall be applied for NR FDD operation in order to prevent the receivers of own or a different BS of the same band being desensitised by emissions from a BS type 1-O.

This requirement is a co-location requirement as defined in subclause 4.9, in TS 38.104 [2], the power levels are specified at the CLTA output.

The total power of any spurious emission from both polarizations of the CLTA connector output shall not exceed the limits in table 6.7.5.3.5.1-1.

Table 6.7.5.3.5.1-1: BS type 1-O OTA spurious emissions limits for protection of the BS receiver

| BS class | Frequency range | Maximum Level for bands below 3GHz | Maximum Level for bands between 3 and 4.2GHz | Maximum Level for bands between 4.2 and 6GHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
| Wide Area BS | FUL_low – FUL_high | -113.9 dBm | -113.7 dBm | -113.6 dBm | 100 kHz |
| Medium Range BS |  | -108.9 dBm | -108.7 dBm | -108.6 dBm |  |
| Local Area BS |  | -105.9 dBm | -105.7 dBm | -105.6 dBm |  |

#### 6.7.5.4	Additional spurious emissions requirements

##### 6.7.5.4.1	Definition and applicability

These requirements may be applied for the protection of systems operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an NR operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in subclause 4.4.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM, CDMA, UTRA, E-UTRA, NR, etc.).

The requirement shall apply at each RIB supporting transmission in the operating band.

All additional spurious requirements are TRP unless otherwise stated.

##### 6.7.5.4.2	Minimum Requirement

The minimum requirement for BS type 1-O is specified in TS 38.104 [2], subclause 9.7.5.2.4.

The minimum requirement for BS type 2-O is specified in TS 38.104 [2], subclause 9.7.5.3.3.

##### 6.7.5.4.3	Test purpose

The test purpose is to verify the radiated spurious emissions from the BS at the RIB are within the specified additional spurious emissions requirements.

##### 6.7.5.4.4	Method of test

###### 6.7.5.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier:

-	For FR1:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

RF bandwidth positions to be tested in single-band multi-carrier operation:

-	For FR1:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

RF bandwidth positions to be tested in multi-band multi-carrier operation:

-	For FR1:

-	BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

-	B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested: As the FR1 requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

###### 6.7.5.4.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in in Annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 5, 7 and 10.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 6.7.5.2.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the BS type 1-O to transmit:

-	For RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test model NR-FR1-TM1.1 in subclause 4.9.2, at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier and/or CA operation, set the RIB to transmit according to NR-FR1-TM1.1 in subclause 4.9.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.7.2 and 4.8.

6)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

7)	Measure the emission at the specified frequencies with specified measurement bandwidth.

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I).

NOTE 1: the TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2: the frequency sweep or the TRP measurement grid sweep may be done in any order.

9)	Calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.7.5.4.5	Test requirement

###### 6.7.5.4.5.1	Test requirement for BS type 1-O

The power of any spurious emission shall not exceed the test limits in table 6.7.5.4.5-1 for a BS where requirements for co-existence with the system listed in the first column apply. For a multi-band RIB, the exclusions and conditions in the Note column of table 6.7.5.4.5-1 apply for each supported operating band.

Table 6.7.5.4.5-1: BS spurious emissions test limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Test limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| GSM900 | 921 – 960 MHz | -45.4 dBm | 100 kHz | This requirement does not apply to BS operating in band n8. |
|  | 876 – 915 MHz | -49.4 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band n8, since it is already covered by the requirement in subclause 6.7.5.3. |
| DCS1800 | 1805 – 1880 MHz | -35.4 dBm | 100 kHz | This requirement does not apply to BS operating in band n3. |
|  | 1710 – 1785 MHz | -49.4 dBm | 100 kHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in subclause 6.7.5.3. |
| PCS1900 | 1930 – 1990 MHz | -35.4 dBm | 100 kHz | This requirement does not apply to BS operating in band n2, n25 or band n70. |
|  | 1850 – 1910 MHz | -49.4 dBm | 100 kHz | This requirement does not apply to BS operating in band n2 or n25 since it is already covered by the requirement in subclause 6.7.5.3. |
| GSM850 or CDMA850 | 869 – 894 MHz | -45.4 dBm | 100 kHz | This requirement does not apply to BS operating in band n5. |
|  | 824 – 849 MHz | -49.4 dBm | 100 kHz | This requirement does not apply to BS operating in band n5, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 2110 – 2170 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65. |
|  | 1920 – 1980 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1930 – 1990 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n2 or n70. |
|  | 1850 – 1910 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1805 – 1880 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. |
|  | 1710 – 1785 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n66. |
|  | 1710 – 1755 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 869 – 894 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n5. |
|  | 824 – 849 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n5, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 | 860 – 890 MHz | -40.4 dBm | 1 MHz |  |
|  | 815 – 830 MHz | -37.4 dBm | 1 MHz |  |
|  | 830 – 845 MHz | -37.4 dBm | 1 MHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2620 – 2690 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n7. |
|  | 2500 – 2570 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n7, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 925 – 960 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n8. |
|  | 880 – 915 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 – 1879.9 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. |
|  | 1749.9 – 1784.9 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n66 |
|  | 1710 – 1770 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 – 1510.9 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n74 or n75. |
|  | 1427.9 – 1447.9 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76. |
|  | 1447.9 – 1462.9 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n74 or n75. |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 729 – 746 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n12. |
|  | 699 – 716 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n12, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band XIII or E-UTRA Band 13 | 746 – 756 MHz | -40.4 dBm | 1 MHz |  |
|  | 777 – 787 MHz | -37.4 dBm | 1 MHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 758 – 768 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n14. |
|  | 788 – 798 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n14, since it is already covered by the requirement in sub-clause 6.7.5.3. |
| E-UTRA Band 17 | 734 – 746 MHz | -40.4 dBm | 1 MHz |  |
|  | 704 – 716 MHz | -37.4 dBm | 1 MHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 791 – 821 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n20 or n28. |
|  | 832 – 862 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in subclause 6.7.5.3. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 MHz | -40 dBm | 1 MHz | This requirement does not apply to BS operating in Band n77 or n78. |
|  | 3410 – 3490 MHz | -37 dBm | 1 MHz | This requirement does not apply to BS operating in Band n77 or n78. |
| E-UTRA Band 24 | 1525 – 1559 MHz | -40.4 dBm | 1 MHz |  |
|  | 1626.5 – 1660.5 MHz | -37.4 dBm | 1 MHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 – 1995 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, n25 or n70. |
|  | 1850 – 1915 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n25 since it is already covered by the requirement in subclause 6.7.5.3. For BS operating in Band n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in subclause 6.7.5.3. |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 – 894 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n5. |
|  | 814 – 849 MHz | -37.4 dBm | 1 MHz | For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in subclause 6.7.5.3. |
| E-UTRA Band 27 | 852 – 869 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n5. |
|  | 807 – 824 MHz | -37.4 dBm | 1 MHz | This requirement also applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band (Note 5). |
| E-UTRA Band 28 or NR Band n28 | 758 – 803 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n20 or n28. |
|  | 703 – 748 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in subclause 6.7.5.3. |
| E-UTRA Band 29 | 717 – 728 MHz | -40.4 dBm | 1 MHz |  |
| E-UTRA Band 30 or NR Band n30 | 2350 – 2360 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n30. |
|  | 2305 – 2315 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n30, since it is already covered by the requirement in subclause 6.7.5.3. |
| E-UTRA Band 31 | 462.5 -467.5 MHz | -40.4 dBm | 1 MHz |  |
|  | 452.5 -457.5 MHz | -37.4 dBm | 1 MHz |  |
| UTRA FDD band XXXII or E-UTRA band 32 | 1452 – 1496 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n74 or n75. |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 – 1920 MHz | -40.4 dBm | 1 MHz |  |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 – 2025 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n34. |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -40.4 dBm | 1 MHz |  |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n2 or n25. |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -40.4 dBm | 1 MHz |  |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n38. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n39. |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Bands n30 or n40. |
| E-UTRA Band 41 or NR Band n41 | 2496 – 2690 MHz | -40.4 dBm | 1 MHz | This is not applicable to BS operating in Band n41. |
| E-UTRA Band 42 | 3400 – 3600 MHz | -40 dBm | 1 MHz | This requirement does not apply to BS operating in Band n77 or n78. |
| E-UTRA Band 43 | 3600 – 3800 MHz | -40 dBm | 1 MHz | This requirement does not apply to BS operating in Band n77 or n78. |
| E-UTRA Band 44 | 703 – 803 MHz | -40.4 dBm | 1 MHz | This is not applicable to BS operating in Band n28. |
| E-UTRA Band 45 | 1447 – 1467 MHz | -40.4 dBm | 1 MHz |  |
| E-UTRA Band 46 | 5150 – 5925 MHz | -39.5 dBm | 1 MHz |  |
| E-UTRA Band 47 | 5855 – 5925 MHz | -39.5 dBm | 1 MHz |  |
| E-UTRA Band 48 | 3550 – 3700 MHz | -40 dBm | 1 MHz | This requirement does not apply to BS operating in Band n77 or n78. |
| E-UTRA Band 50 or NR Band n50 | 1432 – 1517 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76. |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75 or n76. |
| E-UTRA Band 65 or NR Band n65 | 2110 – 2200 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65. |
|  | 1920 – 2010 MHz | -37.4 dBm | 1 MHz | For BS operating in Band n1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in subclause 6.7.5.3. This requirement does not apply to BS operating in band n65, since it is already covered by the requirement in subclause 6.7.5.3. |
| E-UTRA Band 66 or NR Band n66 | 2110 – 2200 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n66. |
|  | 1710 – 1780 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in subclause 6.7.5.3. |
| E-UTRA Band 67 | 738 – 758 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n28. |
| E-UTRA Band 68 | 753 -783 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n28. |
|  | 698-728 MHz | -37.4 dBm | 1 MHz | For BS operating in Band n28, this requirement applies between 698 MHz and 703 MHz, while the rest is covered in subclause 6.7.5.3. |
| E-UTRA Band 69 | 2570 – 2620 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n38. |
| E-UTRA Band 70 or NR Band n70 | 1995 – 2020 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, n25 or n70 |
|  | 1695 – 1710 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n70, since it is already covered by the requirement in subclause 6.7.5.3. |
| E-UTRA Band 71 or NR Band n71 | 617 – 652 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 |
|  | 663 – 698 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band n71, since it is already covered by the requirement in subclause 6.7.5.3. |
| E-UTRA Band 72 | 461 – 466 MHz | -40.4 dBm | 1 MHz |  |
|  | 451 – 456 MHz | -37.4 dBm | 1 MHz |  |
| E-UTRA Band 74 or NR Band n74 | 1475 – 1518 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n74 or n75. |
|  | 1427 – 1470 MHz | -37.4 dBm | 1MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76. |
| E-UTRA Band 75 or NR Band n75 | 1432 – 1517 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76. |
| E-UTRA Band 76 or NR Band n76 | 1427 – 1432 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75 or n76. |
| NR Band n77 | 3.3 – 4.2 GHz | -40 dBm | 1 MHz | This requirement does not apply to BS operating in Band n77 or n78 |
| NR Band n78 | 3.3 – 3.8 GHz | -40 dBm | 1 MHz | This requirement does not apply to BS operating in Band n77 or n78 |
| NR Band n79 | 4.4 – 5.0 GHz | -39.5 dBm | 1 MHz | This requirement does not apply to BS operating in Band n79 |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in 6.7.5.4.5-1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see TS 38.104 [2], table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.7.5.4.5-1 assumes that two operating bands, where the frequency ranges in TS 38.104 [2] table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 4:	For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band.

The following requirement may be applied for the protection of PHS. This requirement is also applicable at specified frequencies falling between ΔfOBUE below the lowest BS transmitter frequency of the downlink operating band and ΔfOBUE above the highest BS transmitter frequency of the downlink operating band. ΔfOBUE is defined in subclause 6.7.1.

The power of any spurious emission shall not exceed:

Table 6.7.5.4.5-2: BS spurious emissions test limits for BS for co-existence with PHS

| Frequency range | Test limit | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 1884.5 – 1915.7 MHz | -32 dBm | 300 kHz | Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |

In certain regions, the following requirement may apply to BS operating in Band n50 and n75 within 1432-1452 MHz, and in Band n51 and Band n76. Emissions shall not exceed the test levels specified in table 6.7.5.4.5-3. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

Table 6.7.5.4.5-3: Additional operating band unwanted emission test limits for BS operating in Band n50 and n75 within 1432-1452 MHz, and in Band 51 and 76

| Filter centre frequency, Ffilter | Test limit | Measurement bandwidth |
| --- | --- | --- |
| Ffilter = 1413.5 MHz | -39.4 | 27 MHz |

In certain regions, the following requirement may apply to BS operating in NR Band n50 within 1492-1517 MHz. The level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to table 6.7.5.4.5-4, shall neither exceed the maximum emission level PEM,n50,a nor PEM,B50,b declared by the manufacturer.

Table 6.7.5.4.5-4: Operating band n50, n74 and n75 declared emission above 1518 MHz

| Filter centre frequency, Ffilter | Declared emission level (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 1518.5 MHz ≤ Ffilter ≤ 1519.5 MHz | PEM, n50,a | 1 MHz |
| 1520.5 MHz ≤ Ffilter ≤ 1558.5 MHz | PEM,n50,b | 1 MHz |

In certain regions, the following requirement shall be applied to BS operating in Band n14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The power of any spurious emission shall not exceed:

Table 6.7.5.4.5-5: BS Spurious emissions limits for protection of 700 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth |
| --- | --- | --- | --- |
| n14 | 769 – 775 MHz | -37 dBm | 6.25 kHz |
| n14 | 799 – 805 MHz | -37 dBm | 6.25 kHz |

The following requirement may apply to NR BS operating in Band n30 in certain regions. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The power of any spurious emission shall not exceed:

Table 6.7.5.4.5-6: Additional NR BS Spurious emissions limits for Band n30

| Frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 2200 – 2345 MHz | -33.4 dBm | 1 MHz |
| 2362.5 – 2365 MHz | -13.4 dBm |  |
| 2365 – 2367.5 MHz | -28.4 dBm |  |
| 2367.5 – 2370 MHz | -30.4 dBm |  |
| 2370 – 2395 MHz | -33.4 dBm |  |

NOTE:	The regional requirement, included in ECC/DEC/(17)06 [15], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in TS 38.104 [2] annex E.

#### 6.7.5.5	Co-location requirements

##### 6.7.5.5.1	Definition and applicability

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume co-location with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

This requirement is a co-location requirement as defined in subclause 4.9, in TS 38.104 [2], the power levels are specified at the CLTA output.

##### 6.7.5.5.2	Minimum requirements

The minimum requirement for BS type 1-O is defined in TS 38.104 [2], subclause 9.7.5.2.

##### 6.7.5.5.3	Test purpose

For OTA co-locate spurious emission, the test purpose is to verify that the emission is within the specified requirement limits at the CLTA conducted output(s).

##### 6.7.5.5.4	Method of test

###### 6.7.5.5.4.1	Initial conditions

Test environment: normal; see subclause B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band RIB, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band RIB, see subclause 4.9.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, emission testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, emission testing below the lowest operating band may be omitted.

Directions to be tested: The FR1 requirement is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

The co-location spurious emission is measured at the CLTA conducted output(s).

###### 6.7.5.5.4.2	Procedure

1)	Select and place the NR BS and CLTA as described in subclause 4.12, with parameters as specified in table 4.12.2.2-1 and table 4.12.2.3-1.

2)	Several CLTAs might be required to cover the whole co-location spurious emission frequency ranges.

3)	Place test antenna in reference direction at far-field distance, aligned in all supported polarizations (single or dual) with the NR BS as depicted in annex E.1.3.

4)	The test antenna shall be dual (or single) polarized with the same frequency range as the NR BS for co-location spurious emission test case.

5)	Connect test antenna and CLTA to the measurement equipment as depicted in annex E.1.3.

6)	OTA co-location spurious emission is measured as the power sum over all supported polarizations at the CLTA conducted output(s).

7)	The measurement device (signal analyzer) characteristics shall be:

-	Detection mode: True RMS.

8)	Set the BS type 1-O to transmit:

-	Set the NR BS to transmit maximum power according to the applicable test configuration in subclause 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2.

-	For the NR BS declared to be capable of multi-carrier and/or CA operation, set the BS to transmit according to the applicable test configuration and corresponding power setting specified in subclause 4.7.2 and 4.8 using the corresponding test models on all carriers configured.

9)	Measure the emission at the specified frequencies with specified measurement bandwidth.

In addition, for multi-band RIB, the following steps shall apply:

10)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.7.5.5.5	Test requirements

###### 6.7.5.5.5.1	Test requirement for BS type 1-O

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume co-location with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

This requirement is a co-location requirement as defined in subclause 4.9, in TS 38.104 [2], the power levels are specified at the CLTA output.

The output of the CLTA of any spurious emission shall not exceed the test limitin table 6.7.5.5.5.1-1.

For a multi-band RIB, the exclusions and conditions in the notes column of table 6.7.5.5.5.1-1 apply for each supported operating band.

Table 6.7.5.5.5.1-1: BS type 1-O OTA spurious emissions limits for BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Test limit |  |  | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | WA BS | MR BS | LA BS |  |  |
| GSM900 | 876-915 MHz | -115.9 dBm | -108.9 dBm | -87.9 dBm | 100 kHz |  |
| DCS1800 | 1710 – 1785 MHz | -115.9 dBm | -108.9 dBm | --97.9 dBm | 100 kHz |  |
| PCS1900 | 1850 – 1910 MHz | -115.9 dBm | -108.9 dBm | --97.9 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 – 849 MHz | -115.9 dBm | -108.9 dBm | -87.9 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1850 – 1910 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1710 – 1785 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 – 1755 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 824 – 849 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 – 845 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2500 – 2570 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 880 – 915 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 – 1784.9 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 – 1770 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 – 1447.9 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band n50 or n75 |
| UTRA FDD Band XII or E-UTRA Band 12 | 699 – 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 – 787 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 788 – 798 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 17 | 704 – 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 18 | 815 – 830 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 832 – 862 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 – 1462.9 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band n50 or n75 |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band n77 or n78 |
| E-UTRA Band 23 | 2000 – 2020 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1850 – 1915 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 814 – 849 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 27 | 807 – 824 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 30 or NR Band n30 | 2305 – 2315 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 31 | 452.5 -457.5 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 – 1920 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 34 | 2010 – 2025 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band n2 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band n38. |
| UTRA TDD Band f) or E-UTRA Band 39 | 1880 – 1920 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA TDD Band e) or E-UTRA Band 40 | 2300 – 2400MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 41 or NR Band n41 | 2496 – 2690 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band n41 |
| E-UTRA Band 42 | 3400 – 3600 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band n77 or n78 |
| E-UTRA Band 43 | 3600 – 3800 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band n77 or n78 |
| E-UTRA Band 44 | 703 – 803 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band n28 |
| E-UTRA Band 45 | 1447 – 1467 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 46 | 5150 – 5925 MHz | N/A | -108.6 dBm | -105.6 dBm | 100 kHz |  |
| E-UTRA Band 48 | 3550 – 3700 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band n77 or n78 |
| E-UTRA Band 50 or NR Band n50 | 1432 – 1517 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band n74 or n75 |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | N/A | N/A | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band n50, n75 or n76 |
| E-UTRA Band 65 or NR Band n65 | 1920 – 2010 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 66 or NR Band n66 | 1710 – 1780 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 68 | 698 – 728 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 70 or NR Band n70 | 1695 – 1710 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 71 or NR Band n71 | 663 – 698 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 72 | 451 – 456 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 74 or NR Band n74 | 1427 – 1470 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band n50 |
| NR Band n77 | 3.3 – 4.2 GHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band n77 or n78 |
| NR Band n78 | 3.3 – 3.8 GHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band n77 or n78 |
| NR Band n79 | 4.4 – 5.0 GHz | -113.6 dBm | -108.6 dBm | -105.6 dBm | 100 kHz |  |
| NR Band n80 | 1710 – 1785 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| NR Band n81 | 880 – 915 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| NR Band n82 | 832 – 862 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| NR Band n83 | 703 – 748 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| NR Band n84 | 1920 – 1980 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 85 | 698 - 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| NR Band n86 | 1710 – 1780 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.7.5.5.5.1-1 do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band (see table 5.2-1 in TS 38.104 [2]). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [27].

NOTE 2:	Table 6.7.5.5.5.1-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in table 5.2-1 in TS 38.104 [2] would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations (except in Band n46), special co-location requirements may apply that are not covered by the 3GPP specifications.

## 6.8	OTA transmitter intermodulation

### 6.8.1	Definition and applicability

The OTA transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station. The requirement applies during the transmitter ON period and the transmitter transient period.

The requirement applies at each RIB supporting transmission in the operating band.

The transmitter intermodulation level is the total radiated power of the intermodulation products when an interfering signal is injected into the CLTA.

For BS type 1-O, the transmitter intermodulation requirement is captured by the co-location transmitter intermodulation scenario case, in which the interfering signal is injected into the CLTA.

### 6.8.2	Minimum requirement

The minimum requirement for BS type 1-O operation is defined in TS 38.104 [2], subclause 9.8.2.

The OTA transmitter intermodulation requirement is not applicable for BS type 2-O.

### 6.8.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to restrict the generation of intermodulation products in its nonlinear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station to below specified levels.

### 6.8.4	Method of test

#### 6.8.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier:	M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band RIB, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band RIB, see subclause 4.9.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, emission testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, emission testing below the lowest operating band may be omitted.

Directions to be tested: The FR1 requirement is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

#### 6.8.4.2	Procedure

1)	Select a CLTA according to the description in subclause 4.12 and parameters given in table 4.12.2.2-1.

2)	Place the CLTA according to the description in subclause 4.12 and parameters given in table 4.12.2.3-1.

3)	The test antenna(s) shall be dual (or single) polarized covering the same frequency range as the NR BS and the emission frequencies.

4)	Several test antennas are required to cover both the NR BS and the whole emission frequency range.

5)	Connect test antenna and CLTA to the measurement equipment as shown in annex E.1.5.

6)	During the OTA emission measurements at the test antenna conducted output(s), both NR BS and CLTA are rotated around same axis.

7)	The OTA emission measurement method shall be TRP, according to the procedure described in annex I.

8)	The measurement device (signal analyzer) characteristics shall be:

-	Detection mode: True RMS.

9)	Set the BS type 1-O to transmit:

-	Set the NR BS to transmit maximum power according to the applicable test configuration in subclause 4.8 using the corresponding test models or set of physical channels in subclause 4.9.2.

-	For the NR BS declared to be capable of multi-carrier and/or CA operation, set the BS to transmit according to the applicable test configuration and corresponding power setting specified in subclause 4.7.2 and 4.8 using the corresponding test models on all carriers configured.

10)	Generate the interfering signal via the CLTA. The CLTA is fed with a power level equal to declared Prated,t,TRP, divided over all the supported polarizations, from the same signal generator source:

-	using test model as defined in subclause 4.9.2, at a centre frequency offset according to the conditions in table 9.8.2-1 in TS 38.104 [2], but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

11)	Adjust the interfering signal level at the CLTA conducted input(s) as defined in:

-	transmitter intermodulation table 9.8.2-1 in TS 38.104 [2].

12)	If the interferer signal is applicable according to subclause 4.7, perform the unwanted emission tests specified in subclauses 6.7.3 (OTA ACLR) and 6.7.4 (OTA OBUE) for all third and fifth order intermodulation products which appear in the frequency ranges defined in subclauses 6.7.3 and 6.7.4 (Note 2). The width of the intermodulation products shall be taken into account.

13)	If the interferer signal is applicable according to subclause 4.7, perform the Transmitter spurious emissions test as specified in subclause 6.7.5 (OTA spurious emission), except OTA co-location spurious emission, for all third and fifth order intermodulation products which appear in the frequency ranges defined in subclause 6.7.5 (Note 2). The width of the intermodulation products shall be taken into account.

14)	Verify that the emission level does not exceed the required level in subclause 6.8.5 (Test requirements) with the exception of interfering signal frequencies.

15)	Repeat the test for the remaining interfering signal centre frequency offsets according to the conditions of:

-	transmitter intermodulation table 9.8.2-1 in TS 38.104 [2].

16)	Repeat the test for the remaining interfering signals defined in subclause 4.7 for requirements 6.7.3 (OTA ACLR), 6.7.4 (OTA OBUE) and 6.7.5 (OTA spurious emission), except OTA co-location spurious emission.

In addition, for multi-band RIB, the following steps shall apply:

17)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

NOTE 1:	The third order intermodulation products are centred at 2F1F2 and 2F2F1. The fifth order intermodulation products are centred at 3F12F2, 3F22F1, 4F1F2, and 4F2F1 where F1 represents the test signal centre frequency or centre frequency of each sub-block and F2 represents the interfering signal centre frequency. The widths of intermodulation products are:

-	(n*BWF1 + m* BWF2) for the nF1mF2 products;

-	(n* BWF2 + m* BWF1) for the nF2mF1 products;

where BWF1 represents the test wanted signal RF bandwidth or channel bandwidth in case of single carrier, or sub-block bandwidth and BWF2 represents the interfering signal channel bandwidth.

NOTE 2:	During the conformance test the interferer signal can be applied on one side of the wanted signal, while the transmitter intermodulation emission is measured only on the opposite side of the wanted signal. This applies for intermodulation products which are within the operating band or OBUE region.

### 6.8.5	Test requirements

#### 6.8.5.1	Requirement for BS type 1-O

The transmitter intermodulation level shall not exceed the TRP unwanted emission limits specified for OTA transmitter spurious emission in subclause 6.7.5 (except co-location with other base stations), OTA out-of-band emissions in subclause 6.7.4 and OTA ACLR in subclause 6.7.3 in the presence of a wanted signal and an interfering signal, defined in table 6.8.5.1-1.

The requirement is applicable outside the Base Station RF Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For RIBs supporting operation in multiple operating bands, the requirement shall apply relative to the Base Station RF Bandwidth edges of each operating band. In case the inter RF Bandwidth gap is less than 3*BWChannel MHz (where BWChannel is the minimal BS channel bandwidth of the band), the requirement in the gap shall apply only for interfering signal offsets where the interfering signal falls completely within the inter RF Bandwidth gap.

Table 6.8.5.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in subclause 5.3.5 of TS 38.104 [2] |
| Interfering signal level | The interfering signal level is the same power level as the BS (Prated,t,TRP) fed into a CLTA. |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE 3: The Prated,t,TRP is split between supported polarizations at the CLTA input ports. |  |

# 7	Radiated receiver characteristics

## 7.1	General

General test conditions for receiver tests are given in clause 4, including interpretation of measurement results and configurations for testing. BS configurations for the tests are defined in clause 4.5.

Unless otherwise stated, the following arrangements apply for radiated receiver characteristics requirements in clause 7:

-	Requirements apply during the BS receive period.

-	Requirements shall be met for any transmitter setting.

-	For FDD operation the requirements shall be met with the transmitter unit(s) ON.

-	Throughput requirements defined for the radiated receiver characteristics do not assume HARQ retransmissions.

-	When BS is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

-	For ACS, blocking and intermodulation characteristics, the negative offsets of the interfering signal apply relative to the lower Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap, and the positive offsets of the interfering signal apply relative to the upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap.

NOTE 1:	In normal operating condition the BS in FDD operation is configured to transmit and receive at the same time.

NOTE 2:	In normal operating condition the BS in TDD operation is configured to TX OFF power during receive period.

Each requirement, except OTA receiver spurious emissions, shall be met over the RoAoA specified.

For FR1 requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔOTAREFSENS = 44.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) (dB) for the reference direction.

And

ΔOTAREFSENS = 41.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) (dB) for all other directions.

For requirements which are to be met over the minSENS RoAoA absolute requirement values are offset by the following term:

ΔminSENS = PREFSENS – EISminSENS (dB)

For FR2 requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔFR2_REFSENS = -3 dB for the reference direction

and

ΔFR2_REFSENS = 0 dB for all other directions

## 7.2	OTA sensitivity

### 7.2.1	Definition and applicability

The OTA sensitivity requirement is based upon the declaration of one or more OTA sensitivity direction declarations (OSDD), related to a BS type 1-H and BS type 1-O receiver.

The BS type 1-H and BS type 1-O receiver may optionally be capable of redirecting/changing the receiver target by means of adjusting BS settings resulting in multiple sensitivity RoAoA. The sensitivity RoAoA resulting from the current BS settings is the active sensitivity RoAoA.

If the BS is capable of redirecting the receiver target related to the OSDD then the OSDD shall include:

-	BS channel bandwidth and declared minimum EIS level applicable to any active sensitivity RoAoA inside the receiver target redirection range in the OSDD.

-	A declared receiver target redirection range, describing all the angles of arrival that can be addressed for the OSDD through alternative settings in the BS.

-	Five declared sensitivity RoAoA comprising the conformance testing directions as detailed in TR 37.842 [6].

-	The receiver target reference direction.

NOTE 1:	Some of the declared sensitivity RoAoA may coincide depending on the redirection capability.

NOTE 2:	In addition to the declared sensitivity RoAoA, several sensitivity RoAoA may be implicitly defined by the receiver target redirection range without being explicitly declared in the OSDD.

If the BS is not capable of redirecting the receiver target related to the OSDD, then the OSDD includes only:

-	BS channel bandwidth and declared minimum EIS level applicable to the sensitivity RoAoA in the OSDD.

-	One declared active sensitivity RoAoA.

-	The receiver target reference direction.

NOTE 3:	For BS without target redirection capability, the declared (fixed) sensitivity RoAoA is always the active sensitivity RoAoA.

The OTA sensitivity EIS level declaration shall apply to each supported polarization, under the assumption of polarization match.

### 7.2.2	Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in TS 38.104 [2] subclause 7.2.2 shall be met when the level of the arriving signal is equal to the minimum EIS level in the respective declared set of EIS level and BS channel bandwidth.

### 7.2.3	Test Purpose

The test purpose is to verify that the BS can meet the throughput requirement for a specified measurement channel at the EIS level and the range of angles of arrival declared in the OSDD.

### 7.2.4	Method of test

#### 7.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Directions to be tested:

-	receiver target reference direction (D.31),

-	conformance test directions (D.33).

#### 7.2.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.1.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OSDD being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in subclause 7.2.5.

8)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified in subclause 7.2.5.

9)	Measure the throughput according to annex A.1 for each supported polarization.

10)	Repeat steps 3 to 9 for all OSDD(s) declared for the BS (D.23), and supported polarizations.

For multi-band capable BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.2.5	Test requirements

#### 7.2.5.1	General

The minimum EIS level is a declared figure (D.27, D.28) for each OSDD (D.23). The test requirement is calculated from the declared value offset by the EIS Test Tolerance specified in subclause 4.1.

#### 7.2.5.2	Test requirements for BS type 1-H and BS type 1-O

For each measured carrier, the throughput measured in step 9 of subclause 7.2.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.5.2-1.

Table 7.2.5.2-1: EIS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (annex A.1) | EIS level (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 | Declared minimum EIS + 1.6 |
| 10, 15 | 30 | G-FR1-A1-2 |  |  |  |
| 10, 15 | 60 | G-FR1-A1-3 |  |  |  |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 |  |  |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 |  |  |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 |  |  |  |
| NOTE: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

#### 7.2.5.3	Test requirements for BS type 2-O

There is no OTA sensitivity requirement for FR2, the OTA sensitivity is the same as the OTA reference sensitivity in subclause 7.3.

## 7.3	OTA reference sensitivity level

### 7.3.1	Definition and applicability

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received at the RIB at which a reference performance requirement shall be met for a specified reference measurement channel.

The OTA REFSENS EIS level declaration shall apply to each supported polarization, under the assumption of polarization match.

### 7.3.2	Minimum requirement

For BS type 1-O the minimum requirement is in TS 38.104 [2], subclause 10.3.2.

For BS type 2-O the minimum requirement is in TS 38.104 [2], subclause 10.3.3.

### 7.3.3	Test Purpose

The test purpose is to verify that the BS can meet the throughput requirement for a specified measurement channel at the EISREFSENS level and the range of angles of arrival within the OTA REFSENS RoAoA.

### 7.3.4	Method of test

#### 7.3.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

-	B, M and T; see subclause 4.9.1.

Directions to be tested:

-	OTA REFSENS receiver target reference direction (D.54),

-	OTA REFSENS conformance test directions (D.55).

#### 7.3.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.1.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OTA REFSENS RoAoA being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in subclause 7.3.5.

8)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified in subclause 7.3.5.

9)	Measure the throughput according to annex A.1 for each supported polarization.

10)	Repeat steps 3 to 9 for all OTA REFSENS conformance test directions of the BS (D.55), and supported polarizations.

For multi-band capable FR1 BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.3.5	Test requirements

#### 7.3.5.1	General

The FR1 EISREFSENS level is the conducted REFSENS requirement value offset by ΔOTAREFSENS. The test requirement is calculated from the EISREFSENS level offset by the EISREFSENS Test Tolerance specified in subclause 4.1.

#### 7.3.5.2	Test requirements for BS type 1-O

For each measured carrier, the throughput measured in step 9 of subclause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in tables 7.3.5.2-1 to 7.3.5.2-3.

Table 7.3.5.2-1: Wide Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (annex A.1) | EISREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -100.4 – ΔOTAREFSENS | -100.3 – ΔOTAREFSENS | -100.1 – ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -100.5 – ΔOTAREFSENS | -100.4 – ΔOTAREFSENS | -100.2 – ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -97.6 – ΔOTAREFSENS | -97.5 – ΔOTAREFSENS | -97.3 – ΔOTAREFSENS |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 | -94 – ΔOTAREFSENS | -93.9 – ΔOTAREFSENS | -93.7 – ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -94.3 – ΔOTAREFSENS | -94.2 – ΔOTAREFSENS | -94 – ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -94.4 – ΔOTAREFSENS | -94.3 – ΔOTAREFSENS | -94.1 – ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.2-2: Medium Range BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (annex A.1) | EISREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -95.4 – ΔOTAREFSENS | -95.3 – ΔOTAREFSENS | -95.1 – ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -95.5 – ΔOTAREFSENS | -95.4 – ΔOTAREFSENS | -95.2 – ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -92.6 – ΔOTAREFSENS | -92.5 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 | -89 – ΔOTAREFSENS | -88.9 – ΔOTAREFSENS | -88.7 – ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -89.3 – ΔOTAREFSENS | -89.2 – ΔOTAREFSENS | -89 – ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -89.4 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS | -89.1 – ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.2-3: Local Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (annex A.1) | EISREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -92.4 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS | -92.1 – ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -92.5 – ΔOTAREFSENS | -92.4 – ΔOTAREFSENS | -92.2 – ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -89.6 – ΔOTAREFSENS | -89.5 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 | -86 – ΔOTAREFSENS | -85.9 – ΔOTAREFSENS | -85.7 – ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -86.3 – ΔOTAREFSENS | -86.2 – ΔOTAREFSENS | -86 – ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86.4 – ΔOTAREFSENS | -86.3 – ΔOTAREFSENS | -86.1 – ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

#### 7.3.5.3	Test requirements for BS type 2-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50MHZ BS channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50MHz BS channel bandwidth it does not imply that BS has to support 50MHz BS channel bandwidth.

For wide area BS, EISREFSENS_50M is an integer value in the range -96 to -119 dBm. The specific value is declared by the vendor.

For medium range BS, EISREFSENS_50M is an integer value in the range -91 to -114 dBm. The specific value is declared by the vendor.

For local area BS, EISREFSENS_50M is an integer value in the range -86 to -109 dBm. The specific value is declared by the vendor.

Table 7.3.5.3-1 FR2 OTA reference sensitivity requirement

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (annex A.1) | EISREFSENS level (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 2.4 + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 2.4 + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + 2.4 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified in table 10.3.3-2. |  |  |  |

## 7.4	OTA dynamic range

### 7.4.1	Definition and applicability

The OTA dynamic range is a measure of the capability of the receiver unit to receive a wanted signal in the presence of an interfering signal inside the received BS channel bandwidth.

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

### 7.4.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 10.4.2.

### 7.4.3	Test purpose

The test purpose is to verify that at the BS receiver dynamic range, the relative throughput shall fulfil the specified limit.

### 7.4.4	Method of test

#### 7.4.4.1	Initial conditions

Test environment: Normal: see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Directions to be tested: OTA REFSENS receiver target reference direction (D.54).

#### 7.4.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.2.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OTA REFSENS RoAoA being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.4.5.2-1 to 7.4.5.2-3.

b)	Set the signal generator for the AWGN interfering signal at the same frequency as the wanted signal to transmit as specified in table 7.4.5.2-1 to 7.4.5.2-3.

8)	Measure the throughput according to annex A.2 for each supported polarization.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.4.5	Test requirement

#### 7.4.5.1	General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA dynamic range Test Tolerance specified in subclause 4.1.

#### 7.4.5.2	Test requirements for BS type 1-O

For each measured carrier, the throughput measured in step 6 of subclause 7.4.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in tables 7.4.5.2-1 to 7.4.5.2-3.

Table 7.4.5.2-1: Wide Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.2) | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -82.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -79.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -76.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -75.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -74.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -73.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -72.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -71.4 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.8 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.6 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.2-2: Medium Range BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.2) | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -74.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -72.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -71.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -70.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -69.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | --59.2 – ΔOTAREFSENS | --59.2 – ΔOTAREFSENS | --59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -66.4 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.8 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.6 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.2-3: Local Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.2) | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -74.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -71.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -69.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -68.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -66.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -63.4 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.8 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.6 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

## 7.5	OTA in-band selectivity and blocking

### 7.5.1	OTA adjacent channel selectivity

#### 7.5.1.1	Definition and applicability

OTA Adjacent channel selectivity (ACS) is a measure of the receiver’s ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system. The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

#### 7.5.1.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 10.5.1.2.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 10.5.1.3.

#### 7.5.1.3	Test purpose

The test purpose is to verify the ability of the BS receiver filter to suppress interfering signals in the channels adjacent to the wanted channel.

#### 7.5.1.4	Method of test

##### 7.5.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

-	M; see subclause 4.9.1.

Base Station RF Bandwidth edge position to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see subclause 4.9.1;

-	BRFBW_T’RFBW and B’RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested:

-	For BS type 1-O, receiver target reference direction (D.31),

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

##### 7.5.1.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the BS so that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OSDD or OTA REFSENS RoAoA being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.1.4.1-1 for BS type 1-O and table 7.5.1.4.2-1 for BS type 2-O.

b)	Set the signal generator for the interfering signal at the adjacent channel frequency of the wanted signal to transmit as specified in table 7.5.1.4.1-1 for BS type 1-O and table 7.5.1.4.2-1 for BS type 2-O.

8)	Measure throughput according to annex A.1 for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclauses 4.7.2 and 4.8.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

#### 7.5.1.5	Test requirement

##### 7.5.1.5.1	General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA ACS Test Tolerance specified in subclause 4.1.

##### 7.5.1.5.2	Test requirements for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1, the OTA wanted and the interfering signal are specified in table 7.5.1.5.2-1 and table 7.5.1.5.2-2 for ACS. The reference measurement channel for the OTA wanted signal is identified in subclause 7.3.5.2 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.5.1.5.2-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the OTA ACS requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 7.5.1.5.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.5.1.5.2-1: OTA ACS requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) |  |  | Interfering signal mean power (dBm) |
| --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |
| 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 (Note 1) | EISminSENS + 6 dB |  |  | Wide Area: -52 – ΔminSENS Medium Range: -47– ΔminSENS Local Area: -44– ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], subclause 10.2.1. |  |  |  |  |

Table 7.5.1.5.2-2: OTA ACS interferer frequency offset for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5MHz DFT-s-OFDM NR signal, 15kHz SCS, 25 RBs |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20MHz DFT-s-OFDM NR signal, 15kHz SCS, 100 RBs |
| 30 | ±9.4725 |  |
| 40 | ±9.4675 |  |
| 50 | ±9.4625 |  |
| 60 | ±9.4725 |  |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

##### 7.5.1.5.3	Test requirements for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2, the OTA wanted and the interfering signal are specified in table 7.5.1.5.3-1 and table 7.5.1.5.3-2 for ACS. The reference measurement channel for the OTA wanted signal is identified in subclause 7.3.5.3 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.5.1.5.3-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 7.5.1.5.3-1: OTA ACS requirement for BS type 2-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) |
| --- | --- | --- | --- |
|  | 24.24 GHz < f ≤ 33.4 GHz | 37 GHz < f ≤ 52.6 GHz |  |
| 50, 100, 200, 400 | EISREFSENS + 6 dB (Note 3) | EISREFSENS + 6 dB (Note 3) | EISREFSENS_50M + 27.7 + ΔFR2_REFSENS (Note 1) EISREFSENS_50M + 26.7 + ΔFR2_REFSENS (Note 2) |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 3: EISREFSENS is specified in TS 38.104 [2], subclause 10.3.3. |  |  |  |

Table 7.5.1.5.3-2: OTA ACS interferer frequency offset for BS type 2-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 50 | ±24.29 | 50MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| 100 | ±24.31 |  |
| 200 | ±24.29 |  |
| 400 | ±24.31 |  |

### 7.5.2	OTA in-band blocking

#### 7.5.2.1	Definition and applicability

The OTA in-band blocking characteristics is a measure of the receiver’s ability to receive a OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one RB for narrowband blocking.

#### 7.5.2.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 10.5.2.2.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 10.5.2.3.

#### 7.5.2.3	Test purpose

The test purpose is to verify the ability of the BS receiver to withstand high-levels of in-band interference from unwanted signals at specified frequency offsets without undue degradation of its sensitivity.

#### 7.5.2.4	Method of test

##### 7.5.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see subclause 4.9.1.

Base Station RF Bandwidth edge position to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see subclause 4.9.1;

-	BRFBW_T’RFBW and B’RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested:

For BS type 1-O:

-	receiver target reference direction for the minSENS OSDD (D.31),

-	OTA REFSENS conformance test directions (D.55),

For BS type 2-O:

-	OTA REFSENS receiver target reference direction (D.54),

-	OTA REFSENS conformance test directions (D.55).

##### 7.5.2.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OSDD or OTA REFSENS RoAoA being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

For general OTA blocking:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.2.5.2-1 for BS type 1-O and table 7.5.2.5.3-1 for BS type 2-O.

b)	Set the signal generator for the interfering signal at the specified frequency offset from the wanted signal to transmit as specified in table 7.5.2.5.2-1 for BS type 1-O and table 7.5.2.5.3-1 for BS type 2-O. The interfering signal shall be swept with a step size of 1 MHz starting from the minimum offset to the channel edges of the wanted signals.

For OTA narrowband blocking:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.2.5.2-2 for BS type 1-O.

b)	Set the signal generator for the interfering signal at the specified frequency offset from the wanted signal to transmit as specified in tables 7.5.2.5.2-2 and 7.5.2.5.2-3 for BS type 1-O. Set-up and sweep the interfering RB centre frequency offset to the channel edge of the wanted signal according to table 7.5.2.5.2-3.

8)	Measure throughput according to annex A.1 for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclauses 4.7.2 and 4.8.

9)	Repeat steps 3 to 8 for all the specified measurement directions.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

#### 7.5.2.5	Test requirement

##### 7.5.2.5.1	General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA in-band blocking Test Tolerance specified in subclause 4.1.

##### 7.5.2.5.2	Test requirements for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted and OTA interfering signal specified in tables 7.5.2.5.2-1, table 7.5.2.5.2-2 and table 7.5.2.5.2-3 for general OTA and narrowband OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in subclause 7.3.5.2 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For BS type 1-O the OTA in-band blocking requirement shall apply in the in-band blocking frequency range, which is defined within frequency range from FUL_low - ΔfOOB to FUL_high + ΔfOOB, excluding the downlink frequency range of the FDD operating band, where the ΔfOOB for BS type 1-O is defined in table 7.5.2.5.2-0.

Table 7.5.2.5.2-0: ΔfOOB offset for NR operating bands in FR1

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 1-O | FUL_high – FUL_low < 100 MHz | 20 |
|  | 100 MHz ≤ FUL_high – FUL_low ≤ 900 MHz | 60 |

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 7.5.2.5.2-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the OTA in-band blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 7.5.2.5.2-1 and 7.5.2.5.2-3.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA narrowband blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the interfering signal minimum offset in table 7.5.2.5.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band RIBs, the OTA narrowband blocking requirements apply in the narrowband blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the interfering signal minimum offset in table 7.5.2.5.2-3.

Table 7.5.2.5.2-1: General OTA blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |  |
| 5, 10, 15, 20 | EISREFSENS + 6 dB (NOTE 2) |  |  | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS (NOTE 2) | ±7.5 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
|  | EISminSENS + 6 dB (NOTE 3) |  |  | Wide Area: -43 - ΔminSENS Medium Range: -38 - ΔminSENS Local Area: -35 - ΔminSENS (NOTE 3) |  |  |
| 25 ,30, 40, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB (NOTE 2) |  |  | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS (NOTE 2) | ±30 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
|  | EISminSENS + 6 dB (NOTE 3) |  |  | Wide Area: -43 - ΔminSENS Medium Range: -38 - ΔminSENS Local Area: -35 - ΔminSENS (NOTE 3) |  |  |
| NOTE 1: EISREFSENS and EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], subclause 10.3.2 and 10.2.1. NOTE 2: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 3: This test requirement is only applied in the OTA minSENS receiver target reference direction. |  |  |  |  |  |  |

Table 7.5.2.5.2-2: OTA narrowband blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA Wanted signal mean power (dBm) |  |  | OTA Interfering signal mean power (dBm) |
| --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |
| 5, 10, 15, 20 | EISREFSENS + 6 dB (NOTE 3) |  |  | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB (NOTE 4) |  |  | Wide Area: -49 - ΔminSENS Medium Range: -44 - ΔminSENS Local Area: -41 - ΔminSENS |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB (NOTE 3) |  |  | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB (NOTE 4) |  |  | Wide Area: -49 - ΔminSENS Medium Range: -44 - ΔminSENS Local Area: -41 - ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: EISREFSENS and EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], subclause 10.3.2 and 10.2.1. NOTE 3: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 4: This test requirement is only applied in the OTA minSENS receiver target reference direction. NOTE 5: 7.5 kHz shift is not applied to the wanted signal. |  |  |  |  |

Table 7.5.2.5.2-3: OTA narrowband blocking interferer frequency offsets for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±(350 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 10 | ±(355 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±(360 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±(350 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 30 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 40 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 50 | ±(560 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 60 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 70 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 80 | ±(560 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 90 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 100 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| NOTE 1: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

##### 7.5.2.5.3	Test requirements for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS type 2-O, the OTA wanted and OTA interfering signals are provided at RIB using the parameters in table 7.5.2.5.3-1 for general OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in subclause 7.3.5.3 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA blocking requirements are applicable outside the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

For BS type 2-O the OTA blocking requirement shall apply in the in-band blocking frequency range, which is defined within frequency range from FUL_low - ΔfOOB to FUL_high + ΔfOOB,where the ΔfOOB for BS type 2-O is defined in table 7.5.2.5.3-0.

Table 7.5.2.5.3-0: ΔfOOB offset for NR operating bands in FR2

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 2-O | FUL_high – FUL_low ≤ 3250 MHz | 1500 |

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 7.5.2.5.3-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 7.5.2.5.3-1: General OTA blocking requirement for BS type 2-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA wanted signal mean power (dBm) |  | OTA interfering signal mean power (dBm) | OTA interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of OTA interfering signal |
| --- | --- | --- | --- | --- | --- |
|  | 24.24 GHz < f ≤ 33.4 GHz | 37 GHz < f ≤ 52.6 GHz |  |  |  |
| 50, 100, 200, 400 | EISREFSENS + 6 dB | EISREFSENS + 6 dB | EISREFSENS_50M + 33 + ΔFR2_REFSENS dB | ±75 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| NOTE: EISREFSENS and EISREFSENS_50M are given in TS 38.104 [2], subclause 10.3.3. |  |  |  |  |  |

## 7.6	OTA out-of-band blocking

### 7.6.1	Definition and applicability

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

For the general OTA out-of-band blocking the requirement applies to the wanted signal for each supported polarization, under the assumption of polarization match. The interferer shall be polarization matched for in-band frequencies and the polarization maintained for out-of-band frequencies.

### 7.6.2	Minimum requirement

The minimum requirement for BS type 1-O is defined in TS 38.104 [2], subclause 10.6.2.

The minimum requirement for BS type 2-O is defined in TS 38.104 [2], subclause 10.6.3.

### 7.6.3	Test purpose

The test stresses the ability of the receiver unit associated with the RIB under test to withstand high-level interference from unwanted signals at specified frequency bands, without undue degradation of its sensitivity.

### 7.6.4	Method of test

#### 7.6.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier (MC):

-	MRFBW in single-band RIB, see subclause 4.9.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band RIB, see subclause 4.9.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, blocking testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, blocking testing below the lowest operating band may be omitted.

Directions to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

#### 7.6.4.2	Procedure

##### 7.6.4.2.1	BS type 1-O procedure for out-of-band blocking

1)	Place BS and the test antenna(s) according to annex E.2.4.1.

2)	Align the BS and test antenna(s) according to the directions to be tested.

3)	Connect test antenna(s) to the measurement equipment as shown in annex E.2.4.1.

4)	The test antenna(s) shall be dual (or single) polarized covering the same frequency ranges as the BS and the blocking frequencies. If the test antenna does not cover both the wanted and interfering signal frequencies, separate test antennas for the wanted and interfering signal are required.

5)	The OTA blocking interferer is injected into the test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

6)	Generate the wanted signal in receiver target reference direction, according to the applicable test configuration (see subclause 4.8) using applicable reference measurement channel to the RIB, according to annex A.1.

7)	Configure the beam peak direction for the transmitter units associated with the RIB under test according to the declared reference beam direction pair for the appropriate beam identifier with the carrier set-up and power allocation according to the applicable test configuration(s) (see subclause 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

8)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.1.1-1. The distance between the test object and test antenna injecting the interferer signal is adjusted when necessary to ensure specified interferer signal level to be received.

9)	The CW interfering signal shall be swept with a step size of 1 MHz within the frequency range specified in sub-clause 7.6.5.1.1.

10)	Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the subclause 7.6.5, for the relevant carriers specified by the test configuration in subclause 4.7 and 4.8.

11)	Repeat for all supported polarizations.

In addition, for multi-band RIB, the following steps shall apply:

12)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.6.4.2.2	BS type 1-O procedure for co-location blocking

1)	Place NR BS and CLTA as specified in subclause 4.12.2.3.

2)	Several CLTA are required to cover the whole co-location blocking frequency ranges. The CLTA shall be selected according to subclause 4.12.2.2.

3)	Align the NR BS and test antenna(s) according to the directions to be tested.

4)	Connect test antenna and CLTA to the measurement equipment as depicted in annex E.2.4.2.

5)	The NR BS receives the wanted signal in all supported polarizations, in the receiver target reference direction from the test antenna.

6)	The OTA co-location blocking interferer is injected via the CLTA. The CLTA is fed with the specified co-location blocking interferer power per supported polarization.

7)	Generate the wanted signal in receiver target reference direction, all supported polarizations, from the test antenna, according to the applicable test configuration (see subclause 4.8) using applicable reference measurement channel to the RIB, according to annex A.1.

8)	Configure the beam peak direction for the transmitter units associated with the RIB under test according to the declared reference beam direction pair for the appropriate beam identifier with the carrier set-up and power allocation according to the applicable test configuration(s) (see subclause 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

9)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.1.1-1 and, when applicable, for co-location test requirements in table 7.6.5.1.2-1.

10)	The CW interfering signal shall be swept with a step size of 1 MHz within the frequency range corresponding to downlink operating bands releated to co-located systems (according to declaration D.43).

11)	Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the subclause 7.6.5, for the relevant carriers specified by the test configuration in subclause 4.7 and 4.8.

In addition, for multi-band RIB, the following steps shall apply:

12)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.6.4.2.3	BS type 2-O procedure for out-of-band blocking

1)	Place BS and the test antenna(s) according to annex E.2.4.1.

2)	Align the BS and test antenna(s) according to the directions to be tested.

3)	Connect test antenna(s) to the measurement equipment as shown in annex E.2.4.1.

4)	The test antenna(s) shall be dual (or single) polarized covering the same frequency ranges as the BS and the blocking frequencies. If the test antenna does not cover both the wanted and interfering signal frequencies, separate test antennas for the wanted and interfering signal are required.

5)	The OTA blocking interferer is injected into the test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

6)	Generate the wanted signal, according to the applicable test configuration (see subclause 4.7 and 4.8) using applicable reference measurement channel to the RIB, according to annex A.1.

7)	Configure the beam peak direction for the transmitter unit associated with the RIB under test according to the declared reference beam direction pair for the appropriate beam identifier with the carrier set-up and power allocation according to the applicable test configuration(s) (see subclause 4.7 and 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

8)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.2.1-1. The distance between the test object and test antenna injecting the interferer signal is adjusted when necessary to ensure specified interferer signal level to be received.

9)	The interfering signal shall be swept within the frequency range specified in table 7.6.5.2.1-1 with the step size specified in table 7.6.4.2.3-1.

10)	Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the subclause 7.6.5, for the relevant carriers specified by the test configuration in subclause 4.7 and 4.8.

Table 7.6.4.2.3-1: Interferer signal step size

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000 | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |

11)	Repeat for all supported polarizations.

### 7.6.5	Test requirements

#### 7.6.5.1	Requirement for BS type 1-O

The test requirement consists of general and co-location requirements.

##### 7.6.5.1.1	General

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 7.6.5.1.1-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in subclause 10.3.2 in TS 38.104 [2] for each BS channel bandwidth and further specified in annex A.1.

For a multi-band RIB, the OTA out-of-band requirement shall apply for each supported operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to subclause 7.4.2.2 in TS 38.104 [2] shall be excluded from the OTA out-of-band blocking requirement.

For BS type 1-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL_low - ΔfOOB and from FUL_high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for BS supporting FDD. The ΔfOOB for BS type 1-O is defined in table 7.5.2.5.2-0.

Table 7.6.5.1.1-1: OTA out-of-band blocking performance requirement

| Wanted signal mean power (dBm) | Interfering signal RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- |
| EISminSENS + 6 dB (Note 1) | 0.36 V/m | CW carrier |
| NOTE 1: EISminSENS depends on the channel bandwidth as specified in TS 38.104 [2], subclause 10.2.1. NOTE 2: The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m; for example, 0.36 V/m is equivalent to 36 dBm at fixed distance of 30 m. |  |  |

##### 7.6.5.1.2	Co-location requirement

This additional OTA out-of-band blocking requirement may be applied for the protection of BS receivers when NR, E-UTRA BS, UTRA BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with a BS.

The interferer power level is specified at the CLTA conducted input(s) as the signal power per supported polarization.

For OTA wanted and OTA interfering signal provided at the RIB using the parameters in table 7.6.5.1.2-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in subclause 10.3.2 in TS 38.104 [2] for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D in TS 38.104 [2].

For BS type 1-O the OTA blocking requirement for co-location with BS in other frequency bands is applied for all operating bands for which co-location protection is provided.

Table 7.6.5.1.2-1: OTA blocking requirement for co-location with BS in other frequency bands

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | EISminSENS + 6 dB (Note 1) | +46 | +38 | +24 | CW carrier |
| NOTE 1: EISminSENS depends on the BS class and on the BS channel bandwidth as specified in TS 38.104 [2], subclause 10.2.1. NOTE 2: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOBimmediately outside any of the supported uplink operating band(s). NOTE 3: The specified interferer signal power level is applied to all supported CLTA input ports. |  |  |  |  |  |

#### 7.6.5.2	Requirement for BS type 2-O

The test requirement consists of general requirements.

##### 7.6.5.2.1	General requirement

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 7.6.5.2.1-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in subclause 10.3.3 in TS 38.104 [2] for each BS channel bandwidth and further specified in annex A.1.

For BS type 2-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL_low – ΔfOOB and from FUL_high + ΔfOOB up to min(2nd harmonic of the upper frequency edge of the operating band, 60 GHz). The ΔfOOB for BS type 2-O is defined in table 7.5.2.5.3-0.

Table 7.6.5.2.1-1: OTA out-of-band blocking performance requirement

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.36 | CW carrier |
| 12750 to FUL_low – ΔfOOB |  | 0.1 |  |
| FUL_high + ΔfOOB to min(2nd harmonic of the upper frequency edge of the operating band, 60000) |  | 0.1 |  |
| NOTE: EISREFSENS is given in TS 38.104 [2], subclause 10.3.3. |  |  |  |

## 7.7	OTA receiver spurious emissions

### 7.7.1	Definition and applicability

The OTA RX spurious emission is the power of the emissions radiated from the antenna array from a receiver unit.

Unless otherwise stated, all requirements are measured as mean power.

The OTA receiver spurious emission limits for FR1 shall apply from 30 MHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in subclause 6.7.1. For some operating bands, the upper limit of the spurious range might be higher than 12.75 GHz in order to comply with the 5th harmonic limit of the uplinkoperating band, as specified in ITU-R recommendation SM.329 [5].

For multi-band RIB the above exclusion applies for each supported operating band.

The OTA receiver spurious emission limits for FR2 shall apply from 30 MHz to 2nd harmonic of the upper frequency edge of the downlink operating band, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in subclause 6.7.1.

For a BS operating in FDD, OTA RX spurious emissions requirement do not apply as they are superseded by the OTA TX spurious emissions requirement. This is due to the fact that TX and RX spurious emissions cannot be distinguished in OTA domain.

For a BS operating in TDD, the OTA RX spurious emissions requirement shall apply during the transmitter OFF period only.

The metric used to capture OTA receiver spurious emissions for BS type 1-O and BS type 2-O is total radiated power (TRP), with the requirement defined at the RIB.

### 7.7.2	Minimum requirement

The minimum requirement for BS type 1-O is specified in TS 38.104 [2], subclause 10.7.2.

The minimum requirement for BS type 2-O is specified in TS 38.104 [2], subclause 10.7.3.

### 7.7.3	Test purpose

The test purpose is to verify if the receiver radiated spurious emissions from the BS at the RIB are within the specified minimum requirements.

### 7.7.4	Method of test

#### 7.7.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier, see subclause 4.9.1:

-	For FR1:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	For FR2:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in single-band operation, see subclause 4.9.1:

-	For FR1:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	For FR2:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in multi-band operation, see subclause 4.9.1:

-	For FR1:

-	BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

-	B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

#### 7.7.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in in Annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 5, 7 and 10.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 7.7.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the TDD BS to receive only.

6)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

7)	Measure the emission at the specified frequencies with specified measurement bandwidth

8)	Repeat step 6-9 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I).

NOTE 1: the TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2: the frequency sweep or the TRP measurement grid sweep may be done in any order

9)	Calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For BS type 1-O and multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.7.5	Test requirement

#### 7.7.5.1	Test requirement for BS type 1-O

For RX only multi-band RIB, the OTA receiver spurious emissions requirements are subject to exclusion zones in each supported operating band.

The power of any spurious emission shall not exceed the levels in table 7.7.5.1-1:

Table 7.7.5.1-1: General OTA BS receiver spurious emission limits for BS type 1-O

| Spurious frequency range | Test limits (Note 6) | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -54.5 + X dBm | 100 kHz | Note 1, Note 6 |
| 1 GHz – 6 GHz | -44.5 + X dBm | 1 MHz | Note 1, Note 2, Note 6 |
| 6 GHz – 12.75 GHz | -42.8 + X dBm | 1 MHz | Note 1, Note 2, Note 6 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the UL operating band in GHz | -42.8 + X dBm | 1 MHz | Note 1, Note 2, Note 3, Note 6 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the UL operating band is reaching beyond 12.75 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the BS transmitter operating band to ΔfOBUE above the highest frequency of the BS transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in subclause 6.7.1. For multi-band RIBs, the exclusion applies for all supported operating bands. NOTE 5: Void NOTE 6: X = 9 dB, unless stated differently in regional regulation. NOTE 7: Void |  |  |  |

#### 7.7.5.2	Test requirement for BS type 2-O

The power of any receiver spurious emission shall not exceed the limits in table 7.7.5.2-1.

Table 7.7.5.2-1: General OTA BS receiver spurious emission limits for BS type 2-O

| Spurious frequency range | Test limits | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -54.5 dBm | 100 kHz | Note 1 |
| 1 GHz – 6 GHz | -44.5 dBm | 1 MHz | Note 1 |
| 6 GHz – 12.75 GHz | - 44.3dBm | 1 MHz | Note 1 |
| 12.75 GHz – min(2nd harmonic of the upper frequency edge of the UL operating band in GHz; [60] GHz) | -36 dBm | 1 MHz | Note 1, Note 2 |
| NOTE 1: Measurement bandwidth as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 3: The frequency range from ΔfOBUE below the lowest frequency of the BS transmitter operating band to ΔfOBUE above the highest frequency of the BS transmitter operating band, may be excluded from the requirement. ΔfOBUE is defined in subclause 6.7.1. |  |  |  |

## 7.8	OTA receiver intermodulation

### 7.8.1	Definition and applicability

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver unit to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. The requirement is defined as a directional requirement at the RIB.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

### 7.8.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 10.8.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 10.8.3.

### 7.8.3	Test purpose

To verify that the BS receiver dynamic range, the relative throughput shall fulfil the specified limit.

### 7.8.4	Method of test

#### 7.8.4.1	Initial conditions

Test environment: Normal, annex B.2.

RF channels to be tested for single carrier:	M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW for single-band operation, see subclause 4.9.1.

-	BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band operation, see subclause 4.9.1.

Directions to be tested:

-	OTA REFSENS receiver target reference direction (D.54).

-	In addition, for BS type 1-O, receiver target reference direction (D.31).

#### 7.8.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.6.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction of the BS according to declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit the beam(s) of the same operational band as the OTA REFSENS RoAoA or OSDD being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.8.5.1-1 and 7.8.5.1-3 for BS type 1-O and table 7.8.5.2-1 for BS type 2-O.

b)	Set the signal generator for the interfering signal at the same frequency as the wanted signal to transmit as specified in table 7.8.5.1-1 and 7.8.5.1-3 for BS type 1-O and table 7.8.5.2-1 for BS type 2-O.

8)  Set the signal generator for the interfering signal to transmit at the frequency offset and as specified in table 7.8.5.1-2.

9)	Measure the throughput according to annex A.1 for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclause 4.7.

10)	Repeat for all the specified measurement directions and supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

11)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.8.5	Test requirement

#### 7.8.5.1	BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the FR1 OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals at the RIB with the conditions specified in tables 7.8.5.1-1 and 7.8.5.1-2 for intermodulation performance and in tables 7.8.5.1-3 and 7.8.5.1-4 for narrowband intermodulation performance.

The reference measurement channel for the wanted signal is identified in table 7.3.5.2-1, table 7.3.5.2-2 and table 7.3.5.2-3 for each BS channel bandwidth and further specified in annex A.1.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60 kHz and BS channel bandwidth ≤ 20 MHz, for which the subcarrier spacing of the interfering signal should be 30 kHz.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement shall apply in addition inside any sub-block gap in case the sub-block gap is at least as wide as the BS channel bandwidth of the NR interfering signal in tables 7.8.5.1-2 and 7.8.5.1-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the Base Station RF Bandwidth edge.

For multi-band RIBs, the narrowband intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the NR interfering signal in tables 7.8.5.1-2 and 7.8.5.1-4. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.8.5.1-1: General intermodulation requirement

| BS class | Wanted Signal mean power (dBm) | Mean power of interfering signals (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB | -52 - ΔOTAREFSENS | See table 7.8.5.1-2 |
|  | EISminSENS + 6 dB | -52 - ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6 dB | -47 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB | -44 - ΔminSENS |  |
| NOTE: EISREFSENS and EISminSENS depend on the BS class and on the BS channel bandwidth as specified in TS 38.104 [2], subclause 10.3.2 and 10.2.1. |  |  |  |

Table 7.8.5.1-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper base station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 10 | ±7.45 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 15 | ±7.43 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 20 | ±7.38 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 30 | ±7.43 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 1) |
| 25 | ±7.45 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 40 | ±7.45 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 50 | ±7.35 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 60 | ±7.49 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 70 | ±7.42 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 80 | ±7.44 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 90 | ±25 | CW |
|  | ±7.43 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 100 | ±7.45 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| NOTE 1: For the 15 kHz subcarrier spacing, the number of RB is 25. For the 30 kHz subcarrier spacing, the number of RB is 10. NOTE 2: For the 15 kHz subcarrier spacing, the number of RB is 100. For the 30 kHz subcarrier spacing, the number of RB is 50. For the 60 kHz subcarrier spacing, the number of RB is 24. |  |  |

Table 7.8.5.1-3: Narrowband intermodulation performance requirement in FR1

| BS class | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB (Note 1) | -52 - ΔOTAREFSENS | See table 7.8.5.1-4 |
|  | EISminSENS + 6 dB (Note 1) | -52 - ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6 dB (Note 1) | -47 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB (Note 1) | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -44 - ΔminSENS |  |
| NOTE: EISREFSENS and EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], subclause 10.3.2 and 10.2.1. |  |  |  |

Table 7.8.5.1-4: Interfering signals for narrowband intermodulation requirement in FR1

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±360 | CW |
|  | ±1420 | 5MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 10 | ±325 | CW |
|  | ±1780 | 5MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 15 (NOTE 2) | ±380 | CW |
|  | ±1600 | 5MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 20 (NOTE 2) | ±345 | CW |
|  | ±1780 | 5MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 25 (NOTE 2) | ±325 | CW |
|  | ±1990 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 30 (NOTE 2) | ±320 | CW |
|  | ±1990 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 40 (NOTE 2) | ±310 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 50 (NOTE 2) | ±330 | CW |
|  | ±3250 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 60 (NOTE 2) | ±350 | CW |
|  | ±3790 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 70 (NOTE 2) | ±400 | CW |
|  | ±4870 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 80 (NOTE 2) | ±390 | CW |
|  | ±4870 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 90 (NOTE 2) | ±340 | CW |
|  | ±5770 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| 100 (NOTE 2) | ±340 | CW |
|  | ±5770 | 20MHz DFT-s-OFDM NR signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

#### 7.8.5.2	BS type 2-O

Throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted signal at the assigned channel frequency and two OTA interfering signals provided at the RIB using the parameters in tables 7.8.5.2-1 and 7.8.5.2-2. All of the OTA test signals arrive from the same direction, and the requirement is valid if the signals arrive from any direction within the FR2 OTA REFSENS RoAoA. The reference measurement channel for the wanted signal is identified in table 7.3.5.3-1 for each BS channel bandwidth and further specified in annex A.1.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

Table 7.8.5.2-1: General intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Mean power of interfering signals (dBm) | Wanted signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS_50M + 25 + ΔFR2_REFSENS dB | EISREFSENS + 6dB | See table 7.8.5.2-2 |
| NOTE: EISREFSENS and EISREFSENS_50M are given in TS 38.104 [2], subclause 10.3.3. |  |  |  |

Table 7.8.5.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- |
| 50 MHz | ±7.5 | CW |
|  | ±40 | 50MHz DFT-s-OFDM NR signal (Note) |
| 100 MHz | ±6.88 | CW |
|  | ±40 | 50MHz DFT-s-OFDM NR signal (Note) |
| 200 MHz | ±5.64 | CW |
|  | ±40 | 50MHz DFT-s-OFDM NR signal (Note) |
| 400 MHz | ±6.02 | CW |
|  | ±45 | 50MHz DFT-s-OFDM NR signal (Note) |
| NOTE: For the 60 kHz subcarrier spacing, the number of RB is 64. For the 120 kHz subcarrier spacing, the number of RB is 32. |  |  |

## 7.9	OTA in-channel selectivity

### 7.9.1	Definition and applicability

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel. The interfering signal shall be an NR signal as specified in annex E in TS 38.141-1 [3] and shall be time aligned with the wanted signal.

### 7.9.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 10.9.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 10.9.3.

### 7.9.3	Test purpose

The purpose of this test is to verify the BS receiver ability to suppress the IQ leakage.

### 7.9.4	Method of test

#### 7.9.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:	 M; see subclause 4.9.1.

Directions to be tested:

-	For BS type 1-O, receiver target reference direction (D.31),

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

#### 7.9.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.7.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OTA REFSENS RoAoA or OSDD being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

For each supported NR channel BW:

7)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified as follows:

a)	Adjust the signal generator for the wanted signal as specified in:

For BS type 1-O, table 7.9.5.1-1 for BS of Wide Area BS class, in table 7.9.5.1-2 for BS of Local Area BS class and in table 7.9.5.1-3 for BS of Medium Range BS class on one side of the FC.

For BS type 2-O, table 7.9.5.2-1 on one side of the FC.

b)	Adjust the signal generator for the interfering signal as specified in:

For BS type 1-O, table 7.9.5.1-1 for BS of Wide Area BS class, in table 7.9.5.1-2 for BS of Local Area BS class and in table 7.9.5.1-3 for BS of Medium Range BS class at opposite side of the FC and adjacent to the wanted signal.

For BS type 2-O, table 7.9.5.2-1 at opposite side of the FC and adjacent to the wanted signal.

8)	Measure throughput according to annex A.1 for each supported polarization.

9)	Repeat the measurement with the wanted signal on the other side of the FC, and the interfering signal at opposite side of the FC and adjacent to the wanted signal.

10)	Repeat for all the specified measurement directions and supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

9)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.9.5	Test requirement

#### 7.9.5.1	BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted and interfering signals applies to each supported polarization, under the assumption of polarization match.

For a wanted and an interfering signal coupled to the RIB, the following requirements shall be met:

-	For BS type 1-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.9.5.1-1 for Wide Area BS, in table 7.9.5.1-2 for Medium Range BS and in table 7.9.5.1-3 for Local Area BS.

Table 7.9.5.1-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.1) | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -98.9-ΔminSENS | -98.5-ΔminSENS | -98.2-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -97-ΔminSENS | -96.6-ΔminSENS | -96.3-ΔminSENS | -77.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 50 | 15 | G-FR1-A1-4 | -90.6-ΔminSENS | -90.2-ΔminSENS | -89.9-ΔminSENS | -71.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -99.6-ΔminSENS | -99.2-ΔminSENS | -98.9-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -97.1-ΔminSENS | -96.7-ΔminSENS | -96.4-ΔminSENS | -78.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -90.9-ΔminSENS | -90.5-ΔminSENS | -90.2-ΔminSENS | -71.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -96.5-ΔminSENS | -96.1-ΔminSENS | -95.8-ΔminSENS | -78.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -91-ΔminSENS | -90.6-ΔminSENS | -90.3-ΔminSENS | -71.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.9.5.1-2: Medium Range BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.1) | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -93.9-ΔminSENS | -93.5-ΔminSENS | -93.2-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -92-ΔminSENS | -91.6-ΔminSENS | -91.3-ΔminSENS | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 50 | 15 | G-FR1-A1-4 | -85.6-ΔminSENS | -85.2-ΔminSENS | -84.9-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -94.6-ΔminSENS | -94.2-ΔminSENS | -93.9-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -92.1-ΔminSENS | -91.7-ΔminSENS | -91.4-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -85.9-ΔminSENS | -85.5-ΔminSENS | -85.2-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -91.5-ΔminSENS | -91.1-ΔminSENS | -90.8-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86-ΔminSENS | -85.6-ΔminSENS | -85.3-ΔminSENS | -66.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.9.5.1-3: Local area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.1) | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -90.9-ΔminSENS | -90.5-ΔminSENS | -90.2-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -89-ΔminSENS | -88.6-ΔminSENS | -88.3-ΔminSENS | -69.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 50 | 15 | G-FR1-A1-4 | -82.6-ΔminSENS | -82.2-ΔminSENS | -81.9-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -91.6-ΔminSENS | -91.2-ΔminSENS | -90.9-ΔminSENS | -73.4 - ΔminSENS | DFT-s- NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -89.1-ΔminSENS | -88.7-ΔminSENS | -88.4-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -82.9-ΔminSENS | -82.5-ΔminSENS | -82.2-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -88.5-ΔminSENS | -88.1-ΔminSENS | -87.8-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -83-ΔminSENS | -82.6-ΔminSENS | -82.3-ΔminSENS | -63.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

#### 7.9.5.2	BS type 2-O

For BS type 2-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.9.5.2-1.

The wanted and interfering signals applies to each supported polarization, under the assumption of polarization match.

Table 7.9.5.2-1: OTA in-channel selectivity requirement for BS type 2-O

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.1) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | EISREFSENS_50M + 3.4 + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 32 RBs |
| 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 6.4 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| 50 | 120 | G-FR2-A1-5 | EISREFSENS_50M + 3.4 + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 16 RBs |
| 100, 200, 400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 6.4 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RBs |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is defined in TS38.104 [2], subclause 7.3.3. |  |  |  |  |  |

# 8	Radiated performance requirements

## 8.1	General

### 8.1.0	Scope and definitions

Radiated performance requirements specify the ability of the BS type 1-O or BS type 2-O to correctly demodulate radiated signals in various conditions and configurations. Radiated performance requirements are specified at the RIB.

Radiated performance requirements for the BS are specified for the fixed reference channels and propagation conditions defined in TS 38.104 [2] annex A and annex J, respectively. The requirements only apply to those FRCs that are supported by the BS.

The radiated performance requirements for BS type 1-O and for the BS type 2-O are limited to two OTA demodulations branches as described in subclause 8.1.1. Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the number of polarizations supported by the BS, with the required SNR applied separately per polarization.

NOTE 1: BS can support more than 2 demodulation branches, however OTA conformance testing can only be performed for 1 or 2 demodulation branches.

Unless stated otherwise, radiated performance requirements apply for a single carrier only. Radiated performance requirements for a BS supporting CA are defined in terms of single carrier requirements.

For BS type 1-O in FDD operation the requirements in clause 8 shall be met with the transmitter units associated with the RIB in the operating band turned ON.

NOTE 2:	BS type 1-O in normal operating conditions in FDD operation is configured to transmit and receive at the same time. The transmitter unit(s) associated with the RIB may be OFF for some of the tests.

In tests performed with signal generators a synchronization signal may be provided from the BS to the signal generator, to enable correct timing of the wanted signal.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in a slot on a RIB.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot.

### 8.1.1	OTA demodulation branches

Radiated performance requirements are only specified for up to 2 demodulation branches.

If the BS type 1-O, or the BS type 2-O uses polarization diversity and has the ability to maintain isolation between the signals for each of the demodulation branches, then radiated performance requirements can be tested for up to two demodulation branches (i.e. 1RX or 2RX test setups). When tested for two demodulation branches, each demodulation branch maps to one polarization.

If the BS type 1-O, or the BS type 2-O does not use polarization diversity then radiated performance requirements can only be tested for o a single demodulation branch (i.e. 1RX test setup).

### 8.1.2	Applicability rule

#### 8.1.2.0	General

Unless otherwise stated, for a BS declared to support more than 2 demodulation branches (for BS type 1-O and BS type 2-O), the performance requirement tests for 2 demodulation branches shall apply, and the mapping between connectors and demodulation branches is up to BS implementation.

#### 8.1.2.1	Applicability of PUSCH performance requirements

##### 8.1.2.1.1	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, PUSCH requirement tests shall apply only for each subcarrier spacing declared to be supported (see D.7 in table 4.6-1).

##### 8.1.2.1.2	Applicability of requirements for different channel bandwidths

For each subcarrier spacing declared to be supported, the tests for a specific channel bandwidth shall apply only if the BS supports it (see D.7 in table 4.6-1).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

##### 8.1.2.1.3	Applicability of requirements for different configurations

Unless otherwise stated, for BS type 1-O, PUSCH requirement tests shall apply only for the mapping type declared to be supported (see D.100 in table 4.6-1). If both mapping type A and type B are declared to be supported, the tests shall be done for either type A or type B; the same chosen mapping type shall then be used for all tests.

Unless otherwise stated, for BS type 2-O, PUSCH requirement tests shall apply only for the additional DM-RS position declared to be supported (see D.101 in table 4.6-1). If both options (i.e., pos0 and pos1) are declared to be supported, the tests shall be done for pos1.

Unless otherwise stated, for BS type 2-O, PUSCH requirement tests shall apply for the PT-RS option declared to be supported (see D.106 in table 4.6-1). If both PT-RS options (without and with PT-RS) are declared to be supported, the tests shall be done for either without or with PT-RS only; the same chosen option shall then be used for all tests.

#### 8.1.2.2	Applicability of PUCCH performance requirements

##### 8.1.2.2.1	Applicability of requirements for different formats

Unless otherwise stated, PUCCH requirement tests shall apply only for each PUCCH format declared to be supported (see D.102 in table 4.6-1).

##### 8.1.2.2.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, PUCCH requirement tests shall apply only for each subcarrier spacing declared to be supported (see D.7 in table 4.6-1).

##### 8.1.2.2.3	Applicability of requirements for different channel bandwidths

For each subcarrier spacing declared to be supported by the BS, the tests for a specific channel bandwidth shall apply only if the BS supports it (see D.7 in table 4.6-1).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

##### 8.1.2.2.4	Applicability of requirements for different configurations

Unless otherwise stated, PUCCH format 3 requirement tests shall apply only for the additional DM-RS configuration declared to be supported (see D.104 in table 4.6-1). If both options (without and with additional DM-RS) are declared to be supported, the tests shall be done for either without or with additional DM-RS; the same chosen option shall then be used for all tests.

Unless otherwise stated, PUCCH format 4 requirement tests shall apply only for the additional DM-RS configuration declared to be supported (see D.105 in table 4.6-1). If both options (without and with additional DM-RS) are declared to be supported, the tests shall be done for either without or with additional DM-RS; the same chosen option shall then be used for all tests.

##### 8.1.2.2.5	Applicability of requirements for multi-slot PUCCH

Unless otherwise stated, multi-slot PUCCH requirement tests shall apply only if the BS supports it (see D.107 in table 4.6-1).

#### 8.1.2.3	Applicability of PRACH performance requirements

##### 8.1.2.3.1	Applicability of requirements for different formats

Unless otherwise stated, PRACH requirement tests shall apply only for each PRACH format declared to be supported (see D.103 in table 4.6-1).

##### 8.1.2.3.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, for each PRACH format with short sequence declared to be supported, for each FR, the tests shall apply only for the smallest supported subcarrier spacing in the FR (see D.103 in table 4.6-1).

##### 8.1.2.3.3	Applicability of requirements for different channel bandwidths

Unless otherwise stated, for the subscarrier spacing to be tested, the tests shall apply only for anyone channel bandwidth declared to be supported (see D.7 in table 4.6-1).

## 8.2	OTA performance requirements for PUSCH

### 8.2.1	Performance requirements for PUSCH with transform precoding disabled

#### 8.2.1.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 	8.1.2.1.

#### 8.2.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.1.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.2.1.

#### 8.2.1.3	Test purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

#### 8.2.1.4	Method of test

##### 8.2.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested: M, see subclause 4.9.1.

Direction to be tested:

OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

##### 8.2.1.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.1.4.2-1.

Table 8.2.1.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 | {pos0, pos1} |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | {0}, {0,1} |  |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |  |
| Time domain resource assignment | PUSCH mapping type | A, B | B |
|  | Start symbol | 0 | 0 |
|  | Allocation length | 14 | 10 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PTRS configuration | Frequency density (KPT-RS) | N.A. | 2 |
|  | Time density (LPT-RS) | N.A. | 1 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.1.5.1 and 8.2.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.1.4.2-2.

Table 8.2.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36MHz |
|  |  | 20 | -80.2 - ΔOTAREFSENS dBm / 19.08MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64MHz |
|  |  | 20 | -80.4 - ΔOTAREFSENS dBm / 18.36MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16MHz |
|  |  | 100 | -73.1 - ΔOTAREFSENS dBm / 98.28MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the BS, measure the throughput.

#### 8.2.1.5	Test Requirement

##### 8.2.1.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5.1-1 to table 8.2.1.5.1-14 for 1Tx and for 2Tx two layer spatial multiplexing transmission.

Table 8.2.1.5.1-1: Test requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-1.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [10.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [13.2] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [19.0] |

Table 8.2.1.5.1-2: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [11.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [13.2] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [2.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [19.3] |

Table 8.2.1.5.1-3: Test requirements for PUSCH, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [10.9] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [13.3] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [2.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [19.2] |

Table 8.2.1.5.1-4: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-2.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [10.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [12.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [18.8] |

Table 8.2.1.5.1-5: Test requirements for PUSCH, Type A, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-2.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [12.8] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [18.9] |

Table 8.2.1.5.1-6: Test requirements for PUSCH, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [12.3] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [1.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [18.9] |

Table 8.2.1.5.1-7: Test requirements for PUSCH, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-2.3] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [10.8] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [13.3] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [19.4] |

Table 8.2.1.5.1-8: Test requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | [-2.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | [10.5] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | [12.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | [1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | [18.7] |

Table 8.2.1.5.1-9: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | [-2.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | [13.1] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | [19.5] |

Table 8.2.1.5.1-10: Test requirements for PUSCH, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | [-1.8] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | [12.8] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | [1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | [19.0] |

Table 8.2.1.5.1-11: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | [12.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | [1.5] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | [18.9] |

Table 8.2.1.5.1-12: Test requirements for PUSCH, Type B, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | [-2.4] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | [12.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | [19.0] |

Table 8.2.1.5.1-13: Test requirements for PUSCH, Type B, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | [10.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | [12.8] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | [2.1] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | [19.1] |

Table 8.2.1.5.1-14: Test requirements for PUSCH, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | [-2.2] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | [10.7] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | [13.4] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | [2.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | [19.6] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 8.2.1.5.2	Test requirement for BS type 2-O

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5.2-1 to 8.2.1.5.2-5.

Table 8.2.1.5.2-1: Test requirements for PUSCH, 50 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-1 | pos0 | No | [-1.4] |
|  |  |  |  |  | G-FR2-A3-13 | pos1 | No | [-1.6] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-1 | pos0 | Yes | [12.6] |
|  |  |  |  |  |  |  | No | [12.1] |
|  |  |  |  |  | G-FR2-A4-11 | pos1 | Yes | [11.3] |
|  |  |  |  |  |  |  | No | [11.3] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-1 | pos0 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.7] |
|  |  |  |  |  | G-FR2-A5-6 | pos1 | Yes | [14.0] |
|  |  |  |  |  |  |  | No | [13.5] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-6 | pos0 | No | [2.3] |
|  |  |  |  |  | G-FR2-A3-18 | pos1 | No | [2.0] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-6 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  | G-FR2-A4-16 | pos1 | Yes | [20.4] |
|  |  |  |  |  |  |  | No | [18.9] |

Table 8.2.1.5.2-2: Test requirements for PUSCH, 100 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-2 | pos0 | No | [-1.5] |
|  |  |  |  |  | G-FR2-A3-14 | pos1 | No | [-1.8] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-2 | pos0 | Yes | [12.8] |
|  |  |  |  |  |  |  | No | [11.8] |
|  |  |  |  |  | G-FR2-A4-12 | pos1 | Yes | [11.8] |
|  |  |  |  |  |  |  | No | [11.2] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-2 | pos0 | Yes | [14.8] |
|  |  |  |  |  |  |  | No | [13.9] |
|  |  |  |  |  | G-FR2-A5-7 | pos1 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.7] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-7 | pos0 | No | [2.3] |
|  |  |  |  |  | G-FR2-A3-19 | pos1 | No | [2.0] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-7 | pos0 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  | G-FR2-A4-17 | pos1 | Yes | [19.6] |
|  |  |  |  |  |  |  | No | [19.1] |

Table 8.2.1.5.2-3: Test requirements for PUSCH, 50 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-3 | pos0 | No | [-1.2] |
|  |  |  |  |  | G-FR2-A3-15 | pos1 | No | [-1.5] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-3 | pos0 | Yes | [12.2] |
|  |  |  |  |  |  |  | No | [11.5] |
|  |  |  |  |  | G-FR2-A4-13 | pos1 | Yes | [11.5] |
|  |  |  |  |  |  |  | No | [11.1] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-3 | pos0 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.7] |
|  |  |  |  |  | G-FR2-A5-8 | pos1 | Yes | [13.8] |
|  |  |  |  |  |  |  | No | [13.6] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-8 | pos0 | No | [2.2] |
|  |  |  |  |  | G-FR2-A3-20 | pos1 | No | [2.1] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-8 | pos0 | Yes | [21.9] |
|  |  |  |  |  |  |  | No | [19.4] |
|  |  |  |  |  | G-FR2-A4-18 | Pos1 | Yes | [20.4] |
|  |  |  |  |  |  |  | No | [18.4] |

Table 8.2.1.5.2-4: Test requirements for PUSCH, 100 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-4 | pos0 | No | [-1.8] |
|  |  |  |  |  | G-FR2-A3-16 | pos1 | No | [-1.9] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-4 | pos0 | Yes | [12.5] |
|  |  |  |  |  |  |  | No | [11.1] |
|  |  |  |  |  | G-FR2-A4-14 | pos1 | Yes | [11.7] |
|  |  |  |  |  |  |  | No | [11.1] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-4 | pos0 | Yes | [14.1] |
|  |  |  |  |  |  |  | No | [13.5] |
|  |  |  |  |  | G-FR2-A5-9 | pos1 | Yes | [14.0] |
|  |  |  |  |  |  |  | No | [13.4] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-9 | pos0 | No | [2.2] |
|  |  |  |  |  | G-FR2-A3-21 | pos1 | No | [2.0] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-9 | pos0 | Yes | [21.6] |
|  |  |  |  |  |  |  | No | [20.2] |
|  |  |  |  |  | G-FR2-A4-19 | pos1 | Yes | [19.3] |
|  |  |  |  |  |  |  | No | [18.8] |

Table 8.2.1.5.2-5: Test requirements for PUSCH, 200 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-5 | pos0 | No | [-1.2] |
|  |  |  |  |  | G-FR2-A3-17 | pos1 | No | [-1.5] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-5 | pos0 | Yes | [11.9] |
|  |  |  |  |  |  |  | No | [11.5] |
|  |  |  |  |  | G-FR2-A4-15 | pos1 | Yes | [11.8] |
|  |  |  |  |  |  |  | No | [11.1] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-5 | pos0 | Yes | [14.7] |
|  |  |  |  |  |  |  | No | [13.7] |
|  |  |  |  |  | G-FR2-A5-10 | pos1 | Yes | [14.3] |
|  |  |  |  |  |  |  | No | [13.6] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-10 | pos0 | No | [2.2] |
|  |  |  |  |  | G-FR2-A3-22 | pos1 | No | [1.9] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-10 | pos0 | Yes | [22.3] |
|  |  |  |  |  |  |  | No | [21.0] |
|  |  |  |  |  | G-FR2-A4-20 | pos1 | Yes | [19.8] |
|  |  |  |  |  |  |  | No | [19.0] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 8.2.2	Performance requirements for PUSCH with transform precoding enabled

#### 8.2.2.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

#### 8.2.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.2.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.2.2.

#### 8.2.2.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

#### 8.2.2.4	Method of test

##### 8.2.2.4.1	Initial Conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	M, see subclause 4.9.1.

Direction to be tested:

-	For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1)..

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

##### 8.2.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.2.4.2-1.

Table 8.2.2.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 | pos0, pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | 0 |  |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |  |
| Time domain resource assignment | PUSCH mapping type | A, B | B |
|  | Start symbol | 0 | 0 |
|  | Allocation length | 14 | 10 |
| Frequency domain resource assignment | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth | 30 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PT-RS |  | Not configured |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.2.5.1 and 8.2.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.2.4.2-2.

Table 8.2.2.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15dBm / 47.52MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and section 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in section 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the BS, measure the throughput.

#### 8.2.2.5	Test Requirement

##### 8.2.2.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in table 8.2.2.5.1-1 to table 8.2.2.5.1-4.

Table 8.2.2.5.1-1: Test requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-31 | pos1 | [-2.1] |

Table 8.2.2.5.1-2: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-32 | pos1 | [-2.2] |

Table 8.2.2.5.1-3: Test requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-31 | pos1 | [-2.0] |

Table 8.2.2.5.1-4: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-32 | pos1 | [-2.4] |

##### 8.2.2.5.2	Test requirement for BS type 2-O

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in table 8.2.2.5.2-1 to table 8.2.2.5.2-2.

Table 8.2.2.5.2-1: Test requirements for PUSCH, Type B, 50 MHz channel bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-11 | pos1 | [-1.2] |
|  |  |  |  |  | G-FR2-A3-23 | pos1 | [-1.3] |

Table 8.2.2.5.2-2: Test requirements for PUSCH, Type B, 50 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-12 | pos1 | [-1.2] |
|  |  |  |  |  | G-FR2-A3-24 | pos1 | [-1.3] |

### 8.2.3	Performance requirements for UCI multiplexed on PUSCH

#### 8.2.3.1	Definition and applicability

The performance requirement of UCI multiplexed on PUSCH is determined by two paramters: block error probabitiy (BLER) of CSI part 1 and block error probability of CSI part 2. The performance is measured by the required SNR at block error probability of CSI part 1 not exceeding 0.1 %, and the required SNR at block error probability of CSI part 2 not exceeding 1 %.

The CSI part 1 BLER is defined as the probability of incorrectly decoding the CSI part 1 information when the CSI part 1 information is sent.

The CSI part 2 BLER is defined as the probability of incorrectly decoding the CSI part 2 information when the CSI part2 information is sent.

In the test of UCI multiplexed on PUSCH, the UCI information only contains CSI part1 and CSI part2 information, there is no HACK/ACK information transmitted.

The number of UCI information bit payload per slot is defined for two cases as follows:

-	7 bits: 5 bits in CSI part 1, 2 bits in CSI part 2

-	40 bits: 20 bits in CSI part 1, 20 bits in CSI part 2

The 7 bits UCI information case is further defined with the bitmap [c0 c1 c2 c3 c4] = [0 1 0 1 0] for CSI part 1information, where c0 is mapping to the RI information, and with the bitmap [c0 c1] = [1 0] for CSI part 2 information.

The 40 bits UCI information case is assumed random codeword selection.

In both tests, PUSCH data, CSI part 1 and CSI part 2 are transmitted simultaneously.

Which specifc test(s) is applicable to BS is based on the test applicability rule defined in subclause 8.1.2.

#### 8.2.3.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2] subclause 11.2.1.3.

For BS type 2-O, the minimum requirement is in TS 38.104 [2] subclause 11.2.2.3.

#### 8.2.3.3	Test purpose

The test shall verify the receiver’s ability to detect UCI with CSI part 1 and CSI part 2 bits multiplexed on PUSCH under multipath fading propagation conditions for a given SNR.

#### 8.2.3.4	Method of test

##### 8.2.3.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

##### 8.2.3.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters lised in table 8.2.3.4.2-1. The UCI information bit payload per slot is equal to 7bits with CSI part 1 5bits, CSI part 2 2bits, and the UCI information bit payload per slot is equal to 40 bits with CSI part 1 20bits, CSI part 2 20 bits.

Table: 8.2.3.4.2-1 Test parameters for testing UCI multiplexed on PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |  |
| --- | --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |  |
| Uplink-downlink allocation for TDD |  | 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 120 kHz SCS: 3D1S1U, S=10D:2G:2U |  |
| HARQ | Maximum number of HARQ transmissions | 1 |  |  |
|  | RV sequence | 0 |  |  |
| DM-RS | DM-RS configuration type | 1 |  |  |
|  | DM-RS duration | Single-symbol DM-RS |  |  |
|  | Additional DM-RS position | pos1 | pos0,pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |  |
|  | DM-RS port(s) | {0} | {0} |  |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |  |
| Time domain resource assignment | PUSCH mapping type | A,B | B |  |
|  | Start symbol | 0 |  |  |
|  | Allocation length | 14 | 10 |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |  |
|  | Frequency hopping | Disabled |  |  |
| Code block group based PUSCH transmission |  | Disabled |  |  |
| PT-RS configuration | PT-RS | Disabled |  | Enabled |
|  | Frequency density (KPT-RS) | N.A. |  | 2 |
|  | Time density (LPT-RS) | N.A. |  | 1 |
| UCI | Number of CSI part1 and CSI part2 information bit payload | {5, 2}, {20,20} |  |  |
|  | scaling | 1 |  |  |
|  | betaOffsetACK-Index1 | 11 |  |  |
|  | betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part1-Index2 | 13 |  |  |
|  | betaOffsetCSI-Part2-Index1 and betaOffsetCSI-Part2-Index2 | 13 |  |  |
|  | UCI partition for frequency hopping | Disabled |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.3.5.1 and 8.2.3.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.3.4.2-2.

Table 8.2.3.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 30 | 10 | [-83.6] - ΔOTAREFSENS dBm / 8.64MHz |
| BS type 2-O | 120 | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz] |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as declared in subclause 7.1. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	The signal genentors sends a test partten where UCI with CSI part 1 and CSI part 2 information can be multiplexed on PUSCH. The following statistics are kept: the number of incorrectly decoded CSI part 1 informationtransmitted, the number of incorrectly decoded CSI part 2 information transmitted during UCI mupltiplexed on PUSCH transmission.

#### 8.2.3.5	Test Requirement

##### 8.2.3.5.1	Test requirement for BS type 1-O

The fraction of incorrectly decoded UCI with CSI part 1 according to subclause 8.2.3.4.2 shall be less than 0.1 % for the SNR lised in table 8.2.3.5.1-1 and table 8.2.3.5.1-2. The fraction of incorrectly decoded UCI with CSI part 2 according to subclause 8.2.3.4.2 shall be less than 1 % for the SNR lised in table 8.2.3.5.1-3 and table 8.2.3.5.1-4.

Table 8.2.3.5.1-1: Test requirements for UCI multiplexed on PUSCH, Type A, CSI part 1, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | pos1 | G-FR1-A4-11 | [5.7] |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | pos1 | G-FR1-A4-11 | [4.9] |

Table 8.2.3.5.1-2: Test requirements for UCI multiplexed on PUSCH, Type B, CSI part 1, 10MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | pos1 | G-FR1-A4-11 | [6.4] |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | pos1 | G-FR1-A4-11 | [4.7] |

Table 8.2.3.5.1-3: Test requirements for UCI multiplexed on PUSCH, Type A, CSI part 2, 10MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | pos1 | G-FR1-A4-11 | [0.4] |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | pos1 | G-FR1-A4-11 | [3.0] |

Table 8.2.3.5.1-4: Test requirements for UCI multiplexed on PUSCH, Type B, CSI part 2, 10MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 7 (5, 2) | pos1 | G-FR1-A4-11 | [0.9] |
|  | 2 | Normal | TDLC300-100 Low | 40 (20,20) | pos1 | G-FR1-A4-11 | [3.2] |

##### 8.2.3.5.2	Test requirement for BS type 2-O

The fraction of incorrectly decoded UCI with CSI part 1 measured according to subclause 8.2.3.4.2 shall be less than 0.1 % for the SNR lised in table 8.2.3.5.2-1 and table 8.2.3.5.2-2. The fraction of incorrectly decoded UCI with CSI part 2 measured according to subclause 8.2.3.4.2 shall be less than 1 % for the SNR lised in table 8.2.3.5.2-3 and table 8.2.3.5.2-4.

Table 8.2.3.5.2-1: Test requirements for UCI multiplexed on PUSCH, Type B, With PT-RS, CSI part 1, 50 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation braches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | pos0 | G-FR2-A4-3 | [7.8] |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | pos0 | G-FR2-A43 | [6.4] |
|  | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | pos1 | G-FR2-A4-13 | [8.4] |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | pos1 | G-FR2-A4-13 | [6.5] |

Table 8.2.3.5.2-2: Test requirements for UCI multiplexed on PUSCH, Type B, Without PT-RS, CSI part 1, 50MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation braches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | pos0 | G-FR2-A4-3 | [7.7] |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | pos0 | G-FR2-A4-3 | [6.4] |
|  | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | pos1 | G-FR2-A4-13 | [7.9] |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | pos1 | G-FR2-A4-13 | [6.1] |

Table 8.2.3.5.2-3: Test requirements for UCI multiplexed on PUSCH, Type B, With PT-RS, CSI part 2, 50 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation braches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | pos0 | G-FR2-A4-3 | [1.7] |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | pos0 | G-FR2-A4-3 | [4.6] |
|  | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | pos1 | G-FR2-A4-13 | [1.9] |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | pos1 | G-FR2-A4-13 | [4.6] |

Table 8.2.3.5.2-4: Test requirements for UCI multiplexed on PUSCH, Type B, Without PT-RS, CSI part 2, 50MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation braches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | pos0 | G-FR2-A4-3 | [1.7] |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | pos0 | G-FR2-A4-3 | [4.5] |
|  | 2 | Normal | TDLA30-300 Low | 7 (5, 2) | pos1 | G-FR2-A4-13 | [1.8] |
|  | 2 | Normal | TDLA30-300 Low | 40 (20,20) | pos1 | G-FR2-A4-13 | [4.3] |

## 8.3	OTA performance requirements for PUCCH

### 8.3.1	Performance requirements for PUCCH format 0

#### 8.3.1.1	Definition and applicability

The performance requirement of single user PUCCH format 0 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

The transient period as specified in TS 38.101-1 [24] subclause 6.3.3.1 and TS 38.101-2 [25] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

#### 8.3.1.2	Minimum Requirement

For BS type 1-O, the minimum requirements are in TS 38.104 [2] subclause 11.3.1.1 and 11.3.1.2.

For BS type 2-O, the minimum requirements are in TS 38.104 [2] subclause 11.3.2.1 and 11.3.2.2.

#### 8.3.1.3	Test purpose

The test shall verify the receiver’s ability to detect ACK under multipath fading propagation conditions for a given SNR.

#### 8.3.1.4	Method of test

##### 8.3.1.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested:	single carrier (SC) M; see subclause 4.9.1.

Direction to be tested:

OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

##### 8.3.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20] and according to additional test parameters listed in table 8.3.1.4.2-1.

Table 8.3.1.4.2-1: Test parameters

| Parameter | BS type 1-O | BS type 2-O |
| --- | --- | --- |
| nrofBits | 1 | 1 |
| number of PRBs | 1 | 1 |
| First PRB prior to frequency hopping | 0 | 0 |
| Intra-slot frequency hopping | enabled | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither | neither |
| Hopping ID | 0 | 0 |
| initial cyclic shift | 0 | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols | 13 for 1 symbol 12 for 2 symbols |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex TBD.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.1.5.1 and 8.3.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level quoted in table 8.3.1.4.2-2.

Table 8.3.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -83.5 dBm - ΔOTAREFSENS / 4.5 MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS / 9.36 MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS / 19.08 MHz |
|  | 30 | 10 | -80.6 dBm - ΔOTAREFSENS / 8.64 MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS / 18.36 MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS / 38.16 MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS / 98.28 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1 NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1 since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.1.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 8.3.1.4.2-1: Test signal pattern for single user PUCCH format 0 demodulation tests

#### 8.3.1.5	Test Requirement

##### 8.3.1.5.1	Test requirement for BS type 1-O

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 8.3.1.5.1-1 and in table 8.3.1.5.1-2.

Table 8.3.1.5.1-1: Test requirements for PUCCH format 0 and 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | TDLC-300-100 Low | 1 | [10.0] | [9.4] | [9.9] |
|  |  |  | 2 | [3.4] | [4.3] | [3.9] |

Table 8.3.1.5.1-2: Test requirements for PUCCH format 0 and 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | TDLC-300-100 Low | 1 | [10.4] | [10.4] | [10.1] | [9.8] |
|  |  |  | 2 | [4.8] | [4.2] | [4.0] | [3.7] |

##### 8.3.1.5.2	Test requirement for BS type 2-O

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 8.3.1.5.2-1 and in table 8.3.1.5.2-2.

Table 8.3.1.5.2-1: Test requirements for PUCCH format 0 and 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | TDLA30-300 Low | 1 | [9.9] | [9.6] |
|  |  |  | 2 | [4.8] | [4.6] |

Table 8.3.1.5.2-2: Test requirements for PUCCH format 0 and 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | TDLA30-300 Low | 1 | [10.1] | [9.8] | [10.3] |
|  |  |  | 2 | [4.7] | [4.4] | [4.6] |

### 8.3.2	Performance requirements for PUCCH format 1

#### 8.3.2.1	NACK to ACK detection

##### 8.3.2.1.1	Definition and applicability

The performance requirement of PUCCH format 1 for NACK to ACK detection is determined by the two parameters: probability of false detection of the ACK and the NACK to ACK detection probability. The performance is measured by the required SNR at probability of the NACK to ACK detection equal to 0.1% or less. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK at particular bit position when input is only noise. Each false bit detection is counted as one error.

The NACK to ACK detection probability is the probability of detecting an ACK bit when an NACK bit was sent on particular bit position. Each NACK bit erroneously detected as ACK bit is counted as one error. Erroneously detected NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

The transient period as specified in TS 38.101-1 [24] and TS 38.101-2 [25] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

##### 8.3.2.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.3.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.3.

##### 8.3.2.1.3	Test purpose

The test shall verify the receiver’s ability not to falsely detect NACK bits as ACK bits under multipath fading propagation conditions for a given SNR.

##### 8.3.2.1.4	Method of test

###### 8.3.2.1.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

Direction to be tested:

OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

###### 8.3.2.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.2.1.4.2-1.

Table 8.3.2.1.4.2-1: Test parameters

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.2.1.5.1 and 8.3.2.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.2.1.4.2-2.

Table 8.3.2.1.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	The signal generator sends random codeword from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of NACK bits detected as ACK.

##### 8.3.2.1.5	Test Requirement

###### 8.3.2.1.5.1	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1 % and the fraction of NACK bits falsely detected as ACK shall be less than 0.1 % for the SNR listed in tables 8.3.2.1.5.1-1 and table 8.3.2.1.5.1-2.

Table 8.3.2.1.5.1-1: Required SNR for PUCCH format 1 with 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [-3.3] | [-2.9] | [-3.1] |

Table 8.3.2.1.5.1-2: Required SNR for PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [-2.4] | [-2.7] | [-2.5] | [-3.5] |

###### 8.3.2.1.5.2	Test Requirement for BS type 2-O

The fraction of falsely detected ACK bits shall be less than 1 % and the fraction of NACK bits falsely detected as ACK shall be less than 0.1 % for the SNR listed in tables 8.3.2.1.5.2-1 and table 8.3.2.1.5.2-2.

Table 8.3.2.1.5.2-1: Required SNR for PUCCH format 1 with 60 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [TBD] | [-2.1] |

Table 8.3.2.1.5.2-2: Required SNR for PUCCH format 1 with 120 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [-3.3] | [-2.1] | [-2.4] |

#### 8.3.2.2	ACK missed detection

##### 8.3.2.2.1	Definition and applicability

The performance requirement of PUCCH format 1 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

The transient period as specified in TS 38.101-1 [24] and TS 38.101-2 [25] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

##### 8.3.2.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.3.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.3.

##### 8.3.2.2.3	Test purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

##### 8.3.2.2.4	Method of test

###### 8.3.2.2.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

Direction to be tested:

OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

###### 8.3.2.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.2.2.4.2-1.

Table 8.3.2.2.4.2-1: Test Parameters

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex TBD.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.2.2.5.1 and 8.3.2.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.2.2.4.2-2.

Table 8.3.2.2.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 47.52 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8) The signal generator sends random codewords from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits falsely detected in the idle periods and the number of missed ACK bits. Each falsely detected ACK bit in the idle periods is accounted as one error for the statistics of false ACK detection, and each missed ACK bit is accounted as one error for the statistics of missed ACK detection.

Note that the procedure described in this subclause for ACK missed detection has the same condition as that described in subclause 8.3.2.1.4.2 for NACK to ACK detection. Both statistics are measured in the same testing.

Figure 8.3.2.2.4.2-1: Void

##### 8.3.2.2.5	Test Requirement

###### 8.3.2.1.5.1	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of correctly detected ACK bits shall be larger than 99% for the SNR listed in tables 8.3.2.2.5-1 and table 8.3.2.2.5-2.

Table 8.3.2.2.5.1-1: Required SNR for PUCCH format 1 with 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [-4.4] | [-3.7] | [-4.4] |

Table 8.3.2.2.5.1-2: Required SNR for PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [-3.3] | [-3.8] | [-3.8] | [-3.8] |

###### 8.3.2.2.5.2	Test Requirement for BS type 2-O

The fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in tables 8.3.2.2.5.2-1 and table 8.3.2.2.5.2-2.

Table 8.3.2.2.5.2-1: Required SNR for PUCCH format 1 with 60 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [-3.2] | [-3.6] |

Table 8.3.2.2.5.2-2: Required SNR for PUCCH format 1 with 120 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [-4.1] | [-3.8] | [-4.1] |

### 8.3.3	Performance requirements for PUCCH format 2

#### 8.3.3.1	ACK missed detection performance requirements

##### 8.3.3.1.1	Definition and applicability

The performance requirement of PUCCH format 2 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK on the wanted signal. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as probability of detection of the ACK when the signal is present.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

##### 8.3.3.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2] subclause 11.3.1.4.

For BS type 2-O, the minimum requirement is in TS 38.104 [2] subclause 11.3.2.4.

##### 8.3.3.1.3	Test Purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

##### 8.3.3.1.4	Method of test

###### 8.3.3.1.4.1	Initial conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table.4.6-1).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table.4.6-1).

###### 8.3.3.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.3.1.4.2-1.

Table 8.3.3.1.4.2-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index - (Number of PRBs-1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.3.1.5.1 and 8.3.3.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.3.1.4.2-2.

Table 8.3.3.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | [-80.3] - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | [-77.2] -ΔOTAREFSENS dBm / 19.08MHz |
|  | 30 kHz | 10 | [-80.6] - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | [-77.4] - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | [-74.2] - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | [-70.1] - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 kHz | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz] |
|  | 120 kHz | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz] |
|  |  | 200 | [EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz] |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as declared in subclause 7.1. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	The signal generator sends a test pattern with pattern outlined in figure 8.3.3.1.4.2-1. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of missed ACKs.

Figure 8.3.3.1.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

##### 8.3.3.1.5	Test requirement

###### 8.3.3.1.5.1	Requirements for BS type 1-O

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 8.3.3.1.5.1-1 and table 8.3.3.1.5.1-2.

Table 8.3.3.1.5.1-1: Required SNR for PUCCH format 2 with 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [6.4] | [6.2] | [6.5] |

Table 8.3.3.1.5.1-2: Required SNR for PUCCH format 2 with 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [6.1] | [6.2] | [6.1] | [6.3] |

###### 8.3.3.1.5.2	Requirements for BS type 2-O

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 8.3.3.1.5.2-1 and table 8.3.3.1.5.2.-2

Table 8.3.3.1.5.2-1: Required SNR for PUCCH format 2 with 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [TBD] | [7.8] |

Table 8.3.3.1.5.2-2: Required SNR for PUCCH format 2 with 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [7.2] | [6.9] | [7.2] |

#### 8.3.3.2	UCI BLER performance requirements

##### 8.3.3.2.1	Definition and applicability

The UCI block error probability is defined as the probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part2.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

The transient period as specified in TS 38.101-1 [24] and TS 38.101-2 [25] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

##### 8.3.3.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2] subclause 11.3.1.4.

For BS type 2-O, the minimum requirement is in TS 38.104 [2] subclause 11.3.2.4.

##### 8.3.3.2.3	Test Purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

##### 8.3.3.2.4	Method of test

###### 8.3.3.2.4.1	Initial conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

###### 8.3.3.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branches signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.3.2.4.2-1.

Table 8.3.3.2.4.2-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index - (Number of PRBs-1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.3.2.5.1 and 8.3.3.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.3.2.4.2-2.

Table 8.3.3.2.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | [-80.3] - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | [-77.2] -ΔOTAREFSENS dBm/ 19.08MHz |
|  | 30 kHz | 10 | [-80.6] - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | [-77.4] - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | [-74.2] - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | [-70.1] - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 kHz | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz] |
|  | 120 kHz | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz] |
|  |  | 200 | [EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz] |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as declared in subclause 7.1. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.3.2.4.2-1. The following statistics are kept: the number of incorrectly decoded UCI.

Figure 8.3.3.2.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

##### 8.3.3.2.5	Test requirement

###### 8.3.3.2.5.1	Requirements for BS type 1-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.3.2.5.1-1 and table 8.3.3.2.5.1-2.

Table 8.3.3.2.5.1-1: Required SNR for PUCCH format 2 with 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [0.8] | [1.4] | [1.8] |

Table 8.3.3.2.5.1-2: Required SNR for PUCCH format 2 with 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [1.1] | [1.7] | [1.0] | [0.9] |

###### 8.3.3.2.5.2	Requirements for BS type 2-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.3.2.5.2-1 and table 8.3.3.2.5.2-2.

Table 8.3.3.2.5.2-1: Required SNR for PUCCH format 2 with 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [3.2] | [1.7] |

Table 8.3.3.2.5.2-2: Required SNR for PUCCH format 2 with 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [1.8] | [1.8] | [1.7] |

### 8.3.4	Performance requirements for PUCCH format 3

#### 8.3.4.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-1 [24] subclause 6.3.3.1 and TS 38.101-2 [25] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.2.

A test with or without additional DMRS configured is only applicable if the BS support it.

#### 8.3.4.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.5.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.5.

#### 8.3.4.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

#### 8.3.4.4	Method of test

##### 8.3.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

##### 8.3.4.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.4.4.2-1.

Table 8.3.4.4.2-1: Test parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hoppingstartingPRB | 0 |  |
| Intra-slot frequency hoppingintraSlotFrequencyHopping | enabled |  |
| First PRB after frequency hoppingsecondHopPRB | The largest PRB index - (Number of PRBs -1) |  |
| Group and sequence hoppingpucch-GroupHopping | neither |  |
| Hopping IDhoppingId | 0 |  |
| Number of PRBsnrofPRBs | 1 | 3 |
| Number of symbolsnrofSymbols | 14 | 4 |
| The number of UCI information bitsthe number of UCI bits | 16 | 16 |
| First symbolstartingSymbolIndex | 0 | 0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.4.5.1 and 8.3.4.5.2 for BS type 1-O and BS type 2-O respectively, and the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.4.4.2-2.

Table 8.3.4.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm/ 47.52MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm/ 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm/ 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm/ 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm/ 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS receiver target reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

#### 8.3.4.5	Test requirement

##### 8.3.4.5.1	Test requirement for BS type 1-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.4.5.1-1 and table 8.3.4.5.1-2.

Table 8.3.4.5.1-1: Required SNR for PUCCH format 3 with 15 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [0.8] | [1.7] | [0.9] |
|  |  |  |  |  | Additional DM-RS | [0.5] | [1.1] | [0.5] |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [2.0] | [2.8] | [2.6] |

Table 8.3.4.5.1-2: Required SNR for PUCCH format 3 with 30 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [1.5] | [1.2] | [1.2] | [1.5] |
|  |  |  |  |  | Additional DM-RS | [1.1] | [0.9] | [0.6] | [0.7] |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [2.4] | [2.6] | [2.6] | [2.1] |

##### 8.3.4.5.2	Test requirement for BS type 2-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.4.5.2-1 and table 8.3.4.5.2-2.

Table 8.3.4.5.2-1: Required SNR for PUCCH format 3 with 60 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 50 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [2.2] | [1.3] |
|  |  |  |  |  | Additional DM-RS | [1.9] | [1.5] |
| 2 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [3.6] | [3.0] |

Table 8.3.4.5.2-2: Required SNR for PUCCH format 3 with 120 kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [2.0] | [1.3] | [1.3] |
|  |  |  |  |  | Additional DM-RS | [1.9] | [2.0] | [1.5] |
| 2 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [1.7] | [3.5] | [2.0] |

### 8.3.5	Performance requirements for PUCCH format 4

#### 8.3.5.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-1 [24] and TS 38.101-2 [25] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.2.

A test with or without additional DMRS configured is only applicable if the BS support it.

#### 8.3.5.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.6.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.6.

#### 8.3.5.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

#### 8.3.5.4	Method of test

##### 8.3.5.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

##### 8.3.5.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.4.4.2-1.

Table 8.3.5.4.2-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hoppingintraSlotFrequencyHopping | enabled |
| First PRB after frequency hoppingsecondHopPRB | The largest PRB index - (Number of PRBs - 1) |
| Group and sequence hoppingpucch-GroupHopping | neither |
| Hopping IDhoppingId | 0 |
| Number of symbolsnrofSymbols | 14 |
| The number of UCI information bitsthe number of UCI bits | 22 |
| First symbolstartingSymbolIndex | 0 |
| Length of the orthogonal cover codeocc-Length | n2 |
| Index of the orthogonal cover codeocc-Index | n0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.5.5.1 and 8.3.5.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.5.4.2-2.

Table 8.3.5.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15dBm/ 47.52MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18dBm/ 95.04 MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15dBm/ 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18dBm/ 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21dBm/ 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause 7.1, since the OTA REFSENS receiver target reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

#### 8.3.5.5	Test requirement

##### 8.3.5.5.1	Test requirement for BS type 1-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.5.5.1-1 and table 8.3.5.5.1-2.

Table 8.3.5.5.1-1: Required SNR for PUCCH format 4 with 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [2.4] | [3.2] | [2.8] |
|  |  |  |  | Additional DM-RS | [2.2] | [3.0] | [2.4] |

Table 8.3.5.5.1-2: Required SNR for PUCCH format 4 with 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [3.7] | [3.6] | [3.7] | [3.3] |
|  |  |  |  | Additional DM-RS | [3.3] | [2.9] | [3.7] | [2.8] |

##### 8.3.5.5.2	Test requirement for BS type 2-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.5.5.2-1 and table 8.3.5.5.2-2.

Table 8.3.5.5.2-1: Required SNR for PUCCH format 4 with 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [3.6] | [3.3] |
|  |  |  |  | Additional DM-RS | [3.7] | [4.1] |

Table 8.3.5.5.2-2: Required SNR for PUCCH format 4 with 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (annex J) | Additional DM-RS configuration | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 50 MHz | 100 MHz | 200MHz |
| 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [3.4] | [3.4] | TBD |
|  |  |  |  | Additional DM-RS | TBD | [4.4] | TBD |

### 8.3.6	Performance requirements for multi-slot PUCCH format

#### 8.3.6.1	Performance requirements for multi-slot PUCCH format 1

##### 8.3.6.1.1	NACK to ACK detection

###### 8.3.6.1.1.1	Definition and applicability

The performance requirement of PUCCH format 1 for NACK to ACK detection is determined by the two parameters: probability of false detection of the ACK and the NACK to ACK detection probability. The performance is measured by the required SNR at probability of the NACK to ACK detection equal to 0.1% or less. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK at particular bit position when input is only noise. Each false bit detection is counted as one error.

The NACK to ACK detection probability is the probability of detecting an ACK bit when an NACK bit was sent on particular bit position. Each NACK bit erroneously detected as ACK bit is counted as one error. Erroneously detected NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

A test for a specific combination of SCS and channel bandwidth is only applicable if the BS declares to support it.

For a BS supporting multiple combinations of SCS and channel bandwidth, the applicable rule is 8.1.2.2.

###### 8.3.6.1.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.7.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.7.

###### 8.3.6.1.1.3	Test purpose

The test shall verify the receiver’s ability not to falsely detect NACK bits as ACK bits under multipath fading propagation conditions for a given SNR.

###### 8.3.6.1.1.4	Method of test

8.3.6.1.1.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

Direction to be tested: OTA REFSENS receiver target reference direction (D.54).

8.3.6.1.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1) Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2) Align the manufacturer declared coordinate system orientation of the BS with the test system.

3) Set the BS in the declared direction to be tested.

4) Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5) The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.6.1.1.4.2-1.

Table 8.3.6.1.1.4.2-1: Test Parameters for multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index - (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

6) The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7) Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.6.1.1.5.1 and 8.3.6.1.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.6.1.1.4.2-2

Table 8.3.6.1.1.4.2-2 AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz] |
|  |  | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz] |
|  |  | 200 | [EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz] |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8) The signal generator sends random codeword from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of NACK bits detected as ACK.

###### 8.3.6.1.1.5	Test Requirement

8.3.6.1.1.5.1	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in table 8.3.6.1.1.5.1-1.

Table 8.3.6.1.1.5.1-1: Required SNR for multi-slot PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel bandwidth (MHz) / SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 40 MHz |
| 1 | 2 | Normal | TDLC-300-100 Low | [ TBD ] |

8.3.6.1.1.5.2	Test Requirement for BS type 2-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in table 8.3.6.1.1.5.2-1.

Table 8.3.6.1.1.5.2-1: Required SNR for multi-slot PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel bandwidth (MHz) / SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 40 MHz |
| 1 | 2 | Normal | TDLC-300-100 Low | [ TBD ] |

##### 8.3.6.1.2	ACK missed detection

###### 8.3.6.1.2.1	Definition and applicability

The performance requirement of PUCCH format 1 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

A test for a specific combination of SCS and channel bandwidth is only applicable if the BS declares to support it.

For a BS supporting multiple combinations of SCS and channel bandwidth, the applicable rule is defined in 8.1.2.2.

###### 8.3.6.1.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.7.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.7.

###### 8.3.6.1.2.3	Test purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

###### 8.3.6.1.2.4	Method of test

8.3.6.1.2.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see sub-clause 4.9.1

Direction to be tested: OTA REFSENS receiver target reference direction (D.54).

8.3.6.1.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.6.1.2.4.2-1.

Table 8.3.6.1.2.4.2-1: Test Parameters for multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index - (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex TBD.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.6.1.2.5.1 and 8.3.6.1.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.6.1.2.4.2-2.

Table 8.3.6.1.2.4.2-2 AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | -83.5 dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | -80.3 dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | -77.2 dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | -80.6 dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | -77.4 dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | -74.2 dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | -70.1 dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz] |
|  |  | 50 | [EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz ] |
|  |  | 100 | [EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz ] |
|  |  | 200 | [EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz] |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in subclause7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8) The signal generator sends a test pattern with the pattern outlined in figure 8.3.6.1.2.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 8.3.6.1.2.4.2-1: Test signal pattern for PUCCH format 1 demodulation tests

###### 8.3.6.1.2.5	Test Requirement

8.3.6.1.2.5.1	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of correctly detected ACK bits shall be larger than 99% for the SNR listed in table 8.3.6.1.2.5.1-1.

Table 8.3.6.1.2.5.1-1: Required SNR for multi-slot PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel bandwidth (MHz) / SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 40 MHz |
| 1 | 2 | Normal | TDLC-300-100 Low | [ TBD ] |

8.3.6.1.2.5.2	Test Requirement for BS type 2-O

The fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in table 8.3.6.1.2.5.2-1.

Table 8.3.6.1.2.5.2-1: Required SNR for multi-slot PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel bandwidth (MHz) / SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 40 MHz |
| 1 | 2 | Normal | TDLC-300-100 Low | [ TBD ] |

## 8.4	OTA performance requirements for PRACH

### 8.4.1	PRACH false alarm probability and missed detection

#### 8.4.1.1	Definition and applicability

The performance requirement of PRACH for preamble detection is determined by the two parameters: total probability of false detection of the preamble (Pfa) and the probability of detection of preamble (Pd). The performance is measured by the required SNR at probability of detection, Pd of 99%. Pfa shall be 0.1% or less.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

Pd is defined as conditional probability of detection of the preamble when the signal is present. The erroneous detection consists of several error cases – detecting only different preamble(s) than the one that was sent, not detecting any preamble at all, or detecting the correct preamble but with the out-of-bounds timing estimation value. For AWGN, TDLC300-100 and TDLA30-300, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than the time error tolerance values given in table 8.4.1.1-1.

Table 8.4.1.1-1: Time error tolerance for AWGN, TDLC300-100 and TDLA30-300

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |  |
| --- | --- | --- | --- | --- |
|  |  | AWGN | TDLC300-100 | TDLA30-300 |
| 0 | 1.25 | 1.04 us | 2.55 us | N/A |
| A1, A2, A3, B4, C0, C2 | 15 | 0.52 us | 2.03 us | N/A |
|  | 30 | 0.26 us | 1.77 us | N/A |
|  | 60 (FR2) | 0.13 us | N/A | 0.28 us |
|  | 120 | 0.07 us | N/A | 0.22 us |

The test preambles for normal mode are listed in table A.6-1 and A.6-2.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.

#### 8.4.1.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2] subclause 11.4.1.1 and 11.4.1.2.

For BS type 2-O, the minimum requirement is in TS 38.104 [2] subclause 11.4.2.1 and 11.4.2.2.

#### 8.4.1.3	Test purpose

The test shall verify the receiver’s ability to detect PRACH preamble under static conditions and multipath fading propagation conditions for a given SNR.

#### 8.4.1.4	Method of test

##### 8.4.1.4.1	Initial conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	for single carrier (SC): M; see subclause 4.9.1.

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (see D.31 in table 4.6-1).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

##### 8.4.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameter msg1-FrequencyStart is set to 0.

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the AWGN generator, according to the SCS and channel bandwidth. The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.4.1.4.2-1.

Table 8.4.1.4.2-1: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -83.5 - ΔOTAREFSENS dBm / 4.5MHz |
|  |  | 10 | -80.3 - ΔOTAREFSENS dBm / 9.36MHz |
|  |  | 20 | -77.2 - ΔOTAREFSENS dBm / 19.08MHz |
|  | 30 | 10 | -80.6 - ΔOTAREFSENS dBm / 8.64MHz |
|  |  | 20 | -77.4 - ΔOTAREFSENS dBm / 18.36MHz |
|  |  | 40 | -74.2 - ΔOTAREFSENS dBm / 38.16MHz |
|  |  | 100 | -70.1 - ΔOTAREFSENS dBm / 98.28MHz |
| BS type 2-O | 60 | 50 | [EISREFSENS_50M + 12 dBm / 47.52MHz] |
|  |  | 100 | [EISREFSENS_50M + 15 dBm / 95.04 MHz] |
|  | 120 | 50 | [EISREFSENS_50M + 12 dBm / 46.08 MHz] |
|  |  | 100 | [EISREFSENS_50M + 15 dBm / 95.04 MHz] |
|  |  | 200 | [EISREFSENS_50M + 18 dBm / 190.08 MHz] |

8)	Adjust the frequency offset of the test signal according to table 8.4.1.5.1-1 or 8.4.1.5.1-2 or 8.4.1.5.1-3 or 8.4.1.5.2-1 or 8.4.1.5.2-2.

9)	Adjust the equipment so that the SNR specified in table 8.4.1.5.1-1 or 8.4.1.5.1-2 or 8.4.1.5.1-3 or 8.4.1.5.2-1 or 8.4.1.5.2-2 is achieved at the BS input during the PRACH preambles.

10) The test signal generator sends a preamble and the receiver tries to detect the preamble. This pattern is repeated as illustrated in figure 8.4.1.4.2-1. The preambles are sent with certain timing offsets as described below. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 8.4.1.4.2-1: PRACH preamble test pattern

The timing offset base value for PRACH preamble format 0 is set to 50% of Ncs. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.9us. Then the loop is being reset and the timing offset is set again to 50% of Ncs. The timing offset scheme for PRACH preamble format 0 is presented in Figure 8.4.1.4.2-2.

Figure 8.4.1.4.2-2: Timing offset scheme for PRACH preamble format 0

The timing offset base value for PRACH preamble format A1, A2, A3, B4, C0 and C2 is set to 0. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.8us. Then the loop is being reset and the timing offset is set again to 0. The timing offset scheme for PRACH preamble format A1, A2, A3, B4, C0 and C2 is presented in Figure 8.4.1.4.2-3.

Figure 8.4.1.4.2-3: Timing offset scheme for PRACH preamble format A1 A2, A3, B4, C0 and C2

#### 8.4.1.5	Test requirement

##### 8.4.1.5.1	Test requirement for BS type 1-O

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in tables 8.4.1.5.1-1 to 8.4.1.5.1-3.

Table 8.4.1.5.1-1: PRACH missed detection test requirements for Normal Mode, 1.25 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | 0 | [-14.2] |
|  |  | TDLC300-100 Low | 400 Hz | [-6.0] |

Table 8.4.1.5.1-2: PRACH missed detection test requirements for Normal Mode, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-9.0] | [-12.3] | [-13.9] | [-16.5] | [-6.0] | [-12.2] |
|  |  | TDLC300-100 Low | 400 Hz | [-1.5] | [-4.2] | [-6.0] | [-8.2] | [1.4] | [-4.3] |

Table 8.4.1.5.1-3: PRACH missed detection test requirements for Normal Mode, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-8.8] | [-11.7] | [-13.5] | [-16.2] | [-5.8] | [-11.6] |
|  |  | TDLC300-100 Low | 400 Hz | [-2.2] | [-5.1] | [-6.8] | [-9.3] | [0.7] | [-5.0] |

##### 8.4.1.5.2	Test requirement for BS type 2-O

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in tables 8.4.1.5.2-1 to 8.4.1.5.2-2.

Table 8.4.1.5.2-1: PRACH missed detection test requirements for Normal Mode, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-8.6] | [-11.6] | [-13.2] | [-16.3] | [-5.7] | [-11.5] |
|  |  | TDLA30-300 Low | 4000 Hz | [-1.0] | [-3.2] | [-4.2] | [-6.3] | [1.7] | [-3.3] |

Table 8.4.1.5.2-2: PRACH missed detection test requirements for Normal Mode, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex J) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-8.4] | [-11.2] | [-13.0] | [-15.6] | [-5.5] | [-11.1] |
|  |  | TDLA30-300 Low | 4000 Hz | [-1.1] | [-3.8] | [-5.2] | [-6.9] | [1.8] | [-3.6] |

# Annex A (normative):
Reference measurement channels

# A.1	Fixed Reference Channels for OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity (QPSK, R=1/3)

The parameters for the reference measurement channels are specified in table A.1-1 for FR1 OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity.

The parameters for the reference measurement channels are specified in table A.1-2 for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity.

Table A.1-1: FRC parameters for FR1 OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR1-A1-1 | G-FR1-A1-2 | G-FR1-A1-3 | G-FR1-A1-4 | G-FR1-A1-5 | G-FR1-A1-6 | G-FR1-A1-7 | G-FR1-A1-8 | G-FR1-A1-9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 60 | 15 | 30 | 60 | 15 | 30 | 60 |
| Allocated resource blocks | 25 | 11 | 11 | 106 | 51 | 24 | 15 | 6 | 6 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 2152 | 984 | 984 | 9224 | 4352 | 2088 | 1320 | 528 | 528 |
| Transport block CRC (bits) | 16 | 16 | 16 | 24 | 24 | 16 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | 24 | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 2 | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 2168 | 1000 | 1000 | 4648 | 4376 | 2104 | 1336 | 544 | 544 |
| Total number of bits per slot | 7200 | 3168 | 3168 | 30528 | 14688 | 6912 | 4320 | 1728 | 1728 |
| Total symbols per slot | 3600 | 1584 | 1584 | 15264 | 7344 | 3456 | 2160 | 864 | 864 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in TS 38.212 [19], subclause 5.2.2. |  |  |  |  |  |  |  |  |  |

Table A.1-2: FRC parameters for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR2-A1-1 | G-FR2-A1-2 | G-FR2-A1-3 | G-FR2-A1-4 | G-FR2-A1-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 | 120 | 60 | 120 |
| Allocated resource blocks | 66 | 32 | 66 | 33 | 16 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 5632 | 2792 | 5632 | 2856 | 1416 |
| Transport block CRC (bits) | 24 | 16 | 24 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 5656 | 2808 | 5656 | 2872 | 1432 |
| Total number of bits per slot | 19008 | 9216 | 19008 | 9504 | 4608 |
| Total symbols per slot | 9504 | 4608 | 9504 | 4752 | 2304 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in TS 38.212 [19], subclause 5.2.2. |  |  |  |  |  |

# A.2	Fixed Reference Channels for OTA dynamic range (16QAM, R=2/3)

The parameters for the reference measurement channels are specified in table A.2-1 for FR1 OTA dynamic range.

Table A.2-1: FRC parameters for FR1 OTA dynamic range

| Reference channel | G-FR1-A2-1 | G-FR1-A2-2 | G-FR1-A2-3 | G-FR1-A2-4 | G-FR1-A2-5 | G-FR1-A2-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 60 | 15 | 30 | 60 |
| Allocated resource blocks | 25 | 11 | 11 | 106 | 51 | 24 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 2/3 | 2/3 | 2/3 | 2/3 | 2/3 | 2/3 |
| Payload size (bits) | 9224 | 4032 | 4032 | 38936 | 18960 | 8968 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | - | - | 24 | 24 | 24 |
| Number of code blocks - C | 2 | 1 | 1 | 5 | 3 | 2 |
| Code block size including CRC (bits) (Note 3) | 4648 | 4056 | 4056 | 7816 | 6352 | 4520 |
| Total number of bits per slot | 14400 | 6336 | 6336 | 61056 | 29376 | 13824 |
| Total symbols per slot | 3600 | 1584 | 1584 | 15264 | 7344 | 3456 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: MCS index 16 and target coding rate = 658/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in TS 38.212 [19], subclause 5.2.2. |  |  |  |  |  |  |

# A.3	Fixed Reference Channels for performance requirements (QPSK, R=193/1024)

The parameters for the reference measurement channels are specified in table A.3-2, table A.3-4 and table A.3-6 for FR1 PUSCH performance requirements:

-	FRC parameters are specified in table A.3-2 for FR1 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

-	FRC parameters are specified in table A.3-4 for FR1 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers.

-	FRC parameters are specified in table A.3-6 for FR1 PUSCH with transform precoding enabled, additional DM-RS position = pos1 and 1 transmission layer.

The parameters for the reference measurement channels are specified in table A.3-7 to table A.3-12 for FR2 PUSCH performance requirements:

-	FRC parameters are specified in table A.3-7 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos0 and 1 transmission layer.

-	FRC parameters are specified in table A.3-8 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos0 and 2 transmission layer.

-	FRC parameters are specified in table A.3-9 for FR2 PUSCH with transform precoding enabled, additional DM-RS position = pos0 and 1 transmission layer.

-	FRC parameters are specified in table A.3-10 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

-	FRC parameters are specified in table A.3-11 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layer.

-	FRC parameters are specified in table A.3-12 for FR2 PUSCH with transform precoding enabled, additional DM-RS position = pos1 and 1 transmission layer.

Table A.3-1: Void

Table A.3-2: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=193/1024)

| Reference channel | G-FR1-A3-8 | G-FR1-A3-9 | G-FR1-A3-10 | G-FR1-A3-11 | G-FR1-A3-12 | G-FR1-A3-13 | G-FR1-A3-14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 |
| Payload size (bits) | 1352 | 2856 | 5768 | 1320 | 2792 | 5768 | 14856 |
| Transport block CRC (bits) | 16 | 16 | 24 | 16 | 16 | 24 | 24 |
| Code block CRC size (bits) | - | - | 24 | - | - | 24 | 24 |
| Number of code blocks - C | 1 | 1 | 2 | 1 | 1 | 2 | 4 |
| Code block size including CRC (bits) (Note 2) | 1368 | 2872 | 2920 | 1336 | 2808 | 2920 | 3744 |
| Total number of bits per slot | 7200 | 14976 | 30528 | 6912 | 14688 | 30528 | 78624 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39312 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |  |  |

Table A.3-3: Void

Table A.3-4: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers (QPSK, R=193/1024)

| Reference channel | G-FR1-A3-22 | G-FR1-A3-23 | G-FR1-A3-24 | G-FR1-A3-25 | G-FR1-A3-26 | G-FR1-A3-27 | G-FR1-A3-28 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 |
| Payload size (bits) | 2728 | 5640 | 11528 | 2600 | 5512 | 11528 | 29736 |
| Transport block CRC (bits) | 16 | 24 | 24 | 16 | 24 | 24 | 24 |
| Code block CRC size (bits) | - | 24 | 24 | - | 24 | 24 | 24 |
| Number of code blocks - C | 1 | 2 | 4 | 1 | 2 | 4 | 8 |
| Code block size including CRC (bits) (Note 2) | 2744 | 2856 | 2912 | 2616 | 2792 | 2912 | 3744 |
| Total number of bits per slot | 14400 | 29952 | 61056 | 13824 | 29376 | 61056 | 157248 |
| Total symbols per slot | 7200 | 14976 | 30528 | 6912 | 14688 | 30528 | 78624 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |  |  |

Table A.3-5: Void

Table A.3-6: FRC parameters for FR1 PUSCH performance requirements, transform precoding enabled, additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=193/1024)

| Reference channel | G-FR1-A3-31 | G-FR1-A3-32 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 |
| Allocated resource blocks | 25 | 24 |
| DFT-s-OFDM Symbols per slot (Note 1) | 12 | 12 |
| Modulation | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 |
| Payload size (bits) | 1352 | 1320 |
| Transport block CRC (bits) | 16 | 16 |
| Code block CRC size (bits) | - | - |
| Number of code blocks - C | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 1368 | 1336 |
| Total number of bits per slot | 7200 | 6912 |
| Total symbols per slot | 3600 | 3456 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |

Table A.3-7: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos0 and 1 transmission layer (QPSK, R=193/1024)

| Reference channel | G-FR2-A3-1 | G-FR2-A3-2 | G-FR2-A3-3 | G-FR2-A3-4 | G-FR2-A3-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 |
| Payload size (bits) | 2664 | 5384 | 1320 | 2664 | 5384 |
| Transport block CRC (bits) | 16 | 24 | 16 | 16 | 24 |
| Code block CRC size (bits) | - | 24 | - | - | 24 |
| Number of code blocks - C | 1 | 2 | 1 | 1 | 2 |
| Code block size including CRC (bits) (Note 2) | 2680 | 2728 | 1336 | 2680 | 2728 |
| Total number of bits per slot | 14256 | 28512 | 6912 | 14256 | 28512 |
| Total symbols per slot | 7128 | 14256 | 3456 | 7128 | 14256 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos0 with l0= 0 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

Table A.3-8: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos0 and 2 transmission layers (QPSK, R=193/1024)

| Reference channel | G-FR2-A3-6 | G-FR2-A3-7 | G-FR2-A3-8 | G-FR2-A3-9 | G-FR2-A3-10 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 |
| Payload size (bits) | 5384 | 10752 | 2600 | 5384 | 10752 |
| Transport block CRC (bits) | 24 | 24 | 16 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | - | 24 | 24 |
| Number of code blocks - C | 2 | 3 | 1 | 2 | 3 |
| Code block size including CRC (bits) (Note 2) | 2728 | 3616 | 2616 | 2728 | 3616 |
| Total number of bits per slot | 28512 | 57024 | 13824 | 28512 | 57024 |
| Total symbols per slot | 14256 | 28512 | 6912 | 14256 | 28512 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos0 with l0= 0 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

Table A.3-9: FRC parameters for FR2 PUSCH performance requirements, transform precoding enabled, additional DM-RS position = pos0 and 1 transmission layer (QPSK, R=193/1024)

| Reference channel | G-FR2-A3-11 | G-FR2-A3-12 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 |
| Allocated resource blocks | 30 | 30 |
| DFT-s-OFDM Symbols per slot (Note 1) | 9 | 9 |
| Modulation | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 |
| Payload size (bits) | 1224 | 1224 |
| Transport block CRC (bits) | 16 | 16 |
| Code block CRC size (bits) | - | - |
| Number of code blocks - C | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 1240 | 1240 |
| Total number of bits per slot | 6480 | 6480 |
| Total symbols per slot | 3240 | 3240 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos0 with l0= 0 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |

Table A.3-10: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=193/1024)

| Reference channel | G-FR2-A3-13 | G-FR2-A3-14 | G-FR2-A3-15 | G-FR2-A3-16 | G-FR2-A3-17 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 |
| Payload size (bits) | 2408 | 4744 | 1160 | 2408 | 4744 |
| Transport block CRC (bits) | 16 | 24 | 16 | 16 | 24 |
| Code block CRC size (bits) | - | 24 | - | - | 24 |
| Number of code blocks - C | 1 | 2 | 1 | 1 | 2 |
| Code block size including CRC (bits) (Note 2) | 2424 | 2408 | 1176 | 2424 | 2408 |
| Total number of bits per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| Total symbols per slot | 6336 | 12672 | 3072 | 6336 | 12672 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1 with l0 = 0 and l = 8 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

Table A.3-11: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers (QPSK, R=193/1024)

| Reference channel | G-FR2-A3-18 | G-FR2-A3-19 | G-FR2-A3-20 | G-FR2-A3-21 | G-FR2-A3-22 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 |
| Payload size (bits) | 4744 | 9480 | 2408 | 4744 | 9480 |
| Transport block CRC (bits) | 24 | 24 | 16 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | - | 24 | 24 |
| Number of code blocks - C | 2 | 3 | 1 | 2 | 3 |
| Code block size including CRC (bits) (Note 2) | 2408 | 3192 | 2424 | 2408 | 3192 |
| Total number of bits per slot | 25344 | 50688 | 12288 | 25344 | 50688 |
| Total symbols per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1 with l0= 0 and l = 8 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

Table A.3-12: FRC parameters for FR2 PUSCH performance requirements, transform precoding enabled, additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=193/1024)

| Reference channel | G-FR2-A3-23 | G-FR2-A3-24 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 |
| Allocated resource blocks | 30 | 30 |
| DFT-s-OFDM Symbols per slot (Note 1) | 8 | 8 |
| Modulation | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 |
| Payload size (bits) | 1128 | 1128 |
| Transport block CRC (bits) | 16 | 16 |
| Code block CRC size (bits) | - | - |
| Number of code blocks - C | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 1144 | 1144 |
| Total number of bits per slot | 5760 | 5760 |
| Total symbols per slot | 2880 | 2880 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1 with l0 = 0 and l = 8 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |

# A.4	Fixed Reference Channels for performance requirements (16QAM, R=658/1024)

The parameters for the reference measurement channels are specified in table A.4-2 and table A.4-4 for FR1 PUSCH performance requirements:

-	FRC parameters are specified in table A.4-2 for FR1 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

-	FRC parameters are specified in table A.4-4 for FR1 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers.

The parameters for the reference measurement channels are specified in table A.4-5 to table A.4-8 for FR2 PUSCH performance requirements:

-	FRC parameters are specified in table A.4-5 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos0 and 1 transmission layer.

-	FRC parameters are specified in table A.4-6 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos0 and 2 transmission layers.

-	FRC parameters are specified in table A.4-7 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

-	FRC parameters are specified in table A.4-8 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers.

Table A.4-1: Void

Table A.4-2: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR1-A4-8 | G-FR1-A4-9 | G-FR1-A4-10 | G-FR1-A4-11 | G-FR1-A4-12 | G-FR1-A4-13 | G-FR1-A4-14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 |
| Payload size (bits) | 9224 | 19464 | 38936 | 8968 | 18960 | 38936 | 100392 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 2 | 3 | 5 | 2 | 3 | 5 | 12 |
| Code block size including CRC (bits) (Note 2) | 4648 | 6052 | 7816 | 4520 | 6352 | 7816 | 8392 |
| Total number of bits per slot | 14400 | 29952 | 61056 | 13824 | 29376 | 61056 | 157248 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39312 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |  |  |

Table A.4-3: Void

Table A.4-4: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers (16QAM, R=658/1024)

| Reference channel | G-FR1-A4-22 | G-FR1-A4-23 | G-FR1-A4-24 | G-FR1-A4-25 | G-FR1-A4-26 | G-FR1-A4-27 | G-FR1-A4-28 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 |
| Payload size (bits) | 18432 | 38936 | 77896 | 17928 | 37896 | 77896 | 200808 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 5 | 10 | 3 | 5 | 10 | 24 |
| Code block size including CRC (bits) (Note 2) | 6176 | 7816 | 7816 | 6008 | 7608 | 7816 | 8392 |
| Total number of bits per slot | 28800 | 59904 | 122112 | 27648 | 58752 | 122112 | 314496 |
| Total symbols per slot | 7200 | 14976 | 30528 | 6912 | 14688 | 30528 | 78624 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |  |  |

Table A.4-5: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos0 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-A4-1 | G-FR2-A4-2 | G-FR2-A4-3 | G-FR2-A4-4 | G-FR2-A4-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 |
| Payload size (bits) | 18432 | 36896 | 8968 | 18432 | 36896 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 5 | 2 | 3 | 5 |
| Code block size including CRC (bits) (Note 2) | 6176 | 7408 | 4520 | 6176 | 7408 |
| Total number of bits per slot | 28512 | 57024 | 13824 | 28512 | 57024 |
| Total symbols per slot | 7128 | 14256 | 3456 | 7128 | 14256 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos0 with l0= 0 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

Table A.4-6: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos0 and 2 transmission layers (16QAM, R=658/1024)

| Reference channel | G-FR2-A4-6 | G-FR2-A4-7 | G-FR2-A4-8 | G-FR2-A4-9 | G-FR2-A4-10 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 |
| Payload size (bits) | 36896 | 73776 | 17928 | 36896 | 73776 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 5 | 9 | 3 | 5 | 9 |
| Code block size including CRC (bits) (Note 2) | 7408 | 8224 | 6008 | 7408 | 8224 |
| Total number of bits per slot | 57024 | 114048 | 27648 | 57024 | 114048 |
| Total symbols per slot | 14256 | 28512 | 6912 | 14256 | 28512 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos0 with l0= 0 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

Table A.4-7: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR2-A4-11 | G-FR2-A4-12 | G-FR2-A4-13 | G-FR2-A4-14 | G-FR2-A4-15 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 |
| Payload size (bits) | 16392 | 32776 | 7936 | 16392 | 32776 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | - | 24 | 24 |
| Number of code blocks - C | 2 | 4 | 1 | 2 | 4 |
| Code block size including CRC (bits) (Note 2) | 8232 | 8224 | 7960 | 8232 | 8224 |
| Total number of bits per slot | 25344 | 50688 | 12288 | 25344 | 50688 |
| Total symbols per slot | 6336 | 12672 | 3072 | 6336 | 12672 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1 with l0 = 0 and l = 8 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

Table A.4-8: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers (16QAM, R=658/1024)

| Reference channel | G-FR2-A4-16 | G-FR2-A4-17 | G-FR2-A4-18 | G-FR2-A4-19 | G-FR2-A4-20 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 |
| Payload size (bits) | 32776 | 65576 | 15880 | 32776 | 65576 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 4 | 8 | 2 | 4 | 8 |
| Code block size including CRC (bits) (Note 2) | 8224 | 8224 | 7976 | 8224 | 8224 |
| Total number of bits per slot | 50688 | 101376 | 24576 | 50688 | 101376 |
| Total symbols per slot | 12672 | 25344 | 6144 | 12672 | 25344 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1 with l0 = 0 and l = 8 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

# A.5	Fixed Reference Channels for performance requirements (64QAM, R=567/1024)

The parameters for the reference measurement channels are specified in table A.5-2 for FR1 PUSCH performance requirements with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

The parameters for the reference measurement channels are specified in table A.5-3 to table A.5-4 for FR2 PUSCH performance requirements:

-	FRC parameters are specified in table A.5-3 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos0 and 1 transmission layer.

-	FRC parameters are specified in table A.5-4 for FR2 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

Table A.5-1: Void

Table A.5-2: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (64QAM, R=567/1024)

| Reference channel | G-FR1-A5-8 | G-FR1-A5-9 | G-FR1-A5-10 | G-FR1-A5-11 | G-FR1-A5-12 | G-FR1-A5-13 | G-FR1-A5-14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Code rate (Note 2) | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 |
| Payload size (bits) | 12040 | 25104 | 50184 | 11528 | 24576 | 50184 | 131176 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 2 | 3 | 6 | 2 | 3 | 6 | 16 |
| Code block size including CRC (bits) (Note 2) | 6056 | 8400 | 8392 | 5800 | 8224 | 8392 | 8224 |
| Total number of bits per slot | 21600 | 44928 | 91584 | 20736 | 44064 | 91584 | 235872 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39312 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |  |  |

Table A.5-3: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos0 and 1 transmission layer (64QAM, R=567/1024)

| Reference channel | G-FR2-A5-1 | G-FR2-A5-2 | G-FR2-A5-3 | G-FR2-A5-4 | G-FR2-A5-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 |
| Modulation | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Code rate (Note 2) | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 |
| Payload size (bits) | 23568 | 47112 | 11528 | 23568 | 47112 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 2 | 3 | 6 |
| Code block size including CRC (bits) (Note 2) | 7888 | 7880 | 5800 | 7888 | 7880 |
| Total number of bits per slot | 42768 | 85536 | 20736 | 42768 | 85536 |
| Total symbols per slot | 7128 | 14256 | 3456 | 7128 | 14256 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos0 with l0= 0 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

Table A.5-4: FRC parameters for FR2 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (64QAM, R=567/1024)

| Reference channel | G-FR2-A5-6 | G-FR2-A5-7 | G-FR2-A5-8 | G-FR2-A5-9 | G-FR2-A5-10 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 60 | 120 | 120 | 120 |
| Allocated resource blocks | 66 | 132 | 32 | 66 | 132 |
| CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 | 8 | 8 |
| Modulation | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Code rate (Note 2) | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 |
| Payload size (bits) | 21000 | 42016 | 10248 | 21000 | 42016 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 5 | 2 | 3 | 5 |
| Code block size including CRC (bits) (Note 2) | 7032 | 8432 | 5160 | 7032 | 8432 |
| Total number of bits per slot | 38016 | 76032 | 18432 | 38016 | 76032 |
| Total symbols per slot | 6336 | 12672 | 3072 | 6336 | 12672 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1 with l0 = 0 and l = 8 as per table 6.4.1.1.3-3 of TS 38.211 [20]. NOTE 2: Code block size including CRC (bits) equals to K' in subclause 5.2.2 of TS 38.212 [19]. |  |  |  |  |  |

# A.6	PRACH Test preambles

Table A.6-1 Test preambles for Normal Mode in FR1

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| A1, A2, A3, B4, C0, C2 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

Table A.6-2 Test preambles for Normal Mode in FR2

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| A1, A2, A3, B4, C0, C2 | 60 | 69 | 0 | 0 |
|  | 120 | 69 | 0 | 0 |

Annex B (normative):
Environmental requirements for the BS equipment

# B.1	General

For each test in the present document, the environmental conditions under which the BS is to be tested are defined.

For OTA requirements where it is not possible to environmentally control the entire calibrated OTA chamber either localised control of the BS hardware or alternative OTA measurements which are then related to the original specification are acceptable, see annex B.7.

# B.2	Normal test environment

When a normal test environment is specified for a test, the test should be performed within the minimum and maximum limits of the conditions stated in table D.1.

Table B.1: Limits of conditions for normal test environment

| Condition | Minimum | Maximum |
| --- | --- | --- |
| Barometric pressure | 86 kPa | 106 kPa |
| Temperature | 15 C | 30 C |
| Relative humidity | 20 % | 85 % |
| Power supply | Nominal, as declared by the manufacturer |  |
| Vibration | Negligible |  |

The ranges of barometric pressure, temperature and humidity represent the maximum variation expected in the uncontrolled environment of a test laboratory. If it is not possible to maintain these parameters within the specified limits, the actual values shall be recorded in the test report.

NOTE:	This may, for instance, be the case for measurements of radiated emissions performed on an open field test site.

# B.3	Extreme test environment

The manufacturer shall declare one of the following:

1)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-3 [7];

2)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-4 [8];

3)	The equipment that does not comply with the mentioned classes, the relevant classes from IEC 60 721 [9] documentation for temperature, humidity and vibration shall be declared.

NOTE:	Reduced functionality for conditions that fall outside of the standard operational conditions is not tested in the present document. These may be stated and tested separately.

## B.3.1	Extreme temperature

When an extreme temperature test environment is specified for a test, the test shall be performed at the standard minimum and maximum operating temperatures defined by the manufacturer's declaration for the equipment under test.

Minimum temperature:

The test shall be performed with the environment test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-1 [10].

Maximum temperature:

The test shall be performed with the environmental test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-2 [11].

NOTE:	It is recommended that the equipment is made fully operational prior to the equipment being taken to its lower operating temperature.

# B.4	Vibration

When vibration conditions are specified for a test, the test shall be performed while the equipment is subjected to a vibration sequence as defined by the manufacturer’s declaration for the equipment under test. This shall use the environmental test equipment and methods of inducing the required environmental phenomena in to the equipment, conforming to the test procedure of IEC 60 068-2-6 [12]. Other environmental conditions shall be within the ranges specified in annex B.2.

NOTE:	The higher levels of vibration may induce undue physical stress in to equipment after a prolonged series of tests. The testing body should only vibrate the equipment during the RF measurement process.

# B.5	Power supply

When extreme power supply conditions are specified for a test, the test shall be performed at the standard upper and lower limits of operating voltage defined by manufacturer's declaration for the equipment under test.

Upper voltage limit:

The equipment shall be supplied with a voltage equal to the upper limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [10] Test Ab/Ad and IEC 60 068-2-2 [11] Test Bb/Bd: Dry heat.

Lower voltage limit:

The equipment shall be supplied with a voltage equal to the lower limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [10] Test Ab/Ad and IEC 60 068-2-2 [11] Test Bb/Bd: Dry heat.

# B.6	Measurement of test environments

The measurement accuracy of the BS test environments defined in annex B shall be:

Pressure:	5 kPa

Temperature:	2 degrees

Relative humidity:	5 %

DC voltage:	1.0 %

AC voltage:	1.5 %

Vibration:	10 %

Vibration frequency:	0.1 Hz

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

# B.7	OTA extreme test methods

## B.7.1	Direct far field method

The BS under test is placed inside a sealed RF transparent environmental enclosure, as showed in Figure B.7.1-1. This is connected to an environment control system which regulates the temperature inside the enclosure. The remaining equipment inside the OTA chamber (any suitable antenna test range chamber type is acceptable) is outside the environmental control and is at nominal temperature. Positioners, test antennas and all other OTA test equipment do not need to be specified over the extreme temperature range.

Figure B.7.1-1: Measurement set up for extreme conditions for EIRP accuracy using direct far field method

The presence of the environmental chamber inside the OTA chamber may affect the measurement accuracy due to additional reflections and refractions, also the loss through the environmental enclosure may not be consistent with direction as the path through the radome may vary with angle. Hence the system should be calibrated in all tested directions, frequencies and temperatures if necessary.

NOTE: Currently only a single direction is specified for extreme testing so a single calibration direction is sufficient.

Conformance may be demonstrated by measuring the difference between the nominal measurement and the extreme measurement (Δsample) or by measuring Pmax,c,EIRP, extreme directly.

Measure EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

## B.7.2	Relative method

The BS under test is placed inside a small (compared to a far field chamber) anechoic chamber which is both RF a screened and suitable for environmental conditioning. The RF conditions inside the chamber are absorptive and capable of dissipating the power of the BS when radiating. A sample antenna or RF probe are placed in a location which gives a sample of the main beam EIRP but does not have to accurately measure the EIRP directly, instead the near-field response is measured. For this method test components are exposed to the full temperature range for example the test antenna/probe, cables, absorbers etc. may change as a function of temperature.

Using the relative method it is also necessary to measure the EIRP under nominal conditions using an appropriately calibrated far field (or near filed) test range to obtain Pmax,c,EIRP.

Figure B.7.2-1: Measurement set up for extreme conditions for EIRP accuracy using difference method

Measurements from the test antenna/probe are taken under nominal conditions and extreme conditions to calculate (Δsample). The difference between the nominal and extreme conditions (Δsample) is then used along with the nominal EIRP measurement (Pmax,c,EIRP) made in the appropriate far field or near field chamber and compared against the extreme requirement. As follows:

Pmax,c,EIRP, extreme = Pmax,c,EIRP + Δsample.

Measure EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

Annex C (informative):
Test tolerances and derivation of test requirements

The test requirements explicitly defined in this specification have been calculated by relaxing the minimum requirements of the core specification TS 38.104 [2] using the test tolerances (TT) defined here. When the TT value is zero, the test requirement will be the same as the minimum requirement. When the TT value is non-zero, the test requirements will differ from the minimum requirements, and the formula used for this relaxation is given in the following tables.

The TTOTA values are derived from OTA Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the TTOTA values may sometimes be set to zero.

The TTOTA values should not be modified for any reason e.g. to take account of commonly known OTA Test System errors (such as mismatch, cable loss, etc.).

Note that a formula for applying TTOTA values is provided for all OTA tests, even those with a test tolerance of zero. This is necessary in the case where the OTA Test System uncertainty is greater than that allowed in subclause 4.1.2. In this event, the excess error shall be subtracted from the defined TTOTA value in order to generate the correct tightened test requirements as defined in this annex.

# C.1	Measurement of transmitter

Table C.1-1: Derivation of test requirements (FR1 OTA transmitter tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See TS 38.104 [2], subclause 9.2 | Normal conditions: 1.1 dB, f ≤ 3.0 GHz 1.3 dB, 3.0 GHz < f ≤ 4.2 GHz 1.3 dB, 4.2 GHz < f ≤ 6.0 GHz Extreme conditions: 2.5 dB, f ≤ 3.0 GHz 2.6 dB, 3.0 GHz < f ≤ 4.2 GHz 2.6 dB, 4.2 GHz < f ≤ 6.0 GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 OTA base station output power | See TS 38.104 [2], subclause 9.3 | 1.4 dB, f ≤ 3.0 GHz 1.5 dB, 3.0 GHz < f ≤ 4.2 GHz 1.5 dB, 4.2 GHz < f ≤ 6.0 GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4 OTA output power dynamics | See TS 38.104 [2], subclause 9.4 | 0.4 dB | Formula: Total power dynamic range – TT |
| 6.5.1 OTA transmitter OFF power | See TS 38.104 [2], subclause 9.5.2 | 3.4 dB , f ≤ 3.0GHz 3.6 dB, 3.0GHz < f ≤ 4.2GHz 3.6 dB, 4.2GHz < f ≤ 6.0GHz | Formula: Minimum Requirement + TT |
| 6.6.2 OTA frequency Error | See TS 38.104 [2], subclause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.6.3 OTA Modulation quality (EVM) | See TS 38.104 [2], subclause 9.6.2 | 1% | Formula: EVM limit + TT |
| 6.6.4 OTA time alignment error | See TS 38.104 [2], subclause 9.6.3 | 25 ns |  |
| 6.7.2 OTA occupied bandwidth | See TS 38.104 [2], subclause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | See TS 38.104 [2], subclause 9.7.3 | Relative: 1.0 dB, f ≤ 3.0GHz 1.2 dB, 3.0GHz < f ≤ 4.2GHz 1.2 dB, 4.2GHz < f ≤ 6.0GHz Absolute: 0 dB | Formula: Relative limit - TT Absolute limit +TT |
| 6.7.4 OTA operating band unwanted emissions | See TS 38.104 [2], subclause 9.7.4 | Offsets < 10MHz 1.8 dB, f ≤ 3.0GHz 2 dB, 3.0GHz < f ≤ 4.2GHz 2 dB, 4.2GHz < f ≤ 6.0GHz Offsets ≥ 10MHz 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category A | See TS 38.104 [2], subclause 9.7.5.2.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category B | See TS 38.104 [2], subclause 9.7.5.2.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.3 Protection of the BS receiver of own or different BS | See TS 38.104 [2], subclause 9.7.5.2.3 | 3.1 dB, f ≤ 3.0GHz 3.3 dB, 3.0GHz < f ≤ 4.2GHz 3.4 dB, 4.2GHz < f ≤ 6.0GHz | Formula: Minimum Requirement + TT |
| 6.7.5.4 Additional spurious emissions requirements | See TS 38.104 [2], subclause 9.7.5.2.4 | 2.6 dB, f ≤ 3 GHz 3.0 dB, 3 GHz < f ≤ 4.2 GHz 3.5 dB, 4.2 GHz < f ≤ 6 GHz For co-existence with PHS 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.5 Co-location with other base stations | See TS 38.104 [2], subclause 9.7.5.2.5 | 3.1 dB, f ≤ 3.0GHz 3.3 dB, 3.0GHz < f ≤ 4.2GHz 3.4 dB, 4.2GHz < f ≤ 6.0GHz | Formula: Minimum Requirement + TT |
| 6.8 OTA transmitter intermodulation | See TS 38.104 [2], subclause 9.8 | 0 dB |  |

Table C.1-2: Derivation of test requirements (FR2 OTA transmitter tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See TS 38.104 [2], subclause 9.2 | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 40GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 40GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 OTA base station output power | See TS 38.104 [2], subclause 9.3 | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 40GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4 OTA output power dynamics | See TS 38.104 [2], subclause 9.4 | 0.4 dB | Formula: Total power dynamic range – TT |
| 6.5.1 OTA transmitter OFF power | See TS 38.104 [2], subclause 9.5.2 | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 40GHz dB | Formula: Minimum Requirement + TT |
| 6.6.2 OTA frequency Error | See TS 38.104 [2], subclause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.6.3 OTA Modulation quality (EVM) | See TS 38.104 [2], subclause 9.6.2 | 1 % | Formula: EVM limit + TT |
| 6.6.4 OTA time alignment error | See TS 38.104 [2], subclause 9.6.3 | 25 ns |  |
| 6.7.2 OTA occupied bandwidth | See TS 38.104 [2], subclause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | See TS 38.104 [2], subclause 9.7.3 | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦ 40GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦ 40GHz | Formula: Relative limit - TT Absolute limit +TT |
| 6.7.4 OTA operating band unwanted emissions | See TS 38.104 [2], subclause 9.7.4 | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦ 40GHz 0.1*BWcontiguous f < fmax 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category A | See TS 38.104 [2], subclause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category B | See TS 38.104 [2], subclause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.4 Additional spurious emissions requirements | See TS 38.104 [2], subclause 9.7.5.3.3 | TBD dB | Formula: Minimum Requirement + TT |

# C.2	Measurement of receiver

Table C.2-1: Derivation of test requirements (FR1 OTA receiver tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.2 OTA sensitivity | See TS 38.104 [2], subclause 10.2 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz 1.6 dB, 4.2 GHz < f ≤ 6.0 GHz | Formula: Declared Minimum EIS + TT |
| 7.3 OTA reference sensitivity level | See TS 38.104 [2], subclause 10.3 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz 1.6 dB, 4.2 GHz < f ≤ 6.0 GHz | Formula: EISREFSENS + TT |
| 7.4 OTA dynamic range | See TS 38.104 [2], subclause 10.4 | 0.3 dB, f ≤ 6 GHz | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.1 OTA adjacent channel selectivity | See TS 38.104 [2], subclause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking (General) | See TS 38.104 [2], subclause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking (Narrowband) | See TS 38.104 [2], subclause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking (General) | See TS 38.104 [2], subclause 10.6 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking (Co-location) | See TS 38.104 [2], subclause 10.6 | 0 dB | Formula: Wanted signal power unchanged Interferer signal power - TT. |
| 7.7 OTA receiver spurious emissions | See TS 38.104 [2], subclause 10.7 | 2.5dB, 30 MHz ≤ f ≤ 6 GHz 4.2dB, 6 GHz < f ≤ 26 GHz | Formula: Minimum Requirement + TT |
| 7.8 OTA receiver intermodulation | See TS 38.104 [2], subclause 10.8 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| 7.9 OTA in-channel selectivity | See TS 38.104 [2], subclause 10.9 | 1.7 dB, f ≤ 3.0 GHz 2.1 dB, 3.0 GHz < f ≤ 4.2 GHz 2.4 dB, 4.2 GHz < f ≤ 6.0 GHz | Formula: Wanted signal power + TT Interferer signal power unchanged |

Table C.2-2: Derivation of test requirements (FR2 OTA receiver tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.3 OTA reference sensitivity level | See TS 38.104 [2], subclause 10.3 | 2.4 dB, 24.25 GHz < f ≦ 33.4 GHz 2.4 dB, 37 GHz < f ≦ 52.6 GHz | Formula: EISREFSENS+ TT |
| 7.5.1 OTA adjacent channel selectivity | See TS 38.104 [2], subclause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking | See TS 38.104 [2], subclause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking | See TS 38.104 [2], subclause 10.6 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| 7.7 OTA receiver spurious emissions | See TS 38.104 [2], subclause 10.7 | 2.5 dB, 30 MHz ≤ f ≤ 6 GHz 2.7 dB, 6 GHz < f ≤ 12.75 GHz 0 dB, 12.75 GHz < f ≤ 60 GHz | Formula: Minimum Requirement + TT |
| 7.8 OTA receiver intermodulation | See TS 38.104 [2], subclause 10.8 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.9 OTA in-channel selectivity | See TS 38.104 [2], subclause 10.9 | 3.4 dB, 24.25 GHz < f ≦ 33.4 GHz 3.4 dB, 37 GHz < f ≦ 52.6 GHz | Formula: Wanted signal power + TT Interferer signal power unchanged. |

# C.3	Measurement of performance requirements

Table C.3-1: Derivation of test requirements (FR1 OTA performance tests)

| Test | Minimum Requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | See subclause 11.2.1.1 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | See subclause 11.2.1.2 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 8.3.1 Performance requirements for PUCCH format 0 | See subclause 11.3.1.2 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.2 Performance requirements for PUCCH format 1 | See subclause 11.3.1.3 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.3 Performance requirements for PUCCH format 2 | See subclause 11.3.1.4 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.4 Performance requirements for PUCCH format 3 | See subclause 11.3.1.5 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 8.3.5 Performance requirements for PUCCH format 4 | See subclause 11.3.1.6 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 8.4.1 PRACH false alarm probability and missed detection | See subclause 11.4.1 | [0.6] dB for fading cases [0.3] dB for AWGN cases | Formula: SNR + TTOTA PRACH False detection limit unchanged PRACH detection limit unchanged |

Table C.3-2: Derivation of test requirements (FR2 OTA performance tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | See subclause 11.2.2.1 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | See subclause 11.2.2.2 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 8.3.1 Performance requirements for PUCCH format 0 | See subclause 11.3.2.2 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.2 Performance requirements for PUCCH format 1 | See subclause 11.3.2.3 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.3 Performance requirements for PUCCH format 2 | See subclause 11.3.2.4 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.4 Performance requirements for PUCCH format 3 | See subclause 11.3.2.5 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 8.3.5 Performance requirements for PUCCH format 4 | See subclause 11.3.2.6 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 8.4.1 PRACH false alarm probability and missed detection | See subclause 11.4.2 | [0.6] dB for fading cases [0.3] dB for AWGN cases | Formula: SNR + TTOTA PRACH False detection limit unchanged PRACH detection limit unchanged |

Annex D (normative):
Calibration

OTA test requirements specific and OTA measurement chamber specific calibration (and measurement) procedures were captured in TR 37.843 [16] for the OTA AAS BS for the following requirements sets:

-	TX and Rx directional requirements

-	In-band and out-of-band TRP requirements

-	Co-location requirements

-	In-band and out-of-band blocking requirements

All the calibrations procedures captured for OTA AAS BS in TR 37.843 [16] for the frequency range up to 4.2 GHz, are assumed to be also applicable to BS type 1-H and BS type 1-O for the FR1 frequency range, i.e. up to 6 GHz.

Editor’s note: OTA test requirements specific and OTA measurement chamber specific calibration procedures for FR2 are FFS. Potential reuse of the FR1 calibration procedures for FR2 is FFS.

Annex E (informative):
OTA measurement system set-up

# E.1	Transmitter

## E.1.1	Radiated transmit power, OTA output power dynamics, OTA transmitted signal quality, OTA occupied bandwidth, and OTA transmit ON/OFF power (BS type 2-O)

Figure E.1.1-1: Measurement set up for radiated transmit power, OTA output power dynamics, OTA transmitted signal quality, OTA occupied bandwidth, and OTA transmit ON/OFF power (BS type 2-O)

The OTA chamber shown in figure E.1.1-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

## E.1.2	OTA base station output power, OTA ACLR, OTA operating band unwanted emissions

Figure E.1.2-1: Measurement set up for OTA base station output power, OTA ACLR, OTA operating band unwanted emissions

The OTA chamber shown in figure E.1.2-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

## E.1.3	OTA spurious emissions

Figure E.1.3-1: Measurement set up for OTA spurious emissions

The OTA chamber shown in figure E.1.3-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

## E.1.4	OTA co-location emissions, OTA transmit ON/OFF power (BS type 1-O)

Figure E.1.4-1: Measurement set up for OTA co-location emissions, OTA transmit ON/OFF power (BS type 1-O)

The OTA chamber shown in figure E.1.4-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

## E.1.5	OTA transmitter intermodulation

Figure E.1.5-1: Measurement set up for OTA transmitter intermodulation

The OTA chamber shown in figure E.1.5-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.). When injecting the interferer signal into the CLTA ports, a splitter might be needed. For testing emission far out-of-band an additional test antenna might be needed.

# E.2	Receiver

## E.2.1	OTA sensitivity and OTA reference sensitivity level

Figure E.2.1-1: Measurement set up for OTA sensitivity and OTA reference sensitivity level

The OTA chamber shown in figure E.2.1-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

## E.2.2	OTA dynamic range

Figure E.2.2-1: Measurement set up for OTA dynamic range

The OTA chamber shown in figure E.2.2-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

## E.2.3	OTA adjacent channel selectivity, general OTA blocking, and OTA narrowband blocking

Figure E.2.3-1: Measurement set up for OTA ACS and OTA narrowband blocking

The OTA chamber shown in figure E.2.3-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

Figure E.2.3-2: Measurement set up for general OTA blocking

The OTA chamber shown in figure E.2.3-2 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

## E.2.4	OTA blocking

### E.2.4.1	General OTA out-of-band blocking

Figure E.2.4.1-1: Measurement set up for general OTA out-of-band blocking

The OTA chamber shown in figure E.2.4.1-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

### E.2.4.2	OTA co-location blocking

Figure E.2.4.2-1: Measurement set up for OTA co-location blocking

The OTA chamber shown in figure E.2.4.2-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.). For testing blocking far out-of-band several CLTAs might be needed.

## E.2.5	OTA receiver spurious emissions

Figure E.2.5-1: Measurement set up for OTA receiver spurious emissions

The OTA chamber shown in figure E.2.5-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

## E.2.6	OTA receiver intermodulation

Figure E.2.6-1: Measurement set up for OTA receiver intermodulation

The OTA chamber shown in figure E.2.6-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

## E.2.7	OTA in-channel selectivity

Figure E.2.7-1: Measurement set up for OTA in-channel selectivity

The OTA chamber shown in figure E.2.7-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

# E.3	Performance requirements

Figure E.3-1: Measurement set up for single TX, single demodulation branch radiated performance requirements

Figure E.3-2: Measurement set up for single TX, dual polarization radiated performance requirements

Figure E.3-3: Measurement set up for dual TX, dual polarization radiated performance requirements

The OTA chambers shown in figures E.3-1, E.3-2 and E.3-3 are intended to be generic and can be replaced with any suitable OTA chamber (e.g. far field anechoic chamber, CATR, etc.). The PA(s) depicted in figures E.3-1, E.3-2 and E.3-3 is optional. Fading channel emulators are included when needed according to the requirement description.

Annex F (normative):
Void

Annex G (informative):
Transmitter spatial emissions declaration

# G.1	General

The transmitter spatial emission declaration is an optional declaration which provides additional information on the power level of emission in the intended (in cell) spatial directions and the unintended (out of cell) spatial directions. The declarations are only valid when the beam is configured in one of the EIRP conformance directions.

Figure G.1-1: Example of out of cell directions set and declared single beam at a single extreme steering direction

The declaration of unwanted spatial emission may in many circumstances not directly relate to system performance on its own. This is because it is often not possible to differentiate wanted and unwanted radiation, and furthermore because the benefits of optimizing beamforming performance may outweigh the impacts of “unwanted” radiation, leading to systems with apparently higher unwanted radiation also providing superior throughput performance. System performance should additionally be characterized taking all factors into account.

# G.2	Declarations

Table G.2-1: Optional manufacturer declarations

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| Dxx.1 | Out of cell directions set | The set of directions which are outside the intended directions of radiation or outside the wanted cell. Declared per operating band. |
| Dxx.2 | Out of cell power level | Declared in band average power inside each of the out of cell directions set(s) (DE.1) declared for each of the 5 conformance directions (D9.x) |
| Dxx.3 | In cell power level | Declared in band average power outside the out of cell directions set(s) (DE.1) declared for each of the 5 conformance directions (D9.x) |
| Dxx.4 | Average out of cell power level | Declared in band average power inside each of the out of cell directions set(s) (DE.1) averaged over the 5 conformance directions (D9.x). |
| DE.5 | Average in cell power level | Declared in band average power inside each of the out of cell directions set(s) (DE.1) averaged over the 5 conformance directions (D9.x) |

NOTE 1:	The declaration of unwanted spatial emission may in many circumstances not directly relate to system performance on its own. This is because it is often not possible to differentiate wanted and unwanted radiation, and furthermore because the benefits of optimizing beamforming performance may outweigh the impacts of “unwanted” radiation, leading to systems with apparently higher unwanted radiation also providing superior throughput performance. System performance should additionally be characterized taking all factors into account.

NOTE 2:	The average out of cell power level reflects the impact of out of cell radiation on other cells more accurately than the out of cell power level for individual test beams.

Annex H (normative):
Characteristics of the interfering signals

The interfering signal shall be a PUSCH containing data and DMRS symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 6 of TS 38.211 [20]. Mapping of PUSCH modulation to receiver requirement are specified in table H-1.

Table H-1: Modulation of the interfering signal

| Receiver requirement | Modulation |
| --- | --- |
| OTA in-channel selectivity | 16QAM |
| OTA adjacent channel selectivity and narrow-band blocking | QPSK |
| General OTA blocking | QPSK |
| OTA receiver intermodulation | QPSK |

Annex I (normative):
TRP measurement procedures

# I.1	General

The annex describes various procedures for BS OTA TRP measurments. These procedures can provide either an accurate or an over-estimate of TRP values. The procedures for an accurate estimate can be applied to all TRP requirements. However, if a TRP requirement does not need accurate TRP estimate then the procedures for over-estimate of TRP may be used in order to have a reasonable OTA test time. Pre-scan does not provide an accurate TRP estimate or over-estimate of TRP. Pre-scan is a fast but coarse method that is used to identify the spurious emission frequencies with emission power as described in annex I.13. A sequential measurement is then made at the emission frequencies, to assess the TRP as described in annex I.2 to annex I.9.

# I.2	Spherical equal angle grid

## I.2.1	General

TRPEstimate is defined as

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

when EIRP measurements is used or as

TRPEstimate=4πd2π2NMn=1N-1m=0M-1PD(θn,ϕm)sinθn

when power density measurements are used, and d is the test distance. N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM. The sampling intervals Δθ and Δϕ are described in I.2.2.

## I.2.2	Reference angular step criteria

The reference angular steps Δϕref and Δθref in degrees are defined as:

Δϕref= min⁡(180∘πλDcyl,15∘)

Δθref=⁡min(180∘πλD,15∘)

The upper limit for these reference angular steps of 15∘ ensures a low Summation Error (SE) when λ is large compared to the DUT dimensions.

Dcyl and D are calculated as:

`	Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in figure I2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure I2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Optionally, in the case of Uniform Linear Array (ULA), when d is negligible (d ≈0) and the EUT is mounted along the yz plane as shown in figure I2.2-2, the reference angular steps, in degrees, can be determined by

∆θref=180°πarcsin⁡(λ/Dz)

Δϕref=180°πarcsin⁡(λDy)

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis and  is wavelength for the measured frequency.

Figure I2.2-2: Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as

SF=maxΔθgridΔθref,ΔϕgridΔϕref

Where Δθgrid and Δϕgrid are the actual angular steps used in the measurement.

Alternatively, when EUT radiating dimensions are not known, for each frequency within the downlink operating band including ΔfOBUE, the reference angular steps can be specified in terms of the beamwidth of the wanted signal as

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

BeWφ and BeWθ may be set to beamwidth declared for the OTA BS radiated transmit power requirement provided the same beam is applied to test in-band TRP requirements.

NOTE:	Beamwidth is approximately equal to half the first-null beam width.

# I.3	Spherical equal area grid

TRPEstimate is defined as

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

N is the total number of samples and specified as

The sampling intervals Δθref and Δϕref are described in annex I.2.2. Each (θn,ϕn) is a sampling point.

# I.4	Spherical Fibonacci grid

TRPEstimate is defined as

TRPEstimate=1Nn=0N-1EIRP(θn,ϕn)

N is the total number of samples and specified as

The sampling intervals Δθref and Δϕref are described in annex I.2.2. Each (θn,ϕn) is a sampling point, where θn and ϕn, in degrees, are defined as:

θn=cos-11-2n+1N

ϕn=360°nΨ,  Ψ=1+52

# I.5	Orthogonal cut grid

## I.5.1	General

Here, at least two cuts (default) shall be used, an optional third cut can be used. The alignment of the cuts must be along the symmetry planes of the antenna array. No alignment is required for spurious emissions.

When alignment is required:

1)	The first mandatory cut is a horizontal cut passing through the peak direction of the main beam.

2)	The second mandatory is a vertical cut passing through the peak direction of the main beam. Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

3)	The third optional cut is a vertical cut orthogonal to the first and the second cut.

When alignment is not required, the cuts can be aligned arbitrarily.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as

EIRPav,cut-n=1Pj=1PEIRP(j)

where P is the number of sampling points in the cut. The final contribution for all cuts is calculated as

TRPEstimate=1Nn=1NEIRPav,cut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the intersection points are measured multiple times and the repeated values can be removed from the samples before averaging.

When two cuts measurements are used, a conditional pattern multiplication can be applied. The following are the conditions for applying pattern multiplication:

i.	The vertical cut (and the main beam) is in the xz -plane

ii.	The frequency of the emission is within the downlink operating band.

iii.	The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

iv.	The emission appears/disappears when the Tx power is turned on/off.

v.	The antenna arrays of the EUT

1)	Have rectangular grids of antenna element positions

2)	Have symmetry planes that are vertical and horizontal.

3)	Have parallel antenna planes

The antenna array is here assumed to be placed in the yz-plane. The pattern multiplication is performed in uv-coordinates and the data in the two cuts are denoted EIRPcut1ϕ at θ=θH and a vertical cut with data EIRPcut2θ at ϕ=0. The data is split in two parts corresponding to the forward and backward hemispheres. The uv-coordinates are the projections of the angular directions onto the antenna plane, here the yz-plane. Using the spherical coordinates as depicted in figure I.2.2-1 the u and v coordinates are defined as:

u=sinθsinϕv=cosθ

Note that only the data on the cuts are measured.

Calculate power density/EIRP values outside the two cardinal cuts as

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

The pattern multiplication is applied separately for the forward (fwd) and backward (bwd) hemisphere. The TRP is then calculated as

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

NOTE:	The numerical singularity at u2+v2=1 must be treated with care, e.g. by change of variables.

## I.5.2	Operating band unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex I.5.1 for the first two mandatory cuts and calculate the TRPEstimate.

2)	Compare the TRPEstimate to the limit.

3)	If the TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 2.

## I.5.3	Spurious unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex I.5.1 for two cuts and calculate the preliminary TRPEstimate.

2)	Add the appropriate correction factor ΔTRP according to table I.5.3-1 to ensure overestimation with 95% confidence.

3)	Compare the corrected TRPEstimate (including ΔTRP) to the limit.

4)	If the corrected TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 3.

Table I.5.3-1: The correction factor for two or three cuts dense sampling

|  | Three cuts | Two cuts |
| --- | --- | --- |
| Correction factor ΔTRP (dB) | 2.0 | 2.5 |

# I.6	Wave vector space grid

If EUT is mounted along the yz plane as shown in figure I2.2-1, the reference step in wave vector space can be determined by

Δuref=λDy

Δvref=λDz

where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be represented as following:

u=sinθsinϕ, v=cos⁡(θ)

The total radiated power (TRP) in the wave vector space is determined by:

TRPestimate=ΔurefΔvref4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

For spurious Tx or Rx emissions and where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as

SF=max∆ugrid∆uref,∆vgrid∆vref

Where ∆ugridand ∆vgrid are the actual steps used in the wave vector space in the measurement and the upper bound of their value is 1/12, corresponding to 15 degree steps close to boresight.

# I.7	Void

# I.8	Void

# I.9	Full sphere with sparse sampling

The procedure is as follows:

1)	Set the angular grid:

a.	Non-harmonic frequencies: choose the angular steps Δϕ and Δθ smaller than or equal to [15] degrees. Calculate the sparsity factor (SF) as

SF=maxΔϕΔϕref,ΔθΔθref

and the correction factor as:

ΔTRP=SF-1SFmax-1⋅1.0 dB,

where SFmax corresponds to 15 degrees angular step. If the sparsity factor is smaller than 1, the correction factor ΔTRP is 0 dB.

Harmonic frequencies with fixed beam test signal: choose the angular steps smaller than or equal to the reference angular steps Δϕref and Δθref. Correction factor ΔTRP is 0 dB.

[Harmonic frequencies with beam sweeping test signal: set the angular steps to [15] degrees. Correction factor is ΔTRP 0 dB].

2)	Apply a suitable numerical integration to calculate the preliminary TRPEstimate.

3)	Add the appropriate correction factor ΔTRP according to step 1 to ensure an overestimation with 95% confidence.

4)	Compare the corrected TRPEstimate (including ΔTRP) with the limit. If the corrected TRPEstimateis above the limit, choose a smaller angular step and repeat steps 2-4. If the sparsity factor is less than one, no significant improvement of accuracy is expected.

# I.10 Beam-based directions

Beam-based direction can be used in the base station operating band only if the directivity of the radiation pattern of the emssions being measured is known. TRPEstimate is defined as

TRPEstimate=EIRPpeakDEUT, where EIRPpeak is the maximum EIRP in the beam peak direction within a particular beam direction pair and DEUT is the directivity of the EUT.

# I.11	Peak method

The peak method can be used when frequencies with unwanted peak emissions are identified during pre-scan. The method does not provide an estimate of TRP.

For each peak emission frequency identified during pre-scan, measure peak EIRP or power density as follows:

1)	Move EUT and test antenna to the same position where the peak emission is recorded during the pre-scan.

2)	Move the EUT around the position and test antenna orientation to find the final peak EIRP or power density.

3)	The measured peak power density or EIRP shall be used to demonstrate conformance.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarized components.

# I.12	Equal sector with peak average

Equal sector with peak average can be performed on frequencies with unwanted peak emission, which are considered by the peak method for further measurements.

The spherical angle ϕ is divided into K equal sectors. If the largest dimension of EUT is less than 60 cm, then each sector is a half quadrant of 45°.

For each peak emission frequency, measure peak EIRP of beams belonging to different sectors of the sphere as follows:

1)	Move EUT and test antenna to the same position where the emission peak is recorded during the pre-scan.

2)	Move EUT around the position and test antenna orientation to find the final peak EIRP.

3)	Repeat Steps 1 to 2 until all sectors are covered.

4)	Calculate TRPEstimate as

TRPEstimate=1Kk=1KEIRPk, where EIRPk is the peak EIRP in the kth sector.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarized components.

# I.13 Pre-scan

Pre-scan is used to identify frequencies with unwanted emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP. An emission frequency identified by a pre-scan may be further investigated by any of the TRP measurement methods in this annex.

The procedure for pre-scan is as follows:

1)	Scan the entire surface around EUT.

2)	Rotate test antenna to cover all possible polarizations of emissions to detect maximum emissions.

3)	Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur.

4)	Emissions which 20 dB or more below the specified limit shall not require further measurements.

Annex J (normative):
Propagation conditions

# J.1	Static propagation condition

The propagation for the static performance measurement is an Additive White Gaussian Noise (AWGN) environment. No fading or multi-paths exist for this propagation model.

# J.2	Multi-path fading propagation conditions

The multipath propagation conditions consist of several parts:

-	A delay profile in the form of a "tapped delay-line", characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency.

-	Different models are used for FR1 (410 MHz - 7.125GHz) and FR2 (24.25 GHz – 52.6 GHz).

## J.2.1	Delay profiles

The delay profiles are simplified from the TR 38.901 [23] TDL models. The simplification steps are shown below for information. These steps are only used when new delay profiles are created. Otherwise, the delay profiles specified in annex J.2.1.1 and J.2.1.2 can be used as such.

Step 1: Use the original TDL model from TR 38.901 [23].

Step 2: Re-order the taps in ascending delays.

Step 3: Perform delay scaling according to the procedure described in subclause 7.7.3 in TR 38.901 [23].

Step 4: Apply the quantization to the delay resolution 5 ns. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution.

Step 5: If multiple taps are rounded to the same delay bin, merge them by calculating their linear power sum.

Step 6: If there are more than 12 taps in the quantized model, merge the taps as follows

-	Find the weakest tap from all taps (both merged and unmerged taps are considered)

-	If there are two or more taps having the same value and are the weakest, select the tap with the smallest delay as the weakest tap.

-	When the weakest tap is the first delay tap, merge taps as follows

-	Update the power of the first delay tap as the linear power sum of the weakest tap and the second delay tap.

-	Remove the second delay tap.

-	When the weakest tap is the last delay tap, merge taps as follows

-	Update the power of the last delay tap as the linear power sum of the second-to-last tap and the last tap.

-	Remove the second-to-last tap.

-	Otherwise

-	For each side of the weakest tap, identify the neighbour tap that has the smaller delay difference to the weakest tap.

-	When the delay difference between the weakest tap and the identified neighbour tap on one side equals the delay difference between the weakest tap and the identified neighbour tap on the other side.

-	Select the neighbour tap that is weaker in power for merging.

-	Otherwise, select the neighbour tap that has smaller delay difference for merging.

-	To merge, the power of the merged tap is the linear sum of the power of the weakest tap and the selected tap.

-	When the selected tap is the first tap, the location of the merged tap is the location of the first tap. The weakest tap is removed.

-	When the selected tap is the last tap, the location of the merged tap is the location of the last tap. The weakest tap is removed.

-	Otherwise, the location of the merged tap is based on the average delay of the weakest tap and selected tap. If the average delay is on the sampling grid, the location of the merged tap is the average delay. Otherwise, the location of the merged tap is rounded towards the direction of the selected tap (e.g. 10 ns & 20 ns  15 ns, 10 ns & 25 ns  20 ns, if 25 ns had higher or equal power; 15 ns, if 10 ns had higher power). The weakest tap and the selected tap are removed.

-	Repeat step 6 until the final number of taps is 12.

Step 7: Round the amplitudes of taps to one decimal (e.g. -8.78 dB  -8.8 dB)

Step 8: If the delay spread has slightly changed due to the tap merge, adjust the final delay spread by increasing or decreasing the power of the last tap so that the delay spread is corrected.

Step 9: Re-normalize the highest tap to 0 dB.

Note 1:	Some values of the delay profile created by the simplification steps may differ from the values in tables J.2.1.1-2, J.2.1.1-3, J.2.1.1-4, and J.2.1.2-2 for the corresponding model.

Note 2:	For Step 5 and Step 6, the power values are expressed in the linear domain using 6 digits of precision. The operations are in the linear domain.

### J.2.1.1	Delay profiles for FR1

The delay profiles for FR1 are selected to be representative of low, medium and high delay spread environment. The resulting model parameters are specified in J.2.1.1-1 and the tapped delay line models are specified in tables J.2.1.1-2 ~ J.2.1.1-4.

Table J.2.1.1-1: Delay profiles for NR channel models

| Model | Number of channel taps | Delay spread (r.m.s.) | Maximum excess tap delay (span) | Delay resolution |
| --- | --- | --- | --- | --- |
| TDLA30 | 12 | 30 ns | 290 ns | 5 ns |
| TDLB100 | 12 | 100 ns | 480 ns | 5 ns |
| TDLC300 | 12 | 300 ns | 2595 ns | 5 ns |

Table J.2.1.1-2 TDLA30 (DS = 30 ns)

| Tap # | Delay [ns] | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 |  |
| 3 | 15 | -5.1 |  |
| 4 | 20 | -5.1 |  |
| 5 | 25 | -9.6 |  |
| 6 | 50 | -8.2 |  |
| 7 | 65 | -13.1 |  |
| 8 | 75 | -11.5 |  |
| 9 | 105 | -11.0 |  |
| 10 | 135 | -16.2 |  |
| 11 | 150 | -16.6 |  |
| 12 | 290 | -26.2 |  |

Table J.2.1.1-3 TDLB100 (DS = 100ns)

| Tap # | Delay [ns] | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 10 | -2.2 |  |
| 3 | 20 | -0.6 |  |
| 4 | 30 | -0.6 |  |
| 5 | 35 | -0.3 |  |
| 6 | 45 | -1.2 |  |
| 7 | 55 | -5.9 |  |
| 8 | 120 | -2.2 |  |
| 9 | 170 | -0.8 |  |
| 10 | 245 | -6.3 |  |
| 11 | 330 | -7.5 |  |
| 12 | 480 | -7.1 |  |

Table J.2.1.1-4 TDLC300 (DS = 300 ns)

| Tap # | Delay [ns] | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Rayleigh |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 |  |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 |  |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 |  |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 |  |
| 12 | 2595 | -16.0 |  |

### J.2.1.2	Delay profiles for FR2

The delay profiles for FR2 are specified in J.2.1.2-1 and the tapped delay line models are specified in table J.2.1.2-2.

Table J.2.1.2-1: Delay profiles for NR channel models

| Model | Number of channel taps | Delay spread (r.m.s.) | Maximum excess tap delay (span) | Delay resolution |
| --- | --- | --- | --- | --- |
| TDLA30 | 12 | 30 ns | 290 ns | 5 ns |

Table J.2.1.2-2: TDLA30 (DS = 30 ns)

| Tap # | Delay [ns] | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 |  |
| 3 | 15 | -5.1 |  |
| 4 | 20 | -5.1 |  |
| 5 | 25 | -9.6 |  |
| 6 | 50 | -8.2 |  |
| 7 | 65 | -13.1 |  |
| 8 | 75 | -11.5 |  |
| 9 | 105 | -11.0 |  |
| 10 | 135 | -16.2 |  |
| 11 | 150 | -16.6 |  |
| 12 | 290 | -26.2 |  |

## J.2.2	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., TDLA<DS>-<Doppler>, TDLB<DS>-<Doppler> or TDLC<DS>-<Doppler> where ‘<DS>‘ indicates the desired delay spread and ‘<Doppler>’ indicates the maximum Doppler frequency (Hz).

Table J.2.2-1 and J.2.2-2 show the propagation conditions that are used for the performance measurements in multi-path fading environment for low, medium and high Doppler frequencies for FR1 and FR2, respectively.

Table J.2.2-1: Channel model parameters for FR1

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| TDLA30-5 | TDLA30 | 5 Hz |
| TDLA30-10 | TDLA30 | 10 Hz |
| TDLB100-400 | TDLB100 | 400 Hz |
| TDLC300-100 | TDLC300 | 100 Hz |

Table J.2.2-2: Channel model parameters for FR2

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| TDLA30-75 | TDLA30 | 75 Hz |
| TDLA30-300 | TDLA30 | 300 Hz |

## J.2.3	MIMO channel correlation matrices

The MIMO channel correlation matrices defined in J.2.3 apply for the antenna configuration using uniform linear arrays at both gNB and UE and for the antenna configuration using cross polarized antennas.

### J.2.3.1	MIMO correlation matrices using Uniform Linear Array

The MIMO channel correlation matrices defined in J.2.3.1 apply for the antenna configuration using uniform linear array (ULA) at both gNB and UE.

#### J.2.3.1.1	Definition of MIMO correlation matrices

Table J.2.3.1.1-1 defines the correlation matrix for the gNB.

Table J.2.3.1.1-1: gNB correlation matrix

|  | gNB correlation |
| --- | --- |
| One antenna |  |
| Two antennas |  |
| Four antennas |  |
| Eight antennas |  |

Table J.2.3.1.1-2 defines the correlation matrix for the UE:

Table J.2.3.1.1-2: UE correlation matrix

|  | One antenna | Two antennas | Four antennas |
| --- | --- | --- | --- |
| UE correlation |  |  |  |

Table J.2.3.1.1-3 defines the channel spatial correlation matrix. The parameters, α and β in table J.2.3.1.1-3 defines the spatial correlation between the antennas at the gNB and UE respectively.

Table J.2.3.1.1-3: correlation matrices

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 2x2 case |  |
| 2x4 case |  |
| 4x4 case |  |

For cases with more antennas at either gNB or UE or both, the channel spatial correlation matrix can still be expressed as the Kronecker product of  and according to.

#### J.2.3.1.2	MIMO correlation matrices at high, medium and low level

The  and  for different correlation types are given in table J.2.3.1.2-1.

Table J.2.3.1.2-1: Correlation for high, medium and low level

| Low correlation |  | Medium correlation |  | High correlation |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0.9 | 0.3 | 0.9 | 0.9 |

The correlation matrices for high, medium and low correlation are defined in table J.2.3.1.2-2, J.2.3.1.2-3 and J.2.3.1.2-4 as below.

The values in table J.2.3.1.2-2 have been adjusted for the 2x4 and 4x4 high correlation cases to insure the correlation matrix is positive semi-definite after round-off to 4 digit precision.  This is done using the equation:

Where the value “a” is a scaling factor such that the smallest value is used to obtain a positive semi-definite result. For the 2x4 high correlation case, a=0.00010. For the 4x4 high correlation case, a=0.00012.

The same method is used to adjust the 4x4 medium correlation matrix in table J.2.3.1.2-3 to insure the correlation matrix is positive semi-definite after round-off to 4 digit precision with a =0.00012.

Table J.2.3.1.2-2: MIMO correlation matrices for high correlation

| 1x2 case |  |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |
| 4x4 case |  |

Table J.2.3.1.2-3: MIMO correlation matrices for medium correlation

| 1x2 case | [N/A] |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |
| 4x4 case |  |

Table J.2.3.1.2-4: MIMO correlation matrices for low correlation

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 1x8 case |  |
| 2x2 case |  |
| 2x4 case |  |
| 2x8 case |  |
| 4x4 case |  |

In table J.2.3.1.2-4,  is a  identity matrix.

NOTE:	For completeness, the correlation matrices were defined for high, medium and low correlation but performance requirements exist only for low correlation.

### J.2.3.2	Multi-antenna channel models using cross polarized antennas

The MIMO channel correlation matrices defined in J.2.3.2 apply to two cases as presented below:

-	One TX antenna and multiple RX antennas case, with cross polarized antennas used at gNB

-	Multiple TX antennas and multiple RX antennas case, with cross polarized antennas used at both UE and gNB

The cross-polarized antenna elements with +/-45 degrees polarization slant angles are deployed at gNB. For one TX antenna case, antenna element with +90 degree polarization slant angle is deployed at UE. For multiple TX antennas case, cross-polarized antenna elements with +90/0 degrees polarization slant angles are deployed at UE.

For the cross-polarized antennas, the N antennas are labelled such that antennas for one polarization are listed from 1 to N/2 and antennas for the other polarization are listed from N/2+1 to N, where N is the number of TX or RX antennas.

#### J.2.3.2.1	Definition of MIMO correlation matrices using cross polarized antennas

For the channel spatial correlation matrix, the following is used:

Where

-	 is the spatial correlation matrix at the UE with same polarization,

-	 is the spatial correlation matrix at the gNB with same polarization,

-	 is a polarization correlation matrix,

-	 is a permutation matrix, and

-	denotes transpose.

Table J.2.3.2.1-1 defines the polarization correlation matrix.

Table J.2.3.2.1-1 : Polarization correlation matrix

|  | One TX antenna | Multiple TX antennas |
| --- | --- | --- |
| Polarization correlation matrix |  |  |

The matrixis defined as

where  and  is the number of TX and RX antennas respectively, and  is the ceiling operator.

The matrix  is used to map the spatial correlation coefficients in accordance with the antenna element labelling system described in J.2.3.2.

#### J.2.3.2.2	Spatial correlation matrices at UE and gNB sides

##### J.2.3.2.2.1	Spatial correlation matrices at UE side

For 1-antenna transmitter, .

For 2-antenna transmitter using one pair of cross-polarized antenna elements, .

For 4-antenna transmitter using two pairs of cross-polarized antenna elements, .

##### J.2.3.2.2.2	Spatial correlation matrices at gNB side

For 2-antenna receiver using one pair of cross-polarized antenna elements, .

For 4-antenna receiver using two pairs of cross-polarized antenna elements,.

For 8-antenna receiver using four pairs of cross-polarized antenna elements,.

#### J.2.3.2.3	MIMO correlation matrices using cross polarized antennas

The values for parameters α, β and γ for low spatial correlation are given in table J.2.3.2.3-1.

Table J.2.3.2.3-1: Values for parameters α,  and γ

| Low spatial correlation |  |  |
| --- | --- | --- |
|  |  |  |
| 0 | 0 | 0 |
| Note 1: Value of α applies when more than one pair of cross-polarized antenna elements at gNB side. Note 2: Value of β applies when more than one pair of cross-polarized antenna elements at UE side. |  |  |

The correlation matrices for low spatial correlation are defined in table J.2.3.2.3-2 as below.

Table J.2.3.2.3-2: MIMO correlation matrices for low spatial correlation

| 1x8 case |  |
| --- | --- |
| 2x8 case |  |

In table J.2.3.2.3-2,  is a  identity matrix.

Annex K (informative):
Measuring noise close to noise-floor

As the emission level seen by the measurement receiver (PUEM) for co-location requirements are very low, it is suggested to measure relative noise change instead of absolute noise level. For the situation where the noise level is stable, the noise level change can be identified by a relative noise measurement method. This method measures the relative noise change extrated from when the test object is operating and when the power is disconnected. From the two measured noise levels the relative noise change can be determined. The relations between measured noise change 1, noise floor N0 and the relation to PUEM with respect to the noise floor denoted 2 is visualized in the left drawing in figure K-1.

Figure K-1: Relative noise measurement

The absolute emission level in decibel scale is determined from a relative measurement of 1 as:

PUEM=N0-δ2

, where N0 is the noise floor of the measurement receiver and 2 is plotted as function of 1 at the right in figure K-1. The absolute noise floor of the measurement receiver, including probe antenna, cables, filter and LNA is determined by a calibration procedure. The calibration will determine the absolute emission level (N0) accuracy of measuring out-of-band unwanted emission close to the thermal noise floor.

Annex L (normative):
In-channel TX tests

# L.1	General

The in-channel TX test enables the measurement of all relevant parameters that describe the In-channel quality of the output signal of the TX under test in a single measurement process.

The parameters describing the in-channel quality of a transmitter, however, are not necessarily independent. The algorithm chosen for description inside this annex places particular emphasis on the exclusion of all interdependencies among the parameters.

# L.2	Basic principles

The process is based on the comparison of the actual output signal of the TX under test, received by an ideal receiver, with an ideal signal, that is generated by the measuring equipment and represents an ideal error free received signal. All signals are represented as equivalent (generally complex) baseband signals.

The description below uses numbers and illustrations as examples only. These numbers are taken from a TDD frame structure with normal CP length, 120 kHz SCS and a transmission bandwidth configuration of 400 MHz (NRB = 264). The application of the text below, however, is not restricted to this parameter set.

## L.2.1	Output signal of the TX under test

The output signal of the TX under test is acquired by the measuring equipment and stored for further processsing. It is sampled at a sampling rate which is the product of the SCS and the FFT size, and it is named zυ.

For FR1, FFT size is determined by the transmission bandwidth in TS 38.141-1 [3] table 6.5.3.5-2 for 15 kHz SCS, table 6.5.3.5-3 for 30 kHz SCS and table 6.5.3.5-4 for 60 kHz SCS.

For FR2, FFT size is determined by the transmission bandwidth in table 6.6.3.5.2-2 for 60 kHz SCS, and table 6.6.3.5.2-3 for 120 kHz SCS.In the time domain it comprises at least 10 ms. It is modelled as a signal with the following parameters:

-	demodulated data content,

-	carrier frequency,

-	amplitude and phase for each subcarrier.

For the example in the annex, the FFT size is 4096 based on table 6.6.3.5.2-3. The sampling rate of 491.52 Msps is the product of the FFT size and SCS.

## L.2.2	Ideal signal

Two types of ideal signals are defined:

The first ideal signal i1υ is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters:

-	demodulated data content,

-	nominal carrier frequency,

-	nominal amplitude and phase for each subcarrier.

It is represented as a sequence of samples at the sampling rate determined from annex L.2.1 in the time domain. The structure of the signal is described in the test models.

The second ideal signal i2υ is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters for FR1 and FR2:

-	nominal demodulation reference signal and nominal PT-RS if present (all other modulation symbols are set to 0 V),

-	nominal carrier frequency,

-	nominal amplitude and phase for each applicable subcarrier,

-	nominal timing.

It is represented as a sequence of samples at the sampling rate determined from annex L.2.1 in the time domain.

## L.2.3	Measurement results

The measurement results, achieved by the in-channel TX test are the following:

-	Carrier frequency error

-	EVM

-	Resource element TX power

-	OFDM symbol TX power (OSTP)

Other side results are: residual amplitude- and phase response of the TX chain after equalisation.

## L.2.4	Measurement points

The resource element TX power is measured after the FFT box as described in figure L.2.4-1 for FR1 and in figure L.2.4.2. The EVM shall be measured at the point after the FFT and a zero-forcing (ZF) equalizer in the receiver, as depicted in for FR1 in figure L.2.4-1 and for FR2 in figure L.2.4-2. The FFT window of FFT size samples out of (FFT size + cyclic prefix length) samples in the time domain is selected in the “Remove CP” box.

For FR1, The FFT size and the cyclic prefix length are obtained from TS 38.141-1 [3] table 6.5.3.5-2 for 15 kHz SCS, table 6.5.3.5-3 for 30 kHz SCS and table 6.5.3.5-4 for 60 kHz SCS.

For FR2, FFT size and the cyclic prefix length is determined from table 6.6.3.5.2-2 for 60 kHz SCS, and table 6.6.3.5.2-3 for 120 kHz SCS.

In one subframe, there are two symbols with the length of the cyclic prefix larger than the values listed in TS 38.141-1 [3] tables 6.5.3.5-2, 6.5.3.5-3 and 6.5.3.5-4 for FR1 and tables 6.6.3.5.2-2 and table 6.6.3.5.2-3 for FR2. Table L.2.4-1 lists the slot number and the symbol number and the formula how to compute the length of cyclic prefix for those two symbols according to the sampling rate.

Table L.2.4-1: Slot number and symbol number identifying the longer CP length for normal CP

| SCS (kHz) | Frequency Range | # slots in subframe | Symbol # and slot # with longer CP | Longer CP length |
| --- | --- | --- | --- | --- |
| 15 | FR1 | 1 | (symbol 0, slot 0) (symbol 7, slot 0) | CP length + FFT size / 128 |
| 30 |  | 2 | (symbol 0, slot 0) (symbol 0, slot 1) | CP length + FFT size / 64 |
| 60 |  | 4 | (symbol 0, slot 0) (symbol 0, slot 2) | CP length + FFT size / 32 |
| 60 | FR2 | 4 | (symbol 0, slot 0) (symbol 0, slot 2) | CP length + FFT size / 32 |
| 120 |  | 8 | (symbol 0, slot 0) (symbol 0, slot 4) | CP length + FFT size / 16 |

For the example used in the annex, the “Remove CP” box selects 4096 samples out of 4384 samples. Symbol 0 of slot 0 and slot 4 has 256 more samples in the cyclic prefix than the other symbols (the longer CP length = 544).

Figure L.2.4-1: Reference point for FR1 EVM measurements

Figure L.2.4-2: Reference point for FR2 EVM measurements

# L.3	Pre-FFT minimization process

Sample timing, carrier frequency in zυ are varied in order to minimise the difference between zυ and i1υ, after the amplitude ratio of zυ and i1υ has been scaled. Best fit (minimum difference) is achieved when the RMS difference value between zυ and i1υ is an absolute minimum.

The carrier frequency variation is the measurement result: carrier frequency error.

From the acquired samples, one value of carrier frequency error can be derived.

NOTE 1:	The minimisation process, to derive the RF error can be supported by post-FFT operations. However the minimisation process defined in the pre-FFT domain comprises all acquired samples (i.e. it does not exclude the samples inbetween the FFT widths and it does not exclude the bandwidth outside the transmission bandwidth configuration).

NOTE 2:	The algorithm would allow to derive carrier frequency error and sample frequency error of the TX under test separately. However there are no requirements for sample frequeny error. Hence the algorithm models the RF and the sample frequency commonly (not independently). It returns one error and does not distinuish between both.

After this process the samples zυ are called z0υ.

# L.4	Timing of the FFT window

The FFT window length is FFT size samples per OFDM symbol. For FDD, there are 14Ndl FFTs performed where Ndl is the number of slots in a 10 ms measurement interval, and the number of symbols in a slot for normal CP is 14.

The position in time for the FFT shall be determined.

For the example used in the annex, the FFT window length is 4096 samples per OFDM symbol. 1120 FFTs (i.e. 4,587,520 samples) cover less than the acquired number of samples (i.e. 4,915,200 samples in 10 ms).

In an ideal signal, the FFT may start at any instant within the cyclic prefix without causing an error. The TX filter, however, reduces the window. The EVM requirements shall be met within a window W < CP. There are three different instants for FFT:

-	Centre of the reduced window, called ∆c,

-	∆c-W/2, and

-	∆c+W/2,

The value of EVM window length W is obtained from the transmission bandwidth and TS 38.141-1 [3] table 6.5.3.5-2 for 15 kHz SCS, table 6.5.3.5-3 for 30 kHz SCS and table 6.5.3.5-4 for 60 kHz SCS for FR1, and table 6.6.3.5.2-2 for 60 kHz SCS, and table 6.6.3.5.2-3 for 120 kHz SCS for FR2.

The BS shall transmit a signal according to the test models intended for EVM. The demodulation reference signal of the second ideal signal shall be used to find the centre of the FFT window.

The timing of the measured signal is determined in the pre FFT domain as follows, using z0υ and i2υ:

1.	The measured signal is delay spread by the TX filter. Hence the distinct borders between the OFDM symbols and between data and CP are also spread and the timing is not obvious.

2.	In the ideal signal i2υ, the timing is known.

Correlation between bullet (1) and (2) will result in a correlation peak. The meaning of the correlation peak is approximately the “impulse response” of the TX filter.

3.	The meaning of “impulse response” assumes that the autocorrelation of the ideal signal i2υ is a Dirac peak and that the correlation between the ideal signal i2υ and the data in the measured signal is 0. The correlation peak, (the highest, or in case of more than one highest, the earliest) indicates the timing in the measured signal.

The number of samples, used for FFT is reduced compared to z0υ. This subset of samples is called z'υ.

From the acquired samples one timing can be derived.

The timing of the centre ∆c is determined according to the cyclic prefix length of the OFDM symbols. For normal CP, there are two values for ∆c in a 1 ms period:

-	∆c = length of cylic prefix / 2,

-	∆c = Longer CP length - length of cylic prefix / 2,

Where the length of cyclic prefix is obtained from TS 38.141-1 [3] table 6.5.3.5-2 for 15 kHz SCS, table 6.5.3.5-3 for 30 kHz SCS and table 6.5.3.5-4 for 60 kHz SCS for FR1, and table 6.6.3.5.2-2 for 60 kHz SCS, and table 6.6.3.5.2-3 for 120 kHz SCS for FR2, and the longer CP length is obtained from table L.2.4-1.

As per the example values.

-	∆c=144 within the CP of length 288 for most OFDM symbols in 1 ms,

-	∆c=400 (= 544 – 144) within the CP of length 544 for OFDM symbol 0 of slot 0 and slot 4.

# L.5	Resource element TX power

Perform FFT on z'ν with the FFT window timing ∆c.

The result is called Z't,f. The RE TX (RETP) power is then defined as:

RETP=Z't,f2SCSΔf

where ΔfSCS is the subcarrier spacing in Hz.

From RETP, the OFDM symbol TX power (OSTP) is derived as follows:

OSTP=RETP

Where the summation accumulates NRBNscRB values of the 4th OFDM symbol. The 4th (out of 14 OFDM symbols within a slot) contains exclusively PDSCH, NscRB=12.

From the acquired samples Ndl values for each OSTP can be obtained and averaged where Ndl is the number of slots in a 10 ms measurement interval for FDD. For TDD, Ndl is the number of slots with downlink symbols in a 10 ms measurement interval and is computed according to the values in table 4.9.2.2-1.

For the example used in the annex, Ndl=64 and NRB=264.

# L.6	Post-FFT equalisation

Perform 14Ndl FFTs on z'ν, one for each OFDM symbol within 10 ms measurement interval with the FFT window timing to produce an array of samples, 14Ndl in the time axis t by FFT size in the frequency axis f.

For the example in the annex, 1120 FFTs are performed on z'ν. The result is an array of samples, 1120 in the time axis by 4096 in the frequency axis.

The equalizer coefficients af and φf are determined as follows:

1.	Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal Z't,f and the post-FFT ideal signal I2t,f, for each demodulation reference signal, over 10 ms measurement interval. This process creates a set of complex ratios:

at,fejφt,f=Z't,fI2t,f

2.	Perform time averaging at each demodulation reference signal subcarrier of the complex ratios, the time-averaging length is 10 ms measurement interval. Prior to the averaging of the phases  φti,f  an unwrap operation must be performed according to the following definition:

-	The unwrap operation corrects the radian phase angles of  φti,f by adding multiples of 2 * π when absolute phase jumps between consecutive time instances ti are greater then or equal to the jump tolerance of π radians.

-	This process creates an average amplitude and phase for each demodulation reference signal subcarrier (i.e. every second subcarrier).

af=i=1Nati,fN

φf=i=1Nφti,fN

Where N is the number of demodulation reference signal time-domain locations ti from Z't,f for each demodulation reference signal subcarrier f.

3.	The equalizer coefficients for amplitude and phase af  and φf at the demodulation reference signal subcarriers are obtained by computing the moving average in the frequency domain of the time-averaged demodulation reference signal subcarriers. The moving average window size is 19. For reference subcarriers at or near the edge of the channel the window size is reduced accordingly as per figure L.6-1.

4.	Perform linear interpolation from the equalizer coefficients af and φf to compute coefficients af, φf for each subcarrier.

Figure L.6-1: Reference subcarrier smoothing in the frequency domain

a)	In case of FR2 EVM, to account for the common phase error (CPE) experienced in millimetre wave frequencies, φ(f), in the estimated coefficients contain phase rotation due to the CPE, θ, in addition to the phase of the equalizer coefficient φf, that is:

φf=φf+θ(t)

For OFDM symbols where PT-RS does not exist, θ(t) can be estimated by performing linear interpolation from neighboring symbols where PT-RS is present.

In order to separate component of the CPE, θ, contained in, φ(f), estimation and compensation of the CPE needs to follow. θ(t) is the common phase error (CPE), that rotates all the subcarriers of the OFDM symbol at time t.

Estimate of the CPE, θ(t), at OFDM symbol time, t, can then be obtained from using the PT-RS employing the expression:

θ(t)=argf∈fptrsZ't,fIptrst,f afe-jφf

In the above equation, fptrs is the set of subcarriers where PT-RS are mapped, t∈tptrs where tptrs is the set of OFDM symbols where PT-RS are mapped while Z't,f and Iptrst,f are is the post-FFT acquired signal and the ideal PT-RS signal respectively. That is, estimate of the CPE at a given OFDM symbol is obtained from frequency correlation of the complex ratios at the PT-RS positions with the conjugate of the estimated equalizer complex coefficients. The estimated CPE can be subtracted from φf to remove influence of the CPE, and obtain estimate of the complex coefficient’s phase:

φf=φf-θ(t)

# L.7	EVM

For EVM create two sets of Zeq't,f, according to the timing ∆c-W/2 and ∆c+W/2, using the equalizer coefficients from L.6.

The equivalent ideal samples are calculated from i1υ (annex L.2.2) and are called It,f.

The EVM is the difference between the ideal signal and the equalized measured signal.

EVM=tϵTfϵFiZeq't,f-It,f2tϵTfϵFiIt,f2

Where:

-	T is the set of symbols with the considered modulation scheme being active within the slot,

-	Ft is the set of subcarriers within the resource blocks with the considered modulation scheme being active in symbol t,

-	It,f is the ideal signal reconstructed by the measurement equipment in accordance with relevant test models,

-	It,f  is the equalized signal under test.

NOTE:	Although the basic unit of measurement is one slot, the equalizer is calculated over the entire 10 ms measurement interval to reduce the impact of noise in the reference signals.

## L.7.1	Averaged EVM (FDD)

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl downlink slots where Ndl is the number of slots in a 10 ms measurement interval.

The averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step.

EVMframe=1i=1NdlNii=1Ndlj=1NiEVMi,j2

Where Ni is the number of resource blocks with the considered modulation scheme in subframe i.

The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

Thus EVMframe,l is calculated using t=∆tl in the expressions above and EVMframe,h is calculated using t=∆th in the EVMframe calculation where (l and h, low and high; where low is the timing ∆c-W/2 and high is the timing ∆c+W/2).

Thus:

EVMframe=maxEVMframe,l,EVMframe,h

The averaged EVM with the minimum averaging length of at least Ndl slots is then achieved by further averaging of the EVMframe results:

EVM=1Nframek=1NframeEVMframe,k2

Where

Nframe=10Ndl

The resulting EVM is compared against the limit.

## L.7.2	Averaged EVM (TDD)

Let NdlTDD be the number of slots with downlink symbols within a 10 ms measurement interval. For TDD, the averaging in the time domain can be calculated from NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

EVMframe  is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

The EVMframe is calculated, using the maximum of EVMframeat the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated usingt=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by

Nframe=10NdlTDD

Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

The resulting EVMis compared against the limit.

Annex M (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-11 | RAN4#84bis | R4-1711983 | - | - | - | TS skeleton | 0.0.1 |
| 2018-04 | RAN4#86bis | R4-1805875, R4-1805876, R4-1804931, R4-1805877, R4-1805386, R4-1805916, R4-1805915, R4-1805878 | - | - | - | Implementation of TPs agreed during RAN4#86bis, on top of the agreed R4-1803913: - R4-1805875 TP for TS 38.141-2: Addition of applicability table in sub-clause 4.7.2 - R4-1805876 TP for TS 38.141-2: Addition of co-location reference antenna description to sub-clause 4.13 - R4-1804931 TP to TS 38.141-2: clauses 1-3 - R4-1805877 TP to TS 38.141-2: clauses 4, 5 - R4-1805386 TP to TS 38.141-2: clause 6 - R4-1805916 TP to TS 38.141-2: clause 7 - R4-1805915 TP to TS 38.141-2: NR BS OTA sensitivity conformance test (7.2) - R4-1805878 TP to TS 38.141-2: NR BS OTA REFSENS conformance test (7.3) | 0.1.0 |
| 2018-06 | RAN4#87 | R4-1714157, R4-1806599, R4-1807591, R4-1807747, R4-1808325, R4-1808329, R4-1808331, R4-1808332, R4-1808333, R4-1808334, R4-1808336, R4-1808337, R4-1808483, R4-1808487 | - | - | - | Implementation of TPs agreed during RAN4#87, on top of R4-1807255: - R4-1714157 TP to TS 38.141-2 - annex with spatial declarations definitions - R4-1806599 TP to TS 38.141-2: NR BS OTA dynamic range conformance test (7.4) - R4-1807591 TP to TS 38.141-2 - update to Rx general section (7.1) - R4-1807747 TP to TS 38.141-2: Annexes - R4-1808325 TP to TS 38.141-2: Improvement of RIB interface in Figures 4.2-1, 4.2-2 and 4.2-3, in sub-clause 4.2 - R4-1808329 TP to TS 38.141-2: NR BS OTA in-band selectivity and blocking conformance test (7.5) - R4-1808331 TP to TS 38.141-2: Definitions, symbols and abbreviations (Sections 3) - R4-1808332 TP to TS 38.141-2: NR BS OTA occupied bandwidth (6.7.2) - R4-1808333 TP to TR 38.141-2: Clarifications on OTA sensitivity requirement (7.2.1, 7.3.1) - R4-1808334 TP to TS 38.141-2 - OTA base station output power (6.3) - R4-1808336 TP to TS 38.141-2 - OTA in-band receiver intermodulation (7.8) - R4-1808337 TP to TS 38.141-2 - OTA in-channel selectivity (7.9) - R4-1808483 TP to TS 38.141-2: multi-band operation - R4-1808487 TP to TS 38.141-2 - OTA output power dynamics (6.4) | 0.2.0 |
| 2018-07 | RAN4-AH-1807 | R4-1808823 R4-1808874 R4-1809109 R4-1809465 R4-1809485 R4-1809486 R4-1809487 R4-1809488 R4-1809489 R4-1809490 R4-1809491 R4-1809493 R4-1809494 R4-1809495 R4-1809496 R4-1809497 R4-1809499 R4-1809501 R4-1809516 R4-1809561 R4-1809562 | - | - | - | Implementation of TPs approved during RAN4-AH-1807, on top of R4-1809266 (TS 38.141-2, v0.2.0): - R4-1808823 TP to TS 38.141-2: Applicability of requirements (Sections 4.7) - R4-1808874 TP to TS 38.141-2: Introduction of the transmit, receive and co-location configurations, in subclause 4.5 - R4-1809109 TP to TS 38.141-2 - Annex D, TX and RX Test setup - R4-1809465 TP to TS 38 141-2 - 4.8.2 Test signal Configurations - R4-1809485 TP to TS 38.141-2: NR BS acceptable uncertainty of OTA Test System (4.1.2) - R4-1809486 TP to TS 38.141-2: NR BS derivation of test requirement (Annex C) - R4-1809487 TP to TS 38.141-2: Correction of RX procedures - R4-1809488 TP to TS 38.141-2: Correction of TX directional power related requirements - R4-1809489 TP to TS 38.141-2 – OTA unwanted emissions – General (6.7.1) - R4-1809490 TP to TS 38.141-2: NR BS OTA occupied bandwidth (6.7.2) - R4-1809491 TP to TS 38.141-2 – OTA ACLR (6.7.3) - R4-1809493 TP to TS 38.141-2 Annex XX - measuring extreme conditions - R4-1809494 TP to TS 38 141-2 Test requirement for Radiated transmit power - R4-1809495 TP to TS 38.141-2 Transmitter spurious emissions (6.7.5) - R4-1809496 TP to TS 38.141-2: Adding requirement text for OTA Tx IMD in sub-clause 6.8 and Annex E1.7 - R4-1809497 TP to TS 38.141-2 Receiver spurious emissions (7.7) - R4-1809499 TP to TS38.141-2: OTA frequency error (6.6.2) - R4-1809501 TP to TS38.141-2: OTA time alignment error (6.6.4) - R4-1809516 TP to TR 38.141-2: NR BS OTA manufacturers declarations for radiated test requirements (4.6) - R4-1809561 TP to TS38.141-2: OTA modulation quality (6.6.3) - R4-1809562 TP to TS 38.141-2 – OTA operating band unwanted emissions (6.7.4) | 0.3.0 |
| 2018-08 | RAN4#88 | R4-1810818, R4-1810822, R4-1810823, R4-1811150, R4-1811538, R4-1811592, R4-1811618, R4-1811621, R4-1811626, R4-1811742, R4-1811743, R4-1811745, R4-1811746, R4-1811747, R4-1811748, R4-1811749, R4-1811750, R4-1811751, R4-1811752, R4-1811754, R4-1811760, R4-1811766, R4-1811767, R4-1811848, R4-1811879, R4-1811886, R4-1811887 | - | - | - | Implementation of TPs approved during RAN4#88, on top of R4-1810576 (TS 38.141-2, v0.3.0): - R4-1810818 TP to TS 38.141-2: Operating bands and channel arrangement (5) - R4-1810822 TP to TS 38.141-2: initial conditions for FR2 Rx spur test (7.7) - R4-1810823 TP to TS 38.141-2: FRC annex (A) - R4-1811150 TP: Add parameters band n50 in TS 38.141-2 - R4-1811538 TP for introduction of band n74 for TS38.141-2 - R4-1811592 TP to TS38.141-2: OTA operating band unwanted emissions requirements (6.7.4) - R4-1811618 Correction on general clause for 38.141-2 - R4-1811621 TP to TS 38.141-2 Section 4.8.2.1 Test signal used to build Test Configurations - R4-1811626 TP to TS 38.141-2: Section 6.1 NR Test Models - R4-1811742 TP to TS38.141-2: Regional requirements (4.4) - R4-1811743 TP to TS 38.141-2: Remaining issues and corrections for Radiated Tx power (EIRP) (6.2) - R4-1811745 TP to TS 38.141-2: Corrections and improvements to the OTA Tx spurious emissions test (6.7.5) - R4-1811746 TP to TS 38.141-2: Calibration annex (D) - R4-1811747 TP to TS 38.141-2: Improvements of co-location requirement description in sub-clause 4.12 - R4-1811748 TP to TS 38.141-2: NR BS OTA occupied bandwidth (6.7.2) - R4-1811749 TP to TS 38.141-2: Adding requirement text for OTA co-location spurious emission in subclause 6.7.5 and Annex E1.3 - R4-1811750 TP to TS 38.141-2 on MU and TT for Rx requirements for FR1 and FR2 - R4-1811751 TP to TS 38.141-2 on MU and TT for transmission in-band TRP emission and directional requirements or FR2 and FR1 - R4-1811752 TP to TS38.141-2 on MU and TT for extreme EIRP for FR1 and FR2 - R4-1811754 TP to TS 38.141-2: Improvement of requirement text for OTA TX IMD in subclause 6.8 and Annex E.1.5 - R4-1811760 TP to TS 38.141-2: test tolerance table (Annex C) - R4-1811766 TP to TS 38.141-2: wideband operation corrections and FBW declarations (4.6) - R4-1811767 TP to TS 38.141-2: OTA declarations cleanup (4.6) - R4-1811848 TP to TS 38.141-2: Adding requirement text for OTA out-of-band blocking in subclause 7.6 and Annex E2.4.1 and E2.4.2 - R4-1811879 TP to TS 38.141-2 – Overview of radiated Tx and Rx requirements (4.13) - R4-1811886 TP to TS 38.141-2: OBUE correction - R4-1811887 TP to TS 38.141-2 on OTA Tx ON/OFF power requirements | 0.4.0 |
| 2018-09 | RAN#81 | RP-181664 | - | - | - | Presented to TSG RAN for information. | 1.0.0 |
| 2018-11 | RAN4#88bis | R4-1812584, R4-1812666, R4-1812683, R4-1813300, R4-1813301, R4-1813310, R4-1813532, R4-1813754, R4-1813877, R4-1813881, R4-1813883, R4-1813896, R4-1813899, R4-1813900, R4-1813901, R4-1813902, R4-1813903, R4-1813907, R4-1813908, R4-1813911, R4-1813912, R4-1813913, R4-1813914, R4-1813915, R4-1813993, R4-1814074, R4-1814078, R4-1814080, R4-1814120, R4-1814193, R4-1814250, R4-1814251, R4-1814253, R4-1814254 | - | - | - | Implementation of TPs approved during RAN4#88bis, on top of RP-181664 (TS 38.141-1, v1.0.0): - R4-1812584 TP to TS 38.141-2: Correction on NOTE for wanted signal mean power for NR BS RX requirements - R4-1812666 TP to TS 38.141-2: Correction of directions for OTA requirements - R4-1812683 TP to TS 38.141-2: Clarification Note on non-zero Test Tolerance - R4-1813300 TP to TS 38.141-2: correction of the OSDD definition for single RAT NR BS specification - R4-1813301 TP to TS 38.141-2: alignment with TS 38.104 modifications after RAN4#88 - R4-1813310 TP to TS 38.141-2: structure alignments with TS 38.141-1 - R4-1813532 TP to TS 38.141-2: Corrections to Modulation quality test in Clause 6.6.3 - R4-1813754 TP to TS 38.141-2: Radiated performance requirements (8) - R4-1813877 TP for TS38.141-2: RF channel for BS OTA conformance test - R4-1813881 TP to TS 38.141-2: Section 4.9.2.3 Data content of PHY channels - R4-1813883 TP to TS 38.141-2: FR2 test model(Section 4.9.3) - R4-1813896 TP to TS 38.141-2: Addition of MU for OTA performance requirements for FR1 - R4-1813899 TP to TS 38.141-2: alignment of directions to be tested for OTA requirements - R4-1813900 TP to TS 38.141-2 on CLTA definition - R4-1813901 TP to TS 38.141-2 on MU and TT corrections for FR1 and FR2 - R4-1813902 TP to TS 38.141-2 on Rx requirement corrections for FR1 and FR2 - R4-1813903 TP to 38.141-2: Clause 4.6 - correction for manufacturer declaration - R4-1813907 TP to TS 38.141-2: frequency range for the inband blocking requirement for FR2 - R4-1813908 TP to TS 38.141-2 – adding TRP measurement grids to the annex - R4-1813911 TP to TS 38.141-2: Update for NR BS occupied bandwidth requirement (6.7.2) - R4-1813912 TP to 38.141-2: Corrections to OTA co-location spurious emission (6.7.5 and E.1.3) - R4-1813913 TP to 38.141-2: Corrections to OTA transmitter intermodulation in sub-clause 6.8 and Annex E.1.5 - R4-1813914 TP to TS 38.141-2: Correction of the RX intermodulation interferer - R4-1813915 TP to TS 38.141-2: In-channel selectivity (7.9) - R4-1813993 TP to TS 38.141-2: Radiated performance requirements for CP-OFDM based PUSCH - R4-1814074 TP to TS 38.141-2: Corrections on OTA transmit ON/OFF power - R4-1814078 TP to TS38.141-2: OTA CACLR absolute limits (6.7.3) - R4-1814080 TP to TS 38.141-2: OTA declarations numbering and cross-referencing - R4-1814120 TP to TS 38.141-2: Correction on the FRCs in Annex A1 and A2 - R4-1814193 TP to TS38.141-2: OTA UEM(Section 6.7.4) - R4-1814250 TP to TS 38.141-2: operating bands applicable for spurious emissions testing above 12.75 GHz - R4-1814251 TP to TS 38.141-2: correction for the narrowest supported CHBW and SCS - R4-1814253 TP to TS 38.141-2: Improvement of out-of-band blocking requirement in sub-clause 7.6 - R4-1814254 TP to TS 38.141-2 on CLTA related MU | 1.1.0 |
| 2018-11 | RAN4#89 | R4-1816288 R4-1814444 R4-1814504 R4-1814622 R4-1815005 R4-1815268 R4-1815304 R4-1815305 R4-1815330 R4-1815375 R4-1815381 R4-1815686 R4-1815689 R4-1815963 R4-1816277 R4-1816290 R4-1816291 R4-1816292 R4-1816293 R4-1816294 R4-1816295 R4-1816296 R4-1816297 R4-1816298 R4-1816300 R4-1816305 R4-1816306 R4-1816309 R4-1816312 R4-1816313 R4-1816315 R4-1816317 R4-1816318 R4-1816319 R4-1816350 R4-1816353 R4-1816356 R4-1816359 R4-1816361 R4-1816362 R4-1816371 R4-1816446 R4-1816484 R4-1816485 R4-1816593 R4-1816720 R4-1816726 R4-1816732 R4-1816735 R4-1816740 | - | - | - | Implementation of TPs approved during RAN4#89, on top of R4-1815277 (TS 38.141-1, v1.1.0): R4-1816288 TP to TS 38.141-2: general cleanup R4-1814444 TP to TS 38.141-2: FRC definitions for PUSCH and test parameters for PRACH R4-1814504 TP to TS38.141-2: Removal of the multi-band test for BS type 2-O R4-1814622 TP to TS 38.141-2 on Characteristics of the interfering signals R4-1815005 TP to 38.141-2: MU clarifications R4-1815268 TP to TS 38.141-2 on General radiated receiver characteristics R4-1815304 TP to TS 38.141-2: Alignment of test procedure for OTA out-of-band blocking in sub-clause 7.6 R4-1815305 TP to TS 38.141-2: Improvement of test specification text with respect to directions for OTA out-of-band blocking in sub-clause 7.6 R4-1815330 TP to TS 38.141-2: Correction to FBW definition in sub-clause 3.1 R4-1815375 TP to TS 38.141-2: Interpretation of measurement results and the Shared Risk principle R4-1815381 TP to 38.141-2: OTA demodulation alignment with TS38.104 (8.1) R4-1815686 TP to 38.141-2: alignment of OTA requirement names R4-1815689 TP to 38.141-2: OTA out-of-band blocking co-location requirement (7.6) R4-1815963 TP to TS 38.141-2: corrections of notes in declarations table (4.6) R4-1816277 TP to TS 38.141-2: remaining annexes R4-1816290 TP to TS 38.141-2 on manufacturer declarations for NR radiated requirements testing R4-1816291 TP to TS 38.141-2: narrowest beam selection for OTA testing R4-1816292 TP to 38.141-2: Radiated transmit power testing extreme environment conditions (6.2) R4-1816293 TP to TS38.141-2: Radiated transmit power requirement with wideband operation (6.2) R4-1816294 TP to TS38.141-2: OTA total power dynamic range(Section 6.4.3) R4-1816295 TP to TS 38.141-2: OTA transmitter OFF power (Section 6.5.1) R4-1816296 TP to TS38.141-2: OTA ACLR, UEM and spurious emission (Section 6.7.3) R4-1816297 TP to TS 38.141-2: additional spurious emissions requirement corrections (6.7.5.4.5.1) R4-1816298 TP to TS 38.141-2: Correction to RX receiver test directions R4-1816300 TP to TS 38.141-2 – adding further details on reference steps to the annex R4-1816305 TP to TS 38.141-2 - polarization wording improvements for OTA sensitivity and reference sensitivity R4-1816306 TP to TS 38.141-2_Corrections on transmitter intermodulation (section 3.2 and 6.8) R4-1816309 TP to TS 38.141-2 - update FR2 extreme MU and TT R4-1816312 TP to TS 38.141-2: Addition of calibration procedure for extreme temperature testing in Annex B.7 R4-1816313 TP to TS 38.141-2: Test distance for blocking interferer signal in sub-clause 7.6 R4-1816315 TP to 38.141-2: Corrections to co-location requirements R4-1816317 TP to 38.141-2 Corrections to TRP grids formula in Annex I and adding reference in Section 6.5 R4-1816318 Operating band orthogonal cuts measurement R4-1816319 TP to TS 38.141-2 Correction on declaration R4-1816350 TP for introducing propagation conditions in TS 38.141-2 R4-1816353 TP to TS 38.141-2: Radiated test requirements for DFT-s-OFDM based PUSCH R4-1816356 TP for TS 38.141-2 on NR PUCCH format2 radiated performance requirements R4-1816359 TP to TS38.141-2: Performance requirements for PRACH R4-1816361 TP for TS38.141-2: PUCCH format 1 OTA conformance test R4-1816362 TP to TS 38.141-2: Radiated test requirements for CP-OFDM based PUSCH in FR1 R4-1816371 TP to 38.141-2 – PUSCH requirements with CP-OFDM for FR2 R4-1816446 TP to TS 38.141-2: Cleanup to OTA requirements text R4-1816484 TP to TS 38.141-2: Improvement of specification text related to injection of interferer power for OTA TX IMD in sub-clause 6.8 R4-1816485 TP on 38.141-2: Deletion of test procedure on OFF power and transient period for FR2 R4-1816593 TP to TS 38.141-2: PUCCH format 0 requirement testing R4-1816720 TP for introducing PUCCH format 3 and 4 radiated conformance requirements for OTA test in 38.141-2 R4-1816726 TP to TS 38.141-2: FR2 test model(Section 4.9.2) R4-1816732 TP to TS 38.141-2: OTA transmitted signal quality (Section 6.6) R4-1816735 TP to TS 38.141-2: Section 4.9.2.3 Data content for PHY channels R4-1816740 TP to TS 38.141-2: Correction to FR2 OTA REFSENS requirement | 1.2.0 |
| 2018-12 | RAN#82 | RP-182584 |  |  |  | Presented to TSG RAN for approval. | 2.0.0 |
| 2018-12 | RAN#82 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |
| 2019-03 | RAN#83 | RP-190403 | 0001 |  | B | CR to TS 38.141-2 Implementation of the following draft CRs, which were Endorsed during RAN4#90, on top of v15.0.0: R4-1900286 Draft CR on NR PUCCH format2 radiated performance requirements for TS 38.141-2 R4-1900629 Draft CR to TS 38.141-2_Clean up the test requirements for some Rx requirements R4-1900739 Draft CR to TS 38.141-2:OTA dynamic range test requirement (7.4.5) R4-1900742 Draft CR to TS 38.141-2:Correction on OTA total power dynamic range requirement (6.4.3) R4-1900765 Draft CR to TS 38.141-2: Update of test requirement numbers for DFT-s-OFDM based PUSCH R4-1900830 Draft CR to TR 38.141-02: Correction to manufacturer declaration in sub-clause 4.6 R4-1900877 Draft CR to TS 38.141-2: On RX spurious emissions requirement R4-1900970 Draft CR for 38.141-2: Radiated test requirements for NR PUCCH format 1 R4-1901009 Draft CR to 38.141-2: Addition of coordinates system definition R4-1901325 Draft CR to 38.141-2: Correction to subclause 6.4.3 OTA total power dynamic range - correction R4-1901332 Draft CR to 38.141-2: Updates for Abbreviations section R4-1901372 CR to TS 38.141-2: Section 3.2 Missing Beam width Symbol Definition R4-1901389 Draft CR to TS 38.141-2 BS demodulation PUCCH format 0 requirements R4-1901476 Draft CR to TS 38.141-2 Corrections on transmitter co-existence and co-location requirements R4-1901486 Draft CR to TS 38.141-2: Corrections on OTA in-band blocking requirements R4-1901538 TS 38.141-2: Editorial corrections R4-1901743 DraftCR to TS 38.141-2: addition of the Iuant BS modem section R4-1902261 draft CR to TS 38.141-2 - update emissions scaling R4-1902274 Draft CR to TS 38.141-2: Addition of missing EIRP/EIS terminology in Clause 3 R4-1902275 Draft CR to 38.141-2; Correction to definition of OTA reference sensitivity R4-1902276 Draft CR to TS 38.141-2:Overview of radiated Tx requirements (4.1.1) R4-1902277 Draft CR to TS 38.141-2: Corrections on Measurement uncertainties and test requirements R4-1902278 CR to TS 38.141-2: Adding subclause 4.8 reference to test procedures R4-1902279 Draft CR to TS 38.141-2_Correction on test procedures for single-carrier and multi-carrier operation for Tx requirements R4-1902281 Draft CR to 38.141-2: Cleanup of RX procedures R4-1902282 Draft CR to TS 38.141-2:Test tolerance for radiated transmit power (C.1) R4-1902283 Draft CR to TS 38.141-2_Correction on multi-band operation related requirements R4-1902285 Correction of FR2 RoAoA declaration R4-1902287 Draft CR to 38.141-2; clarification of BS power limits R4-1902289 Draft CR to TR 38.141-2: Editorial clean-up of TRP measurement section in Annex I R4-1902291 Draft CR to 38.141-2: Addition of measurement system setup for radiated performance requirements R4-1902293 Draft CR for TS 38.141-2: Correction on TM applicability R4-1902295 Corrections to 38.141-2 subclause 4.9.2 base conformation test models R4-1902319 Draft CR to TS38.141-2 Tx OFF and transient measurement procedure R4-1902343 CR to TS 38.141-2: FR frequency limit corrections R4-1902385 Draft CR to TS 38.141-2: Applicability rule for BS radidated demodulation test R4-1902391 draftCR for TS 38.141-2: Radiated test requirements for CP-OFDM based PUSCH in FR1 R4-1902395 Draft CR to 38.141-2 – PUSCH requirements with CP-OFDM for FR2 R4-1902398 draftCR: Updates to PUCCH format 3 and 4 radiated conformance testing in TS 38.141-2 R4-1902401 Draft CR for updating PRACH performance requirements in TS38.141-2 R4-1902446 Draft CR to TS 38.141-2: Editorial CR for BS radidated demodulation test R4-1902573 Corrections to 38.141-2 Delay profile calculation R4-1902647 CR to TS 38.141-2: NR TM Multicarrier Configuration | 15.1.0 |
| 2019-06 | RAN#84 | RP-191240 | 0005 |  | B | CR to TS 38.141-2: Implementation of endorsed draft CRs from RAN4#90bis and RAN4#91 Implementation of Draft CRs into the merged Draft CR based on the endorsed tdocs during RAN4#90bis, on top of TS 38.141-2, v15.1.0: R4-1903326 Draft CR to TS 38.141-2: cleanup R4-1903331 Draft CR to TS 38.141-2: performance measure correction for the Rx requirements R4-1903459 Draft CR to TS 38.141-2: Correction to TX requirements with respect to total EIRP in subclause 6.2.4.2, subclause 6.3.4.2 and Annex B.7.1 R4-1903490 Draft CR to 38.141-2: Addition of measurement uncertainty for FR2 performance requirements R4-1903615 Draft CR to 38.141-2_Correction on OTA BS output power requirements (6.3) R4-1903616 Draft CR to 38.141-2: Correction on OTA occupied bandwidth requirements (6.7.2) R4-1903617 Draft CR to 38.141-2: Correction on OTA transmitter spurious emissions requirements (6.7.5) R4-1904207 Editorial corrections to TS 38.141-2 R4-1904236 draftCR: Correlation matrix for 8Rx in TS 38.141-2 R4-1904722 Draft CR to TS 38.141-2: Update of applicability rule for BS radiated demodulation test R4-1904725 Draft CR to TS 38.141-2: Update of radiated test requirements for DFT-s-OFDM based PUSCH R4-1904728 draftCR for TS 38.141-2: Radiated test requirements for CP-OFDM based PUSCH in FR1 R4-1904731 Draft CR on PRACH performance requirements in TS38.141-2 R4-1904733 Draft CR on TS 38.141-2 Radiated test requirements for PUCCH format 1 R4-1904737 Draft CR on NR PUCCH format2 radiated performance requirements for TS 38.141-2 R4-1904738 Draft CR to TS 38.141-2 Adding required vendor declaration items for BS demodulation R4-1904741 Draft CR to TS 38.141-2 BS demodulation PUCCH format 0 requirements R4-1904744 draftCR: Measurement system set-up and TT in TS 38.141-2 R4-1904747 draftCR: Updates to PUCCH format 3 and 4 radiated conformance testing in TS 38.141-2 R4-1904803 Draft CR to TS 38.141-2: FRC update for PUSCH FR1 mapping type B and FR2 DMRS 1+1 R4-1904818 Draft CR: Clarification on step 5 and step 6 for delay profiles calculation (38.141-2) R4-1904843 Draft CR to TS 38.141-2 BS demodulation CP-OFDM PUSCH FR2 requirements R4-1905112 Draft CR to TS 38.141-2:Overview of radiated Tx requirements (4.1.1) R4-1905118 Corrections to TS38.141-2 subclause 6.6.3.5 EVM measurement R4-1905119 draft CR to TS38.141-2 on section 4.9.2.3 header R4-1905122 draft CR to TS38.141-2 on test model(Section 4.9.2) R4-1905125 draft CR to 38.141-2 for TAE requirements R4-1905129 draft CR to TS38.141-2 on FR2 OFF power test procedure R4-1905130 draft CR to TS38.141-2 on TT and MU tables for FR2 OFF power R4-1905142 Draft CR: editorial correction on FR1 spurious emission requirement in TS38.141-2 R4-1905147 Draft CR to TS 38.141-2: Clarification on application of interfering signal offsets for ACS, blocking and intermodulation requirements R4-1905150 Draft CR to TS 38.141-2: Corrections on out-of-band blocking requirement R4-1905175 Draft CR to TS 38.141-2: FRC reference corrections for the Rx requirements R4-1905177 Draft CR to TS 38.141-2: consideration of suppoted frequency range of the operating band R4-1905183 Draft CR for TS 38.141-2: Improvements of figures in Annex E R4-1905184 Draft CR to TS 38.141-2: Correction on manufacturer’s declrations related to multi-band operation(section 4.6) R4-1905185 Draft CR for TS 38.141-2: Addition of NOTE for transmitter intermodulation requirements in certain regions R4-1905200 Draft CR to TS 38.141-2: Addition of RC test method for spurious emission in subclause 6.7 and 7.7 R4-1902647 CR to TS 38.141-2: NR TM Multicarrier Configuration Implementation of Draft CRs into the merged CR based on the endorsed tdocs during RAN4#91, on top of TS 38.141-1 in R4-1905216: R4-1905410 DraftCR to TS 38.141-2 Editorial Corrections R4-1905536 draftCR to TS 38.141-2 OTA RX spurious emission (subclause 7.7) R4-1905901 draft CR to TS 38.141-2 - corrections to test set up diagrams R4-1906004 Draft CR to 38.141-2: 6.8 OTA transmitter intermodulation – correction of interfering signal type R4-1906120 Draft CR to TS 38.141-2 Correction on multi-band test configurations R4-1906313 Draft CR to 38.141-2: Correction on FRC (Annex A) R4-1906783 Draft CR to TS38.141-2 on FR2 MU tables correction on frequency range (4.1.2.2, 4.1.2.3) R4-1906920 Draft CR to TS 38.141-2: Clarification on type of interfering signal for ACS, in-band blocking and ICS requirements R4-1906968 Draft CR to TS 38.141-2: adding further details to spherical Fibonacci grids (I.4) R4-1907112 Draft CR to TS 38.141-2: correction of the fundamental frequency limit of 2.55GHz for the spurious emissions R4-1907245 Draft CR to TS 38.141-2: Update of applicability rule for BS radiated demodulation test R4-1907248 Draft CR to TS 38.141-2: Update of radiated test requirements for DFT-s-OFDM based PUSCH R4-1907251 Draft CR to TS 38.141-2: Correction on the terminology in PUSCH FRC tables R4-1907254 Draft CR to TS38.141-2: Updates of PRACH performance requirements R4-1907257 Draft CR on NR PUCCH format2 radiated performance requirements for TS 38.141-2 R4-1907260 Draft CR on NR UCI on PUSCH radiated performance requirements for TS 38.141-2 R4-1907263 draftCR: Updates to PUCCH format 3 and 4 radiated conformance testing in TS 38.141-2 R4-1907265 Draft CR on TS 38.141-2 Radiated test requirements for PUCCH format 1 R4-1907271 Draft CR to TS 38.141-2 Manufacturer declaration for BS demodulation R4-1907274 Draft CR to TS 38.141-2 BS demodulation PUCCH format 0 requirements R4-1907276 Draft CR to TS 38.141-2 BS demodulation CP-OFDM PUSCH FR2 requirements R4-1907279 draftCR for TS 38.141-2: Radiated test requirements for CP-OFDM based PUSCH in FR1 R4-1907631 Draft CR to 38.141-2: Term “reference signal” replacing by term “ideal signal” in EVM context R4-1907633 Draft CR to 38.141-2: corrections to the Annex F structure R4-1907637 Draft CR to TS38.141-2 on target resource block clarification for EVM measurement (4.9.2.2, 4.9.2.3, 6.6.3.5) R4-1907639 Draft CR to TS38.141-2: Correction on test model R4-1907642 Draft CR to TS38.141-2 on TT and MU tables for FR2 Tx OFF power measurement (4.1.2.2, C.1) R4-1907660 Draft CR to TS 38.141-2 on Spurious emission Category B in FR2 R4-1907663 Draft CR to 38.141-2: BS TAE requirements R4-1907666 Draft CR to 38.141-2: Clarification of interferer RB frequency for narrowband blocking R4-1907670 Draft CR to TS 38.141-2: Corrections to Annex I (TRP) R4-1907671 Draft CR to TS 38.141-2: Correction on usage of terms TRP and EIRP R4-1907677 Draft CR to 38.141-2: Correction on OTA measurement setup (Annex E) R4-1907678 Draft CR to TS 38.141-2: Added description of symbol TRP Estimate R4-1907682 Draft CR to 38.141-2: Correction on procedure for general OTA transmitter spurious emissions requirements (6.7.5) R4-1907683 Draft CR to 38.141-2: Correction on OTA transmitter spurious emissions co-location requirements (6.7.5) R4-1907684 Draft CR to 38.141-2 Definition of contiguous transmission bandwidth R4-1907847 Draft CR to TS 38.141-02: Test model (section 4.9.2) R4-1907851 Draft CR to TS 38.141-2: Editorial cleanup of OTA transmit ON/OFF power in subclause 6.5 R4-1907853 Draft CR to TS 38.141-2: Clarification om beam identifier declaration in subclause 4.6 | 15.2.0 |
| 2019-06 | RAN#84 | RP-191242 | 0002 |  | B | Introduction of band n14 - CR to TS 38.141-2 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191246 | 0003 |  | B | Introduction of band n30 - CR to TS 38.141-2 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191250 | 0004 | 1 | B | n65 introduction to 38.141-2 | 16.0.0 |