TSG-RAN Working Group 4 (Radio) meeting #92	R4-1910505

Ljubljana, EU, 26th – 30th August 2019

Source:	Ericsson

Title:	TP to TR 38.820: Addition of BS PA trends conclusion in subclause 5.4.1

Agenda item:	11.4.7

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#91 in Reno) technical background information for different PA technologies was included in TR 38.820 by a text proposal [1]. Unfortunately, the conclusion from the original version [2] was left out. Depending on type of base station and operating frequency, different architectures can be used; centralized or distributed. Based on the choice of architecture the conclusion is highly relevant for BS station implementations for the frequency range 7 to 24 GHz.

In this contribution the remaining information is added with a text proposal to TR 38.820, subclause 5.4.1. The text proposal is attached at the end of this contribution and is presented for approval.

This is a revised version of R4-1908740.

# Discussion

In [3], the trends for power amplifier semi-conductor technologies in terms of achievable peak saturated power and power added efficiency was originally presented. Due to complex dependency between output power, ACLR/unwanted emission and efficiency, it is important to properly document theses aspects including performance and feasibility during the SI to ensure that proper considerations when WI for standardizing specific frequency bands in 7-24 GHz range is taken. In this contribution missing relevant information concluding for base station is added.

The test proposal implements the following changes to the current version of TR 38.820.

- In subclause 5.4.1.1, a conclusion for BS is added for PA saturated output power.

- In subclause 5.4.1.2, a conclusion for BS is added for PA efficiency.

# Conclusion

In this contribution the remaining information for PA trends is added. The remaining parts consists of a conclusion relevant for BS.

# References

[1]	R4-1907796, “TP to TR 38.820: Addition of PA trends in subclause 6.3.1”, Ericsson, RAN4#91

[2] 	R4-1906161, “TP to TR 38.820: Addition of PA trends in subclause 6.3.1”, Ericsson, RAN4#91

[3]	R4-1900832, “7-24 GHz technical aspects and considerations”, Ericsson, RAN4#90

TEXT PROPOSAL:

## 5.4	RF technology considerations in the 7-24 GHz range

Editor’s note: This section is intended to capture conclusions on the performance of key technologies in the BS and UE in the range, such as power amplifiers, filters, receivers, phase noise etc.

### 5.4.1	PA trends

For 7 – 24 GHz frequency range, it is essential to perform in-depth analysis of power amplifiers based on different semi-conductor technologies. In addition, complex relation between output power, ACLR (linearity) and efficiency is an important aspect considering the performance and feasibility for example frequencies.

The PA technology and trends are based on professor Hua Wang [3] work where a large power amplifier survey consisting of more than 2000 data points were published in 2008. Wang’s database covers published results from year 2000 and beyond, both from the open literature, as well as commercial amplifiers from various vendors.

Based on this material a more comprehensive analysis of achievable power amplifier performance has been investigated. The analysis covers the peak output power and power added efficiency. It should be noted that for all presented characteristics, the results are based on peak power, non-linearized power amplifiers without considering the bandwidth impact to show the trends with respect to frequency for different technologies.

#### 5.4.1.1	Saturated peak output power

In figure 5.4.1.1-1, a scatter diagram of saturated output power as a function of operating frequency for silicon technologies is shown. The technologies are grouped in three categories: LDMOS, CMOS and SiGe. The attainable output power at a given operating frequency is limited by the saturated electron velocity and the breakdown field strength in a given semiconductor material. This is captured in Johnsons’ figure of merit which states that the maximum output power will decrease with 20 dB/decade as the operating frequency is increased. The black line conveys the limitation due to Johnsons’ figure of merit.

Figure 5.4.1.1-1: Saturated output power versus frequency for LDMOS, CMOS and SiGe

Figure 5.4.1.1-2 presents the maximum output power as function of operating frequency for two compound semiconductor technologies: GaAs and GaN. Higher output power can be achieved but would necessitate excessive power combining associated with decreased operating efficiency.

Figure 5.4.1.1-2: Saturated output power versus frequency for GaAs and GaN

For PA operating within the frequency range of 7 GHz to 24 GHz the trends can be summarized as:

- Silicon technologies are limited to maximum 2 W of peak power.

- With GaN technology the peak output power can be increased to maximum 20 W peak power (at 24 GHz).

#### 5.4.1.2	Efficiency

In the previous section the output power capability was analyzed using published power amplifier data. In this subclause the same dataset will be used to study the efficiency of available semiconductor technologies.

In figure 5.4.1.2-1 (left), a scatter diagram of peak power added efficiency as function of operating frequency for power amplifiers made using silicon transistors (i.e. LDMOS, CMOS and SiGe). In figure 5.4.1.2-1 (right) the same parameters are plotted for compound semiconductor transistors (i.e. GaAs and GaN). As expected the efficiency is mainly dependent on the operating frequency and not the transistor technology. The wide spread of data is mainly due to different power levels and different amplifier architectures (e.g. class-A, class-AB, Doherty etc.).

Figure 5.4.1.2-1: Peak power added efficiency versus frequency

Grouping power amplifiers in different power categories gives a better view on the efficiency performance of different semiconductor technologies.

It should be noted that for all presented characteristics, the results are based on peak power, non-linearized power amplifiers without considering the bandwidth impact to show the trends with respect to frequency for different technologies. The presented characteristic neither consider practical implementation as the achievable average power (RMS) would being ~10 dB lower compared to saturated peak power to fulfil the needed modulation quality (EVM) or possibly necessary linearization range.