3GPP TSG- RAN WG4 Meeting #92b	R4-1910816

# Chongqing, China, 14th - 18th Oct, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update requirements for UE UL Tx timing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The value H for one shot timing adjustment is still TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the value H for one shot timing adjustment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

#### 7.1.2.2	One shot timing adjustment

When the magnitude of the T exceeds H then the UE shall adjust its transmission timing in one adjustment only once provided that the following conditions are met at the UE. Otherwise when the the magnitude of the T ≤ H then the UE shall adjust its transmission timing according to the rules defined in clause 7.1.2.1.

-	SSB_RP and SSB Ês/Iot according to Annex B.2.6.1 for a corresponding operating Band,

The UE transmit timing immediately after applying the one shot timing adjustment shall be: T2-(NTA+NTA offset)+2(T1-T2). After applying the one shot timing adjustment the UE shall adjust its transmission timing according to the rules defined in clause 7.1.2.1.

Where: T= T1-T2

-	T1 is the reception time at the UE just before the one shot timing adjustment,

-	T2 is the reception time to be used at the UE just after the one shot timing adjustment,

-	H is 33% of the uplink CP.

Table 7.1.2.2-1: Void

<End of change>