3GPPRAN4#94-e-Bis	R4-2004784

Online Meeting, April 20th  – 30th, 2020

Title: 	Discussion on timeline for BWP switch for multiple cells

Source: 	Qualcomm Incorporated

Agenda item:	6.15.1.4	BWP switching on multiple CCs	[NR_RRM_Enh_Core]

Document for:	Discussion

# Introduction

In RAN4-94-e, there was a WF [1] agreed on BWP switch on multiple cells. The following were the open issues.

For simultaneous switching

- Agreement: 𝑇𝐵𝑊𝑃𝑆𝑤𝑖𝑡𝑐ℎ𝐷𝑒𝑙𝑎𝑦+𝐷∗(⌈𝑁/𝐾⌉−1); N: Number of CCs with simultaneous BWP switch; K is number of CCs that can be processed simultaneously; D is incremental delay for BWP switch processing on additional CCsInterruption requirements are FFSIn this contribution we provide our views on the above.

  - FFS on D and K

- Switching delay fro RRC based BWP switching

- Interruption requirements

In this contribution, we present our views on the above.

# Discussion

## DCI Based Simultaneous Switching

The following is closely tracking our previous contribution.  For DCI based switching, the BWP switch time on single cell was divided into PDCCH decode, DCI parsing, UE processing and RF tuning. Of these, the PDDCH decode and DCI parsing should happen in parallel, since the UE needs to be able to do these on each CC individually at the same time in regular operation. However, the UE processing and RF tuning times would scale with number of carriers. From the single cell BWP switching analysis, the following were the approximate assumptions various components for Type 1 and Type 2

|  | Type 1 | Type 2 |
| --- | --- | --- |
| PDCCH decode/DCI parsing | 200 us | 300 us |
| SW processing | 250 us | 1.3 ms |
| RF re-tune | 200us | 200us |

Thus, for each additional cell on which the UE needs to do BWP switch it would need 450us for Type 1 and  1.5 ms for Type 2.

Proposal 1: : For DCI based simulatneous switch on N carriers, the BWP switch delay will be, 𝑇𝐵𝑊𝑃𝑆𝑤𝑖𝑡𝑐ℎ𝐷𝑒𝑙𝑎𝑦+𝐷∗(⌈𝑁/𝐾⌉−1) with K=1 and D = 450 us for Type 1 and 1.5 ms for Type 2.

For timer based simultaneous switching, the same requirements as DCI based simultaneous switch to apply. This is because the same scaling in SW processing and RF tuning would apply even for timer-based switch too.

Proposal 2: For timer based simultaneous switch, same requirements as simultaneous DCI based switch to apply.

## RRC based Switch

For RRC triggered simultaneous switch, similar to single CC, there would again be two components, TRRCprocessingDelay and TBWPswitchDelayRRC. TRRCprocessingDelayshould not change as this is still triggered via a single RRC message. Also, we can formulate the RRC switching in a fashion similar to that for DCI based switching, so that, total time to do RRC switching is  TRRCprocessingDelay +TBWPswitchDelayRRC + (N-1)*D1 . Here D1 should be less than TBWPswitchDelayRRC.

Proposal 3: For simultaneous BWP switch triggered of N cells via RRC, the total delay will be   TRRCprocessingDelay +TBWPswitchDelayRRC + (N-1)*D1

## Interruption Requirements

Each carrier that undergoes a BWP switch would entail re-programming the PLL for that carrier. Thus, each carrier switch would cause an interruption on all other active carrier. The length of each interruptions should be the same as that for single CC switch in Rel-15.

Proposal 4: For DCI or timer-based switch on N carrier, there will be N separate interruptions on all other active carriers. The length of each interruption will be the same as that for single CC as define in Rel-15.

# Conclusions

Proposal 1: : For DCI based simulatneous switch on N carriers, the BWP switch delay will be, 𝑇𝐵𝑊𝑃𝑆𝑤𝑖𝑡𝑐ℎ𝐷𝑒𝑙𝑎𝑦+𝐷∗(⌈𝑁/𝐾⌉−1) with K=1 and D = 450 us for Type 1 and 1.5 ms for Type 2.

Proposal 2: For timer based simultaneous switch, same requirements as simultaneous DCI based switch to apply.

Proposal 3: For simultaneous BWP switch triggered of N cells via RRC, the total delay will be   TRRCprocessingDelay +TBWPswitchDelayRRC + (N-1)*D1

Proposal 4: For DCI or timer-based switch on N carrier, there will be N separate interruptions on all other active carriers. The length of each interruption will be the same as that for single CC as define in Rel-15.

# References

[1] R4-2002244​ WF on R16 NR RRM enhancements - BWP switching on multiple CCs