3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2004583

# Online, 20th April – 30th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to add NR intra-band FR1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2020-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding approved NR Intra-band FR1 combinations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Approved NR Intra-band FR1 combinations at RAN4 94bis-e: CA_n41B_UL_n41B CA_n48(3A) CA_n48(4A) CA_n71B BCS1 Correction: CA_n48(2A) configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Approved NR Intra-band FR1 combinations are not added |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.5, 7.3, 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 series |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n1B | - | 10 | 10,15 |  |  |  | 40 | 0 |
|  |  | 15 | 15,20 |  |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |  |
| CA_n7B | CA_n7B | 10, 15, 20 | 10, 15, 20, 30, 35, 40 |  |  |  | 50 | 0 |
| CA_n40B | - | 20 | 80 |  |  |  | 100 | 0 |
|  |  | 50 | 50 |  |  |  |  |  |
| CA_n41B | CA_n41B | 10, 20, 30, 40, 50 | 10, 20, 30, 40, 50 |  |  |  | 100 | 0 |
| CA_n41C | - | 40 | 80, 100 |  |  |  | 180 | 0 |
|  |  | 50, 60, 80 | 60, 80, 100 |  |  |  |  |  |
|  |  | 10, 15, 20, 40, 50, 60, 80, 90 | 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  | 190 | 1 |
| CA_n48B | CA_n48B | 5, 10 | 10, 15, 20 |  |  |  | 40 | 0 |
|  |  | 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |
|  | - | 10 | 50, 60, 80, 90 |  |  |  | 100 | 1 |
|  |  | 15, 20 | 40, 50, 60, 80 |  |  |  |  |  |
|  |  | 40 | 40, 50, 60 |  |  |  |  |  |
|  |  | 50 | 50 |  |  |  |  |  |
| CA_n48C | - | 10, 15 | 90, 100 |  |  |  | 150 | 0 |
|  |  | 20 | 80, 90, 100 |  |  |  |  |  |
|  |  | 40 | 60, 80, 90, 100 |  |  |  |  |  |
|  |  | 50 | 50, 60, 80, 90, 100 |  |  |  |  |  |
|  |  | 60 | 40, 50, 60, 80, 90 |  |  |  |  |  |
|  |  | 80 | 20, 40, 50, 60 |  |  |  |  |  |
|  |  | 90 | 10, 15, 20, 40, 50, 60 |  |  |  |  |  |
|  |  | 100 | 10, 15, 20, 40, 50 |  |  |  |  |  |
| CA_n66B | - | 5 1 | 20, 40 |  |  |  | 50 | 0 |
|  |  | 10 | 15, 20, 40 |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 5 1, 10, 15 |  |  |  |  |  |
|  |  | 40 | 5 1, 10 |  |  |  |  |  |
| CA_n71B | - | 5 | 20 |  |  |  | 25 | 0 |
|  |  | 10 | 15 |  |  |  |  |  |
|  |  | 15 | 10 |  |  |  |  |  |
|  |  | 20 | 5 |  |  |  |  |  |
|  |  | 10 | 20 |  |  |  | 35 | 1 |
|  |  | 15 | 15,20 |  |  |  |  |  |
|  |  | 20 | 10,15 |  |  |  |  |  |
| CA_n77C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70,80,90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70,80,90, 100 |  |  |  | 200 | 1 |
| CA_n78C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70,80,90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70,80,90, 100 |  |  |  | 200 | 1 |
| CA_n78B | - | 20 | 50 |  |  |  | 70 | 0 |
| CA_n77D CA_n78D CA_n79D | - | 100 | 100 | 100 |  |  | 300 | 0 |
| CA_n79C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
| NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. |  |  |  |  |  |  |  |  |

Table 5.5A.1-2: Void

### 5.5A.2	Configurations for intra-band non-contiguous CA

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3(2A) | - | 5,10,15,20 | 5,10,15,20 |  |  | 40 | 0 |
| CA_n7(2A) | - | 5,10,15,20 | 5,10,15,20 |  |  | 40 | 0 |
| CA_n25(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
| CA_n41(2A) | - | 40, 50, 60, 80, 100 | 40, 50, 60, 80, 100 |  |  | 180 | 0 |
|  |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 190 | 1 |
| CA_n48(2A) | - |  |  |  |  |  | 0 |
|  |  | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 |  |  | 1402 |  |
| CA_n48(3A) | - | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 |  | 1402 | 0 |
| CA_n48(4A) | - | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 1352 | 0 |
| CA_n66(2A) | - | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 |  |  | 60 | 0 |
| CA_n77(2A) | - | 20, 40, 80, 100 | 20, 40, 80, 100 |  |  | 200 | 0 |
| CA_n78(2A) | - | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 |  |  | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 200 | 1 |
| NOTE 1: Void. NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150 MHz), and the minimum frequency gaps in between NR non-contiguous component carriers. |  |  |  |  |  |  |  |

---End of changes---