3GPP TSG-RAN4 #94bis-e 	R4-2005137

Online, 20th – 30th Apr. 2020

Source:	Charter Communications

Title:	TP for TR 38.716-01-01 for CA_n48(3A) and CA_n48(4A)

Agenda item:			8.1.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal to include TP for TP for TR 38.716-01-01 for CA_n48(3A) and n48(4A)

as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 6.x	CA_2DL_n48(3A)_1UL_n48A, CA_2DL_n48(4A)_1UL_n48A

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported bandwidth combinations for CA_2DL_n48(3A)_1UL _n48A

| NR CA configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n48(3A) | - | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 1402 | 0 |

Table 6.x.1-2: Supported bandwidth combinations for CA_2DL_n48(4A)_1UL _n48A

| NR CA configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48(4A) | - | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 1352 | 0 |

### 6.x.2	Co-existence studies

There are no co-existence issues for this combination.

### 6.x.3	REFSENS

There are no REFSENS exceptions for this combination. However, UL configuration for REFSENS needs to be captured after general principles for RX requirements have been agreed

---End of changes---

# Reference

[1] RP-200201, “Revised WID on Rel-16 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) ”, Ericsson