3GPP TSG-RAN WG4 Meeting #94bis-e			  						R4-2005147

Electronic Meeting, 20th Apr. 2020 – 24th Apr. 2020

Source: 	Nokia, Optus

Title: 		TP for 37.716-21-11 to introduce DC_3-7_n40

Agenda Item: 			8.4.2 [DC_R16_2BLTE_1BNR_3DL2UL-Core]

Document for:	Agreement

# 1	Introduction

This contribution is a TP for TR 37.716-21-11 to introduce DC_3-7_n40.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.1.x	DC_3-7_n40

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n40 | CA_3-7 | n40 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A_n40A | DC_3A_n40A DC_7A_n40A |

### 5.1.x.3	Coexistence studies

Co-existence studies for DC_3A_n40A are captured in 38.101-3 and for DC_7A_n40A in TR 37.716-11-. IMD5 might fall in Rx band of band 3.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_3-7_n40, the TIB,c and RIB,c values are reused from CA_3-7-40 in TS 36.101.

Table 5.1.x.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |

Table 5.1.x.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n40 | 3 | 0 |
|  | 7 | 0.3 |
|  | n40 | 0.8 |

### 5.1.x.5	REFSENS requirements

MSD is derived from similar combinations already captured for CA in 36.101 and EN-DC in 38.101-3. In this case CA_3A-7A-40A, DC_3A_n40A and DC_7A_n40A.

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-7A_n40A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |

*********************** End of the TP ***************************************************

# 3	References