3GPP TSG-RAN WG4 Meeting #94e-Bis	R4-20xxxxx

Electronic Meeting, April 20th – May 1st 2020

Agenda item:			6.5.3, 6.5.3.1, 6.5.3.2, 6.5.3.3, 6.5.3.4, 6.5.3.5

Source:	Qualcomm Incorporated

Title:	Email discussion summary of the 1st round for RAN4#94e_Bis#111_NR_IAB_RRM

Document for:	Information

# Introduction

This contribution summarizes different companies’ views regarding IAB RRM features and provides the recommended way forwards.

The email discussion plans to focus on open issues and TPs of RRC mobility control and MT timing in the first round. The discussion plans to focus on TPs of RLM and CBD requirements in the 2nd round.

# Topic #1: General

## Companies’ contributions summary

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2004247 | Huawei, Hisilicon | Proposal: For IAB-MT, the only mandatory features (1-8 Active BWP switching delay) for Rel-15 UE shall be optional, which results in no mandatory features related to RRM in TR 38.822 4.3 for IAB-MT. |

## Open issues summary

### Sub-topic 1-1

Issue 1-1:  Active BWP switching delay feature for IAB-MTs

- Proposals: For IAB-MT, the only mandatory features (1-8 Active BWP switching delay) for Rel-15 UE shall be optional

  - Option 1: Support the proposal

  - Option 2: Oppose the proposal

  - Option 3: Leave this issue to be discussed in RAN4 feature list session

- Recommended WF

Leave this issue to be discussed in RAN4 feature list session

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| ZTE | Sub topic 1-1: Support Option 3. |
| E/// | Issue 1-1: Support option 3. |
| AT&T | Support the Recommended WF. |

| Qualcomm | Support the recommended WF. |
| --- | --- |
| Samsung | Support the Recommended WF. |
| Nokia | Issue 1-1: Support the recommended WF |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2003531 Samsung | ZTE: Since no plenary meeting will be help after RAN4 94b, at least we can discuss this during RAN4 95. Now we should focus on the technical details. |
|  | Samsung: I agree with focusing on the technical details, but considering that this IAB TR is supposed to be approved in the next RAN plenary meeting, we have only one RAN4 meeting left in addition to this meeting. Thus I think it is better to reach an agreement on the skeleton of the spec structure in this meeting in order to help companies submit the detail contents next meeting. Another way is that companies can defined the structure as well as detailed contents all by themselves for the subsection they are assigned. Then companies can submit TP to the TR next meeting respectively. If we go with this option, an agreed WF on this is needed. |
|  | E///: The work on RRM core requirements is on track and discussion has sufficiently converge to compete the core requirements in timely manner. Adding RRM in TR will increase unnecessary workload. Therefore, for the time being we don’t see the need to add RRM requirements or related information in the TR. We propose that the TP is noted. It is not necessary to have RRM part in the TR to approve the TR if RRM session concludes no RRM part is needed. In the past we had TR but didn’t include RRM parts e.g. in NR general. |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Issue 1-1: Active BWP switching delay feature for IAB-MTs | Tentative agreements: “Active BWP switching delay” feature for IAB-MTs will be discussed in RAN4 feature list session. Candidate options: Recommendations for 2nd round: Further discussion is not needed. |
| Issue 1-2: Inclusion of RRM requirement in IAB TR | Tentative agreements: No consensus in the 1st round to accept the RRM related text for IAB TR that are proposed in R4-2003531. Candidate options (proposed by different companies): - Accept the RRM related text for IAB TR that are proposed in R4-2003531. - Discuss this TP, i.e., the RRM related headings for IAB TR, after RAN4 95. - Do not introduce RRM requirements in IAB TR. Recommendations for 2nd round: Companies should first decide the following two questions: - Should RRM requirements be captured in IAB TR? - If yes, can the description of RRM requirements from IAB TS be reused in the relevant sections of IAB TR? If answer to both I and II are yes, the editor of IAB TR spec can fill the relevant RRM sections simply based on IAB TS spec. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Title: Remaining issues for IAB RRM requirements Description of the WF’s focus: This WF will discuss all issues that were not resolved in the first round except RLM requirement and sharing factor related ones. Those issues have been delegated to a separate WF that would be led by ZTE. | Qualcomm |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2003531 Samsung | - This TP can be noted during the first round. - Depending on the outcome of the 2nd round discussion regarding issue 1-2, the editor of IAB TR spec can update the RRM portions of IAB TR spec during the email discussion after the meeting. |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Details of RRC mobility control requirements

Companies have submitted explicit proposals and TPs. The explicit proposals will be treated in the 1st round and the TPs will be treated in the second round.

## Companies’ contributions summary

| Tdoc number | Company | Comments |
| --- | --- | --- |
| R4-2004841 | ZTE | - Proposal 1: Confirm the WF from last meeting that IAB MT is not required to successfully identify a cell when the SMTC window is larger than 160ms and the serving cell is in low SNR. - Proposal 2: Confirm the WF from last meeting that there is no requirement for RRC re-establishment for IAB-MTs if the SSB transmission periodicity is larger than 160 ms. - Proposal 3: Confirm the WF from last meeting that there is no requirement for RRC release with re-direction when the periodicity of SSB is greater than 160 ms. - Proposal 4: Cell identification time shall be extended by a factor of 8 compared to UE requirements. |
| R4-2004246 | Huawei, Hisilicon | Proposal 1: The requirements of RRC connection re-establishment and RRC release with redirection for IAB shall be extended accordingly considering the 160ms SSB periodicity. Proposal 2: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency and inter-frequency cell shall be: Table I Serving cell SSB Ês/Iot (dB); Frequency range (FR) of target NR cell; Tidentify_intra_NR [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (1600 ms, 5 x TSMTC); MAX (6400 ms, 10 x TSMTC). ≥ -8; FR2; N/A; MAX (8000 ms, 80 x TSMTC)). < -8; FR1; N/A; 6400Note1. < -8; FR2; N/A; 28160Note1. Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC > 160 ms and serving cell SSB Ês/Iot < -8 dB.. Table II Serving cell SSB Ês/Iot (dB); Frequency range (FR) of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (1600 ms, 6 x TSMTC, i); MAX (6400 ms, 13 x TSMTC, i). ≥ -8; FR2; N/A; MAX (8000 ms, 104 x TSMTC, i)). < -8; FR1; N/A; 6400Note1. < -8; FR2; N/A; 32000Note1. Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 160 ms and serving cell SSB Ês/Iot < -8 dB.. Proposal 3: Time to identify target NR cell for RRC connection release with redirection to NR shall be: Frequency range (FR) of target NR cell; Tidentify-NR. FR1; MAX (5440 ms, 11 x Trs). FR2; MAX (7040 ms, 8x11 x Trs). Table III |
| R4-200408 | Ericsson | - Observation#1: UE shall be able to meet the RRC re-establishment or RRC release with redirection requirements even if it is not provided with the SMTC configuration assume the target cell’s SSB periodicity of no larger than 20 ms according to sections 6.2.1 and 6.2.3.2.1 of TS 36.133. - Observation#2: The SSB periodicity for initial access for IAB-MT is 160 ms, which is 8 times longer than that of the UE (i.e. 20 ms) according to TS 38.213. - Proposal #1: The time to identify target cell in RRC re-establishment delay requirements for IAB-MT is derived by increasing the numerical figures in the corresponding UE’s cell identification delay for RRC re-establisment by a factor of 8. - Proposal #2: The time to identify target cell in RRC redirection delay requirements for IAB-MT is derived by increasing the numerical figures in the corresponding UE’s cell identification delay for RRC redirection by a factor of 8. Table 1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell [section 6.3.1, 38.133] Serving cell SSB Ês/Iot (dB); Frequency range (FR) of target NR cell; Tidentify_intra_NR [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2; N/A; MAX (1000 ms, 80 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2; N/A; 3520Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 2: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell for IAB-MT Serving cell SSB Ês/Iot (dB); Frequency range (FR) of target NR cell; Tidentify_intra_NR [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (1600 ms, 5 x TSMTC); MAX (6400 ms, 10 x TSMTC). ≥ -8; FR2; N/A; MAX (8000 ms, 80 x TSMTC)). < -8; FR1; N/A; 6400Note1. < -8; FR2; N/A; 28160Note1. Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC >160 ms and serving cell SSB Ês/Iot < -8 dB.. Table 3: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell for IAB-MT Serving cell SSB Ês/Iot (dB); Frequency range (FR) of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (1600 ms, 6 x TSMTC, i); MAX (6400 ms, 13 x TSMTC, i). ≥ -8; FR2; N/A; MAX (8000 ms, 104 x TSMTC, i)). < -8; FR1; N/A; 6400Note1. < -8; FR2; N/A; 32000Note1. Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC,i >160 ms and serving cell SSB Ês/Iot < -8 dB.. Table 4: Time to identify target NR cell for RRC connection release with redirection to NR [section 6.2.3.2.1, TS 38.133] Frequency range (FR) of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2; MAX (880 ms, 8x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. Table 5: Time to identify target NR cell for RRC connection release with redirection to NR for IAB-MT Frequency range (FR) of target NR cell; Tidentify-NR. FR1; MAX (5440 ms, 11Trs). FR2; MAX (7040 ms, 811Trs). |
| R4-2004807 | Qualcomm | Observation 1: Rel-15 allowed network to configure up to two SMTC windows to each UE in intra-frequency. Observation 2: The section describing the core requirements for RRC re-establishment delay mentions the number of SMTC windows that can be configured for the UEs. Observation 3: Rel-16 allows network to configure up to four SMTC windows to each IAB MT node. Observation 4: RAN1 is discussing to generate an IAB-MT capability to support up to 4 SMTC per frequency layer. Proposal 1: The description of TSMTC in RRC re-establishment delay requirement should mention that up to four SMTC windows can be configured per frequency layer for an IAB MT. |

## Oppen issues summary

### Sub-topic 2-1

Issue 2-1: RRC re-establishment delay

- Proposal: The time to identify target cell in RRC re-establishment delay requirements for IAB-MT is derived by increasing the numerical figures in the corresponding UE’s cell identification delay for RRC re-establisment by a factor of 8.

Table 2: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell for IAB-MT

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (1600 ms, 5 x TSMTC) | MAX (6400 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | N/A | MAX (8000 ms, 80 x TSMTC)) |
| < -8 | FR1 | N/A | 6400Note1 |
| < -8 | FR2 | N/A | 28160Note1 |
| Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC >160 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Table 3: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell for IAB-MT

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (1600 ms, 6 x TSMTC, i) | MAX (6400 ms, 13 x TSMTC, i) |
| ≥ -8 | FR2 | N/A | MAX (8000 ms, 104 x TSMTC, i)) |
| < -8 | FR1 | N/A | 6400Note1 |
| < -8 | FR2 | N/A | 32000Note1 |
| Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC,i >160 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

  - Option 1: Support above proposal.

  - Option 2: Don’t support above proposal.

- Recommended WF: Support above proposal and table.

### Sup-topic 2-2

Issue 2-2: RRC release with redirection delay

- Proposal: The time to identify target cell in RRC redirection delay requirements for IAB-MT is derived by increasing the numerical figures in the corresponding UE’s cell identification delay for RRC redirection by a factor of 8.

  - Table 5: Time to identify target NR cell for RRC connection release with redirection to NR for IAB-MT

| Frequency range (FR) of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (5440 ms, 11Trs) |
| FR2 | MAX (7040 ms, 811Trs) |

  - Option 1: Support above proposal.

  - Option 2: Don’t support above proposal.

- Recommended WF: Support above proposal.

### Sub-topic 2-3

Issue 2-3: Applicability of higher number of SMTC windows in RRC re-establishment delay requirement

- Proposals: The description of TSMTC  should mention that up to four SMTC windows can be configured per frequency layer for an IAB MT.

  - Option 1: Support above proposal.

  - Option 2: Don’t support above proposal

- Recommended WF

  - Support above proposal.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| ZTE | Sub topic 2-1: Support recommended WF. Sub topic 2-2: Support recommended WF. Sub topic 2-3: Support recommended WF. |
| E/// | Issue 1-1: Support recommended WF. Issue 1-2: Support recommended WF. Issue 1-3: Do not support recommended WF. 4 SMTC windows per frequency is unnecessary requirement on IAB-DU. RRC re-establishment and RRC release are requirements on IAB-MT. IAB-MT shall be able to meet requirements for any of the SMTC configurations supported in RAN1/RAN2 specs like currently UE meets requirements for any configured SMTC configuration. |
| Huawei | Issue 1-1: Support recommended WF Issue 1-2: Support recommended WF Issue 1-3: We support Option 2. We share the similar views as Ericsson. The exact capability to support 4 SMTC configuration is still under discussion in RAN1. |
| AT&T | We support the recommended WF for all Sub-topics. Whether support for up to 4 SMTCs is mandatory/optional can be discussed independently by RAN1 |

| Qualcomm | We support the recommended WF for all three issues, i.e., issue 2-1, 2-2 and 2-3. Regarding issue 2-3, we share same views as AT&T. This is NOT a requirement for IAB-DU. This is a requirement for IAB-MT. RAN1 has already agreed to define 4 SMTC windows per frequency layer for IAB-MTs. And RAN4 should define requirements for all four SMTC. Whether supporting 4 SMTC windows is mandatory/optional can be discussed independently by RAN1. |
| --- | --- |
| Samsung | Sub Topic 2-1: Support recommended WF. Sub Topic 2-2: Support recommended WF. Sub Topic 2-3: Prefer to recommended WF. We think 4 SMTC window is necessary to be mentioned. If RAN1 define it as mandatory, there is stronger reason to capture it. |
| E/// | Issue 1-3: Regarding specifying requirements on 4 SMTC support for IAB-MT we should wait for the outcome of RAN1 capability discussion. RAN1 has not yet concluded on the capability. Whether up to 4 SMTC windows is necessary or not is not RAN4 rather RAN1 discussion. For example if this feature is optional then such requirements will only apply for IAB-MT which supports this capability. |
| Nokia | Issue 2-1: Support recommended WF Issue 2-2: Support recommended WF Issue 2-3: We agree with Ericsson’s comments. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004252 Huawei, Hisilicon | Samsung: In the last meeting, a WF is agreed on the TP writing principles when referencing will be used and when it is not possible. As UE requirement of random access is reused for IAB-MT, the TP is supposed to be written in a reference manner according to the WF. |
|  | Huawei: To Samsung: The power for preamble of IAB-MT may be different from UE’s and the Sub-clause for SUL is also not applicable for IAB-MT. According to the agreed WF on TP references rules, for the above two reasons, the references manners may not be recommended here. So we suggest not to use the references method for the TP of RA. |
|  | Nokia: Most of the requirement will be the same from 38.133. I wonder if we prefer add reference to 38.133 to avoid maintain both two specifications. We can consider the rules in R4-2002484 (WF on TS drafting referencing rules) approved in RAN4#94e RF session. |
|  | Huawei: To Nokia: As explained in our latest comments to Samsung. According to the rules in R4-2002484. The reference manner is not applicable here. 1. the requirement for the power for preamble of IAB is different.2 The sub-clause for SUL is not applicable. According to the rules, the reference manner is not recommended. We copy the rules here: - Referencing shall not be done if requirements are different, i.e. value or principle differs. - Referencing is not recommended if a complete specification sub-clause cannot be referenced - No referencing when it results in formulation ”BS type 2-O requirements in clause x.x.x [ref X] except [bad requirement] will apply for IAB-DU” - No referencing of individual tables or figures Thus, we prefer to keep the original version. |
| R4-2004409 Ericsson | Qualcomm: This TP assumes that only one SMTC can be configured for each frequency layer. As we have shown in our contribution (R4-2004807), according to RAN1 agreements, up to four SMTC windows can be configured for IAB-MTs for each frequency layer. The TP needs to be updated to capture the RAN1 agreement. We propose to incorporate the TP of R4-2004807 within this TP to describe the RRC re-establishment delay requirement. |
|  | E///: As stated above it is not clear of all IAB-MT will have to support this feature as being discussed in RAN1. Adding text as in R4-2004807 may cause confusion and misleading information. To move forward one suggestion is to add an editor’s note e.g. Requirement for support of up to 4 SMTC windows depends on the outcome of RAN1 agreements on IAB-MT capability discussion. |
|  | Nokia: Same comments in R4-2004252 on the rule to introduce the requriements. Could Ericsson explain why smtc2 is removed? Some minor errors like mixed UE and IAB-MT. |
| R4-2004410 Ericsson | Nokia: Same comments in R4-2004252 on the rule to introduce the requriements. Could Ericsson explain why smtc2 is removed? |
|  | Company B |
| R4-2004807 Qualcomm | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Issue 2-1 RRC re-establishment delay | Tentative agreement: - The time to identify target cell in RRC re-establishment delay requirements for IAB-MT is derived by increasing the numerical figures in the corresponding UE’s cell identification delay for RRC re-establisment by a factor of 8. Table 2: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell for IAB-MT Serving cell SSB Ês/Iot (dB); Frequency range (FR) of target NR cell; Tidentify_intra_NR [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (1600 ms, 5 x TSMTC); MAX (6400 ms, 10 x TSMTC). ≥ -8; FR2; N/A; MAX (8000 ms, 80 x TSMTC)). < -8; FR1; N/A; 6400Note1. < -8; FR2; N/A; 28160Note1. Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC >160 ms and serving cell SSB Ês/Iot < -8 dB.. Table 3: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell for IAB-MT Serving cell SSB Ês/Iot (dB); Frequency range (FR) of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (1600 ms, 6 x TSMTC, i); MAX (6400 ms, 13 x TSMTC, i). ≥ -8; FR2; N/A; MAX (8000 ms, 104 x TSMTC, i)). < -8; FR1; N/A; 6400Note1. < -8; FR2; N/A; 32000Note1. Note 1: The IAB-MT is not required to successfully identify a cell on any NR frequency layer when TSMTC,i >160 ms and serving cell SSB Ês/Iot < -8 dB.. Candidate options: Recommendations for 2nd round: No further discussion is needed. |
| Issue 2-2: RRC release with redirection | Tentative agreements: - The time to identify target cell in RRC redirection delay requirements for IAB-MT is derived by increasing the numerical figures in the corresponding UE’s cell identification delay for RRC redirection by a factor of 8. - Table 5: Time to identify target NR cell for RRC connection release with redirection to NR for IAB-MT Frequency range (FR) of target NR cell; Tidentify-NR. FR1; MAX (5440 ms, 11Trs). FR2; MAX (7040 ms, 811Trs). Candidate options: Recommendations for 2nd round: No further discussion is needed. |
| Issue 2-3: Number of SMTC windows for RRC re-establishment | Tentative agreement: No consensus regarding the requirements related to the number of SMTC windows per frequency layer. Candidate options: - Option 1: Requirements should be derived based on Rel-16 RAN1 agreements, i.e., by assuming each IAB-MT can be configured up to four SMTC windows per frequency layer. Supported by: ZTE, AT&T, Qualcomm, Samsung - Option 2: Requirements should be derived based on Rel-15 UE requirements, i.e., by assuming each IAB-MT can be configured up to two and one SMTC windows per intra-frequency and inter-frequency layers respectively. - Supported by: Ericsson, Nokia, Huawei Recommendations for 2nd round: Discuss this issue further. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2004252 Huawei | Status: To be revised Reason for status: The proponent mentions that SUL might not be applicable to IAB-MTs and the power of RACH preamble for IAB-MTs might be different than that for UEs. However, as several companies pointed that RAN4 RRM session already agreed to reuse the same RACH requirements for IAB-MTs and UEs. So, it is not clear why the power of RACH preamble cannot refer to existing text of 38.101. Besides, the RACH sections for IAB-MTs can still contain SUL section. If SUL is not applicable for IAB-MTs, this section will automatically not be applicable for IAB-MTs. Some UEs don’t support SUL also and the presence of SUL requirements in RACH RRM requirements do not impact those UEs. Suggestion for the revised TP: The proponent is suggested to simply add reference to 38.133 with the following editor’s note: [Editor’s note: The CR can be modified later to align the accuracy requirements for absolute power applied to the first preamble and the relative power applied to the additional preamble with RF’s conclusion]. |
| R4-2004409 Ericsson | Status: To be revised Reason for status: Several companies have mentioned that the description regarding T_SMTC need to be updated. Besides, the description depends on the outcome of 2nd round discussion. Suggestion for the revised TP: So, proponent is suggested to remove the following text from the updated TP: “TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the IAB-MT may assume that the target SSB periodicity is no larger than 160 ms.” Proponent is also suggested to add the following editor’s note: “[Editor’s not: The description regarding TSMTC and TSMTC,i will be added after RAN4 finalizes the requirements regarding the number of SMTC windows]”. |
| R4-2004410 Ericsson | Status: Agreeable with the following editor’s note “[Editor’s note: The description regarding the relationship between Trs and SMTC configuration will be added after RAN4 finalizes the requirements regarding the number of SMTC windows]”. Comment to chair: Is a separate TP necessary that will capture the editor’s note? Or can the editor add the note while updating the IAB TS spec? If the 2nd option is suggested, then the chair is requested to capture the editor’s note in chairman’s notes. |
| R4-2004807 Qualcomm | Status: Can be noted. |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: Details of MT Timing Related Requirements

## Companies’ contributions summary

| Tdoc number | Company | Comments |
| --- | --- | --- |
| R4-2004840 | ZTE | - Proposal 1: The Te requirement for IAB MT reuses Te requirement for Rel-15. - Proposal 2: IAB MT shall only transmit in uplink if it can meet Te requirement. |

## Oppen issues summary

Feature lead’s note:

Both proposals of R4-2004840 were already captured in the chairman’s note as agreements. So, these don’t need to be discussed further.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| ZTE | Agree with feature lead that paper can be noted. No need to discuss. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004250 Huawei, Hisilicon | Nokia: Since the requirement will be the same from 38.133. I wonder if we prefer add reference to 38.133 to avoid maintain both two specifications. We can consider the rules in R4-2002484 (WF on TS drafting referencing rules) approved in RAN4#94e RF session. |
|  | Huawei: To Nokia: As explained in our response to the TP for RA. According to the rules in R4-2002484. The reference shall not be used as the Table for UE including requirement for both TDD and FDD, which is not applicable for IAB-MT. Thus, we suggest to keep the original version. |
| R4-2004251 Huawei, Hisilicon | Nokia: same view as R4-2004250 |
|  | Huawei: To Nokia: We prefer not to use reference, because the description about EN-DC and NE-DC, etc. will lead to misunderstanding. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2004250 Huawei, Hisilicon | Status: Agreeable. Reason for status: One company proposed to modify this TP to simply add reference to 38.133. There is a good justification for that proposal. However, given that this TP is different from the corresponding text of 38.133 in several places, e.g.: 1) DRX mode, 2) FDD band, 3) pTAG/sTAG, etc., it is perhaps better to agree to the TP in its current form. |
| R4-2004251 Huawei, Hisilicon | Status: To be revised Suggestion for the revised TP: The proponent is requested to simply add reference to 38.133. Reason for status: Note that, according to R4-2002484, 38.133 can be cited if values and principles of the requirements are same. That seems to be the case here. Some of the scenarios of 38.133 are not applicable for IAB-MTs (e.g. EN-DC, NE-DC). But they will be clear from other parts of RAN4 spec and should not prohibit referencing to 38.133 in this scenario. |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: RLM requirements

## Companies’ contributions summary

| Tdoc number | Company | Comments |
| --- | --- | --- |
| R4-2004837 | ZTE | - Proposal 1: Micro type of IAB MTs should reuse most of the RLM requirements for R15 UEs, with some relaxation on certain aspects. - Proposal 2: Extend the evaluation period of IS and OOS by multiplying K to the evaluation period for UEs. Different values of K can be used for FR1 and FR2, for SSB based evaluation and CSI-RS based evaluation. - Proposal 3: Omit the sharing factor P from the RLM requirements (P = 1 always). - Proposal 4: The discussion on whether to remove the sharing factor should be combined across all relevant sub-agendas and a decision should be made for all scenarios where P is involved. - Proposal 5: Have a separate WF discussion on RLM requirements for IAB MTs. |
| R4-2003532 | Samsung | Observation 1: Consider non-mobility IAB in Rel-16, RLM procedure for IAB-MT would be much simpler compared to UE’s procedure. Observation 2: Even if unexpected blockage occurs, temperately link outage can be quickly recovered by beam failure recovery procedure where MT switches active beam to the alternative path to maintain the link. Observation 3: Compared to UE, radio link failure has much less chance to happen for IAB. Proposal 1: Apply relaxed RLM requirement of NR UE to the requirement of IAB-MT. Proposal 2: For IAB RLM requirement, increase the number of samples and the lower boundary of the Evaluation Period in both SSB and CSI-RS based measurement cases. Proposal 3: Remove the sharing factor P in Evaluation Period calculation for IAB RLM requirement provided that the number of samples is increased and the lower boundary is relaxed. Proposal 4: The Evaluation Period for IAB RLM requirement could be relaxed as follows. Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1 Configuration; TEvaluate_out_SSB (ms); TEvaluate_in_SSB (ms). no DRX; Max([1000], [50] TSSB); Max([500], [25] TSSB). NOTE: TSSB is the periodicity of the SSB configured for RLM.. Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2 Configuration; TEvaluate_out_SSB (ms); TEvaluate_in_SSB (ms). no DRX; Max([1000], Ceil([50] N) TSSB); Max([500], Ceil([25] N) TSSB). NOTE: TSSB is the periodicity of the SSB configured for RLM.. |
| R4-2004248 | Huawei, Hisilicon | Proposal 1: It is suggested to extend the evaluation period in TS 38.133 by scaling factor of 5 for IAB MT RLM. Proposal 2: There is no need to remove the sharing factor. Proposal 3 : The CSI-RS requirements shall apply only for CSI-RS density is 3 over the bandwidth ≥ 24 PRBs. |
| R4-2004806 | Qualcomm | Observation 1: Reusing Rel-15 RLM evaluation period works for IAB-MTs. But, since Rel-16 IAB-MTs are stationary, relaxing RLM evaluation period saves power and reduces complexity of IAB-MTs without harming its link monitoring performance. Observation 2: Although RAN4 has not allowed measurement requirements, network can still configure these measurement gaps and the IAB-MTs that support this feature can follow those configurations. Observation 3: Local area IAB-MTs are likely to be in NLOS scenarios and may experience similar delay spread as UEs. Observation 4: A reduced density of CSI-RS will also reduce total CSI-RS power (assuming that the tones FDMed with CSI-RS are used for other purposes). Proposal 1: Support relaxed RLM evaluation period requirement for IAB-MTs compared to that for NR UEs. - Increase the number of samples and the lower boundary of the SSB and CSI-RS based RLM evaluation period. Proposal 2: Sharing factor P does not need to be removed from the RLM evaluation period of IAB-MTs. Proposal 3: - Use RLM-CSI-RS with density = 3 for local area IAB-MTs. - Discuss further if it is essential to introduce RLM-CSI-RS with density = 1 for wide area IAB-MTs. Proposal 4: The following RLM related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs. - Measurement restrictions for SSB based RLM and CSI-RS based RLM - Minimum requirement at transitions - Minimum requirement for L1 indication. - Scheduling availability during RLM |

## Open issues summary

### Sub-topic 4-1

Issues: Relaxed RLM requirement.

Proposals: Apply relaxed RLM requirement of NR UE to the requirement of IAB-MT.

- Scale the number of samples and the lower boundary of the SSB and CSI-RS based RLM evaluation period by a factor of 5.

  - Option 1: Support above proposal

  - Option 2: Don’t support above proposal

- Recommended WF

  - Apply relaxed RLM requirement of NR UE to the requirement of IAB-MT.

    - Scale the number of samples and the lower boundary of the SSB and CSI-RS based RLM evaluation period by a factor of 5.

### Sub-topic 4-2

Issues: Sharing factor P.

Proposals: Remove the sharing factor P in Evaluation Period calculation for IAB RLM/BFD/CBD requirement.

  - Option 1: Support above proposal

  - Option 2: Don’t support above proposal

- Recommended WF

  - Decide based on inputs.

### Sub-topic 4-3

Issues: Density of CSI-RS to define CSI-RS based RLM/BFD/CBD requirements.

Proposals: The CSI-RS requirements shall apply only for CSI-RS density 3 over the bandwidth ≥ 24 PRBs.

  - Option 1: Support above proposal

  - Option 2: Don’t support above proposal

- Recommended WF

  - The CSI-RS requirements for RLM/BFD/CBD shall apply only for CSI-RS density 3 over the bandwidth ≥ 24 PRBs.

### Sub-topic 4-4

Issues: Other requirements

Proposals: The following RLM related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

- Measurement restrictions for SSB based RLM and CSI-RS based RLM

- Minimum requirement at transitions

- Minimum requirement for L1 indication.

- Scheduling availability during RLM

  - Option 1: Support above proposal

  - Option 2: Don’t support above proposal

- Recommended WF

  - The following RLM related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

    - Measurement restrictions for SSB based RLM and CSI-RS based RLM

    - Minimum requirement at transitions

    - Minimum requirement for L1 indication.

    - Scheduling availability during RLM

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| ZTE | Sub topic 4-1: We also propose to introduce a scaling factor in our paper to extend the IS and OOS, while we think the value of the factor can be further discussed. Sub topic 4-2: Support Option 1. The IAB MTs don’t need to measure as many reference signals as UE so the sharing factor is not really necessary and only complicates things. P should be omitted (or set to 1 always). Sub topic 4-3: Support Option 1. Sub topic 4-4: support recommended WF, which is also the way we prepared the TP R4-2004838. Others: For 4-1 and 4-2 it’s possible for companies to reach compromise. We can lead a separate WF discussion if needed. |
| Huawei | Sub topic 4-1: Support the recommended WF. Sub topic 4-2: Support Option 2. Sub topic 4-3: Support the recommended WF. Sub topic 4-4 Support the recommended WF. |
| AT&T | Sub-topic 4-1: Support the recommended WF. Sub-topic 4-2: We prefer Option 2 as it gives flexibility to the network in case measurement gaps are supported by IAB-MTs and configured. Sub-topic 4-3: Support the recommended WF. Sub-topic 4-4: Support the recommended WF. |

| Qualcomm | Sub-topic 4-1: Support the recommended WF. Sub-topic 4-2: We prefer option 2. Sub-topic 4-3: We support the recommended WF. Sub-topic 4-4: We support the recommended WF. |
| --- | --- |
| Samsung | Sub-topic 4-1: Support the recommended WF. Sub-topic 4-2: Support Option 1. If the RLM evaluation period is relaxed by scaling factor 5, then the lower boundary of the evaluation period will be 1000ms. At this time P will further prolong the evaluation period, causing a too long evaluation period which may cause a lagging link failure recovery procedure. Though radio link failure does not easily happen on IAB, its occurrence will lead to more severe consequence than UE. So we are worrying about P results in a too long period beyond expectation in some cases. And in the light of less measurements on IAB-MT, P is not necessary any more. If some companies still think P is necessary, maybe we need to prevent from a too long evaluation period by, for example, defining a maximal value of scaling factor P. Sub-topic 4-3: Support the recommended WF. Sub-topic 4-4: Support the recommended WF. |
| Nokia | Sub-topic 4-1: We support option 2. The Rel-15 specification could also applicable for IAB-MT. Why do we need to relax the RLM requirements? Even though IAB-MT is not mobility, there have other reasons which will cause the link broken. Sub-topic 4-2: We support option 2. The Rel-15 specification is fulfilled current IAB-MT requirement. It is not necessary to have this change. Sub-topic 4-3: Support the recommended WF. Sub-topic 4-4: Support the recommended WF. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 Relaxed RLM requirement | Tentative agreement: Downselect from following options during the 2nd round discussion: - Relax the RLM evaluation period of IAB-MTs with respect to that of NR UEs - Scale the number of samples and the lower boundary of the SSB and CSI-RS based RLM evaluation period by a factor. - FFS: Scaling factor - Do not relax RLM evaluation period of IAB-MTs with respect to that of NR UEs Candidate options of scaling factor: If relaxation of RLM evaluation period is agreed, consider following scaling factors: Option 1: Scaling factor of 5 Option 2: Other scaling factors Recommendations for 2nd round: Discuss the down-selection of two choices of the tentative agreement and the exact scaling factor in a separate WF during the 2nd round Reason for current version of the agreement: Five companies proposed to increase the number of samples and the lower boundary of the period. Nokia opposed and proposed not to relax the requirement. Hence, both options are mentioned in the agreement. |
| Sub-topic 4-2 Sharing factor P in RLM/BFD/CBD requirement | Tentative conclusion: No consensus in removing the sharing factor P in evaluation period for IAB RLM/BFD/CBD requirement in the 1st round. Candidate options: Option 1: Remove the sharing factor Option 2: Keep the sharing factor Recommendations for 2nd round: Discuss this issue further in a separate WF in the 2nd round |
| Sub-topic 4-3 Density of CSI-RS to define CSI-RS based RLM/BFD/CBD requirements | Tentative agreement: The CSI-RS requirements shall apply only for CSI-RS density 3 over the bandwidth ≥ 24 PRBs. Candidate options: Recommendations for 2nd round: No further discussion is needed. |
| Sub-topic 4-4 Other RLM requirements | Tentative agreement: The following RLM related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs. - Measurement restrictions for SSB based RLM and CSI-RS based RLM - Minimum requirement at transitions - Minimum requirement for L1 indication. - Scheduling availability during RLM Candidate options: Recommendations for 2nd round: No further discussion is needed. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | RLM requirement and sharing factor in RLM/BFD/CBD evaluation for IAB-MTs Suggestion to the assigned company: Discuss R4-2004838 as part of this WF and revise, if necessary. | ZTE |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004838 ZTE | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #5: Link recovery requirements

## Companies’ contributions summary

| Tdoc number | Company | Comments |
| --- | --- | --- |
| R4-2004839 | ZTE | - Proposal 1: Micro type of IAB MTs should reuse most of the BFD and BFR requirements for R15 Ues, with some relaxation on certain aspects. - Proposal 2: Omit the sharing factor P from the BFD and BFR requirements (P = 1 always). - Proposal 3: The discussion on whether to remove the sharing factor should be combined across all relevant sub-agendas and a decision should be made for all scenarios where P is involved. - Proposal 4: Beam sweeping factor N = 4. |
| R4-2003533 | Samsung | Observation 1: Consider non-mobility IAB in Rel-16, link recovery procedure for IAB-MT would be much simpler compared to UE’s procedure. Observation 2: For FR2, 3dB beamwidth of UE beam is normally much larger than that of MT beam; PC1 UE can be considered as candidate reference for IAB-MT. Observation 3: It is fatal for a MT to wrongly select the active beam because of its very narrow 3dB beamwidth. Beam failure recovery procedure is crucial for recovering the link, which should be performed timely and precisely. Proposal 1: Carefully define the beam recovery requirement for IAB MT and the same level requirement of candidate beam detection as UE should be applied for IAB-MT. Proposal 2: For IAB CBD requirement, beam sweeping factor N=8 in Evaluation Period calculation for FR2 should be reduced as less beam candidates for beam switching and more importance of beam selection. Proposal 3: For IAB CBD requirement, reduce the beam sweeping factor to N=4 of Evaluation Period in both SSB and CSI-RS based measurement cases. |
| R4-2004249 | Huawei, Hisilicon | Proposal 1: For the issues in terms of extending evaluation period, sharing factor and density of CSI-RS, keep the same principles for both RLM and BFR considering the immobility of IAB node. Proposal 2: Adopt the same beam sweeping factor which is 8 for RLM and Link recovery for FR2 in TS 38.174. |
| R4-2004803 | Qualcomm | Observation 1: Although RAN4 has not defined cell level measurement requirements, network can still configure these measurement gaps and the IAB-MTs that support this feature can follow those configurations. Observation 2: IAB-MTs may need to use narrower RX beams to obtain higher link budget. Observation 3: the beam sweeping factor for fixed Ues (power class 1) was also decided to be 8 in Rel-15. Proposal 1: Sharing factor P does not need to be removed from the BFD/CBD evaluation period of IAB-MTs. Proposal 2: For IAB CBD requirement, use beam sweeping factor N=8 in Evaluation Period calculation. Proposal 3: Reuse the SSB and CSI-RS based CBD evaluation period of Rel-15 Ues to define the requirements for IAB-MT nodes. Proposal 4: The following CBD related requirements, that have been defined in 38.133 for Rel-15 Ues, can be reused for IAB-MTs. - Measurement restrictions for SSB based CBD and CSI-RS based CBD - Scheduling availability during CBD |

## Open issues summary

Feature lead’s note:

- The issue of sharing factor for RLM/BFD/CBD is being discussed in the RLM sub-section.

- The issue of CSI-RS density for RLM/BFD/CBD is being discussed in the RLM sub-section

### Sub-topic 5-1

Issues: Beam sweeping factor

Proposal: For IAB CBD requirement, define beam sweeping factor N for FR2.

- Option 1: N = 8

  - Option 2: N = 4

- Recommended WF

  - Decide based on feedback.

### Sub-topic 5-2

Issues: Other requirements

Proposal: The following CBD related requirements, that have been defined in 38.133 for Rel-15 Ues, can be reused for IAB-MTs.

- Measurement restrictions for SSB based CBD and CSI-RS based CBD

- Scheduling availability during CBD

  - Option 1: Support above proposal

  - Option 2: Don’t support above proposal

Recommended WF:

The following CBD related requirements, that have been defined in 38.133 for Rel-15 Ues, can be reused for IAB-MTs.

- Measurement restrictions for SSB based CBD and CSI-RS based CBD

- Scheduling availability during CBD

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| ZTE | Sub topic 5-1: Prefer Option 2, N = 4. Sub topic 5-2: Support recommended WF. |
| Huawei | Sub topic 5-1: Support option 1, N=8 Sub topic 5-2: Support recommended WF |
| AT&T | Sub-topic 5-2: Prefer Option 1, N=8 to align with Rel-15 PC1 UE requirements Sub-topic 5-2: Support the recommended WF |

| Qualcomm | Sub-topic 5-1: Support option 1. The behavior of IAB-MTs is similar to fixed UE (PC 1). N = 8 for fixed UEs and it should be the same for IAB-MTs. Sub-topic 5-2: We support the recommended WF. |
| --- | --- |
| Samsung | Sub topic 5-1: Support option 2. Generally speaking IAB-MT have less effective candidate beams to be selected because of the immobility of IAB and prior known topology of IAB network. N = 4 can help IAB quickly recovery possible beam failures to maintain the quality of service. IAB beam failure may lead to more severe consequence since a large amount of date traffic will be transmitted on IAB which contains multiple UE packages. We believe the functionality of IAB differs from PC1 UE hence different requirement, then a lower N is proposed. Sub topic 5-2: Support recommended WF, which is aligned with our TP for CBD requirement. |
| Nokia | Sub-topic 5-1: Support option 1. Sub-topic 5-2; Support the recommended WF |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#5-1 Beam sweeping factor in CBD evaluation period | Tentative working assumption: For the CBD evaluation period of IAB-MTs, define beam sweeping factor N = 8 for FR2. Reason for working assumption: Companies’ positions for the beam sweeping factor N during the first round are given below: N = 8 is supported by: Nokia, Qualcomm, AT&T, Huawei N = 4 is supported by: ZTE, Samsung Since most of the companies did not seem to change their proposal from the previous meeting and a specific value of N needs to be down-selected, N = 8 is proposed to reflect the majority view. Candidate options: Recommendations for 2nd round: Discuss further if supporters of N = 4 cannot compromise to the proposed working assumption. Otherwise, no further discussion is necessary. |
| Sub-topic #5-2 Other CBD related requirements | Tentative agreement: The following CBD related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs. - Measurement restrictions for SSB based CBD and CSI-RS based CBD - Scheduling availability during CBD Candidate options: Recommendations for 2nd round: No further discussion is necessary. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2003534 Samsung | Company A |
|  | Company B |
|  | Company C |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Other contributions

R4-2004777 was submitted to IAB RRM agenda item. But it is not clear how it is related to IAB RRM discussion.