3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2005407

Electronic Meeting, 20 – 30 Apr. 2020

Agenda item:			6.16.1.3 /6.16.1.4

Source:	Moderator (OPPO)

Title:	Email discussion summary for [94e Bis][125] NR_CSIRS_L3meas_RRM_Part_2

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

In order to make progress on CSI-RS L3 measurement, we would like to discuss the following topics/issues and try to get some agreements in this meeting.

Topic #1: Measurement capability

RAN4 further discuss and conclude the remaining issues for measurement capability

- Clarify the understanding of MO configuration, intra-frequency measurement definition and layer definition

- Whether to restrict MO configuration to specify requirements for the limited or selected scenarios

- Define requirements of measurement capability for CSI-RS L3 measurement

Topic #2: Measurement requirements for CSI-RS intra-frequency and inter-frequency measurements

RAN4 to discuss whether/how to define measurement requirements for Case 1 and Case 2.

- Case 1: if associatedSSB is not configured

- Case 2: if associatedSSB is configured for CSI-RS

The following issues should be discussed in RAN4:

- CSSF

- Scaling factor for RX beam sweeping

- Scheduling restriction

- Restrict for CSI-RS resources outside of DRX/MG duration

- Note: Others are not precluded.

List of candidate target of email discussion for 1st round and 2nd round

As agreed in last meeting, decision on intra-frequency and inter-frequency measurement definition shall be made no later than in RAN4 #94bis. So moderator suggest to postpone some issues which are highly related to definition of CSI-RS intra/inter-frequency measurement in 1st round discussion.

- 1st round: Collect views from companies, and try to achieve agreements on the two sub-agenda:

  - Measurement capability

  - Measurement requirement for intra/inter-frequency measurement

- 2nd round: Discuss the remaining issues and propose a way forward for next meeting

Topic #1: Measurement capability

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003203 | Intel Corporation | Observation 1: for previous SSB based measurement, there can be only one frequency layer in one MO. The measurement capabilities based on MO or layer are the same. Proposal 1: To avoid the confusion about the capabilities dependency on MO or frequency layer for CSI-RS, it’s better to align the one MO configuration with one frequency layer. Observation 2: The definition of positioning frequency layer is defined as: the same SCS and CP type the same centre frequency the same point-A the same value of DL PRS Bandwidth and Start PRB. Proposal 2: Similar with definition of positioning frequency layer agreed in RAN1, CSI-RS frequency layer can be defined as: the same SCS and CP type the same centre frequency the same value of CSI-RS bandwidth. Proposal 3: all CSI-RS resources in the same MO will have the same bandwidth. Observation 3: RSRP performance degrade a lot especially for SCS=30KHz and 60KHz even with 3us timing offset. Proposal 4: side condition of the receiving timing difference between serving cell and neighbouring cell needs to be defined if associatedSSB is not configured for CSI-RS based measurement. |
| R4-2003260 | CATT | Proposal 1: CSI-RS based UE measurement capabilities shall specified in terms of: - Number of carrier to be monitored - Number of cell to be monitored per frequency layer - Number of CSI-RS resources to be monitored per frequency layer Proposal 2: UE shall be able to measure at least 3 CSI-RS frequency layers. Proposal 3: UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers. Proposal 4: UE shall be able to measure at least 14 carriers of all RATs in total. Proposal 5: The requirement of number of cell to be monitored defined for SSB based UE measurement capability can be reused to define the number of cells to be monitored for CSI-RS based UE measurement capabilities. Proposal 6: For the number of CSI-RS resource, UE shall monitor at least 32 CSI-RS resources per frequency layer considering mobility performance and UE’s complexity. |
| R4-2003645 | MediaTek inc. | Observation 1: SSB-based cell detection is always required for CSI-RS based L3 measurement. Observation 2: If proposal 1 is not agreed, CSI-RS based L3 measurement will add additional frequency layers to be monitored on top of the number specified for SSB based measurement, resulting the extended measurement delay of every frequency layer and increased complexity in UE measurement scheduling. Observation 3: If RAN4 allows some CSI-RS resources in the same MO to be intra-frequency and the other to be inter-frequency, the total number of frequency layers to be monitored will be increased further. Proposal 1: Requirement is only defined when the CSI-RS based measurement is configured with associated SSB and with SSB based measurement within the same MO. Proposal 2: Given proposal 1 is agreed, CSI-RS based L3 measurement does not add additional frequency layers to be monitored on top of the number specified for SSB based measurement. Proposal 3: There is no need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB. Proposal 4: Regarding the number of CSI-RS (beams) to be monitored per layer based on L3 CSI-RS, requirements defined the same requirements as those for SSB. Proposal 5: If network configures more CSI-RS resources in an MO than the UE measurement capability, the UE behavior is undefined. |
| R4-2003856 | CMCC | Proposal 1: it is proposed to specify requirements for the case with associatedSSB and the case without associatedSSB. Proposal 2: for the case without associatedSSB, UE base the timing on its serving cell; for the case with associatedSSB, UE base the timing on the cell given by the cellId of the CSI-RS resource configuration. Proposal 3: for the number of frequency layers, it is proposed that UE is capable of measuring: - [3] CSI-RS frequency layers - [8] NR frequency layers including SSB frequency layers and CSI-RS frequency layers - [14] effective carrier frequency layers comprising of different RAT Proposal 4: For each intra-frequency layer, it is proposed that UE is capable of measuring [8] CSI-RS cell Proposal 5: For each inter-frequency layer, it is proposed that UE is capable of measuring [4] CSI-RS cell Proposal 6: For each intra-frequency layer, the number of CSI-RS resource is proposed to be [32]. Proposal 7: For each inter-frequency layer, the number of CSI-RS resource is proposed to be [24]. |
| R4-2003984 | OPPO | Proposal 1: Only define the requirements when the CSI-RS based measurement is configured with SSB based measurement within the same MO, so that UE can perform CSI-RS and SSB measurement with overlapped bandwidth in the same time duration. Proposal 2: No requirements for MOs configured with only CSI-RS based L3 measurement. Observation 1: CSI-RS resource for mobility is beneficial to be configured with associated SSB, which can reduce the complexity of both network and UE implementation. Proposal 3: CSI-RS based L3 measurement does not add additional frequency layers and cells to be monitored on top of the number specified for SSB based measurement. - UE shall be able to measure at least 7 NR inter-frequency frequency layers in total, including SSB frequency layers and CSI-RS frequency layers. - UE shall be able to measure a total of at least 13 effective carrier frequency layers of all RATs in total, including SSB frequency layers and CSI-RS frequency layers. Proposal 4: No need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB. Proposal 5: UE shall monitor at least 32 CSI-RS resources per frequency layer |
| R4-2004114 | ZTE | Proposal 1. No restriction on MO configuration except centre frequency and total number of CSI-RS resources. Proposal 2. One frequency layer means centre frequencies of configured CSI-RS resources are the same. Proposal 3. One or multiple MOs can be one frequency layer. Proposal 4. A UE shall be capable of monitoring at least 7 NR inter frequency layers configured by serving cell. The UE shall be capable of performing measurements including (SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, CSI-SINR, E-UTRAN RSRP, E-UTRAN RSRQ, E-UTRAN RS-SINR measurements, etc.) of detected cells on all the layers. Proposal 5. UE capability of number of cells and number of CSI-RS resources that the UE shall be capable of performing CSI-RS based measurement for L3 mobility should be specified. Proposal 6. UE shall be capable of performing CSI-RS based measurements for at least [8] identified cells in FR1 for intra frequency measurement and at least [4] identified cells in FR1 for inter frequency measurement. Proposal 7. UE shall be capable of performing CSI-RS based measurements for at least [6] identified cells in FR2 for intra frequency measurement and at least [4] identified cells in FR2 for inter frequency measurement. Proposal 8. UE shall be capable of performing SSB and CSI-RS based measurements for at least [12] identified cells in FR1 for intra frequency measurement and at least [6] identified cells in FR1 for inter frequency measurement. Proposal 9. UE shall be capable of performing CSI-RS based measurements for at least [9] identified cells in FR2 for intra frequency measurement and at least [6] identified cells in FR2 for inter frequency measurement. Proposal 10. Number of CSI-RS resources shall be monitored by UE is specified as in option 2. Proposal 11. Not to define UE capability to indicate maximum CSI-RS resources in a slot per MO. Proposal 12. LS RAN1/2 for clarification on the background of UE capability maxNumberCSI-RS-RRM-RS-SINR, potential contradiction to requirements on number of CSI-RS resources UE shall monitor and necessity of this UE capability. Proposal 13. Gap configuration, frequency layer etc. should be taken into account if UE capability of the minimum separation between two slots with CSI-RS resources is to be introduced. |
| R4-2004374 | Huawei, HiSilicon | Proposal 1: CSI-RS measurement capability requirements are defined for the case where all the CSI-RS resources in an MO have the same BW. There is no other restriction on the MO configuration. Proposal 2: Measurement capability requirements are defined based on MO. A frequency layer is identical to an MO for measurement capability requirements. Proposal 3: The definition of intra/inter-frequency should be such that there is only one intra-frequency layer for each serving cell, and RAN4 only defines the number of inter-frequency layers UE shall monitor. Proposal 4: UE shall be able to measure at least 3 CSI-RS layers for CSI-RS based mobility. Proposal 5: UE shall be able to measure at least 7 SSB frequency layers, including MO with SSB as mobility RS, and MO with CSI-RS as mobility RS with associated SSB. Proposal 6: UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers. Proposal 7: UE shall be able to measure at least 14 carriers of all RATs in total. Proposal 8: Re-use the SSB requirements for CSI-RS on number of cells UE shall monitor per layer. Proposal 9: UE shall monitor at least 32 CSI-RS resources per layer. Proposal 10: Introduce UE capability to indicate the maximum number of CSI-RS resources per MO in a slot. Proposal 11: Introduce UE capability on the minimum separation between two slots with CSI-RS resources. |
| R4-2004826 | Qualcomm | Observation1: when a CSI-RS resource does not have an associated SSB for measuring a neighbour cell, UE measurement accuracy is affected by how well the neighbor cells are synchronized with the serving cell. Proposal 3: Requirements shall be defined when CSI-RS is configured with an associated SSB. Proposal 3a: Number of monitored cells is determined by the UE capability based on SSB based measurements. Proposal 3b: “The total number of CSI resources that UE can monitor should come from the UE capability maxNumberCSI-RS-RRM-RS-SINR.” [3] Proposal 4: Performance requirements should also be defined based the assumption that UE uses a single FFT window for processing all CSI-RSs on a single frequency layer. |
| R4-2003408 | Apple | Proposal 1: - When the SCS of CSI-RS on serving cell and target cell is the same and the CSI-RS resource of the target cell is fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as intra-frequency measurement without gap. - When the SCS of CSI-RS on serving cell and target cell is NOT the same or the CSI-RS resource of the target cell is fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as inter-frequency measurement. Proposal 2: Introduce CSI-RS Measurement Timing Configuration (CMTC). - All CSI-RS resources for L3 meaurement should be configured within CMTC window - CMTC window duration: considering CSI-RS periodicity is up to 40ms, the CMTC window should be less than 5ms. - Up to 2 CMTC periodicities can be configured per CSI-RS intra-frequency layer - Up to 1 CMTC periodicity can be configured per CSI-RS inter-frequency layer Proposal 3: Further restriction on CSI-RS MO configuration for mobility in Rel-16 include - A fixed channel bandwidth per MO should be configured - Up to 2 CSI-RS resources periodicities can be configured per intra-frequency MO - Up to 1 CSI-RS resource periodicity can be configured per inter-frequency MO |
| R4-2003809 | Nokia, Nokia Shanghai Bell | Proposal1: The restriction of all CSI-RS resources in the same MO having the same bandwidth does not bring any benefit. Proposal2: For intra-frequency measurement, the requirement is defined when all the measured CSI-RS resources in one MO are within the active BWP. Proposal3: It can be assumed maximum one MO is configured per center frequency and SCS. Proposal4: Do not define the UE requirements when associatedSSB is not configured in Rel16. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: Assumption and applicability

Sub-topic description

In this section, RAN4 further discuss and conclude the remaining issues for measurement capability

- Clarify the understanding of MO configuration, intra-frequency measurement definition and layer definition

- Whether to restrict MO configuration to specify requirements for the limited or selected scenarios

- Define requirements of measurement capability for CSI-RS L3 measurement

Open issues and candidate options before e-meeting:

Issue 1-1-1: Applicability for CSI-RS based measurement capability

- FFS on whether to introduce restriction of MO configuration - FFS whether to specify requirements for the limited or selected scenarios if no restriction of MO configuration - Candidate options: - Option 1(MediaTek, OPPO): Only define the requirements when the CSI-RS based measurement is configured with SSB based measurement within the same MO, and UE can perform CSI-RS and SSB measurement with overlapped bandwidth in the same time duration - Option 2(Ericsson, CATT, CMCC, Huawei, DOCOMO, ZTE, Nokia): No restriction on MO configurations - Option 3: Other possible restriction in order to specify requirements for the limited or selected scenarios.

- Proposals

  - Option 1 (MediaTek, OPPO): Only define the requirements when the CSI-RS based measurement is configured with SSB based measurement within the same MO

  - Option 1a (Qualcomm): Requirements shall be defined when CSI-RS is configured with an associated SSB.

  - Option 2 (CMCC): Specify requirements for the case with associatedSSB and the case without associatedSSB.

  - Option 2a (Huawei): CSI-RS measurement capability requirements are defined for the case where all the CSI-RS resources in an MO have the same BW. There is no other restriction on the MO configuration.

  - Option 2b (ZTE): No restriction on MO configuration except centre frequency and total number of CSI-RS resources.

  - Option 2c (Nokia): The restriction of all CSI-RS resources in the same MO having the same bandwidth does not bring any benefit.

  - Option 3 (Intel): Side condition of the receiving timing difference between serving cell and neighbouring cell needs to be defined if associatedSSB is not configured for CSI-RS based measurement.

- Recommended WF

  - Option 1 or 1a ?

  - Depend on the definition of intra-frequency and inter-frequency measurement

Issue 1-1-2: Measurement capabilities per MO or per layer

- A frequency layer for CSI-RS is - Option 1: just the same center frequency of CSI-RS resource - Option 2: the same center frequency and BW of CSI-RS resource - Option 3: one MO, i.e. same center frequency, same SCS - FFS how to align the understanding of MO and frequency layer in RAN4. - Depend on the definition/restriction of intra-frequency and inter-frequency measurement

- Proposals

  - Option 1 (Intel): Align one MO configuration with one frequency layer.

    - CSI-RS frequency layer can be defined as: the same SCS and CP type, the same centre frequency, the same value of CSI-RS bandwidth,

    - All CSI-RS resources in the same MO will have the same bandwidth.

  - Option 2 (ZTE): One or multiple MOs can be one frequency layer.

    - One frequency layer means centre frequencies of configured CSI-RS resources are the same.

  - Option 3 (Huawei): Measurement capability requirements are defined based on MO.

    - A frequency layer is identical to an MO for measurement capability requirements.

  - Option 4 (Nokia): It can be assumed maximum one MO is configured per center frequency and SCS.

- Recommended WF

  - Based on all proposals and RAN1’s LS [R1-1903856], can we agree on working assumption firstly?

  - Working assumption

    - CSI-RS resources in the same MO shall at least have the same centre frequency (i.e., startPRB + floor(nrofPRBs/2)) value and the same SCS and CP type. FFS the same CSI-RS bandwidth

  - If the assumption is agreed, measurement capabilities per MO or per layer are the same.

  - Depend on the definition of intra-frequency and inter-frequency measurement

Sub-topic 1-2: Requirements of measurement capabilities

Sub-topic description

In this section, we discuss measurement capabilities for CSI-RS based L3 measurement including:

- number of frequency layers to be monitored

- number of cells to be monitored per layer

- number of CSI-RS resource/beams to be monitored per layer

Open issues and candidate options before e-meeting:

Issue 1-2-1: number of frequency layers to be monitored

- Option 1 (MediaTek, OPPO, ZTE, Apple, Ericsson, DOCOMO): - CSI-RS based L3 measurement does not add additional frequency layers and cells to be monitored on top of the number specified for SSB based measurement. - Option 2(Huawei): - Assuming no restriction on MO configurations: - UE shall be able to measure at least 3 CSI-RS frequency layers. - UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers. - UE shall be able to measure at least 14 carriers of all RATs in total. - Option 3(CATT): - If CSI-RS is configured with associated SSB, the measurement capability can be considered the same as SSB based measurement capability.(as option 1) - If CSI-RS is not configured with any associated SSB, UE is capable to perform CSI-RS based layers.(as option 2) - Not preclude other options.

- Proposals

  - Option 1 (MediaTek, OPPO, Qualcomm, Nokia, Apple):

    - CSI-RS based L3 measurement does not add additional frequency layers and cells to be monitored on top of the number specified for SSB based measurement.

  - Option 1a (ZTE):

    - A UE shall be capable of monitoring at least 7 NR inter frequency layers configured by serving cell.

    - The UE shall be capable of performing measurements including (SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, CSI-SINR, E-UTRAN RSRP, E-UTRAN RSRQ, E-UTRAN RS-SINR measurements, etc.) of detected cells on all the layers.

  - Option 2 (Huawei):

    - The definition of intra/inter-frequency should be such that there is only one intra-frequency layer for each serving cell, and RAN4 only defines the number of inter-frequency layers UE shall monitor.

    - UE shall be able to measure at least 3 CSI-RS frequency layers.

    - UE shall be able to measure at least 7 SSB frequency layers, including MO with SSB as mobility RS, and MO with CSI-RS as mobility RS with associated SSB.

    - UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers.

    - UE shall be able to measure at least 14 carriers of all RATs in total.

  - Option 3 (CATT, CMCC):

    - UE shall be able to measure at least 3 CSI-RS frequency layers.

    - UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers.

    - UE shall be able to measure at least 14 carriers of all RATs in total.

- Recommended WF

  - TBA

Issue 1-2-2: number of cells to be monitored per layer/MO

- Option 1 (MediaTek, OPPO, Apple, Ericsson): No need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB. - Option 2 (ZTE): UE shall be capable of performing CSI-RS based measurements for at least [8] identified cells in FR1 for intra frequency measurement and at least [4] identified cells in FR1 for inter frequency measurement, at least [6] identified cells in FR2 for intra frequency measurement and at least [4] identified cells in FR2 for inter frequency measurement. - Option 3 (Huawei, CATT): RAN4 should discuss the total number of cells and CSI-RS resources UE should measure per MO.

- Proposals

  - Option 1: Shared capability for CSI-RS and SSB

    - Option 1a (MediaTek, OPPO): No need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB.

    - Option 1a (Qualcomm): Number of monitored cells is determined by the UE capability based on SSB based measurements.

  - Option 2:  Separated capability for CSI-RS

    - Option 2a (Huawei, CATT):

      - Re-use the SSB requirements for CSI-RS on number of cells UE shall monitor per layer.

    - Option 2b (CMCC):

      - For each intra-frequency layer, UE is capable of measuring [8] CSI-RS cell;

      - For each inter-frequency layer, UE is capable of measuring [4] CSI-RS cell.

  - Option 3: Capabilities for CSI-RS only and CSI-RS&SSB

    - Option 2a (ZTE):

      - UE shall be capable of performing CSI-RS based measurements for at least [8] identified cells in FR1 for intra frequency measurement and at least [4] identified cells in FR1 for inter frequency measurement, at least [6] identified cells in FR2 for intra frequency measurement and at least [4] identified cells in FR2 for inter frequency measurement.

      - UE shall be capable of performing SSB and CSI-RS based measurements for at least [12] identified cells in FR1 for intra frequency measurement and at least [6] identified cells in FR1 for inter frequency measurement.

      - UE shall be capable of performing CSI-RS based measurements for at least [9] identified cells in FR2 for intra frequency measurement and at least [6] identified cells in FR2 for inter frequency measurement.

  - Recommended WF

  - At least define capability for CSI-RS&SSB based measurements

    - FFS: reuse the requirements for SSB, or additional cells on top of those for SSB？

  - FFS if separated capability for CSI-RS based L3 measurement is needed

Issue 1-2-3: number of CSI-RS resource/beams to be monitored per layer/MO

- Option 1 (CATT, Huawei, OPPO): UE shall monitor at least 32 CSI-RS resources per frequency layer - Option 2 (ZTE, Ericsson, CATT): Number of CSI-RS resources shall be monitored by UE, - [24] CSI-RS resources for intra frequency measurements in FR1, - [48] CSI-RS resources for intra frequency measurements in FR2, - [16] CSI-RS resources for inter frequency measurements in FR1, - [24] CSI-RS resources for inter frequency measurements in FR2. - Option 3 (MediaTek): Requirements defined the same requirements as those for SSB.

- Proposals

  - Option 1(CATT, Huawei, OPPO): UE shall monitor at least 32 CSI-RS resources per frequency layer

  - Option 2(ZTE): Define different UE capability for different scenarios, and number of CSI-RS resources shall be monitored by UE,

    - [24] CSI-RS resources for intra frequency measurements in FR1

    - [48] CSI-RS resources for intra frequency measurements in FR2,

    - [16] CSI-RS resources for inter frequency measurements in FR1,

    - [24] CSI-RS resources for inter frequency measurements in FR2.

  - Option 3 (MediaTek): Requirements defined the same requirements as those for SSB.

    - If network configures more CSI-RS resources in an MO than the UE measurement capability, the UE behavior is undefined.

  - Recommended WF

  - Tentative agreement:  Option 3

    - Requirements defined the same requirements as those for SSB.

    - Do not define UE behaviour if network configures more CSI-RS resources in an MO than the UE measurement capability

Sub-topic 1-3: Buffering and processing capability

Issue 1-3-1: UE capability to indicate maximum number of CSI-RS resources in a slot per MO

- RAN to further discuss whether and how to to indicate maximum CSI-RS resources in a slot per MO - RAN4 decide whether to send LS to ask for RAN1 and RAN2 in next meeting

- Proposals

  - Option 1(Huawei): Introduce UE capability to indicate the maximum number of CSI-RS resources per MO in a slot.

  - Option 2(ZTE): Not to define UE capability to indicate maximum CSI-RS resources in a slot per MO.

  - Option 3(Qualcomm): The total number of CSI resources that UE can monitor should come from the UE capability maxNumberCSI-RS-RRM-RS-SINR.

    - FFS how to split up

  - Recommended WF

  - TBA

  - LS RAN1/2 for clarification on the background of UE capability maxNumberCSI-RS-RRM-RS-SINR, potential contradiction to requirements on number of CSI-RS resources UE shall monitor and necessity of this UE capability

Issue 1-3-2:  UE buffering and processing capability

- FFS on UE buffering and processing capability - Option 1: UE capability on UE buffering and processing time RAN1 defined for PRS is re-used for CSI-RS L3 measurement. - Option 2: Define UE capability on the minimum separation between two slots with CSI-RS resources.

- Proposals

  - Option 1: Introduce UE capability on the minimum separation between two slots with CSI-RS resources.

  - Option2 (ZTE): Gap configuration, frequency layer etc. should be taken into account if UE capability of the minimum separation between two slots with CSI-RS resources is to be introduced.

  - Recommended WF

  - TBA

Sub-topic 1-4: On CSI-RS resources configurations

Issue 1-4-1: How flexible CSI-RS resources per PCI per MO can be configured.

- Proposals

  - Option 1: (Apple):

    - A fixed channel bandwidth per MO should be configured

    - Up to 2 CSI-RS resources periodicities can be configured per intra-frequency MO

    - Up to 1 CSI-RS resource periodicity can be configured per inter-frequency MO

    - Introduce CSI-RS Measurement Timing Configuration (CMTC).

      - All CSI-RS resources for L3 meaurement should be configured within CMTC window

      - CMTC window duration: considering CSI-RS periodicity is up to 40ms, the CMTC window should be less than 5ms.

      - Up to 2 CMTC periodicities can be configured per CSI-RS intra-frequency layer

      - Up to 1 CMTC periodicity can be configured per CSI-RS inter-frequency layer

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

- Sub-topic 1-1: Assumption and applicability

| Company | Comments |
| --- | --- |
| MTK | Issue 1-1-1: Applicability for CSI-RS based measurement capability Option 1 and 1a are not contradicting to each other. We support to consider both Option 1 and 1a together. Option 2a might be resolved if the MO restriction is concluded in [94e Bis][124] NR_CSIRS_L3meas_RRM_Part_1. Issue 1-1-2: Measurement capabilities per MO or per layer Support Option 3 which is already clear at this moment. Whether to have restriction on BW in one MO is another discussion. |
| Apple | Issue 1-1-1: Possible restriction should be discussed and agreed. Option 2 can result in a huge number of scenarios, which are unnecessariliy complicating the following discussion. We suggest combine this issue with issue 1-4. Among options, we are fine with option 1 and 1a. Issue 1-1-2: we think it is depending on the definition of intra-/inter-frequency layer. Theoretically, there can be multiple MO per layer. It makes more sense to define UE capability based on frequency layer. |
| CATT | Issue 1-1-1: Support option 2, specify requirements for the case with associatedSSB and the case without associatedSSB. In synchronous network deployment, it would be quicker and more efficient to configure non-associated SSB to UE to perform CSI-RS measurement. Issue 1-1-2: Measurement capability requirements are defined based on frequency layer, and a frequency layer is identical to an MO according to RAN1’s agreement. |
| Intel | Issue 1-1-1: We are fine with option 1. If option 2 is chosen, then some side condition about timing offset should be considered Issue 1-1-2: Fine with option 3. Besides, for option 1, we further define how to align the two concepts together. |
| CMCC | Issue 1-1-1: Applicability for CSI-RS based measurement capability MO configuration is specified in RAN1/2 in Rel-15, it is not preferred to have restriction on the MO configuration. For the requirements specification, we can further discuss the applied scenario in RAN4, e.g. same BW. As for the scenario without associatedSSB, considering that it is possible scenario for the CA case, we do not prefer to preclude it. Issue 1-1-2: Measurement capabilities per MO or per layer According to RAN1 agreements, same MO only consider the centre frequency, CP and SCS, BW is not considered. It is better to avoid the case that different/multiple MO are considered as one layer. In our view, if the intra-frequency measurement is defined only consider the centre frequency, CP, and SCS, MO is the same with frequency layer. |
| Huawei, HiSilicon | Issue 1-1-1: Applicability for CSI-RS based measurement capability We understand whether to define requirements for CSI-RS without associated SSB is addressed in Issue 2-1-1. If RAN4 decides to define the measurement requirements then the measurement capability should be also defined. The MO restriction is discussed in email thread [124] and we can wait for the conclusion. Issue 1-1-2: Measurement capabilities per MO or per layer Support option 3. Based on RAN1 LS, for CSI-RS measurement frequency layer is identical as MO, and we do not see the need for RAN4 to further develop the concept of frequency layer, as it may complicate the discussion. |
| ZTE | Issue 1-1-1: According to current MO configuration, the centre frequency, SCS for all the CSI-RS resources are the same and there is limitation on the total number of CSI-RS resources can be configured. From RRM requirements perspective we don’t see any further restriction are needed. There are different use cases for w/wo associatedSSB in MO configuration. Requirements should be specified for both use cases. Issue 1-1-2: A frequency layer is defined for the purpose that measurements on the frequency layer can be conducted at the same time. For SSB based measurements the centre frequency of SSB indicates all the SSBs of different cells with the same centre frequency. However there is limitation on the total number of CSI-RS resources that can be configured in MO, i.e. 96 or 64 depending on if associatedSSB is configured. It is high likely that 1 or 2 cells will have CSI-RS resources in total beyond the limitation. It is not true that a UE can see 1 or 2 neighbour cells. To configure more cells with CSI-RS on the same center frequency, multiple MOs should be used. As the center frequency of CSI-RS resources all multiple MOs are the same, it should be regarded as one frequency layer as the measurement can be done at a time. |
| Qualcomm | Issue 1-1-1: Applicability for CSI-RS based measurement capability Clarification on the use case of measuring CSI-RS without associated SSB is desired since UE is not able to identify the neighbor cell given the CSI-RS alone especially this is in the context of mobility. Otherwise, we agree with MTK on combining option 1 and option 1a. Issue 1-1-2: Measurement capabilities per MO or per layer We agree with the recommended WF and think we need to further require same BW for all the CSI-RS per MO/layer for defining the intra-frequency measurements. |
| Nokia, Nokia Shanghai Bell | Issue 1-1-1: Prefer not having any restriction to MO configuration, but we can define the requirements only when associatedSSB is configured in Rel16 i.e. Option 1a. For associatedSSB, it is understood the network will probably configure it if the timing difference between the two cells may exceed single FFT. To facilitate the discussion, we may focus on the most likely scenario to define the requirements. Issue 1-1-2: We share the understanding of Option 3. According to RAN1, one frequency layer is one MO i.e. Option 3. From RAN2 spec, it is allowed to configure multiple MOs over the same carrier frequency i.e. there could be multiple MOs/frequency layers with same center frequency. If we want to change the concept of “frequency layer”, we need send LS to RAN1 checking the feasibility. |
| Docomo | Issue 1-1-1: Applicability for CSI-RS based measurement capability We don’t understand why both of measurement with SSB (e.g., associatedSSB) based measurement and CSI-RS based measurement should be performed in the same MO. We think associatedSSB is not equal to SSB for measurement, thus configuration of them should be considered separately. On the other hand, we prefer to avoid making any restriction on MO configuration for keeping flexibility of CSI-RS. Thus, both of the case with associated SSB and the case without associatedSSB, and the case that CSI-RS resources for measurement are configured with different BWs in the same MO should be into consideration because they are possible according to RAN2 specification. Therefore, option 2, 2b and 2c are preferable for us. Issue 1-1-2: Measurement capabilities per MO or per layer We agree with the recommended WF as CSI-RS resources in the same MO should have the same center frequency, SCS and CP type. |
| OPPO | Issue 1-1-1: Applicability for CSI-RS based measurement capability Fine with option 1 and 1a. Whether to define requirements for CSI-RS without associated SSB has impact on both measurement capability and requirements in issue 2-1-1. And we can further come back after agreement on CSI-RS resources configurations in sub-topic 1-4 and MO restriction in [124] part1. Issue 1-1-2: Measurement capabilities per MO or per layer Agree with the recommended WF. And if the assumption is agreed, measurement capabilities per MO or per layer are the same. |

- Sub-topic 1-2: Requirements of measurement capabilities

| Company | Comments |
| --- | --- |
| MTK | Issue 1-2-1: number of frequency layers to be monitored Support Option 1. Here we are discussing inter-frequency measurement. Without SSB to provide information about timing, frequency error, AGC reference (although may not be perfect) and Rx beam direction, how should UE perform CSI-RS measurement? Therefore, the frequency layers for CSI-RS measurement should always be the subset of frequency layers for SSB measurement. Further comment to HW: We agree with HW that the engines for CSI-RS measurement and SSB measurement are different. But the thing we want to highlight is that it is not possible to perform CSI-RS measurement without detecting SSB first for an inter-frequency. Therefore, the frequency layers that UE can perform CSI-RS measurement is always a subset of SSB-based frequency layers Issue 1-2-2: number of cells to be monitored per layer/MO Support Option 1. Same comment as Issue 1-2-1. Further comment to HW: We think one clarification is needed when we say ‘re-using’ the SSB requirement. For an example, if UE is already monitoring 8 cells based on SSB: Cell#0, #1, … #7. In our understanding, the 8 cells to be monitored by CSI-RS should also be Cell#0, #1, … #7. But we are not sure if Option 2a means UE is required to monitor other 8 different cells based on CSI-RS, e.g., Cell#8, #9, …, #15. Issue 1-2-3: number of CSI-RS resource/beams to be monitored per layer/MO Support Option 3. We would like to see system-level analysis for new numbers. Otherwise, those numbers for SSB should be the baseline, which was concluded based on simulation alignment. Further comment: We wonder how more CSI-RS beams from gNB is going to maintain the same mobility robustness as SSB-based measurement. More and finer CSI-RS beams means the coverage of each CSI-RS beam becomes narrower than a SSB beam. This is going to increase the change of handover failure because the best CSI-RS beam reported by UE could be highly likely not a suitable beam to that UE after handover. |
| Apple | Issue 1-2-1: We agree with the principle in Option 1 that CSI-RS L3 measurement should not increase the existing intra-/inter-frequency measurement capability including “UE shall be capable of monitoring a total of at least 13 effective carrier frequency layers” Issue 1-2-2: Depending on the agreementon on issue 1-4, if CSI-RS resources configuration per cell is too flexible, the corresponding UE capability is impacted. However, if SSB-liked CSI-RS configuration restriction can be assumed, UE capability can be defined based on SSB based one. Issue 1-2-3: similar comments as 1-2-2 that it depends on how flexible CSI-RS resources can be configured. Otherwise, it is hard to define a specific number to cover all scenarios. |
| CATT | Issue 1-2-1: Both option 2 and option 3 are OK for us. As we support to specify requirements for the case with associatedSSB and the case without associatedSSB. Issue 1-2-2: support option 2a, re-use the SSB requirements in FR1 and FR2 to define CSI-RS requirement on number of cells UE shall monitor per layer. Issue 1-2-3: Support option 1. 32 or the number for SSB (24) for CSI-RS resources can be as the baseline. |
| CMCC | Issue 1-2-1: number of frequency layers to be monitored If both the case with associatedSSB and the case without associatedSSB are considered, it is not acceptable for us to reuse the number of frequency layers specified for SSB. Issue 1-2-2: number of cells to be monitored per layer/MO We prefer to introduce the separate capability on the number of cells for CSI-RS. As for the detail value, we are open to discuss. Issue 1-2-3: number of CSI-RS resource/beams to be monitored per layer/MO Considering the number of beams for CSI-RS is larger than that for SSB, reusing the requirements specified for SSB is not preferred. For intra-frequency measurement and inter-frequency measurement, different value can be considered. Our suggestion is: - For each intra-frequency layer, the number of CSI-RS resource is proposed to be [32]. For each inter-frequency layer, the number of CSI-RS resource is proposed to be [24]. |
| Huawei, HiSilicon | Issue 1-2-1: number of frequency layers to be monitored Support option 2. We do not quite understand option 1. Even CSI-RS is configured with associated SSB, the CSI-RS measurement does not come for free. CSI-RS is in different time and frequency locations than SSB, and all the actions for measurement e.g. sampling, processing and filtering need to be done separately for CSI-RS. Therefore, we need to define separate capabilities for the numbers of SSB layers and number of CSI-RS layers. Issue 1-2-2: number of cells to be monitored per layer/MO Support option 2a. Take example of CSI-RS with associated SSB, UE only measures the CSI-RS after detecting the associated SSB. If UE is required to measure X cells for SSB measurement, it means UE is also required to measure X cells for CSI-RS measurement, so it is meaningful to define the requirements for CSI-RS by reusing the requirements for SSB. However, in addition to the SSB measurement, UE is taking CSI-RS measurement for the X cells, so separate capability for CSI-RS is needed. Issue 1-2-3: number of CSI-RS resource/beams to be monitored per layer/MO Support option 1. In our understanding, there may be more than one CSI-RS resources associated to the same SSB, so the number of CSI-RS resources can be larger than number of SSBs. |
| ZTE | Issue 1-2-1: We understand that UE measurement capability on total number of frequency layers may not be increased by introducing CSI-RS based L3 measurement. On a frequency layer all kinds of measurement, e.g. SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, CSI-SINR, E-UTRAN RSRP, E-UTRAN RSRQ, E-UTRAN RS-SINR measurements, etc. shall be performed if applicable. Option 2 also makes sense in terms of more detailed requirements. If going with option 2 we would like to change the 2nd sub-bullet to ‘UE shall be able to measure at least 7 CSI-RS frequency layers’. The other parts are agreeable to us. Issue 1-2-2: The UE measurement capability are specified with separated number and total number in principle. The Total number is much smaller than summation of all separate numbers. The same rule can be applied here. Without specifying number of cells for CSI-RS based L3 measurement, the UE capability for SSB based measurement will be compromised and the mobility performance will be degraded. Issue 1-2-3: Option 2 is good balance between UE complexity and use cases in our view. We disagree with the recommend WF. |
| Qualcomm | Issue 1-2-1: number of frequency layers to be monitored We support option1. Issue 1-2-2: number of cells to be monitored per layer/MO We support option1 to have shared capability with SSB based measurement. After all, it is required to have corresponding SSB to identify/monitor the cell. Issue 1-2-3: number of CSI-RS resource/beams to be monitored per layer/MO Please note our proposal on the capability of maxNumberCSI-RS-RRM-RS-SINR is not intended to address the [total] number of CSI-RS resource/beams to be monitored for this issue. Our view towards this issue is unless there is a benefit to monitor extra CSI-RS resources, the baseline shall be based on what is required for the SSB based measurements set forth in 38.133 9.2.3 and 9.3.3. |
| Nokia, Nokia Shanghai Bell | Issue 1-2-1: Prefer with the principle of Option1, but it also depends on how we define a “frequency layer” in Issue 1-1-2. If we follow RAN1 definition, there would be multiple frequency layers/MOs with the same center frequency for CSI-RS based measurement. With the same capability, the number of “frequency layers i.e. MO” will be increased. Alternatively, we may consider having one MO including all the CSI-RS resources with the same center frequency. This will result one frequency layer/MO per center frequency. With Option 2 in Issue 1-1-2, it helps align the “frequency layer” definition with SSB-based measurement. But as it now changed the RAN1 decision, we need send LS to RAN1 asking for the feasibility. Also we need define the number of MOs per frequency layer as one of measurement capabilities. Issue 1-2-2: Prefer Option 1a. If we only define the requirement for the case where associatedSSB is configured, the UE is required to detect the associatedSSB before monitoring CSI-RS. So the number of cells would be the same as the number defined for SSB-based measurement. Issue 1-2-3: We are fine with either Option 1 or Option2. maxNumberCSI-RS-RRM-RS-SINR is defined as the capability across all measurement frequencies per slot. In addition to this, we need define the number of CSI-RS resources per frequency layer or MO, which depends on how a frequency layer is defined in Issue 1-1-2. |
| OPPO | Issue 1-2-1: number of frequency layers to be monitored Agree with MTK and support option 1. Issue 1-2-2: number of cells to be monitored per layer/MO Still support option 1 to have shared capability with SSB based measurement. Issue 1-2-3: number of CSI-RS resource/beams to be monitored per layer/MO Agree with the recommended WF. If no conclusion in this meeting, as moderator I’d like to encourage people to bring possible simulation analysis to move forwards. |

- Sub-topic 1-3: Buffering and processing capability

| Company | Comments |
| --- | --- |
| MTK | Issue 1-3-1: UE capability to indicate maximum number of CSI-RS resources in a slot per MO We are OK with Option 1. In addition, we may need to discuss how to deal with exiting capability maxNumberCSI-RS-RRM-RS-SINR. Issue 1-3-2: UE buffering and processing capability FFS We understand that the UE processing capability is an issue. But some more study is still needed on how to address it effectively. The main issue comes from the extremely flexible time domain configuration of CSI-RS. UE may experience a very crowed slot with a huge number of CSI-RS configured from one MO or kind of uniformly distributed configuration where every slot has 1 or 2 CSI-RS. To address this issue, RAN4 may need to think whether to have some time domain restriction for CSI-RS configuration first. |
| Apple | Issue 1-3-1: in SSB case, we specify UE capability as # of SSB per frequency layer, which can be associated with different SSB index or PCI. We should reuse the similar approach to define UE capability to indicate maximum number of CSI-RS resources per MO/frequency layer with different CSI-RS indices and/or PCI. This capability may not be limited to per slot based. |
| CMCC | Issue 1-3-1: UE capability to indicate maximum number of CSI-RS resources in a slot per MO For option 1, we have one question for clarification. Considering the number of CSI-RS resource/beams to be monitored per layer/MO will be specified, as discussed in issue 1-2-3, do we need to introduce additional UE capability for to indicate the maximum number of CSI-RS resources per MO? |
| Huawei, HiSilicon | Issue 1-3-1: UE capability to indicate maximum number of CSI-RS resources in a slot per MO Support option 1. To address the comments from Apple and CMCC above and also ZTE R4-2004114, we would like to clarify that the capability in option 1 is about the maximum number of CSI-RS resources UE can measure per slot per MO. It is different from RAN4 requirements on the measurement capability in terms of number of CSI-RS resources per layer/MO (Issue 1-2-3). For example, with capability reporting UE indicates it can measure at most 8 CSI-RS resources per slot per MO, but the UE is still required to measure 32 CSI-RS resources per MO across different slots according to RAN4 requirements. It is also noted that for CLI measurement and PRS measurement, RAN1 has also defined per layer capability and per slot/window capability. For MTK comment above regarding the exiting capability maxNumberCSI-RS-RRM-RS-SINR, we understand it can stay as it is, and it would be up to UE to indicate the values for the two. Of course the reported number per UE cannot be smaller than the number per MO. Issue 1-3-2: UE buffering and processing capability Support option 1. Similar as for PRS measurement, the UE buffering and processing capability needs to be addressed. For MTK comments above, we are open to discussion on the time domain restriction in CSI-RS configuration, but we understand it is separate issue. The buffering and processing capability is on a smaller scale compared to the C-MTC (if defined), e.g. it will define what UE can buffer and process within the C-MTC window. |
| ZTE | Issue 1-3-1: We should first study the impact of UE capability maxNumberCSI-RS-RRM-RS-SINR to the number of CSI-RS resources UE shall be monitoring. E.g. if UE reports 4 for the maxNumberCSI-RS-RRM-RS-SINR and there are two frequency layers for CSI-RS resources. Then only 2 CSI-RS resources can be monitored in one slot on one frequency layer. Then how does UE meet the requirements of monitoring number of beams since the requirement are general and applicable to all UE. We need clarification on UE capability maxNumberCSI-RS-RRM-RS-SINR, which is for across frequency layers before we decide to introduce new UE capability per frequency layer. Issue 1-3-2: We need to understand the issue better before we introduce any UE capability. |
| Qualcomm | Issue 1-3-1: UE capability to indicate maximum number of CSI-RS resources in a slot per MO We support option3 to reuse this UE capability maxNumberCSI-RS-RRM-RS-SINR. Issue 1-3-2: UE buffering and processing capability FFS if a new UE capability shall be introduced to address the minimum separation in slots between consecutive two CSI-RS measurement sessions. We also think maxNumberCSI-RS-RRM-RS-SINR should already reflect a UE’s capability in the worst case where UE has to run the measurements every slot. |
| Nokia, Nokia Shanghai Bell | Issue 1-3-1: Agree with the tentative WF. We understood this issue is about number of CSI-RS resources per slot, different from the number of CSI-RS resources per MO/frequency layer in Issue 1-2-3. As maxNumberCSI-RS-RRM-RS-SINR also defines the number per slot. Would be good to send LS to RAN1/2 clarifying the possibility of reuse. Issue 1-3-2: Does existing number of CSI-RS resource per slot and/or per MO/frequency layer already reflect the buffering and processing capability? Why do we need these additional ones? |
| OPPO | Issue 1-3-1: UE capability to indicate maximum number of CSI-RS resources in a slot per MO We can support option 1. The comments by Huawei make sense to me that what we talk about here is the maximum number of CSI-RS resources UE can measure per slot per MO, different with the capability in issue 1-2-3. Issue 1-3-2: UE buffering and processing capability We also need to understand this issue more. Maybe we can de-prioritize it in this meeting. |

- Sub-topic 1-4: On CSI-RS resources configurations

| Company | Comments |
| --- | --- |
| MTK | Issue 1-4-1: How flexible CSI-RS resources per PCI per MO can be configured We can agree with Option 1 with all numbers TBD as a starting point. A time-domain structure like SMTC (of directly re-use SMTC) is also very important for RAN4 to finalize the remaining work of this WI. Otherwise, there will be too many scenarios to be considered. |
| Apple | Issue 1-4-1: similar comment as MTK. We should restrict CSI-RS configuration in time domain to make it align with existing measurement gap design and avoid too frequent scheduling restriction. |
| CATT | Question for clarification, introduction of CMTC is only for requirement defined in RAN4, or formally introduce CMTC definition? Currently, there is no CMTC configuration in CSI-RS for mobility measurement in 38.331. |
| Intel | Issue 1-4-1: It’s fine to have some restrictions about time-domain scheduling since the configuration of CSI-RS is too flexible. |
| Huawei, HiSilicon | Issue 1-4-1: How flexible CSI-RS resources per PCI per MO can be configured. We are open to discuss the concept of CMTC. We agree with MTK that all the parameters should be FFS at this stage. |
| ZTE | Issue 1-4-1: CSI-RS resources is different from SSB. If SMTC like window to be introduced for CSI-RS measurement it should be decided by RAN1 as for SMTC. From RRM requirements perspective, we could have some assumption on the CSI-RS configuration rather than introduce any new terminology. |
| Qualcomm | Issue 1-4-1: How flexible CSI-RS resources per PCI per MO can be configured We propose FFS or discussions on the need of introducing the CMTC configuration or reusing SMTC for scheduling the CSI-RS measurement especially when CSI-RS and SSB are associated. Please also note the introduction of new signaling could add tremendous amount of workload to finalize the WI. |
| Nokia, Nokia Shanghai Bell | Issue 1-4-1: Why to introduce CMTC for CSI-RS based measurement? CSI-RS can be configured at any time/frequency location in order to better reflect beam status. What is the benefit to apply such restriction? |
| OPPO | Issue 1-4-1: How flexible CSI-RS resources per PCI per MO can be configured Agree with MTK and Apple in principle. It is helpful to restrict CSI-RS configuration in time domain to make it align with existing measurement gap design and finalize the main work of this WI. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

- Sub-topic 1-1: Assumption and applicability

|  | Status summary |
| --- | --- |
| Sub-topic 1-1 | Tentative agreements: Candidate options: Issue 1-1-1: Applicability for CSI-RS based measurement capability - Option 1 (Qualcomm, MediaTek, OPPO, Apple, Intel, Nokia): - Requirements shall be defined when CSI-RS is configured with an associated SSB. - Option 2 (CMCC, CATT, ZTE, DOCOMO): - Specify requirements for the case with associatedSSB and the case without associatedSSB. Issue 1-1-2: Measurement capabilities per MO or per layer - Option 1(CATT, Nokia, ZTE, Apple): per frequency layer - Option 1a (CATT, Nokia): a frequency layer is identical to an MO - Option 1b (ZTE, Apple): One or multiple MOs can be one frequency layer. - Option 2(Huawei, MTK, Intel, Nokia): per MO - A frequency layer is identical to an MO - Option 3(Qualcomm, DOCOMO, OPPO): per MO and per frequency layer are the same - CSI-RS resources in the same MO should have the same center frequency, SCS and CP type. Recommendations for 2nd round: Issue 1-1-1: Wait for the conclusion of definition and MO configuration in email thread [124]. Continue to discuss CSI-RS resource configuration in issue 1-4-1. Issue 1-1-2: Depending on the definition of intra-/inter-frequency layer in email thread [124]. |

- Sub-topic 1-2: Requirements of measurement capabilities

| Sub-topic 1-2 | Tentative agreements: Candidate options: Issue 1-2-1: number of frequency layers to be monitored - The total number of carrier UE shall be capable of monitoring - Option 1(MediaTek, OPPO, Qualcomm, Apple, Nokia, ZTE): - at least 13 effective carrier frequency layers - Option 2(CATT, Huawei, CMCC): - at least 14 carriers of all RATs in total. - CSI-RS frequency layers - Option 1 (MediaTek, OPPO, Qualcomm): - UE shall be able to measure at least 7 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers. - Option 2 (CATT, Huawei, CMCC): - UE shall be able to measure at least [3] CSI-RS frequency layers. - UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers. - Option 3 (ZTE): - UE shall be able to measure at least [7] CSI-RS frequency layers Issue 1-2-2: number of cells to be monitored per layer/MO - Option 1 (MediaTek, OPPO, Qualcomm, Nokia): Shared capability for CSI-RS and SSB - Option 1a (MediaTek, OPPO): No need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB. - Option 1a (Qualcomm, Nokia): Number of monitored cells is determined by the UE capability based on SSB based measurements. - Option 2(Huawei, CATT, CMCC): Separated capability for CSI-RS - Option 2a (Huawei, CATT): - Re-use the SSB requirements for CSI-RS on number of cells UE shall monitor per layer. - Option 2b (CMCC): - For each intra-frequency layer, UE is capable of measuring [8] CSI-RS cell; - For each inter-frequency layer, UE is capable of measuring [4] CSI-RS cell. - Option 3(ZTE): Capabilities for CSI-RS only and CSI-RS&SSB - Option 3a (ZTE): - UE shall be capable of performing CSI-RS based measurements for at least [8] identified cells in FR1 for intra frequency measurement and at least [4] identified cells in FR1 for inter frequency measurement, at least [6] identified cells in FR2 for intra frequency measurement and at least [4] identified cells in FR2 for inter frequency measurement. - UE shall be capable of performing SSB and CSI-RS based measurements for at least [12] identified cells in FR1 for intra frequency measurement and at least [6] identified cells in FR1 for inter frequency measurement. - UE shall be capable of performing CSI-RS based measurements for at least [9] identified cells in FR2 for intra frequency measurement and at least [6] identified cells in FR2 for inter frequency measurement. Issue 1-2-3: number of CSI-RS resource/beams to be monitored per layer/MO - Option 1(CATT, Huawei, OPPO): UE shall monitor at least 32 CSI-RS resources per frequency layer - Option 2(ZTE): Define different UE capability for different scenarios, and number of CSI-RS resources shall be monitored by UE, - [24] CSI-RS resources for intra frequency measurements in FR1 - [48] CSI-RS resources for intra frequency measurements in FR2, - [16] CSI-RS resources for inter frequency measurements in FR1, - [24] CSI-RS resources for inter frequency measurements in FR2. - Option 3 (MediaTek): Requirements defined the same requirements as those for SSB. - If network configures more CSI-RS resources in an MO than the UE measurement capability, the UE behavior is undefined. - Option 4 (CMCC)： - For each intra-frequency layer, the number of CSI-RS resource is proposed to be [32]. - For each inter-frequency layer, the number of CSI-RS resource is proposed to be [24]. Recommendations for 2nd round: Continue discussion. |
| --- | --- |

- Sub-topic 1-3: Buffering and processing capability

| Sub-topic 1-3 | Tentative agreements: Candidate options: Issue 1-3-1: UE capability to indicate maximum number of CSI-RS resources in a slot per MO We also need to understand this issue more. Maybe we can de-prioritize it in this meeting. - Option 1(Huawei, MTK, OPPO): Introduce UE capability to indicate the maximum number of CSI-RS resources per MO in a slot. - Option 2(ZTE): Not to define UE capability to indicate maximum CSI-RS resources in a slot per MO. - Option 3(Qualcomm): The total number of CSI resources that UE can monitor should come from the UE capability maxNumberCSI-RS-RRM-RS-SINR. - FFS how to split up Issue 1-3-2: UE buffering and processing capability - Option 1: Introduce UE capability on the minimum separation between two slots with CSI-RS resources. - Option 2: Not to introduce UE capability on the minimum separation between two slots with CSI-RS resources. Recommendations for 2nd round: Continue discussion. |
| --- | --- |

- Sub-topic 1-4: On CSI-RS resources configurations

| Sub-topic 1-4 | Tentative agreements: Candidate options: - Option 1: (Apple) - A fixed channel bandwidth per MO should be configured - Up to [2] CSI-RS resources periodicities can be configured per intra-frequency MO - Up to [1] CSI-RS resource periodicity can be configured per inter-frequency MO - Introduce CSI-RS Measurement Timing Configuration (CMTC). - All CSI-RS resources for L3 meaurement should be configured within CMTC window - CMTC window duration: considering CSI-RS periodicity is up to 40ms, the CMTC window should be less than [5] ms. - Up to [2] CMTC periodicities can be configured per CSI-RS intra-frequency layer - Up to [1] CMTC periodicity can be configured per CSI-RS inter-frequency layer Recommendations for 2nd round: Continue discussion on restrictions of CSI-RS resource configuration, including CMTC and the values. |
| --- | --- |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | R4-2005357, WF on measurement capabilities and requirements CSI-RS based L3 measurement | OPPO |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

- Sub-topic 1-1: Assumption and applicability

Issue 1-1-1: Applicability for CSI-RS based measurement capability

  - Option 1 (Qualcomm, MediaTek, OPPO, Apple, Intel, Nokia):

    - Requirements shall be defined when CSI-RS is configured with an associated SSB.

  - Option 2 (CMCC, CATT, ZTE, DOCOMO):

    - Specify requirements for the case with associatedSSB and the case without associatedSSB.

Moderator: suggest to try to conclude this issue to move forward in this meeting.

| Company | Comments |
| --- | --- |
| OPPO | Prefer option 1 |
| CATT | We can compromise to option to move forward. |
| Intel | prefer option 1. Since there may be timing offset if associated SSB is not configured, and the measurement performance will degrade. |
| MTK | Prefer Option 1. Suggest to converge on this issue in this meeting so that RAN4 can start to discuss the requirements and CRs in May meeting. |
| Apple | Option 1 |
| Huawei, HiSilicon | Option 1. |
| Nokia, Nokia Shanghai Bell | We support Option1. |
| Qualcomm | We support Option 1. |
| ZTE | We can compromise to Requirements shall be defined only when CSI-RS is configured with an associated SSB in Rel-16 |
| Docomo | We still prefer option 2. Firstly, we think it is desirable to make less restriction on MO configuration for keeping configurability of CSI-RS. In addition, as CMCC mentioned in 1st discussion, if the UE belongs to synchronized network, there will be little timing difference between the serving cell and the target. Therefore, the UE will not need to acquire timing information from associatedSSB, in other words, associatedSSB will be not necessary for this case. This can result in quicker processing. Also, configuring associatedSSB is optional according to RAN2 specification, thus specifying requirements only for the case with associatedSSB seems to conflict to RAN2 specification. From the above reasons, option 2 is our preference. |

Issue 1-1-2: Measurement capabilities per MO or per layer

  - Option 1(CATT, Nokia, ZTE, Apple): per frequency layer

    - Option 1a (CATT, Nokia): a frequency layer is identical to an MO

    - Option 1b (ZTE, Apple): One or multiple MOs can be one frequency layer.

  - Option 2(Huawei, MTK, Intel, Nokia): per MO

    - A frequency layer is identical to an MO

  - Option 3(Qualcomm, DOCOMO, OPPO): per MO and per frequency layer are the same

    - CSI-RS resources in the same MO should have the same center frequency, SCS and CP type.

Moderator: suggest to try to conclude this issue in this meeting. Maybe we can wait for the conclusion on definition of intra-/inter-frequency layer in GTW session.

| Company | Comments |
| --- | --- |
| OPPO | Prefer option 3 |
| CATT | Option 3 |
| Intel | Option 3. |
| MTK | OK to Option 3 |
| Apple | OK with option3. It can be further clarified as single MO can be configured per frequency layer. |
| Huawei, HiSilicon | It seems to us option 1a, option 2 and option 3 are identical. If this is the case we are also ok with option 3. |
| Nokia, Nokia Shanghai Bell | We understood Option 1a and Option 3 are the same? According to RAN1, a MO is identical to a frequency layer. So there could be multiple MOs sharing the same center frequency, i.e. multiple intra-f frequency layers. But for SSB-based measurement, frequency layer includes all the MOs on the center frequency. We may send LS to RAN1 clarifying their intention to define frequency layer as one MO for CSI-RS based measurement? |
| Qualcomm | We support Option 3. Further according to the GTW meeting agreement on the intra-frequency definition, shall we further discuss the case when two or more frequency layers/MOs can share the same centre frequency? This is in line with the question raised by Nokia. |
| ZTE | Option 1b. Multiple MOs with same center frequency is allowed at least by RAN2 spec. If one MO is one frequency layer, then there may be only one or two cells can be configured for one frequency layer. The CSI-RS based L3 mobility doesn’t work at all. |
| Docomo | Option 3. |

- Sub-topic 1-2: Requirements of measurement capabilities

Issue 1-2-1: number of frequency layers to be monitored

- The total number of carrier UE shall be capable of monitoring

  - Option 1(MediaTek, OPPO, Qualcomm, Apple, Nokia, ZTE):

    - at least 13 effective carrier frequency layers

  - Option 2(CATT, Huawei, CMCC):

    - at least 14 carriers of all RATs in total.

- CSI-RS frequency layers

  - Option 1 (MediaTek, OPPO, Qualcomm):

    - UE shall be able to measure at least 7 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers.

  - Option 2 (CATT, Huawei, CMCC):

    - UE shall be able to measure at least [3] CSI-RS frequency layers.

    - UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers.

  - Option 3 (ZTE):

    - UE shall be able to measure at least [7] CSI-RS frequency layers

| Company | Comments |
| --- | --- |
| OPPO | For the total number of carrier, we suggest to keep the same as Rel-15. For CSI-RS frequency layers, we still propose to share the capability with SSB, assuming that associated SSB should be configured for CSI-RS L3 measurement. To Huawei: yes, I share the same understanding with you. To Qualcomm: in our view, it means CSI-RS based NR inter-frequency layers. |
| CATT | For the total number of carrier, ok with option 1. For CSI-RS carrier, OK with option 1. |
| MTK | For total number, keep the same as Rel-15 For CSI-RS carrier, Option 1, given that associated SSB is configured. |
| Apple | Option 1 for both sub-issues |
| Huawei, HiSilicon | For total number of layers, we can compromise to option 1. For number of CSI-RS layers, we would like to clarify the meaning of option 1. Could moderator or proponent of option 1 clarify if our understanding below is correct? UE is required to measure at least 7 NR inter-frequency layers, which can be - 0 SSB layer + 7 CSI-RS layers, or - 1 SSB layer + 6 CSI-RS layers, or - … - 7 SSB layers + 0 CSI-RS layer. If this is the case we can also compromise to option 1. |
| Nokia, Nokia Shanghai Bell | We prefer keeping same number as SSB-based measurement capability, assuming “frequency layer” here refers to the center frequency. If we only define the requirements when associatedSSB is configured, the UE shall detect the associated SSB before performing CSI-RS based measurement. The total number of carriers a UE shall at least measure remains the same. |
| Qualcomm | We support Option 1 for “The total number of carrier UE shall be capable of monitoring”; But we have a question on the concept of “CSI-RS frequency layers” if Moderator can help to elaborate? @Moderator |
| ZTE | Option 1. Clarification on how the total number are distributed between SSB based measurements and CSI-RS based measurements are necessary. |

Issue 1-2-2: number of cells to be monitored per layer/MO

- Option 1 (MediaTek, OPPO, Apple, Qualcomm, Nokia): Shared capability for CSI-RS and SSB

  - Option 1a (MediaTek, OPPO): No need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB.

  - Option 1a (Qualcomm, Nokia): Number of monitored cells is determined by the UE capability based on SSB based measurements.

- Option 2(Huawei, CATT, CMCC):  Separated capability for CSI-RS

  - Option 2a (Huawei, CATT, ZTE):

    - Re-use the SSB requirements for CSI-RS on number of cells UE shall monitor per layer.

  - Option 2b (CMCC, ZTE):

    - For each intra-frequency layer, UE is capable of measuring [8] CSI-RS cell;

    - For each inter-frequency layer, UE is capable of measuring [4] CSI-RS cell.

- Option 3(ZTE): Capabilities for CSI-RS only and CSI-RS&SSB

  - Option 3a (ZTE):

    - UE shall be capable of performing CSI-RS based measurements for at least [8] identified cells in FR1 for intra frequency measurement and at least [4] identified cells in FR1 for inter frequency measurement, at least [6] identified cells in FR2 for intra frequency measurement and at least [4] identified cells in FR2 for inter frequency measurement.

    - UE shall be capable of performing SSB and CSI-RS based measurements for at least [12] identified cells in FR1 for intra frequency measurement and at least [6] identified cells in FR1 for inter frequency measurement.

    - UE shall be capable of performing CSI-RS based measurements for at least [9] identified cells in FR2 for intra frequency measurement and at least [6] identified cells in FR2 for inter frequency measurement.

| Company | Comments |
| --- | --- |
| OPPO | Similar to issue 1-2-1, we support option 1 using shared capability for CSI-RS and SSB. |
| CATT | For number of cells to be monitored, we think the separate capability shall be defined. |
| MTK | Option 1. All cells to be measured are based on the cell search results from SSB. |
| Apple | Option1 |
| Huawei, HiSilicon | Option 2a. Not sure if option 2a is same as option 1. Response to MTK comments in the 1st round for clarification, for the case with associated SSB, with option 2a, it will be the same 8 cells UE measures for SSB and CSI-RS. In our view, we need to define separate capabilities for SSB and CSI-RS because they are different measurements even they are performed on the same set of cells. |
| Nokia, Nokia Shanghai Bell | We prefer Option1 i.e. same as the value defined for SSB-based measurement. |
| Qualcomm | We support Option 1. The two sub-options seem the same to us. |
| ZTE | Agree with Huawei that the capabilities for SSB based measurement and CSI-RS based measurement should be separated. Option 3 and option 2a, 2b are similar. We can support option 2a and 2b |

Issue 1-2-3: number of CSI-RS resource/beams to be monitored per layer/MO

- Option 1(CATT): UE shall monitor at least 32 CSI-RS resources per frequency layer

- Option 2(ZTE): Define different UE capability for different scenarios, and number of CSI-RS resources shall be monitored by UE,

  - [24] CSI-RS resources for intra frequency measurements in FR1

  - [48] CSI-RS resources for intra frequency measurements in FR2,

  - [16] CSI-RS resources for inter frequency measurements in FR1,

  - [24] CSI-RS resources for inter frequency measurements in FR2.

- Option 3 (MediaTek, Apple, OPPO): Requirements defined the same requirements as those for SSB.

  - If network configures more CSI-RS resources in an MO than the UE measurement capability, the UE behavior is undefined.

- Option 4 (CMCC, Huawei, ZTE)：

  - For each intra-frequency layer, the number of CSI-RS resource is proposed to be [32].

  - For each inter-frequency layer, the number of CSI-RS resource is proposed to be [24].

| Company | Comments |
| --- | --- |
| OPPO | We can compromise to option 3 requirements defined the same requirements as those for SSB |
| CATT | Option 1 |
| MTK | Option 3. Need to see convincing analysis before agreeing on larger numbers. |
| Apple | Support option 3. For FR1, 14 and 7 CSI-RS resources for intra-f and inter-f, respectively. For FR2, 24 and 10 CSI-RS resources for inra- and inter-frequency, respectively and at least 1 CSI-RS resources per cell. |
| Huawei, HiSilicon | Option 4. |
| Nokia, Nokia Shanghai Bell | This depends on how we define a frequency layer. We think the starting point could be the existing number for SSB-based measurement. We’d like to know why a higher number is required for CSI-RS based measurement and how the higher numbers are derived? |
| Qualcomm | FFS. |
| ZTE | Option 2. We also support option 4. |

- Sub-topic 1-3: Buffering and processing capability

Issue 1-3-1: UE capability to indicate maximum number of CSI-RS resources in a slot per MO

- Option 1(Huawei, MTK, OPPO): Introduce UE capability to indicate the maximum number of CSI-RS resources per MO in a slot.

- Option 2(ZTE, Nokia): Not to define UE capability to indicate maximum CSI-RS resources in a slot per MO.

- Option 3(Qualcomm, Apple, Huawei, Nokia): The total number of CSI resources that UE can monitor should come from the UE capability maxNumberCSI-RS-RRM-RS-SINR.

  - FFS how to split up

| Company | Comments |
| --- | --- |
| OPPO | Option 1 |
| MTK | Option 1. Since the detail is FFS at this moment, maybe we can postpone this issue to next meeting. |
| Apple | Option 3. It will depend on the situation when multiple MO collide in the same slot. Even we restrict single MO per frequency layer, there is case intra-frequency and inter-frequency without gap mesaurements can happen in the same slot. So, it is hard to define the maximum number per MO. |
| Huawei, HiSilicon | We can compromise to option 3, i.e. to work with existing capability. |
| Nokia, Nokia Shanghai Bell | We are fine with Option2 and Option3. In Issue 1-2-3, we are defining the number of CSI-RS resources per MO/frequency layer. According to the definition for SSB-based measurement, it refers to the number the UE shall be able of measure “within each L1 measurement period”, where the time period is already reflected. In addition, maxNumberCSI-RS-RRM-RS-SINR also defines the number per slot. Would be good to send LS to RAN1/2 clarifying the possibility of reusing it. Anyway, it is not clear why we need define additional capability per slot. |
| Qualcomm | We support Option 3 |
| ZTE | Option 2. For option 3, the impact to number of beams UE shall monitor as in Issue 1-2-3 should be studied and clarification from RAN1 are needed. |

Issue 1-3-2:  UE buffering and processing capability

- Option 1: Introduce UE capability on the minimum separation between two slots with CSI-RS resources.

- Option 2: Not to introduce UE capability on the minimum separation between two slots with CSI-RS resources.

| Company | Comments |
| --- | --- |
| OPPO | We need more time to understand it and suggest to further discuss this issue in next meeting. |
| MTK | Since the detail is FFS at this moment, maybe we can postpone this issue to next meeting. |
| Huawei, HiSilicon | We can have further discussion next meeting but we would like to share our thinking on this capability. In our understanding, the existing capability maxNumberCSI-RS-RRM-RS-SINR provides the information for the case where UE is configured to measure in every slot. The proposed capability allows higher flexibility for both network and UE, e.g. network can configure more CSI-RS resources per slot than indicated by maxNumberCSI-RS-RRM-RS-SINR, and UE can still measure the resources as long as there is enough time to the next slot with CSI-RS resource. |
| Nokia, Nokia Shanghai Bell | We don’t see the reason to do so right now. Could further discuss it next meeting. |
| Qualcomm | FFS whether it is needed to define a new capability for addressing this. In fact Issue 1-3-1 should be answered assuming every slot has an maxNumberCSI-RS-RRM-RS-SINR of CSI-RS resources to measure. |
| ZTE | FFS |

- Sub-topic 1-4: On CSI-RS resources configurations

Issue 1-4-1: How flexible CSI-RS resources per PCI per MO can be configured.

- Option 1: (Apple)

  - A fixed channel bandwidth per MO should be configured

  - Up to [2] CSI-RS resources periodicities can be configured per intra-frequency MO

  - Up to [1] CSI-RS resource periodicity can be configured per inter-frequency MO

  - Introduce CSI-RS Measurement Timing Configuration (CMTC).

    - All CSI-RS resources for L3 meaurement should be configured within CMTC window

    - CMTC window duration: considering CSI-RS periodicity is up to 40ms, the CMTC window should be less than [5] ms.

    - Up to [2] CMTC periodicities can be configured per CSI-RS intra-frequency layer

    - Up to [1] CMTC periodicity can be configured per CSI-RS inter-frequency layer

Moderator suggests to focus on

- How to restrict of CSI-RS resource configuration on time-domain

- Whether to introduce CMTC

| Company | Comments |
| --- | --- |
| OPPO | Depending on the conclusion of email thread [124]. We need more time to understand it and suggest to further discuss this issue in next meeting. Consider a simple way to mandate the CSI-RS resource to occur only within the SMTC of the same MO.（proposed by MTK R4-2003644） |
| CATT | The requirement can be defined assuming the CSI-RS configuration in time domain follows the proposal from Apple. We think it is not necessary to restrict the CSI-RS resources to occur only with the SMTC duration in the same MO. |
| Intel | Since the configuration of CSI-RS is too flexible, some restrictions about time-domain scheduling is preferred, which can make the WI work first. |
| MTK | Support Option 1, but have to change all numbers to TBD. |
| Apple | Option 1. One issue to restrict CSI-RS within SMTC is CSI-RS resources and SSB have different set of periodicity. Aligning between them means the restriction for SMTC configuration. |
| Huawei, HiSilicon | In principle we are fine to have the concept of C-MTC, but all the parameters should be FFS for now. |
| Nokia, Nokia Shanghai Bell | We would leave it FFS. Why to introduce CMTC for CSI-RS based measurement? CSI-RS can be configured at any time/frequency location in order to better reflect beam status. What is the benefit to apply such restriction? |
| Qualcomm | We prefer not to introduce CMTC for Rel-16 due to workload. |
| ZTE | We should focus on how to restrict CSI-RS configuration in time domain in the RRM requirements. CMTC is not necessary in Rel-16. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

- Sub-topic 1-1: Assumption and applicability

Issue 1-1-1: Applicability for CSI-RS based measurement capability

  - Option 1 (Qualcomm, MediaTek, OPPO, Apple, Intel, Nokia, CATT, ZTE):

    - Requirements shall be defined when CSI-RS is configured with an associated SSB in Rel-16.

  - Option 2 (DOCOMO):

    - Specify requirements for the case with associatedSSB and the case without associatedSSB.

Recommended WF: Adopt Option 1 as majority view to move forwards.

Tentative agreement:

- Requirements shall be defined when CSI-RS is configured with an associated SSB in Rel-16.

  - No requirements in Rel-16 for the case associatedSSB is not configured for CSI-RS

Issue 1-1-2: Measurement capabilities per MO or per layer

  - Option 1(Nokia, ZTE): per frequency layer

    - Option 1a (Nokia): a frequency layer is identical to an MO

    - Option 1b (ZTE): One or multiple MOs can be one frequency layer.

  - Option 2: per MO

    - A frequency layer is identical to an MO

  - Option 3(Qualcomm, OPPO, Intel, CATT, MTK, Huawei, Apple, DOCOMO): per MO and per frequency layer are the same

    - CSI-RS resources in the same MO should have the same center frequency, SCS and CP type.

Recommended WF:  No consensus. Need decision in next meeting.

- Sub-topic 1-2: Requirements of measurement capabilities

Issue 1-2-1: number of frequency layers to be monitored

- The total number of carrier UE shall be capable of monitoring

  - Option 1(MediaTek, OPPO,CATT, Qualcomm, Apple, Nokia, ZTE):

    - at least 13 effective carrier frequency layers

  - Option 2(CMCC):

    - at least 14 carriers of all RATs in total.

- CSI-RS frequency layers

  - Option 1 (MediaTek, OPPO,CATT, Qualcomm, Apple, Nokia, ZTE):

    - UE shall be able to measure at least 7 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers.

  - Option 2 (CMCC):

    - UE shall be able to measure at least [3] CSI-RS frequency layers.

    - UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers.

Recommended WF:

Tentative agreement:

- The total number of carrier UE shall be capable of monitoring at least 13 effective carrier frequency layers.

FFS: CSI-RS frequency layers

Issue 1-2-2: number of cells to be monitored per layer/MO

- Option 1 (MediaTek, OPPO, Apple, Qualcomm, Nokia): Shared capability for CSI-RS and SSB

- Option 2(Huawei, CATT, CMCC):  Separated capability for CSI-RS

  - Option 2a (Huawei, CATT, ZTE)

  - Option 2b (CMCC, ZTE)

- Option 3(ZTE): Capabilities for CSI-RS only and CSI-RS&SSB

Recommended WF:  No consensus. Need decision in next meeting.

Issue 1-2-3: number of CSI-RS resource/beams to be monitored per layer/MO

- Option 1(CATT)

- Option 2(ZTE)

- Option 3 (MediaTek, Apple, OPPO)

- Option 4 (CMCC, Huawei, ZTE)

Recommended WF:  No consensus. Need decision in next meeting.

- Sub-topic 1-3: Buffering and processing capability

Issue 1-3-1: UE capability to indicate maximum number of CSI-RS resources in a slot per MO

- Option 1(Huawei, MTK, OPPO): Introduce UE capability to indicate the maximum number of CSI-RS resources per MO in a slot.

- Option 2(ZTE, Nokia): Not to define UE capability to indicate maximum CSI-RS resources in a slot per MO.

- Option 3(Qualcomm, Apple, Huawei, Nokia): The total number of CSI resources that UE can monitor should come from the UE capability maxNumberCSI-RS-RRM-RS-SINR.

  - FFS how to split up

Recommended WF:  No consensus. Need decision in next meeting.

Issue 1-3-2:  UE buffering and processing capability

- Option 1: Introduce UE capability on the minimum separation between two slots with CSI-RS resources.

- Option 2: Not to introduce UE capability on the minimum separation between two slots with CSI-RS resources.

Recommended WF:  No consensus.

- Sub-topic 1-4: On CSI-RS resources configurations

Issue 1-4-1: How flexible CSI-RS resources per PCI per MO can be configured.

Recommended WF:  More discussion based option 1 by Apple with all value TBD in next meeting.

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2005357 | Agreeable |

Topic #2: Measurement requirements for CSI-RS intra-frequency and inter-frequency measurements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003226 | LG Electronics Inc. | - Proposal 1: Consider case that UE measures the CSI-RS resource with deriveSSB-IndexFromCell when associatedSSB is not detected. - Proposal 2: Define the CSI-RS measurement requirement for Case 1 and Case 2 with tight synchronization level between serving and neighbour cell. - Proposal 3: Re-use the principle of SSB based L3-measurement for scaling factor with RX beam sweeping ~8. - Proposal 4: For Case 3-1, since the UE waits the SSB beam sweeping to end, measurement period time should take into account waiting time. - Proposal 5: RAN4 should clarify collision between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell. - Alt 1: Do not define CSI-RS measurement requirements for the collision case. - Alt 2: Network should configure L1 measurement resource to avoid collision with CSI-RS L3 measurement resource of neighbour cell. - Proposal 6: Define scheduling restriction on one data symbol before and after CSI-RS symbol to be measured due to Rx beam sweeping. - Proposal 7: Do not define scheduling restriction if the timing difference between serving and neighbor cell including cell phase synchronization is guaranteed to be less then CP length. |
| R4-2003261 | CATT | Proposal 1: The CSI-RS measurement delay requirements shall be defined for both associated SSB case and non-associated SSB case. Proposal 2: For CSI-RS based measurement requirement, the following scenarios are prioritized to be defined in Rel-16: - Intra-frequency measurement without gap - Inter-frequency measurement with gap Proposal 3: If associated SSB is not configured for CSI-RS resources, the CSI-RS identification time can be expressed as: TCSI-RS_identification_without_associatedSSB = TCSI-RS_measurement_period. Proposal 4: If associated SSB is configured for CSI-RS resources, the CSI-RS identification time can be expressed as: TCSI-RS_identification_with_associatedSSB = TPSS/SSS_sync + TCSI-RS_measurement_period. Proposal 5: If the CSI-RS resource is configured with associated SSB and QCL-ed to the associated SSB, no Rx sweeping is needed only after SSB has been detected. Proposal 6: It is proposed to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell. Proposal 7: the scheduling restriction for CSI-RS based measurement shall be introduced for the following cases: - Mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell - RX beam sweeping in FR2 - Collision between UL transmission and DL measurement for TDD carrier Proposal 8: If additional dedicated searcher is assumed for CSI-RS measurement, no impact on existing CSSF defined for SSB based measurement specified in 38.133. Otherwise, the CSSFs for FR1/FR2 SCC shall be updated by considering the CSI-RS based intra-frequency and inter-frequency measurement without gap and within gap respectively. Proposal 9: For CSI-RS based measurement, the following options can be considered to define the corresponding requirements: - Option 1: UE is allowed not to measure the CSI-RS resources that are not within DRX on-duration or measurement gap; - Option 2: No UE performance requirement is defined for the CSI-RS resources that are not within DRX on-duration or measurement gap; - Option 3: the network needs to make sure the configured CSI-RS resources falls within the configured measurement gaps |
| R4-2003484 | NTT DOCOMO, INC. | Observation 1: If associatedSSB is not configured, UE can determine CSI-RS reception timing based on serving cell indicated by refServCellIndex or PCell. Observation 2: According to RAN2 specification, the SCS and the center frequency should be same between the CSI-RS resources in the same MO, but the BW can differ from each other. Proposal 1: MRTD value for intra-band CA can be reused for synchronization level for CSI-RS intra-frequency measurement regardless of associatedSSB. Proposal 2: UE shall base the timing on its serving cell, and the requirement needs at least consider the CSI-RS measurement time if associatedSSB is not configured. Proposal 3: RRM requirements should be specified based on the configuration of associatedSSB, e.g., if both ssbIndex and isQuasiColocated are provided, Rx beam sweeping is not needed. Proposal 4: Define RRM requirements for Case 2 :all CSI-RS resources in the same MO have the same center frequency, SCS, CP type configured. BW can be different. |
| R4-2003646 | MediaTek inc. | Observation 1: According to TS38.331, if associatedSSB is configured, the timing (SFN and slot) reference of the CSI-RS follows the target cell. - If deriveSSB-IndexFromCell is not indicated, UE has to decode the PBCH of the target cell to get SFN, frame boundary and symbol-level timing. - If deriveSSB-IndexFromCell is indicated, UE can resolve the remaining symbol-level timing ambiguity of deriveSSB-IndexFromCell through the timing estimated from SSB. Observation 2: If associatedSSB is not configured, UE has to measure up to 96 CSI-RS resources per MO. This is a huge waste of UE’s computation power. Observation 3: The purpose of PBCH decoding is to acquire the frame and slot timing of the target cell. Observation 4: Without proper limitation on timing domain configuration, CSSF will be RS-specific, which contradicts to previous RAN4 agreement. Proposal 1: RAN4 does not specify requirements for the case that associatedSSB is not configured. Proposal 2: For PBCH, 5 samples are needed to guarantee >90% detection rate at SNR -6dB. If UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated, then UE can skip PBCH decoding. Proposal 3: For inter-frequency CSI-RS measurement, at least additional [3] AGC samples are needed. Proposal 4: The detail requirements for CSSF is pending on the conclusion of intra and inter-frequency definition. Proposal 5: RAN4 to consider some time-domain limitation on CSI-RS configurations in the same MO to avoid extension of existing SSB measurement delay as well as reduce RAN4 workload and UE scheduling complexity. Proposal 6: The FFT window timing always follows the serving cell timing for intra frequency measurement Proposal 7: The scheduling restriction on the additional OFDM symbols before and after CSI-RS is not needed. Proposal 8: RAN4 to discuss in the performance part on how to address the degraded measurement accuracy for those CSI-RS which comes with large timing misalignment to UE’s FFT window timing. |
| R4-2003809 | Nokia, Nokia Shanghai Bell | Proposal1: The restriction of all CSI-RS resources in the same MO having the same bandwidth does not bring any benefit. Proposal2: For intra-frequency measurement, the requirement is defined when all the measured CSI-RS resources in one MO are within the active BWP. Proposal3: It can be assumed maximum one MO is configured per center frequency and SCS. Proposal4: Do not define the UE requirements when associatedSSB is not configured in Rel16. |
| R4-2003985 | OPPO | Observation 1: The synchronization between serving and target cell is difficult to be guaranteed in all networks. Proposal 1: If associatedSSB is not configured, no requirements is specified for CSI-RS L3 measurement Proposal 2: If associated SSB is configured for CSI-RS resources, CSI-RS based cell identification can consider: 1) Cell search via SSB, 2) PBCH decoding and 3) CSI-RS measurement. Proposal 3: Study the impact of AGC control and if any define AGC margin additionally. Observation 2: From specification’s perspective, the structure of CSI-RS L3 measurement can be more concise than those of SSB. Proposal 4: Intra-frequency measurement can all be configured without gaps if the definition of intra-frequency includes the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of the UE. Otherwise, all others are inter-frequency measurement with gaps. Proposal 5: RAN4 to re-use the existing requirement of CSSF as much as possible, and the framework of CSSF can be shared by SSB and CSI-RS based L3 measurement. Proposal 6: RAN4 to consider the collision with UL transmission and DL measurement for TDD band on FR1, and Rx beam sweeping in FR2, for scheduling restriction of CSI-RS based L3 measurement without gaps. Proposal 7: If CSI-RS based L3 measurement is configured without gaps, the requirements of scheduling restriction can be chosen from two options: - Option 1: the scheduling restriction on the additional OFDM symbols before and after SSB is not needed for CSI-RS based L3 measurement. - Option 2: define scheduling restriction on CSI-RS to be measurement and additional X OFDM symbol before and after consecutive CSI-RS symbols to be measured, FFS on X value. Proposal 8: If CSI-RS is configured with associated SSB and QCL-ed to the associated SSB, no additional scaling factor for RX beam sweeping should not be considered for CSI-RS L3 measurement. Proposal 9: If CSI-RS configured with associated SSB but not QCL-ed to the associated SSB, Rx sweeping is needed and FFS on the scaling factor N. |
| R4-2004292 | Huawei, Hisilicon | Proposal 1: CSI-RS measurement requirement is the CSI-RS measurement periodicity when associatedSSB is not configured assuming that the timing between serving cell (which is timing reference cell) and the neighbour cell is frame boundary synchronized. Proposal2: - If the MO includes the serving CSI-RS resource and associated SSB, for the cells in the same MO, - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO. - For TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO. - If the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured, UE needs to perform PSS/SSS detection, DMRS matching and PBCH decoding to acquire SFN, frame boundary timing, slot timing and symbol level timing of one cell. For the other cells in the same MO, - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO. - For FR1 TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO. Proposal 3: AGC adjustment time shall be considered when UE needs to retune RF to an inter-frequency layer to perform measurement. Proposal 4: If CSI-RS is not QCL-ed to the associated SSB, UE needs to sweep the RX beam. If the CSI-RS is QCL-ed to the associated SSB, no Rx sweeping is needed. Proposal 5: If a UE is configured with both CSI-RS-Resource-Mobility and ssb-ConfigMobility in one MO, the CSSF calculation shall consider SSB MO and CSI-RS MO. Proposal 6: If a UE is configured with one MO with CSI-RS-Resource-Mobility with associatedSSB and another MO with ssb-ConfigMobility, CSSF calculation shall consider 2 SSB MOs and 1 CSI-RS MO. Proposal 7: If UE is not able to support mixed numerology of data and CSI-RS L3 mobility, the following scheduling restrictions apply due to CSI-RS based L3 intra-frequency measurement: -if the associatedSSB is configured, UE is not expected to transmit or receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured. -if the associatedSSB is not configured, UE is not expected to transmit or receive on the data OFDM symbol impacted by the CSI-RS resource symbol to be measured, provided timing difference between the CSI-RS resource and the serving cell should be less than half CP corresponding to the SCS of the CSI-RS. Proposal 8: When UE performs CSI-RS intra-frequency measurements in a TDD band, UE is not expected to transmit and receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured. Proposal 9: Scheduling restriction shall be considered when UE performs RX beam sweeping. |
| R4-2004826 | Qualcomm | Proposal 1: RAN4 to define requirements for CSI-RS based measurements for the following configuration only: 48 PRB’s and density D =3. Proposal 2: All CSI-RS resources in the same MO have the same center frequency, BW, SCS, CP type configured Proposal 2a: CSI-RS measurements to be considered intra-frequency if the CSI-RS of neighbor cell has the same frequency, bandwidth, SCS and CP duration as that of serving cell and the CSI-RS lies within the active BWP. Proposal 2b: All CSI-RS measurements that don’t fall under proposal 2a are considered inter-frequency. Proposal 3: Requirements shall be defined when CSI-RS is configured with an associated SSB within the MO. Proposal 3a: Number of monitored cells is determined by the UE capability based on SSB based measurements. Proposal 3b: “The total number of CSI resources that UE can monitor should come from the UE capability maxNumberCSI-RS-RRM-RS-SINR.” [3] Proposal 4: Performance requirements should also be defined based the assumption that UE uses a single FFT window for processing all CSI-RSs on a single frequency layer. Proposal 5: The UE will need GAPs for CSI-RS L3 measurements if - The SCS of CSI-RS is different from active BWP - The CSI-RS is not fully confined within the active BWP - The CP of cells to be measured is different from that of active BWP (60 kHz SCS only) Proposal 6: The tuning time for CSI-RS based measurements that are outside UE’s active BWP will be same as that for BWP switch. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

RAN4 to discuss whether/how to define measurement requirements for Case 1 and Case 2.

- Case 1: if associatedSSB is not configured

- Case 2: if associatedSSB is configured for CSI-RS

The following issues should be discussed in RAN4:

- CSSF

- Scaling factor for RX beam sweeping

- Scheduling restriction

- Restrict for CSI-RS resources outside of DRX/MG duration

- Note: Others are not precluded.

Sub-topic 2-1: Case 1- if associatedSSB is not configured

Sub-topic description

- The synchronization level between serving and target cell needs to be clarified in RAN4.

  - FFS whether to introduce requirements of synchronization level for intra-frequency and inter-frequency measurement

- Measurement requirements :

  - Option 1: No requirements specified for CSI-RS L3 measurement

  - Option 2: CSI-RS identification time is the CSI-RS measurement periodicity

  - Option 2a: If sufficient synchronization of severing cell and neighbor cell can be guaranteed, CSI-RS identification time is the CSI-RS measurement periodicity

  - Option 2b: UE shall base the timing on its serving cell, and the requirement needs at least consider the CSI-RS measurement time.

Open issues and candidate options before e-meeting:

Issue 2-1-1: Measurement requirements

- Proposals

  - Option 1 (OPPO, Qualcomm, MediaTek, Nokia): No requirements in Rel-16.

  - Option 2 (DOCOMO): UE shall base the timing on its serving cell (indicated by refServCellIndex or PCell), and the requirement needs at least consider the CSI-RS measurement time if associatedSSB is not configured.

  - Option 3 (CATT): CSI-RS identification time is the CSI-RS measurement periodicity

  - Option 4 (Huawei): CSI-RS measurement requirement is the CSI-RS measurement periodicity when associatedSSB is not configured assuming that the timing between serving cell (which is timing reference cell) and the neighbour cell is frame boundary synchronized.

- Recommended WF

  - Tentative agreement: Option 1

Sub-topic 2-2: Case 2- if associatedSSB is configured for CSI-RS

Sub-topic description

- Measurement requirements ：

  - CSI-RS based cell identification can consider:

    - 1) Cell search via SSB

    - 2) PBCH decoding

    - 3) CSI-RS measurement,

    - AGC margin should be considered as well.

- FFS the case if the MO includes the serving CSI-RS resource with associated SSB

- FFS the case if the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured

Open issues and candidate options before e-meeting:

Issue 2-2-1: General measurement requirements

- Proposals

  - Option 1 (OPPO, MediTek): CSI-RS based cell identification can consider

    - 1) Cell search via SSB, 2) PBCH decoding and 3) CSI-RS measurement.

  - Option 1a (MediaTek):

    - For PBCH decoding, 5 samples are needed to guarantee >90% detection rate at SNR -6dB.

    - If UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated, then UE can skip PBCH decoding.

  - Option 2 (CATT):

    - If associated SSB is configured for CSI-RS resources, the CSI-RS identification time can be expressed as: TCSI-RS_identification_with_associatedSSB = TPSS/SSS_sync + TCSI-RS_measurement_period.

  - Option 3 (Huawei):

- If the MO includes the serving CSI-RS resource with associated SSB, for the cells in the same MO,

  - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO.

  - For TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO.

- If the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured, UE needs to perform PSS/SSS detection, DMRS matching and PBCH decoding to acquire SFN, frame boundary timing, slot timing and symbol level timing of one cell. For the other cells in the same MO,

  - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO.

  - For FR1 TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO.

  - Option 4 (LGE):

- Consider case that UE measures the CSI-RS resource with deriveSSB-IndexFromCell when associatedSSB is not detected.

- Recommended WF

  - Tentative agreement:

    - CSI-RS based cell identification can consider: 1) Cell search via SSB, 2) PBCH decoding and 3) CSI-RS measurement.

    - If UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated, then UE can skip PBCH decoding.

    - FFS on the value of delay

Issue 2-2-2: AGC

- Proposals

  - Option 1 (MediaTek):

    - For inter-frequency CSI-RS measurement, at least additional [3] AGC samples are needed.

  - Option 2 (Huawei):

    - AGC adjustment time shall be considered when UE needs to retune RF to an inter-frequency layer to perform measurement.

  - Option 3 (Qualcomm):

    - The tuning time for CSI-RS based measurements that are outside UE’s active BWP will be same as that for BWP switch.

- Recommended WF

  - Comment understanding: AGC adjustment time shall be needed when UE needs to retune RF

  - FFS on when UE needs to retune RF

  - FFS the value of AGC adjustment time

Sub-topic 2-3: Requirement of Measurement Gap

Sub-topic description

How to define requirements for intra-frequency and inter-frequency measurement with or w/o gaps.

Open issues and candidate options before e-meeting:

Issue 2-3-1: intra-frequency and inter-frequency measurement with or w/o gaps?

- Proposals

  - Option 1(Qualcomm): The UE will need GAPs for CSI-RS L3 measurements if

    - The SCS of CSI-RS is different from active BWP

    - The CSI-RS is not fully confined within the active BWP

    - The CP of cells to be measured is different from that of active BWP (60 kHz SCS only)

    - The tuning time for CSI-RS based measurements that are outside UE’s active BWP will be same as that for BWP switch.

  - Option 2 (OPPO):

    - Intra-frequency measurement can all be configured without gaps if the definition of intra-frequency includes the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of the UE.

    - Otherwise, all others are inter-frequency measurement with gaps.

  - Option 3 (CATT):：For CSI-RS based measurement requirement, the following scenarios are prioritized to be defined in Rel-16:

    - Intra-frequency measurement without gap

    - Inter-frequency measurement with gap

  - Option 4 (Nokia): For intra-frequency measurement, the requirement is defined when all the measured CSI-RS resources in one MO are within the active BWP.

- Recommended WF

  - Pending on the conclusion of intra and inter-frequency definition.

  - Tentative agreement: Option 3

    - Intra-frequency measurement without gap

    - Inter-frequency measurement with gap

Sub-topic 2-4: Scaling Factor

Sub-topic description

CSSF and Scaling factor for RX beam sweeping are to be discussed in this section.

Open issues and candidate options before e-meeting:

Issue 2-4-1: CSSF

- Proposals

  - Option 1(MediaTek):

    - Without proper limitation on timing domain configuration, CSSF will be RS-specific.

    - RAN4 to consider some time-domain limitation on CSI-RS configurations in the same MO to avoid extension of existing SSB measurement delay as well as reduce RAN4 workload and UE scheduling complexity.

  - Option 2 (OPPO):

    - RAN4 to re-use the existing requirement of CSSF as much as possible, and the framework of CSSF can be shared by SSB and CSI-RS based L3 measurement.

  - Option 3 (CATT):

    - If additional dedicated searcher is assumed for CSI-RS measurement, no impact on existing CSSF defined for SSB based measurement specified in 38.133.

    - Otherwise, the CSSFs for FR1/FR2 SCC shall be updated by considering the CSI-RS based intra-frequency and inter-frequency measurement without gap and within gap respectively.

  - Option 3(Huawei):

    - If a UE is configured with both CSI-RS-Resource-Mobility and ssb-ConfigMobility in one MO, the CSSF calculation shall consider SSB MO and CSI-RS MO.

    - If a UE is configured with one MO with CSI-RS-Resource-Mobility with associatedSSB and another MO with ssb-ConfigMobility, CSSF calculation shall consider 2 SSB MOs and 1 CSI-RS MO.

- Recommended WF

  - The detail requirements for CSSF is pending on the conclusion of intra and inter-frequency definition.

Issue 2-4-2: Scaling factor for RX beam sweeping

- Proposals

  - Option 1 (OPPO):

    - If CSI-RS is configured with associated SSB and QCL-ed to the associated SSB, no additional scaling factor for RX beam sweeping should not be considered for CSI-RS L3 measurement.

    - If CSI-RS configured with associated SSB but not QCL-ed to the associated SSB, Rx sweeping is needed and FFS on the scaling factor N.

  - Option 1a (DOCOMO):

    - RRM requirements should be specified based on the configuration of associatedSSB, e.g., if both ssbIndex and isQuasiColocated are provided, Rx beam sweeping is not needed.

  - Option 1b (CATT):

    - If the CSI-RS is QCL-ed to the associated SSB, no Rx sweeping is needed only after SSB has been detected.

  - Option 2 (LGE):

    - Case 3: CSI-RS is QCL-ed with the associated SSB and the SSB is detected.

      - Case 3-1: The SSB beam sweeping is finished

      - Case 3-2: The SSB beam sweeping is not finished

        - The measurement period time could take into account waiting time for SSB beam sweeping.

    - Case 4: CSI-RS is not QCL-ed with any SSB

      - Re-use the principle of SSB based L3-measurement for scaling factor with RX beam sweeping ~8

- Recommended WF

  - Option 1 as baseline for discussion

  - FFS on waiting time for SSB beam sweeping.

  - FFS on the value of scaling factor with RX beam sweeping if CSI-RS is not QCL-ed with any SSB

Sub-topic 2-5: Scheduling Restriction

Sub-topic description

- The requirements for scheduling restriction are only defined for CSI-RS L3 measurement without gaps

- Identify all possible factors which would cause scheduling restriction in next meeting:

  - Collision with UL transmission and DL measurement on TDD carrier

  - The need of Rx beam sweeping in FR2

  - Mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell

Open issues and candidate options before e-meeting:

Issue 2-5-1: Factors to consider for scheduling restriction

- Proposals

  - Option 1(OPPO, MediaTeK):

    - collision with UL transmission and DL measurement on TDD carrier and

    - Rx beam sweeping in FR2.

  - Option 2 (CATT, Huawei):

    - Mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell

    - RX beam sweeping in FR2

    - Collision between UL transmission and DL measurement for TDD carrier

- Recommended WF

  - Tentative Agreement:

    - collision with UL transmission and DL measurement on TDD carrier and

    - Rx beam sweeping in FR2.

    - FFS the impact of mix-numerology in 1st round and try to achieve agreement in 2nd round

Issue 2-5-2: General requirement

- Proposals

  - Option 1 (Huawei):

    - If UE is not able to support mixed numerology of data and CSI-RS L3 mobility, the following scheduling restrictions apply due to intra-frequency CSI-RS based L3 measurement:

      - if the associatedSSB is configured, UE is not expected to transmit or receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured.

      - if the associatedSSB is not configured, UE is not expected to transmit or receive on the data OFDM symbol impacted by CSI-RS resource symbol to be measured, provided timing difference between the CSI-RS resource and the serving cell should be less than half CP corresponding to the SCS of the CSI-RS.

    - When UE performs CSI-RS intra-frequency measurements in a TDD band, UE is not expected to transmit and receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured.

    - Scheduling restriction shall be considered when UE performs RX beam sweeping.

  - Option 2 (MediaTek):

    - The scheduling restriction on the additional OFDM symbols before and after SSB is not needed.

  - Option 3 (OPPO):

    - Option 1: the scheduling restriction on the additional OFDM symbols before and after SSB is not needed for CSI-RS based L3 measurement.

    - Option 2: define scheduling restriction on CSI-RS to be measurement and additional X OFDM symbol before and after consecutive CSI-RS symbols to be measured, FFS on X value.

  - Option 4 (LGE):

    - Define scheduling restriction on one data symbol before and after CSI-RS symbol to be measured due to Rx beam sweeping.

    - Do not define scheduling restriction if the timing difference between serving and neighbor cell including cell phase synchronization is guaranteed to be less than CP length

- Recommended WF

  - Option 1 can be as baseline for discussion.

Issue 2-5-3: Whether to restrict CSI-RS resources outside of DRX/MG duration

- Proposals (CATT)

  - Option 1: UE is allowed not to measure the CSI-RS resources that are not within DRX on-duration or measurement gap;

  - Option 2: No UE performance requirement is defined for the CSI-RS resources that are not within DRX on-duration or measurement gap;

  - Option 3: the network needs to make sure the configured CSI-RS resources falls within the configured measurement gaps

- Recommended WF

  - Tentative Agreement: Option 2

Issue 2-5-4: Collision between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell

- Proposal (LGE):

  - Option 1: Do not define CSI-RS measurement requirements for the collision case.

  - Option 2: Network should configure L1 measurement resource to avoid collision with CSI-RS L3 measurement resource of neighbour cell.

- Recommended WF

  - Tentative Agreement: Option 1

Issue 2-5-5: whether to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell

- Option 1:  (CATT)

  - It is proposed to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell.

- Option 2:  (CMCC)

  - Whether SimultaneousRxDataSSB-DiffNumerology can be reused or new UE capability is introduced can be further discussed.

Companies views’ collection for 1st round

Open issues

- Sub-topic 2-1: Case 1- if associatedSSB is not configured

| Company | Comments |
| --- | --- |
| MTK | Issue 2-1-1: Measurement requirements Support Option 1. Measuring all CSI-RS without checking the detection results of SSB is actually a waste of UE’s computation power. We are also curious about the difficulty in network side to provide information for associated SSB. |
| Apple | Issue 2-1-1: Option 1 |
| CATT | Support option 3/4. |
| Intel | Issue 2-1-1: Fine with option 1. |
| CMCC | Issue 2-1-1: Measurement requirements According to RAN1 agreements, if associatedSSB is not configured, UE base the timing of the CSI-RS resource on the timing of the serving cell. In this case, PSS/SSS detection is not necessary, only the requirements on measurement delay need to be specified. |
| Huawei, HiSilicon | Issue 2-1-1: option 1. UE didn’t know the actual timing difference between serving cell and to-be-measured cells. If the timing is not guaranteed, the measurement performance will be degraded seriously. |
| ZTE | Issue 2-1-1: Option 2 and option 4 are quite similar. Option 2 is more general and we support at this stage. |
| Qualcomm | Issue 2-1-1: Measurement requirements We support option 1 since without associated SSB being configured, UE has to measure the CSI-RS resource based on the serving cell’s timing, which implies the NW needs to maintain a smaller timing error between the serving cell and neighbor cells. Further, without associated SSB, UE cannot identify the cell. |
| Nokia, Nokia Shanghai Bell | Issue 2-1-1: Agree with Option 1. |
| Docomo | Issue 2-1-1: Measurement requirements We have the similar opinion to CMCC, and we still prefer option 2. |
| OPPO | Agree with MTK, Apple, Intel, Huawei, Qualcomm and Nokia. At least no requirements for this case in Rel-16. |

- Sub-topic 2-2: Case 2- if associatedSSB is configured for CSI-RS

| Company | Comments |
| --- | --- |
| MTK | Issue 2-2-1: General measurement requirements Support Option 1 or 1a. Actually PBCH can be skipped if UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated. Issue 2-2-2: AGC Support Option 1. Simply follow what we have for SSB-based requirement. We believe that Option 3 is addressing a different issue rather than AGC. |
| Apple | Issue 2-2-1: we prefer not to specify CSI-RS based cell identification requirements. In Rel-16, we can focus on CSI-RS based cell measurement only. Issue 2-2-2: No sure if we need to agree on a specific number here. Eventually, only thing to be captured in the spec is the total number of samples for FR1 and FR2 respectively. We can consider to reuse SSB based measurement requirement if CSI-RS configuration is similar as SMTC. |
| LGE | Issue 2-2-1 (General measurement requirements) If associatedSSB is configure for CSI-RS and UE does not detect SSB, UE is not require to measure CSI-RS resource since UE does not know timing of CSI-RS resource. However, if the deriveSSB-IndexFromCell is indicated, the UE can know the timing of CSI-RS resource. In this case, UE can measure CSI-RS resource and hence the requirement of this case need to be considered. |
| CATT | Issue 2-2-1: Support option2, if associated SSB is configured, ssb-Index will be configured to UE, thus PBCH decoding is not needed. Issue 2-2-2: OK with option 1/2. |
| Huawei, HiSilicon | Issue 2-2-1: support option 3. Multiple cases shall be considered. The detail can refer to document [R4-2004292]. In summary: - If the MO includes the serving CSI-RS resource with associated SSB, for the cells in the same MO, - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO. - For TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO. - If the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured, UE needs to perform PSS/SSS detection, DMRS matching and PBCH decoding to acquire SFN, frame boundary timing, slot timing and symbol level timing of one cell. For the other cells in the same MO, - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO. - For FR1 TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO. Issue 2-2-2: agree with option 1 and option2. Disagree with option3, the retuning time is the same as RF retuning time: 0.5ms for FR1. |
| ZTE | Issue 2-2-1: associatedSSB If this field is present, the UE may base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the cell indicated by the cellId in the CSI-RS-CellMobility. In this case, the UE is not required to monitor that CSI-RS resource if the UE cannot detect the SS/PBCH block indicated by this associatedSSB and cellId. According to RAN2 procedure the cell detection is needed for sure. If PBCH decoding is needed depending on configurations. It can be discussed case by case. CSI-RS measurement is of course necessary. Issue 2-2-2: If cell detection is performed, e.g. if associated SSB is configured, then there is no need for AGC. |
| Qualcomm | Issue 2-2-1: General measurement requirements We agree that the recommended WF provides two scenarios for measuring the CSI-RS depending on whether the configured SSB has been detected. And each scenario incurs different measurement latency. We want to add that, 1. If the configured SSB fails to be detected, requirement should not be defined. 2. Our proposal 4 imposes a further requirement that UE can measure the intra-frequency CSI-RS resources from the same MO/frequency layer without adjusting the FFT window a.k.a with a single FFT operation. Note this requires the network maintain the synchronization of the serving and its intra-frequency neighbor cells within a timing error. This applies to the case when the serving CSI-RS configuration is available in the MO. 3. If an MO doesNOT have the serving CSI-RS configuration (e.g. inter-frequency measurement), UE assumes the recommended WF to derive the target cell timing. Issue 2-2-2: AGC We agree on the recommended WF. |
| Nokia, Nokia Shanghai Bell | Issue 2-2-1: Agree with the recommended WF. If associatedSSB is configured, the UE need detect SSB before measuring CSI-RS. We could reuse existing cell detection time period for SSB-based measurement. Issue 2-2-2: Why do we need AGC when RF retuning to inter-frequency carriers? Further discussion is required. |
| Docomo | Issue 2-2-1: General measurement requirements We have similar view to CATT and MTK. If CSI-RS for L3 measurement is configured with associatedSSB, ssb-Index is also configured, then there is no necessity for UE to decode PBCH. Even if associatedSSB is not configured, UE can skip PBCH decoding when UE has already detected the SSB of the target cell and deriveSSB-IndexFromCell is congifured. Therefore, we prefer option 2 and 1a. |
| OPPO | Issue 2-2-1: General measurement requirements Agree to discuss the measurement delay case by case. In principle, we can follow these, - If configured SSB fails to be detected, requirement should not be defined. - If UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated, PBCH decoding can be skipped. - For the case if the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured, we can further discuss whether and how to define requirements based on Huawei’s proposal. Issue 2-2-2: AGC Agree on the recommended WF. AGC adjustment time is needed when UE needs to retune RF. FFS on when UE needs to retune RF and the value of AGC adjustment time. |

- Sub-topic 2-3: Requirement of Measurement Gap

| Company | Comments |
| --- | --- |
| MTK | Issue 2-3-1: intra-frequency and inter-frequency measurement with or w/o gaps? Support Option 3 and Option 4 together. We suggest to first clarify the scope of this WI. If we only define requirement for intra-frequency measurement without gap and inter-frequency measurement with gap, then the discussion here will become much easier. |
| Apple | Related to intra-/inter-f definition. Agree with option 1 in principle. |
| CATT | Issue 2-3-1: For the WI scope, we support option 3 as a compromise, for the need gap for CSI-RS intra-/inter-frequency measurement, we can agree the principle in option1. |
| Intel | fine with option 1. |
| CMCC | Issue 2-3-1: intra-frequency and inter-frequency measurement with or w/o gaps? Our preference is to specify requirements for intra-f without MG, intra-f with MG, inter-f without MG, inter-f with MG. However, if companies have concern on the limited timeline or workload, we can compromise to option 3. |
| Huawei. HiSilicon | Issue 2-3-1: related with intra-f and inter-f definition. Whether need gaps depend on whether UE need to perform RF retuning. |
| ZTE | Issue 2-3-1: It depends on definition of intra frequency measurement. In general 4 type of requirements as for SSB based measurement are needed. RAN4 can also consider to define requirements just for gap based and gapless measurement if possible. |
| Qualcomm | Issue 2-3-1: intra-frequency and inter-frequency measurement with or w/o gaps? Please note we added our proposal in the option 1 regarding the GAP tuning time for CSI-RS measurement that lies outside the active BWP. Other than that, we agree on the recommended WF. |
| Nokia, Nokia Shanghai Bell | Prefer Option 4. This also depends on how we define the intra-frequency measurement. We expect the intra-frequency measurement do not need gaps. |
| Docomo | Issue 2-3-1: intra-frequency and inter-frequency measurement with or w/o gaps? As we commented in [94e Bis][124] NR_CSIRS_L3meas_RRM_Part_1, we think that if all of CSI-RS resources are included in active BWP, UE can perform intra-frequency measurement, in other words, UE can measure them without measurement gap. On the other hand, if any one of the conditions of intra-frequency measurement (e.g., CSI-RS resources are partially or fully configured out of active BWP), it is treated as inter-frequency measurement and UE will need measurement gap. In conclusion, option 4 is preferable for us. |
| OPPO | This issue depends on the definition of intra-f and inter-f measurement. Generally agree with option 3 to define requirement for intra-frequency measurement without gap and inter-frequency measurement with gap. And the condition of without gap can be further discussed based on option 1/2/4. |

- Sub-topic 2-4: Scaling Factor

| Company | Comments |
| --- | --- |
| MTK | Issue 2-4-1: CSSF Support Option 1 In addition to frequency-domain limitation, we also need time-domain limitation. Otherwise, there are too many scenarios to be handled. We can start from Issue 1-4-1. Further comment to CATT: In our understanding a dedicated engine will be used for CSI-RS. We prefer not to call it searcher because we cannot search for cells based on CSI-RS. Even with a separate engine, we still see the impact to exiting CSSF for SSB at least for inter-frequency case. Because UE can only pick one frequency layer to measure at one gap. If one gap is to be used to measure CSI-RS, then the SSB-based measurement on other frequency layers need to wait for next opportunity. In other words, CSSF may be increased, resulting a longer measurement period for frequency layers based on SSB. Issue 2-4-2: Scaling factor for RX beam sweeping Either Option 1, 1a or 1b is OK to us. One thing to be noted is that if the CSI-RS overlapped with symbol of SSB or SMTC (during which UE has to sweep Rx beam), UE may have to sweep the Rx beam anyway. We can also handle this in the measurement restriction. |
| Apple | Issue 2-4-1: Same comment as MTK, Issue 1-4-1 should be discussed first. Issue 2-4-2: Option 1 is OK. |
| LGE | Issue 2-4-2 (Scaling factor for RX beam sweeping) If the CSI-RS is QCL-ed with the associated SSB and the SSB is detected as in option 2, we support no RX beam sweeping as the first bullet in option 1. If the CSI-RS is not QCL-ed with any SSB as in option 2, we support the second bullet in option 1 with scaling factor N = 8. If the CSI-RS is QCL-ed with the associated SSB and the SSB is detected but SSB beam sweeping is not finished, the case 3-2 in option 2 needs to be considered. Since we think that no RX sweeping is needed only after SSB beam sweeping has been finished, the measurement period time could take into account waiting time for SSB beam sweeping. |
| CATT | Issue 2-4-1: For CSSF, first of all, we have a question for UE vendor, whether there is a dedicated searcher for CSI-RS measurement? If yes, there is no impact on existing CSSF defined for SSB based measurement specified in 38.133. Otherwise, the CSSFs for FR1/FR2 SCC shall be updated by considering the CSI-RS based intra-frequency and inter-frequency measurement without gap and within gap respectively. Issue 2-4-2: If the CSI-RS is QCL-ed to the associated SSB, no Rx sweeping is needed only after SSB has been detected, otherwise, Rx sweeping is needed. |
| Huawei, HiSilicon | Issue 2-4-1: support option4. In principle option 2 and option 3 are ok as well (To option3, we don’t think there are additional searcher). i.e, regarding CSSFoutside_gap, the SSB MOs and CSI-RS MOs shall compete searchers. Regarding CSSFwithingap, the SSB MOs and CSI-RS MOs shall compete the gap measurement opportunities In detail, two cases shall be considered: Case 1: If a UE is configured with both CSI-RS-Resource-Mobility and ssb-ConfigMobility in one MO, it shall be regarded that there are two type measurements, that is, one SSB based measurement and one CSI-RS based measurement. The CSSF calculation shall consider both SSB based MO and CSI-RS based MO. Case 2: If a UE is configured with one MO with CSI-RS-Resource-Mobility with associatedSSB and another MO with ssb-ConfigMobility, then two SSB based measurements (associatedSSB and SSB) and one CSI-RS based measurements will be performed for the UE. For the CSSF calculation, the two SSB MOs and CSI-RS MO shall be considered. Issue 2-4-2: option 1, 1a, 1b have the similar meaning. The principle is ok. The detailed wording can be further discussion. |
| ZTE | Issue 2-4-1: How the CSI-RS measurement is performed under different configurations needs to be decided firstly. Then how the gap can be shared by CSI-RS measurement and SSB measurement can be further discussed. |
| Qualcomm | Issue 2-4-1: CSSF We appreciate more comments to this regard. In our view, CSI-RS measurement doesnot necessarily reserve a searcher. Hence it is not in the same situation as SSB based measurement which we shall discuss the CSSF. Please clarify if we need to prioritize the CSSF in the discussions. Issue 2-4-2: Scaling factor for RX beam sweeping We support option 1 as the baseline for discussions. |
| Nokia, Nokia Shanghai Bell | Issue 2-4-1: Agree with the recommended WF. Issue 2-4-2: Agree with the recommended WF. |
| Docomo | Issue 2-4-2: Scaling factor for RX beam sweeping Our proposal option 1a seems to be reflected to option 1, thus option 1 is also OK for us. |
| OPPO | Issue 2-4-1: CSSF Agree with MTK, Apple, ZTE and Qualcomm to start with issue 1-4-1 CSI-RS resources configurations and deprioritize the discussion of CSSF in this meeting. Issue 2-4-2: Scaling factor for RX beam sweeping Support option 1 as the baseline. And Case 3-2 provided by LGE at least makes sense to me and more discussion is needed on waiting time. |

- Sub-topic 2-5: Scheduling Restriction

| Company | Comments |
| --- | --- |
| MTK | Issue 2-5-1: Factors to consider for scheduling restriction Option 2 is also agreeable to us Issue 2-5-2: General requirement Support Option 2. Note that UE has only one single FFT timing for the measurement of all cells in one frequency. And the FFT timing for intra-frequency should always be UE’s serving cell. So we do not need to consider the additional OFDM symbol margin for scheduling restriction as we did for SSB. Issue 2-5-3: Whether to restrict CSI-RS resources outside of DRX/MG duration OK with the recommended WF Issue 2-5-4: Collision between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell Either Option 1 or 2 is OK to us. Issue 2-5-5: whether to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell It is not clear whether Option 1 is try to addressing the mix-numerology case or not. Regarding Option 2, if there is a need for mix –numerology between CSI-RS measurement and serving cell data reception, it is better to use a new capability to avoid confusion. |
| LGE | 2-5-2 (General requirement) If the timing difference between serving and neighbour cell including cell phase synchronization is guaranteed to be less than CP length, the scheduling restriction for additional OFDM symbol is not needed in CSI-RS L3 measurement. Otherwise, the scheduling restriction for additional OFDM symbol before and after CSI-RS are needed since the network does not know the timing difference. |
| CATT | Issue 2-5-1:Support option 2. If UE is not support mix-numerology between SSB of serving cell and CSI-RS of neighbour cell, scheduling restriction is needed. Issue 2-5-3: OK with the recommended WF Issue 2-5-5: To MTK, option 1 is to introduce UE capability to support mix-numerology case. And we prefer to introduce a new capability other than the parameter of “SimultaneousRxDataSSB-DiffNumerology” |
| CMCC | Issue 2-5-1: Factors to consider for scheduling restriction Option 2. For the mixed numerology, it is proposed to consider the UE capability on supporting of simultaneous reception of mixed numerology, like we did for SSB based measurement. If UE is supporting the simultaneous reception of mixed numerology, scheduling restriction is not needed. Issue 2-5-5: whether to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell Our preference is to consider the UE capability on supporting of simultaneous reception of mixed numerology, as for reusing SimultaneousRxDataSSB-DiffNumerology or introducing new UE capability, we are open to discussion. |
| Huawei, HiSilicon | Issue 2-5-1: support option 2 since the numerology between data/SSB of serving cell and CSI-RS of neighbour cell may be different. Issue 2-5-2: agree with the recommended WF. Issue 2-5-3: please clarify the scenario. Does it refer to the measurement with gap? Issue 2-5-4: needs to further study since it is the first time to raise the issue. Issue 2-5-5: option 2. |
| ZTE | Issue 2-5-1: Option 1 Issue 2-5-3 For intra frequency measurement without gap, there should be no restrictions. Issue 2-5-4 This needs more study. At least for intra-frequency measurement without gap, L3 and L1 measurement requirements should apply. Issue 2-5-4 Option 2 can be considered as starting point for discussion. |
| Qualcomm | Issue 2-5-1: Factors to consider for scheduling restriction We agree with the recommended WF. Issue 2-5-2: General requirement We agree on comments by MTK and LG and open towards further discussions. Issue 2-5-3: Whether to restrict CSI-RS resources outside of DRX/MG duration We support option 2. Issue 2-5-4: Collision between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell We support option 2 that requires NW to resolve the collision. Issue 2-5-5: whether to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell We support option 2 for further discussions. |
| Nokia, Nokia Shanghai Bell | Issue 2-5-1: Agree with the recommended WF. Issue 2-5-2: We can discuss the details after we conclude on the scenario scope and the definition. |
| Docomo | Issue 2-5-1: Factors to consider for scheduling restriction We support the recommended WF. Issue 2-5-2: General requirement We have the same opinion as MTK and prefer option 2. Issue 2-5-5: whether to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell Option 2 is OK. |
| OPPO | Issue 2-5-1: Factors to consider for scheduling restriction Fine with option 2. Issue 2-5-2: General requirement More discussion seems needed. Issue 2-5-3: Whether to restrict CSI-RS resources outside of DRX/MG duration Agree with the recommended WF. Issue 2-5-4: Collision between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell Either Option 1 or 2 is OK to us. Issue 2-5-5: whether to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell We support option 2 for further discussions. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

- Sub-topic 2-1: Case 1- if associatedSSB is not configured

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Tentative agreements: Candidate options: - Option 1 (MTK, Apple, Intel, Huawei, Qualcomm and Nokia): - No requirements in Rel-16. - Option 2 (DOCOMO, ZTE, CMCC): - the requirement needs at least consider the CSI-RS measurement time, if associatedSSB is not configured, assuming UE shall base the timing on its serving cell (indicated by refServCellIndex or PCell) - Option 3 (Huawei, CATT): - CSI-RS measurement requirement is the CSI-RS measurement periodicity, when associatedSSB is not configured assuming that the timing between serving cell (which is timing reference cell) and the neighbour cell is frame boundary synchronized. Recommendations for 2nd round: Continue discussion. We have discussed this for several meetings. Regarding option 1 as majority view, can we compromise to option 1 due to limited time of this WID? If so, no more discussion is needed for 2nd round. |

- Sub-topic 2-2: Case 2- if associatedSSB is configured for CSI-RS

|  | Status summary |
| --- | --- |
| Sub-topic#2-2 | Tentative agreements: Candidate options: Issue 2-2-1: General measurement requirements - Option 1 (OPPO, MediTek, ZTE): CSI-RS based cell identification can consider - 1) Cell search via SSB, 2) PBCH decoding and 3) CSI-RS measurement. - If UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated, then UE can skip PBCH decoding. - Option 2 (CATT, DOCOMO): - TCSI-RS_identification_with_associatedSSB = TPSS/SSS_sync + TCSI-RS_measurement_period. - Option 3 (Huawei): - If the MO includes the serving CSI-RS resource with associated SSB, for the cells in the same MO, - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO. - For TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO. - If the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured, UE needs to perform PSS/SSS detection, DMRS matching and PBCH decoding to acquire SFN, frame boundary timing, slot timing and symbol level timing of one cell. For the other cells in the same MO, - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO. - For FR1 TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO. - Option 4 (LGE): - Consider case that UE measures the CSI-RS resource with deriveSSB-IndexFromCell when associatedSSB is not detected. - Option 5 (Apple): - Do not specify CSI-RS based cell identification requirements and focus on CSI-RS based cell measurement only in Rel-16. - Option 6 (Qualcomm): Based on option 1, we want to add that, - If the configured SSB fails to be detected, requirement should not be defined. - Proposal 4(in R4-2004826) imposes a further requirement that UE can measure the intra-frequency CSI-RS resources from the same MO/frequency layer without adjusting the FFT window a.k.a with a single FFT operation. This applies to the case when the serving CSI-RS configuration is available in the MO. - If an MO doesNOT have the serving CSI-RS configuration (e.g. inter-frequency measurement), UE assumes option 1 to derive the target cell timing. Issue 2-2-2: AGC - Option 1 (MediaTek): - For inter-frequency CSI-RS measurement, at least additional [3] AGC samples are needed. - Option 2 (Huawei): - AGC adjustment time shall be considered when UE needs to retune RF to an inter-frequency layer to perform measurement. - Option 3 (ZTE): - No need for AGC if cell detection is performed. Recommendations for 2nd round: Continue to discuss the measurement delay case by case. In principle, we could follow the guideline below, - If configured SSB fails to be detected, requirement should not be defined. - If UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated, PBCH decoding can be skipped. - For the case if the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured, we can further discuss whether and how to define requirements based on Huawei’s proposal. Continue to discuss AGC adjustment time. FFS on AGC when UE needs to retune RF and the value. |

- Sub-topic 2-3: Requirement of Measurement Gap

|  | Status summary |
| --- | --- |
| Sub-topic#2-3 | Tentative agreements: Candidate options: Requirements with or w/o gaps - Option 1 (CATT, CMCC, OPPO)：For CSI-RS based measurement requirement, the following scenarios are prioritized to be defined in Rel-16: - Intra-frequency measurement without gap - Inter-frequency measurement with gap The principle for gap-needed or gapless - Option 1(Qualcomm, Intel, Apple, OPPO): The UE will need GAPs for CSI-RS L3 measurements if - The SCS of CSI-RS is different from active BWP - The CSI-RS is not fully confined within the active BWP - The CP of cells to be measured is different from that of active BWP (60 kHz SCS only) - The tuning time for CSI-RS based measurements that are outside UE’s active BWP will be same as that for BWP switch. - Option 2 (OPPO): - Intra-frequency measurement can all be configured without gaps if the definition of intra-frequency includes the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of the UE. - Otherwise, all others are inter-frequency measurement with gaps. - Option 3 (Nokia, MTK, DOCOMO): For intra-frequency measurement, the requirement is defined when all the measured CSI-RS resources in one MO are within the active BWP. Recommendations for 2nd round: For the scenarios to be prioritized: Option 1 by CATT seems acceptable and can be as baseline for discussion. Continue the discussion in 2nd round when the intra-f measurement definition is clear. For the principle for gap-needed or gapless: Depend on definition of intra frequency measurement. We can further discuss based on QC’s option 1. |

- Sub-topic 2-4: Scaling Factor

|  | Status summary |
| --- | --- |
| Sub-topic#2-4 | Tentative agreements: Candidate options: Issue 2-4-1: CSSF The detail requirements for CSSF is pending on the conclusion of intra and inter-frequency definition Issue 2-4-2: Scaling factor for RX beam sweeping - Option 1(OPPO, CATT, MTK, Apple, Huawei, Qualcomm, DOCOMO): - If the CSI-RS is QCL-ed to the associated SSB, no Rx sweeping is needed only after SSB has been detected. (Nokia) - If CSI-RS configured with associated SSB but not QCL-ed to the associated SSB, Rx sweeping is needed. FFS on the scaling factor N =8. - Option 2 (LGE): - Case 3: CSI-RS is QCL-ed with the associated SSB and the SSB is detected. - Case 3-1: The SSB beam sweeping is finished - No Rx sweeping is needed - Case 3-2: The SSB beam sweeping is not finished - The measurement period time could take into account waiting time for SSB beam sweeping. - Case 4: CSI-RS is not QCL-ed with any SSB - Re-use the principle of SSB based L3-measurement for scaling factor with RX beam sweeping ~8 Recommendations for 2nd round: Issue 2-4-1: CSSF Depending on CSI-RS resource configuration in issue 1-4-1 and intra and inter-frequency definition. Focus on issue 1-4-1 CSI-RS resources configurations in this email thread. And suggest to deprioritize the discussion of CSSF in this meeting. Issue 2-4-2: Scaling factor for RX beam sweeping Support option 1 as the baseline for further discussion. FFS the Case 3-2 provided by LGE about waiting time for SSB beam sweeping. |

- Sub-topic 2-5: Scheduling Restriction

|  | Status summary |
| --- | --- |
| Sub-topic#2-5 | Tentative agreements: Candidate options: Issue 2-5-1: Factors to consider for scheduling restriction - Option 2 (CATT, Huawei, MTK, OPPO, CMCC, DOCOMO): - Mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell - RX beam sweeping in FR2 - Collision between UL transmission and DL measurement for TDD carrier Issue 2-5-2: General requirement - Option 1 (Huawei): - If UE is not able to support mixed numerology of data and CSI-RS L3 mobility, the following scheduling restrictions apply due to intra-frequency CSI-RS based L3 measurement: - if the associatedSSB is configured, UE is not expected to transmit or receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured. - if the associatedSSB is not configured, UE is not expected to transmit or receive on the data OFDM symbol impacted by CSI-RS resource symbol to be measured, provided timing difference between the CSI-RS resource and the serving cell should be less than half CP corresponding to the SCS of the CSI-RS. - When UE performs CSI-RS intra-frequency measurements in a TDD band, UE is not expected to transmit and receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured. - Scheduling restriction shall be considered when UE performs RX beam sweeping. - Option 2 (MediaTek, DOCOMO): - The scheduling restriction on the additional OFDM symbols before and after SSB is not needed. - Option 3 (LGE): - Define scheduling restriction on one data symbol before and after CSI-RS symbol to be measured due to Rx beam sweeping. - Do not define scheduling restriction if the timing difference between serving and neighbor cell including cell phase synchronization is guaranteed to be less than CP length Issue 2-5-3: Whether to restrict CSI-RS resources outside of DRX/MG duration - Option 2 (CATT, MTK, Qualcomm, OPPO, Huawei?, ZTE?): - No UE performance requirement is defined for the CSI-RS resources that are not within DRX on-duration or measurement gap Issue 2-5-4: Collision between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell - Option 1(LGE, MTK, OPPO): Do not define CSI-RS measurement requirements for the collision case. - Option 2(LGE, MTK, Qualcomm, OPPO, ZTE): Network should configure L1 measurement resource to avoid collision with CSI-RS L3 measurement resource of neighbour cell. Issue 2-5-5: whether to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell - Option 1: (CATT) - new UE capability - Option 2: (CMCC, Huawei, Qualcomm, DOCOMO, OPPO) - Resuing SimultaneousRxDataSSB-DiffNumerology or new UE capability is introduced can be further discussed. Recommendations for 2nd round: No consensus in 1st round and continue to discuss based on the following guideline. Issue 2-5-1: Option 2 as majority view. Issue 2-5-2: Option 1 can be as baseline for further discussion. Issue 2-5-3: Option 2 as majority view. The scenario needs to be clarified for intra-f measurement without gaps. Issue 2-5-4: Option 2 can be as baseline for further discussion. Issue 2-5-5: Option 2 can be as baseline for further discussion. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | R4-2005357, WF on measurement capabilities and requirements CSI-RS based L3 measurement | OPPO |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

- Sub-topic 2-1: Case 1- if associatedSSB is not configured

Issue 2-1-1: Measurement requirements

- Option 1 (MTK, Apple, Intel, Huawei, Qualcomm, Nokia, OPPO):

  - No requirements in Rel-16.

- Option 2 (DOCOMO, ZTE, CMCC):

  - the requirement needs at least consider the CSI-RS measurement time, if associatedSSB is not configured, assuming UE shall base the timing on its serving cell (indicated by refServCellIndex or PCell)

- Option 3 (CATT):

  - CSI-RS measurement requirement is the CSI-RS measurement periodicity, when associatedSSB is not configured assuming that the timing between serving cell (which is timing reference cell) and the neighbour cell is frame boundary synchronized.

Moderator: Similar to issue 1-1-1, we suggest to conclude this issue to move forward in this meeting. Regarding we have discussed this for several meetings, can we compromise to option 1 as majority view in Rel-16 due to limited time of this WID?.

| Company | Comments |
| --- | --- |
| OPPO | Support option 1, similar to issue 1-1-1. |
| CATT | We can compromise to option 1 |
| Intel | Support option 1 |
| MTK | Prefer Option 1. Suggest to converge on this issue in this meeting so that RAN4 can start to discuss the requirements and CRs in May meeting. |
| Apple | Option 1 |
| Nokia, Nokia Shanghai Bell | We prefer Option1. |
| Qualcomm | We support Option 1. |
| ZTE | Option 1 considering the status of the WI. |
| Docomo | We still prefer option 2. As we commented for issue 1-1-1 in 2nd discussion, the case without associatedSSB is possible, thus we should specify requirements for the case without associatedSSB. |

- Sub-topic 2-2: Case 2- if associatedSSB is configured for CSI-RS

Issue 2-2-1: General measurement requirements

  - Option 1 (OPPO, MediTek, ZTE): CSI-RS based cell identification can consider

    - 1) Cell search via SSB, 2) PBCH decoding and 3) CSI-RS measurement.

    - If UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated, then UE can skip PBCH decoding.

  - Option 2 (CATT, DOCOMO):

    - TCSI-RS_identification_with_associatedSSB = TPSS/SSS_sync + TCSI-RS_measurement_period.

  - Option 3 (Huawei):

- If the MO includes the serving CSI-RS resource with associated SSB, for the cells in the same MO,

  - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO.

  - For TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO.

- If the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured, UE needs to perform PSS/SSS detection, DMRS matching and PBCH decoding to acquire SFN, frame boundary timing, slot timing and symbol level timing of one cell. For the other cells in the same MO,

  - For FR1 FDD cell, UE can perform PSS/SSS detection and DMRS matching to acquire time index of each cell in the MO.

  - For FR1 TDD cell or FR2 cells, UE can perform PSS/SSS detection to acquire symbol level timing of each cell in the MO.

  - Option 4 (LGE):

- Consider case that UE measures the CSI-RS resource with deriveSSB-IndexFromCell when associatedSSB is not detected.

  - Option 5 (Apple):

- Do not specify CSI-RS based cell identification requirements and focus on CSI-RS based cell measurement only in Rel-16.

  - Option 6 (Qualcomm):

Based on option 1, we want to add that,

- If the configured SSB fails to be detected, requirement should not be defined.

- Proposal 4(in R4-2004826) imposes a further requirement that UE can measure the intra-frequency CSI-RS resources from the same MO/frequency layer without adjusting the FFT window a.k.a with a single FFT operation. This applies to the case when the serving CSI-RS configuration is available in the MO.

- If an MO doesNOT have the serving CSI-RS configuration (e.g. inter-frequency measurement), UE assumes option 1 to derive the target cell timing.

Moderator: Continue to discuss the measurement delay case by case. In principle, we could follow the guideline below,

- If configured SSB fails to be detected, requirement should not be defined.

- If UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated, PBCH decoding can be skipped.

- For the case if the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured, we can further discuss whether and how to define requirements based on option 3.

| Company | Comments |
| --- | --- |
| OPPO | Support option 1 as baseline for discussion. For the case if the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured, we agree with Proposal 7 by MTK in R4-2003644. UE always has to perform serving cell measurement if measConfig is provided according to RAN2 procedure. - No requirement is defined if CSI-RS of serving cell is not configured |
| MTK | Option 1 as the starting point. We can further consider the comments from companies like Option 6 from Qualcomm. |
| Apple | Option 4. What’s the motivation to do CSI-RS based cell identification if it has to rely on SSB based cell search? If SSB based cell search has to be done, why don’t we just do SSB based cell identification. CSI-RS can be used for cell measurement only. |
| Nokia, Nokia Shanghai Bell | We can agree with Option 1 as the starting point for FR1. We can further discuss it based on the agreed applicable scenarios during this meeting. |
| Qualcomm | Option 6 We want to reiterate, regardless of serving CRI-RS availability, requirements shall be defined by assuming UE does only one FFT window. If the working assumption on single FFT requires any change, we need to request a new capability to indicate whether UE supports multiple FFT windows. |
| ZTE | Option 1 as starting point. The procedure can follow what is specified in RAN2 spec. |
| Docomo | Our preference is option 2, but we can compromise with option 1 since it considers the case that UE can skip PBCH decoding. |

Issue 2-2-2: AGC

Continue to discuss AGC adjustment time.  FFS on AGC when UE needs to retune RF and the value.

  - Option 1 (MediaTek):

    - For inter-frequency CSI-RS measurement, at least additional [3] AGC samples are needed.

  - Option 2 (Huawei):

    - AGC adjustment time shall be considered when UE needs to retune RF to an inter-frequency layer to perform measurement.

  - Option 3 (ZTE):

    - No need for AGC if cell detection is performed.

| Company | Comments |
| --- | --- |
| CATT | Option 1 |
| MTK | Option 1. Simply follow SSB. |
| Nokia, Nokia Shanghai Bell | Why do we need AGC when RF retuning to inter-frequency carriers? Further discussion is required. |
| Qualcomm | We would appreciate more info to understand the problem. Option 1: [3] samples meaning [3] measurements on the same CSI-RS resource? Option 2: is AGC adjustment time including the time for measuring the resource firstly? |
| ZTE | Option 3 |

- Sub-topic 2-3: Requirement of Measurement Gap

Issue 2-3-1: intra-frequency and inter-frequency measurement with or w/o gaps?

Q1: Requirements with or w/o gaps

  - Option 1 (CATT, CMCC, OPPO)：For CSI-RS based measurement requirement, the following scenarios are prioritized to be defined in Rel-16:

    - Intra-frequency measurement without gap

    - Inter-frequency measurement with gap

Q2: The principle for gap-needed or gapless

  - Option 1(Qualcomm, Intel, Apple, OPPO): The UE will need GAPs for CSI-RS L3 measurements if

    - The SCS of CSI-RS is different from active BWP

    - The CSI-RS is not fully confined within the active BWP

    - The CP of cells to be measured is different from that of active BWP (60 kHz SCS only)

    - The tuning time for CSI-RS based measurements that are outside UE’s active BWP will be same as that for BWP switch.

  - Option 2 (OPPO):

    - Intra-frequency measurement can all be configured without gaps if the definition of intra-frequency includes the bandwidth of the CSI-RS on the eighbour cell is within the active BWP of the UE.

    - Otherwise, all others are inter-frequency measurement with gaps.

  - Option 3 (Nokia, MTK, DOCOMO): For intra-frequency measurement, the requirement is defined when all the measured CSI-RS resources in one MO are within the active BWP.

| Company | Comments |
| --- | --- |
| OPPO | Q1: option 1 Q2: option 1 We can come back to this once the definition of intra-f and inter-f measurement was concluded. |
| CATT | Q1: Option 1 Q2: option 1 |
| Intel | Q1: option 1 Q2: option 1 |
| MTK | Q1: Option 1 is fine, but this issue is also discussed in [124] Q2: There is some common ground among 3 proposals. We are OK to take the first 3 bullets of Option 1 as the starting point. We have concern on the 4th point of Option 1. It is far longer than the RF switching time in R15 (500us in FR1 and 250us in FR2) |
| Nokia, Nokia Shanghai Bell | With agreed definition for intra-f, no gaps are required for intra-frequency measurement. We should prioritize defining the requirements for intra-f measurement. Afterwards, we may work on inter-f w/ gaps as proposed in Option1. |
| Qualcomm | We support Option 1 for Q1 to prioritize intra-frequency measurement w/o GAP and inter-frequency measurement with GAP. For Q2, we will request FFS about budgeting the tuning time. |
| ZTE | Q1: Option 1 Q2: Option 1a - The SCS of CSI-RS is different from active BWP if UE is not capable of mixed numerology Except the change to the above sub-bullet, other sub-bullet of option 1 is fine. |

- Sub-topic 2-4: Scaling Factor

Issue 2-4-2: Scaling factor for RX beam sweeping

- Option 1(OPPO, CATT, MTK, Apple, Huawei, Qualcomm, DOCOMO):

  - If the CSI-RS is QCL-ed to the associated SSB, no Rx sweeping is needed only after SSB has been detected. (Nokia)

  - If CSI-RS configured with associated SSB but not QCL-ed to the associated SSB, Rx sweeping is needed. FFS on the scaling factor N =8.

- Option 2 (LGE):

  - Case 3: CSI-RS is QCL-ed with the associated SSB and the SSB is detected.

    - Case 3-1: The SSB beam sweeping is finished

      - No Rx sweeping is needed

    - Case 3-2: The SSB beam sweeping is not finished

      - The measurement period time could take into account waiting time for SSB beam sweeping.

  - Case 4: CSI-RS is not QCL-ed with any SSB

    - Re-use the principle of SSB based L3-measurement for scaling factor with RX beam sweeping ~8

| Company | Comments |
| --- | --- |
| OPPO | Option 1 as baseline. For the case SSB beam sweeping is not finished, it can leave to the implementation. |
| CATT | Option 1 |
| Intel | support option 1 |
| LGE | Since option 1 and option 2 are very similar except for case 3-2, we can support option 1. But, we think that the case 3-2 in option 2 needs more discussion. |
| MTK | Option 1. Since measurement and cell search are a long term behaviors, we may not need to go to the detail on whether SSB beam sweeping is finished or not. But we can add something like Qualcomm’s proposal in Issue 2-2-1, like “If the configured SSB fails to be detected, requirement should not be defined.” |
| Apple | Option 1 |
| Nokia, Nokia Shanghai Bell | We would like to study it a bit more and come back next meeting. |
| Qualcomm | We support Option 1 |
| ZTE | FFS. Option 1 can be considered as starting point. |
| Docomo | Option 1. |

- Sub-topic 2-5: Scheduling Restriction

Issue 2-5-1: Factors to consider for scheduling restriction

Moderator: according to majority views, we suggest to consider the following 3 factors.

- Mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell

- RX beam sweeping in FR2

- Collision between UL transmission and DL measurement for TDD carrier

Q1： Whether to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell

  - Option 1: New UE capability

  - Option 2: Reusing SimultaneousRxDataSSB-DiffNumerology

  - Option 3: Do not consider the case mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell

| Company | Comments |
| --- | --- |
| OPPO | Prefer option 1. And the details of new UE capability need to be clarified. |
| CATT | Option 1 |
| MTK | We have concern on Option 2, which extended the current IE definition. Either Option 1 or 3 is fine to us. |
| Apple | Option 1. We also need to agree on scheduling restriction is generally needed for FR2. |
| Nokia, Nokia Shanghai Bell | We would like to study it a bit more and come back next meeting. |
| Qualcomm | We support Option 1 |
| ZTE | FFS if it necessary in Rel-16 based on what requirements are specified. |

Issue 2-5-2: General requirement

- Option 1 (Huawei):

  - If UE is not able to support mixed numerology of data and CSI-RS L3 mobility, the following scheduling restrictions apply due to intra-frequency CSI-RS based L3 measurement:

    - if the associatedSSB is configured, UE is not expected to transmit or receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured.

    - if the associatedSSB is not configured, UE is not expected to transmit or receive on the data OFDM symbol impacted by CSI-RS resource symbol to be measured, provided timing difference between the CSI-RS resource and the serving cell should be less than half CP corresponding to the SCS of the CSI-RS.

  - When UE performs CSI-RS intra-frequency measurements in a TDD band, UE is not expected to transmit and receive on 2 data OFDM symbols impacted by CSI-RS resource symbol to be measured.

  - Scheduling restriction shall be considered when UE performs RX beam sweeping.

- Option 2 (MediaTek, DOCOMO):

  - The scheduling restriction on the additional OFDM symbols before and after SSB is not needed.

- Option 3 (LGE):

  - Define scheduling restriction on one data symbol before and after CSI-RS symbol to be measured due to Rx beam sweeping.

  - Do not define scheduling restriction if the timing difference between serving and neighbor cell including cell phase synchronization is guaranteed to be less than CP length

| Company | Comments |
| --- | --- |
| OPPO | FFS |
| LG | Prefer option 3. |
| MTK | We support Option 2, but we know that this discussion is subject to the conclusion of synchronization assumption in [124] |
| Apple | Prefer to option 3. |
| Nokia, Nokia Shanghai Bell | We would like to study it a bit more and come back next meeting. |
| Qualcomm | FFS |
| ZTE | FFS |

Issue 2-5-3: Whether to restrict CSI-RS resources outside of DRX/MG duration

- Option 2 (CATT, MTK, Qualcomm, OPPO, Huawei?, ZTE?):

  - No UE performance requirement is defined for the CSI-RS resources that are not within DRX on-duration or measurement gap

Moderator: according to majority views, can we take option 2 as agreement?

| Company | Comments |
| --- | --- |
| OPPO | Agree with Option 2 |
| CATT | Agree with option 2 |
| MTK | Option 2 |
| Apple | Option 2 is OK |
| Nokia, Nokia Shanghai Bell | Agree with Option2. |
| Qualcomm | We support Option 2. |
| ZTE | In Rel-16, option 2 would be fine. |

Issue 2-5-4: Collision between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell

- Option 1(LGE, MTK, OPPO): Do not define CSI-RS measurement requirements for the collision case.

- Option 2(LGE, MTK, Qualcomm, OPPO): Network should configure L1 measurement resource to avoid collision with CSI-RS L3 measurement resource of neighbour cell.

| Company | Comments |
| --- | --- |
| OPPO | FFS |
| LG | FFS |
| MTK | Both options have the same meaning to us |
| Apple | Agree both options are the same. Option 1 is more clear from UE requirement point of view. |
| Nokia, Nokia Shanghai Bell | We would like to study it a bit more and come back next meeting. |
| Qualcomm | We support Option 2. |
| ZTE | FFS |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

- Sub-topic 2-1: Case 1- if associatedSSB is not configured

Issue 2-1-1: Measurement requirements

- Option 1 (MTK, Apple, Intel, Huawei, Qualcomm, Nokia, OPPO, CATT, ZTE):

  - No requirements in Rel-16.

- Option 2 (DOCOMO, CMCC):

  - the requirement needs at least consider the CSI-RS measurement time, if associatedSSB is not configured, assuming UE shall base the timing on its serving cell (indicated by refServCellIndex or PCell)

Recommended WF:  Most companies prefer no requirements for case 1 in Rel-16. No consensus is achieved and need decision in next meeting.

- Sub-topic 2-2: Case 2- if associatedSSB is configured for CSI-RS

Still exist 6 option as 1st round for Issue 2-2-1.

Recommended WF:

[Tentative agreements]:

- CSI-RS based cell identification can consider

  - 1) Cell search via SSB, 2) PBCH decoding and 3) CSI-RS measurement.

- If configured SSB fails to be detected, requirement should not be defined.

- FFS: If UE already detects the SSB of the target cell and deriveSSB-IndexFromCell is indicated, PBCH decoding can be skipped.

- FFS the requirements for the cases

  - If the MO includes the serving CSI-RS resource with associated SSB

  - if the MO doesn’t include the serving CSI-RS resource and the CSI-RS resource associated SSB is configured

- FFS AGC adjustment time.

- Sub-topic 2-3: Requirement of Measurement Gap

Issue 2-3-1: intra-frequency and inter-frequency measurement with or w/o gaps?

Recommended WF:  More discussion based on the options as below in next meeting.

Requirements with or w/o gaps

  - Option 1 (CATT, CMCC, OPPO)：For CSI-RS based measurement requirement, the following scenarios are prioritized to be defined in Rel-16:

    - Intra-frequency measurement without gap

    - Inter-frequency measurement with gap

The principle for gap-needed or gapless

  - Option 1(Qualcomm, Intel, Apple, OPPO): The UE will need GAPs for CSI-RS L3 measurements if

    - The SCS of CSI-RS is different from active BWP [if UE is not capable of mixed numerology]

    - The CSI-RS is not fully confined within the active BWP

    - The CP of cells to be measured is different from that of active BWP (60 kHz SCS only)

    - FFS : The tuning time for CSI-RS based measurements that are outside UE’s active BWP will be same as that for BWP switch.

- Sub-topic 2-4: Scaling Factor

Recommended WF:

[Tentative agreement] :

Issue 2-4-2: Scaling factor for RX beam sweeping

  - If the CSI-RS is QCL-ed to the associated SSB, no Rx sweeping is needed only after SSB has been detected.

  - If CSI-RS configured with associated SSB but not QCL-ed to the associated SSB, Rx sweeping is needed. FFS on the scaling factor N =8.

  - For FR 2, if the RX beam sweeping for QCL-ed associatedSSB is not finished, requirement should not be applied.

- FFS CSSF

- Sub-topic 2-5: Scheduling Restriction

Recommended WF:

Issue 2-5-3: Whether to restrict CSI-RS resources outside of DRX/MG duration

[Tentative agreement] : No UE performance requirement is defined for the CSI-RS resources that are not within DRX on-duration or measurement gap

More discussion for issue 2-5-1, 2-5-2, 2-5-4 in next meeting.

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2005357 | Agreeable |