3GPP TSG-RAN WG4 Meeting # 94-e-Bis 								      R4-2005492

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:			6.5.2

Source:	Samsung

Title:	WF on IAB-MT reference sensitivity

Document for:	Approval

Introduction

This a WF to capture agreement on IAB-MT reference sensitivity

Agreement

  - REFSENS for FR2 IAB-MT type 2-O

The open issue on FR2 RFESENS is the decalration range for EISREFSENS_50M:

  - Declaration range of EISREFSENS_50M will be defined for each IAB-MT class respectively to either reuse or merge together different BS 2-O declaration ranges on EISREFSENS_50M as table below

| Class | EISREFSENS_50M range (dBm) |
| --- | --- |
| Wide Area | -96 to -119 |
| [Local Area] | Option 1:-86 to -109 Option 2: -86 to -114 |

  - Conducted REFSENS for FR1 IAB-MT type 1-H

It is agreed that the conducted REFSENS for IAB-MT type 1-H will be defined as:

  - For Wide Area IAB-MT: Reuse FR1 BS NF 5dB and IM 2dB

  - FFS on the other IAB-MT class (es).

  - OTA reference senstivity level for FR1 IAB-MT type 1-O

.FFS on necessity on  this requirement

  - OTA senstivity for FR1 IAB-MT type 1-H and 1-O

FR1 IAB-MT type 1-H and 1-O OTA sensitivity should be declaration based within OSDD just like FR1 BS without dedicated declaration range.

  - FRC for reference senstivty

FFS on FRC for IAB-MT in next meeting with more analysis