TSG-RAN Working Group 4 (Radio) meeting #94bis-E	R4-2005494

Electronic Meeting, April 20th – 30th 2020

Source:	Ericsson

Title:	TP to TS 38.174: Addition of OTA IAB-Node out-of-band receiver blocking requirement in subclause 10.6

Agenda item:	6.5.2.2.3

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#94-E) draft specification text for IAB-Node out-of-band blocking requirement was presented in [1]. The outcome from the discussion is captured in [2, 3].

In this contribution specification text for IAB Node OTA out-of-band blocking requirement have been created in a text proposal. The text proposal for the technical specification is attached at the end of this contribution and it is presented for approval.

This is a revised version of R4-2003759.

# Discussion

From the last meeting discussions, it has been decided to develop the OTA out-of-band requirement based on the NR BS requirement, where;

- The IAB-Node out-of-band blocking interferer level is reused from the BS requirement.

- The IAB-Node frequency range applied for OOBB requirement is reused from BS boundary between in-band blocking and out-of-band blocking

For the coming IAB-Node receiver it is reasonable to define a common requirement for out-of-band blocking applying for both IAB-MT and IAB-DU. Since the frequency range and corresponding interferer level is different for FR1 and FR2 it is suggested to split up the requirement in two subclauses.

The principles used to create the text proposal can be summarized as:

- A general section is created in subclause 10.6.1. The information in this subclause is common between FR1 and FR2.

- A section for FR1 IAB-Node out-of-band blocking requirement is created in subclause 10.6.2.

- A section for FR2 IAB-Node out-of-band blocking requirement is created in subclause 10.6.3.

- Requirement text applicable for FR1 is added in subclause 10.6.2. The text in this section is based on the BS requirement in TS 38.104, subclause 10.6.2 but adapted for an FR1 IAB-Node.

- Requirement text applicable for FR2 is added in subclause 10.6.3. The text in this section is based on the BS requirement in TS 38.104, subclause 10.6.3 but adapted for an FR2 IAB-Node.

Summary of changes in this revision;

- Wanted signal level is put in [].

- Interferer signal level is put in [].

# Conclusion

Based on decisions from previous meetings and the status given in [2, 3] a text proposal for TS 38.174, subclause 10.6 have been created. The text proposal captures specification text for OTA out-of-band blocking relevant for the IAB-Node.

The text proposal is attached at the end of this contribution and is presented for approval.

# References

[1]	R4-2001024, “TP to TS 38.174: Addition of FR2 IAB out-of-band receiver blocking requirement in subclause 10.6”, Ericsson

[2] 	R4-2002374, “Email discussion summary for RAN4#94e_#83_NR_IAB_RF_Rx”, Samsung

[3]	R4-2002483, “WF on IAB RX RF requirement”, Samsung

TEXT PROPOSAL:

## 10.6	OTA out-of-band blocking

### 10.6.1	General

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

### 10.6.2 FR1 OTA out-of-band blocking

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are from the same direction and are within the [minSENS RoAoA or refsens RoAoA].

The wanted signal applies to each supported polarization, under the assumption of polarization match. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 10.6.2-2, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in clause 10.3.2 for each [IAB-Node channel bandwidth] and further specified in [annex A.1].

For a multi-band RIB, the OTA out-of-band requirement shall apply for each supported operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to [clause 7.4.2.2] shall be excluded from the OTA out-of-band blocking requirement.

For OTA out-of-band blocking requirement apply from 30 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz. The ΔfOOB for FR1 OTA out-of-band blocking requirement is defined in table 10.6.2-1.

Table 10.6.2-1: ΔfOOB

| Operating band characteristics | ΔfOOB (MHz) |
| --- | --- |
| FUL,high – FUL,low < 100 MHz | 20 |
| 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 60 |

Table 10.6.2-2: OTA out-of-band blocking performance requirement

| Wanted signal mean power (dBm) | Interfering signal RMS field-strength (V/m) | Type of interfering Signal |
| --- | --- | --- |
| [EISminSENS + TBD dB] (Note 1) | [0.36] | CW |
| NOTE 1: EISminSENS depends on the channel bandwidth as specified in clause 9.2. NOTE 2: The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m; for example, 0.36 V/m is equivalent to 36 dBm at fixed distance of 30 m. |  |  |

### 10.6.3 FR2 OTA out-of-band blocking

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted signal applies to each supported polarization, under the assumption of polarization match. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

For IAB type 2-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL,low – 1500 MHz and from FUL,high + 1500 MHz up to 2nd harmonic of the upper frequency edge of the operating band.

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 10.6.3-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in subclause 10.3.3 for each [IAB Node channel bandwidth].

Table 10.6.3-1: OTA out-of-band blocking performance requirement

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | [EISREFSENS + TBD dB] | [0.36] | CW |
| 12750 to FUL,low – 1500 | [EISREFSENS + TBD dB] | [0.1] | CW |
| FUL,high + 1500 to 2nd harmonic of the upper frequency edge of the operating band | [EISREFSENS + TBD dB] | [0.1] | CW |