3GPP TSG-RAN WG4 Meeting #94bis-e 	R4-2005505

Online, 20 Apr - 01 May 2020

Source: 	Huawei

Title: 	TP to TR 37.941: Tx MU value derivation sub-clause 9 update (FR2)

Agenda Item:	6.19.5

Document for:	Approval

# Introduction

This TP updates the MU value derivation sections for the FR2 Tx directional requirements.

It uses the pre-approved tables agreed in R4-2004529.

After review in RAN4#94e bis, it was discussed that the TE values used in both eth Tx and Rx tables needed further discussion as such introduction of the FR2 tables in this TP are to be delayed until next meeting.

However the TP was submitted with errors as it contains FR1 update as well as the intended FR2 updates, hence the TP is corrected so that it is correct (as a base for the update next meeting when the final TE values are approved).

# TP to TR 39.941 v1.0.0

--- Start of changes ---

#### 9.2.3.4	MU value derivation, FR2

The MU assessment was carried out using a CATR chamber only however other chamber types are not precluded if suitable MU assessment is done.

A CATR MU budget was assessed in order to determine acceptable MU for the EIRP accuracy measurement in FR2. The CATR test setup and calibration and measurement procedures for FR2 are expected to be similar to those of FR1, although the test chamber dimensions and associated MU values will scale due to the shorter wavelengths and larger relative array apertures.

Table 9.2.3.4-1 captures derivation of the expanded measurement uncertainty values for EIRP accuracy measurements in CATR (Normal test conditions, FR2).

Table 9.2.3.4-1: CATR MU value derivation for EIRP accuracy measurements, Normal test conditions, FR2

| UID |  | Uncertainty source |  | Uncertainty value |  |  | Distribution of the probability |  | Divisor based on distribution shape |  | ci |  | Standard uncertainty ui [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25<f <29.5GHz | 37<f <40GHz |  |  |  | 24.25<f <29.5GHz | 37<f <40GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| A2-1a | Misalignment DUT & pointing error (EIRP) | 0.20 | 0.20 | Exp. normal | 2.00 | 1 | 0.10 | 0.10 |
| C1-7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) -High power | 0.50 | 0.70 | Normal | 1.00 | 1 | 0.50 | 0.70 |
| A2-2a | Standing wave between DUT and test range antenna | 0.03 | 0.03 | U-shaped | 1.41 | 1 | 0.02 | 0.02 |
| A2-3 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Normal | 1.00 | 1 | 0.01 | 0.01 |
| A2-4a | QZ ripple with DUT | 0.40 | 0.40 | Normal | 1.00 | 1 | 0.40 | 0.40 |
| A2-12 | Frequency flatness | 0.25 | 0.25 | Normal | 1.00 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| C1-3 | Network Analyzer | 0.30 | 0.30 | Normal | 1.00 | 1 | 0.30 | 0.30 |
| A2-5a | Mismatch of receiver chain | 0.43 | 0.57 | U-shaped | 1.41 | 1 | 0.30 | 0.40 |
| A2-6 | Insertion loss variation in receiver chain | 0.00 | 0.00 | Rectangular | 1.73 | 1 | 0.00 | 0.00 |
| A2-3 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | 0.01 | 0.01 | Normal | 1.00 | 1 | 0.01 | 0.01 |
| A2-7 | Influence of the calibration antenna feed cable | 0.21 | 0.29 | U-shaped | 1.41 | 1 | 0.15 | 0.21 |
| C1-4 | SGH Calibration uncertainty | 0.52 | 0.52 | Rectangular | 1.73 | 1 | 0.30 | 0.30 |
| A2-8 | Misalignment positioning system | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 |
| A2-1b | Misalignment of calibration antenna and test range antenna | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 |
| A2-9 | Rotary joints | 0.00 | 0.00 | U-shaped | 1.41 | 1 | 0.00 | 0.00 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 |
| A2-4b | QZ ripple calibration antenna | 0.01 | 0.01 | Normal | 1.00 | 1 | 0.01 | 0.01 |
| A2-11 | Switching uncertainty | 0.10 | 0.10 | Rectangular | 1.73 | 1 | 0.06 | 0.06 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.89 | 1.06 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.74 | 2.07 |

--- Next change ---

--- Next change ---

#### 9.3.3.4	MU value derivation, FR2

Table 9.3.3.4-1: CATR MU value derivation for EIRP accuracy measurements in Extreme test conditions, FR2

| UID | Uncertainty source | Uncertainty value |  |  |  | Distribution of the probability |  |  | Divisor based on distribution shape |  |  |  | ci |  |  |  | Standard uncertainty ui [dB] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25<f ≤29.5GHz | 37<f ≤40GHz |  |  |  |  |  |  |  |  | 24.25<f ≤29.5GHz |  |  |  | 37<f ≤40GHz |  |  |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-1a | Misalignment DUT & pointing error (EIRP) | 0.20 |  | 0.20 |  |  | Exp. normal |  |  | 2.00 |  |  |  | 1 |  |  |  | 0.10 |  |  |  | 0.10 |  |  |
| C1-7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) -High power | 0.50 |  | 0.70 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.50 |  |  |  | 0.70 |  |  |
| A2-2a | Standing wave between DUT and test range antenna | 0.03 |  | 0.03 |  |  | U-shaped |  |  | 1.41 |  |  |  | 1 |  |  |  | 0.02 |  |  |  | 0.02 |  |  |
| A2-3 | RF leakage, test range antenna cable connector terminated. | 0.01 |  | 0.01 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.01 |  |  |  | 0.01 |  |  |
| A2-13 | QZ ripple with DUT (Extreme) | 0.70 |  | 0.70 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.70 |  |  |  | 0.70 |  |  |
| A2-12 | Frequency flatness | 0.25 |  | 0.25 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.25 |  |  |  | 0.25 |  |  |
| A2-15 | radome loss variation | 0.50 |  | 0.50 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.50 |  |  |  | 0.50 |  |  |
| A2-14 | wet radome loss variation | 0.90 |  | 0.90 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.90 |  |  |  | 0.90 |  |  |
| A2-16 | Change in absorber behavior | 0.50 |  | 0.50 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.50 |  |  |  | 0.50 |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Network Analyzer | 0.30 |  | 0.30 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.30 |  |  |  | 0.30 |  |  |
| A2-5a | Mismatch of receiver chain | 0.43 |  | 0.57 |  |  | U-shaped |  |  | 1.41 |  |  |  | 1 |  |  |  | 0.30 |  |  |  | 0.40 |  |  |
| A2-6 | Insertion loss variation in receiver chain | 0.00 |  | 0.00 |  |  | Rectangular |  |  | 1.73 |  |  |  | 1 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |
| A2-3 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | 0.01 |  | 0.01 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.01 |  |  |  | 0.01 |  |  |
| A2-7 | Influence of the calibration antenna feed cable | 0.21 |  | 0.29 |  |  | U-shaped |  |  | 1.41 |  |  |  | 1 |  |  |  | 0.15 |  |  |  | 0.21 |  |  |
| C1-4 | SGH Calibration uncertainty | 0.52 |  | 0.52 |  |  | Rectangular |  |  | 1.73 |  |  |  | 1 |  |  |  | 0.30 |  |  |  | 0.30 |  |  |
| A2-8 | Misalignment positioning system | 0.00 |  | 0.00 |  |  | Exp. normal |  |  | 2.00 |  |  |  | 1 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |
| A2-1b | Misalignment of calibration antenna and test range antenna | 0.00 |  | 0.00 |  |  | Exp. normal |  |  | 2.00 |  |  |  | 1 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |
| A2-9 | Rotary joints | 0.00 |  | 0.00 |  |  | U-shaped |  |  | 1.41 |  |  |  | 1 |  |  |  | 0.00 |  |  |  | 0.00 |  |  |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 |  | 0.09 |  |  | U-shaped |  |  | 1.41 |  |  |  | 1 |  |  |  | 0.06 |  |  |  | 0.06 |  |  |
| A2-4b | QZ ripple calibration antenna | 0.01 |  | 0.01 |  |  | Gaussian |  |  | 1.00 |  |  |  | 1 |  |  |  | 0.01 |  |  |  | 0.01 |  |  |
| A2-11 | Switching uncertainty | 0.10 |  | 0.10 |  |  | Rectangular |  |  | 1.73 |  |  |  | 1 |  |  |  | 0.06 |  |  |  | 0.06 |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.56 |  |  |  | 1.66 |  |  |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.05 |  |  |  | 3.25 |  |  |  |

.

--- Next change ---

Table 9.3.4-2: Test system specific measurement uncertainty values for the EIRP accuracy in Extreme test conditions, FR2

|  | Expanded uncertainty ue (dB) |  |
| --- | --- | --- |
|  | 24.25<f <29.5GHz | 37GHz < f ≦ 40 GHz |
| Compact Antenna Test Range | 3.05 | 3.25 |
| Common maximum accepted test system uncertainty | 3.1 | 3.3 |

An overview of the MU values for all the requirements is captured in clause 16.

--- Next change ---

#### 9.10.2.3	MU value derivation, FR2

The MU assessment was carried out using a CATR chamber only. However other chamber types are not precluded if suitable MU assessment is done.

The CATR test setup and calibration and measurement procedures for FR2 are expected to be similar to those of FR1, although the test chamber dimensions and associated MU values will scale due to the shorter wavelengths and larger relative array apertures. However, it is noted that in order to achieve the test instrument uncertainties that were assumed, calibration of the spectrum analyzer may be needed.

Table 9.10.2.3-1: Compact antenna test range uncertainty assessment for EIRP measurements for transmitter OFF power and transmitter transient period

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor based on distribution shape |  | ci |  | Standard uncertainty ui [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25<f ≤29.5GHz | 37<f ≤40GHz |  |  |  |  |  | 24.25<f ≤29.5GHz |  | 37<f ≤40GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-1a | Misalignment DUT & pointing error (EIRP) | 0.20 | 0.20 | Exp. normal | 2.00 |  | 1 |  | 0.10 |  | 0.10 |
| C1-10 | RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2 | 1.25 | 1.45 | Gaussian | 1.00 |  | 1 |  | 1.25 |  | 1.45 |
| A2-2a | Standing wave between DUT and test range antenna | 0.03 | 0.03 | U-shaped | 1.41 |  | 1 |  | 0.02 |  | 0.02 |
| A2-3 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Gaussian | 1.00 |  | 1 |  | 0.01 |  | 0.01 |
| A2-4a | QZ ripple with DUT | 0.40 | 0.40 | Gaussian | 1.00 |  | 1 |  | 0.40 |  | 0.40 |
| A2-12 | Frequency flatness | 0.25 | 0.25 | Gaussian | 1.00 |  | 1 |  | 0.25 |  | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Network Analyzer | 0.30 | 0.30 | Gaussian | 1.00 |  | 1 |  | 0.30 |  | 0.30 |
| A2-5b | Mismatch of receiver chain for low power | 0.72 | 0.72 | U-shaped | 1.41 |  | 1 |  | 0.51 |  | 0.51 |
| A2-6 | Insertion loss variation in receiver chain | 0.00 | 0.00 | Rectangular | 1.73 |  | 1 |  | 0.00 |  | 0.00 |
| A2-3 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | 0.01 | 0.01 | Gaussian | 1.00 |  | 1 |  | 0.01 |  | 0.01 |
| A2-7 | Influence of the calibration antenna feed cable | 0.21 | 0.29 | U-shaped | 1.41 |  | 1 |  | 0.15 |  | 0.21 |
| C1-4 | SGH Calibration uncertainty | 0.52 | 0.52 | Rectangular | 1.73 |  | 1 |  | 0.30 |  | 0.30 |
| A2-8 | Misalignment positioning system | 0.00 | 0.00 | Exp. normal | 2.00 |  | 1 |  | 0.00 |  | 0.00 |
| A2-1b | Misalignment of calibration antenna and test range antenna | 0.00 | 0.00 | Exp. normal | 2.00 |  | 1 |  | 0.00 |  | 0.00 |
| A2-9 | Rotary joints | 0.00 | 0.00 | U-shaped | 1.41 |  | 1 |  | 0.00 |  | 0.00 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | 0.09 | U-shaped | 1.41 |  | 1 |  | 0.06 |  | 0.06 |
| A2-4b | QZ ripple calibration antenna | 0.01 | 0.01 | Gaussian | 1.00 |  | 1 |  | 0.01 |  | 0.01 |
| A2-11 | Switching uncertainty | 0.10 | 0.10 | Rectangular | 1.73 |  | 1 |  | 0.06 |  | 0.06 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.50 | 1.68 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.95 | 3.29 |

.

### 9.10.3	Maximum accepted test system uncertainty

The MU assessment was carried out using a CATR chamber only. However other chamber types are not precluded if suitable MU assessment is done.

Table 9.10.3-1: Maximum accepted test system uncertainty values for the EIRP accuracy, FR2

|  | 24.25 < f < 29.5 GHz | 37 < f < 40 GHz |
| --- | --- | --- |
| Maximum accepted test system uncertainty (dB) | 2.9 | 3.3 |

--- End of changes ---