3GPP TSG-RAN WG4 Meeting #95-e	R4-2006026

Electronic Meeting, 25 May – 5 June, 2020

Source:	ZTE

Title:	Pending issues on cell detection and serving cell measurement under NR-U

Agenda item:	6.1.5.11

Document for:	Discussion

# 1	Introduction

In RAN4 94-e, the topic of UE behaviour under certain cases in cell detection and serving cell measurement has been discussed. Companies’ views were summarized in WF [1], the content copied below:

- In RAN4 95-e, the WF [2] has been updated by adding another option:

- This paper discusses the open issues listed above in the WF and provide our view.

2	Pending issues

Issue 2-4-1:	UE behaviour in case of successive DL LBT failures during measurements

From the discussions during last meeting, most companies can agree to Option 1), which introduces a new UE behavior. In our view this is fine since it can help UE to avoid wasting power when the channel condition is extremely poor. After N unsuccessful measurement attempts due to exceeding the max number of unavailable SMTC occasions, UE should restart from the detection stage again. To us this seems like a very extreme condition because it suggests that successive DL LBT failures happen often in a period of time.

As to the value of N, we think N = 4 would be reasonable. Having N too small would force the UE to go back to detection stage and will eventually consume more power from the UE and decrease network efficiency at the same time. Having N too small may also cause ping-pang effect in some cases.

After 4 unsuccessful measurement attempts due to exceeding the max number of unavailable SMTC occasions, UE should restart from the detection stage again.

3	Conclusion

Proposal 1: After 4 unsuccessful measurement attempts due to exceeding the max number of unavailable SMTC occasions, UE should restart from the detection stage again.

References

- R4-2002278, WF on NR-U RRM requirements (Part 3), Nokia, Nokia Shanghai Bell

- R4-2005375