3GPP TSG-RAN4 Meeting #95-e	R4-2006221

# Electronic meeting, 25th May – 5th June, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0635 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on minimum requirement at transition period for UE power saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  |  | Date: |  | 2020-5-5 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RRM measurement relaxation was introduced for UE power saving, hence the minimum requirement at transition between normal mode and relaxed mode shall be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce minimum requirement at transition between normal mode and relaxed mode in power saving. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Minimum requirement at transition for power saving is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 4.2.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

4.2.2.8	Minimum requirement at transitions

When the intra/inter-frequency measurement transitions from low mobility condition or not-in-cell-edge condition to low mobility condition and not-in-cell-edge condition or vice versa during one cell-reselection period, the cell re-selection requirements with longer delay apply.

When the intra/inter-frequency measurement transitions from normal mode to low mobility condition or not-in-cell-edge condition or low mobility condition and not-in-cell-edge condition or vice versa during one cell-reselection period, the cell re-selection requirements with longer delay apply.

<< End of Change #1>>