3GPP RAN WG4 Meeting #95e		R4-2006349

Electronic meeting, 25th May – 5th June 2020

Agenda item:	7

Source:	Apple Inc.

Title:	Views on NR UE feature list for FR2

Release:	Rel-16

Document for:	Discussion

# 1	Introduction

With one quarter remaining in the Rel-16 Core requirement work, the RAN4 #94bis-e meeting is the first opportunity to collect company views on the NR UE feature list for FR2.  This contribution outlines our views on the features of interest in Section 3 of the paper.

# 2	References

- RP-200246, “Revised WID on NR RF Requirement Enhancements for FR2,” Nokia, Nokia Shanghai Bell, 3GPP RAN #87-e, March 2020

- R4-2006330, “Clarification for the definition of the UL duty cycle,” Apple Inc., 3GPP RAN4 #95e, May 2020

- R4-2006332, “Further considerations on the uplink duty cycle enhancements for the MPE scenario,” Apple Inc., 3GPP RAN4 #95e, May 2020

- R4-2006357, “Remaining issues with beam correspondence in Rel-16,” Apple Inc., 3GPP RAN4 #95e, May 2020

- R4-2006634, “CR to 38.101-2 on FR2 frequency separation class enhancement,” Apple Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, 3GPP RAN4 #95e, April 2020

# 3	Proposals for the feature list

## 3.1	Maintenance of NR RF features for FR2

As described in [2], the feature description for maxUplinkDutyCycle-FR2 needs to be updated to clarify the reference UE output power.  Table 1 below provides the proposed wording.

Table 1: Proposed correction of maximum uplink duty cycle for FR2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. UE RF | 2-15 | Maximum uplink duty cycle for FR2 | 1) Maximum percentage of uplink transmission time that can be scheduled within 1s time window at the UE maximum total radiated power in order to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. The value range is {15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%}. |  | Yes | N/A | UE is not able to provide the maximum UL duty cycle information to the network, thereby leading to less efficient scheduling of UL resources | Type 2 (per band) | No | Applicable only to FR2 | N/A | Indicates the maximum percentage of symbols during 1s that can be scheduled for uplink transmission at the UE maximum total radiated power so as to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. This field is applicable for all power classes UE in FR2 as specified in TS38.101-2 [3]. Value n15 corresponds to 15%, value n20 corresponds to 20% and so on. If the field is absent or the percentage of uplink symbols transmitted within any 1s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE behaviour is specified in TS38.101-2 [3]. | Optional with capability signaling |

Proposal 1:	Introduce the correction of the UE capability for maxUplinkDutyCycle-FR2 as proposed in Table 1.

## 3.2	NR RF enhancements for FR2

With the work item on NR RF enhancements for FR2 nearing completion [1], our views on a number of relevant features are provided in [3], [4], [5], Error! Reference source not found., Error! Reference source not found..  Table 2 below summarizes the impact of these proposals on the UE feature list.

Table 2: Proposed features for NR RF enhancements in FR2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBD | TBD | MPE | P-MPR reporting. UE capability to report applied P-MPR. |  | Yes | N/A | Potential radio link failure as the UE applies P-MPR to comply with RF exposure regulation | Type 1 (per UE) | N/A | Applicable to FR2 only | N/A |  | Optional with capability signaling |
| TBD | TBD | Beam correspondence | [1) UE is capable of a beam refinement procedure based only on the SSB reference signal (FFS whether this component will be introduced)] 2) UE is capable of a beam refinement procedure based only on the CSI reference signal (FFS whether this component will be introduced)] | 2-20 | Yes | N/A | UE is not capable of enhanced beam refinement procedures | Type 1 (per UE) | N/A | Applicable to FR2 only | N/A |  | Optional with capability signaling |
| TBD | TBD | Non-contiguous intra-band CA frequency separation class for FR2 | 1) Support of frequency separation classes to handle the total frequency span for DL for intra-band non-contiguous CA as specified in TS38.101-2 2) Support of frequency separation classes to handle the total frequency span for UL for intra-band non-contiguous CA as specified in TS38.101-2 3) Support of Frequency separation class for DL-only frequency separation span of DL CCs that UE can support per band. The spectrum covered by the DL-only frequency separation extends on one-side relative to the bidirectional spectrum. The sum value of the frequency separation (Fs) and frequency separation class for DL-only spectrum (Fsd) cannot exceed 2400 MHz |  | Yes | N/A | UE is not capable of supporting non-contiguous carrier aggregation configurations | Type 3 (per band per band combination) | N/A | Applicable to FR2 only | N/A |  | Mandatory to support a frequency separation class within {I, II, III} specified in TS38.101-2 with capability if UE supports non-contiguous CA in FR2 |

Proposal 2:	Introduce UE RF features related to FR2 as shown in the Table 2.