3GPP TSG-RAN WG4 Meeting#95-e	R4-2006615

Online, 25th May – 5th Jun, 2020

Source: 	Qualcomm Incorporated

Title: 	Parameters and simulation results on PMI reporting requirements with larger number of Tx ports

Agenda item:	6.18.1.2

Document for:	Discussion

# Introduction

In WF [1], some of the test parameters were discussed and options for different parameters were provided. In this paper, we discuss our views on those parameters for PMI reporting requirements with larger number of Tx ports.

# Test Parameters

Type I SB PMI Reporting

In this section, we discuss our view on different parameters for PMI reporting requirements for larger number of Tx ports.

In the last meeting, the question of whether to define the subband (SB) PMI requirements or not, was left open. Simulation assumptions were agreed in [2] to evaluate subband and WB PMI reporting. Below are our simulation results under TDLC300-5 channel model for FDD.

Table 1: Simulation results for WB vs SB PMI reporting under TDLC300-5 channel model

| Test case | SNR in dB at 90% of peak throughput | PMI ratio |
| --- | --- | --- |
| SB PMI, 2Rx | 14.49 | 3.24 |
| SB PMI, 4Rx | 9.81 | 3.95 |
| WB PMI, 2Rx | 14.49 | 2.82 |
| WB PMI, 4Rx | 9.89 | 3.36 |

As PMI ratios for SB PMI are better than WB PMI, and it was already agreed to define WB PMI test cases for 32 Tx ports, RAN4 should define subband PMI tests for 16Tx ports to have a good coverage and performance. Therefore, we propose the following.

Proposal 1: Define subband Type -I PMI reporting requirements for 16 Tx ports.

Type II PMI Reporting

In [1], there were two options listed for test setup: SU-MIMO and MU-MIMO. As RAN4 is discussing to define minimum requirements for UE and UE implementation is unaware of whether it is SU-MIMO setup or MU-MIMO setup, it does not make sense to complicate the setup without testing anything new on UE implementation. Therefore, we propose the following.

Proposal 2: Use SU-MIMO test setup for defining Type II PMI reporting tests.

Type II codebook has two components: Wideband and Subband. So, it makes more sense to have Subband PMI reporting for Type II codebook so that this codebook can be used to its full potential. Therefore, we propose the following.

Proposal 3: Use subband PMI reporting for defining Type II PMI reporting tests.

In [1], there were many options listed for each parameter to define Type II PMI reporting requirements. We present simulation results below for different cases based on [3] assuming subband PMI reporting and SU-MIMO setup. We use the test metric as throughput ratio between following PMI and random PMI.

Table 2: Simulation results for Type II subband PMI reporting for TDD 16x2 XP High

| N_PSK | subbandAmplitude | SNR in dB at 90% of peak throughput | PMI ratio |
| --- | --- | --- | --- |
| 4 | TRUE | 13.57 | 3.04 |
| 4 | FALSE | 13.80 | 2.99 |
| 8 | TRUE | 10.04 | 5.88 |
| 8 | FALSE | 10.29 | 5.49 |

Based on above simulation results, we can clearly see that N_PSK = 8 and subbandAmplitude = true provide the best throughput ratios. Therefore, we propose the following.

Proposal 4: Define Type II PMI reporting requirements with N_PSK = 8 and subbandAmplitude = true

Next, we compare XP High vs XP Medium performance based on simulation results in Table 3 below.

Table 3: Simulation results for Type II PMI reporting for TDD XP High, N_PSK =8, subbandAmplitude = true

| NTx x NRx | SNR in dB at 90% of peak throughput | PMI ratio |
| --- | --- | --- |
| 16x2 | 10.04 | 5.88 |
| 16x4 | 6.46 | 6.47 |
| 32x2 | 6.77 | 30.91 |
| 32x4 | 3.68 | 27.41 |

Based on above simulations, we notice that 32Tx ports provide too large throughput ratios compared to 16Tx ports. Therefore, we propose the following.

Proposal 5: Define Type II PMI reporting requirements for only 16Tx ports.

Next, we compare XP High vs XP Medium performance in Table 4 below.

Table 4: Simulation results for Type II PMI reporting for TDD N_PSK =8, subbandAmplitude = true

| Case | SNR in dB at 90% of peak throughput | PMI ratio |
| --- | --- | --- |
| 16x2 XP High | 10.04 | 5.88 |
| 16x4 XP High | 6.46 | 6.47 |
| 16x2 XP Med | 11.13 | 3.19 |
| 16x4 XP Med | 7.65 | 2.76 |

As we have defined other PMI reporting tests with XP High correlation and based on results in Table 4, XP High provides better performance than XP Medium correlation, we propose the following.

Proposal 6: Define Type II PMI reporting requirements for XP High MIMO correlation.

Another open issue in the last meeting was beam steering approach. As we have already agreed to use 2 beams for Type II codebook, there is no need to discuss extending the beam steering approach to more independent clusters under this WI. Therefore, we propose the following.

Proposal 7: Discuss extension of beam steering approach to more than 2 clusters under eMIMO WI and use the 2 cluster beam steering approach from 36.101 for defining Type II PMI reporting requirements under NR performance enhancement WI.

# Simulation Results for Type I PMI Reporting

As per simulation assumptions in [3], we provide the simulation results below.

Table 5: Simulation results for 32Tx ports WB PMI reporting

| Test case | SNR in dB at 90% of peak throughput | PMI ratio |
| --- | --- | --- |
| FDD, 2Rx | 12.41 | 6.55 |
| FDD, 4Rx | 7.95 | 11.13 |
| TDD, 2Rx | 13.02 | 5.29 |
| TDD, 4Rx | 8.64 | 9.56 |

# Conclusions

This paper proposes parameters related to PMI reporting requirements for larger number of Tx ports and also presents simulation results for Type I WB PMI reporting. Following has been proposed:

Proposal 1: Define subband Type -I PMI reporting requirements for 16 Tx ports.

Proposal 2: Use SU-MIMO test setup for defining Type II PMI reporting tests.

Proposal 3: Use subband PMI reporting for defining Type II PMI reporting tests.

Proposal 4: Define Type II PMI reporting requirements with N_PSK = 8 and subbandAmplitude = true

Proposal 5: Define Type II PMI reporting requirements for only 16Tx ports.

Proposal 6: Define Type II PMI reporting requirements for XP High MIMO correlation.

Proposal 7: Discuss extension of beam steering approach to more than 2 clusters under eMIMO WI and use the 2 cluster beam steering approach from 36.101 for defining Type II PMI reporting requirements under NR performance enhancement WI.

# References

[1] R4-2005549, “Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, 3GPP TSG RAN WG4 Meeting #94-bis-e, Online, Apr 20 - Apr 30, 2020.

[2] R4-1915859, “Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports”, 3GPP TSG RAN WG4 Meeting #93, Reno, USA, Nov 18-22, 2019.

[3] R4-2005550, “Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports”, 3GPP TSG RAN WG4 Meeting #94-bis-e, Online, Apr 20 - Apr 30, 2020.