3GPP TSG RAN WG4 Meeting #95-e	R4-2006679

Online, 25th May – 5th June 2020

Agenda item:	8.4.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-11: EN-DC_1-42_n28

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_1-42_n28 was approved in RAN#87[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-42_n28 | CA_1-42 | n28 | No |

Table 2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-42A_n28A | DC_1A_n28A DC_42A_n28A | CA_1A-42A | n28A |
| DC_1A-42C_n28A | DC_1A_n28A DC_42A_n28A DC_42C_n28A | CA_1A-42C | n28A |

3.  Co-existence

When Uplink EN-DC configuration is DC_1A_n28A, (1) IMD3 of (B1 - n28) will fall into Rx band of Band 42. When Uplink EN-DC configuration is DC_42A_n28A, (2) IMD3 of (B42 - n28) will fall into Rx band of Band 1. Thus, it is proposed to evaluate (1) and (2).

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 3: ΔTIB of constituent 2 band combo.

| ΔTIB | B1+B42 | B1+n28 | B42+n28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 |  | 0.3 |
| B42 | 0.8 |  | 0.5 | 0.8 |
| n28 |  | 0.6 | 0.8 | 0.8 |

Table 4: ΔRIB of constituent 2 band combo.

| ΔRIB | B1+B42 | B1+n28 | B42+n28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0 | 0 |  | 0 |
| B42 | 0.5 |  | 0.2 | 0.5 |
| n28 |  | 0.2 | 0.5 | 0.5 |

Then the proposal for this 2LTE-1NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n28 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n28 | 1 | 0 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |

5.  REFSENS

As mentioned in section 3, REFSENS exceptions due to IMD3 are expected. Based on the MSD analysis results of DC_1-28_n78 and DC_1_n28-n78, the following values are proposed.

Table 5: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | 42 | 3416 | 5 | 25 | 3416 | 15.7 | TDD | IMD3 |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 | 3580 | N/A | TDD | N/A |
|  | n28 | 723 | 5 | 25 | 778 | N/A | FDD | N/A |
|  | 1 | 1944 | 5 | 25 | 2134 | 15.7 | FDD | IMD3 |

6.  Conclusion

This paper proposes study results of EN-DC 1-42_n28.

TP is shown in Annex.

7.  Reference

[1]  RP-200271  WID revision: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL); rapporteur: Huawei

Annex.   Proposed text for TR37.716-21-11:

[Unchanged Parts Skipped]

5.1.x	DC_1-42_n28

### 5.1.x.1	Operating bands for DC_1-42_n28

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-42_n28 | CA_1-42 | n28 | No |

### 5.1.x.2	Configurations for DC_1-42_n28

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-42A_n28A | DC_1A_n28A DC_42A_n28A | CA_1A-42A | n28A |
| DC_1A-42C_n28A | DC_1A_n28A DC_42A_n28A DC_42C_n28A | CA_1A-42C | n28A |

### 5.1.x.3	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n28A, (1) IMD3 of (B1 - n28) will fall into Rx band of Band 42. When Uplink EN-DC configuration is DC_42A_n28A, (2) IMD3 of (B42 - n28) will fall into Rx band of Band 1.

### 5.1.x.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n28 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |

Table 5.1.x.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n28 | 1 | 0 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |

### 5.1.x.5	REFSENS requirements

As mentioned above, IMD3 of B1 and n28 to Band42 Rx and IMD3 of B42 and n28 to Band1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | 42 | 3416 | 5 | 25 | 3416 | 15.7 | TDD | IMD3 |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 | 3580 | N/A | TDD | N/A |
|  | n28 | 723 | 5 | 25 | 778 | N/A | FDD | N/A |
|  | 1 | 1944 | 5 | 25 | 2134 | 15.7 | FDD | IMD3 |

[Unchanged Parts Skipped]