3GPP TSG-RAN WG4 Meeting #95-e	R4-2006776

May-Jun 2020

Title:	FR2 23dBm FWA pk. EIRP and REFSENS budgets

Source:	Qualcomm Incorporated

Agenda item:	10.1.1

Release:	Rel-17

Work Item:	NR_FR2_FWA_Bn257_Bn258

Responsible WG:	RAN4

Document for:	Approval

# 1.	Introduction

In this contribution, we present out proposals for pk. EIRP and REFSENS respectively, as outlined in [2]. Our estimates are built around a 16-element array.

# 2. 	Discussion

## 2.1	On using an existing power class

The WID [1] explicitly identifies Japan as the target geographic region for deployment of the proposed UE type. While it is possible to reduce the output power per element in PC1 to stay compliant with the TRP limit (based on Pmax signalling or similar) [4], it may not be a complete solution. Additional signalling must be instituted to convey to the UE other considerations new to existing FR2 UEs, like the upper bound on antenna gain in Japan. We address these aspects in greater detail in [3]

RAN4 would benefit from clarification of Japanese regulation on max. antenna gain, and if there are any changes anticipated.

## 2.2	New power class for Japan FWA

In case RAN4 decides to proceed with definition of a dedicated new power class, referred to as PC5 for the rest of the contribution, we present our estimates for peak EIRP and REFSENS assume a single 16-element array, with each element carried over from PC3 application.

### 2.2.1 Peak EIRP

See table 2.2.1-1 for our estimated peak EIRP. The estimate accounts for PVT corners, and further modified by calibration accuracy. The estimate below also includes the effect of PA output limiting to ensure that the UE stays under the hard TRP limit of 23.0 dBm with margin (our target upper bound for TRP is 22.0 dBm). Finally, we assume that the UE has 2 identical Tx chains arranged to transmit in mutually orthogonal polarizations, with each radiating element capable of transmitting both polarizations simultaneously. This type of implementation was used in Rel-15 UEs. For Rel-16, UL full power Tx modes defined in RAN1 further clarify this arrangement when a UE is configured for 2 or more SRS ports. For our assumed UE topology, the UEs would declare ULFPTx mode 1 or mode 2, pending detailed discussions in the Rel-16 e-MIMO WI. Note that the UE is still expected to support Rel-15 behaviour, where full power is available when the UE is configured for a single SRS port.

Observation 1: The FR2 23 dBm FWA UE may use ULFPTx PA mode 1 or PA mode 2 to deliver peak EIRP.

| PC5 peak EIRP budget | n258 |  | n257 |  |
| --- | --- | --- | --- | --- |
|  | Low extremity | High extremity | Low extremity | High extremity |
| Pout per element (w/ cal uncert) (dB) | 7.5 | 10.5 | 7.5 | 10.5 |
| # elem | 16 | 16 | 16 | 16 |
| total conducted power per pol (dBm) | 19.5 | 22.5 | 19.5 | 22.5 |
| ant. element gain |  |  |  |  |
| gain roll-off |  |  |  |  |
| realized array gain (dB) | 13.6 | 16.6 | 14.1 | 16.9 |
| pol gain (dB) | 2.0 | 3.0 | 2.0 | 3.0 |
| mismatch and TL loss |  |  |  |  |
| BF loss (phase shifter ) |  |  |  |  |
| finite beam table |  |  |  |  |
| BF loss (one table fits all) |  |  |  |  |
| form factor loss |  |  |  |  |
| total implementation loss (dB) | 6.5 | 3.5 | 7.7 | 3.5 |
| EIRP (dBm) | 28.7 | 38.7 | 28.0 | 39.0 |
| TRP (dBm) | 15.0 | 22.0 | 13.8 | 22.0 |

Table 2.2.1-1: FR2 23 dBm FWA peak EIRP estimate

Observation 2: Min. peak EIRP estimate for FR2 23 dBm FWA is 28.7 dBm and 28.0 dBm, respectively, for n258 and n257

### 2.2.2 REFSENS

Our REFSENS estimates are captured in table 2.2.2-1. Receive chain NF accounts for PVT corners. Our REFSENS assumptions draw from Rel-15 agreements [5]:

- DL incidence:	Plane wavefront, single AoA

- DL polarization:	Linearly polarized DL signal, polarization plane agnostic of UE pol. axes

- Reference plane:	DL power measured by 0dBi antenna at ‘radiated requirements reference point’

- Receiver topology: Dual-pol receiver with MRC

| PC5 REFSENS budget | n258 | n257 |
| --- | --- | --- |
|  | Worst Case | Worst Case |
| SNR (dB) | -1.0 | -1.0 |
| BW (MHz) | 50.0 | 50.0 |
| thermal noise floor (dBm/MHz) | -173.8 | -173.8 |
| noise figure (dB) | 9.0 | 9.0 |
| num elem | 16 | 16 |
| element gain |  |  |
| element gain roll off |  |  |
| realized array gain (dB) | 13.6 | 14.1 |
| diversity gain (combining o/h) | -0.8 | -0.8 |
| bf loss + impl. Loss + fll |  |  |
| form factor loss |  |  |
| total implementation loss (dB) | 8.3 | 9.5 |
| REFSENS (dBm) | -94.2 | -93.5 |

Table 2.2.2-1: FR2 23 dBm FWA REFSENS estimate

Observation 3: REFSENS estimate for FR2 23 dBm FWA is -94.2 dBm and -93.5 dBm, respectively, for n258 and n257, assuming -1 dB target SNR and 50 MHz CC

## 2.3	Multi-band relaxations

It is unclear at this time if multi-band relaxations will be required for a fundamentally non-portable device, and one with relaxed packaging constraints and potential for favourable material choice for radome.

## 2.4	Beam correspondence

The PC5 UE is intended as an FWA device. The WID [1] suggested parallels to PC1 for deriving spherical coverage requirements, and consensus in the WF [2] has confirmed this common understanding. Beam correspondence as defined in RAN4 is closely tied to spherical coverage, so it follows that PC5 should adopt PC1 practice for beam correspondence also. Note that a PC1 variant that can only support beam correspondence with assistance from UL beam sweeping (‘bit 0 UE’) is not defined in Rel-15. To introduce such a variant for PC5, whose mission is the same as for PC1, would represent standards regression.

Proposal 1: The FR2 23 dBm FWA UE that declares beamCorrespondenceWithoutUL-BeamSweeping = 0 shall not be supported

# 3.	Conclusion

Our estimates for min. peak EIRP were presented, and the following observations were made:

Observation 1: The FR2 23 dBm FWA UE may use ULFPTx PA mode 1 or PA mode 2 to deliver peak EIRP.

Observation 2: Min. peak EIRP estimate for FR2 23 dBm FWA is 28.7 dBm and 28.0 dBm, respectively, for n258 and n257

Our estimates for REFSENS were presented, and the following observations were made:

Observation 3: REFSENS estimate for FR2 23 dBm FWA is -94.2 dBm and -93.5 dBm, respectively, for n258 and n257, assuming -1 dB target SNR and 50 MHz CC

Regarding beam correspondence for PC5, we proposed:

Proposal 1: The FR2 23 dBm FWA UE that declares beamCorrespondenceWithoutUL-BeamSweeping = 0 shall not be supported

# 4. 	References

[1]	RP-200503, ‘WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258’, Softbank, RAN#87-2, Mar 2020

[2]	R4-2005177, ‘WF on FR2 new FWA UE RF and RRM requirements’, Huawei et al, RAN4#94Bis-e, Apr 2020

[3] R4-2008053, ‘p-max feasibility for Fr2’, Qualcomm, RAN4#95-e, May-Jun 2020

[4]	R4-2004429, ‘How to introduce FR2 new power class feature’, NTTDocomo, RAN4#94-e-Bis, Apr 2020

[5]	R4-1900671, ‘On FR2 EIS’, Qualcomm, RAN4 #90, Athens, GR, Feb-Mar 2019

# 5. 	Appendix

RP-200503, the WID for ‘… Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258’ has the following objectives: