3GPP TSG-RAN WG4 Meeting #95-e 	R4-2006815

# Online, May 25th – June 5th, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0169 | rev | - | Current version: | 15.9.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-2: Intra-band non-contiguous CA configuration clarifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - This is a resubmission of endorsed draft CR R4-2005201. - NOTE 1 in Table 5.5A.2-1 and Table 5.5A.2-2 for intra-band non-contigous CA configurations was removed in CR R4-1907999 to align with the non-contiguous CA channel spacing definiton in Clause 5.4A.1. That means the minimum frequency gap between sub-blocks in FR2 is not constrained at 50 MHz. As a result, the maximum bandwidth from the summation of sub-block bandwidths for some CA configuations need to be re-calculated. - Term “Maximum aggregated bandwidth” in Table 5.5A.2-1 and Table 5.5A.2-2 was replaced by (BWChannel,block), the summation of sub-block bandwidths in CR R4-2002922. However, (BWChannel,block) can be any value from the summation for all supported sub-block bandwidths. It is better to clarify the real meaning of (BWChannel,block) so that the numbers specified in this column are consistent with the (BWChannel,block) definition. - A few CA configurations where the values under the (BWChannel,block) column still have NOTE 1 index which should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add NOTE 5 in Table 5.5A.2-1 and NOTE 4 in Table 5.5A.2-2 to clarify the definition of (BWChannel,block) which should be “the maximum total bandwidth from the summation of the sub-block bandwidths and shall be less than the bandwidth of the operating band”. - Remove NOTE 1 index for values under (BWChannel,block) column for certain CA configurations which should have been removed in previous CR R4-1907999. - Re-calculate the maximum total bandwidth for some CA configurations to align with the (BWChannel,block) definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The meaning of (BWChannel,block) is not clearly defined which may cause confusion on how the values under this column were derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changed sections>>>

### 5.5A.2	Configurations for intra-band non-contiguous CA

Configurations listed in this clause apply to downlink carrier aggregation only.

Table 5.5A.2-1: NR CA configurations with single CA bandwidth class defined for intra-band non-contiguous CA

| NR configuration | Uplink CA configurations | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | (BWChannel,block) (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n257(2A) | - | n257A | n257A |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2A) | - | n260A | n260A |  |  |  |  |  |  | 800 | 0 |
| CA_n260(3A) | - | n260A | n260A | n260A |  |  |  |  |  | 1200 | 0 |
| CA_n260(4A) | - | n260A | n260A | n260A | n260A |  |  |  |  | 1600 | 0 |
| CA_n261(2A) | - | n261A | n261A |  |  |  |  |  |  | 800 | 0 |
| CA_n261(3A) | - | n261A | n261A | n261A |  |  |  |  |  | 800 | 0 |
| CA_n261(4A) | - | n261A | n261A | n261A | n261A |  |  |  |  | 800 | 0 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: Channel bandwidth per operating band defined in Table 5.3.5-1 NOTE 5: (BWChannel,block) denotes the maximum total bandwidth from the summation of the sub-block bandwidths and shall be less than the bandwidth of the operating band. |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5A.2-2: NR CA configurations and bandwidth combination sets for intra-band non-contiguous CA

| CA configuration | Uplink CA configurations | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | (BWChannel,block) (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n260(A-I) | CA_n260I | n260A | CA_n260I |  |  |  |  |  | 800 | 0 |
| CA_n260(D-G) | CA_n260D CA_n260G | CA_n260D | CA_n260G |  |  |  |  |  | 600 | 0 |
| CA_n260(D-H) | CA_n260D CA_n260H | CA_n260D | CA_n260H |  |  |  |  |  | 700 | 0 |
| CA_n260(D-I) | CA_n260D CA_n260I | CA_n260D | CA_n260I |  |  |  |  |  | 800 | 0 |
| CA_n260(D-O) | CA_n260D CA_n260O | CA_n260D | CA_n260O |  |  |  |  |  | 600 | 0 |
| CA_n260(D-P) | CA_n260D CA_n260P | CA_n260D | CA_n260P |  |  |  |  |  | 700 | 0 |
| CA_n260(D-Q) | CA_n260D CA_n260Q | CA_n260D | CA_n260Q |  |  |  |  |  | 800 | 0 |
| CA_n260(E-O) | CA_n260E CA_n260O | CA_n260O | CA_n260E |  |  |  |  |  | 800 | 0 |
| CA_n260(E-P) | CA_n260E CA_n260P | CA_n260E | CA_n260P |  |  |  |  |  | 900 | 0 |
| CA_n260(E-Q) | CA_n260E CA_n260Q | CA_n260E | CA_n260Q |  |  |  |  |  | 1000 | 0 |
| CA_n260(G-I) | CA_n260G CA_n260I | CA_n260G | CA_n260I |  |  |  |  |  | 600 | 0 |
| CA_n261(D-G) | CA_n261D CA_n261G | CA_n261D | CA_n261G |  |  |  |  |  | 600 | 0 |
| CA_n261(D-H) | CA_n261D CA_n261H | CA_n261D | CA_n261H |  |  |  |  |  | 700 | 0 |
| CA_n261(D-I) | CA_n261D CA_n261I | CA_n261D | CA_n261I |  |  |  |  |  | 800 | 0 |
| CA_n261(D-O) | CA_n261D CA_n261O | CA_n261D | CA_n261O |  |  |  |  |  | 600 | 0 |
| CA_n261(D-P) | CA_n261D CA_n261P | CA_n261D | CA_n261P |  |  |  |  |  | 700 | 0 |
| CA_n261(D-Q) | CA_n261D CA_n261Q | CA_n261D | CA_n261Q |  |  |  |  |  | 800 | 0 |
| CA_n261(E-O) | CA_n261E CA_n261O | CA_n261E | CA_n261O |  |  |  |  |  | 800 | 0 |
| CA_n261(E-P) | CA_n261E CA_n261P | CA_n261E | CA_n261P |  |  |  |  |  | 800 | 0 |
| CA_n261(E-Q) | CA_n261E CA_n261Q | CA_n261E | CA_n261Q |  |  |  |  |  | 800 | 0 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Unless otherwise stated, BCS0 is referred in each constituent CA configuration NOTE 4: (BWChannel,block) denotes the maximum total bandwidth from the summation of the sub-block bandwidths and shall be less than the bandwidth of the operating band. |  |  |  |  |  |  |  |  |  |  |

<<< End of changed sections>>>