3GPP TSG-RAN4  #95-e    													           R4-2006820

25th May – 5th June 2020

Source: 	Qualcomm Incorporated

Title: 	TP on 5G V2X MPR_AMPR for S-SSB transmission

Agenda item:	6.4.3.1

Document for:	Approval

# Introduction

In RAN4-94e-bis a WF was agreed for capturing the MPR and AMPR requirements for S-SSB transmission [1].  Based on [1] the MPR and AMPR specification tables are presented below.

****************** Start of the TP in clause 8.1.2.1 & 8.1.3 of TR38.886 ************************

# 8	Transmitter characteristics

## 8.1	NR V2X UE Tx requirements in FR1

### 8.1.2	UE maximum output power reduction

#### 8.1.2.1	MPR for Power class 3 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be specified as inner/outer RB allocations in Table 8.1.2.1-1 for power class 3.

Table 8.1.2.1-1: Maximum Power Reduction (MPR) for power class 3 V2X (Contiguous PSCCH and PSSCH transmission)

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ [4.5] | ≤ [2.0] |
|  | 16QAM | ≤ [4.5] | ≤ [2.0] |
|  | 64 QAM | ≤ [4.5] |  |
|  | 256 QAM | ≤ [7.0] |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 in TS38.101-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

For single V2X UE’s PSFCH transmission for PC3 NR V2X UE, the required MPR is defined as follow

MPR_PSFCH =  [3.5] dB

For contiguous and non-contiguous allocation for simultaneous PSFCH transmission for PC3 NR V2X UE, the required MPR are specified as follow

MPR_PSFCH = CEIL {MA_PSFCH, 0.5}

Where MA is defined as follows

MA_PSFCH  =	7.5		; 0.00 <  NGap/NRB  ≤ 0.55

= 12.0	; 0.55 < NGap/NRB ≤ 1.0

Where

NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart)

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The allowed MPR for the maximum output power for V2X physical channels on S-SSB transmission are specified as follow

Table 8.1.2.1-2: Maximum Power Reduction (MPR) for S-SSB transmission for power class 3 V2X

|  |  |  | MPR (dB) |  |
| --- | --- | --- | --- | --- |
| Carrier Frequency (MHz) | Channel BW (MHz) | SCS (kHz) | Outer RB Allocations2 | Inner RB Allocations2 |
| TBD | 10 | 15, 30 | ≤ 4.8 | ≤ 1.6 |
|  |  | 60 | ≤ 5.4 | na |
| TBD | 20 | 15, 30, 60 | ≤ 5 | ≤ 1.6 |
|  | 30 | 15, 30 | ≤ TBD | ≤ 1.6 ≤ 2 |
|  |  | 60 |  |  |
|  | 40 | 15, 30 | ≤ 5.5 | ≤ 2 |
|  |  | 60 |  |  |
| NOTE 1: Channel center RB placement designates the signal to be placed at the center of the band, it can take advantage of the lowest possible MPR. NOTE 2: Inner and Outer RB allocations are defined in section 6.2.2 in TS38.101-1. |  |  |  |  |

### 8.1.3	UE maximum output power with additional requirements

To comply the EN 302 571 standard emission mask, RAN4 need to derive A-MPR requirements considering with the additional A-SEM and A-SE requirements in EN 302 571 standard as shown in Table 8.1.3-1 and Table 8.1.3-2.

NR V2X UE shall satisfy the additional SEM and additional SE requirements when NS_33 is configured from pre-configured radio parameters or the cell and the indication from upper layers has indicated was informed.

Table 8.1.3-1: Additional SEM requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex G in TS38.101-1.

Table 8.1.3-2: Additional SE requirements

| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43 |
| NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. |  |  |  |  |  |  |  |

Also, FCC had regulatory requirements for 40MHz in ITS spectrum as shown in Table 8.1.3-3

Table 8.1.3-3: Additional SEM requirements for 40MHz channel bandwidth (fc =5885 MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0 - 2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40 - 100 | -50 | 100kHz |

To comply the regional regulatory requirements, RAN4 specifiy the A-MPR requirements according to different network signalling by pre-configured radio parameters or the cell and the indication from upper layers has indicated to NR V2X UE.

The allowed additional Maximum Power Reduction (A-MPR) for the maximum output power due to higher order modulation and transmit bandwidth configuration (resource blocks) will be specified in TS38.101-1 as below.

Table 8.1.3-4: Additional Maximum Power Reduction (A-MPR) for NR V2X

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_33 | Table 8.1.3-1 (A-SEM) Table 8.1.3-2 (A-SE) | n47 | 10 | Table 8.1.3-5 Table 8.1.3-6 Table 8.1.3-7 |  |
| NS_48 | Table 8.1.3-3 (A-SEM) | n47 | 40 | Table 8.1.3-8 |  |

Table 8.1.3-5: A-MPR for NS_33 (15kHz SCS)

| Carrier frequency(MHz) | Resources Blocks (NRB) Note1 | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | [≤15] | [0] | [20] |  |  |
|  | [15> and ≤25] | [0] | [18.5] |  |  |
|  | [25 > and ≤40] | [0] | [17] |  |  |
|  | [> 40] | [0] | [15.5] |  |  |
|  | [≤ 40] | [10] | [12] |  |  |
|  | [≤ 30] | [15] | [9.5] |  |  |
|  | [≤ 30] | [20] | [7.5] |  |  |
|  | [≤ 25] | [25] | [6] |  |  |
|  | [≤ 20] | [> 30] | [5] |  |  |
| 5870, 5910, 5920, 5880, 5890, 5900 | [≤ 50] | [0] | [3] |  | [4] |
|  | [≤ 40] | [10] | [2.5] | [2.5] | [4] |
|  | [≤ 20] | [15, 20, 40] | [1] | [2] | [4] |
|  | [20 > ] | [20] | [2.5] | [2.5] | [4] |
|  | [≤ 25] | [25, 30] | [2.5] | [2.5] | [4] |
| Note 1: The resource blocks are limited by LCRB. LCRB = {10,15,20,25,30,40,45,50}. |  |  |  |  |  |

Table 8.1.3-6: A-MPR for NS_33 (30kHz SCS)

| Carrier frequency(MHz) | Resources Blocks (NRB) Note1 | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | [10,15] | [0] | [19] |  |  |
|  | [20] | [0] | [17] |  |  |
|  | [10] | [10] | [9] |  |  |
| 5870, 5910, 5920, 5880, 5890, 5900 | [≤ 20] | [0] | [2.5] |  | [4] |
|  | [10] | [10] | [1.5] | [2] | [4] |
| Note 1: The resource blocks are limited by LCRB. LCRB = {10,15,20}. |  |  |  |  |  |

Table 8.1.3-7: A-MPR for NS_33 (60kHz SCS)

| Carrier frequency(MHz) | Resources Blocks (NRB) | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | [10] | [0] | [18] |  |  |
| 5870, 5910, 5920 |  |  | [5] |  |  |
| 5880, 5890, 5900 |  |  | [2.5] |  | [4.5] |

Table 8.1.3-8: A-MPR for NS_48

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocation |
| 5885 | QPSK | ≤ (9.5 + ΔNote1) | [≤ 8.0] | [≤ 3.5] |
|  | 16QAM |  | [≤ 8.0] | [≤ 3.5] |
|  | 64QAM |  | [≤ 8.5] | [≤ 4.5] |
|  | 256QAM |  | [≤ 8.5] | [≤ 6.0] |
| Note1: Δ is 0, 3, and 5 for 60kHz, 30kHz, and 15kHz SCS, respectively. |  |  |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

RBStart,Low = max(1, floor(NRB /3.5))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ ceil(NRB/3.5)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ floor(NRB*0.2) RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

[Editor Notes]: A-MPR requirements for PSSCH/PSCCH, simultaneous PSFCH and S-SSB will be updated based on the RAN4 agreements. The captured A-MPR requirements for PSSCH/PSCCH is based on approved TP (R4-2002783)

The AMPR tables for ETSI (NS_New) and ETSI with coexistence (NS_33) are given below:

Table 8.1.3-9: A-MPR for S-SSB transmission for 15kHz SCS for NS 33 or NS NEW in n47

| Carrier Frequency (MHz) | RBStart | A-MPRBase (dB) | AMPR1Step (dB) |
| --- | --- | --- | --- |
| 58601 | 0 to 1 | ≤ 25 | TBD |
|  | 2 to 3 | ≤ 25 |  |
|  | 4 to 17 | ≤ 17 |  |
|  | 18 | ≤ 11 |  |
|  | 19 to 30 | ≤ 6.5 |  |
|  | 31 to 35 | ≤ 5.5 |  |
|  | 36 to 40 | ≤ 7 |  |
| 58602, 5870, 5880, 5890, 5900, 5910, 5920 | 0 to 1 | ≤ 5.5 | 0.5 |
|  | 2 to 7 | ≤ 4.5 |  |
|  | 8 to 11 | ≤ 3.5 |  |
|  | 12 to 13 | ≤ 2.5 |  |
|  | 14 to 25 | 0 |  |
|  | 26 to 29 | ≤ 1.5 |  |
|  | 30 to 34 | ≤ 3.5 |  |
|  | 35 to 36 | ≤ 4.5 |  |
|  | 37 to 50 | ≤ 5.5 |  |
| Note 1: Applies when NS_33 is signalled Note 2: Applies when NS_NEW is signalled |  |  |  |

Table 8.1.3-10: A-MPR for S-SSB transmission for 30kHz SCS for NS 33 or NS NEW in n47

| Carrier Frequency (MHz) | RBStart | A-MPRBase (dB) | AMPR1Step (dB) |
| --- | --- | --- | --- |
| 58601 | 0 to 1 | ≤ 25 | TBD |
|  | 2 to 4 | ≤ 12 |  |
|  | 5 to 7 | ≤ 10.5 |  |
|  | 8 to 10 | ≤ 8 |  |
|  | 11 to 13 | ≤ 7 |  |
| 58602, 5870, 5880, 5890, 5910, 5920 | 0 to 1 | ≤ 6.5 | 0.25 |
|  | 2 to 4 | ≤ 5.5 |  |
|  | 5 to 6 | ≤ 4.0 |  |
|  | 7 to 9 | ≤ 3.0 |  |
|  | 10 to 12 | ≤ 5.5 |  |
|  | 13 | ≤ 6.5 |  |
| Note 1: Applies when NS_33 is signalled Note 2: Applies when NS_NEW is signalled |  |  |  |

Table 8.1.3-11: A-MPR for S-SSB transmission for 60kHz SCS for NS 33 or NS NEW in47

| Carrier Frequency (MHz) | RBStart | A-MPRBase (dB) | AMPR1Step (dB) |
| --- | --- | --- | --- |
| 58601 | 0 | ≤ 25 | 0 |
| 58602, 5870, 5880, 5890, 5910, 5920 | 0 | ≤ 5 | 0.625 |
| Note 1: Applies when NS_33 is signalled Note 2: Applies when NS_NEW is signalled |  |  |  |

<End of Changes>

# Reference

- R4-2005225,”WF on MPR/A-MPR requirements for S-SSB transmission”, RAN#94-e-bis, April 2020