3GPP TSG-RAN4 Meeting #95-e	R4-2006982

# Online, , 25th May 2020 - 5th Jun 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6847 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to DAPS HO requirements in 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Power imbalance requirement for DAPS handover is not required since the consequence of exceeding imbalance is fallback to source cell or handover failure/RRC establishment as per legacy HO Definition of sycnronous DAPS handover is missing Clean up of Editor’s note |  |  |  |  |  |  |  |  |
| Summary of change: |  | Power imbalance requirement removed Editor’s note on synchronous and asyncronous requirements added : The exact definition of synchronous / asynchronous DAPS handover, and the possible requirements for asynchronous intrafrequency DAPS handover and asynchronous intraband interfrequency DAPS handover are FFS Editor’s note on BW removed as it is decided not to have requirements in case target BW>Source BW Add the interruption requirements for the case when target cell BW is larger than source cell BW in section 5.7.2.1 Add the reference for TDD in section 5.7.2.1 Replaced [TBD] for source cell release message to RRC command Clarified Dhandover2 defintion as also agreed in NR DAPS CR (R4-2002223) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete DAPS handover requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Additional changes on top of technically endorsed CR R4-2005425 are highlighted and shown with author “Further Changes” |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.7	E-UTRAN DAPS Handover

### 5.7.1	Introduction

The purpose of DAPS handover is to change the PCell to another cell.

An intrafrequency DAPS handover is synchronous if the recive time difference between source and target cell is less than or equal to 3µS. In a synchronous intrafrequency DAPS handover, transmission time difference between source and target cell shall be nominally the same if the UE supports simultaneous transmission to source and target cell.

An interfrequency DAPS handover is synchronous if the receive time difference between source and target cell is less than or equal to 33 µS. In a synchronous intrafrequency DAPS handover, transmission time difference between source and target cell shall be less than 34.6 µS if the UE supports simultaneous transmission to source and target cell.

### 5.7.2	Requirements

#### 5.7.2.1	E-UTRAN FDD – FDD

The requirements in this clause are applicable to both intra-frequency and inter-frequency DAPS handovers.

##### 5.7.2.1.1	DAPS Handover delay

Procedure delays for the procedure that can command a DAPS handover are specified in TS 36.331 [2]. DAPS delay is comprised of Dhandover1 and Dhandover2.

When the UE receives an RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 seconds from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure+ Tsearch + TIU + 20 ms

Where

TRRC_procedure is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

A cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 8.1.2.2.1 (FDD) and Clause 8.1.2.2.2 (TDD) for intra-frequency handover and Clause 8.1.2.3.1 (FDD) and Clause 8.1.2.3.2 (TDD) for inter-frequency handover.

After successful RACH procedure of the target cell, when the UE receives an RRC message implying source cell release command, the UE shall accomplish the release actions specified in TS 38.331 [2] within Dhandover2.

Dhandover2 = TRRC_procedure+ Tinterrupt2

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

Tinterrupt2 is defined in clause 5.7.2.1.2.

##### 5.7.2.1.2	Interruption time

During Dhandover1 the UE is allowed an interruption of up to Tinterrupt1 on source cell:

-	Tinterrupt1 is 1 ms for intra-frequency synchronous DAPS handover, when the bandwidth of target cell is no larger than the bandwidth of source cell,

-     Tinterrupt1 is [2]ms for intra-frequency synchronous DAPS handover when the bandwidth of target cell is larger than the bandwidth of source cell,

-	Tinterrupt1 is 2 ms for intra-frequency asynchronous DAPS handover, when the bandwidth of target cell is no larger than the bandwidth of source cell,

-     Tinterrupt1 is [3]ms for intra-frequency asynchronous DAPS handover when the bandwidth of target cell is larger than the bandwidth of source cell,

-	Tinterrupt1 is 5 ms for intra-band inter-frequency DAPS handover, where the source cell and target cell are synchronous,  and 6 ms for asynchronous intra-band inter-frequency DAPS handover.

-	Tinterrupt1 is 1 ms for synchronous inter-band inter-frequency DAPS handover, and 2 ms for asynchronous inter-band inter-frequency DAPS handover.

During Dhandover2 the UE is allowed an interruption of up to Tinterrupt2 on target cell:

-	Tinterrupt2 is 2 ms for synchronous intra-frequency DAPS handover when the bandwidth of target cell is smaller than the bandwidth of source cell

Tinterrupt2 is 3 ms for asynchronous intra-frequency DAPS handover when the bandwidth of target cell is smaller than the bandwidth of source cell

-	Tinterrupt2 is 1 ms for intra-frequency synchronous DAPS handover when the bandwidth of target cell is not smaller than the bandwidth of source cell

Tinterrupt2 is 2 ms for intra-frequency asynchronous DAPS handover when the bandwidth of target cell is not smaller than the bandwidth of source cell

-	Tinterrupt2 is 5 ms for intra-band inter-frequency synchronous DAPS handover, where the source cell and target cell are synchronous,

-	Tinterrupt2 is 6 ms for intra-band inter-frequency asynchronous DAPS handover, where the source cell and target cell are synchronous,

-	Tinterrupt2 is 1 ms for synchronous inter-band inter-frequency DAPS handover, and 2ms for asynchronous inter-band inter-frequency DAPS handover.

#### 5.7.2.2	E-UTRAN FDD – TDD

The requirements in this clause are applicable to DAPS handover from FDD to TDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.7.2.1 apply for this section.

#### 5.7.2.3	E-UTRAN TDD – FDD

The requirements in this clause are applicable to DAPS handover from TDD to FDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.7.2.1 apply for this section.

#### 5.7.2.4	E-UTRAN TDD – TDD

The requirements in this clause are applicable to DAPS handover from TDD to TDD. The requirements in this clause shall apply to UE supporting TDD.

The requirements in clause 5.7.2.1 apply for this section.