3GPP TSG-RAN WG4 #94bis-e	R4-2006984

Online, 25th May – 5th June, 2020

Title: 					InterRAT requirements for high speed train

Source: 				Ericsson

Agenda item:	6.17.1.5

Document for:	Discussion

# Introduction

The outcome of RAN4#94bis was a way forward [1]and an LS on signalling [2]. Main remaining open issues are

- FFS whether additional note such as Note x: Operation with scaling factor M=1.5, M=2 may not be sufficient in all high speed deployments considered in this release of the specifications should be added in NR high speed specifications for both idle and RRC connected state

- FFS whether additional note is added to the spec, such as “Requirements with 0.64s and 1.28s DRX cycle may not be sufficient in all high speed deployments considered in this release of the specifications” should be added in NR high speed specifications

- Cell re-selection requirements on NR- EUTRA inter-RAT measurement in idle mode

- NR- EUTRA Inter-RAT measurement, Cell identification with DRX in connected mode

- Cell re-selection requirements on EUTRA-NR inter-RAT in idle mode

- EUTRA - NR Inter-RAT measurement, Cell identification without DRX and cell identification with DRX in connected mode

- Whether to specify SS-SINR accuracy requirement.

# Discussion

## InterRAT mobility requirements analysis

We begin by reviewing some of the possible use cases for interRAT measurement between NR and LTE in both directions.

- InterRAT reselection from LTE to NR, e.g. so that a user is camped in idle mode on NR to experience 5G service immediately on connection

- InterRAT reselection from NR to LTE e.g. when leaving an area where there is 5G coverage.

- Adding an NR PS Cell to configure EN-DC to get better throughput

- Adding an LTE PSCell to configure NE-DC

- InterRAT handover from LTE to NR-SA to access 5G service

- InterRAT handover from NR to LTE, e.g. when leaving an area where there is 5G coverage.

In all cases there is more time available to perform interRAT mobility, compared with the corresponding scenario for intrafrequency handover/reselection. The reason is that for an intrafrequency procedure, the UE is not expected to be able to detect or receive the neighbor cell when Es/Iot<-6dB, and in this context the serving cell is an interferer. Moreover, the handover is time critical, in that if the UE serving cell Es/Iot becomes too low (e.g. below -6dB or so) before handover is completed, radio link failure will occur.

This is illustrated in figure 1 where we assume that a UE is travelling at 500km/h past a deployment of trackside gNB with ISD=700m. Theoretical Es/Iot based on pathloss is plotted, assuming a carrier frequency of 2.7GHz although an almost identical plot is obtained for any frequency in the FR1 range. Es/Iot reaches a peak when the UE passes a trackside site, and falls below 0dB when the UE would be better served by another cell.

For interRAT mobility, if we assume that the UE can detect another cell when Es/Iot is better than -6dB, the time available is illustrated by T1 for detecting cell 2, and numerically, ~12.96s is available. For intrafrequency mobility, we additionally assume a condition that the serving cell must have -6dB or better Es/Iot to maintain the connection. This is shown by T2, which numerically is ~2.88 seconds.

In practice, the situation for interRAT mobility may be changed by the need to have a trigger for starting interRAT measurements, or starting measurement gaps which may be based on a measurement event trigger from the serving cell. However, in this case it is up to network implementation to define a suitable trigger which works well. For use cases 3 and 4 this trigger is not likely to be based on RRM measurements at all, and in use cases 1,2,5 and 6 a simple solution is to enable interfrequency measurements all the time in the last cell or two before coverage ends (use case 2,6) or the first few LTE cells where the UE is moving into NR coverage. Alternatively RSRP or Ec/Io based triggers can be used, and in any rate the thresholds can be designed so that the UE still has significantly longer to detect the target cell, since it is a less time critical operation.

Figure 1 : Es/Iot of each cell as a UE progresses along a track with 700m ISD, speed = 500km/h

Observation: InterRAT mobility procedures are typically be less time critical than interRAT mobility procedures

In light of this observation, although our view in RAN4#94e was that intrafrequency HST RRM interRAT requirements should be aligned with the corresponding intrafrequency requierment, we now think this may be too demanding from a UE point of view, especially as supporting such performance may have negative implications such as additional power consumption. A more suitable critera seems to be that interRAT delays are within a factor of 2x – 4x the intrafrequency delays. Clealy RAN4 should also avoid overspecifying interRAT requirements, so options relating to direct reuse of  HS requirements for intrafrequency as interRAT requierments may be too strict. On the other hand, some HST intra requierments might have been enhanced less than would ideally be desirable due to power constraints etc, so we propose to target not more than 2x the intrafrequency delay for both PSS/SSS sync (cell detection in LTE) and for meausrement period

Proposal 1 : RAN4 should target an RRM delay for interRAT measurements of not more than 2x the intrafrequency HST delay.

## Cell re-selection requirements for NR- EUTRA inter-RAT

Based on proposal 1, the requirements for idle mode are:

| DRX cycle length [s] | Tdetect,EUTRAN_IRAT_HS [s] (number of DRX cycles) | Tmeasure,EUTRAN_IRAT_HS [s] (number of DRX cycles) | Tevaluate,E-UTRAN_RAT_HS [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 (16) | 0.32(2) | 0.96(6) |
| 0.64 | 5.12 16) | 0.64 (2) | 1.92 (6) |
| 1.28 | 8.96 (14) | 1.28 (2) | 3.84 (6) |
| 2.56 Note1 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

Considering the options in [1], options 1 and 3 exceed the requirements in table 1, and are suitable from a delay perspective. Option 2 is also close to suitable. Option 3 seems to offer a good tradeoff between reselection delay and power consumption.

Proposal 2 : Requirements for Cell re-selection for NR- EUTRA inter-RAT are specified as:

| DRX cycle length [s] | Tdetect,EUTRAN_Intra [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 5.76(16) | 0.64 (2) | 0.96(3) |
| 0.64 | 7.68 (12) | 1.28 (2) | 1.92 (3) |
| 1.28 | 8.96(7) | 1.28 (1) | 3.84 (3) |
| 2.56 Note1 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

## NR- EUTRA Inter-RAT measurement, Cell identification with DRX in connected mode

Based on proposal 1, the requirements for RRC connected mode are:

| DRX cycle length [s] | Tdetect,EUTRAN_IRAT_HS [s] (number of DRX cycles) | Tmeasure,EUTRAN_IRAT_HS [s] (number of DRX cycles) | Tevaluate,E-UTRAN_RAT_HS [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 (16) | 0.32(2) | 0.96(6) |
| 0.64 | 5.12 16) | 0.64 (2) | 1.92 (6) |
| 1.28 | 8.96 (14) | 1.28 (2) | 3.84 (6) |
| 2.56 Note1 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

Considering the options in [1], options 1 and 2 both exceed the requirements in table 1, and are suitable from a delay perspective. Option 1 seems to offer a good tradeoff between reselection delay and power consumption, although there is not a very large difference between option 1 and option 2

Proposal 3 : Requirements for Cell identification for NR- EUTRA inter-RAT are specified as:

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.3.2 apply | Non-DRX requirements in clause 9.4.3.2 apply |
| 0.16<DRx cycle<0.32 | Note1 (15) | Note1 (15) |
| 0.32<= DRx cycle <= 1.28 | Note1 (10) | Note1 (10) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20) | Note1 (20) |
| NOTE 1: The time depends on the DRX cycle length. |  |  |

## Cell re-selection requirements on E-UTRA - NR inter-RAT measurement in idle mode

Following a similar approach as proposal 1, it would also appear any option 1 - 3 from [1] is acceptable. However, since the intra-frequency reselection may not offer sufficient performance when longer DRX cycles are configured, we propose to adopt option 3 as a tradeoff between performance and power consumption.

Proposal 4 : Requirements for Cell re-selection for EUTRA-NR inter-RAT are specified as:

| DRX cycle length [s] | Tdetect,NR [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |
| 1.28 | 8.96(7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2 = M3 = M4 = 1 when SMTC < =40, and M2 = 1.5, M3 = M4 = 2 when SMTC >40 |  |  |  |

## EUTRA-NR Inter-RAT measurement, Cell identification with DRX in connected mode

For this, the main difference between the options seems to be scaling by Nfreq or using CSSF. Since 36.133 exising requirements in section 8.1.2.4.21 are based on Nfreq, we propose to adopt option 1.

Proposal 5 : Requirements for Cell identification for EUTRA-NR inter-RAT are specified as:

| DRX cycle | TPSS/SSS_sync_intra | T SSB_measurement_period_intra | TSSB_time_index_intra |  |
| --- | --- | --- | --- | --- |
| No DRX | Max(600ms, 8 x max(MGRP, SMTC period))×Nfreq | Max(200ms, 8 x max(MGRP, SMTC period))×Nfreq | Max(120ms, 3 x max(MGRP, SMTC period)) ×Nfreq |  |
| DRX cycle < 320ms | Max(600ms, ceil( 8 x M) x max(MGRP, SMTC period, DRX cycle)) ×Nfreq | Max(200ms, ceil(8 x M) x max(MGRP, SMTC period, DRX cycle))×Nfreq | Max(120ms, ceil(3 x M) x max(MGRP, SMTC period, DRX cycle)) ×Nfreq |  |
| DRX cycle≥320ms | 4xM x DRX cycle ×Nfreq | 4xM DRX cycle ×Nfreq | [3] x DRX cycle ×Nfreq |  |
| Note 1: M = 1 when SMTC < =40, and M = 1.5 when SMTC >40 |  |  |  |

# Conclusions

Proposal 1 : RAN4 should target an RRM delay for interRAT measurements of not more than 2x the intrafrequency HST delay.

Proposal 2 : Requirements for Cell re-selection for NR- EUTRA inter-RAT are specified as:

| DRX cycle length [s] | Tdetect,EUTRAN_Intra [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 5.76(16) | 0.64 (2) | 0.96(3) |
| 0.64 | 7.68 (12) | 1.28 (2) | 1.92 (3) |
| 1.28 | 8.96(7) | 1.28 (1) | 3.84 (3) |
| 2.56 Note1 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

Proposal 3 : Requirements for Cell identification for NR- EUTRA inter-RAT are specified as:

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.3.2 apply | Non-DRX requirements in clause 9.4.3.2 apply |
| 0.16<DRx cycle<0.32 | Note1 (15) | Note1 (15) |
| 0.32<= DRx cycle <= 1.28 | Note1 (10) | Note1 (10) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20) | Note1 (20) |
| NOTE 1: The time depends on the DRX cycle length. |  |  |

Proposal 4 : Requirements for Cell re-selection for EUTRA-NR inter-RAT are specified as:

| DRX cycle length [s] | Tdetect,NR [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |
| 1.28 | 8.96(7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2 = M3 = M4 = 1 when SMTC < =40, and M2 = 1.5, M3 = M4 = 2 when SMTC >40 |  |  |  |

Proposal 5 : Requirements for Cell identification for EUTRA-NR inter-RAT are specified as:

| DRX cycle | TPSS/SSS_sync_intra | T SSB_measurement_period_intra | TSSB_time_index_intra |  |
| --- | --- | --- | --- | --- |
| No DRX | Max(600ms, 8 x max(MGRP, SMTC period))×Nfreq | Max(200ms, 8 x max(MGRP, SMTC period))×Nfreq | Max(120ms, 3 x max(MGRP, SMTC period)) ×Nfreq |  |
| DRX cycle < 320ms | Max(600ms, ceil( 8 x M) x max(MGRP, SMTC period, DRX cycle)) ×Nfreq | Max(200ms, ceil(8 x M) x max(MGRP, SMTC period, DRX cycle))×Nfreq | Max(120ms, ceil(3 x M) x max(MGRP, SMTC period, DRX cycle)) ×Nfreq |  |
| DRX cycle≥320ms | 4xM x DRX cycle ×Nfreq | 4xM DRX cycle ×Nfreq | [3] x DRX cycle ×Nfreq |  |
| Note 1: M = 1 when SMTC < =40, and M = 1.5 when SMTC >40 |  |  |  |

# References

- R4-2005358	WF on NR HST RRM requirements CMCC

- R4-2005359	LS on NR HST inter-RAT measurement CMCC