# 3GPP TSG-RAN WG4 Meeting #95-e                     	R4-2007117

# Electronic Meeting, 25 May – 5 June, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE aspects for NR positioning

Agenda item:	6.8.2.4

Document for:	Discussion and Agreement

Introduction

This contribution discusses three open issues on UE aspects for NR positioning, as listed in [2] and [6]. In particular, aspects related to scheduling restriction of PRS symbols in FR1, accuracy requirements for UE Rx-Tx time difference and related to the introduction of new MG patterns are addressed.

Discussion

The discussion in this section refers to the matters depicted above.

Concurrent processing of PRS and other DL channels

At RAN4 #94 there was an incoming LS from RAN1 [1] which included following agreements at RAN1 #99 on NR positioning. An extract of the LS is reflected below.

Agreement: In case DL PRS Resources are processed in the active BWP and there is no measurement gap configured to the UE, at least in FR2, the UE is not expected to process DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE. Behavior in FR1 is up to RAN4 to decide.

The marked aspect above on UE behavior in FR1 was discussed at RAN4 #94-e with no conclusion being reached.

At RAN4 #94bis-e, the discussion was continued whether a scheduling restriction for PRS in FR1 should be introduced and this aspect was left for further study in the WF [2].

Scheduling restriction in FR1 - Further discuss scheduling restrictions for PRS symbols in FR1

In case DL PRS is transmitted in the Active BWP and no measurement gap is configured, RAN1 agreed that the UE is not expected to process DL PRS from serving TRP or neighbor TRP in the same OFDM symbol where other DL signals and channels are transmitted to the UE from the serving TRP, valid in FR2. RAN1 asks RAN4 to decide the UE behavior in FR1.

In our view, the UE behavior in FR1 and FR2 should be the same, i.e. if the serving cell/TRP schedules the UE e.g. for DL data, then the UE in FR1 is not expected to measure PRS of serving cell/TRP or neighbor cell/TRP in the same OFDM symbol, as collision may occur and corrupt the PRS measurement.

The following observation is made:

DL PRS reception from serving or neighbor cell/TRP should have lower priority than reception of DL data/control channels from serving cell/TRP and may be dropped in case of collision.

At RAN4 #94-e, one contribution [3] argued that this can be left UE implementation dependent, provided that there is no significant impact on RSTD measurement accuracy from other signals in the same OFDM symbol.

Proceeding that way, we have a concern that it is unclear how a significant impact can be quantified in terms of acceptable degradation in measurement accuracy. Second, the specification of appropriate UE behavior in terms of a minimum requirement for the UE will get more challenging and an additional UE capability will likely be needed. Third, the UE behaviors in FR1 and FR2 are not aligned, which may thus require higher UE complexity in FR1.

In addition, as already discussed during RAN4 #94-e [4], it is obvious from TS 38.214 that the UE behavior in FR1 and in FR2 are aligned and hence the scheduling restriction applies for both frequency ranges.

Extract from TS 38.214 v16.1.0, clause 5.1.6.5, PRS reception procedure

…

The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS from a non-serving cell is not mapped to any symbol that contains the SS/PBCH block of the same non-serving cell.

…

The UE does not expect to process the DL PRS in the same symbol where other DL signals and channels are transmitted to the UE when there is no measurement gap configured to the UE.

End of Extract

At RAN4#94bis-e, it was commented in [that the rationale for introducing scheduling restriction for PRS is to allow maximum possible bandwidth to be used for PRS, i.e. avoid FDM’ing other data/channels with PRS. We support this reasoning. For achieving best positioning performance, the entire PRS bandwidth is needed and overlaps with SSB have to be avoided.

Therefore, the following proposal is made:

Apply the RAN1 rule for scheduling restriction of PRS symbols with other DL signals and channels in FR2, also for FR1.

Accuracy requirements for UE Rx-Tx time difference

At RAN4 #94bis-e, the discussion on accuracy requirements for UE Rx-Tx time difference was continued and items for further study were captured in the WF [2].

Types of accuracy requirements - Serving vs. neighbor UE Rx-Tx timing difference - Option 1. Define requirements for: - Serving cell - Neighbor cell - Option 2. No separate requirement for serving and neighbor cells

This item was discussed in issue 11-5-3 in [5]. In our view, UE Rx-Tx timing difference requirements should be distinguished between serving cell and neighbor cell. This is reasoned by different side conditions that are applicable for serving cell and neighbor cell.

Thus, the following proposal is made:

Specify UE Rx-Tx time difference accuracy requirements for serving cell and for neighbor cell separately (Option 1).

New measurement gap patterns

At RAN4 #94bis-e, the discussion on new measurement gap patterns was continued and candidate measurement gap patterns were defined in the WF [6].

New MG Patterns - FFS: Introduction of new measurement gap pattern with MGL > 6 ms. - Candidate MGL and MGRP if new MG patterns are specified: - MGL = {10, 20, 40 and 50} ms - MGRP = {80, 160, 320 and 640} ms - Combination of MGL and MGRP is FFS - Independent MG patterns for positioning and RRM measurements is deferred to future release.

As commented in [7], in our view a limited set of new MG patterns with MGL > 6ms should be introduced in Rel-16 to improve performance, in particular for inter-frequency PRS measurements. On the candidates for new MGL, we support all of them except MGL = 50 ms. We are not sure, what is the motivation to introduce an additional gap that is just 25% longer than the next smaller gap of 40 ms. As commented in [7], we propose to replace it by MGL=80 ms in order to keep the factor 2 between new measurement gap lengths.

Therefore, the following proposal is made:

For new MG patterns, replace MGL=50 ms by MGL=80 ms in the list of Candidate MGL’s.

Conclusion

This contribution has discussed three open issues on UE aspects for NR positioning, i.e. on scheduling restriction of PRS symbols in FR1, on accuracy requirements for UE Rx-Tx time difference and on introduction of new MG patterns. Following proposals for agreement are made:

- Apply the RAN1 rule for scheduling restriction of PRS symbols with other DL signals and channels in FR2, also for FR1.

- Specify UE Rx-Tx time difference accuracy requirements for serving cell and for neighbor cell separately (Option 1).

For new MG patterns, replace MGL=50 ms by MGL=80 ms in the list of Candidate MGL’s.

References

- R4-2002765 / R1-1913522, "LS on agreements related to NR Positioning", source: RAN1

- R4-2005378, "WF on NR Positioning UE measurements and reporting", source: Qualcomm Inc

- R4-2000388, "Discussion on UE PRS processing behavior", source: Intel Corporation

- R4-2002311, "Email discussion summary for RAN4#94e_#57_NR_pos_RRM_Part_2", source: Moderator (Ericsson)

- R4-2005837, "Email discussion summary for [94e Bis][115] NR_pos_RRM_Part_1", source: Moderator (Qualcomm Inc)

- R4-2005379, "WF on impact of positioning on RRM requirements", source: Moderator (Ericsson)

- R4-2005398, "Email discussion summary for [94e Bis][116] NR_pos_RRM_Part_2", source: Moderator (Ericsson)