3GPP TSG-RAN WG4 Meeting #95-e	R4-2007185

Electronic Meeting, May 25th – June 5th 2020

Agenda item:	6.17.2.2.2

Source: 		NTT DOCOMO, INC.

Title: 	Views on NR PRACH for high speed

Document for:	Approval

- Introduction

In RAN4 #94bis-e, WF for NR HST PRACH BS demodulation was approved [1]. In this contribution, we provide our views on remaining issues for NR PRACH HST.

2.	Discussion

2.1	Propagation conditions for long preamble format

The following agreements on channel model for PRACH HST were made as below [1, 2]:

RAN4 #92bis [2] - Channel model - AWGN - TDL-C fading channel, Frequency offset is 400Hz RAN4 #94bis-e [1] - TDLC300-100 propagation conditions for long preamble formats - Option 1: Do not to introduce TDLC300-100 fading channel with frequency offset of 400Hz requirements for long preamble formats for HST requirements. - Option 2: Introduce TDLC300-100 fading channel with frequency offset of 400Hz requirements for long preamble formats for HST requirements.

For PRACH long preamble format, a specific configuration for high speed support called as “restricted set” is defined in RAN1 specification. In order to verify the PRACH HST performance, the requirements for restricted set type A and B were introduced in RAN4 specification. On the other hand, for PRACH short preamble format, no specific configuration for high speed support is defined in RAN1 specification. Therefore, even if BS declares high speed support for PRACH short format, it does not set the configuration of HST such as restricted set. Based on this understanding, we summarized the current test coverage of NR PRACH performance requirements in Table 1.

Table 1: Test coverage of NR PRACH performance requirements

| Channel | Frequency offset | Long format |  |  | Short format |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Unrestricted set | Restricted set type A | Restricted set type B | 15kHz SCS | 30kHz SCS |
| AWGN | 0 | X |  |  | X | X |
| TDLC300-100 | 400 Hz | X |  |  | X | X |
| AWGN | 625 Hz |  | X | X |  |  |
| AWGN | 1340 Hz |  | X |  |  |  |
| AWGN | 1740 Hz |  |  |  | X (only BS that declares high speed support) |  |
| AWGN | 2334 Hz |  |  | X |  |  |
| AWGN | 3334 Hz |  |  |  |  | X (only BS that declares to support high speed) |

From Table 1, only long preamble format with restricted set type A and B are not tested under fading channel condition if TDLC300-100 is not introduced. Moreover, in a real deployment scenario, not only HST users but also normal speed users are connected to a same HST cell. For such a deployment scenario, the requirements for TDLC300-100 is important and beneficial to verify the performance. From above discussion, RAN4 should introduce test cases for TDLC300-100 for PRACH restricted set type A and B.

Proposal 1: RAN4 introduces TDLC300-100 for PRACH restricted set type A and B (Keep original agreement made in RAN4 #92bis).

Based on above proposal, the detailed text example is shown below:

------------------------------------------------------------------------------------------------------------------------------------------------

Table 8.4.2.3-1: PRACH missed detection requirements for high speed train, restricted set type A, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 4 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 8 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |

Table 8.4.2.3-2: PRACH missed detection requirements for high speed train, restricted set type B, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 2334 Hz | TBD |
|  | 4 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 2334 Hz | TBD |
|  | 8 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 2334 Hz | TBD |

------------------------------------------------------------------------------------------------------------------------------------------------

2.2	Declaration for PRACH high speed support

The following agreements on declaration were made in [1]:

RAN4 #94bis-e [1] - High speed support declaration for HST PRACH - speed or feature based - Allow BS to declare support for HST including [restricted set type A] and/or [restricted set type B] and/or [A2 for high speed mode] and/or [B4 for high speed mode] and/or [C2 for high speed mode].

As mentioned in section 2.1, there is no specific configuration for PRACH short preamble format to support high speed. Hence, the wording of “high speed mode” might not be suitable for short preamble format.

Proposal 2: RAN4 introduces the following new declaration for PRACH high speed support:

| Declaration identifier | Declaration | Description | Applicability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H | BS type 1-O |
| D.1xx | High speed support for PRACH | Declaration of high speed support for PRACH: restricted set type A, restricted set type B, format A2 for high speed, format B4 for high speed or/and format C2 for high speed. | x | x | x |

3.	Conclusion

In this contribution, we discuss on remaining issues for PUSCH HST. The following proposals are obtained.

Channel model

Proposal 1: RAN4 introduces TDLC300-100 for PRACH restricted set type A and B (Keep original agreement made in RAN4 #92bis).

Declaration

Proposal 2: RAN4 introduces the following new declaration for PRACH high speed support:

| Declaration identifier | Declaration | Description | Applicability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H | BS type 1-O |
| D.1xx | High speed support for PRACH | Declaration of high speed support for PRACH: restricted set type A, restricted set type B, format A2 for high speed, format B4 for high speed or/and format C2 for high speed. | x | x | x |

References

- R4-2005535, “Way forward on Rel-16 NR HST PRACH BS demodulation requirement”, Huawei, RAN4 #94bis-e, April 2020

- R4-1912729, “Way forward on NR HST PRACH”, Samsung, RAN4 #92bis, October 2019.